# 15 April 2003

# Appendix 5 Fugitive emissions from fuels

## Methodology and references

Fugitive emissions from solid fuels (1.B.1.c)

Storage and handling of coal:

Coal mining is not occurring in Denmark, but power plants use a considerable amount of coal. CH4 emission from storage and handling of coal is included in the Danish inventory.

The CH4 emission inventory is based on tier 1 in IPCC Guidelines for National Greenhouse Gas Inventories: Reference Manual. The CH4 emission occurring in Denmark is assumed to be half the post-mining emission.

Coal import refers to the official Danish energy statistics. In inventories for the years 1990-1999 country of origin of the imported coal and underground fraction of each country is taken into account. The emission factor from the 1999 inventory have been applied to the 2000 and 2001 inventories.

Table 1 CH4 emission factor of coal storage and handling in Denmark

| Year | Emission factor 1) | Unit |
| --- | --- | --- |
| 1990 | 336 | g/Mg coal imported |
| 1991 | 310 | g/Mg coal imported |
| 1992 | 327 | g/Mg coal imported |
| 1993 | 458 | g/Mg coal imported |
| 1994 | 477 | g/Mg coal imported |
| 1995 | 485 | g/Mg coal imported |
| 1996 | 485 | g/Mg coal imported |
| 1997 | 485 | g/Mg coal imported |
| 1998 | 430 | g/Mg coal imported |
| 1999 | 474 | g/Mg coal imported |
| 2000 | 474 | g/Mg coal imported |
| 2001 | 474 | g/Mg coal imported |

1) ½ of total post mining emission factor

Fugitive emissions from natural gas (1.B.2.b)

Natural gas transmission and distribution:

Inventories of CH4 emission from gas transmission and distribution is based on annual environmental reports from the Danish gas transmission company, DONG and on a Danish inventory for the years 1999-2001 reported by the Danish gas sector (transmission and distribution companies). The inventories from the Danish gas sector for the years 1999-2001 are based on the work carried out by Marcogas and the International Gas Union (IGU).

In 1990-1999 fugitive CH4 emissions from storage facilities and the gas treatment plant are included in the transmission emission factor below. In 2000-2001 they are registered separately and added.

Table 2 CH4 emission from natural gas transmission

| TRANSMISSION |  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmission rate Mm3 | 1) | 2739 | 3496 | 3616 | 3992 | 4321 | 4689 | 5705 | 6956 | 6641 | 6795 | 7079 | 7289 |
| CH4 emission Mg | 2) |  | 310 | 93 | 186 | 151 | 536 | 183 | 235 | 156 | 191 | 86 | 157 |
| CH4 IEF kg/Mm3 | 3) | 88,619 | 88,619 | 25,653 | 46,642 | 34,981 | 114,272 | 36,000 | 33,784 | 23,490 | 28,109 | 12,149 | 21,539 |

- In 1990-1997 transmission rates refers to Danish energy statistics, in 1998 transmission rate refers to the annual environmental report of DONG, in 1999-2001 emissions refers to DONG/Danish Gas Technology Centre

- In 1991-95 CH4 emissions are based on the annual environmental report from DONG for the year 1995. In 1996-99 the CH4 emission refers to the annual environmental reports from DONG for the years 1996-99. In 2000-2001 the CH4 emission refers to DONG/Danish Gas Technology Centre

- IEF=Emission/transmission_rate. In 1990 the IEF is assumed to be the same as in 1991

Table 3 Additional fugitive CH4 emissions from natural gas storage facilities and venting in gas treatment plant

|  | 2000 | 2001 |
| --- | --- | --- |
| Gas treatment plant | 7,55 Mg | 0 Mg |
| Gas storage facilities | 76,48 Mg | 72,68 Mg |

Table 4 CH4 emission from natural gas distribution

| DISTRIBUTION |  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distribution rate Mm3 | 1) | 1574 | 1814 | 1921 | 2185 | 2362 | 2758 | 3254 | 3276 | 3403 | 3297 | 3181 | 3675 |
| CH4 emission Mg | 2) |  |  |  |  |  |  |  |  |  | 43 | 49 | 56 |
| CH4 IEF kg/Mm3 | 3) | 14,561 | 14,561 | 14,561 | 14,561 | 14,561 | 14,561 | 14,561 | 14,561 | 14,561 | 13,042 | 15,404 | 15,238 |

- In 1999-2001 distribution rates refers to DONG / Danish Gas Technology Centre / Danish gas distribution companies, In 1990-98 distribution rates are estimated from the Danish energy statistics. Distribution rates are assumed to equal total Danish consumption rate minus the consumption rates of sectors that receive the gas at high pressure. The following consumers are assumed to receive high pressure gas: Town gas production companies, production platforms and power plants

- Danish Gas Technology Centre / DONG/ Danish gas distribution companies

- In the years 1999-2001 IEF=CH4 emission / distribution rate. In 1990-1998 an average IEF of 1999-2001 is assumed.

In the data transfer, from the Danish emission database to the CRF, CH4 emission from gas transmission and distribution have unintentionally been added. Thus CH4 from both transmission and distribution have been included in the CRF source category Transmission. The activity data stated in the CRF table is misleading, please refer to the distribution and transmission data above. The error will be corrected in future inventories.

In the year 2001 the length of transmission pipelines including offshore pipeline is 1439 km. The length of distribution pipelines is 19550 km (cast iron 0 km, steel 2442 km, plastics 17108 km). Two natural gas storages are in operation in Denmark. In 2001 the gas input was 399 Mm3 and the withdrawal was 491 Mm3.

Off-shore activities - Production of oil and gas (CRF: 1.B.2.b  SNAP: 050303):

The emissions of VOC from offshore activities have been estimated to be about 2000 tons/year, of this 350 tons/year NMVOC and 1630 tons/year CH4. The estimates are made from 1994 to 2001 and the uncertainty of the emission data is very high. A project is going on to make consistence emission inventories from 1990 to 2001 for oil and gas production including emissions from gas terminals and ships. Emissions from flaring are included in ’Stationary combustion plants’, Appendix 4.

Fugitive emissions from oil (1.B.2.a)

Oil Refineries – Petroleum products processing:

In the production process at the refineries a part of the volatile hydrocarbons (VOC) is emitted to the atmosphere. It is assumed that CH4 accounts for 1 % and NMVOC for 99% of the emissions. The VOC emissions from petroleum refinery processes cover non-combustion emissions from feed stock handling/storage, petroleum products processing, product storage/handling and flaring. SO2 is also emitted from the non-combustion processes and includes emissions from products processing and sulphur recovery plants. The emission calculations are based on information form the Danish refineries and the Energy statistic (Table 6).

## Recalculation and changes

Coal mining and handling

The coal import 1990-2000 has been recalculated based on the new energy statistics.

Natural gas

The CH4 emission from transmission and distribution has been recalculated. Both emission factors and activity rates have been updated according to new knowledge.

## Uncertainty

Estimation of uncertainty is based on the Tier 1 methodology in IPCC Good Practice Guidance (GPG) /2/. The uncertainty of greenhouse gas emissions and the uncertainty of total Global Warming Potential (GWP) is shown in Table 5.

Table 5 Uncertainty of greenhouse gases

| Pollutant | Uncertainty of emission inventory [%] | Uncertainty of emission trend [%] |
| --- | --- | --- |
| CO2 | 16 | 56 |
| CH4 | 80 | 102 |
| N2O | 52 | 56 |
| GWP | 22 | 73 |

The uncertainty of activity rate (coal import) for fugitive emissions from solid fuels is assumed to be 2% referring to GPG page 2.77. The uncertainty of the post mining emission factor is assumed to be 200% also referring to GPG.

Uncertainty of activity rates for oil and gas activities is 15% referring to GPG page 2.92. The uncertainty of emissions factors for CO2 is the uncertainty of emissions factors for flaring. This emission factor uncertainty is 5% (GPG page 2.15). Uncertainties of CH4 and N2O emission factors are both assumed to be 50%.

Table 6 Uncertainty of activity rates and emission factors

|  | Uncertainty Activity rate | Uncertainty Emission factor |
| --- | --- | --- |
| Solid fuel, CO2 | 2 | 200 |
| Oil and gas, CO2 | 15 | 5 |
| Oil and gas, CH4 | 15 | 50 |
| Oil and gas, N2O | 15 | 50 |

Table 6. Oil Refineries. Processed crude oil, emissions and emission factors: