| NIR 2003 | ELC 15/4 2003 |
| --- | --- |

Appendix 7.

The specific methodologies regarding Industrial Processes. CRF Table 2(I), Table 2(I).A-G, Table 2(II), Table 2(II).C,E, and Table 2(II).F.

Mineral Products

Lime and bricks

The CO2 emission from production of Lime is calculated according to the IPCC guidelines, Reference Manual p2.8, with the emission factor of 0,79 kg CO2 / kg Lime. This emission factor is multiplied by activity data from production statistics published by Statistics Denmark, refer the Table below.

The emission factor from production of yellow bricks is calculated the following way. When limestone (CaCO3) is heated it decomposes into lime (CaO) and CO2. Using the molecular weights, implies that 44 kg of CO2 is emitted for every 100 kg CaCO3 decomposed. Since clay used to produce yellow bricks contains 18% of limestone, the emission factor is 0.18*0.44=0.079 kg CO2 / kg yellow bricks. This emission factor is multiplied by activity data from production statistics published by Statistics Denmark. This Statistics accounts for the number of bricks produced, the average weight of a brick is estimated to 2 kg. Refer the Table below for the data and the emissions calculated.

The activity data and the resulting emissions from production of lime and bricks are aggregated in the CRF, in Table 2(I).A-G sheet 1. The underlying data and the resulting aggregated data are given in the table below.

Table. Production of Lime and Bricks, CO2 emission factors and CO2 emissions.

F-gases, HFCs, PFCs and SF6

The data for emissions on HFCs, PFCs and SF6 has been obtained in continuation of work on inventories for previous years. The methods used has been more and more advanced due to IPCC Guidelines becoming available  and the ability to gather more detailed data.

The determination of  emissions includes the quantification and determination of any import and export of HFCs, PFCs, and SF6 contained in products, and takes into account the substances in stock form. This is in accordance with the latest and most accurate method of determination, which is Tier 2 method of the Guidelines .

The following sources of information has been used:

- Importers, agency enterprises, wholesalers, and suppliers

- Consuming enterprises, and trade and industry associations

- Recycling enterprises and chemical waste recycling plants

- Statistics Denmark

- Danish Refrigeration Installers’ Environmental Scheme (KMO)

- Previous evaluations of HFCs, PFCs and SF6

Data is primarily based on respondents from enterprises and importers resulting from a questionnaire survey.

The Tier 2 – Bottom-up analysis used for determination of emissions from HFCs, PFCs, and SF6 covers the following activities:

- Screening of the market for products in the F-gases are used.

- Determination of averages for the content of F-gases per product unit.

- Determination of emissions during the lifetime of products and disposal.

- Identification of technological development trends that have significance for the emission of F-gases.

- Calculation of import and export on the basis of defined key figures, and information from Statistics Denmark on foreign trade and industry information.

The determination of emissions of F-gases is based on a calculation of the actual emission. The actual emission is the emission in the evaluation year, accounting for the time lapse between consumption and emission. The actual emission includes Danish emissions from production, from products during their lifetimes, and from waste products. The actual emissions for the specific areas of application are determined on the grounds of the following analyses:

Consumption and emissions F-gases are wherever possible carried out for individual substances, even though the consumption of certain HFCs has been very limited. This has been done to ensure transparency of evaluation in the determination of GWP values. However, the continued use of a category for “Other HFCs” has been necessary since not all importers and suppliers have specified records of sales for individual substances.

The substances has in the survey been accounted for according to their trade names. In the transfer to the "pure" substances used in the CRF reporting schemes the following relations has been used.

Table. The composition (weight percent) of HFCs mixtures (trade names) as compared to pure HFCs substances reported in the CRF.

Potential emissions.

For the potential emissions put into the CRF Tables the following has been used.

Potential emission = import + production - export - destruction/treatment.

Uncertainty

Uncertainty varies from substance to substance. Uncertainty is greatest for HFC-134a due to its widespread application in products that are imported and exported. The biggest uncertainty in the areas of application is judged to arise from the consumption of HFC-404a and HFC-134a in commercial refrigerators and mobile refrigerators. The uncertainty on year to year data is influenced by the uncertainty on the rate at which the emissions are released, which results in significant differences in the emission determinations in the short term (approx. five years), differences that balances out in the long term.

The average degree of uncertainty in the report’s consumption figures (sold and bought quantities) is estimated at approx. 10-15%, and slightly greater for data regarding the areas of application. The degree of uncertainty in the determination of actual emissions is estimated at 20-25% depending on import/export information on the actual products.