CONTENTS OF ZIP FILES:

| file | contents | description |
| --- | --- | --- |
| vegetation.zip | VSI grid veg_at.dbf | vegetation sensitivity index vegetation parameters (see description below) |
| climate.zip | CSI grid | climate sensitivity index |
| soil.zip | SSI grid soil_at.dbf | soil sensitivity index soil parameters (see description below) |
| final.zip | DSI grid | Desertification sensitivity index |

DESCRIPTION OF ATTRIBUTE TABLES

| veg_at.dbf |  |  |
| --- | --- | --- |
| FIELD | DESCRIPTION | RANGE |
| id | CLC90 raster value | 0-50 |
| f_risk | Fire resistance | 0-2 |
| e_prot | Erosion protection | 0-2 |
| d_rest | Resistance to drought | 0-2 |
| c_horiz | Density of ground coverage | 0-2 |
| veg_ind | (f_risk + e_prot + d_rest + c_horiz) * 10 / 4 | 0 – 20 |

| soil_at.dbf |  |  |
| --- | --- | --- |
| FIELD | DESCRIPTION | RANGE |
| smu | soil mapping unit (link to soil polygons) | 1-15842 |
| country | country code |  |
| mat1sg | parent material code |  |
| texture | texture code (can be combined, ex: 2/5) | 1-5 |
| slope | slope code (can be combined, ex: ac) | a-c |
| subgroup | subgroup name |  |
| slope1 | slope code 1 | a-c |
| slope2 | slope code 2 | a-c |
| slope3 | slope sensivity index 1 | 0-2 |
| slope4 | slope sensivity index 2 | 0-2 |
| slope5 | arithmetic mean for slope sensitvity index | 0-2 |
| text1 | texture code 1 | 1-5 |
| text2 | texture code 2 | 1-5 |
| text3 | texture sensivity index 1 | 0-2 |
| text4 | texture sensivity index 2 | 0-2 |
| text5 | arithmetic mean for texture sensitvity index | 0-2 |
| depth | depth sensivity index | 0-2 |
| parent | parent material sensitivity index | 0-2 |
| soil_ind | (slope5 * text5 * depth * parent) 1/4 | 0 |

Figure 1. Methodology Scheme. 1) Joining the vegetation and soil attribute tables to Corine Land Cover and EU soil map. 2) Creation of the vegetation sensitivity index grid (VSI) obtaining the values from the joined vegetation attribute table and crossing it with the mask of the interest area.  3) Rasterising the soil sensitivity index to a new grid (SSI) and crossing it with the mask.  4) Reprojecting and reclassifying the aridity index grid to a new grid with the climate sensitivity index values (CSI). The new grids have a resolution of about 800 meters that is the lowest of the three input layers. The mask of the interest area has been created adding the complet boundaries of spain and italy to the boundaries of the mediterranean biogeographic region boundaries in order to cover the complete area of these two countries. 5)  Geometric mean of the three layers.

VSI

SSI

CSI

veg_at

CLC 90

ia_100

soil_at

DSI

Soil Map

mask

2

3

4

5

1

1