# URBAN ATLAS – LUZ DELIVERY REPORT

| LUZ | ROTTERDAM - NL |
| --- | --- |

GENERAL INFORMATION

| Area of product | 708,24 km² |  |
| --- | --- | --- |
| Format (shapefile + prj) | Ok |  |
| SOIL SEALING cover and projection | Ok |  |
| COTS validation | Ok |  |

Coordinate system reference

Projection: UTM 31

Central Meridian: 3°

Latitude of Origin: 0°

False Easting: 500000,00

False Northing: 0,00

Ellipsoid: WGS84

#### IMAGE data used

| Satellite & Sensor | ### Image Name | ### Date (y/m/d) | ### Remark (e.g. clouds) |
| --- | --- | --- | --- |
| Spot 5 2,50 m | 00182800021 (PS) | 2006/09/13 | Pan-sharpening Images created by SPOT IMAGE |
|  | 00182800023 (PS) | 2006/10/15 |  |

Ancillary data used (thematic data, satellite images, aerial photos, city maps)

| Id. | Data source/type | Title (if relevant) | Date de production (y/m/d) | Scale (spatial detail) | Remark |
| --- | --- | --- | --- | --- | --- |
| Google Earth |  |  |  |  |  |
| Inner city map | Falk |  |  | 1 : 13 500 and 1 : 7500 | Scan + georef. |

CAPI/ DIGITAL CLASSIFICATION

#### Photointerpreter(s)

| Name/surname | interpretation |  |  |
| --- | --- | --- | --- |
|  | start (y/m/d) | end (y/m/d) | Remark |
| Courmont Yoann | 09/02/24 | 09/03/11 | Layer well adjusted to roads and railrods but many micropolygons noticed on the central reservations and in between a two-ways road . We could not notice canals within the urban areas |

Internal quality control

| Internal quality check | Date of control (y/m/d) | Result | Remark (errors, corrections, etc.) |
| --- | --- | --- | --- |
| Anne crépin | 09/03/17 | OK | Unsure about specific areas where we could notice countryside house which was coded 11 |

Quality assesment

| Internal quality check | Date of control (y/m/d) | Result | Remark (errors, corrections, etc.) |
| --- | --- | --- | --- |
| IGNFI | 09/06/29 | ok |  |

CONFUSION MATRIX

Urban strata accuracy calculation

The sample population is that of the entire LUZ and non urban classes are aggregated in class 600.

The main advantage of this method is to provide both omission and commission errors on the urban strata (omission - commission class 1xx / class 600).