Indicator Fact Sheet – Theme AIR and CLIMATE CHANGE

CSI 001   Emissions of Acidifying Substances

Key message In the period from 1990 to 2004 decreasing trend in emission of acidifying substances was registered, so the present state may be considered satisfactory. Reduced emissions are mainly the consequence of the fact that during the war in Croatia (1990-1995) main industrial complexes remarkably reduced or even stopped their production. But also, it is not to neglect the fact that nowadays many former polluters introduced “cleaner” modern technologies in their production, which is the clear demand of official policy and market itself as well. Due to the respective weigh of each of the three substances, the proportion of NH3 and NOx emissions increased from 18% in 1980 to 25% in 2004 and for NH3 from 31% in 1990 to 43% in 2004. However, absolute emissions decreased. This is mainly due to the significant decrease of SO2 emissions during the same period (51 % in 1980 to 32 % in 2003).

Figure 1: Indexes of absolute emissions of substances (without nature) that contribute to acidification in the Republic of Croatia, from 1990 to 2004 (base 100 in 1990)

Source: CEA

Figure 2: Percentage of emissions of substances (without nature) that contributed to acidification in the Republic of Croatia in 1990 and 2004

Source: CEA

Results and assessment

Policy relevance

International Conventions:

Croatia is a party to the Convention on Long-range Trans-boundary Air Pollution based on the Notification of Succession (OG 12/93 – International Treaties). The application of The Convention in the Republic of Croatia started 10 August 1991. Among eight Protocols to the Convention, Croatia accepted the Protocol on Long-term Financing of the Cooperative Programme for Monitoring and Evaluation of the Long-range Transmission of Air Pollutants in Europe (EMEP) and Protocol on Further Reduction of Sulphur Emissions. The Republic of Croatia signed but yet not ratified yet the Protocol to Abate Acidification, Eutrophication and Ground level Ozone, Protocol on Persistent Organic Pollutants (POPs) and Protocol on Heavy Metals. Adopting the implemented legislation, subordinate rules and regulations, Croatia fulfilled its obligations under the Convention and ratified Protocols. The legislative framework relevant for the Convention and the ratified Protocols are well developed. The following laws and subordinate regulations are in force:

National Legislastion:

· Law on Air Quality Protection (OG 48/95)

· Decree on Recommended and Limit Air Quality Values (OG 101/96, 2/97)

Assessment

It can be noticed that the acid equivalent has an overall decreasing trend as a result of downward trend of SO2 and approximately continuous trend of NO2 and  NH3.

Sulphur dioxide emissions  SO2

In accordance with the calculated results the level of SO2 emission observed in 2004 reached 60.3 kt. The trend shows that emissions of SO2 have decreased by 17% compared to the emission in 2003 and by 65 % since 1990. It can be observed that SO2 emission in 2004 was lower than the target value set in the framework of the Protocol on further reduction of SO2 emission (117 kt), but also, that it is lower than the target set under the MPME Protocol (70 kt). Therefore, in long-term period the SO2 emission must be retained under 70 kt, which is also one of the goals set according to the Strategy for environmental protection. Since 1990, SO2 emission has the overall decreasing trend due to consumption of fossil fuel with lower sulphur content. In 2004, the emission of SO2 was generally assigned to the combustion in energy transformation industry (42% of total SO2 emissions). The sector - combustion in manufacturing industry, participated with 21% in total emission of SO2, the production processes and road transport participated with 11% and 12%, and non-industrial combustion plants participated with 9%.

During the period from 1990 to 2004, the decrease of SO2 emissions was achieved in almost all sectors and the greatest decrease of SO2 emission was in sector - Other mobile source and machinery (-100%) but the contribution of that sector to the national total emission of SO2 in 2004 was low (around 5%). SO2 emission increased only in road transport (+66%) due to increased number of vehicles in traffic

Nitrogen dioxide NO2

The emission of NOx in 2004 was 68.9 kt which is 3% lower than the year before and 20% lower compared to 1990. The main source of NOx emission is still road transport (40% of total emission), but its contribution has been decreasing steadily since 1999 (-11.1%), as a result of vehicles gradually being equipped with catalytic converters. It is expected that the NOx emissions from this sector will continue decreasing in coming years. Other mobile source and machinery also significantly contributes to national total NOx emissions (24.7%). Emission of NOx from stationary energy sources participated with 33% to the national total amount, mainly from the combustion in energy transformation industry (16.2%). The rest of the emission comes essentially from production processes (3.1% of the national total) and the agriculture sector (1.1%). It can be concluded that the NOx emission is mainly result of fossil fuel consumption.

Ammonia NH3

The emission of NH3 in 2004 was 44.2 t which was approx. 3% higher then in 2003 and approx. 22% lower than in 1990.

The main source of NH3 emission is agriculture sector (approx.91%) with the following main sources: livestock (74%) and crops (20%) in 2004. Slightly higher emission of NH3 in 1996 and 1997 were the result of higher number of domestic animals. The rest of the emissions came essentially from the production processes particularly from ammonia, nitric acid and NPK fertilisers’ production (7.5% of the national total), road transport (1.4%) and stationary energy sectors (around 0.4%). Emissions from road transport increase regularly, due to the vehicles being equipped with catalytic converters considering NH3 being  catalyst in catalytic converters for NOx removing.

NH3 emission shows the decreasing trend in the period from 1990 to 2004, however, the increase in the past few years must not be ignored. If no additional measures are going to be implemented, with the predictable increase of livestock number in coming years, it will be more difficult to reach the target set under MPME Protocol.

In the framework of MPME Protocol, Croatia is obliged to keep the NH3 emission until 2010 at the same level as in the base (1990) year.

Recalculation carried out in the report for the year 2003, showed higher emission of NH3 in the period from 1990 to 2004. A recalculated value for NH3 emission in the base year (57 t) is 35% higher then the target set under the MPME Protocol (37 kt). That could lead to change of the target value for NH3.

Data coverage:

Table 1: Emission of acidifying substances that contribute to the acidification expressed in acid equivalent (*)

| YEAR | SO2 % Aeq | NOx % Aeq | NH3 % Aeq | Aeq(**) kt |
| --- | --- | --- | --- | --- |
| 1990 | 51 | 18 | 31 | 10.60 |
| 1991 | 42 | 18 | 41 | 7.95 |
| 1992 | 47 | 17 | 36 | 7.41 |
| 1993 | 46 | 17 | 36 | 7.29 |
| 1994 | 47 | 18 | 35 | 7.33 |
| 1995 | 40 | 21 | 39 | 6.17 |
| 1996 | 33 | 23 | 44 | 6.18 |
| 1997 | 39 | 21 | 40 | 6.87 |
| 1998 | 47 | 22 | 31 | 7.12 |
| 1999 | 44 | 22 | 33 | 7.20 |
| 2000 | 35 | 25 | 40 | 6.15 |
| 2001 | 36 | 26 | 39 | 6.03 |
| 2002 | 37 | 24 | 39 | 6.33 |
| 2003 | 36 | 24 | 40 | 6.35 |
| 2004 | 32 | 25 | 43 | 5.98 |

Source: CEA

(*) Emissions concern only anthropogenic ones (without nature)

(**) Aeq: indicator of acid equivalent calculated on the bases of potential fixation of H+ ion: 0.0313 for SO2, 0.0217 for NOx and 0.0588 for NH

Table 2: Emissions of the substances which cause acidification on the territory of the

Croatia in 2004

| #### Emissions, the year 2004 – t/yr | SO2 | NOx | NH3 |
| --- | --- | --- | --- |
| Combustion in energy transformation industry | 25655.0 | 11196.2 | 11.9 |
| Non-industrial combustion plants | 5524.9 | 3899.2 | 122.6 |
| Combustion in manufacturing industry | 12558.5 | 6334.2 | 32.0 |
| Production processes | 6575.5 | 2131.7 | 3331.1 |
| Extraction and distribution of fossil fuels and geothermal energy | 0 | 0 | 0 |
| Solvent and other product use | 0 | 0 | 0 |
| Road transport | 7313.8 | 27553.4 | 612.8 |
| Other mobile source and machinery | 2720.0 | 17035.8 | 2.4 |
| Waste treatment and disposal | 0.9 | 25.3 | 0 |
| Agriculture | 0 | 726.8 | 40050.8 |
| Other source and sinks | 0 | 0 | 0 |
| TOTAL | 60349 | 68903 | 44164 |
| Emissions in relation to population kg/citizen | 13.6 | 15.5 | 9.9 |
| Emissions in relation to area kg/km2 | 1066 | 1217 | 780 |
| Emissions in relation to GDP. g/US$ | 1.76 | 2.01 | 1.29 |
| Proportion - % | SO2 | NOx | NH3 |
| Combustion in energy transformation industry | 42.5 | 16.2 | 0 |
| Non-industrial combustion plants | 9.2 | 5.7 | 0.3 |
| Combustion in manufacturing industry | 20.8 | 9.2 | 0.1 |
| Production processes | 10.9 | 3.1 | 7.5 |
| Extraction and distribution of fossil fuels and geothermal energy | 0 | 0 | 0 |
| Solvent and other product use | 0 | 0 | 0 |
| Road transport | 12.1 | 40.0 | 1.4 |
| Other mobile source and machinery | 4.5 | 24.7 | 0 |
| Waste treatment and disposal | 0 | 0 | 0 |
| Agriculture | 0 | 1.1 | 90.7 |
| Other source and sinks | 0 | 0 | 0 |
| TOTAL | 100 | 100 | 100 |

Source: CEA

Table3: amounts (ktonnes) of the substances which cause acidification on the territory

of Croatia 1990 – 2004.

| year | SO2 (kt) | NO2 (kt) | NH3 (kt) | total |
| --- | --- | --- | --- | --- |
| 1990. | 180,006 | 87,567 | 37,102 | 316,975 |
| 1991. | 107,981 | 65,042 | 31,707 | 213,63 |
| 1992. | 106,732 | 56,198 | 26,844 | 197,674 |
| 1993. | 113,714 | 59,257 | 25,483 | 206,104 |
| 1994. | 89,259 | 65,477 | 24,206 | 186,542 |
| 1995. | 70,383 | 65,713 | 24,901 | 168,197 |
| 1996. | 66,167 | 68,603 | 23,368 | 166,338 |
| 1997. | 80,355 | 73,274 | 22,969 | 185,948 |
| 1998. | 89,493 | 76,038 | 23,344 | 198,225 |
| 1999. | 90,723 | 72,326 | 31,826 | 203,525 |
| 2000. | 58,23 | 77,536 | 30,118 | 174,034 |
| 2001. | 62,665 | 69,602 | 52,396 | 192,113 |
| 2002. | 67,474 | 69,29 | 50,657 | 194,921 |
| 2003. | 72,977 | 70,966 | 42,936 | 195,479 |
| 2004. | 60,349 | 68,903 | 44,164 | 181,916 |

Source: CEA

Metadata

Legal bases for the indicator: LRTAP Convention Data source(s) : Emissions of pollutants into the air on the territory of Croatia for the 2004 according to CLRTAP convention , EKONERG. Geographical coverage: national Temporal coverage: 1990. - 2004. Methodology and frequency of data collection: According to SNAP 97 nomenclature of the EMEP/CORINAIR methodology, annually. In the inventory preparation, two approaches were applied: 1. "Bottom-up" approach – used for point sources emission calculation 2. "Top-down" approach – based on aggregated data (energy balance and statistical data) It is possible to obtain satisfactory activity data only for large point sources. Therefore, the application of both methods is justified. Methodology of data manipulation (measurements): According to Air Pollution Studies No. 15, Guidelines for Estimating and Reporting Emission Data under the CLRTAP. Quality information (at data level): Strength and weakness (at data level): Lack of experts/institutions, adequate equipment and financial resources for data collection; involved institutions do not have clear mandates and responsibilities, confidentiality problems, delays in contracting between Ministry/Agency and the executive institutions.

Population in the Republic of Croatia: 4.442.000 – source: Central Bureau of Statistics (CBS)

Land area (km2): 56.594 – source: Central Bureau of Statistics (CBS)

GDP (US$): 2,881E+10 – source: Central Bureau of Statistics (CBS