Indicator Fact Sheet – Theme AIR and CLIMATE CHANGE

CSI 003   Emissions of Primary Particles and Secondary Particulate Precursors

Key message In the year 2004 total emission of primary particles and secondary particulate precursors decreased for 31% in comparison to the year 1990. This is mainly the result of lower production in manufacturing industry, and consequently, decrease in fossil fuel combustion, but partly due to introduction of “cleaner” production process. However, there was a significant increase of PM10 emissions registered in road transport (+67% during the period from 1990 to 2004), which is connected with high increase in number of vehicles on the roads.

Figure 1: The PM10 emission in Croatia from 1990 to 2004

0

1.000

2.000

3.000

4.000

5.000

6.000

7.000

8.000

9.000

10.000

11.000

12.000

13.000

1990

1991

1992

1993

1994

1995

1996

1997

1998

1999

2000

2001

2002

2003

2004

t PM10/year

SEKTOR 01

SEKTOR 02

SEKTOR 03

SEKTOR 04

SEKTOR 05

SEKTOR 06

SEKTOR 07

SEKTOR 08

SEKTOR 09

SEKTOR 10

Source: CEA; Ekonerg

Figure 2: The percentage of the PM10 emission in  Croatia by sectors

Figure 3: The SO2 emissions in Croatia from 1990 to 2004.

0

15.000

30.000

45.000

60.000

75.000

90.000

105.000

120.000

135.000

150.000

165.000

180.000

1990

1991

1992

1993

1994

1995

1996

1997

1998

1999

2000

2001

2002

2003

2004

t SO

2

/year

SECTOR 01

SECTOR 02

SECTOR 03

SECTOR 04

SECTOR 05

SECTOR 06

SECTOR 07

SECTOR 08

SECTOR 09

SECTOR 10

Protocol for SO2

MPME Protocol

Protocol on further decrease in SO2 emissions (117 000 kt)

Source: CEA; Ekonerg

Figure 4: The percentage of the SO2 emissions in Croatia by sectors

Source: CEA; Ekonerg

Figure 5: The NOx emissions in Croatia from 1990 to 2004.

0

10.000

20.000

30.000

40.000

50.000

60.000

70.000

80.000

90.000

100.000

1990

1991

1992

1993

1994

1995

1996

1997

1998

1999

2000

2001

2002

2003

2004

t NOx/year

SEKTOR 01

SEKTOR 02

SEKTOR 03

SEKTOR 04

SEKTOR 05

SEKTOR 06

SEKTOR 07

SEKTOR 08

SEKTOR 09

SEKTOR 10

MPME protokol (87.000)

MPME Protocol (87.000 kt)

Source: CEA; Ekonerg

Figure 6: The percentage of the NOx emissions in Croatia by sectors

Source: CEA; Ekonerg

Figure 7: The NH3 emissions in Croatia from 1990 to 2004

0

5.000

10.000

15.000

20.000

25.000

30.000

35.000

40.000

45.000

50.000

55.000

60.000

1990

1991

1992

1993

1994

1995

1996

1997

1998

1999

2000

2001

2002

2003

2004

t NH

3

/year

SEKTOR 01

SEKTOR 02

SEKTOR 03

SEKTOR 04

SEKTOR 05

SEKTOR 06

SEKTOR 07

SEKTOR 08

SEKTOR 09

SEKTOR 10

MPME protokol (30.000 kt)

MPME protokol (30.000 kt)

Source: CEA; Ekonerg

Figure 8: The precentage of the NH3 emissions in Croatia by sectors

Source: CEA; Ekonerg

Figure 9: Emission of primary and secondary fine particles in Croatia 1990-2004.

Source: CEA; Ekonerg

Results and assessment

Policy relevance

International Conventions:

-Convention on Long-range Trans-boundary Air Pollution; Croatia is a party to

Convention upon the Notification of Succession (OG 12/93 – International Treaties). The application of The Convention in Croatia started in 1991. Among eight Protocols to the Convention, Croatia accepted the Protocol on Long-term Financing of the Cooperative Programme for Monitoring and Evaluation of the Long-range Transmission of Air Pollutants in Europe (EMEP) and Protocol on Further Reduction of Sulphur Emissions. The Republic of Croatia signed but yet not ratified the Protocol to Abate Acidification, Eutrophication and Ground level Ozone, Protocol on Persistent Organic Pollutants (POPs ) and Protocol on Heavy Metals. Adopting the implemented legislation, subordinate rules and regulations, Croatia fulfilled its obligations under the Convention and ratified Protocols. The legislative framework relevant for the Convention and the ratified Protocols are well developed.

National Legioslation:

-Law on Air Quality Protection (OG 48/95)

-Decree on Recommended and Limit Air Quality Values (OG 101/96, 2/97)

Assessment

The PM10 emission was 8.5 kt in 2004, i.e. 31% lower than in 1990.

The following sectors participate in PM10 emissions: non-industrial combustion plants with 46.3% of total national emission in 2004, combustion in manufacturing industry with 21.4%, combustion in energy transformation industry with 11.1%, production processes with 13.9% and road transport with 7.3%.

The significant increase of PM10 emissions was in road transport (+67% during the period from 1990 to 2004). The greatest decrease was in combustion in manufacturing industry (-42%) due to decrease of manufacturing i.e. decrease of the fossil fuel combustion.

Sulphur dioxide emissions SO2

In accordance with the calculated results the level of SO2 emission observed in 2004 reached 60.3 kt. The trend shows that emissions of SO2 have decreased by 17% compared to the emission in 2003 and by 65 % since 1990. It can be observed that SO2 emission in 2004 was lower than the target value set in the framework of the Protocol on further reduction of SO2 emission (117 kt), but also, that it is lower than the target set under the MPME Protocol (70 kt). Therefore, in long-term period the SO2 emission must be retained under 70 kt, which is also one of the goals set according to the Strategy for environmental protection. Since 1990, SO2 emission has the overall decreasing trend due to consumption of fossil fuel with lower sulphur content. In 2004, the emission of SO2 was generally assigned to the combustion in energy transformation industry (42% of total SO2 emissions). The sector - combustion in manufacturing industry accounted with 21% in total emission of SO2, the production processes and road transport accounted with, 11% and 12%, and non-industrial combustion plants accounted with 9%.

During the period from 1990 to 2004, the decrease of SO2 emissions was achieved in almost all sectors and the greatest decrease of SO2 emission was in Other mobile source and machinery (-100%) but the contribution of that sector to the national total emission of SO2 in 2004 was low (around 5%). SO2 emission increased only in road transport (+66%) due to greater number of vehicles on the roads.

Nitrogen dioxide NO2

The emission of NOx in 2004 was 68.9 kt which is 3% lower than the year before and 20% lower compared to 1990. The main source of NOx emission is still road transport (40% of total emission), but its contribution has been decreasing steadily since 1999 (-11.1%), as a result of vehicles gradually being equipped with catalytic converters. It is expected that in coming years, the NOx emissions from this sector will continue decreasing. Other mobile source and machinery also significantly contributes to national total of NOx emissions (24.7%). Emission of NOx from stationary energy sources participated with 33% to the national total, mainly from the combustion in energy transformation industry (16.2%). The rest of the emission comes essentially from production processes (3.1% of the national total) and the agriculture sector (1.1%). It can be concluded that the NOx emission is mainly result of fossil fuel consumption.

Ammonia NH3

The emission of NH3 in 2004 was 44.2 t which was about 3% higher then in 2003 and about 22% lower than in 1990.

The main source of NH3 emission is agriculture (around 91%) with the following main sources: livestock (74%) and crops (20%) in 2004. Slightly higher emission of NH3 in 1996 and 1997 were the result of higher number of animals. The rest of the emissions came essentially from the production processes particularly from ammonia, nitric acid and NPK fertilisers’ production (7.5% of the national total), road transport (1.4%) and stationary energy sectors (around 0.4%). Emission from road transport increases regularly, due to the vehicles being equipped with catalytic converters because NH3 is a catalyst in catalytic converters for NOx removing.

NH3 emission shows the decreasing trend in the period from 1990 to 2004, however, the increase in the past few years must not be ignored. If no additional measures are implemented, with the predictable increase of livestock number in coming years, it will be more difficult to reach the target set under MPME Protocol.

In the framework of MPME Protocol Croatia is obliged to keep the NH3 emission until 2010  at the same level as in the base (1990) year.

Recalculation carried out in the report for the year 2003, showed higher emission of NH3 in the period from 1990 to 2004. A recalculated value for NH3 emission in the base year (57 t) is 35% higher then the target set under the MPME Protocol (37 kt). That could lead to change of the target value for NH3.

Data coverage:

Table1: amounts (ktonnes) of secondary particulate precursors and primary particles by years

| year | SO2 kt | NO2 kt | NH3 kt | PM10 kt | total |
| --- | --- | --- | --- | --- | --- |
| 1990. | 180,006 | 87,567 | 37,102 | 12,3 | 316,975 |
| 1991. | 107,981 | 65,042 | 31,707 | 8,9 | 213,63 |
| 1992. | 106,732 | 56,198 | 26,844 | 7,9 | 197,674 |
| 1993. | 113,714 | 59,257 | 25,483 | 7,65 | 206,104 |
| 1994. | 89,259 | 65,477 | 24,206 | 7,6 | 186,542 |
| 1995. | 70,383 | 65,713 | 24,901 | 7,2 | 168,197 |
| 1996. | 66,167 | 68,603 | 23,368 | 8,2 | 166,338 |
| 1997. | 80,355 | 73,274 | 22,969 | 9,35 | 185,948 |
| 1998. | 89,493 | 76,038 | 23,344 | 9,35 | 198,225 |
| 1999. | 90,723 | 72,326 | 31,826 | 8,65 | 203,525 |
| 2000. | 58,23 | 77,536 | 30,118 | 8,15 | 174,034 |
| 2001. | 62,665 | 69,602 | 52,396 | 7,45 | 192,113 |
| 2002. | 67,474 | 69,29 | 50,657 | 7,5 | 194,921 |
| 2003. | 72,977 | 70,966 | 42,936 | 8,6 | 195,479 |
| 2004. | 60,349 | 68,903 | 44,164 | 8,5 | 181,916 |
| total | 1316,508 | 1045,792 | 492,021 | 127,3 |  |

Source: CEA; Ekonerg

Table 2: amounts for secondary particulate precursors weighted with particulate formation potencial and primary particles

SO2: 0,54;  NOX :0,88;  NH3: 0,64

| year | SO2 kt | NO2 kt | NH3 kt | PM10 kt | total |
| --- | --- | --- | --- | --- | --- |
| 1990. | 97,203 | 77 | 23,7 | 12,3 | 210,203 |
| 1991. | 58,309 | 52,2 | 20,29 | 8,9 | 139,699 |
| 1992. | 57,63 | 49,4 | 17,18 | 7,9 | 132,11 |
| 1993. | 61,405 | 52,1 | 16,3 | 7,6 | 137,405 |
| 1994. | 48,199 | 57,6 | 15,49 | 7,6 | 128,889 |
| 1995. | 38 | 57,8 | 15,93 | 7,2 | 118,93 |
| 1996. | 35,73 | 60,3 | 14,95 | 8,2 | 119,18 |
| 1997. | 43,39 | 64,4 | 14,7 | 9,35 | 131,84 |
| 1998. | 48,32 | 66,9 | 14,9 | 9,35 | 139,47 |
| 1999. | 48,99 | 63,6 | 20,36 | 8,65 | 141,6 |
| 2000. | 31,44 | 68,23 | 19,27 | 8,15 | 127,09 |
| 2001. | 33,83 | 61,2 | 33,53 | 7,45 | 136,01 |
| 2002. | 36,43 | 60,9 | 32,42 | 7,5 | 137,25 |
| 2003. | 39,407 | 62,4 | 27,47 | 8,6 | 137,877 |
| 2004. | 32,58 | 60,6 | 28,26 | 8,5 | 129,94 |

Source: CEA; Ekonerg

Table 3: emissions of the Primary Particles and Secondary Particulate Precursors on the territory of the Republic of Croatia in 2004

| #### Emissions, the year 2004 – t/yr | SO2 | NOx | PM10 | NH3 |
| --- | --- | --- | --- | --- |
| Combustion in energy transformation industry | 25655.0 | 11196.2 | 945.6 | 11.9 |
| Non-industrial combustion plants | 5524.9 | 3899.2 | 3947.8 | 122.6 |
| Combustion in manufacturing industry | 12558.5 | 6334.2 | 1826.4 | 32.0 |
| Production processes | 6575.5 | 2131.7 | 1189.8 | 3331.1 |
| Extraction and distribution of fossil fuels and geothermal energy | 0 | 0 | 0 | 0 |
| Solvent and other product use | 0 | 0 | 0 | 0 |
| Road transport | 7313.8 | 27553.4 | 624.4 | 612.8 |
| Other mobile source and machinery | 2720.0 | 17035.8 | 0 | 2.4 |
| Waste treatment and disposal | 0.9 | 25.3 | 0 | 0 |
| Agriculture | 0 | 726.8 | 0 | 40050.8 |
| Other source and sinks | 0 | 0 | 0 | 0 |
| TOTAL | 60349 | 68903 | 8534 | 44164 |
| Emissions in relation to population kg/citizen | 13.6 | 15.5 | 1.9 | 9.9 |
| Emissions in relation to area kg/km2 | 1066 | 1217 | 151 | 780 |
| Emissions in relation to GDP. g/US$ | 1.76 | 2.01 | 0.25 | 1.29 |
| Proportion - % | SO2 | NOx | PM10 | NH3 |
| Combustion in energy transformation industry | 42.5 | 16.2 | 11.1 | 0 |
| Non-industrial combustion plants | 9.2 | 5.7 | 46.3 | 0.3 |
| Combustion in manufacturing industry | 20.8 | 9.2 | 21.4 | 0.1 |
| Production processes | 10.9 | 3.1 | 13.9 | 7.5 |
| Extraction and distribution of fossil fuels and geothermal energy | 0 | 0 | 0 | 0 |
| Solvent and other product use | 0 | 0 | 0 | 0 |
| Road transport | 12.1 | 40.0 | 7.3 | 1.4 |
| Other mobile source and machinery | 4.5 | 24.7 | 0 | 0 |
| Waste treatment and disposal | 0 | 0 | 0 | 0 |
| Agriculture | 0 | 1.1 | 0 | 90.7 |
| Other source and sinks | 0 | 0 | 0 | 0 |
| TOTAL | 100 | 100 | 100 | 100 |

Source: CEA; Ekonerg

Metadata

Legal bases for the indicator: CLRTAP convention, Protocol on Further Reduction of Sulphur Emissions, Croatian Law on protection of Air (Official Gazette 178/04), Decree on Recommended and Limit Air Quality Values (Official Gazette 101/96, 2/97). Data source(s): Emissions of pollutants into the air on the territory of the Republic of Croatia for the 2004 to CLRTAP convention. Geographical coverage: national Temporal coverage: 1990 - 2004. Methodology and frequency of data collection: According to SNAP 97 nomenclature of the EMEP/CORINAIR methodology, annually. In the inventory preparation, two approaches were applied: 1. "Bottom-up" approach – used for point sources emission calculation 2. "Top-down" approach – based on aggregated data (energy balance and statistical data) It is possible to obtain satisfactory activity data only for large point sources. Therefore, the application of both methods is justified. Methodology of data manipulation (measurements): According to Air Pollution Studies No. 15, Guidelines for Estimating and Reporting Emission Data under the CLRTAP. Quality information (at data level): The legislation for implementation of the Convention and ratified Protocols has been adopted. Cooperation in the programmes for monitoring and evaluation of the long range transmission of air pollutants in Europe through participation in the working groups under the Convention and through the exchange of data within the framework of the relevant ICPs has been established. Strength and weakness (at data level): Lack of experts/institutions, adequate equipment and financial resources for data collection, involved institutions don’t have clear mandates and responsibilities, confidentiality problems, delays in contracting between Ministry/Agency and the executive institution.

Population in the Republic of Croatia: 4.442.000 – source: Central Bureau of Statistics (CBS)

Land area (km2): 56.594 – source: Central Bureau of Statistics (CBS)

GDP (US$): 2,881E+10 – source: Central Bureau of Statistics (CBS