## ENER20 Combined Heat and Power (CHP)

Key message

The share of electricity produced from combined heat and power (CHP) in the EU-27 remained the same between 2006 and 2007 at 10.9%, despite strong policy support to promote the technology in many Member States. High gas prices and relatively low electricity prices reduced the competitiveness of gas-fired CHP-plants. Greater incentives will be needed to reach the EU-15 indicative target of 18 % of CHP electricity in gross electricity production by 2010 (currently 10.2% of total gross electricity production in EU-15). In the new Member States the share of CHP in electricity production is 15.0% of total gross electricity production in these countries, approximately 1.5 times that in the EU-15.

Rationale

CHP or “cogeneration” implies that heat and electricity are produced simultaneously in one process. Use of combined heat and power can help to improve the overall efficiency of electricity and heat production as these plants combine electricity production technologies with heat recovery equipment. Increasing the conversion efficiency of power generation through the use of CHP helps to reduce the environmental impact of power generation. A higher penetration of CHP will also help achieving the 20% target for GHG emissions, the 20% target for renewable energy in final energy consumption and the 20% reduction in gross inland consumption compared to BAU by 2020.

- Key policy question: is the European energy production system becoming more efficient?

- Key assessment: penetration of combined heat and power

- The share of electricity from combined heat and power in total gross electricity production in the EU-27 was 10.9 % in 2007, same as in 2006.

- The preferred fuel for new CHP plants is natural gas due to its flexibility concerning the heat to power ratio as well as a better environmental performance compared for instance with liquid fossil fuels. In recent years, CHP has suffered from increasing natural gas prices and falling electricity prices, development which diminished the cost competitiveness of these plants. This trend may change in the future given that both prices have now started to rise again (see ENER 31). Other barriers included high costs for grid connection to sell surplus electricity, relatively high start-up costs, and the design of the policy framework in some cases.

- Specific policy question: what are the key differences among European countries with respect to the promotion of CHP?

- Specific assessment: cross-country comparison, CHP penetration

- The share of CHP production in gross electricity production was significantly higher in 2007 in the new Member States (15.0 % of total gross electricity production in these countries) than in the EU-15, where the share was 10.2%. In 2006, these shares were 16 % (NMS) and 10.1 % (EU-15) (see Figure 1). Countries with a high market penetration of CHP electricity include Denmark (42.8%), Latvia (40.9 %), Finland (34.4 %) and the Netherlands (30.1 %). In Denmark CHP has received strong government policy support, providing tax incentives and subsidies, and growth has been seen mainly in public supply as a result of investments in district heating infrastructure. Government support was also an important factor in the Netherlands, combined with widespread availability of natural gas. The high level of CHP production in Finland and Latvia reflects the cold climate, which provides scope for a better use of the heat as well as the electricity produced by CHP. In contrast to the process of energy market liberalisation in many other countries, the strong demand for both outputs coupled with a well developed district heating network, has actually helped to stimulate investment in CHP as opposed to hindering its expansion. Poor infrastructure for natural gas and less demand for heat, in particular in Greece and to a lesser extent Ireland and Portugal, has historically hindered CHP development and the share of CHP electricity remains low in these countries. Combined Heat/Cooling-Power Conversion may help to overcome the problem of surplus heat production in summertime and in warmer countries such as Greece and Portugal. A large decline in share was observed in Romania (from 18.0 % in 2006 to 10.7 % in 2007). Although the installed electrical CHP capacity increased with almost 10 %, the load factor decreased substantially, producing 41.6 % less electricity and 26.5 % less heat.

- Most of the EU Member States have in place feed-in tariffs for CHP promotion. Apart from feed-in tariffs, other support mechanisms exist such as investment subsidies (Austria, Belgium, Bulgaria, Czech Republic, Cyprus, Finland, Germany, Greece, Ireland), excise duty exemption, energy tax exemption (Belgium, Finland, Germany), preferential treatment for grid access (Estonia, Slovakia, Bulgaria), fuel subsidy (gas used as input fuel for CHP in Bulgaria, wood chips in Finland), etc. to promote new CHP (COGEN2007a,b). The level of feed-in tariff sometimes depends on the size of the unit, on the fuel use (e.g. in Slovakia the feed-in tariff is linked to share of coal in the input fuel, in Estonia peat, waste and shale gas are promoted) or is linked with the obligation to deliver heat into the district heating system (e.g.Latvia).

- The use of renewable energies (biomass) in combined heat and power provides an opportunity to further improve the environmental performance of CHP, while simultaneously increasing renewable electricity production (see ENER30). However, renewables provided only 11.9 % of the fuel input in CHP plants in the EU-27 in 2007 (see Figure 2a), an increase of 0.3 percentage points compared to 2006. Natural gas accounted for 50.5 % of the fuel input in EU-15 (up from 49.6 % in 2006), and 13.1 % in the new Member States (down from 13.4 % in 2006). Solid fossil fuels such as coal and lignite provided 15.8 % of the fuel input in EU-15 (down from 16.1% in 2006) and 73.7 % of in the new Member States, up from 72.3 % in 2006 (see Figure 2b).

# Fig. 1: Share of combined heat and power in gross electricity production in 2007

Data source: Eurostat

Note: The share is defined as the proportion of CHP electricity production in total electricity production. Note, however not all electricity production from a CHP ‘plant’ may be considered as CHP production as the plant may consist of different types of units (such as heat only, or flexible units where the power-to-heat ratio may be adjusted). The method for data collection by Eurostat on CHP was revised in an earlier version of the factsheet to account for this (as it had previously tended to overestimate the share of electricity generation from CHP) and is described in more detail in the metadata section. Therefore, the current share is not directly comparable to the 18 % target outlined in 1997 by the European Commission (COM (97) 514 final). The division in auto-producers and main activity producers is unknown for Denmark, and The Netherlands.

- Fig. 2a: Fuel input to CHP plants in the EU-27 in 2007

- Data source: Eurostat

- Fig. 2b: Fuel input to CHP plants in EU-27 and EEA countries in 2007

Data source: Eurostat

Note: No data available for Malta, Liechtenstein and Switzerland

Policy target

The Communication from the Commission to the Council, the European Parliament, the Economic and Social Committee and the Committee of the Regions on “A Community strategy to promote combined heat and power (CHP) and to dismantle barriers to its development” (COM97/514 final from 15th October 1997) calls for an increase in the share of CHP production in gross electricity generation to 18 % by 2010 in the EU-15 member states. The share of electricity from combined heat and power in total gross electricity production in the EU-15 was 10.2 % in 2007, hence quite some way from the target. The highest absolute production of CHP-electricity was observed in Germany (77.5 TWh in 2007, or 21.3 % of the total CHP electricity production in the EU-27). The CHP Directive on the promotion of high-efficiency cogeneration (2004/8/EC) encourages Member States to promote CHP up-take and help to overcome the current barriers hindering progress but much remains yet to be done.

Indicator rationale

Environmental context

Combined heat and power is one means of increasing the efficiency of electricity generation as it makes use of the heat that is otherwise lost in conventional thermal power plants. The combined efficiency of heat and electricity production from CHP schemes is therefore usually higher than that from heat and electricity produced independently. A typical efficiency value for CHP is approximately 85 % for the combined heat and electricity generation, compared to a typical efficiency value for conventional thermal electricity-only plant of 35–45 % and for heat only boilers of up to 90 % (ADEME, 1999). CHP plants are often located close to where the heat can be consumed, limiting transmission and distribution losses.

Increasing the conversion efficiency of power generation through the use of CHP helps to reduce the environmental impact of power generation. Additional environmental benefits can be realised if the scheme uses low emission fuels such as natural gas or biomass as opposed to coal or oil. Meeting the indicative EU-15 target of doubling the share of CHP in gross electricity production from 1994 to 2010 could lead to avoided CO2 emissions of over 65 Mt CO2/year by 2010 (see ENER 11).

CHP in industrial processes has a large potential for achieving energy-efficiency and CO2-emission reductions. A typical example: of the four refineries in the port of Rotterdam (the Netherlands) two use CHP for production of steam, the other two not. When the other two would also use CHP for steam-production, this would result in an expansion of production from refineries with app. 500 MW. Technically, a further expansion to app. 1,200 MW would be possible if all refineries would use CHP for heat-production as well. If this 1,200 MW would replace production of electricity by coal-fired power plants, this equals a net reduction of approximately 5 MtCO2 (CE Delft, 2008).

2.2 Policy context

- Council adopted on 6 April 2009 the climate-energy legislative package containing measures to fight climate change and promote renewable energy. This package is designed to achieve the EU's overall environmental target of a 20 % reduction in greenhouse gases and a 20 % share of renewable energy in the EU's total energy consumption by 2020.The climate action and renewable energy (CARE) package includes the following main policy documents

Directive 2009/29/ec of the European parliament and of the Council amending directive 2003/87/ec so as to improve and extend the greenhouse gas emission allowance trading scheme of the community

Directive 2009/31/ec of the European parliament and of the Council on the geological storage of carbon dioxide

Directive 2009/28/ec of the European parliament and of the Council on the promotion of the use of energy from renewable sources

Community guidelines on state aid for environmental protection (2008/c 82/01)

Directive 2008/101/ec of the European parliament and of the Council amending directive 2003/87/ec so as to include aviation activities in the scheme for greenhouse gas Emission allowance trading within the community

Regulation (ec) no 443/2009 of the European parliament and of the Council setting emission performance standards for new passenger cars as part of the community’s integrated approach to reduce CO2 emissions from light-duty vehicles

- Second Strategic Energy Review; COM(2008) 781 final

Strategic review on short, medium and long term targets on EU energy security.

- Energy Services Directive; Directive 2006/32/EC

Sets out clear mandatory targets for annual energy intensity improvements at Member States’ level and for the share of energy efficient public procurement for the period 2006-2012. For the same period, strong incentives were given by the Directive for Member States to ensure that suppliers of energy offer a certain level of energy services.

- Energy Performance Buildings Directive; Directive 2002/91/EC

The Member States must apply minimum requirements as regards the energy performance of new and existing buildings, ensure the certification of their energy performance and require the regular inspection of boilers and air conditioning systems in buildings.

- Communication from the Commission; COM(2008) 771 final

The main objectives of this communication are to report on the current status of the combined heat and power generation (CHP or cogeneration), and to present possibilities for its development.

- Detailed guidelines for the implementation and application of Annex II to Directive 2004/8/EC; 2008/952/EC

Guidelines for the calculation of the electricity from high-efficiency cogeneration.

- Directive on co-generation; 2004/8/EC

Requires Member States to carry out analyses of their potential for high efficiency cogeneration.

References

ADEME, European Commission (1999) Energy Efficiency Indicators: The European Experience.

CE Delft (2008) unpublished results

COD/2008/0013 - Proposal for a Directive of the European Parliament and of the Council amending Directive 2003/87/EC so as to improve and extend the greenhouse gas emission allowance trading system of the Community

COD/2008/0014 - Proposal for a Decision of the European Parliament and of the Council on the effort of Member States to reduce their greenhouse gas emissions to meet the Community’s greenhouse gas emission reduction commitments up to 2020

COGEN (2007a) – Financial and regulatory support for cogeneration in the EU

COGEN (2007b) – Cogeneration in Central and Eastern Europe; market situation and financial support

COM(1997) 514 final - Communication from the Commission on a Community strategy to promote combined heat and power (CHP) and to dismantle barriers to its development.

COM(2008) 771 final - Communication from the Commission to the European Parliament and the Council: Europe can save more energy by combined heat and power generation

COM(2008) 781 final - Second Strategic Energy Review

Directive 2002/91/EC - Directive on the Energy Performance of Buildings

Directive 2009/28/EC  - Directive 2009/28/EC of the European Parliament and of the Council of 23 April 2009 on the promotion of the use of energy from renewable sources and amending and subsequently repealing Directives 2001/77/EC and 2003/30/EC, Brussels, 2009

EU (2008) – 2008/C82/01 Community Guidelines on State Aid for Environmental Protection

EU (2008) – 2008/952/EC Commission Decision of 19 November 2008 establishing detailed guidelines for the implementation and application of Annex II to Directive 2004/8/EC of the European Parliament and of the Council (notified under document number C(2008) 7294)

EU (2009) Climate action and renewable energy package (CARE Package)

http://ec.europa.eu/environment/climat/climate_action.htm

European Commission (2005) Combining heat and power - using cogeneration to improve energy efficiency in the European Union

Eurostat (2007) Panorama of Energy, 2007 edition

SEC(2007) 53 - Directive of the European Parliament and of the Council amending Directive 2003/87/EC so as to improve and extend the EU greenhouse gas emission allowance trading system - Summary of the Impact Assessment

Meta data

Technical information

- Data source: Eurostat http://ec.europa.eu/eurostat/

- Description of data/Indicator definition:
The share of combined heat and power (CHP) electricity production is the ratio between the electricity produced from combined heat and power plants (based upon the consideration of individual units within the plants) and the gross electricity production calculated for a calendar year. The former is expressed as a percentage of the latter. It measures the contribution of electricity produced from CHP to total electricity production. However, there are several important qualifications as not all the electricity and (useful) heat produced in CHP plants can be considered CHP production. This is explained in section 6 of the metadata.
It should also be noted that although the indicator focuses on CHP electricity the quantity of useful heat recovered is almost 2.5 times higher in energy terms.

- Geographical coverage: The Agency had 32 member countries at the time of writing of this fact sheet. These are the 27 European Union Member States and Turkey, plus Iceland, Norway, Liechtenstein and Switzerland.

- Temporal coverage: 2007

- Methodology and frequency of data collection: Data collected bi-annually.
Eurostat definitions for energy statistics http://circa.europa.eu/irc/dsis/coded/info/data/coded/en/Theme9.htm
Eurostat metadata for energy statistics http://epp.eurostat.ec.europa.eu/cache/ITY_SDDS/EN/nrg_base.htm

- Methodology of data manipulation:
The share of electricity produced from combined heat and power as a percentage of total national electricity production. The coding and specific components of the indicator are:
Numerator: Electricity production from combined heat and power plants. However, as mentioned in section 2 there are several important qualifications regarding the quantity of electricity that be considered to be from CHP production.
A power plant which is equipped with heat recovery facilities in connection with electricity generation is called a CHP plant. From the statistical point of view, the simplest approach is to consider all the electricity and useful heat produced in these plants as CHP production. However, there are special features in CHP production, which complicate the collection of the statistics. First, there can be several units in a power plant, not all of which have the possibility of recovering heat. Therefore, the collection of statistics has to be carried out on a unit basis. There might be some supplementary heating in addition to the CHP heat production. This supplementary heat can be produced for example in an extra boiler, completely separated from the CHP process and hence must be subtracted from the total heat generation of the plant. The fuel used to generate this heat also has to be subtracted from the total fuel consumed, when the CHP production is considered. There are also flexible units, where the produced heat to power ratio is adjustable. The flexible units can be operated in a fully CHP mode, or they may also be run without any heat recovery at all, depending on the current demand for heat and electricity. Variable heat loads are typical for district heating plants, whilst in industry the ratio between heat load and electricity generation is generally more stable.
Following a change in methodology the CHP component in electricity production is now calculated from the total production of CHP plants by considering the overall annual efficiency and the power-to-heat ratio of individual units within each plant to account for the above. Under certain conditions all the electricity produced in a CHP unit can be regarded as CHP production and in this case there is no need to separate the non-CHP component. In the past there was no unequivocal quantitative rule to define when this separation into CHP and non-CHP components should be done. Different interpretations of the old methodology were therefore possible. The methodology was strengthened for the survey for the year 2000; a threshold of 75 % for the overall efficiency was set as the criterion to select plants in which the CHP component of the electricity production has to be calculated. If the average annual efficiency is 75 % or higher, all the electricity produced in the plant is considered to come from CHP. If the efficiency is below that threshold, the CHP electricity (ECHP) is calculated by multiplying the CHP heat production by the characteristic power-to-heat ratio of the plant. 
Denominator: 6000 electrical energy 107000 total gross electricity generation.
Average annual rate of growth calculated using: [(last year/base year) ^ (1/number of years) –1]*100
For a significant period of time, the statistical system did not permit the correct assessment of CHP production at European level. EU Directive 2004/8/EC on the promotion of cogeneration then obliged Member States to submit statistics on CHP to the Commission (Eurostat). However, Eurostat had already started collecting CHP statistics at the beginning of the 1990s (in the course of pilot projects). Since then, a common methodology for CHP data collection has been developed and assistance given to Member States in setting up the required statistical reporting system (Eurostat, 2007). The overall efficiency of a CHP unit is used as a measure to determine whether electricity generation is fully CHP or not. If overall efficiency is above a threshold set at 75 % (85 % for steam condensing extraction turbines and combined-cycle units), all the electricity generated is considered to be CHP electricity. On the other hand, if the overall efficiency is below the threshold, the amount of CHP electricity (ECHP) is calculated by multiplying the produced CHP heat by the power to heat ratio. These power to heat ratios are the actual ratios. If these actual ratios are unknown, default values can be used for several types of CHP. For several types of CHP (e.g. Stirling, fuel cell or Organic Rankine Cycles) countries can determine their own default values. These values shall be published and notified to the Commission .

Qualitative information

- Strengths and weaknesses (at data level)
No historical data for new Member States. Changes in the methodology mean that historical time series are not consistent.

- Reliability, accuracy, robustness, uncertainty (at data level):

Indicator uncertainty (historical data)
As mentioned in section 6 the method for data collection by Eurostat on combined heat and power has been revised in recent years. The primary change has been to account for the fact that not all of the electricity and (useful) heat produced in CHP plants can be considered CHP production. The methodology was strengthened for the survey for the year 2000 and again in 2002 and allows a clearer distinction between CHP and non-CHP electricity production and ensures more harmonised CHP statistics across the Member States. The effect of the methodology change on the reported CHP electricity production varies between the Member States. Some Member States were already reporting CHP statistics in which the condensing power, or the non-CHP production, was strictly separated from the gross production of CHP plants, whilst others were not. However, the overall effect has been to reduce the estimates of CHP electricity in the EU.

- Overall scoring – historical data (1 = no major problems, 3 = major reservations):

Relevance: 1

Accuracy: 2 (for EU-15 data)

Comparability over time: 3

Comparability over space: 1

- Natural gas is the favoured fuel for combined heat and power.