# Austria

Information on policies and measures and on projections are included in the Third National Communication and in the Austrian Climate Strategy. The Third National Communication provides two sets of projections: (1) projections based on model calculations; (2) projections based on the draft Austrian Climate Strategy. The Austrian Climate Strategy was adopted after the Third National Communication and includes projections which slightly differ from the projections included in the Third National Communication. Therefore, this document provides quantitative data on projections from the Austrian Climate Strategy.

## Sources of Information

Third National Communication to the UNFCCC:

Federal Ministry of Agriculture, Forestry, Environment and Water Management: Austria’s Third National Climate Report of the Austrian Federal Government – Austria’s Third National Communication in Compliance with the Obligations under the Framework Convention on Climate Change (Federal Law Gazette No. 414/1994), November 2001, Vienna.

Austrian Climate Strategy:

Bundesministerium für Land- und Forstwirtschaft, Umwelt und Wasserwirtschaft: Strategie Österreichs zur Erreichung des Kyoto-Ziels, Klimastrategie 2002/2012, vom Ministerrat angenommen am 18. Juni 2002, Wien.

National Allocation Plan (Austria):

Bundesministerium für Land- und Forstwirtschaft, Umwelt und Wasserwirtschaft: Nationaler Zuteilungsplan für Österreich gemäß § 11 EZG, 31. März 2004, mit Ergänzungen vom 7. April 2004 (http://www.eu-emissionshandel.at/)

Austria´s Emission Projection on Greenhouse Gases 2003-2020 – Interim Report (unpublished):

Umweltbundesamt, Vienna, June 15th 2005, according to the EU Monitoring Mechanism (Council Decision 280/2004/EC).

## Quality and Transparency of Reporting

The Third National Communication draws on the draft Austrian Climate Strategy 2010 and on model calculations. It provides a list of policies and measures to reduce greenhouse gas emissions in a range of sectors. The impact of the measures has been quantified in most cases. The with measures projection includes measures implemented and adopted by 2001; the with additional measures projection includes measures planned.

The table on policies and measures follows the UNFCCC guidelines.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - +++ |  |
| - Objectives of policies | - +++ |  |
| - Which GHGs? | - CO2, CH4, N2O, HFC, SF6 |  |
| - Status of Implementation | - +++ |  |
| - Implementation body specified | - +++ |  |
| - Quantitative assessment of implementation | - +++ | - Estimated mitigation effect for 2010; a few measures are not quantified |
| - Interaction with other P&Ms discussed | - ++ | - In some cases |

- +, ++, +++ level of information available increases as the number of + signs increases

- Note: Information on ‘Type of instrument’ follows largely the UNFCCC Guidelines but introduces the term ‘promotive’, which in many cases seems to correspond with ‘economic’.

- Two approaches are taken to develop projections of greenhouse gas emissions and to calculate the total effect of policies and measures. On the one hand, the emission projections are based on expert judgement (Austrian Climate Strategy). On the other hand, projections for the period 2000–2020 have been developed based on the model calculations (Third National Communication). Both approaches include with measures and with additional measures scenarios and show similar results.

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | With measures - With additional measures | - Austrian Climate Strategy (expert judgment): - Both scenarios are given for sectors partly corresponding with IPCC sectors - Third National Communication (model calculations): - Both scenarios are given for IPCC sectors and gases |
| - Expressed relative to inventory for previous years | - Yes |  |
| - Starting year | - 1999 |  |
| - Split of projections | - +++ | - Austrian Climate Strategy (expert judgment): - Projections split by sectors partly corresponding with IPCC sectors - Third National Communication (model calculations): - Projections split by IPCC sectors and gases |
| - Presentation of results | - +++ | - Results presented in both tabular and graphical form |
| - Description of model (level of detail, approach and assumptions) | - ++ | - Basic description of the models and further references provided |
| - Discussion of uncertainty | - No | - A few results of sensitivity analysis provided |
| - Details of parameters and assumptions | - ++ | - Summary table for key parameters (input and output parameter are marked) |

- +, ++, +++ level of information available increases as the number of + signs increases

## Assessment of Policies and Measures

- Table 3 gives an overview of effects of additional policies and measures from the Austrian Climate Strategy. The effects of existing policies and measures are included in the scenario ‘Trend 2010’.

- Table 3: Summary of the Effect of Policies and Measures by 2010 (MtCO2)

|  | Reduction potential (additional measures) |
| --- | --- |
| CO2 | -10,9 |
| CH4 | -1,6 |
| N2O | -0,2 |
| F-gases | -1,2 |
| Energy demand (small combustion) | 4,0 |
| Energy supply | 2,1 |
| Waste | 1,1 |
| Transport | 3,7 |
| Industry (energy and process) | 1,25 |
| Agriculture | 0,4 |
| HFCs, PFCs, SF6 | 1,2 |
| Other (incl. solvent use) | 0,1 |
| Total | 13,85 |

- Source: Austrian Climate Strategy

Table 4 gives details of the policies and measures.  The measures implemented or adopted by 2001 are included in the with measures  projections.  The measures planned are included in the with additional measures projections.

- Table 4: Detailed Information on Polices and Measures (estimated mitigation effect in 2010, in million tons CO2 equivalent)

| Sector | Name | Objective | GHG | Type of | Status | Implementing | Estimate of savings |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | affected | instrument |  | Entity | (MtCO2) |
| Energy demand | Agreement between Federation and Länder on energy-saving | Energy-savings in buildings, heating installations and electricity consumption | CO2 | regulatory (framework legislation) | Implemented | Federation and Länder | n.q. |
| Energy demand | Minimum thermal standards for buildings | Energy-savings in buildings | CO2 | regulatory (implementation) | implemented / planned | Länder | 0.5 Mt (0.2-0.3 impl.) |
| Energy demand | Housing support schemes | Energy-savings in dwellings (new and renovation) | CO2, HFC | promotive | implemented / adopted / planned | Länder | 2-2.5 Mt (0.5-1.0 impl./adop.) |
| Energy demand | Consumption-related heating costs calculation | Energy-savings in buildings | CO2 | regulatory | implemented | Federation | 0.1 Mt |
| Energy demand | Energy efficiency in federal public buildings | Energy-savings in public buildings | CO2 | voluntary | implemented | Federation | 0.05 Mt |
| Energy demand | ‘Contracting’ for public buildings | Energy-savings in public and private sector buildings | CO2 | economic / voluntary | implemented / adopted / planned | Federation, Länder, municipalities | 0.5 Mt (0.1 impl./ adop.) |
| Energy demand | Replacement of old heating systems | Energy-savings in buildings by improvement of heating energy supply | CO2 | promotive | implemented / adopted / planned | Länder, municipalities | 2.0 Mt (0.5 impl./ adop.) |
| Energy demand | Harmonised energy codes for buildings | Transparent and comparable declaration of energy consumption of buildings | CO2 | economic / information | planned | Federation, Länder | n.q. |
| Energy demand | Regular inspection of heating systems | Energy-savings in buildings by efficiency-raising of heating systems | CO2 | regulatory | implemented / adopted / planned | Länder | 0.3 Mt (0.1 impl./ adop.) |
| Energy supply | Preferential market access for ‘green electricity’ | Raising share of renewable energy sources in electricity supply | CO2, CH4 | economic (regulatory) | implemented | Federation (framework), Länder (executing laws) | 0.5 Mt |
| Energy supply | Public support for renewable energy projects and district heating | Raising share of heat production from renewable energy sources and CHP | CO2, CH4 | promotive | implemented | Federation, Länder, EU | 0.7 Mt |
| Energy supply | Energy tax rebates for CHP | Promotion of combined heat and power production (natural gas and diesel) | CO2 | fiscal | implemented | Federation | n.q. |
| Energy supply | Preferential framework conditions for CHP | Promotion of combined heat and power production by granting improved feed-in conditions or setting obligatory quota | CO2 | economic / regulatory / promotive | framework law implemented; executing laws planned | Federation (framework), Länder (executing laws) EU (future targets) | 0.5 Mt |
| Energy supply | Further development of targets for ‘green electricity’ | Raising share of renewable energy sources in electricity supply | CO2, CH4 | economic (regulatory) | planned | Federation (framework), Länder (executing laws) | 0.2-0.5 Mt |
| Energy supply | Stepped-up public support for GHG mitigation projects | Promotion of heat supply from renewable energy, energy efficiency measures etc. | CO2 | promotive | planned | Federation | 1.5-2 Mt |
| Energy supply | ‘Green electricity’ for public buildings | Raising share of electricity production from renewables through purchase power of public entities | CO2 | economic | adopted / planned | Federation, Länder, municipalities | n.q. |
| Energy supply | Voluntary agreements | Cost efficient GHG reductions in energy industry | CO2 | voluntary / negotiated | planned | Federation | n.q. |
| Waste Management | Waste Management Act 1990 | Framework law regulating waste management - minimisation of environmental impacts | CH4, CO2 | regulatory | implemented | Federation (framework), Länder (executing laws) | n.q. |
| Waste Management | Landfill Regulation 1996 | Minimisation of waste landfilling | CH4 | regulatory | implemented / adopted | Federation (framework), Länder (executing laws), municipalities | 0.9 Mt |
| Waste Management | Landfill Charge Act 1989 | Reduction of disposal of waste on landfills; earmarking of revenue for clean-up of contaminated land | CH4 | regulatory / fiscal / economic | implemented | Federation | n.q. |
| Waste Management | Expansion of waste treatment capacities other than landfilling | Banning disposal on landfills by 2004/2008; expanding share of other capacities, e.g. energy efficient incineration | CH4, CO2 | regulatory | planned / adopted | Länder, municipalities | 1.1 Mt for measures M24-M28 (0.3 adop.) |
| Waste Management | Promotion of waste recovery | higher share of waste recycling | CH4, CO2 | voluntary, information, education, research | implemented / adopted / planned | Federation, Länder, municipalities | n.q. |
| Waste Management | Efficient energy recovery from waste | energy recovery from waste incineration (CHP) | CH4, CO2 | promotive | adopted | Federation | n.q. |
| Waste Management | Other programmes to launch waste prevention and recovery | prevention of waste; higher share of energy recovery / recycling | CH4, CO2 | voluntary, information, research, promotive | implemented / adopted / planned | Ministry of Environment, Länder | n.q. |
| Waste Management | Technical state of art for mechanical-biological treatment of waste | Better investment security for operators of waste treatment sites with respect to technical standards | CH4 | standardization | planned | Federation | n.q. |
| Transport | Reduction of emissions from passenger cars | Raising market share of advanced engine technologies with low fuel consumption | CO2 | voluntary / information | implemented | Federation, EU | 0.5 Mt |
| Transport | Fuel consumption levy | Fiscal incentive for low fuel consumption vehicles | CO2 | fiscal | implemented | Federation | n.q. |
| Transport | Road tolls | Internalisation of external costs for use of highways | CO2 | fiscal | implemented | Federation | n.q. |
| Transport | Vehicle tax adaptation 2000 | Internalisation of external costs, especially for strong engines | CO2 | fiscal | implemented | Federation | n.q. |
| Transport | Rail infrastructure and public transport investments | Changing modal split to the benefit of rail/public transport | CO2 | public investments and promotion | implemented / adopted / planned | Federation, Länder, municipalities | 0.3 Mt (0.1 impl./ adop.) |
| Transport | improvement of fuel quality and promotion of ‘bio-diesel’ | GHG emissions reduction through fuel improvement and renewable energy sources | CO2 | regulatory / fiscal | implemented / planned | Federation | 0.1 Mt (0.05 impl.) |
| Transport | Model projects and programmes for environmentally sound mobility | Model projects with the aim to raise public awareness and to demonstrate new technologies | CO2 | information / education / demonstration / research | implemented | Federation, Länder, municipalities | n.q. |
| Transport | Model projects and programmes for environmentally sound logistics | GHG emissions reductions in transport sector by logistic support and avoidance of insufficient transportation |  | promotion / information / demonstration / research | implemented | Federation | n.q. |
| Transport | Mileage based toll for lorries | Internalisation of external costs of road transport | CO2 | fiscal | adopted | Federation | 0.3 Mt |
| Transport | Promotion of energy efficient and alternative motor concepts | Reduction of fleet fuel consumption | CO2 | promotive | planned | Federation, EU | 0.1 Mt |
| Transport | Public awareness raising measures | Reduction of individual private traffic | CO2 | information, education | adopted / planned | Federation, Länder | 0.3 Mt (0.1 adop.) |
| Transport | Improvement of transport logistics | Avoidance of inefficient and unnecessary transportation of goods, aiming at total reduction of road transport | CO2 | promotive, information | implemented / adopted / planned | Federation, Länder | 0.7 Mt (0.2 impl./ adop.) |
| Transport | Further internalisation of costs | Internalisation of external costs of road transport / private traffic | CO2 | fiscal | planned | Federation | n.q. |
| Transport | Promotion of walking and cycling | Shifting modal split, improving living conditions and safety | CO2 | promotive, information, education | implemented / adopted / planned | (Federation), Länder, municipalities | 0.3 Mt (0.1 impl./ adop.) |
| Transport | Improvement of spatial planning | Avoidance of traffic-inducing settlement structures | CO2 | planning / regulatory | planned | Länder | 0.3 Mt |
| Transport | Traffic management and speed limitation | Avoidance of congestions; promotion of economic driving | CO2 | information / regulatory / fiscal | implemented / planned | Federation, Länder | 0.3 Mt (impl. n.q.) |
| Industry | Promotion of energy efficiency and renewable energy | Energy-savings and increasing share of renewables in industry | CO2 | promotive | implemented | Federation | 0.2 Mt |
| Industry | Implementation of the IPPC directive | Energy-savings and efficiency raising measures in industry | CO2 | regulatory | implemented | Federation, (EU) | n.q. |
| Industry | Energy concepts for energy intensive branches | Evaluation of energy-saving potentials in industry | CO2 | conceptual, consultative | implemented | Länder | n.q. |
| Industry | Energy efficiency programme | Promotion of economic energy-saving in industry | CO2 | consultative, promotive | planned | Federation | 0.5 Mt |
| Industry | Promotion of energy efficiency and renewable energy | Energy-savings and increasing share of renewables in industry | CO2 | promotive | planned | Federation | 0.3 Mt |
| Industry | Voluntary agreements and flexible instruments | Cost efficient GHG reductions in industry | CO2 | voluntary/ negotiated agreements, economic | planned | Federation | 1.0-2.0 Mt |
| Agriculture and Forestry | Extension of ecological farming | Protection of rural environment (soil, water, natural and cultural habitat); production of high quality food | CH4, N2O | promotive | implemented | Federation, Länder, EU | n.q. |
| Agriculture and Forestry | Cultivation of oil-seed crops | Extension of crops for production of bio-fuels | CO2 | promotive, fiscal | implemented | Federation, Länder | n.q. |
| Agriculture and Forestry | Further enforcement of measures to reduce methane and N2O emissions | Protection of rural environment with more specific focus on GHG mitigation | CH4, N2O | promotive | adopted | Federation, Länder, EU | n.q. |
| Agriculture and Forestry | Maintenance and extension of vital forests | Maintaining biodiversity, productivity, regeneration capacity and vitality of forests | CO2 | research, information, regulatory | implemented / adopted / planned | Federation, Länder | n.q. |
| Fluorinated Gases | Phase-out of Montreal gases | Protection of the ozone-layer | ‘Montreal gases’ | regulatory | implemented | Federation | n.q. |
| Fluorinated Gases | Partial phase-out of HFCs and SF6 | Substantial reduction of emissions of gases with high GWP | HFC, SF6 | regulatory | planned | Federation | 0.8 Mt |
| Fluorinated Gases | Public procurement and support measures | Substantial reduction of emissions of gases with high GWP | HFC, SF6 | promotive, public procurement | Implemented / adopted / planned | Federation, Länder | 0.1 Mt (0.05 impl./ adop.) |
| Fluorinated Gases | Avoidance of leakage | Substantial reduction of emissions of gases with high GWP | HFC, SF6 | voluntary, research | planned | Federation | 0.3 |
| Cross-cutting p&ms | Energy related taxes | Revenue-raising with positive side-effect of potential GHG reductions | CO2 | fiscal | implemented | Federation | 0.3 |
| Cross-cutting p&ms | GHG emissions trading | Cost-effective GHG emissions reductions for large stationary emitters | CO2 | economic | planned | Federation | n.q. |

- Source: Austria’s Third National Communication, pages 82-88

## Evaluation of Projections

- Table 5 shows the projections by greenhouse gas for 2010, Table 6 summarises the projections by sector. Transport related emissions are projected to increase by 62% (with measures) and 32% (with additional measures). The largest decreases are projected for energy demand from small combustion with –28% in the with additional measures scenario. The emissions from agriculture and waste reduce in both, the trend 2010 scenario and the scenario with additional measures.

- Table 5: Summary of projections by gas in 2010 (MtCO2)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| CO2 | 62,3 | 70,3 | 59,4 |
| CH4 | 11,3 | 8,8 | 7,2 |
| N2O | 2,3 | 2,3 | 2,1 |
| F-gases | 1,7 | 3,0 | 1,8 |
| Total | 77,6 | 84,4 | 70,5 |
| % change relative to base year |  | 8,8% | -9,1% |

- Source: Austrian Climate Strategy

- Table 5a: Update of projections by gas in 2010 (MtCO2)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| F-gases | 1,4 | 1,5 |  |

- Source: Austria´s Emissions Projection on Greenhouse Gases 2003-2020 – Interim Report (2005)

- Table 6: Summary of projections by sector in 2010 (MtCO2)

|  | Base year | With measures (trend 2010) | % change relative to 1990 | with additional measures | % change relative 1990 (additional measures) |
| --- | --- | --- | --- | --- | --- |
| Energy demand (small combustion) | 14.60 | 14.50 | -0.7% | 10.50 | -28.1% |
| Energy supply | 14.44 | 14.50 | 0.4% | 12.40 | -14.1% |
| Waste | 6.26 | 4.80 | -23.3% | 3.70 | -40.9% |
| Transport | 12.32 | 20.00 | 62.3% | 16.30 | 32.3% |
| Industry (energy and process) | 21.71 | 22.00 | 1.3% | 20.75 | -4.4% |
| of which energy* | 13,12 | 13,29 | 1,3% | 12,54 | -4,4% |
| of which process* | 8,59 | 8,71 | 1,3% | 8,21 | -4,4% |
| Agriculture | 5.60 | 4.80 | -14.3% | 4.40 | -21.4% |
| HFCs, PFCs, SF6 | 1.74 | 3.00 | 72.4% | 1.80 | 3.4% |
| Other (incl. solvent use) | 0.97 | 0.80 | -17.5% | 0.70 | -27.8% |
| Total | 77.64 | 84.40 | 8.7% | 70.55 | -9.1% |

- Source: Austrian Climate Strategy

*Industry emissions were not split into energy and process related emissions in the Austrian Climate Strategy, therefore, the percentage change as reported in the latest GHG inventory for the year 2002 has been applied.

- Table 6a: Update of projections by sector in 2010 (MtCO2)

|  | Base year | With measures (trend 2010) | % change relative to 1990 | with additional measures | % change relative 1990 (additional measures) |
| --- | --- | --- | --- | --- | --- |
| Waste | 4.50 | 2.78 | -38.23% |  |  |
| Agriculture | 8.46 | 7.26 | -14.14% |  |  |
| HFCs, PFCs, SF6 | 1.43 | 1.47 | 2.81% |  |  |

- Source: Austria´s Emissions Projection on Greenhouse Gases 2003-2020 – Interim Report (2005)

The with measures projections show that the currently implemented or adopted measures of Austria do not reduce greenhouse gas emissions by 2010. Instead, greenhouse gas emissions are projected to be 8.7% above the base year level by 2010 in the with measures scenario.

Even in the with additional measures scenario Austria is expected to exceed its commitment of a 13% reduction in greenhouse gas emissions under the EU burden sharing agreement. Additional measures will reduce greenhouse gas emissions to 9.1% below 1990 levels by 2010. The remaining GHG reduction requirement of around 3 Mt CO2 equivalent will be fulfilled by making use of the project-based flexible mechanisms of the Kyoto Protocol (JI/CDM).

- Table 7: Assessment of the Target

|  | Excl. LUCF |  |
| --- | --- | --- |
|  | MtCO2 equiv. | % of base year level (six gas basket) |
| Base year emissions (from projections) | 77.6 | 100% |
| Kyoto Commitment/burden sharing | 67.5 | -13.0% |
| Use of Kyoto mechanisms | 7.0 | 9.0% |
| With existing P&Ms projections | 84.4 | 108.8% |
| Gap (without flex. mechs.)* | 16.9 | 21.8% |
| Gap (with flex. mechs.) | 9.9 | 12.7% |
| With additional P&Ms projections | 70.5 | 90.9% |
| Gap (without flex. mechs.) | 3.0 | 3.9% |
| Gap (with flex. mechs.) | -4.0 | -5.2% |

- Source: Austrian Climate Strategy

## Description of Modelling ApproacH

This section refers to the model-based projections provided in the Third National Communication. Emissions from the energy sector are projected with the modelling framework of the Austrian Institute for Economic Research. The framework includes the energy model DEDALUS and the macroeconomic multisectoral model MULTIMAC. Resulting figures for energy demand in the Austrian economy are split according to the subsectors of the Austrian greenhouse gas emission inventory and emissions are calculated in accordance with inventory methodology.

The structure of the energy-economy-environment (E3) modelling framework is as follows: The energy model DEDALUS is integrated into the macro economic multisectoral model of the Austrian economy MULTIMAC III. The output of DEDALUS determines the energy sector variables, which is the energy-economy link. The outcome of the macroeconomic multisectoral model (GDP, output by industries, capital stock for different energy-relevant purposes) determines together with fully exogenous influences (energy prices, technology diffusion for renewables and district heating, transport equipment, demography, etc.) the energy use and CO2 emissions, which constitute the remaining environmental link.

DEDALUS consists of an econometric model for final energy demand of 13 sectors of the Austrian economy and an input-output model of energy transformation with varying technical coefficients. This model describes the Austrian energy system in sufficient detail to derive emission paths of 13 sectors and the energy transforming processes.

MULTIMAC III is input-output based at a medium aggregation level of 32 industries and combines econometric functions for goods and factor demand, prices, wages and the labour market with the input-output accounting framework.

Emission scenarios for the agriculture, industrial processes and waste sectors are in principle based on the Austrian Carbon Balance Model (ACBM). ACBM is a model that reflects the dynamic behaviour of the carbon cycle in Austria with the aim of establishing a full carbon accounting for Austria. For the current projections, the results of the ACBM scenarios had to be partially adapted according to results of the energy projections (e.g. activities in the industry sector) and to the latest expert judgements (e.g. development of livestock). The emission figures are based on emissions factors from the Austrian greenhouse gas inventory.

### Table 8: Modelling parameters

| Parameter | 1990 | 1995 | 2000 | 2005 | 2010 | 2015 | 2020 | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GDP growth | +4.6 | +1.5 | +3.9 | +2.1 | +2.0 | +1.9 | +2.0 | annual average % (for previous 5 years) |
| ### Oil price Brent |  |  | 24.9 | 16.5 | 16.5 | 19.7 | 22.5 | EURO(1990/bbl |
| Population | 7.73 | 8.05 | 8.11 | 8.17 | 8.21 | 8.25 | 8.30 | million |
| Electricity import/export ratio |  |  | -2.6 | +2.6 | +2.0 | +2.0 | +2.0 | % |
| Number of dwellings | 2.97 |  | 3.26 | 3.36 | 3.44 | 3.51 | 3.59 | million |
| Private cars | 2.99 | 3.59 | 4.03 | 4.40 | 4.77 | 5.10 | 5.37 | million |
| Energy prices |  |  |  |  |  |  |  |  |
| Fuel oil |  |  | 100 | 78 | 76 | 79 | 84 | % of 2000 |
| Electricity |  |  | 100 | 92 | 102 | 103 | 106 | % of 2000 |
| Gasoline |  |  | 100 | 86 | 84 | 86 | 89 | % of 2000 |
| Final energy consumption | 833 | 880 | 945 | 1010 | 1049 | 1085 | 1121 | PJ |
| Steel production activity | 3922 | 4529 | 5366 | 5502 | 5502 | 5447 | 5393 | 1000 tons |
| Energy efficiency | +18% |  |  | +10% |  |  |  | % |
| Cattle | 2584 | 2326 | 2156 | 2092 | 2010 | 1941 | 1875 | 1000 head |
| Municipal waste deposition in landfills | 4115 | 3529 | 3640 | 3463 | 2700 | 2147 | 1783 | 1000 tons |

Note: Input parameters to the models are in bold

## Country Conclusions

The main documents used for this summary were the Third National Communication and the Austrian Climate Strategy. The level of detail and clarity of the documents from Austria is rather good. Details of the methodology for the model-based projections are available and contain a reasonable level of detail on the models and parameters. The policies and measures are summarised in a Table, which makes clear the potential greenhouse gas savings.

The projected increase from the base year with measures implemented and adopted is +8.7% in the basket of six greenhouse gases by 2010. Additional policies and measures are projected to reduce GHG emissions to 9.1% below base year levels by 2010. This reduction is still not enough in order to reach the 13% reduction agreed under the Burden Sharing. The remaining GHG reduction requirement of around 3 Mt CO2 equivalent will be fulfilled by making use of the project-based flexible mechanisms of the Kyoto Protocol (JI/CDM), estimated at 7Mt CO2 equivalent.

The projections in the Austrian Climate Strategy show that transport related emissions are projected to increase by 62% (with measures) and 32% (with additional measures). The largest decreases are projected for energy demand from small combustion with –28% in the with additional measures scenario. The emissions from agriculture and waste reduce in both, the trend 2010 scenario and the scenario with additional measures.