# Slovakia

## Sources of Information

For policies and measures: Slovakia’s Third National Communication under the United Nations Framework Convention on Climate Change, 2001. Information on expired or repealed policies and measures was provided in the report: Biennial Report 2005 Slovak Republic submitted to the Monitoring Mechanism in March 2005,

For projections: Information on updated projections provided in the report: Biennial Report 2005 Slovak Republic submitted to the Monitoring Mechanism in March 2005.

and in Up-date of GHG emissions’ projection data as provided in the  Part 3 of the Biennial Report 2005 of the Slovak Republic submitted in March 15, 2005 submitted in July 2005.

## Quality and Transparency of Reporting

Two chapters in The Third National Communication of Slovakia deal with projections and measures. Policies and measures to reduce greenhouse gas emissions are provided for the Energy, Transport, Industry, Agriculture, Forestry and Waste management sectors.  The impact of the measures has been quantified in most cases. The table on policies and measures follows the UNFCCC guidelines, but not all discussed policies and measures are presented in the table. Information on expired or repealed policies and measures and on updated projections provided in the report: Biennial Report 2005 Slovak Republic submitted to the Monitoring Mechanism in March 2005

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - +++ |  |
| - Objectives of policies | - +++ |  |
| - Which greenhouse gases? | - CO2, CH4, N2O, PFCs,HFCs,SF6 |  |
| - Status of Implementation | - ++ |  |
| - Implementation body specified | - ++ | - In most cases |
| - Quantitative assessment of implementation | - ++ | - Estimated mitigation effect for 2010; a few measures are not quantified |
| - Interaction with other P&Ms discussed | - ++ | - In some cases |

- o, +, ++, +++ level of information available increases as the number of + signs increases

- Two approaches are taken to develop projections of greenhouse gas emissions and to calculate the total effect of policies and measures. Energy and Forestry projections are based on model calculations; the other sectors projections are based on expert judgement. Both approaches include without, with measures and with additional measures scenarios.

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | Without measures With measures - With additional measures | - Scenarios are given for sectors corresponding with IPCC sectors and for CO2 from fossil fuel combustion and transformation |
| - Expressed relative to inventory for previous years | - no |  |
| - Starting year | - 1998 |  |
| - Projections | - 2000,2005,2010,2015, |  |
| - Split of projections | - yes | - Projections split by IPCC main sectors and for CO2 from fossil fuel combustion and transformation |
| - Presentation of results | - +++ | - Results presented in both tabular and graphical form |
| - Description of model (level of detail, approach and assumptions) | - ++ | - Basic description of the models and further references provided |
| - Discussion of uncertainty | - No |  |
| - Details of parameters and assumptions | - ++ | - information on type of indicators used in scenarios provided |

- +, ++, +++ level of information available increases as the number of + signs increases

## Assessment of Policies and Measures

- Table 3 gives an overview of effects of policies and measures. The without measures scenario represents extrapolation of the actual situation from the aspect of source structure and fuel consumption. The figures given for the with measures projection are for policies and measures implemented or adopted by 2001.

- The with additional measures scenario includes the effects of policies applicable and/or planned. The effect of policies planned is derived from the difference between the with measures and the with additional measures scenarios of the model calculations.

- Table 3: Summary of the Effect of Policies and Measures by 2010 included in the Projections (MtCO2  eq.)

|  | with measures scenario | with additional measures scenario |
| --- | --- | --- |
| Energy | 0.8 | 0.3 |
| of which transport | 0.0 | 0.3 |
| Industrial processes | 0.1 | 0.0 |
| Agriculture | 0.0 | 0.4 |
| Waste | 0.2 | 0.4 |
| Total by sector (excl. sinks) | 1.1 | 1.1 |

Note: The effect of policies implemented or adopted is derived from the difference between the ‘without measures’ scenario and the ‘with measures’ scenario.  The’ additional measures’ scenario is subtracted from the ‘with measures’ scenario to get the effect of planned policies and measures.

Table 4 gives details of individual policies and measures.

Table 4 Detailed Information on Polices and Measures (estimated mitigation effect in 2010, in million tons CO2 equivalent). Characteristics and reduction potential of some mitigating measures in Energy sector

#### Characteristics and reduction potential of some mitigating measures in Agriculture

#### Characteristics and reduction potential of some mitigating measures in Forestry

#### Characteristics and reduction potential of some mitigating measures in Waste management

I – policy and measure have been already implemented (using criteria of updated IPCC Guidelines 1999/7)

S – adopted, approved policy or measure

P – planned, prepared policy/measure

The following information comes from the report: Biennial Report 2005 Slovak Republic submitted to the Monitoring Mechanism in March 2005.

# Overview of  expired or repealed policy and measures to limit and/or reduce GHG emissions

Strategies and programmes related to the climate change

| Strategy /Program/Policy | Current stage | Comment |
| --- | --- | --- |
| Energy policy of the Slovak Republic, 2000 | Proposal of new energy policy of the Slovak Republic, Ministry of Economy, 2005 | Open for comments at the web site of the Ministry of Economy: www.economy.gov.sk, among other goals increased share of RES on electricity production (19% in 2010), energy saving and energy efficiency improvement are defined. |

Cross-sectoral measures

| Policy and measures | Current stage | Comment |
| --- | --- | --- |
| Act. No. 309/1991 Coll. on air protection | Replaced by Act No. 478/2002 Coll. on air protection and on amendment of the Act No. 401/1998 Coll. on charges for air pollution | Regulates the rights and duties of legal and natural persons concerning the air protection against the pollutants caused by human activity and in reduction of causes and mitigation of effects of air pollution, objectives in ambient air quality, competence of state administration authorities for air protection and municipalities (hereinafter referred to as “air protection authority” only) and responsibility for breaching the duties in the field of air protection. |
| Act. No. 286/1992 Coll. on income taxes | At present, there is no legal instrument to provide for income tax allowance for the operation of RES in place | Supportive financial programmes of the Ministry of Environment and Ministry of Economy to promote wider use of RES and energy saving measures are available in limited range. Program of MoEnv allows the use of funds up to 30% of total investments (maximum 50.000 SKK) also for physical persons, if they implement projects with the objective to change their fuels into more energy and environmentally acceptable type. |
| Act No.69/1998 Coll. on the state fund of the environment | Act No. 587/2004 Coll. on environmental fund |  |
| National program of environmental evaluation and identification of products | Act No. 469/2002 Coll. on environmental labelling of products |  |

Energy sector – CO2  emissions

| Policy and measures | Current stage | Comment |
| --- | --- | --- |
| Act. No. 70/1998 Coll. on energy | Replaced by Act No. 656/2004 Coll. on energy and by the Act No. 657/2004 on heat energy |  |
| Proposal of the Act on energy efficiency | This Act has not been approved and adopted. | Transposing of the key EU directives in this area: Directive 2002/91/EC on the energy performance of buildings and draft Directive on energy end-use efficiency and energy services will fully cover objectives of the proposed act. |
| Prepared Act on Regulation in network industries | Act No. 276/2001 Coll. on regulation in network industries in amendments (the last one amendment is the Act No. 658/2004). |  |

Industry- CO2  emissions

| Policy and measures | Current stage | Comment |
| --- | --- | --- |
| Iron and steel production in the US Steel Košice | Covered by the EU emission trading scheme according to the Directive 2003/87/EC, Act No. 572/2004 Coll. |  |
| Aluminium production in ZSNP Žiar Nad Hronom | Covered by the EU emission trading scheme according to the Directive 2003/87/EC, Act No. 572/2004 Coll. |  |
| Cement production | Covered by the EU emission trading scheme according to the Directive 2003/87/EC, Act No. 572/2004 Coll. |  |

Agriculture – CH4, N2O  emissions

| Policy and measures | Current stage | Comment |
| --- | --- | --- |
| Act No. 83/2000 Coll. on agricultural soil protection | Act No. 220/2004 Coll. on protection and use of agricultural soil and on amendment of the Act No. 245/2003 Coll. on IPPC (integrated prevention and pollution control) |  |

Waste management – CH4, N2O  emissions

| Policy and measures | Current stage | Comment |
| --- | --- | --- |
| Act.No.238/1991 Coll. on waste | Replaced by the Act No.223/2001 Coll. on waste and later amendments (the last one Act No.733/2004 Coll.) |  |
| Act No. 327/1996 Coll. on charges for waste disposal | Act No. 17/2004 Coll. on charges for waste disposal |  |
| Waste management program of the SR until 2000 | Waste management program of the SR until 2005 |  |

## Evaluation of Projections

The data in Tables 5-7 are based on information from the report: Up-date of GHG emissions’ projection data as provided in the Part 3 of the Biennial Report 2005 of the Slovak Republic submitted in March 15, 2005 submitted in July 2005.

Table 5: Summary of projections by gas in 2010 (MtCO2 equivalent)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| Carbon dioxide (excl. sinks) | 59,4 | 50,4 | 50,1 |
| Methane | 6,3 | 4,0 | 3,6 |
| Nitrous oxide | 6,0 | 3,3 | 2,9 |
| F-gases* | 0,3 | 0,2 | 0,2 |
| Total (excl. sinks) | 72,1 | 57,9 | 56,8 |
| % change relative to base year (excl. sinks) |  | -19,7% | -21,3% |

*base year for F-gases is 1995

- Table 6 summarises the projections by sector.  Transport is projected to increase emissions in the with measures projections, Forestry is decreasing sinks, whereas energy as a whole, industry, agriculture, and waste sector reduce emissions by 2010.

- The with measures projection shows that the currently implemented or adopted measures of Slovakia could reduce greenhouse gas emissions by 2010 by 19.7% and with additional measures by21.3%.

Table 6: Summary of projections by sector in 2010 (MtCO2 eq.)

|  | Base year (1990) | with measures | % change relative to 1990 | with additional measures | % change relative to 1990 |
| --- | --- | --- | --- | --- | --- |
| Energy | 57,7 | 48,0 | -17% | 47,7 | -17% |
| of which Transport | 5,2 | 6,3 | 22% | 6,0 | 16% |
| Industrial Processes | 4,3 | 5,0 | 17% | 5,0 | 17% |
| Agriculture | 8,1 | 2,8 | -66% | 2,3 | -71% |
| Waste | 2,1 | 2,1 | -1% | 1,7 | -19% |
| Total (excl. sinks) | 72,1 | 57,9 | -20% | 56,8 | -21% |

Table 7: Assessment of the Target

|  | Excl. sinks |  |
| --- | --- | --- |
|  | MtCO2 equiv. | % of base year level (six gas basket) |
| Base year emissions (from projections) | 72,1 | 100% |
| Kyoto Commitment/burden sharing | 66,3 | -8,0% |
| With existing P&Ms projections | 57,9 | 80,3% |
| Gap (-ve means no gap) | -8,4 | -11,7% |
| With additional P&Ms projections | 56,8 | 78,7% |
| Remaining gap | -9,6 | -13,3% |
| Effect of flexible mechanisms |  | 0,0% |
| Remaining gap (if target includes flex mechs) |  | 0,0% |

## Description of Modelling ApproacH

Projections of energy related CO2 emissions

Updated Windows modules BALANCE and IMPACT of the ENPEP software package which represents an integrated approach of energy system modelling, were applied for modelling. The key assumptions and input data adopted under the development of modelling scenarios of CO2 emissions from fossil fuel combustion and transformation are as follows:

- Development of final energy consumption in individual sectors;

- Structure of sources which participate in transformation of primary energy sources  (PES);

- Conversion, transport and distribution of fuels and energies from primary sources to their final consumption

Input data for modelling:

- Updated Windows modules BALANCE and IMPACT of the ENPEP software supplemented with the  MESSAGE software package were applied for  modelling.

- Macro-economic and demographic growth rate figures were identical with these one checked by the Slovak Ministry of Environment and submitted in March 2005 to the EK for EU-25 wide GHG projections. Data are given in Table A1 in Annex.

- Forecast of PES consumption.

- Energy supply side forecasts.

- Energy demand side forecasts;

- Scenario of electricity generation in the Slovak Electricity, a.s.;

- Perspectives of crude oil processing in the Slovak Republic;

- Structure of road vehicle fleet, annual vehicle mileage and their specific consumption;

- Transport outputs in railway, water and air transport;

Assumptions applied for modelling:

- 2003 - base year for projections; inventory emission data from the NIR 2005 of the Slovak Republic;

- Emissions from mobile sources are also involved in this category.

- Growth rate of fossil fuel consumption in technologies will directly follow gross value added growth rate.

- Autonomous energy efficiency improvement by 1% yearly (elasticity) for heat generation in the industrial heat plants was assumed;

- Heat consumption will be the driving force for electricity generation in the industrial CHP.

- Description of scenarios:

- Without measures scenario (reference scenario) :

- Excluded all PAMs implemented, adopted or planned after the base year of projections (2003) – in the Slovak case the impact of the Act No. 478/2002 Coll. on Air Protection including the BAT requirements for the large and middle scale emission sources  and domestic Cap &  Trade system for the SO2 emissions.

- With measures scenario:

- Implemented and adopted PAMs related to the CC in and after the base year for projections (2003).

- Impact of the Directive 2003/87/EC establishing a scheme for greenhouse gas emission allowance trading within the Community 2003/87/ES has been followed.

- Due to the former national legislation in the air protection area (focused on basic air pollutants) the only small room remained for implementing of CC related PAMs – assumptions on combined combustion of coal and biomass with 30% share of biomass and on  fuel switching from the liquid fuels to natural gas have been adopted for modelling.

- With additional measures scenario:

- PAMs planned after the base year for projections and related to the CC (2003).

- Directive 2001/77/EC on the promotion of electricity produced from renewable energy sources in the internal electricity market – total reduction potential of RES consisting of small hydropower plants, biomass, wind, geothermal, solar and biogas in 2010 was  476 kt of CO2;

- Directive 2003/30/EC on the promotion of the use of biofuels or other

- renewable fuels for transport- reduction potential in 2010 related to this measure has been established as of 323.5 kt CO2;

- Directive 2002/91/EC on the energy performance of buildings – related input data were not available. Forecasts of heat consumption for new buildings developed under the World Bank project for National energy efficiency study for the Slovak Republic were used.

- Directive 2004/7/EC on the promotion of cogeneration – relevant data to quantify possible impact were not available; this directive will be a subject of analyses during preparation of 4th National Communication on CC;

# Indicators to monitor and evaluate progress with policies and measures over time

Reference: Article 3(2)(a) (iv) of the Decision 2004/280/EC and Annex III of the Decision 2005/166/ES

| No | Eurostat sectors | Indicator | Numerator/denominator | Input data | Source reference | Indicator value | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MACRO | CO2 intensity of GDP, t/Mio Euro | Total CO2 emissions, kt | 42 817 | CRF 2003 |  |  |
|  |  |  | GDP, Bio Euro (EC95) | 19 | Statistical yearbook 2004 | 2278 | t/Mio Euro |
| 2 | TRANSPORT C0 | CO2 emissions from passenger cars, kt |  | 2472 | COPERT III |  |  |
|  |  | Number of kilometres by passenger cars, Mkm |  | 13606 | COPERT III | 0.1817 |  |
| 3 | TRANSPORT D0 | CO2 emissions from freight transport (all modes), kt |  | 2385 | COPERT III |  |  |
|  |  | Freight transport (all modes), Mtkm |  | 10 236 | Sectoral yearbook | 0.2330 |  |
| 4 | INDUSTRY A1 | Energy related CO2 intensity of industry, t/Mio Euro | CO2 emissions from fossil fuel consumption industry, kt | 14 588 | CRF 2003 |  |  |
|  |  |  | Gross value-added total industry, Bio Euro (EC95) | 6 | Statistical yearbook 2004 | 2285 | t/Mio Euro |
| 5 | HOUSEHOLDS A1 | Specific CO2 emissions of households, t/dwelling | CO2 emissions from fossil fuel consumption households, kt | 4 593 | CRF 2003 |  |  |
|  |  |  | Stock of permanently occupied dwellings, 1000 | 1 654 | Statistical census in 2001 | 3 | t/dwelling |
| 6 | SERVICES A0 | CO2 intensity of the services sector, t/Mio Euro | CO2 emissions from fossil fuel consumption services, kt | 980 | CRF 2003 |  |  |
|  |  |  | Gross value-added - services, Bio Euro (EC95) | 9 | Statistical yearbook 2004 | 109 | t/Mio Euro |

| 7 | TRANSFORMATION B0 | Specific CO2 emissions of public and autoproducer power plants, t/TJ | CO2 emissions from public and autoproducer thermal power stations, kt | 10062 | Energy Statistics 2003 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | All products –output by public and autoproducer thermal power stations, PJ | 131.4 | Energy Statistics 2003 | 76.6 | t/ TJ |
| 8 | AGRICULTURE | Specific N2O emissions of fertiliser and manure use, kg/kg | N2O emissions from synthetic fertiliser and manure use, kt | 0.4472 + 0.4396 | CRF 2003 |  |  |
|  |  |  | Use of synthetic fertiliser and manure, kt nitrogen | 81.3+79.9 | Green Report | 0.0055 | kg/kg |
| 9 | AGRICULTURE | Specific CH4 emissions of cattle production, kg/head | CH4 emissions from cattle, kt | 46.5 | CRF 2003 |  |  |
|  |  |  | Cattle population, 1000 heads | 607.8 | Green Report | 0.076 | kg/head |
| 10 | WASTE | Specific CH4 emissions from landfills, kt/kt | CH4 emissions from landfills, kt | 65.75 | CRF 2003 |  |  |
|  |  |  | Municipal solid waste going to landfills, kt | 1 472 | RISO | 0.0447 | kt/kt |

## Country Conclusions

The main document used for this summary were: the Third National Communication and the report: Biennial Report 2005 Slovak Republic submitted to the Monitoring Mechanism in March 2005. The level of detail in policies and measures chapter is appropriate and is line with UNFCCC guidelines.  The details of the methodology for the projections are described.  The scenarios are provided for sectors and for total GHGs , but not separately for CO2 methane and nitrous oxide. The key policies and measures are summarised in a table, which makes clear the potential greenhouse gas savings.

The projected decrease from the base year with measures implemented and adopted is -19.7% in the basket of six greenhouse gases by 2010, excluding land use change and forestry. Additional policies and measures are applicable to reduce GHG emissions to 21.3% below the base year levels by 2010 (excluding LUCF). This shows that Slovakia has potential to go beyond Kyoto targets.

Issued in the Slovak 3rd National Communication on Climate Change, 2001, currently expired or repealed

Policy and/or measure currently adopted

Renewable energy source

Activities and measures, the effect of which occured in several categories of GHG or in several sectors at the same time

Issued in the Slovak 3rd National Communication on Climate Change, 2001, currently expired or repealed

Issued in the Slovak 3rd National Communication on Climate Change, 2001, currently expired or repealed

Energy and Power Evaluation Program, Argonne National Laboratory, Illionois, USA

Energy and Power Evaluation Program, Argonne National Laboratory, Illionois, USA

Energy Policy of the Slovak Republic- Draft proposal, 2005, www.economy.gov.sk

www.enviro.gov.sk

The Slovak Ministry of Transport, Post and Telecommunications, Yearbook 2004

Gross value-added data were calculated using official statistical data for NACE categories (in SKK only)

Preliminary data

Preliminary data

Green Report 2004, The Slovak Ministry of  Agriculture

Green Report 2004, The Slovak Ministry of  Agriculture

Regional Info System on Wastes