# Greece

## Sources of Information

- Greece’s report to the EU Commission of projected progress in accordance with article 3.2 of the Council Decision No280/2004/EC on the mechanism for monitoring Community greenhouse gas emissions and for implementing the Kyoto Protocol. Submitted in June 2005.

## Quality and Transparency of Reporting

The report submitted by Greece under the monitoring mechanism provides information on policies and measures, and a summary of its progress towards a commitment under the EU burden sharing agreement to reduce greenhouse gas emissions to 25% above 1990 levels.

The report provides a summary of policies and measures being taken or planned for implementation. It covers five sectors – energy, transport, waste, agriculture and domestic. These polices and measures are presented showing the possible savings to 2010, though a more comprehensive quantitative assessment by policy, to 2020, would enable a full analysis of Greece’s progress. Further to this, there is a lack of sufficient information relating to the base year of projections.

Projections are presented as ‘with measures’ and ‘with additional measures’.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - ++ | - Expanded to sub sector |
| - Objectives of policies | - + | - Basic description only |
| - Which greenhouse gases? | - CO2, NH4, N20, F-gases |  |
| - Status of Implementation | - ++ | - Good explanation |
| - Implementation body specified | - + | - Acronyms given but no explanation as to the organisation |
| - Quantitative assessment of implementation | - ++ | - 2010 assessment only, but data for each sector |
| - Interaction with other P&Ms discussed | - o | - Not discussed |

- o, +, ++, +++ level of information available increases as the number of + signs increases

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - With and with additional measures |  |
| - Expressed relative to base year | - 1990, 2000 | - Base clearly stated |
| - Starting year | - 1990 | - 2000 |
| - Split of projections | - + | - Split by sector and gas. |
| - Presentation of results | - + | - No gas summary for “with additional measures” |
| - Description of model (level of detail, approach and assumptions) | - ++ | - Tables of assumptions and text. |
| - Discussion of uncertainty | - o | - No discussion |
| - Details of parameters and assumptions | - + |  |

- o, +, ++, +++ level of information available increases as the number of + signs increases

- The 2005 Greece report contains “with measures” and “with additional measures” scenarios for carbon dioxide, methane, nitrous oxide and F-gases. CO2 projections ‘with measures’ also describes the impact of land use and forestry change as sinks. A summary of the effect of policies and measures included in the projections was not provided in this report, but data on the effect of policies and measures was presented.

## Projections are for five year intervals to the year 2020.   A good level of information on the effect of policies and measures is provided.

## Assessment of Policies and Measures

- Table 3: Summary of the Effect of Policies and Measures Included in the Projections (MtCO2)

|  | With measures | With additional measures |
| --- | --- | --- |
| Energy (total, excluding transport) | 23.7 | 9.3 |
| Energy supply | 20.4 | 5.6 |
| Energy – industry, construction | 0.9 | 0.5 |
| Energy – other (commercial, residential, agriculture) | 2.4 | 3.3 |
| Transport (energy) | 1.7 | 0.6 |
| Industrial processes | 0.0 | 0.7 |
| Waste | 2.9 | 0.2 |
| Agriculture | 0.0 | 0.1 |
| Total (excluding sinks) | 28.3 | 11.0 |

- Table 4: Detailed Information on Polices and Measures

| Sector | Name | Objective | GHG affected | Type of instrument |  | Status | Implementing entity | Estimate of savings (MtCO2) |  | ### CCPM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 2010 | 2020 |  |
| Policies and measures in the “with measures” projection |  |  |  |  |  |  |  |  |  |  |
| Energy | Natural gas in electricity generation | Electricity generation | CO2 |  | Economic | Implemented | PPC | 4.8 |  |  |
| Energy | Natural gas in electricity generation | Electricity generation | CO2 |  | Economic | Adopted | PPC/Private | 8.7 |  |  |
| Energy | Electricity generation from industrial auto-producers using natural gas | Electricity generation | CO2 |  | Economic | Adopted | MD | 2.1 |  |  |
| Energy | Cogeneration | Power/heat generation | CO2 |  | Economic | Implemented | MD | 0.16 |  |  |
| Energy | Cogeneration | Power/heat generation | CO2 |  | Economic | Adopted | MD | 0.67 |  |  |
| Energy | Natural gas in industry | Thermal uses | CO2 |  | Economic | Implemented | MD | 0.9 |  |  |
| Energy | Natural gas in residential/tertiary sector | Thermal uses | CO2 |  | Economic | Implemented | MEPPPW | 1.2 |  |  |
| Energy | CNG Buses | Road transport | CO2 |  | Economic | Implemented | MTT | 0.007 |  |  |
| Energy | Improvements in conventional power generation system | Electricity generation | CO2 |  | Economic | Implemented | PPC |  |  |  |
| Energy | Wind energy | Electricity generation | CO2 |  | Economic, regulatory | Implemented | RAE/MD/Private | 0.95 |  |  |
| Energy | Wind energy | Electricity generation | CO2 |  | Economic, regulatory | Adopted | RAE/MD/Private | 2 |  |  |
| Energy | Small hydroelectric units | Electricity generation | CO2 |  | Economic, regulatory | Implemented | RAE/MD/Private | 0.08 |  |  |
| Energy | Small hydroelectric units | Electricity generation | CO2 |  | Economic, regulatory | Adopted | RAE/MD/Private | 0.34 |  |  |
| Energy | Large hydroelectric units | Electricity generation | CO2 |  | Economic | Adopted | RAE/MD/Private | 0.37 |  |  |
| Energy | Photovoltaic units | Electricity generation | CO2 |  | Economic | Implemented | RAE/MD/Private | 0.002 |  |  |
| Energy | Biomass | Thermal and electric uses in industry | CO2 |  | Economic, regulatory | Implemented | RAE/MD/Private | 0.26 |  |  |
| Energy | Solar energy in the residential sector | Electricity substitution | CO2 |  | Economic | Implemented | MEPPPW | 1.2 |  |  |
| Energy | Solar energy in the tertiary sector and in industry | Electricity/oil products substitution | CO2 |  | Economic, regulatory | Implemented | MEPPPW | 0.004 |  |  |
| Energy | Road transport | Road transport | CO2 |  | Economic, regulatory | Adopted | MTT | 1.2 |  |  |
| Transport | ACEA agreement | Road transport | CO2 |  | Regulatory | Adopted | MTT | 0.45 |  |  |
| Waste | Landfill directive | Waste management | CH4 |  | Regulatory | Adopted | MEPPPW | 2.9 |  |  |

Policies and measures in the “with additional measures” projection

| Energy | Cogeneration | Thermal uses and electricity generation | CO2 | Economic | Planned | MD | 0.26 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Energy | Natural gas in residential/tertiary sector | Thermal uses | CO2 | Economic | Planned | MEPWPP | 0.17 |  |  |
| Energy | Wind energy | Electricity generation | CO2 | Economic | Planned | MD/RAE/Private | 1.5 |  |  |
| Energy | Small hydroelectric units | Electricity generation | CO2 | Economic | Planned | MD/RAE/Private | 0.58 |  |  |
| Energy | Photovoltaic units | Electricity generation | CO2 | Economic | Planned | MD/RAE/Private | 0.03 |  |  |
| Energy | Geothermal energy units | Electricity generation | CO2 | Economic | Planned | MD/RAE/Private |  |  |  |
| Energy | Biomass | Electricity generation | CO2 | Economic | Planned | MD/RAE/Private | 0.4 |  |  |
| Energy | Solar energy in the residential sector | Electricity substitution | CO2 | Economic | Planned | MEPWPP | 1 |  |  |
| Energy | Solar energy in the tertiary sector and in industry | Electricity/oil products substitution | CO2 | Economic | Planned | MEPWPP | 0.2 |  |  |
| Energy | Wider use of natural gas in power plants | Electricity generation | CO2 | Regulatory | Planned | RAE/MD/PPC | 2.8 |  |  |
| Energy | Natural gas in industry | Thermal uses | CO2 | Economic | Planned | MD | 0.2 |  |  |
| Industry | Energy conservation interventions | Thermal/electric uses | CO2 | Economic | Planned | MD | 0.28 |  |  |
| Industry | Recovery of F-gases from discarded equipment | F-gasses consumption | HFCs | Regulatory | Planned | MD | 0.7 |  |  |
| Transport | Improvements in road signalling | Road transport | CO2 | Economic | Planned | MTT | 0.07 |  |  |
| Transport | Promotion of public means of transport | Road transport | CO2 | Economic | Planned | MTT | 0.54 |  |  |
| Waste | Flaring of landfill gas | Waste management | CH4 | Regulatory | Planned | MEPPPW | 0.2 |  |  |
| Domestic | Improvement of the thermal behaviour of existing buildings | Energy conservation | CO2 | Economic, regulatory | Planned | MEPPPW | 0.1 |  |  |
| Domestic | Systematic maintenance of central heating boilers | Energy conservation | CO2 | Economic, regulatory | Planned | MEPPPW | 0.2 |  |  |
| Domestic | Replacement of central heating boilers | Energy conservation | CO2 | Economic, regulatory | Planned | MEPPPW | 0.06 |  |  |
| Domestic | External shading of buildings, night ventilation and use of roof fans | Energy conservation | CO2 | Economic, regulatory | Planned | MEPPPW | 0.05 |  |  |
| Domestic | Energy efficient air conditioning units | Energy conservation | CO2 | Regulatory | Planned | MD | 0.23 |  |  |
| Domestic | Energy efficient electric appliances | Energy conservation | CO2 | Regulatory | Planned | MD | 0.26 |  |  |
| Domestic | Replacement of incandescent bulbs by high efficiency ones | Energy conservation | CO2 | Information | Planned | MD | 1 |  |  |
| Domestic | Advanced lighting control systems | Energy conservation | CO2 | Economic, regulatory | Planned | MEPPPW | 0.02 |  |  |
| Agriculture | Manure Management systems | Agriculture | CH4 | Economic | Planned | MA | 0.07 |  |  |
| Agriculture | Organic farming | Agriculture | N20 | Economic | Planned | MA | 0.07 |  |  |

## Evaluation of Projections

- Table 5: Summary of projections by gas in 2010 (MtCO2)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| Carbon dioxide (excl. sinks) | 84.0 | 123.0 |  |
| Methane | 10.0 | 9.0 |  |
| Nitrous oxide | 14.2 | 14.0 |  |
| HFCs* | 3.4 | 4.4 |  |
| PFCs | 0.1 | 0.0 |  |
| SF6 |  |  |  |
| Total (excl. sinks) | 111.7 | 150.4 | 139.5 |

*base year for F-gases is 1995

Table 6:  Summary of projections (6 gas basket) by sector in 2010 (MtCO2eq)

|  | Base year | with measures | % change relative to 1990 | with additional measures | % change relative to 1990 |
| --- | --- | --- | --- | --- | --- |
| Energy (total, excluding transport) | 66.1 | 96.2 | 46% | 86.3 | 31% |
| Transport (energy) | 15.6 | 24.1 | 54% | 24.1 | 54% |
| Industrial processes | 8.7 | 13.9 | 60% | 13.2 | 52% |
| Solvents | 0.2 | 0.2 | -6% | 0.2 | -6% |
| Waste | 5.4 | 3.7 | -31% | 3.5 | -35% |
| Agriculture | 13.5 | 12.3 | -9% | 12.1 | -10% |
| Total (excl. LUCF) | 109.5 | 150.4 | 37% | 139.4 | 27% |

Table 7: Assessment of the Target

|  | MtCO2 equiv. | % of base year level (six gas basket) |
| --- | --- | --- |
| Base year emissions (from projections) | 111.7 | 100% |
| Kyoto Commitment/burden sharing | 139.6 | 25% |
| With existing P&Ms projections | 150.4 | 135% |
| Gap (-ve means no gap) | 10.8 | 10% |
| With additional P&Ms projections | 139.5 | 125% |
| Remaining gap | -0.1 | 0% |
| Remaining gap (if target includes flex mechs) | -0.1 | 0% |

Table above excludes LUCF.  Please refer to the LUCF section in the main report for further information.

## Description of Modelling ApproacH & indicator reporting

For scenario development and projections two main model types / procedures have been used:

1) Economic-technical model (ENPEP) for the energy sector (energy supply and use – fugitive emissions).

2) Spreadsheet models for the non-energy sectors, in which future changes in activity data are mainly derived from statistical analysis while emission factors are derived from expert assessments based on the IPCC/CORINAIR methodology.

Modelling parameters additional to core indicators

| Parameter | 2000 | 2010 | Unit |
| --- | --- | --- | --- |
| Population (M) | 10.92 | 11.26 |  |
| Household size | 2.82 | 2.65 |  |
| GDP (bn Euro 2000) | 123.2 | 179.2 |  |
| International fuel prices |  |  |  |
| Coal ($2000/t) | 33.8 | 37.2 |  |
| Oil ($2000/bbl) | 27 | 23.5 |  |
| Natural gas ($2000/toe) | 121.4 | 125.7 |  |
| Transport activity |  |  |  |
| Passenger transport (bn pkm) | 128 | 193 |  |
| Goods transport (bn tkm) | 27 | 38 |  |

Table 8) Indicators for projections to monitor and evaluate progress with policies and measures (2005/166/EC) Annex III

| Sector | Index | 2005 | 2010 | 2015 | 2020 |
| --- | --- | --- | --- | --- | --- |
| Total | Emissions intensity | 769 | 686 | 612 | 563 |
|  | (t CO2 / M€) |  |  |  |  |
| Transport | Specific CO2 emissions of passenger cars | 0.072 | 0.063 | 0.06 | 0.058 |
|  | (kt CO2/Mp-km) |  |  |  |  |
| Transport | Specific CO2 emissions of road freight transport | 0.35 | 0.32 | 0.31 | 0.29 |
|  | (kt CO2/Mt-km) |  |  |  |  |
| Industry | Emissions intensity | 532 | 496 | 460 | 429 |
|  | (t CO2 / M€) |  |  |  |  |
| Residential | Specific CO2 emissions of households | 2.39 | 2.6 | 2.7 | 2.79 |
|  | (t CO2 / household) |  |  |  |  |
| Tertiary | CO2 emissions intensity of the services sector | 13.27 | 14.08 | 13.72 | 13.44 |
|  | (t CO2 / Μ€) |  |  |  |  |
| Energy | Specific CO2 emissions of conventional power plants | 304 | 274 | 260 | 248 |
|  | (t CO2 / TJ) |  |  |  |  |
| Agriculture | Specific N20 emissions of fertiliser and manure use | 0.02 | 0.02 | 0.02 | 0.02 |
|  | (kt N2O / kt N) |  |  |  |  |
| Agriculture | Specific CH4 emissions of cattle production | 80.2 | 79.86 | 79.52 | 79.19 |
|  | (kg CH4 / head) |  |  |  |  |
| Waste | Specific CH4 emissions from landfills | 0.04 | 0.04 | 0.03 | 0.02 |
|  | (kt CH4/kt waste) |  |  |  |  |

## A comprehensive time series of indicators was provided and is summarised in the table above.  In future years, Greece should aim to also submit the numerator in addition to the indicator.

## Country Conclusions

Greenhouse gas projections for Greece are presented in the June 2005 Emissions Projections. Greek emissions of the basket of six greenhouse gases are projected to be 35% above the base year levels in 2010 for the “with measures” scenario. For the “with additional measures” scenario, emissions are 25% above the base year, which means Greece will meet its Kyoto Target of a 25% increase, with additional measures.

For the “with measures” scenario, Greece is expected to underachieve its commitment – increasing its emissions to 2010 by 10% above its commitment. For the “with additional measures” scenario Greece would meet to its target.  Land use change and forestry are excluded from this projection.

The report split projections into five broad sectors and four gases. Projections were presented clearly and broken down further by sub-sector. Summaries of the data were good, but report did lack a “with additional measures” summary by gas. The reporting of the effect of policies and measures detailed, although savings to 2020 would have given a more comprehensive quantitative assessment. Data on carbon dioxide, methane and nitrous oxide were in included in this table.  Presentation of the effect of policies and measures was clear and detailed, providing useful information for analysis.