BULGARIA

Sources of Information	1

Reporting	1

Assessment of Policies and Measures	1

Evaluation of Projections	8

Kyoto Mechanisms	9

Description of Modelling Approach	9

Country Conclusions	10

- Bulgaria

- Sources of Information

The “Second National Action Plan on Climate Change 2005 – 2008 / NAPCC /” http://www.moew.government.bg/recent_doc/international/climate/NAPCC_Final_English.doc

Personal Communication, Siyka Kalimanova, Bulgaria, October 2005

- Reporting

The information presented in this report is from the Second National Action Plan.  Bulgaria has also submitted three National Communications to the UNFCCC.

Bulgaria reports a with measures projection as a ‘baseline’ scenario and three scenarios of ‘additional measures’ relating to different focus of measures (all in the energy sector):

Scenario 1 (V1). Implementation of measures to improve energy efficiency in the demand-side sectors

- Scenario 2 (V2). Implementation of measures for promoting energy from renewable energy sources.

- Scenario 3(V3). Implementation of measures affecting conventional energy production.

The projections are given only as totals and not by sector or gas.

The main assumptions underlying the projections are given but the methodology used is not described.

The action plan gives details of policies and measures, including estimates of the reduction potential at 2010 and of the cost in terms of three cost categories:

- Low: up to 10 Euro per ton CO2 Eq.

- Medium: 10-50 Euros per ton CO2 Eq.

- High: more than 50 Euro per ton CO2 Eq.

The policies and measures included in the different projections are specified.

- Assessment of Policies and Measures

The following four categories of actions are included in the updated Plan:

- Actions to implement mitigation policies and measures that result in GHG emission reductions in the different economic sectors in Bulgaria;

- Actions to create the necessary conditions for implementation of the mitigation measures, for instance institutional arrangements and awareness raising;

- Actions related to the monitoring and registration of GHG gases and the systematic evaluation of emission trends and projections, including the evaluation of policies and measures;

- Actions for the implementation of the Joint Implementation and Emissions Trading schemes.

The NAPCC comprises mainly measures that do not require budget funding/financing, as the economic situation in Bulgaria does not allow for the allocation of significant funding and other public resources to implementing climate change policies and measures. Although the plan indicates set of “reserve”/”additional” policies and measures which can be applied in case if there is observed/evident accelerated increase of the GHG emissions more than the expected one or when the financial situation in Bulgaria allows more active cooperation/partnership of the Government in this field.

The measures included in the baseline or with measures projections are given in Table 1.

Table 1) Measures included in the baseline projection for Bulgaria

Energy production

- Decommissioning of units 3-4 in NPP Kozloduy (-) - New nuclear unit in 2014 and 2018 (+) - No new large scale hydropower projects (-)

Industry

- Maintenance of liberalization and market principles without subsidies and preferences (+) - Energy efficiency measures with pay-back period less than 2 years (+)

Residential and commercial/institutional buildings

- Efficiency improvement of district heating (+) - Efficiency improvement of street lightning (+)

Transport

- Minimizing of subsidies (-)

Agriculture/forestry

- None

Waste

- Reduction of waste generation (+) - Introduction of methane capture at all new landfills (+)

Note: The effect on GHG emission reduction is indicated with a plus (for emissions reductions) or a minus (for emissions increases)

For the economic sectors reviewed in the Second NAPCC, a total of 24 policies and measures are included that are additional to those included in the baseline scenario. These policies and measures use one or more policy instruments, such as:

- Legal instruments: implementation of a certain EU Directive; implementation of different law’s acts, privatization, etc.

- Market-based instruments: participation in International Emission Trading, EU Emission Trading Scheme;

- Financial instruments: funds and different investment sources like Green Investment Scheme, Energy Efficiency funds, etc. and state loan guarantees.

- Scientific and research activity;

- Voluntary agreements, etc.

The table below shows the additional estimated reduction that can be expected from implementing these sectoral measures.

Table 1) Reductions from additional measures in Bulgaria

| Sector | Additional annual reduction in 2010 (Mton) |
| --- | --- |
| Energy | 4.1 (without measure E2, see Table 3) |
| Industry | 0.7 |
| Residential and commercial/institutional buildings | 2.4 |
| Transport | 0.1 (without measures T4 and T5, see Table 3) |
| Agriculture | 0.2 |
| Waste | 0.9 |

Table 3 gives details of the additional policies and measures from the Second NAPCC.

Table 3) Additional Policies and Measures in the Second NAPCC for Bulgaria

| Measure | Sub-sector | Activity | Instruments | Annual GHG reduction in 2010 (Mton) | Year of implementation |
| --- | --- | --- | --- | --- | --- |
| Energy Sector |  |  |  |  |  |
| E1 | Nuclear | Improvement existing power plant | Finance | 0.9 | 2007 |
| E2 | Hydro | New capacity | Legal Finance | 0.4 (after 2012) | Depending on implementation GC and additional finance |
| E3 | Small and micro hydro | New capacity | Legal Finance | 0.2 | Depending on implementation GC and additional finance |
| E4 | Cogeneration and district heating | Upgrading | Legal Finance | 0.9 | Depending on implementation GC, Large Combustion Plant Directive and additional finance |
| E5 | T&D networks | Reduce losses | Legal | 1.1 | 2005 |
| E6 | Heat transmission networks | Reduce losses | Legal Finance | 0.9 | Gradually (depending on regulation SERC and additional funding) |
| E7 | Biomass | Combustion | Legal Finance | 0.05 | Depending on implementation GC and additional finance |
| Industry |  |  |  |  |  |
| I1 | All sub-sectors | Reduction of thermal losses in industry | Legal Finance VA | 0.12 | Depending on source of financing |
| I2 | All sub-sectors | Increased use of natural gas industry | Finance | 0.05 | Dependent on funding for network construction |
| I3 | Non-ferrous metallurgy Light industry Machine building, electrical and electronic industry | Monitoring systems of energy use in industry | VA | 0.11 | Gradual implementation from 2006 |
| I4 | Light industry Food industry Machine building electrical and electronic industry | Updating of the steam generation and compressed air plants | Legal Finance VA | 0.02 | Gradually from 2007 |
| I5 | Construction | Introduction of highly-efficient construction machines using diesel fuel | Legal | 0.07 | Gradual penetration from 2006 |
| I6 | Building materials industry | Replacement of part of the fuel by motor-car tyres and other combustible waste | Legal VA | 0.34 | Gradually from 2005 |
| Tertiary |  |  |  |  |  |
| H1 | Households | Gas supply to households | Finance Awareness/Information | 2.3 | Gradually, but strongly depending on available financing |
| H2 | Municipal and Public | Installation of solar collectors | Finance | 0.02 | 2006, depending on availability of finance |
| H3 | Municipal and Public | Hybrid and other RES hot water installations | Finance | 0.04 | Dependent on availability of financing |
| Transport |  |  |  |  |  |
| T1 | Road/railway | Introduce cargo dispatch system | Fiscal Awareness and campaign | 0.03 | Depending on private sector initiative |
| T2 | Railway | Introduce railway power dispatch system | Finance Legal Awareness and campaign | 0.09 | 2005 |
| T3 | Railway | Modernization | Legal | 0.04 | Depending on ongoing improvements in the railway sector |
| T4 | City transport | Improving public transportation and reducing transportation in cities | Legal Fiscal Finance Awareness and campaign Strict/strong control | n.e. | Depending on availability of subsidies and regulation |
| T5 | Road transport | Increasing the use of biofuels | Legal Fiscal Finance Awareness and campaign | n.e. | Depending on the production of biofuels |
| Agriculture |  |  |  |  |  |
| A1 | Animal’s breeding | Manure management | Finance R&D | 0.07 | Gradually from 2005 |
| A2 | Agricultural fertilization and Irrigation | Improved fertilization practices Improving the irrigating technologies in order to reduction of the water consumption | Legal Finance R&D SAPARD and State Fund “Agriculture” | 0.17 | Gradually from 2005 |
| Waste |  |  |  |  |  |
| W1 | Landfills | Electricity production from methane capture | Legal Finance | 0.09 | Depending on GC and/or additional funding |

- Evaluation of Projections

The data in Tables 4 and 5 are from the second National Action Plan.

Table 4: Summary of projections in 2010 (MtCO2 equivalent) (no disaggregation by gas available.

|  | Base year 1988 | with measures | with additional measures |
| --- | --- | --- | --- |
| CO2 |  |  |  |
| CH4 |  |  |  |
| N2O |  |  |  |
| HFC |  |  |  |
| PFC |  |  |  |
| SF6 |  |  |  |
| Total | **138.4 | 79 | *73.5 |

Based on the average between the highest (75.8) and lowest (72.3) with additional measures projection.

Source: Projections in “Second National Action Plan on Climate Change 2005 – 2008 / NAPCC /”

**Base year was revised based on Personal Communication from Bulgaria in October 2005

- The with measures projection shows that the currently implemented or adopted measures of Bulgaria could reduce greenhouse gas emissions by 43% from the 1988 level by 2010 and with additional measures by approximately 47%.  Bulgaria therefore more than meets its Kyoto target of an 8% reduction in emissions compared to the base year.

Table 5) Assessment of the Target (2010)

| Assessment of the Target |  |  |
| --- | --- | --- |
|  | Excluding sinks |  |
|  | MtCO2eq | % 1988 level |
| **Base year 1988 | 138.4 | 100.0 |
| Commitment | 127.3 | 92.0 |
| With existing P&Ms | 79.0 | 57.1 |
| Gap (-ve means no gap) | -48.3 | -34.9 |
| *Effect of additional P&Ms | 5.5 | 4.0 |
| Remaining gap | -53.8 | -38.9 |

Based on the average between the highest (75.8) and lowest (72.3) with additional measures projection.

Source: Projections in “Second National Action Plan on Climate Change 2005 – 2008 / NAPCC /”

**Base year was revised based on Personal Communication from Bulgaria in October 2005

- Kyoto Mechanisms

Bulgaria has already gained substantial experience with JI projects development and approval. A number of investing countries, including Austria, Switzerland, Netherlands and Denmark, have already signed a Memorandum of Understanding with Bulgaria.

Bulgaria sees Joint Implementation in its current form as a temporary instrument for the promotion of emission reduction projects. At a later stage, a Green Investment Scheme is regarded as a potentially more effective instrument because of the larger flexibility in the condition for supporting GHG reduction projects.

- Description of Modelling Approach

The with measures projection is based on the assumption that the policies and measures formulated in the First Action Plan will be implemented as well as all European pre-accession policies, which are already approved by the Bulgarian Government.

Economic and demographic developments as well as general economic policies have caused a significant decrease of the GHG emissions in Bulgaria in the period 1988-2002. All subsidies for fuel and energy are assumed to be terminated, and the energy, the energy efficiency and the renewable energy sources develop on the basis of the economical efficiency. All European pre-accession programmes of the Government, currently approved, are implemented.

For the demographic development official projections of the Government are used. According to these figures the population size will further decline with 0.8% per year in the period 2003-2009, followed by an annual reduction of 0.6% in the period 2010-2020. As a result, it is projected that in 2020 the country population will reach 6.9 million people.

The GDP forecast assumes a 5.25% growth for 2004-2005 and 5.5% for the 2006-2016 periods. After then, the growth gradually declines to 3.5% in 2020. These growth figures are the official projections of the Agency of Economic Development to the Council of Ministers taken into account the positive impact of the EU pre-accession programs and the EU accession itself. The potential for accelerated growth exhausts by 2017. After then, it is assumed that the growth stabilizes at the average EU growth level.

The forecast of total GHG emissions are based on the cumulative forecast for each sector but the methodology used is not described in detail.

- Country Conclusions

The main document used for this summary was the Second National Action Plan.  The level of detail in policies and measures chapter is in line with UNFCCC guidelines.  The details of the methodology for the projections is only partly described.  The scenarios are only provided for total greenhouse gas.

Bulgaria did not submit projections in 2006 to the EC, however, they provided comments on the country profile in October 2005.  The revisions stemming from these comments have lead to a reduction in base year by 3.4Mt CO2 eq, but this is a small decrease compared to actual emissions in the base year and corresponds to a decrease of 2% compared to the base year reported previously.  Bulgaria is still comfortably meeting their Kyoto target.

The analysis of the emissions during the First Kyoto period shows that according to the base (with measures) scenario, Bulgaria will have 48 Mt available for trading. If Bulgaria successfully implements the additional planned measures, then it will over achieve the Kyoto target even further and have 54 Mt СО2 eq available for trading.