# Greenhouse Gas Emissions Inventories Report from Spain
1990-2000

communication to the european commission
(decision 1999/296/ce)

Directorate-General for Environmental 
Quality and Assessment

Ministry of the Environment

Madrid, March 2002

### contents

1.- PRESENTAtion	1

2.- GENERAL METHODOLOGY	3

3.- DEvelopment of the inventory principles	7

4.- RECALCULATIONS	10

5.- RESULTS AND KEY SOURCES ANALYSIS	22

6.- RESULTS COMPARISON (1990-1999) ed. 2002 vs. ed. 2001	39

##### 1.- PRESENTAtion

The present document is the edition corresponding to 2002 of the report on the 1990-2000 inventories of Greenhouse Gases (GHG) presented by Spain to the Commission of the European Union by virtue of the provisions of Council Decision 1999/296/EC, amending that of 93/389/EEC, regarding a Monitoring Mechanism of Community Carbon Dioxide and other greenhouse gas emissions.

On the other hand, and bearing in mind that Spain must also report on its emissions of GHG to the United Nations Secretariat on the Framework Convention on Climatic Change (UNFCCC), there is an obvious need to ensure that the format and content of the inventory data is shared by the deliveries to both international institutions, guaranteed by the presentation of the inventories in the Common Reporting Format (CRF) defined in document FCCC/CP/1999/7 drawn up by the UNFCCC Secretariat.

As for the substances referred to in the inventory, the data in the CRF tables are given both in units of mass for each gas and in terms of CO2-equivalent for each of the compounds/groups considered in Annex A of Kyoto Protocol and that includes the six categories below with a direct effect on global warming: a) carbon dioxide (CO2), b) methane (CH4), c) nitrous oxide (N2O), d) hydrofluorocarbons (HFC); e) perfluorocarbons (PFC), and f) sulphur hexafluoride (SF6). In addition, the estimated emissions of four gases with an indirect warming effect are given in terms of mass: a’) nitrogen oxides (NOX), b’) carbon monoxide (CO), c’) non-methane volatile organic compounds (NMVOC), and d’) sulphur oxides (SOX). The amounts sequestered by sinks have been estimated solely with reference to the CO2 fixed as carbon by forestry biomass.

The current edition of the inventories corresponding to 2002 updates, and revises in some cases, the estimates provided in the previous edition corresponding to 2001 for the years in the period 1990-1999, at the same time as it extends the time series to 2000. Where necessary, the revision of the estimates for certain items in the inventories has been due to various factors including: a) the review of the input data and statistics themselves, b) changes in the estimation methodologies (factors and algorithms) as a result of the improved awareness of the emissions-generating processes, and c) in some cases, the correction of errors detected. The changes in the methodological specifications are in some cases noticeable, and have been motivated in many instances by following the complementary methodological directives established by the Intergovernmental Panel of Experts on Climatic Change (IPCC) as they appear in the document entitled “Good practice guidance and uncertainty management in national greenhouse gas inventories”, published at the end of 2000, that due to its late publication could not be take into account in the previous inventory edition.

The following sections 2, “General Methodology” and 3, “Development of Inventory Principles” summarize the methodologies and approaches used to estimate the emissions and describe to which extent we have be able to follow the recommended good practices for the compilation of these inventories. Revisions to the previous inventories from 1990-1999 are described in section 4, “Recalculations”; these are included in this edition with relation to the estimates presented in 2001. In section 5 “Results and Identification of Key Sources”, the results as accumulated inventory, by type of gas and activity sector are presented first, and continue by identifying which activities have the greatest repercussion be it in absolute values of emissions (level) or trend (tendency) of the series of inventories. Finally, section 6 “Results comparison (1990-1999) ed. 2002 vs. ed. 2001”, present and comment on the primary changes registered in the emissions estimates with regard to the previous edition.

##### 2.- GENERAL METHODOLOGY

The data shown in the set of CRF tables for this 2002 edition contain all of the relevant information on the direct and indirect emission/uptake of greenhouse gases produced in Spain during the period 1990 to 2000.

The fulfilment of the CRF tables is effected on the basis of the pertinent data available in the National Air Emissions Inventories into the Atmosphere (Inventarios Nacionales de Emisiones a la Atmósfera), maintained and developed by the Directorate-General for Environmental Quality and Assessment at the Spanish Ministry of the Environment.

In the present 2002 edition of the greenhouse gas inventory, the fulfilment of the CRF tables has been completed following the bottom-up hierarchic link structure of the CRF software itself to produce automatically from the background tables both the derived sectoral tables with the emissions estimated by IPCC activity category and also, at a level of greater aggregation, the summary tables for levels and trends.

A brief description is given below of the work carried out to draw up the greenhouse gas inventory and particularly how the CRF tables are completed on the basis of the data from the National Air Emissions Inventory System.

The approaches for the emissions estimations recommended in the “IPCC Guidelines for national greenhouse gas inventories, 1996 revised version” were adopted for all those activities for which the said approaches were considered the most precise, taking into account the resources and data available. In those cases where a national approach was available and was considered more appropriate than the alternative IPCC approach, the national approach was adopted in line with the recommendations of the IPCC itself. Thus, with respect to the crosstabs of gas type and emission-producing activity, the approaches adopted have been those indicated in the following sub-section.

2.1.- METHODOLOGIES APPLIED BY IPCC ACTIVITY CATEGORY

Energy: Combustion Processes

The carbon mass balance has been used for the estimation of the CO2 emissions, taking the most specific national parameters as the fuel characterization, either provided by the emissions producers themselves, as in the case of the Large Point Sources, or else derived from the standard fuel specifications.

The following approaches have been used for the other pollutants:

-	Estimations based on direct measurements, mainly in the case of SO2 and NOX at the Large Point Sources.

-	Estimations based on mass balances, mainly in the case of SO2 in the emissions from mobile sources and from stationary sources without desulphurisation technologies.

-	Estimations based on the EMEP/CORINAIR methods, including COPERT III for traffic, as these were considered more specific for the combustion installations under consideration than those proposed at the IPCC. This group generally includes the estimations of NMVOC, CH4, CO and N2O.

Energy: Fugitive Emissions

In this activity category national methods have been used when, as in the cases below, it has been possible to call on information regarding processes, emission factors or estimation algorithms considered better adapted to the sector’s activity in Spain:

-	CH4 emissions in the mining and use of coal.

-	CH4, NMVOC and CO2 emissions in the transportation and distribution of natural gas and other gaseous fuels (air with added methane/propane, propane, factory gas).

-	CO2 emissions in the (no combustive) processes for the transformation of fuels, mainly in coke plants, furnaces and oil refineries.

-	NMVOC emissions in the transportation of oil-based products.

For the remaining activities in this sector, the IPCC or EMEP/CORINAIR emission factors have been used, depending on which one was considered more representative. In this case it was found that:

-	The emissions of CH4 and NMVOC generated in the national oil and natural gas production activities; as well as the fugitive emissions of both pollutants generated in the loading-unloading and storage operations for crude oil and oil-based products at the sea terminals.

Industrial Processes

The emissions of the three main greenhouse gases directly from the activities in this sector have been estimated in accordance with the IPCC methodology.

For the indirect greenhouse gases, the approach used has been a combination of national methods complemented, when these data were missing, by EMEP/CORINAIR emission factors.

For the estimation of the emissions of fluorinated gases (HFC, PFC and SF6), the IPCC actual approach methodology was adopted. The complementary potential approach was not considered feasible as, at the maximum level of disaggregation of the foreign trade nomenclature, it was not possible to identify the commercial transactions (and it was also tremendously complex to do so in the chain of importers-distributors-exporters) by individual gas type, in other words, as:

- HFC-23; HFC-125; HFC-134a; HFC-143a; HFC-227ea, HFC-236fa in the HFC group;

- ii) CF4; C2F6; C3F8; C4F10; in the PFC group; and

- iii) SF6.

Use of Solvents and Other Products

Within this group, in which the IPCC methodology itself refers to EMEP/CORINAIR for a large number of activities, the national methods have been used and complemented by EMEP/CORINAIR factors.

Agriculture

Within the group of agricultural activities, it is necessary to distinguish the methodological handling by sub-sectors and by gas type if appropriate. We thus have:

-	With regard to the CH4 emissions from enteric fermentation in cattle, the IPCC methodology has been followed, with an advanced (tier 2) approach for cattle and sheep.

-	For the estimation of the CH4 and N2O emissions from handling manure, the IPCC methodology has been followed, supported by national manure generation and management parameters. Similar treatment (IPCC methodology supported by national emission factors) has been applied to the estimation of CH4 emissions in rice growing.

-	In the case of the N2O emissions from agricultural soils, the IPCC methodology has been followed, supported with regard to the determination of basic activity parameters and variables on national studies.

-	The pollutant emissions generated in the burning of agricultural waste have been estimated: a) for CH4 and CO, in accordance with the IPCC methodology; and b) for N2O and NOx, using this same methodology supported by national parameters.

-	In the estimation of soil NOx emissions and NMVOC emissions from agricultural land (plants) and the burning of agricultural waste, the EMEP/CORINAIR methodology has been used as there is no alternative in IPCC.

Changes in Land Use and Forestry

The estimations of the net uptake of CO2 corresponding to this IPCC category are obtained by the difference between the uptakes of the forest tree biomass and the losses from that same biomass through logging and forest fires. The calculation, shown in Table 5 A of the CRF, has been effected following the IPCC methodology, using data taken from the Second National Forest Inventory and other national data sources for forestry use and monitoring.

2.2.- TREATMENT OF CARBON STORED IN FEEDSTOCKS

The consumption of fuel products for non-energy uses appears in the fuel balance sheets under the homonimous heading. The amounts of all the types of fuel listed under this heading are included in the reference approach analysis, with each of these amounts being appropriately broken down into two fractions: a) the fraction that is stored in products; and b) the fraction that is presumably released in the short-term, giving rise to the corresponding CO2 emissions in accordance with the aforesaid reference approach.

2.3.- TREATMENT OF INTERNATIONAL BUNKERS

For the purposes of pro-memo reporting, that is to say outwith the national total, for the emissions corresponding to international sea and air traffic, the activity variable have been taken to be the fuel consumption  figures appearing in the energy balance sheets as allocated to the respective international traffic flows: a) international marine bunkers; and b) international aviation, respectively.

##### 3.- DEvelopment of the inventory principles

This section comments on the developments in this 2002 edition to the principles that must be taken into account in the drafting of the inventories, and their reference to the CRF tables, as indicated in the FCCC/CP/1999/7 document of the Secretariat for the UNFCCC.

3.1.- TIME SERIES CONSISTENCY

An important feature of the compilation process has been the emphasis placed on guaranteeing that, as far as possible, the time series 1990-2000 of the country was consistently estimated over the years in accordance with the updated Revised 1996 IPCC Guidelines (and corresponding nomenclature) and the complementary document “Good practice guidance and uncertainty management in national greenhouse gas inventories”, published in 2000. The estimated figures for by-gas have been converted in terms of CO2-equivalent using the 1995 revised IPCC global warming potentials (100 years time horizon).

3.2.- RECALCULATIONS PERFORMED

The consistency objective just mentioned forced a recalculation of previously submitted, edition 2001, estimations of emissions and removals covering the period 1990-1999. It is firmly believed that this revision of the previous estimates, that is shown in Tables 8(a) of CRF, has significantly contributed to an improvement in the accuracy and completeness of the inventories series and the emissions trends derived therefrom. Basic information for the year 2000 is partly provisional (it uses projected activity data for some sectors), so it is envisaged that estimations for this year will presumably be recalculated when definite data are available.

3.3.- COHERENCE

The coherence of the estimated CO2 emissions originating from fossil fuel combustion activities has systematically been taken into account through the whole inventorying process of such emitting activities. The use of fuels for these energy purposes has essentially been matched with the national energy balances as reported in the “Energy Balance Sheets” publication of EUROSTAT and “Energy Statistics” of the International Energy Agency. CO2 emissions from biomass fuels combustion is also reported within the specified “Memo” items, although not taken into account in the national CO2 emissions estimated figure. The reference approach shown in Tables 1.A(b) and 1.A(c) can be seen as a coherence checking for CO2 emissions from fuel combustion activities, although, by the very definition of its activity variable coverage, the emissions estimated according to this approach yield normally higher figures as the ones obtained following the national approach.

3.4.- COMPLETENESS

Completeness of the inventories has been assessed according to IPCC estimating status typology: NO (not occurring), EN (not estimated); NA (not available); IE (included elsewhere); C (confidential), 0 (less than half a unit). It is believed that the completeness has been satisfactorily met, with the major caveat that CO2 removals by sinks have only been estimated for IPCC source “Changes in forests and other woody biomass stocks” and that no estimates of potential emissions have been calculated for fluorinated gases (HFC, PFC, SF6).

In short, the list of IPCC categories not covered in the inventories, ordered by gas species affected, is:

Carbon dioxide:

Sector: Land use change and forestry; categories: 5B-Forest and grassland conversion, 5C-Abandonment of managed lands, 5D-CO2 emissions and removals from soil.

All these three categories have not been estimated due mainly to lack of relevant statistical data. Additionally, the IPCC methodology, as it stands in the Revised 1996 IPCC Reference Manual raises, for some items, significant practical difficulties. It is expected that the expected new IPCC Guidelines for this group contribute to clarifying concepts and facilitating their implementation. For the case of Spain, categories 5B and 5D seem a priori the most relevant, category 5C having in principle a limited potential for the net balance of emissions. In any case, additional efforts are to be allocated to all categories of the “Land use change and forestry” sector once the IPCC estimating methodology under way receives approval.

Fluorinated gases, potential emissions

Sector: Industrial processes – potential emissions of fluorinated gases.

It has not been possible to estimate the potential emissions due to the reasons explained in section 2.1 regarding the limitation of data sources. This shortage of estimations only affects Table 2(II) Sheet 2/2.

3.5.- UNCERTAINTY/ESTIMATION QUALITY

The assessment of uncertainty, or symmetrically the estimation quality, has been shown in Table 7 of the CRF using the quality codes H (high), M (medium), and L (low) for each pertinent crosstab of gas type and IPCC activity category. This ordinal classification of quality is only a first stage in the analysis of the uncertainty associated with the inventory estimations. We are currently beginning work on the implementation of a quantitative estimation of uncertainty in accordance with the approach recommended in the “IPCC Good Practice Guidelines” document.

3.6.- TRANSPARENCY

From a formal standpoint, the fulfilment of the CRF background tables with the inclusion of the activity variables, estimated emissions and implicit emission factors, as well as the complementary information included in the said tables, as appropriate, represents the most significant advance towards the achievement of data transparency in the preparation of the inventories. In addition, the transparency requirements are dealt with through the documentation and filing of the background information sources which, over and above what can logically be reflected in the background tables, have been used in preparing the inventories.

##### 4.- RECALCULATIONS

Recalculations have been performed for the period 1990-1999, in some cases with notable influence in the level and trend of the emission inventories, for a group of activities which are subject to comment on an individual basis further below in section 4.2, “Comments by Activity”. These comments have been included also in the first part of tables 8(b) of the CRF format, and are motivated by the changes in the estimation methodology, which in turn, normally leads to the revision of the representative activity rates and of the emission factors, or by corrections to previous errors detected.

For the most recent sub-period 1997-1999, we should point out that in addition, new calculations have been made in all those cases when an updated version of the activity rates was available; these may have been caused by the refreshing of estimates in the original sources, or simply projections used in the previous edition. In section 4.1, titled “General Comments”, the most relevant general sources of information that affect this type of estimates are enumerated. These comments are also included in the second part of the tables 8(b) of the CRF format.

4.1. GENERAL COMMENTS

The changes in the activity rates for the years 1997-1999 within the categories of greatest emission potential are motivated primarily by changes in some of the following sources of information:

4.1.1. Large Point Sources Questionnaires (GFP)

In the case of the Large Point Sources, for which information is gathered through a specific questionnaire, data related to the year 2000 has been included in its entirety to the current edition and simultaneously corrected errors in the questionnaires corresponding to the sub-period 1997-1999. These changes could have affected plants that belong to the following sectors:

*	Utility Power Stations.

*	Integrated Iron/Steel plants.

*	Oil Refineries.

*	Paper Pulp plants.

*	Large Vehicle Paint plants.

*	Fluorinated Gases Production plants.

4.1.2. Activity Statistics

The statistical activity data has been updated for the sub-period 1997-1999 for the following sectors:

*	Transport: road, railways, aviation, other means of transportation.

In the case of agricultural mobile machinery, fishing fleet and coasting traffic, the revision extends to the complete period 1990-1999.

*	Waste Treatment (years 1998 and 1999).

*	Agriculture: crops and livestock.

4.1.3. Energy Balances with special reference to CO2

Although the tallying of the fuels balance was performed in the previous version as well the current, there are some notable discrepancies, especially in the year 1999 fuels consumption data between one version and the other. It must be noted that at the time of writing this report, the final version of the Fuels Balance of 2000 was not available. A similar situation occurred at the time of writing the previous edition with regard to the Fuels Balance of 1999. This motivated the need to operate with projected figures in the previous version, which have provoked a very important recalculation for the year 1999 in the category 1.A.1, “Energy Sector Industries”.

On the other hand, it must be taken into account that the International Energy Agency has revised the historical series of Fuel Balances which appears in its publication titled: “Energy Statistics of OECD Countries”, re-assigning fuel consumption within activities of the same sector, which has motivated recalculations of the activity rates in sectors, such as the Iron/Steel, that cover at least two categories of the IPCC activities (transformation and industrial processes sectors).

4.2. COMMENTS BY ACTIVITY

Recalculations by activity category, discussed in this sub-paragraph, are sorted by type of gas for those sectors whose estimated emissions have experienced significant changes for the period 1990-1999 with regard to the estimations provided in the previous edition. The following codes indicated next to the description of activity are used to reference the reasons of the changes (please note that several codes could be used simultaneously to refer to changes in one activity):

R:	reallocation of activity within the nomenclature of sources/sinks in the IPCC.

A:	addition of the activity in the inventory.

Re:	removal of the activity in the inventory.

EM:	estimation method changes.

AR:	activity rate changes.

EF:	emission factor changes.

Carbon Dioxide

1.A.1. Fuel combustion energy industries

  - Utility power stations: AR. Emissions of a type of fuel have been included which, by omission, had been left out in the previous edition and the estimates of the sector as a whole have been revised for the sub-period 1997-1999. Because the readjustment has been performed to the activity rate, it also implies a change to the CH4 and N2O estimates.

  - Incineration of Municipal Solid Wastes (MSW): R. The inclusion of the incineration activities of MSW in this category exclude them from category 6C, where they had been previously included. The decision for the change is mainly motivated by the fact that the majority of the incineration sites reutilise energy; and according to the IPCC methodology, these must be included in the energy processing activities. The reclassification by category has been performed for all years from 1990 to 2000, although the energy re-utilization at these sites has been more widespread nearing the end of the period studied. Because the readjustment has been performed to the activity rate, it also implies a change to the CH4 and N2O estimates.

1.A.2. Construction and Manufacturing industries

  - Coke used in the Iron/Steel industry: AR. A re-adjustment of the coke used in the Iron/Steel industry has been made in the current edition between the two primary items which appears in the fuels balance: i) as a final fuel in the Iron/Steel industry; and ii) as a reduction agent in the blast furnace processes. This readjustment has been motivated by the revision of the historical series of fuel balances from the International Energy Agency (IEA). The methodology readjustment, in line with the IEA, has caused a reduction of the coke quantity considered as fuel for the Iron/Steel industry, and an increase of the part considered as a reducing agent (the last adjusted to achieve a balance in terms of carbon content between the in and out flows of blast furnaces). The final result of the readjustment has implied a reduction with regard to the estimates of the previous edition. Because the readjustment has been performed to the activity rate, it also implies a change to the CH4 and N2O estimates.

  - Revision of some fuel characteristics. EF. In the current edition, several characteristics have been reviewed (net calorific value and carbon content) of some fuels used in the industry, so as to adjust them to values considered more representative of the reality according to information provided by some industry sector associations.

1.A.3. Transport

  - Fuel consumption revisions in coasting traffic: AR. In the current edition, the fuel consumption in coasting traffic series has been revised according to the new estimates provided by the Spanish Merchant Shipping Association. Because the readjustment has been performed to the activity rate, it also implies a change to the CH4 and N2O estimates.

1.A.4. Other sectors

  - Revision of fuel consumtion of agricultural mobile machinery: AR. In the current edition, the fuel consumption series 1990-2000 used by agricultural machinery has been reviewed. This change has been motivated by the standarization of specific fuel consumption parameters (litres/BHP) with the other types of mobile machinery (forest, public works, etc.). Because the readjustment has been performed to the activity rate, it also implies a change to the CH4 y N2O estimates.

1.B.1. Fugitive Emissions from solid fuels

  - Emissions from cokes: A. In the current edition, the fugitive emissions produced by the cokes, calculated in terms of CO2 as the difference between the carbon content in the obtained products and those using during the coking process, have been included in this category.

2.B. Chemical Industry

  - Reclassification of the silicon carbide production: R. In the current edition, the production of silicon carbide has been reclassified to group 2.B., “Chemical Industry” from previous classification in group 2.C “Metal Production”.

2.C. Metal Production

  - Reclassification of the silicon carbide production: R. In the current edition, the production of silicon carbide has been reclassified to group 2.B., “Chemical Industry” from previous classification in group 2.C., “Metal Production”.

  - Aluminium Production: EF. In the current edition, the CO2 (and CO) emission factors corresponding to the production of pre-baked anodes for the period 1990-1996 have been reviewed since the previous edition.

2.D. Other Industries

  - Paper Pulp Plants: Re According to the non-admittance in the corresponding cells of the CRF base table, the assignment of CO2 emissions from the manufacture of paper pulp have been removed in the current edition.

3. Use of Solvents and other products

  - Manufacture/treatment of chemical products: Re. According to the non-admittance in the corresponding cells of the CRF base table, the conversion of CO2 from the emissions of NMVOC has been removed from this group of activity.

6.C. Residue Incineration

  - Incineration of Municipal Solid Waste: Re This activity category has been removed and included in category 1.A.1. (please refer to 1.A.1. category comments)

Methane

1.A.1. Energy Industries

  - Utility power stations: AR. Please refer to comments for the CO2 case.

  - Incineration of Municipal Solid Waste: R. Please refer to comments for the CO2 case.

1.A.2. Manufacturing and construction industries

  - Coke used in Iron/Steel industry: AR. Please refer to comments for the CO2 case.

  - Revision of some fuel characteristics. EF. Please refer to comments for the CO2 case.

1.A.3. Transport

  - Revision of fuel consumption in coasting traffic: AR. Please refer to comments for the CO2 case.

1.A.4. Other sectors

  - Revision of fuel consumption in agricultural mobile machinery: AR. Please refer to comments for the CO2 case.

1.B.1. Solid Fuels fugitive emissions

  - Coal mining and handling: EF. In the current edition, the emission factors for surface and underground coal mining as well as post-mining activities have been revised. In underground mining, the coefficients have been removed, which depending on the type of coal (anthracite, soft coal, black lignite), were being used in the previous edition to go from the quantities of commercial coal to coal with methane (in strict sense). After a discussion with experts in the industry it became clear that such correction was not adequate as the activity rate (amount gross coal mined) was directly related to the methane content ratios available as per different types of coal. This change is also implemented in the emission factors of surface mining and post-mining activities, as these emission factors are in relation to underground mining factors, according to the average proportions defined in equations 2 to 5 of the sub-paragraphs 1.7.2.3 and 1.7.2.4 of the IPCC Reference Manual.

1.B.2 Liquid and gaseous fugitive emissions

  - Losses in the distribution of natural gas: EM, AR, EF. In the current version, the fugitive emissions from losses during transportation and distribution of natural gas have been estimated with the information obtained by means of questionnaires from the companies in the gas industry sector. The emissions estimates are based on a sampling of losses, which in turn is extrapolated to the network total according to its construction characteristics and functioning; then the same emission factors are applied per unit of volume to other canalised gases (liquefied petroleum gases, methanized air, propanized air, etc.). This estimation method is considered to be more accurate and replaces the one used in the previous edition which was based on the calculation of generic emission factors of canalised gas consumption. It should be noted that to tally the fuel balance, the differences between the figure provided by the Fuel Balance (International Energy Agency and EUROSTAT) and the one obtained with the current method for the concept of distribution losses is now included as consumption of the energy sector (re-gasification plants and other installations of companies in the gas industry sector).

2.B. Chemical Industry

  - Reclassification of silicon carbide production: R. Please refer to comments for the CO2 case.

2.C. Metal Industry

  - Reclassification of silicon carbide production: R. Please refer to comments for the CO2 case.

4.A. Enteric fermentation

  - Enteric fermentation: EM, AR, EF. In the current edition, the tier 2 methodology has been used for the bovine and ovine livestock and the activity rates (number of livestock heads and their distribution by sex, age and production type) have been reviewed considering the averages from the available livestock surveys for each year for every type of animal, when there are more than one survey per year (2 surveys per year for bovine, and 3 for porcine). In the previous edition, default factors from the IPCC Reference Manual were being used for the bovine and ovine; and only one survey per year (December) was being performed on the number of effective livestock for all animal types.

4.B. Manure Management

  - Manure Management: AR. There has been a significant reduction in the estimated emissions as a consequence of the following changes introduced in the current edition compared to the previous one:

- the revision of the nitrogen excretion rates for each type of animal, using in the current edition the values proposed in the IPCC Reference Manual for the “Middle East and Mediterranean” region, as opposed to those used in the previous edition taken from ECETOC (1994) where the IPCC Reference Manual was also referred to.

- the adoption of national data related to the weighing of the manure management systems in replacement of the default weighing used in the previous edition based on the IPCC Reference Manual for the “Western Europe” region.

4.F. Field burning of agricultural residues

  - Field burning of agricultural residues: EM, AR. This category refers to agricultural stubbles burning. The burning of other agricultural residues is included in category 6.D. The number of vegetable species has been reduced in the current edition to those which have parameter information in table 4-17 of the IPCC Reference Manual. In the previous edition, additional species had been included for which the parameters from the abovementioned table had been “extended”.

6.A.Solid waste disposal on land

  - Solid waste disposal on land: EF. The recalculations have been motivated by the revision of the IPCC Good Practice Guidance document compared to the IPCC Reference Manual in regard to the specific parameters of the kinetic first order decay methodology of the degradable matter in MSW. The most determining are:

- the change in the DOCF (degradable organic carbon dissimilated) parameter, the default value of which changes from 0.77 to 0.55 (a reduction of 28.6%).

- the normalization factor so that the probability distribution adds to 1, which did not occur with the formula in the IPCC Reference Manual.

- Additionally, the oxidation factor has changed to 0.05 (replacing the 0 which had been used in the previous edition) and the MCF (methane conversion factor) of the controlled solid waste disposal lands which changed from 0.7 to 0.6 (assuming a 50% distribution between disposal lands with a depth greater than 5 m (MCF = 0.8) and those with less than 5 m (MCF = 0.4)).

6.B.Waste water handling

  - Waste water handling: EF. The recalculation has been motivated by the modifications in the revision of the IPCC Good Practice Guidance document with regards to IPCC Reference Manual in relation to the parameter B0, maximum methane production capacity, which when expressed as BOD (biochemical oxygen demand) changes from 0.25 to 0.6 kg CH4/kg BOD. This change affects the emission estimates in the domestic waste water treatment (not the industrial, whose Bo is referred to as COD (chemical oxygen demand), parameter that does not suffer any variations).

6.C. Residue Incineration

  - Incineration of Municipal Solid Waste: Re This activity category has been removed and included in category 1.A.1. (please refer to 1.A.1. category comments)

6.D.Other waste treatments.

  - Other waste treatments: EF. The methane emission factor in the MSW compost production activity has been considered equal to zero in the current edition; because composting is eminently an aerobic process, it is considered that this gas is not emitted (in the previous edition a factor of methane emission of 20 kg per CH4/ton of compost produced was considered).

Nitrous Oxide

1.A.1. Energy sector industries

  - Utility power stations: AR. Please refer to comments for the CO2 case.

  - Incineration of Municipal solid waste: R. Please refer to comments for the CO2 case.

1.A.2. Manufacturing and construction industries

  - Coke used in Iron/Steel industry: AR. Please refer to comments for the CO2 case.

  - Revision of some fuel characteristics. EF. Please refer to comments for the CO2 case.

1.A.3. Transport

  - Revision of fuel consumption in coasting traffic: AR. Please refer to comments for the CO2 case.

1.A.4. Other sectors

  - Revision of fuel consumption of agricultural mobile machinery: AR. Please refer to comments for the CO2 case.

1.B.2. Fugitive emisions in liquid y gaseous fuels

  - Oil refinery catalytic cracking: Re The accounting of N2O emissions produced in the catalytic cracking during the oil refining has been removed from the current edition according to the non-admittance in the corresponding cells of the CRF base table.

2.C. Metal industry

  - Pig iron tapping: Re The accounting of N2O emissions produced in the pig iron tapping has been removed from the current edition according to the non-admittance in the corresponding cells of the CRF base table.

4.B. Manure management

  - Manure management: AR. There has been a very significant reduction in the estimated emissions during the entire period 1990-1999 as a consequence of the following three changes introduced in the current edition from the previous edition.

i)	the revision of nitrogen excretion rates for the diverse animal types, having opted in the current edition for the values that are proposed in the IPCC Reference Manual for the “Middle East and Mediterranean” region, replacing those used in the previous version taken from ECETOC (1994) where the IPCC Reference Manual was also referred to.

ii)	the adoption of national data related to the weighing of the manure management systems in replacement of the default weighing used in the previous edition based on the IPCC Reference Manual for the “Western Europe” region.

iii)	an assignment error in the emission factors of various manure management systems, although the emission factors were and continue to be those of the IPCC Reference Manual.

4.D. Agricultural soils

  - Agricultural soils: AR. There has also been a significant reduction in this category for the emission estimates. Among the reasons are those described in i) and ii) above in section 4.B, “Manure management”, which have a repercussion in the agricultural soils because of the animal excreta during pasture as well as fertilization of agricultural soils with organic fertilizers from animal manure. On the other hand, several changes in the activity rates of: i) nitrogen fixing crops; and ii) residual crops as a consequence of having adopted national data in reference to nitrogen extraction estimates for the varying agricultural crops. Finally, the activity rates of the subcategory “Indirect emissions of agricultural soils: Atmospheric deposition, and Nitrogen leaching and run-off have been revised as a result of the variation registered in the abovementioned activity rates within this same category.

4.F. Field burning of agricultural residues (stubbles)

  - Field burning of agricultural stubbles: EM, AR. As previously mentioned when managing the recalculations of CH4, this category refers to the burning of agricultural stubbles, as the burning of other agricultural residues is considered to be included in category 6.D. In the current edition, the number of vegetable species entering this category has been expanded. In fact, the quantity of burnt nitrogen has been obtained from national data that estimates the nitrogen extraction for the various agricultural cropping contained in the agricultural residue that is burnt on-site. This is to say that it is not based on the N/C ratio utilization proposed by the IPCC Reference Manual section 4.4.3, which would restrict the group of species considered to the ones for which information about the quantity of carbon burnt in the residues was available.

6.A.Solid waste disposal on land

  - Solid waste disposal on land: Re. These emissions correspond to the combustion of the MSW at un-managed waste disposal sites, estimates that had been included directly in the sectoral table 6 of the previous edition. The assignment of N2O emissions originated from the combustion of these residues have been removed in the current edition according to the non-admittance in the corresponding cells of the CRF base table.

6.B.Waste water treatment

  - Waste water treatment: A. The N2O emission estimates described in section 6.4 of the IPCC Reference Manual have been included in the current edition.

6.C. Residue Incineration

  - Incineration of Municipal Solid Waste: Re This activity category has been removed and included in category 1.A.1. (please refer to 1.A.1. category comments)

Hidrofluorocarbides and sulphur hexafluoride

2.E. Production

  - 2.E.2. Fugitive emissions from the production of HFC’s: EF, Revision of the HFC-23 emission factor for one of the HCFC-22 manufacturing plants, as it contributed data on measured emissions.

2.F. Consumption

  - 2.F.6 Others: Electric equipment. EF, Revision of the fugitive emission factor in the electrical equipment installed base. In the previous edition, a factor of 3% was used; in the current edition this has been changed to 2% according to the IPCC Good Practices Guide.

##### 5.- RESULTS AND KEY SOURCES ANALYSIS

For the analysis of the implications on the potential of global atmospheric warming, the emissions figures are estimated with weightings based on the 1995 IPCC global warming factors (100-year time horizon) for each gas and shown in terms of gigagrammes (Gg) of CO2-equivalent. For the case of Spain, the base period taken for reference purposes is 1990 for CO2, CH4 and N2O and 1995 for the fluorinated gases, HFC, PFC and SF6. The time interval under consideration is from 1990 to 2000.

5.1.- AGGREGATE ANALYSIS

Table 5.1 shows in both absolute terms (Gg CO2-equivalent) and in terms of the time index (100 in the reference year) the values corresponding to the total gross emissions. The graphic representation of this index is shown in Figure 5.1. From the preceding data, it can be seen that the total emissions in 2000 are 33,7% above the reference year, whereas the percentage increase reduces to 20,6% when the average of the last quinquennium, 1996-2000, is compared with the same reference year. Overall, the index has shown sustained growth during the inventoried period, except for 1993 and 1996 when the levels fell by comparison with the preceding years. In terms of the slope of the curve, the interval 1990-1996 is characterized by a more moderate growth than that corresponding to the interval 1996-2000. This variability in the evolution seems to be linked from time to time (annual peaks/troughs) with the greater or lesser degree of rainfall in the year in question and its sequel on the thermal electricity generation, although another series of additional factors such as the general expansion in fuel consumption and industrial activity are at the base of change of slope observable between the two intervals indicated above, 1990-1996 and 1996-2000.

TablE 5.1.
EvoluTION OF AGGREGATE EMISSIONS

Absolute Values (Gg CO2 equivalent)

| Base year | 1990 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- |
| 288.647,97 | 286.428,11 | 310.899,11 | 331.167,51 | 341.929,80 | 370.920,07 | 385.987,44 |

Annual Index

| Base year | 1990 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- |
| 100,00 | 99,22 | 107,70 | 114,72 | 118,45 | 128,49 | 133,71 |

Mean five-yearly index

| Base year | 1990-1994 | 1991-1995 | 1992-1996 | 1993-1997 | 1994-1998 | 1995-1999 | 1996-2000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100,00 | 102,20 | 104,40 | 105,60 | 107,56 | 111,32 | 115,91 | 120,62 |

FigurE 5.1.
Index of the Evolution

5.2.- ANALYSIS BY GAS

Table 5.2 gives the emissions estimates by type of gas for each of the six groups or species indicated above as having a direct warming effect: CO2, CH4, N2O, HFC, PFC and SF6. The top of the table shows the emissions in absolute terms (Gg CO2-equivalent) and the bottom part gives the trend in terms of the time index (100 in the reference year).

Table 5.2.
Evolution of emissions by type of gaS

Absolute Values (Gg CO2 equivalent)

| GAS | Base year | 1990 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 | 227.233,25 | 227.233,25 | 242.214,61 | 261.369,23 | 270.129,81 | 295.232,89 | 306.631,85 |
| CH4 | 29.647,72 | 29.647,72 | 34.760,42 | 35.443,12 | 36.551,92 | 37.305,71 | 38.363,49 |
| N2O | 26.259,79 | 26.259,79 | 27.729,65 | 26.941,56 | 27.715,25 | 28.988,46 | 30.497,08 |
| HFC | 4.645,44 | 2.403,18 | 5.334,16 | 6.507,40 | 6.642,63 | 8.513,06 | 9.877,70 |
| PFC | 790,37 | 828,41 | 758,93 | 784,32 | 749,62 | 695,53 | 408,75 |
| SF6 | 93,58 | 55,75 | 101,34 | 121,87 | 140,57 | 184,42 | 208,56 |
| TOTAL GASES | 288.670,16 | 286.428,11 | 310.899,11 | 331.167,51 | 341.929,80 | 370.920,07 | 385.987,44 |

Annual Index

| GAS | Base year | 1990 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 | 100,00 | 100,00 | 106,59 | 115,02 | 118,88 | 129,93 | 134,94 |
| CH4 | 100,00 | 100,00 | 117,24 | 119,55 | 123,29 | 125,83 | 129,40 |
| N2O | 100,00 | 100,00 | 105,60 | 102,60 | 105,54 | 110,39 | 116,14 |
| HFC | 100,00 | 51,73 | 114,83 | 140,08 | 142,99 | 183,26 | 212,63 |
| PFC | 100,00 | 104,81 | 96,02 | 99,23 | 94,84 | 88,00 | 51,72 |
| SF6 | 100,00 | 59,57 | 108,29 | 130,23 | 150,21 | 197,06 | 222,86 |
| TOTAL GASES | 100,00 | 99,22 | 107,70 | 114,72 | 118,45 | 128,49 | 133,71 |

With reference to Figure 5.2, it is pointed out the dioxide of carbon as the main compound in the absolute weight of the emissions (around 80%). The pair formed by the metane and the nitrous oxide, with values varying, depending on the year, between 20.1% and 17.8%, is seen as the second most important block. Finally, there comes the group of fluorinated compounds with a range of contribution between 1.0 and 2.7% in the period 1990-2000. By comparing the three blocks in Figure 5.2, it can be seen that the contribution by CO2 has shown only a very slight variation, 79.0%-79.4%, between that for the five year average 1996-2000 and those corresponding to 2000 and also to the reference year. This same relative stability can be seen for CH4, with a range of variation of less than 1%, as the extreme values in this period are 9.9%-10.5%. N2O reflects a drop in its relative contribution from 9.2% in the reference year to 8.2%-7.9% respectively in the period 1996-2000 and in the year 2000. For gases containing fluor, the reduction in PFC contrasts with the significant relative increases in HFC and SF6, although all of these remain at low (very low) absolute levels even in 2000: HFC (2.6%), PFC (0.1%) and SF6 (0.05%).

fIGURe 5.2.
Contribution to emissions by GAS

Figure 5.3 presents the variations over time, with the top part indicating the indices corresponding to the three gases with the greatest contribution and the lower part those containing fluor. Observing the evolution of CO2 in the upper section, the relative minimum values for 1993 and 1996 can be seen, as can the increase in the slope when going from the sub-interval 1990-1996 to 1996-2000, patterns reflected to a large extent in the trend commented above for the aggregate index. The evolution in CH4 is more uniform throughout the period. N2O continues to follow a different trend from the two gases above, with a slight drop during the sub-interval 1990-1995, then growing in later years but with a variation rate that puts it in 1998 at the same level as in 1996. As for the fluorinsted gases, differences can be seen in the evolution of PFC, stable-decreasing with a significant decrease in 2000, and that of HFC and SF6, for which a first stable period, 1990-1993, is followed by parallel growth in the interval 1994-2000.

fIGURE 5.3.
INDICES of EVOLUtion of EMIsSIONS by GAS

The heterogeneity of the HFC and PFC sub-groups in terms of the individual gases they comprise makes it particularly difficult to give a general comment. However, with respect to future prospects, the figures seem to point to a stabilization in the emissions from production processes and a significant growth in emissions from consumption activities. For the three remaining gases, the increases seen between 1990 and 2000 for CO2 (34.9%) and CH4, (29.4%) are only partly offset by the moderate increase in N2O (16.4%) when it comes to determining the weighted variation rate (33.7%) for the aggregate of the emissions.

5.3.- ANALYSIS BY SECTORS

Table 5.3 gives the emissions estimates by activity sectors, distinguishing between the following groups in the IPCC nomenclature: Energy, industrial processes, use of solvents and other products, agriculture, and waste treatment and disposal. A pro-memoria summary is given for the group including changes in land-use and forestry, as these no do not include gross emissions figures but net sequestration of CO2. The top of the table shows the emissions in absolute terms (Gg CO2-equivalent) and the lower part gives the evolution over time (100 in the reference year) for each group under consideration.

TablE 5.3.
Evolution of emissions by SECTOR

Absolute Values (Gg CO2 equivalent)

| SECTOR | Base year | 1990 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Energy | 216.532,76 | 216.532,76 | 232.217,96 | 250.600,22 | 258.268,49 | 283.102,86 | 294.355,25 |
| 2. Industrial Processes | 25.015,41 | 22.773,36 | 25.403,53 | 27.456,20 | 28.659,49 | 31.394,74 | 32.772,93 |
| 3. Solvent and Other Product Use | 1.342,89 | 1.342,89 | 1.458,15 | 1.538,26 | 1.641,07 | 1.676,34 | 1.709,24 |
| 4. Agriculture | 36.377,73 | 36.377,73 | 39.295,52 | 38.437,28 | 39.745,48 | 40.739,75 | 42.569,19 |
| 6. Waste | 9.401,37 | 9.401,37 | 12.523,94 | 13.135,55 | 13.615,26 | 14.006,38 | 14.580,83 |
| TOTAL SECTORS | 288.670,16 | 286.428,11 | 310.899,11 | 331.167,51 | 341.929,80 | 370.920,07 | 385.987,44 |
| 5. Land Use Change and Forestry | -29.252,46 | -29.252,46 | -29.252,46 | -29.252,46 | -29.252,46 | -29.252,46 | -29.252,46 |

# Annual Index

| SECTOR | Base year | 1990 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Energy | 100,00 | 100,00 | 107,24 | 115,73 | 119,27 | 130,74 | 135,94 |
| 2. Industrial Processes | 100,00 | 91,04 | 101,55 | 109,76 | 114,57 | 125,50 | 131,01 |
| 3. Solvent and Other Product Use | 100,00 | 100,00 | 108,58 | 114,55 | 122,21 | 124,83 | 127,28 |
| 4. Agriculture | 100,00 | 100,00 | 108,02 | 105,66 | 109,26 | 111,99 | 117,02 |
| 6. Waste | 100,00 | 100,00 | 133,21 | 139,72 | 144,82 | 148,98 | 155,09 |
| TOTAL SECTORS | 100,00 | 99,22 | 107,70 | 114,72 | 118,45 | 128,49 | 133,71 |

With reference to Figure 5.4, the first comment to be pointed out when examining the data by sectors is the dominant contribution by the energy group with a weighting that varies between in 1990 about 75% and around 76% in both the mean value for the period 1996-2000 and in 2000. It must be remembered that this IPCC group includes, in addition to the emissions from combustion in stationary and mobile sources, the fugitive emissions from activities dealing with mining, transport and distribution of fuels, which are of great relevance for certain gases such as NMVOC and CH4. Secondly, although lagging far behind the previous group, there is the contribution by agriculture, with values varying around 13% in the reference year and about 11-12% for the period 1996-2000 and in 2000. The third most important group comprises physical and chemical industrial processes (excluding combustion activities that are listed in the energy group), with a contribution that stays around 8.5% without great variation. The group dealing with waste treatment and disposal shows a relative growth, with a contribution rising from 3.3% in the reference year to figures of around 3.9% in the mean and in the last year of the sub-interval 1996-2000. Finally, mention must be made of the marginal and stable contribution, 0.4-0.5%, of the group comprising the use of solvents and other products, which is relatively unrelated to the direct greenhouse gas effect as the essential contribution comes from NMVOC.

fIGURE 5.4.
Contributions to emissions by sector

Figure 5.5 presents the variations over time, with the top part indicating the indices corresponding to the sectors of energy processing and agriculture and the lower part the industrial processes (excluding combustion), waste treatment and disposal, and the use of solvents and other products. With respect to the energy sector, this reproduces to a large extent the patterns commented on above in connection with aggregate emissions (Figure 5.1) and CO2 (Figure 5.3), justified by the close connection between the emissions of CO2 and the energy sector and between both of these and the aggregate emissions. In the agriculture sector, a period of stability can be seen between 1990 and 1995, followed, in the period 1997-2000, by a monotonous upwards trend. The waste treatment and disposal sector is the one showing the most intense and uniform upward tendency over the entire period 1990-2000. Lastly, the sector for the use of solvents and other products, after the stable phase during the years 1990-1992, shows a sustained increase over the period 1993-2000, although its extremely low absolute level makes it irrelevant in terms of the evolution of the aggregate. In short, a distinction can be drawn between the evolution on the one hand of the energy and industrial processes groups, which in 2000 are around 35% above the reference year, then the waste treatment group reaching a level 55% higher than the reference year, and on the other hand the agricultural sector, which is less than these and able to offset slightly their contribution to the increase in the aggregate emissions, with a increase of around 17% in relation to the reference year.

FigurE 5.5.
Indices of the Evolution of emissions by sector

5.4.- KEY SOURCES ANALYSIS

Aiming to improve the precision and accuracy of the emission inventory estimates, it is necessary to consolidate the quality assurance and quality control procedures on the activities which could be considered key or high-priority from the perspective of their contribution to uncertainty of the estimates.

In this sense the IPCC Good Practice Guidance (GPG) document defines an emission source (alternatively a sink) as key if such source could exert a significant influence in the estimates, be it as a level (level assessment) or trend (trend assessment) within the inventory emissions aggregate.

From an operational perspective the GPG establishes two approaches (tiers) to identify the key sources. The first approach, tier 1, is focused on determining the influence, be it by level or trend, a source of emission could exert on the uncertainty of the global inventory estimates but without requiring the use of more formal procedures of uncertainty analysis. The second approach, tier 2, proposes a similar procedure but expects that prior information derived from a formal uncertainty analysis is available. Considering that the National Emission Inventories have not yet implemented a formal uncertainty analysis (although it is planned to produce one for the near future) the following analysis has been developed according to tier 1.

The tier 1 approach distinguishes between the identification of a source as key be it as a level or trend or both concepts. For level identification, a threshold is first defined (usually at 95%) for the cumulative distribution function of the activities contributing to the aggregate inventory emissions; having sorted these activities by descending order of contribution to the inventory emissions (in terms of CO2-e). One can consider that the activities selected, in relation to the cumulative distribution threshold, can take account of approximately 90% of the inventory uncertainty. To identify the trend, tier 1 also specifies a threshold of 95% but defined in this case on the contribution of activities to the trend metrics.

Once the key source analysis approach has been described, the obtained results for the level of the initial (1990) and final (2000) years are presented in tables 5.4 and 5.5, and for the trend of the year 2000 in table 5.6. A results summary for the corresponding period is presented in table 5.7 which, in addition to the classification by level and trend, includes in certain cases a qualitative judgement of those source activities that deserve attention due to other (qualitative) motives. The latter, influencing these significant activities, are (among others): i) actions to control and or reduce emissions; ii) important changes experienced during the considered period; iii) display evidence of great uncertainties; iv) show unexpectedly high or low emissions; and v) subject to important methodology changes in the emission estimates.

| table 5.4. LEVEL CONTRIBUTION BY ACTIVITIES – YEAR 1990 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IPCC SOURCE CATEGORIES |  | Gas | Gg CO2-equivalent | Contribution (A) | Cumulative (B) |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Liquids Fuels | CO2 | 57779,09 | 0,202 | 0,202 |  |
| 1A2f | CO2 Emissions from Stationary Combustion- Other Manufacturing Industries and Construction | CO2 | 26589,30 | 0,093 | 0,295 |  |
| 1A3b | Mobile Combustion : Road Transportation- Gasoline | CO2 | 25925,54 | 0,091 | 0,385 |  |
| 1A3b | Mobile Combustion : Road Transportation- Diesel Oil | CO2 | 25387,28 | 0,089 | 0,474 |  |
| 4D | N2O Emissions from Agricultural Soils | N2O | 16023,03 | 0,056 | 0,530 |  |
| 1A4b | CO2 Emissions from Stationary Combustion- Residential Sector | CO2 | 12981,70 | 0,045 | 0,575 |  |
| 2A1 | CO2 Emissions from Cement Production Processes | CO2 | 12534,33 | 0,044 | 0,619 |  |
| 4A | CH4 Emissions from Enteric Fermentation | CH4 | 12489,94 | 0,044 | 0,662 |  |
| 1A1b | CO2 Emissions from Stationary Combustion- Petroleum Refining | CO2 | 10203,54 | 0,036 | 0,698 |  |
| 1A4c | CO2 Emissions from Stationary Combustion- Agriculture / Forestry / Fisheries | CO2 | 9287,06 | 0,032 | 0,730 |  |
| 1A2a | CO2 Emissions from Stationary Combustion- Iron and Steel | CO2 | 7478,62 | 0,026 | 0,756 |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Solids Fuels | CO2 | 6015,03 | 0,021 | 0,777 |  |
| 4B | CH4 Emissions from Manure Management | CH4 | 5827,33 | 0,020 | 0,798 |  |
| 6A | CH4 Emissions from Solid Waste Disposal on Land | CH4 | 5390,92 | 0,019 | 0,817 |  |
| 1A3a | Mobile Combustion : Civil Aviation | CO2 | 4371,64 | 0,015 | 0,832 |  |
| 1A4a | CO2 Emissions from Stationary Combustion- Commercial / Institutional Sector | CO2 | 3684,01 | 0,013 | 0,845 |  |
| 1A2e | CO2 Emissions from Stationary Combustion- Food Processing, Beverages and Tobacco | CO2 | 3528,41 | 0,012 | 0,857 |  |
| 1A2c | CO2 Emissions from Stationary Combustion- Chemicals | CO2 | 3368,84 | 0,012 | 0,869 |  |
| 2B2 | Non-CO2 Emissions from Nitric Acid Production | N2O | 2884,16 | 0,010 | 0,879 |  |
| 1B1b | Fugitive Emissions from Solid Fuel Transformation | CO2 | 2521,73 | 0,009 | 0,888 |  |
| 2E1 | HFC-23 Emissions from Production of HCFC-22 | HFC-23 | 2403,18 | 0,008 | 0,896 |  |
| 1A2d | CO2 Emissions from Stationary Combustion- Pulp, Paper and Print | CO2 | 2139,95 | 0,007 | 0,904 |  |
| 1A1c | CO2 Emissions from Stationary Combustion- Manufacture of solid Fuls and Other Energy Industries | CO2 | 2095,01 | 0,007 | 0,911 |  |
| 1B1a | Fugitive Emissions from Coal Mining | CH4 | 1765,67 | 0,006 | 0,917 |  |
| 1A3d | Mobile Combustion : Navegation | CO2 | 1536,50 | 0,005 | 0,922 |  |
| 4B | N2O Emissions from Manure Management | N2O | 1444,20 | 0,005 | 0,927 |  |
| 1A2b | CO2 Emissions from Stationary Combustion- Non-Ferrous Metals | CO2 | 1424,41 | 0,005 | 0,932 |  |
| 1B2a | Fugitive Emissions from Oil Refining | CO2 | 1369,63 | 0,005 | 0,937 |  |
| 2A2 | CO2 Emissions from Lime Production | CO2 | 1065,50 | 0,004 | 0,941 |  |
| 6B2 | CO2 Emissions from Domestic and Commercial Wastewater Handling | CH4 | 1023,45 | 0,004 | 0,945 |  |
| 6B1 | CH4 Emissions from Industrial Wastewater Handling | CH4 | 1002,27 | 0,003 | 0,948 |  |
| 3 | CO2 Emissions from Solvent and Other Product Use | CO2 | 977,09 | 0,003 | 0,951 |  |
| SUBTOTAL |  |  | 272.518,37 |  |  |  |
| TOTAL EMISSIONS |  |  | 286.428,11 |  |  |  |
| PERCENT OF TOTAL |  |  | 95,14 |  |  |  |

(A). Percentage of the activity categories emissions with relation to total inventory emissions.

(B). Cumulative percentage of the activity categories emissions with relation to total inventory emissions.

| tablE 5.5. LEVEL CONTRIBUTION BY ACTIVITIES – YEAR 2000 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IPCC SOURCE CATEGORIES |  | Gas | Gg CO2-equivalent | Contribution (A) |  | Cumulative (B) |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Liquids Fuels | CO2 | 75441,10 | 0,195 | 0,195 |  |  |
| 1A3b | Mobile Combustion : Road Transportation- Diesel Oil | CO2 | 49944,19 | 0,129 | 0,325 |  |  |
| 1A2f | CO2 Emissions from Stationary Combustion- Other Manufacturing Industries and Construction | CO2 | 37180,87 | 0,096 | 0,421 |  |  |
| 1A3b | Mobile Combustion : Road Transportation- Gasoline | CO2 | 27150,74 | 0,070 | 0,492 |  |  |
| 4D | N2O Emissions from Agricultural Soils | N2O | 18570,12 | 0,048 | 0,540 |  |  |
| 1A4b | CO2 Emissions from Stationary Combustion- Residential Sector | CO2 | 16993,85 | 0,044 | 0,584 |  |  |
| 2A1 | CO2 Emissions from Cement Production Processes | CO2 | 15033,87 | 0,039 | 0,623 |  |  |
| 4A | CH4 Emissions from Enteric Fermentation | CH4 | 14069,53 | 0,036 | 0,659 |  |  |
| 1A1b | CO2 Emissions from Stationary Combustion- Petroleum Refining | CO2 | 11998,06 | 0,031 | 0,690 |  |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Solids Fuels | CO2 | 11165,90 | 0,029 | 0,719 |  |  |
| 1A4c | CO2 Emissions from Stationary Combustion- Agriculture / Forestry / Fisheries | CO2 | 10739,78 | 0,028 | 0,747 |  |  |
| 6A | CH4 Emissions from Solid Waste Disposal on Land | CH4 | 10098,85 | 0,026 | 0,773 |  |  |
| 4B | CH4 Emissions from Manure Management | CH4 | 7843,20 | 0,020 | 0,793 |  |  |
| 1A2a | CO2 Emissions from Stationary Combustion- Iron and Steel | CO2 | 6823,52 | 0,018 | 0,811 |  |  |
| 1A4a | CO2 Emissions from Stationary Combustion- Commercial / Institutional Sector | CO2 | 6702,81 | 0,017 | 0,828 |  |  |
| 2E1 | HFC-23 Emissions from Production of HCFC-22 | HFC-23 | 6322,68 | 0,016 | 0,845 |  |  |
| 1A2c | CO2 Emissions from Stationary Combustion- Chemicals | CO2 | 6099,77 | 0,016 | 0,861 |  |  |
| 1A3a | Mobile Combustion : Civil Aviation | CO2 | 5496,88 | 0,014 | 0,875 |  |  |
| 2F | HFC Emissions from Consumption of Halocarbons and SF6 | HFC | 3483,19 | 0,009 | 0,884 |  |  |
| 1A2e | CO2 Emissions from Stationary Combustion- Food Processing, Beverages and Tobacco | CO2 | 3384,68 | 0,009 | 0,893 |  |  |
| 1A2d | CO2 Emissions from Stationary Combustion- Pulp, Paper and Print | CO2 | 2762,70 | 0,007 | 0,900 |  |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Gaseous Fuels | CO2 | 2735,12 | 0,007 | 0,907 |  |  |
| 2B2 | Non-CO2 Emissions from Nitric Acid Production | N2O | 2306,75 | 0,006 | 0,913 |  |  |
| 1A3 | CO2 Emissions from Stationary Combustion- Transport | N2O | 2079,79 | 0,005 | 0,918 |  |  |
| 1B1b | Fugitive Emissions from Solid Fuel Transformation | CO2 | 2004,99 | 0,005 | 0,923 |  |  |
| 1A2b | CO2 Emissions from Stationary Combustion- Non-Ferrous Metals | CO2 | 1951,03 | 0,005 | 0,928 |  |  |
| 1A3d | Mobile Combustion : Navegation | CO2 | 1850,33 | 0,005 | 0,933 |  |  |
| 1B2a | Fugitive Emissions from Oil Refining | CO2 | 1756,26 | 0,005 | 0,938 |  |  |
| 1A1c | CO2 Emissions from Stationary Combustion- Manufacture of solid Fuls and Other Energy Industries | CO2 | 1732,11 | 0,004 | 0,942 |  |  |
| 6B2 | CO2 Emissions from Domestic and Commercial Wastewater Handling | CH4 | 1669,06 | 0,004 | 0,947 |  |  |
| 2A2 | CO2 Emissions from Lime Production | CO2 | 1509,19 | 0,004 | 0,951 |  |  |
| SUBTOTAL |  |  | 366.900,90 |  |  |  |  |
| TOTAL EMISSIONS |  |  | 385.987,44 |  |  |  |  |
| PERCENT OF TOTAL |  |  | 95,06 |  |  |  |  |

(A). Percentage of the activity categories emissions with relation to total inventory emissions.

(B). Cumulative percentage of the activity categories emissions with relation to total inventory emissions.

| tabla 5.6. trend CONTRIBUtion by activities – year 2000 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPCC SOURCE CATEGORIES |  | Gas | Gg CO2-e 1990 | Gg CO2-e 2000) | Trend (A) | Contribution (B) | Cumulated (C) |  |
| 1A3b | Mobile Combustion : Road Transportation- Diesel Oil | CO2 | 25.387,28 | 49.944,19 | 0,030 | 0,21119 | 0,211 |  |
| 1A3b | Mobile Combustion : Road Transportation- Gasoline | CO2 | 25.925,54 | 27.150,74 | 0,015 | 0,10452 | 0,316 |  |
| 2F | HFC Emissions from Consumption of Halocarbons and SF6 | HFC | 0,00 | 3.483,19 | 0,007 | 0,04676 | 0,362 |  |
| 1A2a | CO2 Emissions from Stationary Combustion- Iron and Steel | CO2 | 7.478,62 | 6.823,52 | 0,006 | 0,04369 | 0,406 |  |
| 2E1 | HFC-23 Emissions from Production of HCFC-22 | HFC-23 | 2.403,18 | 6.322,68 | 0,006 | 0,04140 | 0,448 |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Liquids Fuels | CO2 | 6.015,03 | 11.165,90 | 0,006 | 0,04108 | 0,489 |  |
| 4D | N2O Emissions from Agricultural Soils | N2O | 16.023,03 | 18.570,12 | 0,006 | 0,04057 | 0,529 |  |
| 6A | CH4 Emissions from Solid Waste Disposal on Land | CH4 | 5.390,92 | 10.098,85 | 0,005 | 0,03804 | 0,567 |  |
| 4A | CH4 Emissions from Enteric Fermentation | CH4 | 12.489,94 | 14.069,53 | 0,005 | 0,03707 | 0,604 |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Solids Fuels | CO2 | 57.779,09 | 75.441,10 | 0,005 | 0,03250 | 0,637 |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Gaseous Fuels | CO2 | 427,14 | 2.735,12 | 0,004 | 0,02899 | 0,666 |  |
| 2A1 | CO2 Emissions from Cement Production Processes | CO2 | 12.534,33 | 15.033,87 | 0,004 | 0,02493 | 0,691 |  |
| 1A4c | CO2 Emissions from Stationary Combustion- Agriculture / Forestry / Fisheries | CO2 | 9.287,06 | 10.739,78 | 0,003 | 0,02383 | 0,715 |  |
| 1A1b | CO2 Emissions from Stationary Combustion- Petroleum Refining | CO2 | 10.203,54 | 11.998,06 | 0,003 | 0,02352 | 0,738 |  |
| 1A4a | CO2 Emissions from Stationary Combustion- Commercial / Institutional Sector | CO2 | 3.684,01 | 6.702,81 | 0,003 | 0,02333 | 0,761 |  |
| 2B2 | Non-CO2 Emissions from Nitric Acid Production | N2O | 2.884,16 | 2.306,75 | 0,003 | 0,02121 | 0,783 |  |
| 1A2c | CO2 Emissions from Stationary Combustion- Chemicals | CO2 | 3.368,84 | 6.099,77 | 0,003 | 0,02094 | 0,804 |  |
| 1B1b | Fugitive Emissions from Solid Fuel Transformation | CO2 | 2.521,73 | 2.004,99 | 0,003 | 0,01870 | 0,822 |  |
| 1A2e | CO2 Emissions from Stationary Combustion- Food Processing, Beverages and Tobacco | CO2 | 3.528,41 | 3.384,68 | 0,003 | 0,01839 | 0,841 |  |
| 1A2f | CO2 Emissions from Stationary Combustion- Other Manufacturing Industries and Construction | CO2 | 26.589,30 | 37.180,87 | 0,003 | 0,01811 | 0,859 |  |
| 1B1a | Fugitive Emissions from Coal Mining | CH4 | 1.765,67 | 1.188,78 | 0,002 | 0,01598 | 0,875 |  |
| 1A1c | CO2 Emissions from Stationary Combustion- Manufacture of solid Fuls and Other Energy Industries | CO2 | 2.095,01 | 1.732,11 | 0,002 | 0,01465 | 0,889 |  |
| 1A3 | CO2 Emissions from Stationary Combustion- Transport | N2O | 850,72 | 2.079,79 | 0,002 | 0,01253 | 0,902 |  |
| 2C3 | PFC Emissions from Aluminium Production | PFC | 828,41 | 408,75 | 0,001 | 0,00950 | 0,911 |  |
| 6C | CO2 Emissions from Waste Incineration | CO2 | 532,43 | 167,80 | 0,001 | 0,00738 | 0,919 |  |
| 1A1 | CO2 Emissions from Stationary Combustion- Energy Industries- Other Fuels | CO2 | 196,78 | 788,20 | 0,001 | 0,00702 | 0,926 |  |
| 1A4b | CO2 Emissions from Stationary Combustion- Residential Sector | CO2 | 12.981,70 | 16.993,85 | 0,001 | 0,00671 | 0,933 |  |
| 4B | N2O Emissions from Manure Management | N2O | 1.444,20 | 1.450,30 | 0,001 | 0,00666 | 0,939 |  |
| 1A4 | Non-CO2 Emissions from Stationary Combustion- Other Sectors | CH4 | 837,62 | 694,93 | 0,001 | 0,00582 | 0,945 |  |
| 1A3a | Mobile Combustion : Civil Aviation | CO2 | 4.371,64 | 5.496,88 | 0,001 | 0,00529 | 0,950 |  |
| SUBTOTAL |  |  | 259.825,34 | 352.257,90 | 0,136 |  |  |  |
| TOTAL EMISSIONS |  |  | 286.428,11 | 385.987,44 | 0,143 |  |  |  |
| PERCENT OF TOTAL |  |  | 90,71 | 91,26 | 95,03 |  |  |  |

(A). See the formula (2) from the previous footnote 2 previous.

(B). Percentage of the activity categories emissions with relation to total inventory emissions.

(C). Cumulative percentage of the activity categories emissions with relation to total inventory emissions.

table 5.7.
sUMMARY OF THE ACTIVITIES CONTRIBUTION TO THE inventOrY

| IPCC SOURCE CATEGORIES |  | GAS | (1) | (2) | (3) | (4) | COMMENTS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENERGY |  |  |  |  |  |  |  |  |  |
| 1A1 | CO2 Emissions from Stationary Combustion- Energy Industries- Other Fuels | CO2 | YES |  | YES |  | Trend from 1996-2000 |  |  |
| 1A1 | Non-CO2 Emissions from Stationary Combustion- Energy Industries | CH4 |  |  |  |  |  |  |  |
| 1A1 | Non-CO2 Emissions from Stationary Combustion- Energy Industries | N2O | YES | YES | YES |  | Trend in 1992 | Level in 1992 |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Gaseous Fuels | CO2 | YES | YES | YES |  | Trend from 1991,1993-1995,1997-2000 | Level from 1997-2000 |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Solids Fuels | CO2 | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 1A1a | CO2 Emissions from Stationary Combustion- Public Electricity and Heat Production -Liquids Fuels | CO2 | YES | YES | YES |  | Trend from 1991-1992,1994-2000 | Level for all years |  |
| 1A1b | CO2 Emissions from Stationary Combustion- Petroleum Refining | CO2 | YES | YES | YES |  | Trend from 1991-1996,1998-2000 | Level for all years |  |
| 1A1c | CO2 Emissions from Stationary Combustion- Manufacture of solid Fuls and Other Energy Industries | CO2 | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 1A2 | Non-CO2 Emissions from Stationary Combustion- Manufacturing Industries and Construction | CH4 |  |  |  |  |  |  |  |
| 1A2a | CO2 Emissions from Stationary Combustion- Iron and Steel | CO2 | YES | YES | YES |  | Trend from 1992-1993,1995-2000 | Level for all years |  |
| 1A2a | Non-CO2 Emissions from Stationary Combustion- Iron and Steel | N2O |  |  |  |  |  |  |  |
| 1A2b | CO2 Emissions from Stationary Combustion- Non-Ferrous Metals | CO2 | YES | YES | YES |  | Trend in 1992 | Level for all years |  |
| 1A2b | Non-CO2 Emissions from Stationary Combustion- Non-Ferrous Metals | N2O |  |  |  |  |  |  |  |
| 1A2c | CO2 Emissions from Stationary Combustion- Chemicals | CO2 | YES | YES | YES |  | Trend from 1992-2000 | Level for all years |  |
| 1A2c | Non-CO2 Emissions from Stationary Combustion- Chemicals | N2O | YES |  | YES |  | Trend in 1995 |  |  |
| 1A2d | CO2 Emissions from Stationary Combustion- Pulp, Paper and Print | CO2 | YES | YES | YES |  | Trend from 1991-1995,1997-1998 | Level for all years |  |
| 1A2d | Non-CO2 Emissions from Stationary Combustion- Pulp, Paper and Print | N2O |  |  |  |  |  |  |  |
| 1A2e | CO2 Emissions from Stationary Combustion- Food Processing, Beverages and Tobacco | CO2 | YES | YES | YES |  | Trend from 1992-1995,1998-2000 | Level for all years |  |
| 1A2e | Non-CO2 Emissions from Stationary Combustion- Food Processing, Beverages and Tobacco | N2O |  |  |  |  |  |  |  |
| 1A2f | CO2 Emissions from Stationary Combustion- Other Manufacturing Industries and Construction | CO2 | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 1A2f | Non-CO2 Emissions from Stationary Combustion- Other Manufacturing Industries and Construction | N2O | YES | YES |  |  |  | Level in 1997,1998 |  |
| 1A3 | Non-CO2 Emissions from Stationary Combustion- Transport | CH4 |  |  |  |  |  |  |  |
| 1A3 | Non-CO2 Emissions from Stationary Combustion- Transport | N2O | YES | YES | YES |  | Trend from 1993-2000 | Level from 1994,1996-2000 |  |
| 1A3a | Mobile Combustion : Civil Aviation | CO2 | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 1A3b | Mobile Combustion : Road Transportation- Gasoline | CO2 | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 1A3b | Mobile Combustion : Road Transportation- Diesel Oil | CO2 | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 1A3d | Mobile Combustion : Navegation | CO2 | YES | YES | YES |  | Trend from 1991-1994,1997-1998 | Level for all years |  |
| 1A3c+1A3e+1A5b | Mobile Combustion : Other Transportation | CO2 |  |  |  |  |  |  |  |
| 1A4 | Non-CO2 Emissions from Stationary Combustion- Other Sectors | CH4 | YES |  | YES |  | Trend from 1995,1997-2000 |  |  |
| 1A4 | Non-CO2 Emissions from Stationary Combustion- Other Sectors | N2O | YES |  | YES |  | Trend in 1991 |  |  |
| 1A4a | CO2 Emissions from Stationary Combustion- Commercial / Institutional Sector | CO2 | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 1A4b | CO2 Emissions from Stationary Combustion- Residential Sector | CO2 | YES | YES | YES |  | Trend from 1991-1996,1999-2000 | Level for all years |  |
| 1A4c | CO2 Emissions from Stationary Combustion- Agriculture / Forestry / Fisheries | CO2 | YES | YES | YES |  | Trend from 1993,1995,1997-2000 | Level for all years |  |
| 1B1a | Fugitive Emissions from Coal Mining | CH4 | YES | YES | YES |  | Trend for all years | Level from 1990-1997 |  |
| 1B1b | Fugitive Emissions from Solid Fuel Transformation | CH4 |  |  |  |  |  |  |  |
| 1B1b | Fugitive Emissions from Solid Fuel Transformation | CO2 | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 1B2 | Fugitive Emissions from Oil and Natural Gas | CH4 | YES |  | YES |  | Trend from 1991-1994 |  |  |
| 1B2a | Fugitive Emissions from Oil Refining | CO2 | YES | YES | YES |  | Trend from 1991-1996 | Level for all years |  |
| 1B2c | Fugitive Emissions from Oil and Natural Gas: Flaring | CO2 |  |  |  |  |  |  |  |

(1). Key Source Category.

(2). Level Criteria Trend.

(3).Trend Criteria.

(4). Qualitative Criteria.

table 5.7.
sUMMARY OF THE ACTIVITIES CONTRIBUTION TO THE inventOrY

| IPCC SOURCE CATEGORIES |  | GAS | (1) | (2) | (3) | (4) | COMMENTS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INDUSTRIAL PROCESSES |  |  |  |  |  |  |  |  |  |
| 2A1 | CO2 Emissions from Cement Production Processes | CO2 | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 2A2 | CO2 Emissions from Lime Production | CO2 | YES | YES | YES |  | Trend from 1994-1995 | Level for all years |  |
| 2A3+2A4+2A5+2A6+2A7 | CO2 Emissions from Other Mineral Production | CO2 |  |  |  |  |  |  |  |
| 2B | CH4 Emissions from Chemical Industry | CH4 |  |  |  |  |  |  |  |
| 2B | CO2 Emissions from Chemical Industry | CO2 | YES |  | YES |  | Trend for 1993,1999 |  |  |
| 2B2 | Non-CO2 Emissions from Nitric Acid Production | N2O | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 2C | CH4 Emissions from Metal Production | CH4 |  |  |  |  |  |  |  |
| 2C1 | CO2 Emissions from Iron and Steel Production | CO2 | YES |  | YES |  | Trend in 1992 |  |  |
| 2C2+2C3+2C4+2C5 | CO2 Emissions from Production of Other Metals | CO2 | YES |  | YES |  | Trend from 1991-1992 |  |  |
| 2C3 | PFC Emissions from Aluminium Production | PFC | YES |  | YES |  | Trend from 1991-1992,1999-2000 |  |  |
| 2E1 | HFC-23 Emissions from Production of HCFC-22 | HFC-23 | YES | YES | YES |  | Trend for all years | Level for all years |  |
| 2F | HFC Emissions from Consumption of Halocarbons and SF6 | HFC | YES | YES | YES | YES | Trend from 1991-1992,1996-2000 | Level from 1998-2000 | Expected growth |
| 2F7 | SF6 Emissions from Electrical Equipment | SF6 |  |  |  |  |  |  |  |
| SOLVENT AND OTHER PRODUCT USE |  |  |  |  |  |  |  |  |  |
| 3 | CO2 Emissions from Solvent and Other Product Use | CO2 | YES | YES | YES |  | Trend in 1994 | Level from 1990-1991 |  |
| 3 | N2O Emissions from Solvent and Other Product Use | N2O |  |  |  |  |  |  |  |
| AGRICULTURE |  |  |  |  |  |  |  |  |  |
| 4A | CH4 Emissions from Enteric Fermentation | CH4 | YES | YES | YES |  | Trend from 1991, 1992,1994-1995,1997-2000 | Level for all years |  |
| 4B | CH4 Emissions from Manure Management | CH4 | YES | YES | YES |  | Trend from 1991,1993-1994,1996-1998 | Level for all years |  |
| 4B | N2O Emissions from Manure Management | N2O | YES | YES | YES |  | Trend from 1992, 1995,1997,1999-2000 | Level from 1990-1999 |  |
| 4C | CH4 Emissions from Rice Cultivation | CH4 |  |  |  |  | Trend in 1994 |  |  |
| 4D | N2O Emissions from Agricultural Soils | N2O | YES | YES | YES | YES | Trend for all years | Level for all years | Check N excretion |
| 4F | CH4 Emissions from Field Burning of Agricultural Residues | CH4 |  |  |  |  |  |  |  |
| 4F | N2O Emissions from Field Burning of Agricultural Residues | N2O |  |  |  |  |  |  |  |
| WASTE |  |  |  |  |  |  |  |  |  |
| 6A | CO2 Emissions from Solid Waste Disposal on Land | CO2 | YES |  | YES |  | Trend from 1992,1995-1997,1999 |  |  |
| 6A | CH4 Emissions from Solid Waste Disposal on Land | CH4 | YES | YES | YES | YES | Trend for all years | Level for all years | Expected control |
| 6B1 | N2O Emissions from Industrial Wastewater Handling | N2O | YES | YES |  |  |  | Level from 1990-1991,1993,1996 |  |
| 6B1 | CH4 Emissions from Industrial Wastewater Handling | CH4 |  |  |  |  |  |  |  |
| 6B2 | CH4 Emissions from Domestic and Commercial Wastewater Handling | CH4 | YES | YES | YES |  | Trend from 1993-1999 | Level for all years |  |
| 6C | CO2 Emissions from Waste Incineration | CO2 | YES |  | YES |  | Trend from 1995-2000 |  |  |
| 6C | CH4 Emissions from Waste Incineration | CH4 |  |  |  |  |  |  |  |
| 6C | N2O Emissions from Waste Incineration | N2O |  |  |  |  |  |  |  |

##### 6.- RESULTS COMPARISON (1990-1999)
ed. 2002 vs. ed. 2001

In this section, the comparative analysis of the results obtained for the period 1990-1999 in the current edition of the inventories, edition 2002, compared to the corresponding results for the same period in previous edition, 2001, are presented. Please refer to section 4 above for the motivating reasons of the recalculations.

The presentation is based on a series of graphs, 6.1 to 6.9, where each one shows the time profiles, as absolute value emissions (upper panel) or as indexed numbers (lower panel) of both editions. The absolute emission data are expressed homogeneously in CO2-equivalent units; and the indices take the value 100 for the base year.

Figure 6.1 shows the evolution of the emissions inventory aggregate. It is apparent that the estimate in the edition 2002 has meant an average reduction of around twenty thousand kilo tonnes in terms of CO2-e, while during the years 1998 and 1999 there is a more pronounced growth rate in the new series. These facts are reflected in the lower panel in similar values up to the year 1996 of the corresponding indexed numbers in both series, while the new series profile is steeper from there on. The acute drop of N2O agricultural emissions in first place and, the reduction of CH4 emissions in the waste treatment, coal mining and distribution of natural gas in second place (see figures 6.5, 6.6, 6.8 and 6.9 further on) are among the most determining factors of the drop in the level of the series in the current edition. On the other hand, the clear increase in the new series for the year 1999, in relation to 1998, is mainly caused by the revision of the fuel consumption for electricity production after having detected an important omission in the primary information data of the previous edition. Please also refer to figures 6.2 and 6.7 further on.

Figure 6.2 shows the evolution of energy sector. The two most relevant observations are:

-	The great coincidence between both series up to the year 1998, and

-	The pronounced increase in year 1999, caused by the revision of that years’ estimate in the new series. A big part of this difference is caused by the correction of an error in the fuel consumption data of a utility power plant.

Figure 6.3 shows the evolution of the industrial processes emissions not related to the energy consumption, with a similar behaviour in the inventory estimates of both editions.

Figure 6.4 shows the emissions evolution from the use of solvents and other products. The clear parallelism of the series in both estimates can be noted in the upper panel. The descent reflected in the current estimate compared to the previous comes from the fact that, following the IPCC methodology, the volatile non-methane organic compounds (NMVOC) from the IPCC activity category 3.C., “Manufacture and chemical products processing” have not been converted to CO2-e, as had been performed in the previous edition. In terms of indexed numbers the evolution is very much the same.

Figure 6.5 shows the evolution in the agriculture sector emissions. The two most relevant observations are:

-	The great drop which the new series experiments with regard to the previous edition, although as years pass the differences become lesser between them. This reduction in the new series is related, almost in its entirety, to the decrease of the N2O emission in the management of livestock manure. This decrease in the estimated N2O emissions is motivated by three factors: i) the revision nitrogen excretion rates of the animals; ii) the revision of the relative participation of diverse manure management systems; iii) and, very important, the correction of an error in the assignment of emission factors detected in the previous edition.

-	The relative drop which, in the previous edition, represented in the year 1998 and identified as a CH4 agriculture estimation error has been corrected in the current edition. Please also refer to the comment on this subject further down in figure 6.7.

Figure 6.6 shows the emissions evolution of the treatment and disposal of wastes. The most notable fact is the reduction experienced in the new estimates compared to the old ones, becomes even more acute as years pass. There are two fundamental reasons to support this difference:

-	First, the methodology revision according to the IPCC Good Practice Guidance document of methane emissions at waste disposal sites. This revision has implied a change in the parameters of the kinetic first order decay methodology of the degradable matter in MSW (and its subsequent generation of CH4).

-	Second, the reallocation to IPCC category 1.A.1, from the CO2 emissions originated by the MSW incineration, where an energy re-utilization is made; while in the previous edition these emissions were calculated as part of the waste treatment.

The results comparison between the current edition of the inventories and the previous are completed with the following 3 figures (6.7 to 6.9); the evolution in both series reflect the estimates of the three primary gases in the inventory: CO2, CH4 and N2O.

Hence, the evolution in the group of activities that produce CO2 emissions is shown in figure 6.7. The profile is very similar to the one in figure 6.2, which corroborates the close relationship between CO2 emissions and the energy sector.

The evolution in the group of activities that produce CH4 emissions is presented in figure 6.8. One can appreciate the relatively lower level of the current edition estimates. This is basically due to three primary factors:

-	The abovementioned methodology revision on waste disposal emissions.

-	The revision of the emission factors derived from coal mining.

-	The revision of the emission factors related to natural gas distribution activity.

On the other hand, the drop corresponding to the year 1998 seen as a reflection of the previously commented reduction for that year in figure 6.5 is especially clear in the lower panel, considering this is the agricultural sector.

Figure 6.9 shows the evolution in the N2O emissions for the group of activities that produce this gas. One can appreciate the markedly low level of evolution of this new series compared to the previous one. This reduction is directly related to the evolution of the agricultural sector emissions, where there is a significant reduction of gas emissions in the manure management.

Figure 6.1.
total emissions evolution

Figure 6.2.
energy evolution

Figure 6.3.
industrial processes evolution

Figure 6.4.
solvent and other product use evolution

Figure 6.5.
agriculture evolution

Figure 6.6.
waste evolution

Figure 6.7.
co2 Evolution

Figure 6.8.
cH4 Evolution

Figure 6.9.
n2o Evolution

The comparison of the five-year average 1996-2000 with the reference year is similar to what must be done for the quinquennium 2008-2012 with 2010 as the representative value for comparison with the reference year.

The respective metrics for the level and trend are calculated by the following equations:

(1)	Lx,t=Ex,tEt

(2)	Tx,t=Lx,t∗(Ex,t−Ex,0)Ex,t−(Et−E0)Et

where:

Lx,t is the level assessment for source x in year t

Tx,t is the trend assessment for source x in year t

Ex,t y Ex,0  are the emission estimates of source category x in years t and 0, respectively

0  is the base year (i.e. 1990)