Sweden’s National Inventory Report 2002 

Submitted under the United Nations Convention on Climate Change

# Data sheet

Title:	Sweden’s National Inventory Report 2002 – submitted under the United Nations Convention on Climate Change

Date:	April 2002

Authors:	Susanna Fink, Gabriella Hammarskjöld and Håkan Staaf, Swedish Environmental Protection Agency

Maria Lidén, José Luis Berrospi and Rolf Adolfsson, Statistics Sweden

Karin Kindbom, IVL Swedish Environmental Research Institute

Hillevi Eriksson, National Board of Forestry

Internet version:	This Report is also available on www.environ.se

Contact: 	Susanna Fink
Swedish Environmental Protection Agency
SE-106 48 Stockholm, Sweden
e-mail: susanna.fink@environ.se

# Contents

Introduction	3

(a)	The annual inventory information 1990 - 2000	4

(b)	Database information	4

(c)	Methodologies	5

(d)	References regarding methodologies, emission factors and activity data	9

(e)	Assumptions underlying the emission and removal estimates	10

(f)	Feedstocks and bunkers	10

(g)	Recalculations	10

(h)	Uncertainties	10

(i)	Information on quality assurance/quality control (QA/QC)	11

(j)	Changes with respect to the previous years	11

1	Methodology regarding Energy	12

1.1	Recalculation	12

1.2	Fuel combustion – general method	12

1.3	CO2 from Fuel Combustion Activities – Reference Approach	16

1.4	Feedstocks and Non-Energy Use of Fuels	16

1.5	Fugitive Emissions from Solid Fuels, Oil and Natural Gas	16

1.6	International Bunkers and Multilateral Operations	17

1.7	Tables with Thermal Values and used Emission Factors	18

1.8	References	30

2	Methodology regarding Industrial Processes (including F-gases)	31

2.1	Industrial Processes, Emissions of CO2	31

2.2	Industrial Processes, emissions other than CO2	31

2.3	Emissions of HFCs, PFCs and SF6	32

2.4	Specific source categories	34

2.5	References	35

3	Methodology regarding solvent and other product use	36

4	Methodology regarding Agriculture	37

4.1	Introduction	37

4.2	Livestock Groups and Subgroups	38

4.3	Methane from Enteric Fermentation	40

4.4	Methane from Manure Management (including excretion on pasture range)	40

4.5	N2O from manure management	42

4.6	Direct emissions of N2O from agricultural soils	43

4.7	Animal production - N2O from grazing animals	49

4.8	Indirect emission of N2O	49

4.9	Literature references	50

4.10	Overview of emission factors	52

5	Methodology regarding Land-Use Change and Forestry	54

5.1	Forest biomass	54

5.2	Forest soils	57

5.3	Harvested wood products	58

5.4	Cultivation of organic soils	58

5.5	CO2 from liming	60

6	Methodology regarding Waste	61

6.1	The Method	61

6.2	Waste Statistics in Sweden	61

6.3	Waste categories	62

6.4	Methane potentials	64

6.5	Recovered gas	64

6.6	Other parameters	65

6.7	Summary: Used statistics on land filled waste	66

6.8	Literature	67

7	Analysis of Key Source Categories	69

8	Coming improvements	70

8.1	Energy	70

8.2	Transport, emissions of CH4, N2O, NOx, CO, NMVOC and SO2 for the years 1990-1999	70

8.3	Industrial processes	71

8.4	Burning of biomass in the residential sector	71

8.5	Solvent and Other Product Use	71

8.6	Agriculture	71

# Introduction

According to decision 3/CP.5 taken under the United Nations Framework Convention on Climate Change (UNFCCC) by the Conference of the Parties at its fifth session in November 1999, including the adoption of UNFCCC Reporting Guidelines on Annual Inventories contained in the document FCCC/CP/1999/7, each developed country Party to the Convention shall annually submit to the Conference of the Parties, through the secretariat, a national inventory report. The report shall contain detailed and complete information on inventories for all years from the base year to the year of the current annual inventory submission, in order to ensure the transparency of the inventory.

This report is Sweden’s National Inventory Report due by 15 April 2002. The report contains information on Sweden’s inventories for all years from 1990 to 2000.

The inventory accounts for anthropogenic emissions of CO2, CH4, N2O, NOx, CO, NMVOC, SO2, HFCs, PFCs and SF6. Inventories for the years 1990-1999 have been revised/corrected to some extent and are included in the submission.

Emission estimates are mainly based on official Swedish statistics, e.g. energy statistics, agricultural and forestry statistics, environmental reports from industry and emission factors (nationally developed factors as well as internationally recommended ones). Statistics Sweden, IVL Swedish Environmental Research Institute and the Swedish Environmental Protection Agency have carried out the calculations. In a few cases, estimates are based on expert judgements. A more thorough outline of methods used is enclosed in the following chapters.

According to the UNFCCC Reporting Guidelines on Annual Inventories the following issues are addressed in the report:

(a) The annual inventory information 1990-2000

(b) Database information

(c) Methodologies

(d) References regarding methodologies, emission factors and activity data

(e) Assumptions underlying the emission and removal estimates

(f) Feedstocks and bunkers

(g) Recalculations

(h) Uncertainties

(i) Information on quality assurance/quality control (QA/QC)

(j) Changes with respect to the previous years

This report contains the following chapters and appendices:

- Methodology regarding Energy

- Methodology regarding Industrial Processes (including F-gases)

- Methodology regarding Solvent and other Product Use

- Methodology regarding Agriculture

- Methodology regarding Land-Use Change and Forestry

- Methodology regarding Waste

- Analysis of Key Source Categories

- Coming Improvements

Appendix 1: 	Sweden’s National Greenhouse Gas Inventory 2000

Appendix 2: 	Sweden’s National Greenhouse Gas Inventory 1999

Appendix 3: 	Sweden’s National Greenhouse Gas Inventory 1998

Appendix 4: 	Sweden’s National Greenhouse Gas Inventory 1997

Appendix 5: 	Sweden’s National Greenhouse Gas Inventory 1996

Appendix 6: 	Sweden’s National Greenhouse Gas Inventory 1995

Appendix 7: 	Sweden’s National Greenhouse Gas Inventory 1994

Appendix 8: 	Sweden’s National Greenhouse Gas Inventory 1993

Appendix 9: 	Sweden’s National Greenhouse Gas Inventory 1992

Appendix 10: 	Sweden’s National Greenhouse Gas Inventory 1991

Appendix 11: 	Sweden’s National Greenhouse Gas Inventory 1990

Appendix 12:	Comparison of Sectoral and Reference Approach, 1990-2000

Appendix 13:	Key Sources 1990-2000

Appendix 14:	Activity Data for HFCs, PFCs and SF6 1990-2000

## (a)	The annual inventory information 1990 - 2000

Sweden uses the adopted UNFCCC Reporting Guidelines on annual inventories (FCCC/CP/1999/7) including the Common Reporting Format (CRF) for the reporting to the United Nations Framework Convention on Climate Change. Sweden, as all European countries, has to fulfill reporting obligations on air emissions to different international organizations, as well as within the European Union, and in different formats. The Swedish Environmental Protection Agency is responsible for the emission inventories. The different reporting formats have created some problems until now, as there has not been one system to deal with the different reporting formats. All inventories have been carried out on “ad hoc-basis”, i.e. data collection has been conducted once a year but the compiling of inventories to different reporting formats has been done on a manual basis.

The emission inventories for 1990 to 2000 are given in Appendices 1-11 and include the Common Reporting Format (CRF) for each year. The trend tables are only filled in the CRF for year 2000.

## (b)	Database information

As described in the previous section Sweden has not had a single database for storage of inventory data until now. The compiled inventories have been archived at the Swedish EPA together with underlying data in electronic and/or hardcopy form.

This year a consortium constituted of Statistics Sweden (SCB), the Swedish Meteorological and Hydrological Institute (SMHI) and IVL, Swedish Environmental Research Institute, are developing an emission database for air emissions. The consortium are also working with the development of methodologies and are doing the calculations and compiling the data into the different reporting formats. The Swedish Environmental Protection Agency is still involved in the processes as the orderer and as an expert organization. It also does the final quality control of the data before the submission.

This implies that Sweden in the coming years will be able to submit data with adequate time series and quality and in a transparent manner and hopefully will be able to meet all the reporting requirements.

A description of used emission factors and underlying data for different sectors can be found in chapters 1-7.

## (c)	Methodologies

### The general methodology

Sweden’s greenhouse gas emission inventory is in accordance with the Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories.

### Fuel Combustion

#### Stationary combustion

Emissions from stationary combustion in connection to energy production and heating in Sweden are determined as the product of fuel consumption, fuel thermal value and emission factors. Statistics Sweden is doing the calculations. The emission factors are provided by the Swedish Environmental Protection Agency and are based on results from measurements and national studies as well as studies of internationally used emission factors and judgments of relevance to national conditions.

Emission factors for stationary fuel that have been used in the calculations are provided in chapter 1 together with a description of the used methodology.

For the submission on the 2000 inventory and the time series some corrections have been made regarding emissions of NOX from Residual Heating Oil.

#### Mobile sources

Some changes have been made for the calculation of emissions regarding the year 2000 compared to emissions 1990-1999. In the near future emissions from mobile sources for all years will be revised to ensure consistency in the emission trends.

Emissions from military use of aviation fuel were reported together with emissions from civil aviation under 1A3a for the years 1990-1999.For the year 2000 emissions from military use of aviation fuel was separated and reported under 1A5b.

Emissions from electricity production are included in emissions from railways for the years 1990-1999. For the year 2000 emissions from electricity production are not included.

Emissions of N2O from all aviation, emissions of CH4, N2O and NMVOC from railways and emissions of CH4 and NMVOC from marine international bunkers were estimated 2000 but not for the years 1990-1999.

Emissions from road transport are calculated with the aid of a model developed by the Swedish National Road Administration. The model offers several types of calculation possibilities. The emission estimates are based on traffic data, descriptions of different vehicle categories as well as descriptions of different fuel types. There are numerous emission factors available, e.g. for cold starts, hot emissions, evaporation and different fuel types as well as correction and deterioration factors. Engine technology and different modes of driving (rural/urban/highway) are also considered, e.g. have emission factors, developed for the Swedish system for environmental classification of cars from 1992 and onwards, been included in the model.

For the years 1990-1999 emissions from civil aviation, navigation and railways were based on compilations submitted by the respective national authorities to the Swedish EPA (in their annual Environmental Reports). For 2000 the Environmental Reports were used only for civil aviation and railways. To improve the quality of emissions from navigation the emissions in 2000 were calculated on the basis of fuel consumption according to a survey to wholesale dealers by Statistics Sweden.

For 2000, and to some extent 1990-1999, emissions from military mobile sources are calculated on basis on fuel consumption given by the Swedish Armed Forces.

Statistics Sweden on the basis of fuel consumption statistics calculates emissions from international bunkers, for aviation and navigation. For the years 1990-1999 only CO2 were calculated on the basis of fuel consumption, and emission factors for navigation, aviation and railways were not documented. 2000 all emissions from international bunkers are calculated and emission factors are provided in chapter 1.

Emissions from off-road vehicles were calculated by The Swedish EPA 1990-1999 and by Statistics Sweden 2000.

Chapter 1 includes a description of the methodology used.

### Industrial Processes

Data on emissions to air from industrial processes are mainly collected from the annual environmental reports from industries to the regulatory authorities. These data often originate from measurements or surveys and refer to real emissions, known e.g. by measurements or mass balances. The use of emission factors is limited.

Statistics Sweden, on the basis of information about production and use of raw material (e.g. limestone) calculates estimates of CO2-emissions from industrial processes. Estimates for greenhouse gases other than CO2 have been collected mainly from the CORINAIR inventory system.

Actual emissions of halocarbons and sulfur hexafluoride have been estimated with Tier 2 methodology according to the IPCC 1996 Revised Guidelines. Potential emissions of halocarbons and sulfur hexafluoride have been estimated with Tier 1b methodology. A study has been carried out by the Swedish Environmental Research Institute, IVL, for all the years in the time series, 1990-1999 (Kartläggning och beräkning av årliga potentiella och faktiska utsläpp av HFC, FC och SF6 i Sverige, IVL 2000).

In chapter 2 a description of the used methodology can be found.

### Solvent and Other Product Use

The emission estimates originate from data reported to the CORINAIR inventory. Emission estimates are based on results from production statistics, together with national studies as well as expert judgments of relevance to national conditions and for large point sources on data collected from the annual environmental reports from industries to the regulatory authorities. A study will be carried out in the near future to update the methodology used as well as activity data and emission factors.

A description of the used methodology is found in chapter 3.

### Agriculture

Emissions of CH4 from enteric fermentation are calculated with national emission factors, based on a Swedish EPA report from 1992 (”Växthusgasutsläpp från husdjur”, Murphy, NV 4144, Swedish EPA) and emission factors recommended by IPCC. The national emission factors are considered to be more representative for Swedish conditions than IPCC default factors. The EPA report has been found not sufficiently documented for reporting needs. For this reason calculations concerning emissions from cattle are currently under review.

Emissions from manure management are calculated with 1996 Revised IPCC methodology. Both national emission factors and emissions factors recommended by IPCC, where there has been a lack of knowledge, have been used.

Estimates of N2O emissions from agriculture are based on the methods recommended by IPCC in the 1996 Revised Guidelines. Data on the amount of N input have however been available for Sweden and are used in the method. National emission factors have been developed to better reflect Swedish conditions.

Chapter 4 includes information on the used methodology as well as used emissions factors.

### Changes in Forest and Other Woody Biomass Stocks

Estimates of CO2 removals from forestry are calculated with the 1996 Revised IPCC methodology.

Data for ”Forest and Grassland Conversion” and ”Abandonment of Managed Lands” is not reported since these land use changes are considered to be insignificant in Sweden. Data for ”CO2 emissions and removals from soil” is reported for the whole time series. This is possible due to new information on emissions from organic soils and information on liming of soils.

The Swedish data for uptake of carbon dioxide due to changes in forest and other woody biomass stocks submitted in this National Inventory Report differs from data previously submitted by Sweden. The reason for this is changes in the calculation method. The major changes are mainly within the expansion factors. There are also changes in the conversion factors.

Estimates of CO2 emissions from liming are calculated with 1996 Revised IPCC methodology. Emission factors recommended by IPCC are used, and activity data are from Statistics Sweden.

In chapter 5 a description of the used methodology is included.

### Waste

Calculations of methane emissions from landfills are done mainly according to the 1996 Revised IPCC Tier 2 methodology, first order decay. However national gas potentials are used for some waste fractions, namely sludge from wastewater treatment, and organic waste from food processing industries.

Emissions of nitrous oxide and CH4 from wastewater handling are considered to be insignificant since almost all wastewater is treated in aerobic conditions. Emissions from wastewater handling are therefor not reported. (Lustgasutsläpp från kommunala avloppsreningsverk, SNV rapport 4309, Swedish EPA 1994).

Chapter 6 includes information on the used methodology as well as used emission factors.

### Adjustments for climatic conditions

Sweden is developing a method for adjusting CO2-emissions for climatic conditions in Sweden. The climate is cold with long winters when need for heating is big, thus the adjustment to be able to compare the emissions with a “normal” year. In the UNFCCC Reporting Guidelines on Annual Inventories Parties are encouraged to give information on application of adjustments as it is regarded as important information in relation to the monitoring of emission and removal trends and the performance of national policies and measures. Information on CO2-emissions adjusted for climatic conditions in Sweden is included in the Third National Communication on Climate Change due in November 2001.

### Normal-year correction of green-house gas emissions

The Swedish climate varies a great deal from year to year. The variables are temperature, wind conditions, radiation and precipitation. Temperature, radiation and wind influence the amount of energy needed to heat buildings to maintain normal indoor temperatures. Precipitation affects the quantity of water flowing in watercourses and hence the potential for generating electric energy using hydropower. Normal-year corrections allow relevant comparisons between the years. A new normal-year correction model has been used for the Third National Communication. This model differs in terms of calculation method and results. Normal-year correction involves estimating emissions a normal year from the heating of buildings and from electricity generation. These two models are described in more detail in Appendix 3 in Sweden’s Third National Communication on Climate Change. In figure 1 below a comparison between the normal-year correction data and actual emission data are shown for the years 1990 to 2000. In table 1 the normal-year correction data in total and detailed for hydropower and heating are shown, expressed in 1000 ton CO2/year.

Figure 1. Comparison between the normal-year correction data and the actual emission data for the years 1990 to 2000.

Table 1. Normal-year correction data in total and detailed for hydropower and heating, expressed in 1000 ton CO2/year.

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hydropower | 159 | -40 | 349 | 410 | -239 | 140 | -911 | 182 | 389 | 274 | 496 |
| Heating | 3008 | 1040 | 1631 | 291 | 655 | 393 | -936 | 1134 | 391 | 1647 | 2934 |
| Normal-year correction | 3167 | 1000 | 1980 | 701 | 416 | 533 | -1847 | 1316 | 780 | 1921 | 3430 |

## (d)	References regarding methodologies, emission factors and activity data

As mentioned above some parts of the underlying methodologies are taken directly from the Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories. Regarding the calculation of actual emissions of HFCs, PFCs and SF6 further information can be obtained in the report “ Kartläggning och beräkning av årliga potentiella och faktiska utsläpp av HFC, FC och SF6 i Sverige, IVL 2000”, by IVL Swedish Environmental Research Institute.

Regarding activity data the references are national statistics e.g. on energy and agriculture as well as data on production (e.g. cement) and consumption (e.g. F-gases) obtained directly from producers and consumers.

## (e)	Assumptions underlying the emission and removal estimates

The assumptions underlying the emission and removal estimates are in general related to the emission factors chosen and activity data used. Information on the emission factors chosen and activity data used is given above. Information on the rationale for their selection is one of the issues where the level of detail is planned to be further elaborated in the future.

## (f)	Feedstocks and bunkers

### Feedstocks

The Swedish Energy Statistics, which are used as activity data in the emission inventory calculations, includes feedstocks and Non Energy Use of Fuels.

### Bunkers

In the Swedish emission inventories the distinction between domestic marine and aviation emissions, which are to be included in the national totals, and international bunker emissions, is made in accordance with the Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories. This means that domestic marine and aviation emissions are emissions that are coming from the transportation between two national harbors or airports.

## (g)	Recalculations

Since the last Swedish submission of greenhouse gas emission inventories to the UNFCCC, April 2001 changes to the Swedish emission inventories have been carried out.

Some errors concerning NOX from Residual Fuel Oil 1992-1995 and 1997-1998, NMVOC from off road vehicles 1997 and 1998, CH4 and N2O from industrial processes in the pulp and paper industry 1999 and SO2 from cement industry 1990-1998 have been corrected. Emissions of HFCs, PFCs and SF6 have been updated. Information on used methodologies, activity data and emission factors can be found in chapters 1-7.

## (h)	Uncertainties

The uncertainty on the emissions arises from the uncertainty on the activity data, the uncertainty on the emission factors and the uncertainty arising from whether all (major) sources of emissions are included in the inventory.

It is assumed that the top-down estimates based on the energy statistic are more accurate than the bottom-up estimates based on less well known activity data, e.g. average annual mileage of diesel driven cars, which means that discrepancies are eliminated by updating some of the most uncertain parameters in the activity data to fit with the energy statistics, e.g. consumption of diesel sold for road transportation.

For most sectors Swedish official statistics are used for activity data, except for industrial processes where information comes from the industries yearly “Environmental Reports”, F-gases and for solvent use. Used emission factors originate either from measurements from existing Swedish plants or from comparable European installations, where IPCC default emission factors are not used. It is also assumed that the uncertainty is largest for the inventories of NMVOC, CH4 and N2O, perhaps with an uncertainty factor of 2, while the uncertainty on the CO SO2 and NOx inventories is assumed to be less than 30 – 40% and the uncertainty with the CO2 may be as low as 1 – 2%.

## (i)	Information on quality assurance/quality control (QA/QC)

In the preparation of Sweden’s Annual Emission Inventories some quality control (QC) is performed. A part from the UNFCCC’s In-Depth-Reviews, Quality Assurance (QA) with independent review of the inventories has not yet been carried out. The IPCC has developed guidance on good practice, which also includes good practice guidance on QA/QC. For some parts of the Swedish Emission Inventories the good practice has been implemented, except for QA. Sweden will during the next years improve the Swedish Emission Inventories and will include further elaboration of how formal QA/QC procedures could be implemented upon the full adoption of the IPCC’s Good Practice Guidance.

## (j)	Changes with respect to the previous years

As mentioned under (g) on recalculations some changes have been made with respect to the previous years. These changes can be seen in the CRFs for each individual year, see appendix 1-11.

# Methodology regarding Energy

## Recalculation

The Swedish Greenhouse Gas inventories for the years 1992-1995 and 1997-1998 have been corrected, since the submission due by April 2001, concerning emissions of NOX from Residual Fuel Oil. Corrections have also been made for NMVOC from off road vehicles 1997 and 1998.

## Fuel combustion – general method

Emissions from stationary combustion in connection to energy production and heating in Sweden are determined as the product of fuel consumption, fuel thermal value and emission factors.

Statistics Sweden, for each fuel type, has produced thermal values. All thermal values refer to net calorific values. Most thermal values are calculated on the basis of chemical qualities and are considered to be of good quality. For some fuel types (coal, coke oven gas and blast furnace gas) the thermal value vary for different years, due to that these thermal values are based on information from the users. The uncertainty of the thermal value for coal, coke oven gas and blast furnace gas is larger than for other fuels.

The quality of the thermal values can at present not be quantified.

The emission factors are provided by the Swedish Environmental Protection Agency and Statistics Sweden and are based on results from measurements and national studies as well as studies of internationally used emission factors and judgments of their relevance to national conditions. The emission factors depend on type of fuel, type of plant and abatement equipment. Thermal values and emission factors are enclosed in the end of this chapter.

Methods for and the quality in activity data (fuel consumption etc) vary for different sectors as described below.

Although the statistical data generally is of good quality, Reference approach and Sectoral approach does not match for all years (1992, 1994, 1995,1998 and 2000). The difference is largest for 1995, when the difference in energy use is 3,6%. The reason for these differences is that energy statistics in Sweden are not fully adapted to international reporting of environmental statistics. The biggest differences are found in the industry sector. To calculate the total energy use by energy carrier and sector, there are different methods in the energy statistics, which could cause these differences in energy use.

Emissions of CO2 differ some more between Reference approach and Sectoral approach than energy use. The reasons for this will be analysed in the near future and this is also mentioned in chapter 7.

### Fuel Combustion Activities – Sectoral Approach

#### Energy Industries, Manufacturing Industries and Construction

1A1a: Information on fuel consumption comes from Statistics Sweden, energy statistics (1). The statistics are based on a survey to all working units in the country and are considered to be of very good quality.

1A1b-c and 1A2, stationary combustion: Information on fuel consumption comes from Statistics Sweden, energy statistics. Most fuel types have been collected from a survey to all companies with at least 10 employees for the years 1990-1996 and 2000. This survey gives information of high quality for all purchased fuels.

Activity data for 1997-1999 has been taken from a sample survey on working unit level. The results are adjusted upwards to match the statistics for the years 1990-1996.

Information on coal, coke, peat, MSW, sulfur lies and landfill gas is collected from the sample survey 1990-2000. Tall oil and ”other not specified” fuels are collected from the sample survey 1990 and 1997-2000.

Before 1996 the industry survey had focus on economy, not energy. From 1997-1999 the energy statistics part of Statistics Sweden is responsible for this survey and thus the focus is changed more towards energy. The industry survey is a sample survey and thus sent to fewer companies than before. This could of course result in larger uncertainties in point estimates. Both surveys are considered to give high quality information and possible mistakes, when the sample survey is adjusted, are marginal.

1A2, CO2 from off-road vehicles and other machinery: see chapter 1.2.1.2 below.

#### Transport, Emission of CO2

The statistics are based on information of delivered amounts of fuel from wholesale dealers on national level. All figures are double-checked by Statistics Sweden and all wholesale dealers. The quality is very good and any faults that may occur are negligible.

Disaggregated reporting to UNFCCC requires additional information, assumptions and model calculations:

Information on fuel consumption for railway and military use is collected from the Swedish National Rail Administration and the Swedish Armed Forces. The information is considered to be of good quality.

The division between domestic and international flights is made according to the calculations for CORINAIR made by the Swedish EPA (2). These calculations are in turn based on calculations from the Swedish Civil Aviation Administration. LTO-cycles are considered according to the Good Practice Guidance.

Emissions from domestic and international Navigation are calculated on the basis of fuel consumption according to a survey to wholesale dealers. For the year 2000, Military use is separated and reported under 1A5b.

Information on consumption of diesel oil and residual fuel oil for domestic and bunker navigation is given in the survey to the wholesale dealers.

1,5% of delivered gasoline in Sweden is assumed to be used by small boats. The uncertainty in this assumption is not known. It is based on a report made in 1992 (3). All other gasoline is assumed to be used by off-road vehicles and other machinery and road traffic.

Gasoline- and diesel oil consumption by off-road vehicles and other machinery for the years 1995-2000 is calculated according to an inventory made in 1999 (4). Allocation to different sectors is made according to this report for the whole time series, 1990-2000.

Total fuel consumption for off-road vehicles and other machinery in 1990 is based on two reports published in 1989 (5) and 1990 (6). The consumption for the years 1991-1994 is interpolated between 1990 and 1995.

Estimating the numbers of all off-road vehicles and other machinery is very difficult. It is even more difficult to estimate their fuel consumption. For this reason there is a large uncertainty in the estimation of fuel consumption for this category.

Consumption of gasoline and diesel oil by road traffic is calculated by the Swedish National Road Administration. A model is used that accounts for traffic data, descriptions of different vehicle categories as well as descriptions of different fuel types. Engine technology and different modes of driving (rural/urban/highway) are also considered. The model is called EMV, which stands for Emissions from Road traffic. It is developed by VTI, the Swedish Road- and Transport Research Institute, by order of the Swedish EPA.

Delivered amount of diesel varies a lot for different years. Deliveries go up the period before an expected increase in price. Nothing indicates that the consumption should increase in the same way. This suggests that diesel is stored by consumers during the period before an increase in price. In order to improve the estimation of diesel consumption a correction for private storing has been made 1990-1999.

Delivered amount of diesel for the years 1990-1999 is reallocated so that the share for each year is the same as the share of transport volume (in vehicle km) that year compared with total transport volume (in vehicle km) for the years 1990-1999 for diesel vehicles. Yearly transport volume (in vehicle km) data from EMV goes back to 1990.

The quality of this correction can not be quantified, but it is assumed to give a more reliable picture of the diesel consumption each year. The correction has not been made for 2000 since there has been no dramatic pricechanges during the period.

Delivered amount of diesel with correction for private storing does not match modeled consumption by off-road vehicles and other machinery, railway, military use, navigation and road traffic. This is because of the uncertainties in the assumptions and models being used for the different sectors. The difference in diesel is allocated according to the relative size of the different sectors. As a result of this the sum of consumed diesel is the same as delivered amount of diesel with correction for private storing.

To sum up, the estimation of total fuel consumption for mobile sources is very good while the allocation between different sectors is unsure. The uncertainties can at present not be quantified.

#### Transport, emissions of CH4, N2O, NOx, CO, NMVOC and SO2

Some changes have been made for the calculation of emissions regarding the year 2000 compared to emissions 1990-1999. In the near future emissions from mobile sources for all years will be revised to ensure consistency in the emission trends.

Emissions from military use of aviation fuel was reported together with emissions from civil aviation under 1A3a for the years 1990-1999. For the year 2000 emissions from military use of aviation fuel was separated and reported under 1A5b.

Emissions from electricity production are included in emissions from railways for the years 1990-1999. For the year 2000 emissions from electricity production are not included.

Emissions of N2O from all aviation, emissions of CH4, N2O and NMVOC from railways and emissions of CH4 and NMVOC from marine international bunkers were estimated for the year 2000 but not for the years 1990-1999.

In this submission emissions of NMVOC from off road vehicles have been corrected 1997 and 1998 due to errors in the previous submission.

Emissions from road transportation are calculated with a model in use at the Swedish EPA described in chapter 1.2.1.2. The model has been used to calculate emissions for the years 1995-2000 for all gases. The quality is considered to be very good. CH4, N2O, NMVOC and SO2 are calculated with a different model for the years 1990-1994 and are therefore not directly comparable with the years 1995-2000. No division between gasoline and diesel has been made for the years 1990-1994 due to lack of data. Data on these years is going to be revised in the near future in order to make the whole time series consistent with regard to methodology, Good Practice Guidance and to make background data consistent with the methodology used for estimating CO2 emissions.

Emissions from off-road vehicles and other machinery for the years 1997-2000 are estimated on basis from a Swedish report (4). Emissions for 1994-1996 are based on two reports (5,6). For the years 1990-1994 the same division into subgroups has been used as in 1994. A review of data is going to be made in the near future in order to make estimations more transparent and consistent with the methodology used for estimating CO2 emissions.

Information on emissions from railways is collected from the Swedish National Rail Administration. The quality of calculated emissions in this sector is considered to be representative.

Information on emissions from aviation, and division between domestic and international flights, has been collected from the Swedish Civil Aviation Administration. The quality is considered to be good.

Information on emissions from navigation, and division between domestic and international marine operations, has been collected from the Swedish Maritime Administration for the years 1990-1999. The quality of the calculated emissions is considered to be representative, but the division into national and international operations is approximate and not well documented. This has to be revised in the future. For 2000 this division was not necessary since emissions from domestic and international Navigation are calculated on the basis of fuel consumption according to a survey to wholesale dealers and emission factors from Statistics Sweden and CORINAIR.

### Other Sectors

Information on consumption of various fuel types has been collected from Statistics Sweden, energy statistics.

The uncertainties vary a lot for different sectors. Information on household consumption, the largest part of 1A4, is of relative good quality. Fuel consumption for other subsectors has been estimated with the aid of models and extrapolations of old results, which do not give as good quality in the estimations. Total consumption in 1A4 is checked against fuel deliveries. Possible errors only occur in the allocation between sectors.

Statistics Sweden also calculates emissions from off-road vehicles and other machinery used in 1A4, for the years 1990-1999 only of CO2. The method for this is described under chapter 1.2.1.2 above.

### Other (Not elsewhere specified)

In this sector only military mobile sources are reported. The method is described under chapter 1.2.1.2.

## CO2 from Fuel Combustion Activities – Reference Approach

Information on fuel consumption comes from Statistics Sweden, energy statistics. It is the same source as for fuel combustion in 1A1a and the quality is very good. Corrections for international bunkers and private storing of diesel are made 1990-1999 but not for 2000, see chapter 1.2.1.2 and 1.2.1.3.

## Feedstocks and Non-Energy Use of Fuels

“Non-Energy use of Fuels” is collected from Statistics Sweden, energy statistics, the same source as for 1A1a and 1Ab. The quality of the estimates is good.

## Fugitive Emissions from Solid Fuels, Oil and Natural Gas

Reported fugitive emissions from natural gas and gas works gas consists of the difference between delivered amount and consumed amount. The gases are delivered in pipelines and fugitive emissions do not occur according to wholesale dealers. The difference could be due to measure errors. Sweden chooses to report these emissions anyway. The uncertainty in the estimates is therefore high.

Reported fugitive emissions from other fuels consist of transmission losses (transports in pipelines) and some flaring according to Statistics Sweden, energy statistics. These estimates are of good quality.

There also exists some flaring that should be reported in section 1B. Currently these emissions are included in section 1A due to lack of proper background data. This will be revised in the near future.

## International Bunkers and Multilateral Operations

See chapter 1.2.1.2 and 1.2.1.3.

## Tables with Thermal Values and used Emission Factors

### Thermal Values

GJ/unit 1990-2000

| Fuel Category | Unit | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gas/Diesel Oil | m3 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 |
| Residual Fuel Oil | m3 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 |
| Liquefied Petroleum Gas | Ton | 46,05 | 46,05 | 46,05 | 46,05 | 46,05 | 46,05 | 46,05 | 46,05 | 46,05 | 46,05 | 46,05 |
| Gas Works Gas | 1000 m3 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 |
| Natural Gas | 1000 m3 | 34,992 | 34,992 | 34,992 | 34,992 | 34,992 | 34,992 | 34,992 | 34,992 | 34,992 | 34,992 | 34,992 |
| Coke Oven Gas | 1000 m3 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 | 16,75 |
| Blast Furnace Gas | 1000 m3 | 2,83 | 2,89 | 2,86 | 2,83 | 2,87 | 2,82 | 2,78 | 2,75 | 2,81 | 2,9 | 2,9 |
| Blast Furnace Gas | 1000 m3 | 6,09 | 6,1 | 6,1 | 6,61 | 6,73 | 6,87 | 7,23 | 7,07 | 7,54 | 7,2 | 7,2 |
| Coking Coal | ton | 27,21 | 27,21 | 27,21 | 27,21 | 27,21 | 27,21 | 27,21 | 27,21 | 27,21 | 27,21 | 27,21 |
| Coke Oven Coke | ton | 28,05 | 28,05 | 28,05 | 28,05 | 28,05 | 28,05 | 28,05 | 28,05 | 28,05 | 28,05 | 28,05 |
| Biomass Fuel | toe | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 |
| Peat | toe | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 |
| Municipal Solid Waste | toe | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 |
| Sulphur Lies (Black Liquor) | toe | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 |
| Sulphur Lies (Black Liquor) | m3 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 | 38,94 |
| Other | toe | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 | 41,87 |
| Other Oil | m3 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 | 35,59 |

Thermal Values are a stereotyped estimation of respective fuels energy content.

### CO2

Emission Factors 1990-2000 (gram CO2/MJ)

|  |  | Thermal | CO2 |  |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| Fuel Category | Unit | GJ/Unit | g/MJ | Area of Use |
| Gas/Diesel Oil | m3 | 35,59 | 75,3 | All Consumption |
| Residual Fuel Oil | m3 | 38,94 | 76,2 | All Consumption |
| Residual Fuel Oil | m3 | 38,94 | 76,2 | All Consumption |
| Liquefied Petroleum Gas | ton | 46,05 | 65,1 | All Consumption |
| Gas Works Gas | 1 000 m3 | 16,75 | 77,5 | All Consumption |
| Natural Gas | 1 000 m3 | 34,992 | 56,5 | All Consumption |
| Coke Oven Gas | 1 000 m3 | 16,75 | 60,0 | All Consumption |
| Blast Furnace Gas | 1 000 m3 | 3,35 * | 103,0 | All Consumption |
| Blast Furnace Gas | 1 000 m3 | 8,37 * | 199,9 | All Consumption |
| Coking Coal | ton | 27,21 * | 90,7 | All Consumption |
| Coke Oven Coke | ton | 28,05 | 103,0 | All Consumption |
| Wood Waste | toe | 41,87 | 96,0 | All Consumption |
| Peat | ton | 9,91 | 107,3 | Consumption in (electricity-, gas- and Heating Plant) |
| Peat | ton | 6,12 | 97,1 | Industry Consumption |
| Municipal Solid Waste | ton | 9,52 | 32,7 | Consumption in (electricity-, gas- and Heating Plant) |
| Municipal Solid Waste | ton | 10,96 | 28,4 | Industry Consumption |
| Sulfur Lies (Black Liquor) | toe | 41,87 | 108,0 | All Consumption |
| Sulfur Lies (Black Liquor) | m3 | 38,94 | - | Emissions Factor Not Available |
| Other | toe | 41,87 | 60,0 | All Consumption |
| Gasoline | m3 | 31,40 | 72,6 | All Consumption |
| Jet Gasoline | m3 | 32,70 | 72,3 | All Consumption |
| Other Kerosene | m3 | 34,50 | 73,1 | All Consumption |
| Diesel Oil | m3 | 35,59 | 75,3 | All Consumption |
| Transformation losses in refineries | GJ | 1,00 | 76,2 | All Consumption |
| Transformation losses in iron and steel | GJ | 1,00 | 79,0 | All Consumption |

* Biomass

* Thermal value for these fuel categories fluctuate every year; an average is stated here.

Source: Swedish EPA and Statistics Sweden

### CH4

Emission Factors 1990-2000 (gram CH4/MJ)

|  |  |  | District |  | Thermal Value |
| --- | --- | --- | --- | --- | --- |
|  | Industry | Residence | Heating | Electricity | GJ/Unit |
| Diesel Oil | 0,001 | 0,002 | 0,001 | 0,001 | 35,59 |
| Residual Fuel Oil | 0,002 | 0,003 | 0,002 | 0,002 | 38,94 |
| Liquefied Petroleum Gas | 0,001 | 0,001 | 0,001 | 0,001 | 46,05 |
| Gas Works Gas | 0,001 | 0,001 | 0,001 | 0,001 | 16,75 |
| Natural Gas | 0,001 | 0,001 | 0,001 | 0,001 | 34,992 |
| Coke Oven Gas | 0,001 | 0,001 | 0,001 | 0,001 | 16,75 |
| Blast Furnace Gas | 0,001 | 0,001 | 0,001 | 0,001 | 3,35* |
| Blast Furnace Gas | 0,001 | 0,001 | 0,001 | 0,001 | 8,37* |
| Coking Coal | 0,002 | 0,004 | 0,002 | 0,002 | 27,21* |
| Coke Oven Coke | 0,002 | 0,004 | 0,002 | 0,002 | 28,05 |
| Wood Waste | 0,03 | 0,25 | 0,03 | 0,03 | 41,87 |
| Peat | 0,03 | 0,03 | 0,02 | 0,02 | 41,87 |
| Municipal Solid Waste | 0,02 | 0,03 | 0,02 | 0,02 | 41,87 |
| Sulfur Lies (Black Liquor) | 0,03 | 0 | 0 | 0 | 41,87 |
| Sulfur Lies (Black Liquor) | 0,002 | 0,003 | 0,002 | 0,002 | 38,94 |
| Other | 0,001 | 0,001 | 0,001 | 0,001 | 41,87 |
| Other Oil | 0,001 | 0,002 | 0,001 | 0,001 | 35,59 |
| Transformation losses in refineries | 0,002 | - | - | - | 1,00 |
| Transformation losses in iron and steel | 0,003 | - | - | - | 1,00 |

* Thermal Values for these fuel categories fluctuate every year; an average is stated here.

Fuels Consumption Refer to Residence, include Peat and Municipal Solid Waste.

Consumption report is based on “quarterly” statistics every three months, Statistics Sweden

Emission factors from Swedish EPA

Thermal Values are a stereotyped estimation of respective fuels energy content.

### N2O

Emission Factors 1990-2000 (gram N2O/MJ)

|  |  |  | District |  | Thermal Value |
| --- | --- | --- | --- | --- | --- |
|  | Industry | Residence | Heating | Electricity | GJ/Unit |
| Diesel Oil | 0,002 | 0,002 | 0,002 | 0,002 | 35,59 |
| Residual Fuel Oil | 0,005 | 0,005 | 0,005 | 0,005 | 38,94 |
| Liquefied Petroleum Gas | 0,002 | 0,002 | 0,002 | 0,002 | 46,05 |
| Gas Works Gas | 0,002 | 0,002 | 0,002 | 0,002 | 16,75 |
| Natural Gas | 0,002 | 0,002 | 0,002 | 0,002 | 34,992 |
| Coke Oven Gas | 0,002 | 0,002 | 0,002 | 0,002 | 16,75 |
| Blast Furnace Gas | 0,002 | 0,002 | 0,002 | 0,002 | 3,35 * |
| Blast Furnace Gas | 0,002 | 0,002 | 0,002 | 0,002 | 8,37 * |
| Coking Coal | 0,02 | 0,02 | 0,02 | 0,02 | 27,21 * |
| Coke Oven Coke | 0,02 | 0,02 | 0,02 | 0,02 | 28,05 |
| Wood Waste | 0,005 | 0,005 | 0,005 | 0,005 | 41,87 |
| Peat | 0,01 | 0,01 | 0,01 | 0,01 | 41,87 |
| Municipal Solid Waste | 0,005 | 0,005 | 0,005 | 0,005 | 41,87 |
| Sulfur Lies (Black Liquor) | 0,005 | 0 | 0 | 0 | 41,87 |
| Sulfur Lies (Black Liquor) | 0,005 | 0,005 | 0,005 | 0,005 | 38,94 |
| Other | 0,002 | 0,002 | 0,002 | 0,002 | 41,87 |
| Other Oil | 0,002 | 0,002 | 0,002 | 0,002 | 35,59 |
| Transformation losses in refineries | 0,005 | - | - | - | 1,00 |
| Transformation losses in iron and steel | 0,04 | - | - | - | 1,00 |

* Thermal Values for these fuel categories fluctuate every year; an average is stated here.

Fuels Consumption refers to Residence include Peat and Municipal Solid Waste.

Consumption report is based on “quarterly” statistics every three months, Statistics Sweden

Emission factors from Swedish EPA

Thermal Values are a stereotyped estimation of respective fuels energy content.

### NOx

Emission Factors 1990-2000 (gram NOx/MJ)

|  |  | Thermal | NOX |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | kg/MJ |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Category | Unit | GJ/Unit | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | Area of use |
| Gas/Diesel Oil | m3 | 35,59 | 0,60 | 0,60 | 0,60 | 0,30 | 0,20 | 0,20 | 0,20 | 0,20 | 0,20 | 0,20 | 0,20 | Gas Turbine & diesel drifting |
| Gas/Diesel Oil | m3 | 35,59 | 0,07 | 0,07 | 0,07 | 0,06 | 0,06 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | Other sector |
| Gas/Diesel Oil | m3 | 35,59 | 0,07 | 0,07 | 0,07 | 0,06 | 0,06 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | Other Consumption |
| Residual Fuel Oil | m3 | 38,94 | 0,30 | 0,30 | 0,30 | 0,25 | 0,25 | 0,20 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | Gas Turbine & diesel drifting |
| Residual Fuel Oil | m3 | 38,94 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | Consumption in the Cement Industry |
| Residual Fuel Oil | m3 | 38,94 | 0,17 | 0,17 | 0,15 | 0,15 | 0,12 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | Other sector |
| Residual Fuel Oil | m3 | 38,94 | 0,17 | 0,17 | 0,15 | 0,15 | 0,12 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | Other Consumption |
| Residual Fuel Oil | m3 | 38,94 | 0,30 | 0,30 | - | - | - | - | - | - | - | - | - | Gas Turbine & diesel drifting |
| Residual Fuel Oil | m3 | 38,94 | 0,50 | 0,50 | - | - | - | - | - | - | - | - | - | Consumption in the Cement Industry |
| Residual Fuel Oil | m3 | 38,94 | 0,17 | 0,17 | - | - | - | - | - | - | - | - | - | Other sector |
| Residual Fuel Oil | m3 | 38,94 | 0,17 | 0,17 | - | - | - | - | - | - | - | - | - | Other Consumption |
| Liquefied Petroleum Gas | ton | 46,05 | 0,04 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | Other sector |
| Liquefied Petroleum Gas | ton | 46,05 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | Other Consumption |
| Gas Works Gas | 1 000 m3 | 16,75 | 0,04 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | Other sector |
| Gas Works Gas | 1 000 m3 | 16,75 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | Other Consumption |
| Natural Gas | 1 000 m3 | 34,992 | 0,04 | 0,04 | 0,04 | 0,04 | 0,04 | 0,04 | 0,04 | 0,04 | 0,04 | 0,04 | 0,04 | Other sector |
| Natural Gas | 1 000 m3 | 34,992 | 0,07 | 0,06 | 0,06 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | Other Consumption |
| Coke Oven Gas | 1 000 m3 | 16,75 | 0,10 | 0,10 | 0,10 | 0,06 | 0,06 | 0,03 | 0,03 | 0,03 | 0,03 | 0,03 | 0,03 | All Consumption |
| Blast Furnace Gas | 1 000 m3 | 3,35 * | 0,10 | 0,10 | 0,06 | 0,06 | 0,06 | 0,03 | 0,03 | 0,03 | 0,03 | 0,03 | 0,03 | All Consumption |
| Blast Furnace Gas | 1 000 m3 | 8,37 * | 0,10 | 0,10 | 0,10 | 0,06 | 0,06 | 0,03 | 0,03 | 0,03 | 0,03 | 0,03 | 0,03 | All Consumption |
| Coking Coal | ton | 27,21 * | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | Consumption in the Cement Industry |
| Coking Coal | ton | 27,21 * | 0,20 | 0,20 | 0,20 | 0,20 | 0,20 | 0,20 | 0,20 | 0,20 | 0,20 | 0,20 | 0,20 | Consumption in the Cement Industry |
| Coking Coal | ton | 27,21 * | 0,20 | 0,20 | 0,20 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | Other sector |
| Coking Coal | ton | 27,21 * | 0,20 | 0,20 | 0,20 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | Other Consumption |
| Coking Coal | ton | 27,21 * | - | - | - | 0,30 | 0,70 | 0,70 | 0,70 | 0,70 | 0,70 | 0,70 | 0,70 | Mining |

|  |  | Thermal | NOX |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | kg/MJ |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Category | Unit | GJ/Unit | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | Area of use |
| Coke Oven Coke | ton | 28,05 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | 0,50 | Consumption in the Cement Industry |
| Coke Oven Coke | ton | 28,05 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | Other Consumption |
| Wood Waste | toe | 41,87 | 0,07 | 0,07 | 0,07 | 0,06 | 0,06 | 0,06 | 0,06 | 0,06 | 0,06 | 0,06 | 0,06 | Other sector |
| Wood Waste | toe | 41,87 | 0,12 | 0,12 | 0,11 | 0,11 | 0,11 | 0,11 | 0,11 | 0,11 | 0,11 | 0,11 | 0,11 | Consumption in Industry |
| Wood Waste | toe | 41,87 | 0,12 | 0,12 | 0,11 | 0,10 | 0,09 | 0,08 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | Consumption in (electricity-, gas- and Heating Plant) |
| Peat | toe | 41,87 | 0,20 | 0,19 | 0,18 | 0,15 | 0,15 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | All Consumption |
| Municipal Solid Waste | toe | 41,87 | 0,17 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | Consumption in Industry |
| Municipal Solid Waste | toe | 41,87 | 0,17 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | 0,09 | Other Consumption |
| Municipal Solid Waste | toe | 41,87 | 0,25 | 0,25 | 0,25 | 0,25 | 0,25 | 0,25 | 0,25 | 0,25 | 0,25 | 0,25 | 0,25 | All Consumption |
| Sulfur Lies (Black Liquor) | toe | 41,87 | 0,07 | 0,07 | 0,07 | 0,07 | 0,06 | 0,06 | 0,06 | 0,06 | 0,06 | 0,06 | 0,06 | All Consumption |
| Sulfur Lies (Black Liquor) | m3 | 38,94 | 0,10 | 0,10 | 0,10 | 0,10 | 0,13 | 0,13 | 0,13 | 0,13 | 0,13 | 0,13 | 0,13 | All Consumption |
| Other | toe | 41,87 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | All Consumption |
| Gasoline | m3 | 31,40 | * | * | * | * | * | * | * | * | * | * | * | All Consumption |
| Jet Gasoline | m3 | 32,70 | * | * | * | * | * | * | * | * | * | * | * | All Consumption |
| Other Kerosene | m3 | 34,50 | * | * | * | * | * | * | * | * | * | * | * | All Consumption |
| Diesel Oil | m3 | 35,59 | * | * | * | * | * | * | * | * | * | * | * | All Consumption |
| Diesel Oil | m3 | 35,59 | * | * | * | * | * | * | * | * | * | * | * | All Consumption |
| Residual Fuel Oil | m3 | 38,94 | * | * | * | * | * | * | * | * | * | * | * | All Consumption |
| Residual Fuel Oil | m3 | 38,94 | * | * | * | * | * | * | * | * | * | * | * | All Consumption |
| Transformation losses in refineries | GJ | 1,00 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | 0,07 | Refinery |
| Transformation losses in iron and steel | GJ | 1,00 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | Coke Oven Plant |

### CO

Emission Factors 1990-2000 (gram CO/MJ)

|  |  |  | District |  | Thermal Value |
| --- | --- | --- | --- | --- | --- |
|  | Industry | Residence | Heating | Electricity | GJ/Unit |
| Diesel Oil | 0,02 | 0,05 | 0,02 | 0,01 | 35,59 |
| Residual Fuel Oil | 0,025 | 0,075 | 0,025 | 0,01 | 38,94 |
| Liquefied Petroleum Gas | 0,01 | 0,01 | 0,01 | 0,01 | 46,05 |
| Gas Works Gas | 0,01 | 0,01 | 0,01 | 0,01 | 16,75 |
| Natural Gas | 0,01 | 0,025 | 0,01 | 0,01 | 34,992 |
| Coke Oven Gas | 0,01 | 0,01 | 0,01 | 0,01 | 16,75 |
| Blast Furnace Gas | 0,01 | 0,01 | 0,01 | 0,01 | 3,35 * |
| Blast Furnace Gas | 0,01 | 0,01 | 0,01 | 0,01 | 8,37 * |
| Coking Coal | 0,05 | 0,05 | 0,04 | 0,04 | 27,21 * |
| Coke Oven Coke | 0,05 | 0,05 | 0,04 | 0,04 | 28,05 |
| Wood Waste | 0,15 | 3 | 0,15 | 0,15 | 41,87 |
| Peat | 0,1 | 0 | 0,15 | 0,15 | 41,87 |
| Municipal Solid Waste | 0,15 | 0 | 0,05 | 0,05 | 41,87 |
| Sulfur Lies (Black Liquor) | 0,15 | - | - | - | 41,87 |
| Sulfur Lies (Black Liquor) | 0,025 | 0,075 | 0,025 | 0,025 | 38,94 |
| Other | 0,01 | 0,01 | 0,01 | 0,01 | 41,87 |
| Other Oil | 0,02 | 0,05 | 0,02 | 0,02 | 35,59 |
| Transformation losses in refineries | 0,02 | - | - | - | 1,00 |
| Transformation losses in iron and steel | 0,05 | - | - | - | 1,00 |

Consumption report is based on “quarterly” statistics every three months, Statistics Sweden

Emission factors from Swedish EPA

Thermal Values are a stereotyped estimation of respective fuels energy content.

*Thermal Values for these fuel categories fluctuate every year, an average is stated here.

### NMVOC

Emission Factors 1990-2000 (gram NMVOC/MJ)

|  |  |  | District |  | Thermal Value |
| --- | --- | --- | --- | --- | --- |
|  | Industry | Residence | Heating | Electricity | GJ/Unit |
| Diesel Oil | 0,002 | 0,003 | 0,002 | 0,002 | 35,59 |
| Residual Fuel Oil | 0,003 | 0,005 | 0,003 | 0,003 | 38,94 |
| Liquefied Petroleum Gas | 0,002 | 0,002 | 0,002 | 0,002 | 46,05 |
| Gas Works Gas | 0,002 | 0,002 | 0,002 | 0,002 | 16,75 |
| Natural Gas | 0,002 | 0,002 | 0,002 | 0,002 | 34,99 |
| Coke Oven Gas | 0,002 | 0,002 | 0,002 | 0,002 | 16,75 |
| Blast Furnace Gas | 0,002 | 0,002 | 0,002 | 0,002 | 3,35 * |
| Blast Furnace Gas | 0,002 | 0,002 | 0,002 | 0,002 | 8,37 * |
| Coking Coal | 0,008 | 0,008 | 0,008 | 0,005 | 27,21 * |
| Coke Oven Coke | 0,008 | 0,008 | 0,008 | 0,005 | 28,05 |
| Wood Waste | 0,1 | 3,34 | 0,1 | 0,05 | 41,87 |
| Peat | 0,05 | 0,05 | 0,05 | 0,005 | 41,87 |
| Municipal Solid Waste | 0,025 | 0,025 | 0,025 | 0,005 | 41,87 |
| Sulfur Lies (Black Liquor) | 0,1 | - | - | - | 41,87 |
| Sulfur Lies (Black Liquor) | 0,002 | 0,005 | 0,002 | 0,002 | 38,94 |
| Other | 0,002 | 0,002 | 0,002 | 0,002 | 41,87 |
| Other Oil | 0,002 | 0,005 | 0,002 | 0,002 | 35,59 |
| Transformation losses in refineries | 0,37 | - | - | - | 1,00 |
| Transformation losses in iron and steel | 0,01 | - | - | - | 1,00 |

*Thermal Values for these fuel categories fluctuate every year, an average is stated here.

Fuel Consumption refers to Residence includes Sulfur Lies (Black Liquor), Peat and Municipal Solid Waste.

Emission factors have been received from Swedish EPA

*Thermal values are a stereotyped estimation of respective fuels energy content.

### SO2

Emission Factors 1990-2000 (gram SO2/MJ)

|  |  | Thermal | SO2 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | g/MJ |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Category | Unit | GJ/Unit | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | Area of Use |
| Gas/Diesel Oil | m3 | 35,59 | 0,1800 | 0,1500 | 0,1500 | 0,1500 | 0,1500 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | Gas Turbine & diesel drifting |
| Gas/Diesel Oil | m3 | 35,59 | 0,0800 | 0,0800 | 0,0800 | 0,0400 | 0,0400 | 0,0400 | 0,0300 | 0,0300 | 0,0300 | 0,0300 | 0,0300 | Other sector |
| Gas/Diesel Oil | m3 | 35,59 | 0,0800 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0300 | 0,0300 | 0,0300 | 0,0300 | 0,0300 | Other Consumption |
| Residual Fuel Oil | m3 | 38,94 | 0,4800 | 0,4800 | 0,3800 | 0,0300 | 0,2500 | 0,2000 | 0,1700 | 0,1700 | 0,1700 | 0,1700 | 0,1700 | Gas Turbine & diesel drifting |
| Residual Fuel Oil | m3 | 38,94 | 0,3800 | 0,3800 | 0,3800 | 0,3800 | 0,3800 | 0,3800 | 0,3800 | 0,3800 | 0,3800 | 0,3800 | 0,3800 | Into Cement Industry |
| Residual Fuel Oil | m3 | 38,94 | 0,3300 | 0,2400 | 0,2400 | 0,2200 | 0,2000 | 0,1800 | 0,1800 | 0,1800 | 0,1800 | 0,1800 | 0,1800 | Other sector |
| Residual Fuel Oil | m3 | 38,94 | 0,3300 | 0,2400 | 0,2400 | 0,2200 | 0,2000 | 0,1800 | 0,1800 | 0,1800 | 0,1800 | 0,1800 | 0,1800 | Other Consumption |
| Residual Fuel Oil | m3 | 38,94 | 1,2000 | 1,2000 | - | - | - | - | - | - | - | - | - | Gas Turbine & diesel drifting |
| Residual Fuel Oil | m3 | 38,94 | 1,2000 | 1,2000 | - | - | - | - | - | - | - | - | - | Into Cement Industry |
| Residual Fuel Oil | m3 | 38,94 | 1,2000 | 1,2000 | - | - | - | - | - | - | - | - | - | Other sector |
| Residual Fuel Oil | m3 | 38,94 | 1,2000 | 1,2000 | - | - | - | - | - | - | - | - | - | Other Consumption |
| Liquefied Petroleum Gas | ton | 46,05 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | Other sector |
| Liquefied Petroleum Gas | ton | 46,05 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | Other Consumption |
| Gas Works Gas | 1 000 m3 | 16,75 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | Other sector |
| Gas Works Gas | 1 000 m3 | 16,75 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | Other Consumption |
| Natural Gas | 1 000 m3 | 34,992 | 0,0020 | 0,0020 | 0,0020 | 0,0020 | 0,0020 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | Other sector |
| Natural Gas | 1 000 m3 | 34,992 | 0,0020 | 0,0020 | 0,0020 | 0,0020 | 0,0020 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | Other Consumption |
| Coke Oven Gas | 1 000 m3 | 16,75 | 0,1400 | 0,1200 | 0,1200 | 0,1400 | 0,1200 | 0,1500 | 0,1500 | 0,1500 | 0,1500 | 0,1500 | 0,1500 | All Consumption |
| Blast Furnace Gas | 1 000 m3 | 3,35 * | 0,0050 | 0,0050 | 0,0050 | 0,0050 | 0,0050 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | All Consumption |
| Blast Furnace Gas | 1 000 m3 | 8,37 * | 0,0003 | 0,0003 | 0,0003 | 0,0003 | 0,0003 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | All Consumption |
| Coking Coal | ton | 27,21 * | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | Into Cement Industry |
| Coking Coal | ton | 27,21 * | 0,1000 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | Into Cement Industry |
| Coking Coal | ton | 27,21 * | 0,6000 | 0,3600 | 0,3600 | 0,3600 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | Other sector |
| Coking Coal | ton | 27,21 * | 0,3800 | 0,3600 | 0,3600 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | Other Consumption |
| Coking Coal | ton | 27,21 * | - | - | - | 0,2200 | 0,2700 | 0,2700 | 0,2700 | 0,2700 | 0,2700 | 0,2700 | 0,2700 | Mining |

|  |  | Thermal | SO2 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | g/MJ |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Category | Unit | GJ/Unit | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | Area of Use |
| Coke Oven Coke | ton | 28,05 | 0,3000 | 0,3000 | 0,3000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | 0,2000 | Into Cement Industry |
| Coke Oven Coke | ton | 28,05 | 0,4800 | 0,3600 | 0,3600 | 0,3600 | 0,3600 | 0,3600 | 0,3600 | 0,3600 | 0,3600 | 0,3600 | 0,3600 | Other Consumption |
| Wood Waste | toe | 41,87 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | Other sector |
| Wood Waste | toe | 41,87 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | 0,0400 | Other Consumption |
| Peat | toe | 41,87 | 0,2400 | 0,2200 | 0,2200 | 0,2000 | 0,1800 | 0,1500 | 0,1500 | 0,1300 | 0,1300 | 0,1300 | 0,1300 | All Consumption |
| Municipal Solid Waste | toe | 41,87 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | Industry Consumption |
| Municipal Solid Waste | toe | 41,87 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | 0,1100 | Other Consumption |
| Municipal Solid Waste | toe | 41,87 | 0,1750 | 0,1750 | 0,1750 | 0,1750 | 0,1750 | 0,1750 | 0,1750 | 0,1750 | 0,1750 | 0,1750 | 0,1750 | All Consumption |
| Sulfur Lies (Black Liquor) | toe | 41,87 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | 0,0000 | All Consumption |
| Sulfur Lies (Black Liquor) | m3 | 38,94 | 0,1400 | 0,1400 | 0,1400 | 0,1400 | 0,1400 | 0,1400 | 0,1400 | 0,1400 | 0,1400 | 0,1400 | 0,1400 | All Consumption |
| Other | toe | 41,87 | 0,2400 | 0,2400 | 0,2400 | 0,2400 | 0,2400 | 0,1500 | 0,1500 | 0,1500 | 0,1500 | 0,1500 | 0,1500 | All Consumption |
| Gasoline | m3 | 31,40 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | 0,0100 | All Consumption |
| Jet Gasoline | m3 | 32,70 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | All Consumption |
| Other Kerosene | m3 | 34,50 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | 0,0140 | All Consumption |
| Diesel Oil | m3 | 35,59 | 0,1200 | 0,0450 | 0,0350 | 0,0190 | 0,0120 | 0,0120 | 0,0120 | 0,0120 | 0,0120 | 0,0120 | 0,0120 | All Consumption |
| Diesel Oil | m4 | 35,59 | 0,1200 | 0,0450 | 0,0450 | 0,0450 | 0,0450 | 0,0450 | 0,0450 | 0,0450 | 0,0450 | 0,0450 | 0,0450 | All Consumption |
| Diesel Oil | m3 | 35,59 | 0,1000 | 0,1000 | 0,1000 | 0,1000 | 0,0500 | 0,0500 | 0,0500 | 0,0500 | 0,0500 | 0,0500 | 0,0500 | All Consumption |
| Residual Fuel Oil | m3 | 38,94 | 0,3300 | 0,3300 | 0,3300 | 0,4300 | 0,3300 | 0,3300 | 0,3300 | 0,3300 | 0,3300 | 0,3300 | 0,3300 | All Consumption |
| Residual Fuel Oil | m3 | 38,94 | 1,2000 | 1,2000 | 1,2000 | 1,2000 | 1,2000 | 1,2000 | 1,2000 | 1,2000 | 1,2000 | 1,2000 | 1,2000 | All Consumption |
| Transformation losses in refineries | GJ | 1,00 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | 0,05 | Refinery |
| Transformation losses in iron and steel | GJ | 1,00 | 0,035 | 0,035 | 0,035 | 0,035 | 0,035 | 0,035 | 0,035 | 0,035 | 0,035 | 0,035 | 0,035 | Coke Oven Plant |

*Thermal Values for these fuel categories fluctuate every year, an average is stated here.

Other Sector = Residence, Service etc.

Source: Swedish EPA and Statistics Sweden

### Mobile Sources

Emission Factors 2000 (ton/TJ)

| Fuel type | Sector | Subsector | Region | UNFCCC | GJ/M3 | CO2 | SO2 | NOX | NMVOC | CH4 | CO | N2O |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gasoline | Off Road Vehicles and Working Machinery | Other |  | 1A3e | 31,39 | 72,60 | 0,00 | 1,62 | 0,44 | 0,02 | 3,95 | 0,05 |
| Gasoline | Off Road Vehicles and Working Machinery | Forestry |  | 1A4c | 31,39 | 72,60 | 0,00 | 2,52 | 0,42 | 0,02 | 2,79 | 0,08 |
| Gasoline | Off Road Vehicles and Working Machinery | Farming |  | 1A4c | 31,39 | 72,60 | 0,00 | 1,39 | 0,23 | 0,01 | 0,54 | 0,04 |
| Gasoline | Off Road Vehicles and Working Machinery | Households |  | 1A4b | 31,39 | 72,60 | 0,00 | 0,14 | 3,26 | 0,10 | 18,65 | 0,00 |
| Gasoline | Off Road Vehicles and Working Machinery | Industry |  | 1A2f | 31,39 | 72,60 | 0,00 | 1,34 | 0,22 | 0,01 | 0,81 | 0,04 |
| Gasoline | Navigation/Shipping | Small boats |  | 1A3d | 31,40 | 72,60 | 0,00 | 0,10 | 9,39 | 0,10 | 21,18 | 0,00 |
| Gasoline | Road Traffic | Truck –3,5t | Country | 1A3b | 31,40 | 72,60 | 0,00 | 0,38 | 0,20 | 0,03 | 1,48 | 0,01 |
| Gasoline | Road Traffic | Moped | Country | 1A3b | 31,40 | 72,60 | 0,00 | 0,07 | 10,96 | 1,22 | 16,67 | 0,00 |
| Gasoline | Road Traffic | Motorcycle | Country | 1A3b | 31,40 | 72,60 | 0,00 | 0,16 | 2,26 | 0,25 | 11,96 | 0,00 |
| Gasoline | Road Traffic | Passenger Car | Country | 1A3b | 31,40 | 72,60 | 0,00 | 0,31 | 0,25 | 0,04 | 1,57 | 0,01 |
| Gasoline | Road Traffic | Truck –3,5t | Urban | 1A3b | 31,40 | 72,60 | 0,00 | 0,37 | 0,63 | 0,08 | 3,93 | 0,02 |
| Gasoline | Road Traffic | Moped | Urban | 1A3b | 31,40 | 72,60 | 0,00 | 0,09 | 5,51 | 0,61 | 19,08 | 0,00 |
| Gasoline | Road Traffic | Motorcycle | Urban | 1A3b | 31,40 | 72,60 | 0,00 | 0,11 | 2,24 | 0,25 | 7,76 | 0,00 |
| Gasoline | Road Traffic | Passenger Car | Urban | 1A3b | 31,40 | 72,60 | 0,00 | 0,22 | 0,65 | 0,09 | 3,62 | 0,01 |
| Gasoline | Road Traffic | Military |  | 1A5b | 31,40 | 72,60 | 0,00 | 0,46 | 0,34 | 0,04 | 1,92 | 0,01 |
| Gasoline | Navigation/Shipping | Military |  | 1A5b | 31,40 | 72,60 | 0,00 | 0,10 | 9,39 | 0,10 | 21,18 | 0,00 |
| Biogas | Road Traffic | All |  | 1A3b | 0,03 | 56,50 | 0,00 | NE | NE | NE | NE | NE |
| Diesel oil | Off Road Vehicles and Working Machinery | Other |  | 1A3e | 35,22 | 75,30 | 0,00 | 1,62 | 0,44 | 0,02 | 3,95 | 0,05 |
| Diesel oil | Off Road Vehicles and Working Machinery | Forestry |  | 1A4c | 35,22 | 75,30 | 0,00 | 2,52 | 0,42 | 0,02 | 2,79 | 0,08 |
| Diesel oil | Off Road Vehicles and Working Machinery | Farming |  | 1A4c | 35,22 | 75,30 | 0,00 | 1,39 | 0,23 | 0,01 | 0,54 | 0,04 |
| Diesel oil | Off Road Vehicles and Working Machinery | Households |  | 1A4b | 35,22 | 75,30 | 0,00 | 0,14 | 3,26 | 0,10 | 18,65 | 0,00 |
| Diesel oil | Off Road Vehicles and Working Machinery | Industry |  | 1A2f | 35,22 | 75,30 | 0,00 | 1,34 | 0,22 | 0,01 | 0,81 | 0,04 |
| Diesel oil | Railways | Railways |  | 1A3c | 35,39 | 74,71 | 0,00 | 1,53 | 0,08 | 0,00 | 0,31 | 0,03 |
| Diesel oil | Navigation/Shipping | Domestic |  | 1A3d | 35,59 | 75,30 | 0,00 | 0,98 | 0,11 | 0,00 | 0,25 | 0,03 |
| Diesel oil | Navigation/Shipping | Bunkers |  | 1C | 35,59 | 75,30 | 0,00 | 0,98 | 0,11 | 0,00 | 0,25 | 0,03 |
| Diesel oil | Road Traffic | Bus | Country | 1A3b | 35,59 | 75,30 | 0,00 | 0,51 | 0,04 | 0,00 | 0,11 | 0,00 |
| Diesel oil | Road Traffic | Truck 16t- | Country | 1A3b | 35,59 | 75,30 | 0,00 | 1,08 | 0,03 | 0,00 | 0,12 | 0,00 |
| Diesel oil | Road Traffic | Truck 3,5-16t | Country | 1A3b | 35,59 | 75,30 | 0,00 | 0,88 | 0,11 | 0,01 | 0,24 | 0,01 |
| Fuel type | Sector | Subsector | Region | UNFCCC | GJ/M3 | CO2 | SO2 | NOX | NMVOC | CH4 | CO | N2O |
| Diesel oil | Road Traffic | Truck -3,5t | Country | 1A3b | 35,59 | 75,30 | 0,00 | 0,38 | 0,20 | 0,02 | 1,48 | 0,01 |
| Diesel oil | Road Traffic | Passenger Car | Country | 1A3b | 35,59 | 75,30 | 0,00 | 0,31 | 0,26 | 0,03 | 1,57 | 0,00 |
| Diesel oil | Road Traffic | Bus | Urban | 1A3b | 35,59 | 75,30 | 0,00 | 1,71 | 0,12 | 0,01 | 0,36 | 0,01 |
| Diesel oil | Road Traffic | Truck 16t- | Urban | 1A3b | 35,59 | 75,30 | 0,00 | 0,70 | 0,03 | 0,00 | 0,10 | 0,00 |
| Diesel oil | Road Traffic | Truck 3,5-16t | Urban | 1A3b | 35,59 | 75,30 | 0,00 | 1,22 | 0,15 | 0,02 | 0,35 | 0,00 |
| Diesel oil | Road Traffic | Truck -3,5t | Urban | 1A3b | 35,59 | 75,30 | 0,00 | 0,37 | 0,64 | 0,07 | 3,93 | 0,01 |
| Diesel oil | Road Traffic | Passenger Car | Urban | 1A3b | 35,59 | 75,30 | 0,00 | 0,22 | 0,66 | 0,07 | 3,62 | 0,00 |
| Diesel oil | Road Traffic | Military |  | 1A5b | 35,59 | 75,30 | 0,00 | 0,46 | 0,35 | 0,04 | 1,92 | 0,00 |
| Diesel oil | Navigation/Shipping | Military |  | 1A5b | 35,59 | 75,30 | 0,00 | 0,98 | 0,11 | 0,00 | 0,25 | 0,03 |
| Diesel oil | Navigation/Shipping | Domestic |  | 1A3d | 35,59 | 75,30 | 0,05 | 1,66 | 0,06 | 0,01 | 0,17 | 0,00 |
| Diesel oil | Navigation/Shipping | Bunkers |  | 1C | 35,59 | 75,30 | 0,05 | 1,66 | 0,06 | 0,01 | 0,17 | 0,00 |
| Residual Fuel Oil | Navigation/Shipping | Domestic |  | 1A3d | 38,94 | 76,20 | 0,24 | 1,66 | 0,06 | 0,01 | 0,17 | 0,00 |
| Residual Fuel Oil | Navigation/Shipping | Bunkers |  | 1C | 38,94 | 76,20 | 0,24 | 1,66 | 0,06 | 0,01 | 0,17 | 0,00 |
| Etanol | Road Traffic | All |  | 1A3b | 22,46 | 56,50 | 0,01 | NE | NE | NE | NE | NE |
| Aviation Gasoline | Aviation | Domestic | LTO | 1A3a | 32,70 | 72,30 | 0,02 | 0,24 | 0,05 | 0,01 | 0,37 | 0,01 |
| Aviation Gasoline | Aviation | Domestic | Cruise | 1A3a | 32,70 | 72,30 | 0,02 | 0,33 | 0,05 | 0,00 | 0,42 | 0,00 |
| Aviation Gasoline | Aviation | Bunkers | LTO | 1C | 32,70 | 72,30 | 0,02 | 0,29 | 0,04 | 0,00 | 0,22 | 0,02 |
| Aviation Gasoline | Aviation | Bunkers | Cruise | 1C | 32,70 | 72,30 | 0,02 | 0,35 | 0,02 | 0,00 | 0,18 | 0,00 |
| Aviation Gasoline | Aviation | Military |  | 1A5b | 32,70 | 72,30 | 0,02 | 0,34 | 0,03 | 0,00 | 0,24 | 0,00 |
| Kerosene | Aviation | Domestic | LTO | 1A3a | 34,50 | 73,10 | 0,02 | 0,23 | 0,05 | 0,01 | 0,35 | 0,01 |
| Kerosene | Aviation | Domestic | Cruise | 1A3a | 34,50 | 73,10 | 0,02 | 0,31 | 0,05 | 0,00 | 0,40 | 0,00 |
| Kerosene | Aviation | Bunkers | LTO | 1C | 34,50 | 73,10 | 0,02 | 0,28 | 0,04 | 0,00 | 0,21 | 0,02 |
| Kerosene | Aviation | Bunkers | Cruise | 1C | 34,50 | 73,10 | 0,02 | 0,33 | 0,02 | 0,00 | 0,17 | 0,00 |
| Kerosene | Aviation | Military |  | 1A5b | 34,50 | 73,10 | 0,02 | 0,32 | 0,03 | 0,00 | 0,22 | 0,00 |
| Natural Gas | Road Traffic | Bus |  | 1A3b | 0,03 | 56,50 | 0,00 | NE | NE | NE | NE | NE |
| Natural Gas | Road Traffic | not Bus |  | 1A3b | 0,03 | 56,50 | 0,00 | NE | NE | NE | NE | NE |
| RME | Road Traffic | Military |  | 1A5b | 35,59 | 56,50 | 0,00 | NE | NE | NE | NE | NE |
| RME | Road Traffic | Civil |  | 1A3b | 35,59 | 56,50 | 0,00 | NE | NE | NE | NE | NE |

NE= Not Estimated

## References

- E20SM (Statistical reports), Statistics Sweden, 1990-2000.
Used background data:
* A total survey over industries (purchased fuels), yearly
* A sample survey over industries each quarter (consumed fuels)
* A total survey over energy industries, yearly

- Andersson, Kjell: Rapportering av luftutsläpp till CORINAIR. Swedish EPA, 1994 -1999.

- Miljöpåverkan från fritidsbåtar, fiske- och arbetsfartyg, Report 3993, Swedish EPA and Swedish Maritime Administration 1992

- Kindbom, Karin and Persson, Karin: Kartläggning av emissioner från arbetsfordon och arbetsredskap i Sverige. Report by IVL ordered by Statistics Sweden and the Swedish Environmental Protection agency, 1999.

- 3K engineering AB: Kartläggning av förorenande utsläpp från traktorer, arbetsmaskiner m.m. Report ordered by the Swedish EPA, 1989.

- ÅF-industriteknik AB: Kartläggning av förorenande utsläpp från motordrivna arbetsredskap. Report ordered by the Swedish EPA, 1990.

# Methodology regarding Industrial Processes (including F-gases)

## Industrial Processes, Emissions of CO2

The estimates of CO2-emissions from industrial processes are based on surveys on working unit level and are mainly collected from environmental reports and from direct contact with companies. Estimates of CO2-emissions from industrial processes are calculated on the basis of information about production and use of raw material (e.g. limestone, dolomite, carbon electrodes, coke etc) The methodology is in accordance with the IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories and The IPCC Good practice Guidance. Statistics Sweden has carried out the estimations. These estimates are of good quality.

## Industrial Processes, emissions other than CO2

The compilation of data concerning emissions from industrial processes is mainly based on information in industrial companies legal environmental reports to the environmental authorities. The information on emissions from industrial processes has been compiled for the European Environment Agency’s (EEA) CORINAIR inventory. This data is available for 1990, 1994 and 1995-99. In order to be able to publish information on emission from all industries with industrial processes for all the years 1990-99 Statistics Sweden and IVL Swedish Environmental Research Institute was given the task of filling in missing years and correcting data where necessary. This was done by contacts with manufacturing industries, experts at county administrations and the Swedish EPA, using production statistics and in some cases by expert judgments. For some years interpolation have been done and in some cases data from previous years have been used. This study is being documented at the moment. Data on key sources is considered to be of good quality. Uncertainties are mainly due to uncertainties in the measurements and/or estimations at the reporting companies.

In the iron- and steel sector some SO2 emissions have been underestimated during the 1990´s. This concerns blast furnace combustion of coke, which has not been included during the 1990´s. It also concerns SO2 emissions arising from the S-content in the raw material at sinter plants, which has not been consistently treated during the 1990´s. In the reporting of emissions for the year 2000 these emissions have not been included. In the reporting of emissions for 2001, they will be included, as well as recalculated data from these sources for the period 1990-2000.

In this submission emissions of SO2 from cement industry have been updated 1990-1999 due to better background data.

## Emissions of HFCs, PFCs and SF6

### Overview of submitted data

Actual emissions: Emissions reported have been marked with X, not occurring with NO and not estimated with NE.

Table 1	Overview of submitted actual emission data.

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C, Metal production |  |  |  |  |  |  |  |  |  |  |  |
| C3, Aluminum production | X | X | X | X | X | X | X | X | X | X | X |
| C4, SF6 in magnesium foundries | NO | NO | NO | X | X | X | X | X | X | X | X |
| E, Production of halocarbons and SF6 |  |  |  |  |  |  |  |  |  |  |  |
|  | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| F, Consumption of Halocarbons and SF6 |  |  |  |  |  |  |  |  |  |  |  |
| F1, Refrig. and air cond. equip. | X | X | X | X | X | X | X | X | X | X | X |
| F2, Foam blowing | NO | NO | NO | NO | NO | NO | X | X | X | X | X |
| F3, Fire exting. | NO | NO | NO | NO | NO | NO | NO | X | X | X | X |
| F4, Aerosols/Metered dose inhalers | NE | NE | NE | X | X | X | X | X | X | X | X |
| F5, Solvents | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| F6, Semiconductor manufacture | NE | X | X | X | X | X | X | X | X | X | X |
| F7, Electrical equipment | X | X | X | X | X | X | X | X | X | X | X |
| F8, Other |  |  |  |  |  |  |  |  |  |  |  |
| Jogging shoes | NO | NO | NO | NO | NO | NO | NO | NO | X | X | X |
| Thermopanes | X | X | X | X | X | X | X | X | X | X | X |

Potential emissions: data have been submitted for the years 1995-2000. Potential emissions for earlier years have not been estimated for bulk import and export due to lack of information. Estimates of potential emissions for import and export in products have however been made for the years 1990-1994.

### Method of inventory, emission factors, recalculations and quality of data

As far as possible a Tier 2 approach has been used. In Table 2 the emission factors and activity data used in the calculations of actual emissions are presented. A model has been used for calculating the actual emissions, where the changes in accumulated amounts each year, resulting from additional amounts of HFC, PFC and SF6 imported and used within the country, as well as the decline in accumulated stock caused by exports or emissions from operating systems, have been taken into consideration.

In this submission recalculations of actual emissions have been made for the period 1990-99, mainly in the refrigeration and airconditioning sector. More detailed information has become available, as to changing leakage rates during operation of equipment and concerning installed amount per unit in the 1990´s. Formerly, uniform leakage rates as well as installed amounts for each type of equipment was used for the whole period 1990-99.

Potential emissions for the years 1997-1999 have beeen recalculated since a prior underestimate of the imported amounts of HFC-134a and of HFC-152a was discovered.

Table 2	Typical values on life times of equipment, amounts of chemical per unit and emission factors for different applications of HFCs, PFCs or SF6, used in calculations of actual emissions in Sweden. Numbers in parenthesis were formerly used for the whole period 1990-99. Intervals given indicate changes between 1990-2000 used in the calculations.

|  | Fluorinated substances | Life time | Amount installed /unit, kg | Emissions at manufacturing | Emissions per year during use | Remained in product at disposal | Emissions at disposal$ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C, Metal production |  |  |  |  |  |  |  |
| C3, Aluminum production | PFCs | 1 | ** | 100% | NA | NA | NA |
| C4, SF6 in magnesium foundries | SF6 | 1 | * | 100% | NA | NA | NA |
| F, Consumption of Halocarbons and SF6 |  |  |  |  |  |  |  |
| Household refr. and freezers | HFCs | 20 (10) | 0.1 | 2% | 1% | 90% | 5% |
| Small stationary refrigeration/AC | HFCs | 15 (10) | 5 | IE | 10-5%(3) | 90% | 1% |
| Large stationary refr./freezers/AC | HFCs | 15 | * | IE | 20-7%(7) | 90% | 1% |
| Plug in systems | HFCs | 15 (10) | 1-0.5 (0.625) | 3.5% | 5-2% (3) | 90% | 5% |
| Small portable AC | HFCs | 15 (10) | 1.2-0.8 (0.9) | NO | 7-3% (3) | 90% | 5% (100) |
| Heat pumps | HFCs | 20 (15) | 5-1 (3) | 1% (2) | 10-1%(3) | 90% | 5% |
| Ice machines, drinking water coolers etc. | HFCs | 15 (10) | 0.5-0.2 (0.5) | NO | 3-1% (3) | 90% | 5% (100) |
| Refrigerated transport | HFCs | 10 (14) | 10-6 (6) | 4.5% | 30-10% (7) | 90% | 15% |
| Mobile air-conditioning, lorries | HFCs | 6 | 1.2 | 1% | 15% | 90% | 15% |
| Mobile air-conditioning, cars | HFCs | 11 | 0.8 | 1% | 15% | 90% | 15% |
| Mobile air-conditioning, buses | HFCs | 12 | 7 | 1% (4.5) | 10% | 90% | 15% |
| Foam blowing (XPS) | HFCs | 12-100 | * | ~30% | Declining | $$ | 100% |
| Fire extinguishing | HFCs | 30 | * | 0.5% | 2% (7) | IE | 1% (0.5) |
| Aerosols | HFCs | 2 | * | NE | 50% | 50% | 100% |
| Semiconductor manufacturing | HFCs, PFCs, SF6 | 1 | * | 73% | NA | NA | NA |
| Electrical insulation, GIS manufacturing | SF6 | 35 | * | 13-2% (13-5) | 0.1-1% | # | 0.5% |
| Electrical insulation, other | SF6 | 60 | * | IE | 0.5% | # | 0.5% |
| Barrier gas windows | SF6 | 30 | * | 5-40% | 1% | # | 100% |
| Jogging shoes | SF6 | 8 | * | NO | ~0% | NE | 25% (100) |

* Total amount used in Sweden per year used as a basis for calculations

** By-product in process

$ Mainly influences future actual emissions, in many instances no or very small emissions at disposal have occurred until 2000. The emissions factor applies to the amount of remaining substance.

$$ Calculated according to a declining curve, different for HFC-134a and HFC-152a.

# Estimated lifetime at least 30 years, NE.

An emission inventory, covering the whole period 1990-1999, was performed in 2000 (1) which means that the same method of inventory and of calculation of emissions have been used for each specific sector for the whole period. Estimates for the year 2000 were made in accordance with these methods. Recalculations have been made for some sectors prior to the present submission, as indicated above. The quality of activity data, such as figures of estimated emissions or amount of fluid used in different applications are usually of better quality for the later years than for the earlier years of the inventory.

## Specific source categories

### Aluminum production

In Sweden aluminum production is performed with two different processes, Prebaked and Soderberg, which have substantially differing emission factors. It is therefore not possible to fill in the CRF background data table correctly. The activity data presumably are of good quality for the years 1995-2000, while more general estimates have had to be made for the earlier years. Estimates of emissions, using plant specific emission factors, were for later years obtained from the annual environmental reports from industry to the regulatory authorities. Produced amounts and processes have not changed significantly during the period, why the estimates for the whole period 1990-2000 are judged to be of medium quality.

### SF6 in magnesium foundries

Total amount of SF6 used in the magnesium foundries was used. Estimate should be of high quality.

### Refrigeration and air conditioning equipment

In the inventory subdivision according to type of equipment was made, since the emission factors may differ substantially (Table 2). For some sectors within the group estimates are of high quality, while for others they are of medium or low quality. The sectors contributing the most to the emissions are judged to be of medium quality.

Since the subdivision into different types of equipment was made in the inventory, which seemed to be the most logical in view of obtaining good quality estimates, it is impossible at present to correctly fill in the CRF background data table asking for domestic, commercial and industrial applications. It also seems unwise to try to aggregate the different types of equipment into general activity data for, e.g. commercial use, no matter what type of equipment. Doing so would only introduce more uncertainty.

### Foam blowing

Emission estimates are based only on the production and use of XPS foam. Data was obtained from the producer concerning used amount of chemicals, emissions at production, as well as algorithms to calculate leakage of HFC-134a and HFC 152a during the lifetime of products. These estimates are of high quality. Uncertainties in F2 are the existence of other emissions from foam blowing or products in use in Sweden, which were not estimated.

### Fire extinguishers

Estimates are judged to be of medium to high quality. All imports are registered at the Swedish Chemicals Inspectorate. Uncertainties are mainly associated with the exported amounts, which are relatively large.

### Aerosols/Metered dose inhalers

Emission estimates cover aerosols as well as metered dose inhalers. Estimates are judged to be of low quality concerning general aerosols while estimates for metered dose inhalers, which contribute only to a minor extent to the emissions from this sector, are of high quality.

### Solvents

Emissions from solvents have not been estimated.

### Semiconductor manufacture

Emission estimates are judged to be of medium quality. Cross- references were made between the amounts used as given by the manufacturers and the imported amounts as given in a register at the Swedish Chemicals Inspectorate.

### Electrical equipment

These estimates actually consist of two different parts, production of GIS on the one hand and SF6 installed in distribution systems on the other hand. This poses problems when filling out the Background data tables in the CRF, similarly to the problems described for aluminum production and for refrigeration and air conditioning equipment.

The larger part of annual SF6- emission originates from the manufacture of GIS. These estimates, obtained from industry, are of medium to high quality. Emissions from operating systems, which according to the calculations contribute much less to the emissions, are more uncertain as the figure of the total amount installed in operating systems might be somewhat underestimated.

### Other

The estimated emissions from the use of SF6 in jogging shoes and in barrier gas windows (thermopanes) are judged to be of medium quality.

For jogging shoes a more or less rough estimate have been made, but the contributions to the emissions are rather small. For windows, the figures of the amount of SF6 used at manufacture is of high quality, while the estimated emission factors at manufacture, which were estimated by the manufacturers on the basis of experience, are of more uncertain quality.

## References

Kindbom, K. Haeger Eugensson, M. Persson, K. (2001), Kartläggning och beräkning av potentiella och faktiska utsläpp av HFC, FC och SF6 i Sverige. IVL B-1428

# Methodology regarding solvent and other product use

The emission estimates for the 1990´s originate from data reported to the CORINAIR inventory. Emission estimates are based on results from production statistics, together with national studies as well as expert judgments of relevance to national conditions and for large point sources on data collected from the annual environmental reports from industries to the regulatory authorities. In this submission the emission estimates for 1998 have been used for all the years 1990-2000, which is a conservative choice. This has been done since the method is not documented well enough. The data is still considered to be quite representative for Swedish conditions. A study will be carried out in the near future to update the methodology used as well as activity data and emission factors.

# Methodology regarding Agriculture

## Introduction

This is a methodological description of the agricultural sector of the Swedish Greenhouse Gas Emissions Inventories for the years1990-2000. For every emission source, there is a description of the activity data and methodology used. An overview of used emission factors is included as an appendix.

The methodology used is the “Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories” (IPCC 1997). Activity data is mainly based on official Swedish statistics and are essentially the same as in the last reported inventories. Interpolation/extrapolation has been used for the years where no high quality data on stable periods and waste management systems, etc. are available.

In order to improve the quality of the agricultural sector of green house gas inventories, Dr. Åsa Kasimir Klemedtsson has carried out a study, not yet published, on emission factors for N2O from agricultural fields. The study takes the form of a literature study on documented N2O-emission measurements carried out in Northern Europe and Canada, in some cases by the author herself. Changes in the emission factors are suggested where enough data are available. The following table provides a comparison between the IPCC default factors and the new suggested factors. The suggested changes have been carried out in the recent inventory, which also includes a background emission from mineral soils that is considered as anthropogenic.

| Categories | IPCC’s EF | Suggested EF |
| --- | --- | --- |
| Direct Emission |  |  |
| Mineral fertilizer | 1.25 % of N | 0.8 % of N |
| Manure | 1.25 % of N | 4.3 % of N |
| Organic soils | 5 kg N2O-N ha-1 | 1-6 kg N2O-N ha-1 |
| Indirect Emission |  |  |
| Atmospheric deposition | 1% of N deposition | 0.2% |
| N-leaching and run-off | 2.5% of N leaching | 0.25% |
| Other (specify) |  |  |
| Background emission from mineral soils | .. | 0.5 kg N2O-N ha-1 |
| From an unpublished report by Dr. Åsa Kasimir Klemedtsson |  |  |

The rational for applying the suggested emission factors is that the referred studies in Dr. Kasimir Klemedtsson's report are not considered by IPCC 1996; furthermore, they were carried out in a climatic region similar or identical to the Swedish one. The quality of the research when generalized to a whole country is not analysed. There are no estimated confidence intervals, but there is presumably a great variability in the estimates.

Another adjustment concerns N2O-emissions from pasture excretion, which we adjust to 1 per cent N2O-N/kg nitrogen excreted for all animal groups, except for Cattle, of which 80 per cent graze on highly productive grassland (which gives a weighted emission factor of 1.8 per cent). Very few data are available, but we find support from the Revised 1996 IPCC Guidelines, page 4.97, which states that nitrogen losses as N2O probably are lower in colder climates such as Sweden.

Emission factors on methane from enteric fermentation for cattle are at present under review; meanwhile, the old ones are still in use.

## Livestock Groups and Subgroups

In the following table, all the livestock subgroups used in the calculations are presented. As indicated, the list is now completed with reindeer, which contribute to food production and are considered a new animal group in the inventory. The group Fur Animals is a minor contributor to green house gas emissions, and it is not included in the inventory due to lack of well-founded emission factors.

| Categories according to IPCC 1996 revised Guide Lines | Sub-categories Enteric Fermentation | Sub-categories Methane from Manure management | Sub-categories N2O from Manure management | Sub-categories N2O from Grazing animals | References |
| --- | --- | --- | --- | --- | --- |
| Dairy Cattle | Dairy Cattle | Dairy Cattle | Dairy Cattle | Dairy Cattle | LBR |
| Non-Dairy Cattle | Beef cows | Beef cows | Beef cows | Beef cows | “ |
|  | Growing animals (12-24 months) | Growing animals (12-24 months) | Growing animals (12-24 months) | Growing animals (12-24 months) | “ |
|  | Calves > 6 months | Calves > 6 months | Calves > 6 months | Calves > 6 months | “ (*) |
|  | Calves < 6 months | Calves < 6 months | Calves < 6 months | Calves < 6 months | “ (*) |
| Swine | Boars | Sows | Sows | Sows | “ |
|  | Sows with farrow | Boars | Boars | Boars | “ |
|  |  | Pigs for meat production | Pigs for meat production | Pigs for meat production | “ |
|  |  | Piglets | Piglets | Piglets | “ |
| Sheep | Sheep | Sheep | Sheep | Sheep | “ |
|  |  |  | Lambs | Lambs | “ |
| Goats | Goats | Goats | Goats | Goats | “ |
|  |  |  | Kids | Kids | “ |
| Horses | Horses | Horses | Horses | Horses | SCB |
| Poultry | Poultry | Poultry | Laying hens |  | LBR |
|  |  |  | Chickens |  | “ |
|  |  |  | Slaughter Chickens |  | Svensk Fågel |
| Other | Reindeer |  |  | Reindeer | Sametinget |

(*) The age distribution of calves is accomplished by using standard values.

Lantbruksregistret, LBR, is the register of holdings in agriculture and forestry (the farm register) and provides the main basis for the agricultural statistics in Sweden. The register is administrated by the Swedish Board of Agriculture and Statistics Sweden (SCB), and provides yearly information on the total number of animals of different categories on Swedish farms. The information on livestock refers to 11 June for the respective year, and is presumably equivalent to a one-year average. Most of the information on livestock numbers comes from the LBR. Distribution of calves (older respectively younger than 6 months) is model assisted, that is 60 % is assumed to be younger than 6 months, and the rest is assumed to be older than 6 months.

In Sweden, the number of horses on farms is about 80 000. The total numbers of horses is estimated to be about 300 000, according to a survey carried out by Statistics Sweden (SCB) in 2000. This larger number is used over all the years in the calculations, since it better reflects the actual situation.

Since the number of slaughter chickens (mean number of chickens kept during the year) is known to be underestimated by LBR, statistics from Svensk Fågel (Swedish Poultry Meat Association) are used.

Used population sizes are put together in the following table:

| Population Size (1000 head) used in the calculations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Animal Subcategories | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| Dairy Cattle | Dairy Cattle | 576 | 528 | 526 | 525 | 509 | 482 | 466 | 468 | 449 | 449 | 428 |
| Non-Dairy Cattle | Beef cows | 75 | 98 | 136 | 154 | 165 | 157 | 164 | 169 | 170 | 165 | 167 |
|  | Growing animals (12-24 months) | 543 | 543 | 565 | 549 | 561 | 596 | 617 | 614 | 611 | 600 | 589 |
|  | Calves > 6 months | 210 | 215 | 219 | 232 | 237 | 217 | 217 | 212 | 204 | 200 | 200 |
|  | Calves < 6 months | 314 | 322 | 329 | 349 | 355 | 325 | 326 | 318 | 305 | 299 | 300 |
| Swine | Sows | 221 | 219 | 225 | 241 | 241 | 237 | 273 | 269 | 255 | 220 | 202 |
|  | Boars | 9 | 8 | 8 | 8 | 8 | 8 | 7 | 6 | 5 | 4 | 4 |
|  | Pigs for meat production | 1025 | 993 | 1017 | 941 | 1264 | 1300 | 1303 | 1313 | 1293 | 1239 | 1146 |
|  | Piglets | 1009 | 982 | 1029 | 1087 | 815 | 768 | 765 | 764 | 733 | 651 | 566 |
| Sheep | Sheep | 162 | 168 | 180 | 189 | 196 | 195 | 203 | 195 | 187 | 194 | 198 |
|  | Lambs | 244 | 251 | 267 | 282 | 288 | 266 | 266 | 247 | 234 | 244 | 234 |
| Goats | Goats | 3 | 3 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
|  | Kids | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Horses | Horses | 300 | 300 | 300 | 300 | 300 | 300 | 300 | 300 | 300 | 300 | 300 |
| Poultry | Laying hens | 2200 | 2600 | 2200 | 1900 | 2200 | 1800 | 2200 | 1900 | 2200 | 2200 | 1700 |
|  | Chickens | 6400 | 6100 | 6100 | 5800 | 5900 | 6100 | 5700 | 2700 | 5400 | 5600 | 5700 |
|  | Slaughter Chickens | 6600 | 7000 | 7600 | 7600 | 8200 | 8500 | 8700 | 9400 | 9400 | 9400 | 9500 |
| Other | Reindeer | 271 | 271 | 271 | 280 | 284 | 253 | 241 | 239 | 227 | 227 | 221 |

## Methane from Enteric Fermentation

The emission factors used are collected in the following table:

| Livestock subgroups | Method | Kg CH4/ head/year |
| --- | --- | --- |
| Dairy Cattle | National | 154 |
| Beef cows | National | 106 |
| Growing animals (12-24 months) | National | 71 |
| Calves > 6 months | National | 10 |
| Calves < 6 months | National | 0 |
| Boars | Tier 1 | IPCC-default |
| Sows with farrow | Tier 1 | IPCC-default |
| Sheep | Tier 1 | IPCC-default |
| Goats | Tier 1 | IPCC-default |
| Horses | Tier 1 | IPCC-default |
| Poultry |  | No fermentation assumed |
| Reindeer | National | 14.5 |

The national estimates of produced methane are proposed in a report from the Swedish EPA, and are meant to reflect national conditions, and have been used for the reporting of emission inventories to the UNFCCC. However, the method was recently found to be not sufficiently documented for the reporting needs and calculations concerning cattle are currently under review. Preliminary results from this project show that the overall estimated emission level from cattle as reported is justifiable, but the distribution between the subgroups may be skewed. Until the new information is completely available, the emission estimates are still calculated with national values used in previous reports.

IPCC-default values are used for the less significant animal groups, where priority has not been given to developing the national emission factor. For reindeer, where IPCC does not provide default values, the suggested national value from the Swedish EPA is used for the calculations.

## Methane from Manure Management (including excretion on pasture range)

The Tier 2-methodology is applied to cattle and swine; the Tier 1 is used for other animal groups. An overview of the data used in the Tier 2-methodology is provided below. In the report, methane from excretion on pasture range is included in this section.

The Boi- and MCF-factors used are the IPCC-default values, since no national factors are estimated. For all manure, 92 per cent VS/dry matter is assumed, as implied from the IPCC-Reference Manual, table B-1, in the case of cattle.

The Swedish Board of Agriculture provides data on Manure Production from cattle and swine. The data are based on extensive experiments, but not on national surveys, which means that the results may not necessarily be representative for the nation.

Manure Production from Dairy Cattle is assumed to depend on milk production, and therefore interpolated as in the following table.

| Manure production for Dairy Cattle |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| Manure (kg d m/day) | 5.39 | 5.4 | 5.4 | 5.45 | 5.5 | 5.52 | 5.52 | 5.54 | 5.56 | 5.57 | 5.58 |
| Milk production (kg) | 6842 | 6894 | 6882 | 7248 | 7591 | 7706 | 7694 | 7837 | 8012 | 8086 | 8287 |
| Manure coefficients calculated from Swedish Board of Agriculture, report 1995:10; Milk production is high quality data from Svensk Mjölk and represent a weighted average between low and high producing animals. |  |  |  |  |  |  |  |  |  |  |  |

Manure production from other animal groups are stated in the following table:

| Manure production for other animal groups (same values every year) |  |
| --- | --- |
| Animal groups | Manure (kg dm/day) |
| Beef cows | 2.64 |
| Growing animals (12-24 months) | 2.6 |
| Calves > 6 months | 1.12 |
| Calves < 6 months | 0.69 |
| Sows | 0.74 |
| Boars | 0.52 |
| Pigs for meat production | 0.42 |
| Piglets | 0.05 |
| Reference of source: Swedish Board of Agriculture, reports 1993:20 (on Swine) and 1995:10 (on Cattle). |  |

Information on waste management systems is collected from the surveys published in the Statistical report Na/Mi 30 SM “Use of fertilizers and animal manure in agriculture” (Statistics Sweden). There are reports available for every second year, and interpolated values are used for the intermediate years. Hence, this is high quality data, representative for the nation.

| Waste Management Systems, fraction of solid storage |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Animal groups | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| Dairy Cattle | 70% | 70% | 66% | 62% | 61% | 59% | 58% | 56% | 53% | 49% | 49% |
| Beef cows | 70% | 70% | 66% | 62% | 57% | 51% | 62% | 72% | 73% | 74% | 74% |
| Growing animals (12-24 months) | 70% | 70% | 66% | 62% | 61% | 59% | 66% | 72% | 73% | 74% | 74% |
| Calves > 6 months | 70% | 70% | 66% | 62% | 61% | 59% | 66% | 72% | 73% | 74% | 74% |
| Calves < 6 months | 70% | 70% | 66% | 62% | 61% | 59% | 66% | 72% | 73% | 74% | 74% |
| Sows | 56% | 56% | 49% | 42% | 40% | 37% | 76% | 76% | 75% | 74% | 74% |
| Boars | 56% | 56% | 49% | 42% | 40% | 37% | 76% | 76% | 75% | 74% | 74% |
| Pigs for meat production | 56% | 56% | 49% | 42% | 40% | 37% | 20% | 20% | 23% | 25% | 25% |
| Piglets | 56% | 56% | 49% | 42% | 40% | 37% | 20% | 20% | 23% | 25% | 25% |
| Values for 1991, 1993, 1995, 1997 and 1999 from Statistics Sweden, other values are interpolated/extrapolated. |  |  |  |  |  |  |  |  |  |  |  |

Stable periods for cattle are collected from the survey noted above (this information is available for 1997 and 1999), which means high quality data. Until 1994, standard values from LBS, SLU (Swedish University of Agricultural Sciences) are used, which probably means a somewhat lower quality, but the level is comparable to the estimated results of the surveys of 1997 and 1999.

| Stable Periods for cattle, days |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Animal groups | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| Dairy Cattle | 252 | 252 | 252 | 252 | 252 | 225 | 225 | 225 | 225 | 225 | 225 |
| Beef cows | 184 | 184 | 184 | 184 | 184 | 175 | 175 | 175 | 169 | 164 | 164 |
| Growing animals (12-24 months) | 246 | 246 | 246 | 246 | 246 | 250 | 250 | 250 | 241 | 232 | 232 |
| Calves > 6 months | 308 | 308 | 308 | 308 | 308 | 205 | 205 | 205 | 200 | 196 | 196 |
| Calves < 6 months | 308 | 308 | 308 | 308 | 308 | 205 | 205 | 205 | 200 | 196 | 196 |
| Values for 1995, 1997 and 1999 from Statistics Sweden, other values are standard values, or interpolated /extrapolated. |  |  |  |  |  |  |  |  |  |  |  |

## N2O from manure management

The methodology used is in the 1996 Revised IPCC-methodology with national activity data and IPCC default emission factors applied.

N-production is provided by SLU, (Swedish University of Agricultural Sciences) and depends on extensive experiments, which might produce a loss of quality when generalized to a national level. The values on N-production per animal in each animal group are stated in the table below. Stable period and manure management systems are the same as in chapter 4.4 (methane calculations).

| N-production from different animal groups, kg/year |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Animal groups | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| Dairy Cattle | 95 | 95 | 95 | 98 | 98 | 102 | 102 | 118 | 118 | 118 | 118 |
| Beef cows | 66 | 66 | 66 | 66 | 66 | 66 | 66 | 63 | 63 | 63 | 63 |
| Growing animals (12-24 months) | 45 | 45 | 45 | 45 | 45 | 46.2 | 46.2 | 47 | 47 | 47 | 47 |
| Calves > 6 months | 32.5 | 32.5 | 32.5 | 32.5 | 32.5 | 32.5 | 32.5 | 28 | 28 | 28 | 28 |
| Calves < 6 months | 32.5 | 32.5 | 32.5 | 32.5 | 32.5 | 32.41 | 32.41 | 28 | 28 | 28 | 28 |
| Sows | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| Boars | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| Pigs for meat production | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9.5 | 9.5 | 9.5 | 9.5 |
| Piglets | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sheep | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 13 | 13 | 13 | 13 |
| Lambs | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Goats | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 13 | 13 | 13 | 13 |
| Kids | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Horses | 55 | 55 | 55 | 55 | 55 | 55 | 55 | 50 | 50 | 50 | 50 |
| Laying hens | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 640 | 640 | 640 | 640 |
| Chickens | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 280 | 280 | 280 | 280 |
| Slaughter Chickens | 220 | 220 | 220 | 220 | 220 | 220 | 220 | 290 | 290 | 290 | 290 |
| Provided by SLU, (Swedish University of Agricultural Sciences). |  |  |  |  |  |  |  |  |  |  |  |

## Direct emissions of N2O from agricultural soils

### N2O from mineral fertilizers

The sales of fertilizers (including total N-quantity) in Sweden is published annually by Statistics Sweden (Mi 30 SM 9901), and the national estimate is considered to be accurate.

| Total N-content in sold fertilizers in Sweden (tons) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| 224 500 | 208 600 | 178 400 | 207 200 | 216 400 | 198 300 | 192 300 | 204 600 | 205 600 | 179 200 | 189 400 |
| SCB: Mi 30 SM 9901 |  |  |  |  |  |  |  |  |  |  |

The report “Emissions to air of ammonia in Sweden from agriculture and other anthropogenic sources” (Mi 37 SM 9901 + 0001) contains information for 1995, 1997 and 1999 on ammonium emission from mineral fertilizers in Sweden; these are calculated from information on N-content from retailers and ammonium emission from CORINAIR. From this information, the proportion of emitted N-content in fertilizers is given in the following table. For the years before 1995, the value for 1995 is used, and the values are interpolated for 1996 and 1998.

| Year | Ammonium emission (tons of ammonium-N) | N-content in sold fertilizers (tons) | Proportion of N-content emitted |
| --- | --- | --- | --- |
| 1995 | 1 818 | 198 300 | 0.0092 |
| 1997 | 1 612 | 204 600 | 0.0079 |
| 1999 | 1 405 | 179 200 | 0.0078 |
| SCB: Mi 37 SM 9901 + 0001 |  |  |  |

Emission factor is national, 0.8 per cent N2O-N of N-supply (Kasimir Klemedtsson).

### N2O from animal manure

IPCC default methodology is used with estimation of N-content in manure (see chapter 1.4) and a national estimation of ammonium-N-emission (see chapter 1.7.1).

A national emission factor is used, 2.5 per cent emission of N-supply (Kasimir Klemedtsson).

### N2O from N-fixing crops

IPCC default methodology is used with national value on N-fixation (fraction of yield = 0.047), which is the same assumption as in the national “Nitrogen and phosphorus balances in arable land and agricultural sector in Sweden” (Na/Mi 40 SM from Statistics Sweden). No estimation of uncertainty is given.

Total production of dry pulses, etc. is given by multiplying standard yield and total area, according to LBR.

Emission factor used is IPCC default, 1.25 per cent of N-supply. There is not enough available data to justify a revision.

### N2O from hayfields, etc.

The methodology used is a national methodology, where available research is used, together with the IPCC default emission factor, since there is insufficient data to justify a revision. The method accounts for regional differences. The formula is stated as

Emissions=CountyareaCounty∗NfixingFactorCounty∗EF,

where area is the area of leys in each county, and NfixingFactor is the corresponding N-fixing factor in kg/ha. These values are calculated by using the computer program NPK-FLO developed by SLU (Swedish University of Agricultural Sciences). Areas of leys are from LBR, the register of holdings in agriculture and forestry.

| Information on N-fixation (kg/ha) in ley, By counties |  |  |  |
| --- | --- | --- | --- |
| County | Value 1990-95 | Value 1996-99 | Value 2000 |
| Stockholm | 26.1 | 26.6 | 23.7 |
| Uppsala | 31.7 | 32.4 | 28.8 |
| Södermanland | 25.9 | 26.4 | 27.5 |
| Östergötland | 37.3 | 37.8 | 30.3 |
| Jönköping | 35.0 | 35.5 | 30.7 |
| Kronoberg | 32.2 | 32.5 | 30.6 |
| Kalmar | 30.8 | 30.9 | 28.4 |
| Gotland | 28.2 | 28.7 | 34.9 |
| Blekinge | 40.0 | 33.7 | 35.3 |
| Former Malmöhus | 32.3 | 36.1 | 34.3 |
| Former Kristianstad | 39.3 | 36.1 | 33.1 |
| Halland | 39.0 | 39.2 | 36.3 |
| Former Göteborgs- och Bohus | 34.9 | 31.9 | 27.1 |
| Former Älvsborg | 31.2 | 31.6 | 25.0 |
| Former Skaraborg | 39.8 | 40.5 | 33.1 |
| Värmland | 30.0 | 30.4 | 25.6 |
| Örebro | 32.7 | 29.2 | 31.5 |
| Västmanland | 27.6 | 32.9 | 32.1 |
| Dalarna | 31.2 | 27.6 | 32.7 |
| Gävleborg | 31.1 | 31.5 | 28.6 |
| Västernorrland | 28.9 | 28.9 | 27.9 |
| Jämtland | 30.7 | 30.6 | 28.0 |
| Västerbotten | 29.6 | 30.2 | 28.4 |
| Norrbotten | 31.3 | 32.0 | 30.6 |
| Source: Calculations made by the computer program NPK-FLO, SLU (Swedish University of Agricultural Sciences) |  |  |  |

Used area of lay on a national level is stated in the table below.

| Year | Area of Ley (ha) |
| --- | --- |
| 1990 | 778 000 |
| 1991 | 706 000 |
| 1992 | 717 000 |
| 1993 | 779 000 |
| 1994 | 788 000 |
| 1995 | 790 000 |
| 1996 | 780 000 |
| 1997 | 780 000 |
| 1998 | 773 000 |
| 1999 | 790 000 |
| 2000(*) | 790 000 |

(*) For 2000 the Farm register does not differentiate between pasture and Lay. Until we get a better estimate, the number for 1999 is kept.

### N2O from Crop Residue

The methodology used is a modified IPCC-methodology, where IPCC default emission factors are used. National data on removed residues and other parameters are used, which give a more precise estimation of the nitrogen supply in Swedish soils. The formula is stated:

emission=cropyieldcropareacrop∗FracResiduescropFracNcrop(1−ResiduesRemovedcrop)EF,

where yield is standard yield (LBR), FracResidues is the crop residues as a fraction of the harvest, FracN is the fraction of nitrogen in crop residues, and ResiduesRemoved is the fraction of crop residues that is removed.

| Crop | Fraction of crop residues removed (ResiduesRemoved) | Fraction of N in crop residues, per cent (FracN) | Fraction residues in relation to harvest, (FracResidues) |
| --- | --- | --- | --- |
| Winter wheat | 0.28 | 0.71 | 1.3 |
| Spring wheat | 0.25 | 0.82 | 1.1 |
| Winter rye | 0.3 | 0.82 | 1.4 |
| Winter barley | 0.61 | 0.94 | 0.9 |
| Spring barley | 0.4 | 0.94 | 0.9 |
| Oats | 0.33 | 0.82 | 1.2 |
| Mixed crops | 0.59 | 0.88 | 1 |
| Tritcale | 0.37 | 0.82 | 1.3 |
| Sugar beets | 0.09 | 2.25 | 0.75 |
| Winter rape | 0.03 | 1.07 | 1.8 |
| Spring rape | 0.03 | 1.07 | 1.8 |
| Winter turnip rape | 0.03 | 1.07 | 1.8 |
| Spring turnip rape | 0.03 | 1.07 | 1.8 |
| Table potatoes | 0 | 3.25 | 0.75 |
| Potatoes for starch prod. | 0 | 3.25 | 0.75 |
| Lay. without clover | 0 | 1.30 | 0.25 |
| Lay for seed, no clover | 0.5 | 1.30 | 0.94 |
| Green forage | 0 | 1.30 | 0.25 |
| Pasture ground | 0 | 1.30 | 0.25 |
| Peas | 0.03 | 1.86 | 0.8 |
| Peas for conservation | 0 | 1.86 | 0.8 |
| Peas for fodder | 0.02 | 1.86 | 0.8 |
| Lay for seed, clover | 0.5 | 1.86 | 0.94 |
| Green forage, clover | 0 | 1.86 | 0.25 |
| Lay, with clover | 0 | 1.86 | 0.25 |
| References: | SCB, Mi 63 SM 9901 | Claesson & Steineck 1991; Statistics Sweden | Claesson & Steineck 1991; Statistics Sweden |

Areas of different crops, according to LBR, used in the calculations are stated in the following table. Concerning lay and green forage, areas are considered to have “clover” or “no clover” according partly to standard values and partly to information from Swedish University of Agricultural Sciences.

| Areas of different crops used in the calculations (Hectare) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| Winter wheat | 320120 | 225330 | 233678 | 271818 | 214095 | 222304 | 292170 | 299594 | 359024 | 209641 | 353201 |
| Spring wheat | 29595 | 33387 | 36647 | 32581 | 39722 | 39076 | 42392 | 44588 | 39021 | 65777 | 48364 |
| Winter rye | 73460 | 43239 | 34597 | 46390 | 38957 | 39693 | 33558 | 29416 | 34617 | 24507 | 34533 |
| Winter barley (*) |  |  |  |  | 29436 |  | 22061 | 15272 | 15949 | 11883 | 12997 |
| Spring barley | 492027 | 490896 | 454097 | 420437 | 443489 | 453375 | 446503 | 467628 | 429011 | 470104 | 398227 |
| Oats | 387823 | 364272 | 360859 | 321961 | 341415 | 278322 | 283588 | 315465 | 311467 | 305658 | 295544 |
| Mixed crops (**) | 32628 | 40337 | 47420 | 35330 | 25421 | 27124 | 34230 | 30247 | 26972 | 33022 | 45328 |
| Tritcale |  |  |  |  |  |  | 61694 | 66473 | 66751 | 32586 | 40728 |
| Sugar beets | 38502 | 47963 | 51287 | 51287 | 53353 | 57518 | 59223 | 60459 | 58737 | 59881 | 55484 |
| Winter rape | 84598 | 75724 | 51364 | 74460 | 46035 | 56084 | 21737 | 22888 | 23159 | 19626 | 24870 |
| Spring rape | 44203 | 41046 | 56519 | 46203 | 53033 | 23311 | 18976 | 19475 | 16705 | 31273 | 12112 |
| Winter turnip rape | 9068 | 8089 | 3145 | 2455 | 1746 | 1587 | 811 | 1787 | 1470 | 1206 | 1395 |
| Spring turnip rape | 30035 | 26362 | 26366 | 22370 | 27647 | 23661 | 23869 | 19432 | 13238 | 23784 | 9791 |
| Table potatoes | 27305 | 28269 | 30414 | 27815 | 25449 | 27630 | 27577 | 26732 | 25133 | 24422 | 23610 |
| Potatoes for starch prod. | 8866 | 8807 | 8791 | 8469 | 7539 | 7371 | 9060 | 9081 | 8567 | 8391 | 9293 |
| Lay. without clover | 621362 | 627199 | 633036 | 638872 | 647450 | 656028 | 648824 | 645895 | 641774 | 657479 | 661245 |
| Lay for seed, no clover | 9183 | 8360 | 7538 | 6715 | 6740 | 6765 | 6794 | 7325 | 7795 | 7061 | 7363 |
| Green forage | 22628 | 19481 | 16335 | 13188 | 13398 | 13607 | 12915 | 14177 | 12722 | 12683 | 12683 |
| Pasture ground | 190503 | 190503 | 190503 | 190503 | 233715 | 276927 | 247369 | 234677 | 221418 | 198091 | 198091 |
| Peas (***) | 5357 |  |  |  |  |  | 17713 | 32742 | 49150 | 30053 | 27892 |
| Peas for conservation |  |  |  |  |  |  | 8821 | 9028 | 8524 | 8752 | 8525 |
| Peas for fodder (***) | 27368 | 23327 | 14059 | 8720 | 6598 | 11959 | 690 | 921 | 938 | 872 | 835 |
| Lay for seed, clover | 1570 | 1429 | 1289 | 1148 | 1145 | 1142 | 1060 | 1145 | 1218 | 1104 | 1102 |
| Green forage, clover | 17070 | 14696 | 12323 | 9949 | 10018 | 10088 | 9353 | 10266 | 9213 | 9184 | 9184 |
| Lay, with clover | 106228 | 107226 | 108224 | 109222 | 109985 | 110748 | 101261 | 100937 | 100294 | 102748 | 98982 |
| (*) Some of the years, statistics on Winter barley was included in Spring barley |  |  |  |  |  |  |  |  |  |  |  |
| (**) Some of the years, statistics on Tritcale was included in Mixed crops |  |  |  |  |  |  |  |  |  |  |  |
| (***) Some of the years, statistics on all groups of Peas were aggregated |  |  |  |  |  |  |  |  |  |  |  |

Standard yield, according to LBR, of different crops used in the calculations is stated in the following table.

| Standard yield of different crops used in the calculations, kg/hectare |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| Winter wheat | 4945 | 5040 | 5134 | 5228 | 5276 | 5323 | 5434 | 5505 | 5603 | 5528 | 5479 |
| Spring wheat | 4180 | 4206 | 4232 | 4258 | 4260 | 4262 | 4316 | 4378 | 4268 | 4264 | 4300 |
| Winter rye | 3566 | 3605 | 3645 | 3685 | 3738 | 3792 | 3910 | 3999 | 4259 | 4338 | 4423 |
| Spring barley | 3324 | 3355 | 3385 | 3415 | 3430 | 3445 | 3488 | 3530 | 3516 | 3530 | 4423 |
| Oats | 3286 | 3291 | 3297 | 3302 | 3289 | 3275 | 3300 | 3312 | 3157 | 3154 | 3516 |
| Mixed crops | 2644 | 2658 | 2673 | 2687 | 2688 | 2688 | 2715 | 2737 | 2669 | 2673 | 3109 |
| Tritcale |  |  |  |  |  |  | 4347 | 4404 | 4483 | 4422 | 3118 |
| Sugar beets | 8969 | 9054 | 9140 | 9226 | 9289 | 9352 | 9397 | 9368 | 9337 | 9327 | 5157 |
| Winter rape | 2501 | 2509 | 2518 | 2526 | 2527 | 2527 | 2504 | 2489 | 2440 | 2401 | 9260 |
| Spring rape | 1617 | 1603 | 1589 | 1575 | 1561 | 1547 | 1528 | 1529 | 1462 | 1508 | 2374 |
| Winter turnip rape | 1657 | 1642 | 1626 | 1610 | 1590 | 1569 | 1531 | 1476 | 1386 | 1341 | 1565 |
| Spring turnip rape | 1444 | 1436 | 1429 | 1421 | 1418 | 1415 | 1403 | 1395 | 1292 | 1302 | 1339 |
| Table potatoes | 5839 | 5954 | 6069 | 6184 | 6282 | 6380 | 6363 | 6366 | 6982 | 7120 | 1320 |
| Potatoes for starch prod. | 7209 | 7300 | 7392 | 7483 | 7517 | 7551 | 7530 | 7523 | 7941 | 8133 | 7029 |
| Lay | 5778 | 5810 | 5842 | 5873 | 5909 | 5945 | 6025 | 6085 | 6085 | 6085 | 8080 |
| Lay for seed | 5778 | 5810 | 5842 | 5873 | 5909 | 5945 | 6025 | 6085 | 6085 | 6085 | 6129 |
| Green forage | 4175 | 4175 | 4175 | 4175 | 4175 | 4175 | 4175 | 4175 | 4175 | 4175 | 6129 |
| Pasture ground | 4623 | 4648 | 4673 | 4699 | 4727 | 4756 | 4820 | 4868 | 4868 | 4868 | 4175 |
| Peas | 2327 | 2348 | 2369 | 2391 | 2401 | 2412 | 2441 | 2471 | 2461 | 2471 | 2027 |
| Peas for fodder | 2327 | 2348 | 2369 | 2391 | 2401 | 2412 | 2441 | 2471 | 2461 | 2471 | 2027 |

The quality of the suggested values on N-fraction and fraction of residues is not discussed by Claesson/Steineck. The values constitute however an attempt by scientists to differentiate between different crops, in contrast to the IPCC default values.

### Background emission of N2O due to cultivation of mineral and organic soils

The used methodology is a national methodology, since it considers the background emission from mineral soils as well as organic soils, and the emission factors used are national (Kasimir Klemedtsson). The formula is

Emissions=iareai∗EFi,

where area is area of mineral respective organic soils, and EF is the background emission per hectare. Index i depends on whether the soil is mineral or organic.

The total area of arable land in Sweden for each year is given by LBR, the register of holdings in agriculture and forestry.

There is little data on the area of organic soils in Sweden. A survey made by Statistics Sweden in 1998 gives a rough estimate, i.e. about 9 per cent of Swedish soils are organic. This figure is used to divide the total area of arable land into organic and mineral soils, as shown in the table below.

| Area of arable land in Sweden, hectare |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| Organic soils | 256000 | 251000 | 249000 | 250000 | 250000 | 249000 | 253000 | 252000 | 251000 | 247000 | 244000 |
| Mineral soils | 2589000 | 2539000 | 2519000 | 2530000 | 2530000 | 2518000 | 2558000 | 2547000 | 2533000 | 2500000 | 2462000 |

Approximately 45 per cent of Swedish organic soils in 1997 were cultivated with annual crops, which is a rough estimate. This gives a weighted emission factor for background emission from organic soils of 3.25 kg N2O-N/ha.

| Summary: Emission factors for background emission from agricultural soils (Kasimir Klemedtsson) |  |  |
| --- | --- | --- |
|  | Crop | EF (kg N2O-N/ha) |
| Mineral soils | All crops | 0.5 |
| Organic soils | Annual crops | 6 |
|  | Perennial crops | 1 |

## Animal production - N2O from grazing animals

Calculations are according to IPCC methodology, but the ammonium emission is taken into consideration. This is consistent with the calculation of indirect emission and the national ammonium emission estimation. The emission factor is somewhat modified (explained in the introduction). The model used can be described as

N2Ograzing=TNT∗NexT∗GrazFactorT∗GrazPeriodT∗(1−Amm)∗EFT365,

where NT ’number of animals of type T in the country’, NexT  ‘N-excretion of animals of type T’, GrazFactorT ‘factor for extra N-production during grazing’, GrazPeriodT ‘Grazing Period for animals of type T’, Amm ‘Ammonium-N-emission (fraction)’, EFT ‘Emission factor for grazing manure from animals of type T’.

N-production by sheep is also applied for reindeers, due to lack of data concerning reindeer. The GrazFactorT is 1 for all animals, except for cattle, where the N-production is considered to be 20 per cent higher during grazing compared with the stable period. Elsewhere, the same sources/assumptions are applied, as in the calculations of N in manure (chapter 1.4).

## Indirect emission of N2O

### Deposition

The 1996 Revised IPCC-default methodology is used, but with a national emission factor, 0.2 per cent of emitted nitrogen (Kasimir Klemedtsson). Estimates of nitrogen supply from mineral fertilizer are applied (chapter 1.5.1), as well as estimates for nitrogen supply from animal manure (chapter 1.4). Fraction of nitrogen supply emitted as ammonium-N is from Statistics Sweden, reported in Mi 37 SM, “Emission to air of ammonia in Sweden from agriculture and other anthropogenic sources”. The estimates on ammonium emissions are model-based to a great extent, which includes assumptions with unknown errors.

| Ammonia-N-emission from mineral fertilizer and manure, fraction |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1995 | 1996 | 1997 | 1998 | 1999 |
| Mineral fertilizer | 0.0092 | 0.009 | 0.0079 | 0.0079 | 0.0078 |
| Manure | 0.33 | 0.35 | 0.37 | 0.37 | 0.37 |
| Grazing manure | 0.12 | 0.10 | 0.08 | 0.08 | 0.08 |
| Source: Statistics Sweden, Mi 37 SM, bold values from the reports, other values are interpolated. For every year before 1995, the values of 1995 are applied. |  |  |  |  |  |

### Nitrogen Leaching and Run-off

The methodology used is a national methodology, using data from an extensive national model on nitrogen circulation in the ground. The emission factor is national, 0.25 per cent of leaching (Kasimir Klemedtsson). The formula is:

Emissions=area∗leachfactor∗EF,

where area is the total Swedish area of agricultural soil. Information on leaching of nitrogen from Swedish arable land is available from the SOIL-SOIL-N-model, developed at SLU (Swedish University of Agricultural Sciences; Johnsson, 1990). The value, 27 kg N/ha, is an average value for Swedish arable land calculated in 1994. The uncertainty is not estimated, but the method is considered to be the best available in Sweden. This value is, for each year, multiplied by the total area of arable land in the country.

## Literature references

NV 4144, Report 4144 from the Swedish EPA 1992, “Västhusgasutsläpp från husdjur” (Methane Emissions from Live Stock).

STANK, a model on nitrogen production kept by the Swedish Board of Agriculture.

JV 1993:20, report from Swedish Board of Agriculture on manure production from Swine.

JV 1995:10, report from Swedish Board of Agriculture on manure production from Cattle.

JTI 281: Element balances as a sustainability tool. Workshop in Uppsala March 16-17 2001. Page 35 STANK – the official model for input/output accounting on farm level in Sweden, Janne Linder

Fagerberg B. and Salomon E., The Computer Program NPK-FLO, SLU (Swedish University of Agricultural Sciences) 1992.

Claesson & Steineck 1991, Växtnäring: Hushållning – Miljö.

Johnsson H Nitrogen and Water Dynamics in Arable Soil, Reports and Dissertations 6, SLU 1990

Kasimir Klemedtsson, Report written by Åsa Kasimir Klemedtsson on N2O-emissions from agricultural soils with a literature survey and suggestions on Swedish emission factors. Produced for the Swedish EPA, not published at present.

LBR, the register of holdings in agriculture and forestry (the farm register) provides the main basis for the agricultural statistics in Sweden. Results are published by Statistics Sweden. Results on Standard yields are published in series JO 15 SM, results on use of arable land are published in series J 10 SM, and results on livestock are published in series J 20 SM.

Mi 40 SM 9901, Statistics Sweden, a report on Nitrogen and phosphorus balances in arable land and agricultural sector in Sweden.

Mi 63 SM 9901, Survey from Statistics Sweden on the use of crop residue 1997.

Mi 37 SM, Statistics Sweden, Emission to air of ammonia in Sweden from agriculture and other anthropogenic sources.

Na/Mi 30 SM, Statistics Sweden “Sales of lime for agricultural and horticultural purposes, and for lakes and woodlands”.

Svensk Mjölk, Swedish Dairy Association, www.svenskmjolk.se

Svensk Fågel, Swedish Poultry Meat Association, www.svenskfagel.se

RVF 1996:8, Beskrivning av biologiskt avfall, Svenska Renhållningsverksföreningen

## Overview of emission factors

### Methane

|  | Emission factor | Reference of source | Note |
| --- | --- | --- | --- |
| Enteric Fermentation | kg CH4/animal/yr |  |  |
| Dairy Cattle | 154 | National, NV 4144 |  |
| Non-Dairy Cattle | 49 | National, NV 4144 | 1 |
| Sheep | 8 | IPCC |  |
| Goats | 5 | IPCC |  |
| Horses | 18 | IPCC |  |
| Swine | 1.5 | IPCC |  |
| Reindeer | 14.5 | National, NV 4144 |  |
| Poultry | 0 | No fermentation assumed |  |

| Manure management |  |  |  |
| --- | --- | --- | --- |
| Dairy Cattle - volatile solid (VS) | 2 236.4 kg TS/animal/yr | National - STANK |  |
| Dairy Cattle - B0 | 0.24 m3 CH4/kg VS | IPCC | 2 |
| Dairy Cattle - MFC solid manure | 1 % av B0 | IPCC | 3 |
| Dairy Cattle - MFC liquid manure | 10 % av B0 | IPCC | 3 |
| Dairy Cattle - Emission per animal | 12.4 kg CH4/animal/yr | Calculated - 1998 |  |
| Non-Dairy Cattle - volatile solid (VS) | 700.8 kg TS/animal/yr | National - STANK |  |
| Non-Dairy Cattle - B0 | 0.17 m3 CH4/kg VS | IPCC | 2 |
| Non-Dairy Cattle - MFC solid manure | 1 % av B0 | IPCC | 3 |
| Non-Dairy Cattle - MFC liquid manure | 10 % av B0 | IPCC | 3 |
| Non-Dairy Cattle – emission/animal | 2.23 kg CH4/animal/yr | Calculated - 1998 |  |
| Swine - volatile solids (VS) | 109.5 kg TS/animal/yr | National - STANK |  |
| Swine - B0 | 0.45 m3 CH4/kg VS | IPCC | 2 |
| Swine - MFC solid manure | 1% av B0 | IPCC | 3 |
| Swine - MFC liquid manure | 10 % av B0 | IPCC | 3 |
| Swine - emission per animal | 2.58 kg CH4/animal/yr | Calculated - 1988 |  |
| Sheep - emission | 0.19 kgCH4/animal/yr | IPCC |  |
| Goats - emission | 0.12 " | IPCC |  |
| Horses - emission | 1.40 " | IPCC |  |
| Poultry - emission | 0.08 " | IPCC |  |

### N2O

|  | Emission factor | Reference of source | Note |
| --- | --- | --- | --- |
| Direct emissions from soils | % N2O-N of N-supply |  |  |
| Mineral fertilizer | 0.8 | National |  |
| Manure | 2.5 | National |  |
| Crop residue | 1.25 | IPCC |  |
| N-fixing Crops | 1.25 | IPCC |  |
| N-fixing in hayfields with clover | 1.25 | IPCC |  |
| Animal Excretion – grassland | 2 | IPCC | 6 |
| Animal Excretion – pasture | 1 | National | 7 |
| Background emission due to cultivation | Kg N2O-N/ha/yr |  |  |
| Cultivation of Histosoils | 3.25 | National | 4 |
| Cultivation of Mineral Soils | 0.5 | National | 5 |
| Indirect emissions from soils |  |  |  |
| Deposition of N from Swedish agriculture | 0.2 % of emitted N | National |  |
| Leached nitrogen | 0.25 % of leaching | National |  |
| Waste Management System | % N2O-N of N-supply |  |  |
| Liquid manure | 0.1 | IPCC |  |
| Solid manure | 2 | IPCC |  |

Comments

1) Weighted value – more than one animal category

2) B0 = maximum methane producing capacity for manure

3) MCF = Methane Conversion Factor

4) Weighted value. Grazing land 6 kg N20-N/ha/yr, pasture1 kg N2O-N/ha/yr.

5) Refers to arable land

6) Excreted N from cattle: 80% to pasture, 20% to grazing land.

7) Refers to excretion-N from cattle (120%), sheep/goats (100%), horses (100 %), reindeer = sheep (100 %).

# Methodology regarding Land-Use Change and Forestry

## Forest biomass

About 75 % of the Swedish forest area is considered as managed forest land (23 mill. ha). The definition of managed forest land is “all land which is not preserved or primarily used for other active land-use than forestry, which hosts a potential yield of stem-wood exceeding one m3 per year and hectare”. In general, clear-cutting of forest land with lower productivity is not allowed.

Over 95 % of the managed forest land of Sweden can be considered as boreal forest. The rest is temperate and subalpine forests. The main tree species in the boreal forest area are Pinus sylvestris, Picea abies and Betula spp. In the temperate forest areas are also Fagus sylvatica and Quercus spp. common. Betula pendula, Picea abies and Pinus sylvestris dominate the subalpine mountain forests.

The most common species on managed forest land are Pinus sylvestris (38%), Picea abies (44%) and Betula spp (10%). Most of the forest contains a mixture of these and other species. Regeneration is most often provided through planting a few years after final harvesting. Seed tree regeneration is used on a quarter to a third of the harvested area. Fellings include wood-fuel harvesting made by forest owners on their own land. The following reporting information concerns the so called managed forest, and forest on other land types where harvesting is allowed, and to a limited extent also recently protected land (see below).

Input data for the calculations are found in the National Forest Statistics. The conversion factors used were 0.40-ton dw m-3 for wood density (weighed average for all species) and 0.49 ton C ton dw- -1 for wood carbon content. The average so-called biomass expansion factor used (for other tree compartments than the stem) was 1.22 (see below). For simplicity, the addition for this biomass was added together with additions for stock changes on some other land types as a fixed amount each year (cf Tables 1.2 and 1.3). This practice may be revised in next year’s reporting.

All managed forest land and the forest on other land types where harvesting is allowed is covered by a running national forest inventory (NFI), which produces reliable region means values for stem volumes, growth, fellings and natural dieback every five-year period. The inventory is based on a set of measurements (tree heights, ages and diameters, year-ring widths, etc) at a systematic net of plots of 314 m2. Some of the plots are revisited every fifth year whereas the major number is temporary. About 25 000 plots are visited every five-year period.

A major part of the forest (according to FAO definitions) which is not covered by the regular NFI, is growing in protected areas and on low-productive land, such as wetlands, thin soils and in cold climates, where harvesting is not allowed.

The development of the C stock in stem wood on managed land was determined by two, partly independent methods. One method (growth minus harvest, minus natural losses, minus stock on lost area, etc) was used to track the change within the periods 1990-1996 and 1996-2001, respectively. Measured stocks was used as opening stocks for 1990 as well as 1996. Thus, by comparing closing stock for 1995 (Table 5.1a) with opening stock of 1996 (Table 5.1b), the second method (measured stock change) can be used to confirm that the first method produced a reasonably accurate estimation of the accumulated stock change over time.

A minor part of the forest that is covered by the NFI grows on various other land types on which tree cuttings occur (farmland, built-up land, etc). This part added about 24 Tg C to the stem-wood stock in 1990, and the stock increase were c. 6 Tg C between 1990 and 1999 (Table 5.2). About 1-3 % of the annual round wood harvest is made on land types other than managed forestland. However, no estimate of the area cover according to FAO definitions have yet been made of this forest.

Between the forest inventory 1988-92 and 1994-96, the area of managed forest land decreased about 0.3 million hectares, mainly in the northern half of the country. A major part has become protected as nature reserves (c 0,25 million hectares). So far, the C stock and the C stock change of biomass in protected areas of Sweden has not been included in this reporting, because the regular NFI has not covered this land. Thus, the stem wood stock of this recently protected land was lost from the total stem wood stock measured. An estimate of the C stock (on average 39 ton C/ha; in total 4,8 Tg C) of the stem wood on this recently protected land was therefore added to the stem wood stock change on managed land (cf Table 5.1a). A first separate inventory of protected forest areas has been conducted. It will be repeated within the next five-year period.

In 1999-2000, a broad-spectrum scenario study was performed for the Swedish forest (SKA99). The study was described and reported in one main report (National Board of Forestry 2000) and several sub reports. In this work, the Swedish model system HUGIN was used to forecast stand characteristics for all 1993-98 NFI plots year-by-year until year 2100, for a range of scenarios. To estimating the branch biomass at different time points, HUGIN used a model that calculates branch amounts as a function of various tree and stand parameters (Petersson 1999). This model was developed from a large set of field data representing the whole country. According to the results for the base scenario, the branch biomass will hardly increase at all over the coming decades, although the stem wood and coarse root biomass does (National Board of Forestry, 2000). The main explanation is that the average tree stand closure will increase and this will normally result in a decreased proportion between branches and stems. According to the SKA99 results, the increase in branch, stump and coarse root biomass would be c. 22% of the stem-wood increase during the coming decades. This figure was used for this reporting occasion, based on the presumption that the biomass expansion factor was near this value also during the 1990’s. This presumption will be tested during the coming year. The biomass expansion factor was used to calculate the increase in branch, stump and coarse root biomass for the total stem wood growth in all forests in which harvesting occurred during the 1990-99 period (Table 5.2).

Table 5.1a. Development of stem-wood stocks in managed forests (22.9 mill. ha) of Sweden 1990-96. Opening stock 1990 as measured through inventory 1988-92. Average annual growth was 15.3 and 3.1 Tg C/yr for coniferous and deciduous forest, respectively, as determined by year-ring growth analysis on wood samples from the forest inventory 1990-94. Natural loss according to Anon. (2000) [Tg C/yr]

|  |  | Total | Coniferous | Broad-leaved |
| --- | --- | --- | --- | --- |
| 1990 | Opening stock – measured | 506 | 431 | 75 |
|  | Natural loss | 0,9 | 0,78 | 0,14 |
|  | Fellings | 13 | 11 | 2,2 |
| 1991 | Calculated opening stock | 510 | 434 | 76 |
|  | Natural loss | 1,0 | 0,86 | 0,15 |
|  | Fellings | 11 | 9 | 2,0 |
| 1992 | Calculated opening stock | 517 | 440 | 77 |
|  | Natural loss | 1,1 | 0,94 | 0,16 |
|  | Fellings | 12 | 11 | 1,6 |
| 1993 | Calculated opening stock | 522 | 443 | 79 |
|  | Natural loss | 1,3 | 1,14 | 0,20 |
|  | Fellings | 11 | 9 | 1,5 |
| 1994 | Calculated opening stock | 528 | 448 | 80 |
|  | Natural loss | 1,4 | 1,22 | 0,22 |
|  | Fellings | 11 | 10 | 1,4 |
| 1995 | Calculated opening stock | 534 | 453 | 81 |
|  | Natural loss | 1,4 | 1,17 | 0,21 |
|  | Fellings | 13 | 11 | 1,8 |
| 1996 | Calculated opening stock | 538 | 456 | 83 |
| 1990-96 | c. 0.25 million ha transferred to protected land | 4,8 | 4,1 | 0,7 |
|  | Closing stock | 534 | 452 | 82 |

Table 5.1b. Development of stem-wood stocks in managed forests (22.6 mill. ha) of Sweden 1996-2000. Opening stock 1996 as determined as average for 1994-98 (Forest statistics 2000). Average annual growth was 14.9 and 3.3 Tg C/yr for coniferous and deciduous forest, respectively, as determined by year-ring growth analysis on wood samples from the forest inventory 1994-98. Natural loss was assumed to be to 1.1 and 0.2 Tg C/yr for coniferous and deciduous forest, respectively, as determined for 1996 (Anon., 2000) and this level was assumed to remain 1997-2000. The average felling level as determined for 1998-1999 was assumed to remain 1999-2000. [Tg C/yr]

|  |  | Total | Coniferous | Broad-leaved |
| --- | --- | --- | --- | --- |
| 1996 | Opening stock – measured | 535 | 452 | 83 |
|  | Fellings | 12 | 11 | 1.2 |
| 1997 | Calculated opening stock | 540 | 455 | 85 |
|  | Fellings | 11 | 10 | 0.9 |
| 1998 | Calculated opening stock | 546 | 459 | 87 |
|  | Fellings | 12 | 11 | 1.1 |
| 1999 | Calculated opening stock | 553 | 463 | 89 |
|  | Fellings | 10 | 9 | 1.0 |
| 2000 | Calculated opening stock | 559 | 468 | 91 |
|  | Assumed fellings | 10 | 9 | 1.0 |
| 2001 | Calculated opening stock | 566 | 473 | 93 |

Table 5.2 Total increase in C stock in Swedish forest biomass other than stem wood on managed land during the 1990’s (10 years). [Tg C]

| Stock increase in: | Total | Coniferous | Broad-leaved |
| --- | --- | --- | --- |
| Stem-wood on other land types where harvesting is allowed | 6 | 3 | 3 |
| Stem-wood on protected areas1 | 4.8 | 4.1 | 0.7 |
| Branch, stump and coarse roots (d>5 cm) | 14 | 9.4 | 4.5 |
| Sum A. annual average [Tg C/yr] | 24.8 2.5 | 16.5 1.65 | 8.2 0.82 |

1 Includes only the stock increase caused by increased area of protected forest – stock change on older

protected areas was not known.

Table 5.3 Estimated year-by-year changes in C stock in Swedish forest biomass between

1990 - 2000. (Annual stock changes from Tables 5.1a,b plus A. from Table 5.2)

| Year | Total Tg CO2/yr | Total Tg C/yr | Coniferous Tg C/yr | Broad-leaved Tg C/yr |
| --- | --- | --- | --- | --- |
| 1990 | 24 | 6.6 | 5.0 | 1.6 |
| 1991 | 33 | 9.0 | 7.3 | 1.8 |
| 1992 | 27 | 7.4 | 5.3 | 2.2 |
| 1993 | 33 | 9.1 | 6.8 | 2.3 |
| 1994 | 30 | 8.1 | 5.8 | 2.3 |
| 1995 | 25 | 6.9 | 5.0 | 1.9 |
| 1996 | 26 | 7.2 | 4.5 | 2.7 |
| 1997 | 31 | 8.3 | 5.3 | 3.0 |
| 1998 | 28 | 7.8 | 4.9 | 2.9 |
| 1999 | 31 | 8.4 | 5.7 | 2.7 |
| 2000 | 31 | 8.4 | 5.7 | 2.7 |
| Sum (11 years) | 319 | 87 |  |  |

## Forest soils

Parallel to the NFI, a National Forest Site Inventory (NFSI) is presently performed for the second time. In this site inventory, the thickness of the humus layer is measured at most plots, the humus layer is sampled at about every second plot and mineral soil layers are sampled at a few thousand plots in each NFSI run. The first run of this inventory was completed 1983-1987 and the second run started 1993 and will be completed in 2002.

Preliminary estimates of the increase in C stock of the humus layer of naturally well-drained forest soils based on the NFSI results range between 50 -300 kg C/ha/yr (National Board of Forestry 2000; pers. comm Karltun, Dept Forest Soils, SLU, Uppsala). The increase was most significant in the southwestern region.

From drained forest soils (around 1-2 mill. ha), net carbon dioxide emissions probably occur  (cf Lindroth et al. 1998, Gustafsson 2001). Emissions of nitrous oxides may also be raised, whereas methane emissions likely have decreased. At this stage, no estimate of the net effect at the national level can be presented. Moreover, knowledge is lacking about net GHG emissions/sinks at untrained moist soils and in deeper soil layers of common soils. Consequently, no official estimates for reporting on total changes in the C stock of forest soils have yet been established.

## Harvested wood products

## The change in the stock of harvested wood products (HWP) in use in Sweden was estimated to be c. 0.1 Tg C/yr in average between 1983 and 1994 based on national statistics on changes in the stock of houses (Rodhe et al. 1995). A later work based on a long-term input and output analysis indicated that the average change in HWP also in the second half of the 1990’s was at a similar or smaller level (pers. comm. A. Jarnehammar, Trätek, Stockholm).

## Cultivation of organic soils

The area of cultivated organic peat soils (histosoils) in Sweden during 1996/97 was estimated to 9% of the arable land, or about 250 000 ha (Statistics Sweden, 1998). Annual crops are cultivated on nearly half of the area, while the remaining part is used for ley or as permanent pastures. About 10% of the area is cultivated with row crops (mainly potatoes and carrots).

Data from eight Swedish sites have been used for estimating the mean annual subsidence rates for peat soils cultivated with different crops (Berglund, 1989).  Carbon oxidation has been calculated to make up 30-40 % of the total subsidence (Berglund, 1989), and for the present survey a fixed value of 35 % was used. Assuming a bulk density of 0,2 g cm-3 and a carbon fraction of the oxidized soil layer of 0,45 (McAffee, 1985; Andrén & Kätterer, 2001) the mean annual carbon loss per cm soil subsidence was calculated as:

Carbon loss factor (CLF)= 10 000 x 0,01 x 0,2 x 0,45 x 0,35 = 3.15 (Mg C ha-1 cm-1)

Total carbon emissions from organic soils in Sweden was estimated using the formula:

CO2 - C emission = ∑ subsidence rate i x  area i  x CLF
		     i

where area i  and subsidence rate i denotes the area and the subsidence rate of crop type i. The background data and the results are given in the table below.

| Crop type | Subsidence rate (cm yr-1) | Area (103 ha) | Carbon loss (Gg yr-1) | CO2 emission (Gg yr -1) |
| --- | --- | --- | --- | --- |
| Pasture | 0,5 | 30 | 47 |  |
| Ley | 1,0 | 110 | 346 |  |
| Cereals | 1,5 | 85 | 402 |  |
| Row crops | 2,5 | 25 | 197 |  |
| Total |  | 250 | 992 | 3 640 |

The area of cultivated organic soils, as well as the proportion of crop types, is considered to have remained constant during the 1990ies. The same emission figure is thus reported for each year during the period 1990-1999.

### References

Anonymous 2000. Skogsdata 99. Inst f. skoglig resurshushållning och geomatik, Swed. Univ. Agric. Sci., Umeå. ISSN 0280-0543.

Gustafsson M. 2001. Carbon loss after forest drainage of three peatlands in southern Sweden. Examensarbeten vid institutionen för skoglig marklära 1-2001, Swed. Univ. Agric. Sci., Uppsala.

Lindroth A., Grelle A. & Morén A.-S. 1998. Long-term measurments of boreal forest carbon balance reveal large temperature sensitivity. Global Change Biology 4: 443-450.

National Board of Forestry 2000. Skogliga konsekvensanalyser 1999. Skogens möjligheter på 2000-talet. Skogsstyrelsens förlag, 551 83 Jönköping. ISSN 1100-0295.

Petersson H. 1999. Biomassafunktioner för trädfraktioner av tall, gran och björk i Sverige. Inst f skoglig resurshushållning och geomatik, Swed. Univ. Agric. Sci., Umeå. Arbetsrapport 59. ISSN 1401-1204. 31p.

Rodhe H., Hedlund T., Eriksson H., Jonsson P., Klemedtsson L. & Nilsson M. 1995. ”Sveriges växthusgasbudget - Rapport från ett seminarium 1994-11-22”. Report from the Climate Delegation 1/95. Regeringskansliets offsetcentral, Stockholm.

## CO2 from liming

Activity data are taken from the yearly sales statistic produced by Statistics Sweden, published in Na/Mi 30 SM “Sales of lime for agricultural and horticultural purposes and for lakes and woodlands”. On a national level, this survey is considered to give a very accurate estimate on the total quantity of sold (and hence presumably used) lime.

| Sold quantities of lime for agricultural and horticultural purposes in Sweden, by categories, tons |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Lime-stone | Find-ground raw lime | Dolomite | Mg-lime | Lime for gardens | Lime from steel-production | Lime (*) from sugar production | Other lime- products |
| 1999 | 67 900 | 64 500 | 37 200 | 75 300 | 13 600 | 16 000 | 79 170 | 0 |
| 1998 | 61 000 | 48 800 | 30 600 | 47 200 | 21 300 | 4 900 | 82 290 | 0 |
| 1997 | 118 500 | 52 500 | 33 500 | 82 800 | 19 400 | 13 500 | 73 645 | 0 |
| 1996 | 123 200 | 79 100 | 38 400 | 83 600 | 22 200 | 9 600 | 79 430 | 400 |
| 1995 | 110 000 | 71 000 | 27 700 | 55 600 | 27 900 | 9 800 | 80 405 | 400 |
| 1994 | 87 700 | 62 000 | 29 300 | 58 800 | 24 400 | 9 200 | 82 290 | 300 |
| 1993 | 68 500 | 36 000 | 27 200 | 49 100 | 31 700 | 9 000 | 71 695 | 200 |
| 1992 | 46 800 | 37 300 | 24 000 | 45 200 | 32 100 | 10 600 | 49 530 | 300 |
| 1991 | 71 000 | 32 800 | 38 600 | 62 300 | 37 600 | 12 100 | 48 490 | 200 |
| 1990 | 80 500 | 26 600 | 56 800 | 70 800 | 47 100 | 32 600 | 67 860 | 1 500 |
| Source: Statistics Sweden Na/Mi 30 SM |  |  |  |  |  |  |  |  |
| (*) Lime from sugar production has a high water content, about 35 %. Quantity given in dry matter. |  |  |  |  |  |  |  |  |

In the calculations, Dolomite and MG-lime are considered as “Dolomite”, and the other categories are considered “limestone”. Carbon conversion factors from IPCC 1996 Workbook are used, 0.12 for limestone and 0.122 for Dolomite. Recounted to CO2, this gives the following emissions:

| Emission of CO2 from lime, 1000 tons |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 |
| 169.79 | 134.10 | 108.67 | 129.65 | 156.40 | 169.05 | 192.70 | 174.14 | 130.85 | 156.44 |

# Methodology regarding Waste

## The Method

The method used for estimating methane emissions from municipal solid waste is the Tier 2-methodology, the IPCC First Order Decay model, with a slightly different time factor, and with some national estimates on gas potentials. The time factor year i, is written;

1−e−0,5k,i=0e−k(i−0,5)⋅(1−e−k),i=1,2,..., where k is the generation rate constant.

This model corresponds to the assumption that all waste is land filled on 1 July, which is approximately equivalent to a uniformly distributed deposition.

Comparisons between the suggested IPCC-gas potentials and Swedish estimates show that the IPCC-values tend to be higher, but given the large methodological uncertainties we are dealing with, this is perhaps within a reasonable interval.

Historic data is considered for 5 half-life periods back in time, which means that for the calculations for 1990, all land filled gas potentials since 1952 are considered. All available historic information on national land filled quantities is used in the calculation. There are high quality data on household waste since 1980, but data on organic industrial waste are scarce. This has forced us to make many assumptions on historical land filled waste quantities, which, of course, have greater impact on the calculated emissions of 1990 than for 2000.

## Waste Statistics in Sweden

The first Swedish national survey with estimates on land filled waste (household waste and similar only) was undertaken in 1980 by the Swedish EPA. Statistics Sweden collected similar data in 1985, 1990 and 1994. Since 1994, RVF (The Swedish Association of Waste Management) undertakes a yearly inquiry on land filled waste. Thus, household waste is the best-documented waste category, with high quality data available since 1980. This waste category is also the most important category for methane production in landfills. Figures on sludge from households and park and garden waste are available since 1990.

There are also standard values on fractions of land filled household waste from 1970 and 1975 available from RVF.

Statistics on industrial biological waste are much scarcer. There are estimates from the 1980s on industrial waste, but biological fractions were not specified. The Swedish EPA undertook studies in 1993 and 1996 on quantities and treatment of industrial biological waste. The most important waste fraction, land filled sludge from the pulp industry, is documented for the 1990s with high quality data from the Swedish EPA.

## Waste categories

### Household waste, sludge and garden waste

| Land filled waste (1000 tons) |  |  |  |
| --- | --- | --- | --- |
| Year | Household waste (and similar) | Sludge | Garden waste |
| 2000 (RVF) | 820 | 345 | 53 |
| 1999 (RVF) | 915 | 490 | 45 |
| 1998 (RVF) | 1 020 | 490 | 45 |
| 1997 (RVF) | 1 150 | 455 | 50 |
| 1996 (RVF) | 1 110 | 470 | 70 |
| 1995 (RVF) | 1 200 | 540 | 60 |
| 1994 (RVF) | 1 380 | 610 | 80 |
| 1990 (SCB) | 1 400 | 900 | 70 |
| 1988 (RVF) | 1 080 | .. | .. |
| 1986 (RVF) | 1 020 | .. | .. |
| 1985 (SCB, RVF) | 1 040 | .. | .. |
| 1980 (SNV) | 1 450 | .. | .. |

The table above presents the available statistics on three waste categories. Interpolation is used for intermediate year. Before 1980, 290 kg produced household waste per citizen is assumed, of which 76 per cent is land filled before 1973, 66 per cent is land filled 1973-1977 and 58 per cent 1978-1980.

Before 1990, park/garden waste and sludge from households are assumed to be directly proportional to the population, with the same proportion as in 1990.

The composition of household waste is examined closely in some studies (Ohlsson 1998 and FoU 145). Ohlsson (1998) presents a historic overview of Swedish analyses, the first of which was carried out in 1977. The time series indicates a rather constant value for most of the components, except the paper content, which declines during the 1990s. The table below presents the chosen values. The same values as 1990 are chosen for the period before 1990, other values are interpolated. It should be pointed out that this type of analysis contains an unknown variation, and the source of error may be quite great here. Ohlsson also shows that different studies may differ greatly in methods and results.

| Content in Swedish household waste, % |  |  |  |
| --- | --- | --- | --- |
|  | 1990 | 1995 | 1999 |
| A. Paper and textiles | 33 | 28 | 22.8 |
| B. Garden/park waste, and diapers | 14.4 | 14.4 | 14.4 |
| C. Food waste | 40.4 | 40.4 | 40.4 |
| D. Wood | 1.3 | 1.3 | 1.3 |

### Industrial waste

As noted above, data on industrial biological waste is rather scarce; a natural connection between, for example number of inhabitants and waste, is in this case questionable. Statistics on land filled industrial waste are not divided into organic waste categories; hence, in most cases it is better to rely on special studies on organic waste.

The first specific study on organic industrial waste was published in 1993; the waste groups discussed are represented in the following table. The most important subgroup is sludge from the pulp industry; the other subgroups are mainly from the food industry. The land filled gas potentials stated in the report are based on literature studies and rotting experiments; these are used as gas potentials in the methane calculations for 1990.

| Organic Industrial Waste, early 1990s (NV 4158) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Waste category | Produced quantity, 1000 tons/yr | Land filled fraction, % | Landfilled quantity, 1000 tons/yr | Gas-potential, Mm3 CH4/yr |
| Sludge from pulp industry | 1000 | 50 | 500 | 31.5 |
| Carcass | 8 | 35 | 2.8 | 0.63 |
| Waste from slaughterhouses | 40 | 5 | 2 | 0.45 |
| Sludge from slaughterhouses | 45 | 8 | 3.6 | 0.28 |
| Entrails | 30 | 5 | 1.5 | 0.09 |
| Manure from slaughterhouses | 10 | 5 | 0.5 | 0.03 |
| Draff | 5.5 | 0.5 | 0.0275 | 0.03 |
| Waste from sugar beet industry | 100 | 0.5 | 0.5 | 0.02 |
| Waste from potato industry | 46 | 0.5 | 0.23 | 0.01 |
| Returned bread | 13 | 3 | 0.39 | 0.11 |
| Mycelia waste | 2 | 1 | 0.02 | 0.01 |
| Scrows waste | 5.5 | 100 | 5.5 | 0.8 |
| Waste from fishing industry |  | 50 | 0 | 0.5 |
| Whey | 1 000 | 0 | 0 | 0 |
| Tinned-foods industry | 53 | 50 | 26.5 | 1.55 |
| Sum: |  |  |  |  |
| Sludge from pulp industry |  |  | 500 | 31.5 |
| Other |  |  | 43.6 | 4.5 |

Data on land filled sludge from the pulp industry are available from an annual survey carried out 1994-1999 by the Swedish EPA. The reports contain detailed information on waste and waste treatment for each pulp and paper producer. Intermediate values (1991-1993) are interpolated, values before 1990 are assumed to be the same as 1990 (0.50 megatons per year).

| Values of land filled waste water sludge from the pulp industry |  |  |
| --- | --- | --- |
| Year | Quantity (Megatons/year) | Source |
| 2000 | 0.075 | (*) |
| 1999 | 0.13 | NV 5114 |
| 1998 | 0.21 | NV 4987 |
| 1997 | 0.52 | NV 4924 |
| 1995 | 0.31 | NV 4657 |
| 1994 | 0.25 | NV 4434 |
| 1990 | 0.50 | NV 4158 |

(*)Report from Swedish EPA not yet published

Data on other organic industrial waste are presented in a study that was published in 1996, which shows that the land filled quantities of organic waste have declined since the early 1990s. According to the report, the land filled waste categories are represented in the following table. The gas potentials were calculated by VBB Viak.

| Organic Industrial Waste 1996 (NV 4611) |  |  |
| --- | --- | --- |
| Waste category | Land filled quantity, 1000 tons/yr | Gas-potential, Mm3 CH4/yr |
| Waste from slaughterhouses | 22.5 | 0.88 |
| Waste from potato and vegetable industries | 11.5 | 0.64 |
| Sum: | 34 | 1.52 |

The final gas potential is used as gas potentials in the methane calculations for 1996 and later. Using the two reports, values are interpolated between 1990 and 1996.

In addition to the gas potentials from these industries, we must add the paper and cardboard waste from industry, which is not included in the referred reports. However, information is available from the inquiry “Waste from the manufacturing and minerals extraction industries in 1998” from Swedish EPA/Statistics Sweden. In 1998, about 6 000 tons paper and wrapping material were land filled. This quantity is added each year to the industrial material already noted.

## Methane potentials

IPCC-values for gas potentials are used for the different fractions of household waste, as well as garden waste. As noted above, these values are somewhat higher than Swedish estimates, but lie within a reasonable interval.

IPCC gives no gas potential for land filled sludge (already treated, for example by rotting) from wastewater treatment. The DOC-content in sludge from wastewater treatment is approximately 7 per cent. The sludge has a reduced gas potential by 50 per cent because it is treated. This means that we will have a gas potential of 24 kg/tons of sludge.

For wastewater sludge from the pulp industry, we use the national value of 45 kg methane /tons of waste.

Gas potentials in waste from the food industry are stated above (chapter 1.3.2).

## Recovered gas

The first plant for biogas extraction from landfills was started in 1983. The business has increased and in 1998 there were 73 plants in operation. Information on recovered gas (in energy units) is provided by RVF (The Swedish Association of Waste Management); see table below for quantities used.

| Used values on recovered gas |  |  |
| --- | --- | --- |
| Year | Recovered gas, tons | Reference of source |
| 2000 | 34 000 | RVF |
| 1999 | 33 000 | “ |
| 1998 | 30 000 | ” |
| 1997 | 30 000 | " |
| 1996 | 30 000 | " |
| 1995 | 30 000 | ” |
| 1994 | 27 500 | RVF Report 96:7 |
| 1993 | 20 800 | " |
| 1992 | 14 430 | " |
| 1991 | 12 210 | " |
| 1990 | 12 000 | Swedish EPA/RVF |
| 1983 | NE | 1:st plant was started |
| 1982 | 0 | No gas recovery |

## Other parameters

The Methane Correction Factor for modern Swedish landfills is equal to a unit.  Waste management was centralized during the 1970s. Before 1980, landfills were smaller and presumably less compact. Information that helps establish the MCF-factor (cover material, mechanical compacting and leveling of waste) is missing. Hence, before 1980 the IPCC-default value of 0.6 is used. Methane content in landfill gas (F) is taken to be 50 per cent (IPCC-default).

National expertise estimates the mean temperature in Swedish landfills at 30 degrees Celsius, which, according to IPCC-methodology, implies a DOCF-value equal to 0.7. The oxidation factor is estimated to be 10 per cent, and the half-life of the methanogenesis is 7.5 years.

| Other parameters in the calculation |  |  |
| --- | --- | --- |
| Parameter | Value | Motivation |
| MCF - 1979 | 0.6 | IPCC Default |
| MCF 1980 - | 1 | Well managed |
| F | 50 % | IPCC Default |
| DOCF | 0.7 | National |
| OX | 10 % | National |
| t½ | 7.5 years | National |

Until about 1975, waste burning at landfills was a common waste treatment method, but it was ended about five years later. There is no information on the waste fraction that was burned, but we know that in 1975 burning was practiced at 311 of the 847 landfills.  We therefore assume that before 1976, 37 per cent of all land filled household waste was burned.

## Summary: Used statistics on land filled waste

| Year | Standard value: Household waste per Citizen (kg) | Fraction land filled household waste | Fraction of burned household waste on landfills | Land filled household-waste and similar, 1000 tons | Land filled park- and garden waste, 1000 tons | Land filled sludge from waste water treatment, 1000 tons | Land filled sludge from pulp industry, 1000 tons | Land filled organic industrial waste, 1000 tons |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2000 |  |  |  | 820 | 53 | 345 | 75 | 39 |
| 1999 |  |  |  | 915 | 45 | 490 | 130 | 39 |
| 1998 |  |  |  | 1020 | 45 | 490 | 210 | 39 |
| 1997 |  |  |  | 1150 | 50 | 455 | 520 | 39 |
| 1996 |  |  |  | 1110 | 70 | 470 | 410 | 39 |
| 1995 |  |  |  | 1200 | 60 | 540 | 310 | 43 |
| 1994 |  |  |  | 1380 | 80 | 610 | 250 | 46 |
| 1993 |  |  |  | 1390 | 77 | 700 | 386 | 48 |
| 1992 |  |  |  | 1390 | 75 | 750 | 424 | 51 |
| 1991 |  |  |  | 1390 | 72 | 800 | 462 | 53 |
| 1990 |  |  |  | 1400 | 70 | 900 | 500 | 56 |
| 1989 |  |  |  | 1240 | 70 | 895 | 500 | 56 |
| 1988 |  |  |  | 1080 | 69 | 890 | 500 | 56 |
| 1987 |  |  |  | 1050 | 69 | 885 | 500 | 56 |
| 1986 |  |  |  | 1020 | 68 | 881 | 500 | 56 |
| 1985 |  |  |  | 1040 | 68 | 876 | 500 | 56 |
| 1984 |  |  |  | 1100 | 68 | 875 | 500 | 56 |
| 1983 |  |  |  | 1200 | 68 | 874 | 500 | 56 |
| 1982 |  |  |  | 1300 | 68 | 873 | 500 | 56 |
| 1981 |  |  |  | 1400 | 68 | 872 | 500 | 56 |
| 1980 |  |  | 0% | 1450 | 68 | 871 | 500 | 56 |
| 1979 | 290 | 58% | 7% | 1292 | 68 | 869 | 500 | 56 |
| 1978 | 290 | 58% | 15% | 1186 | 67 | 867 | 500 | 56 |
| 1977 | 290 | 66% | 22% | 1229 | 67 | 865 | 500 | 56 |
| 1976 | 290 | 66% | 30% | 1109 | 67 | 862 | 500 | 56 |
| 1975 | 290 | 66% | 37% | 990 | 67 | 860 | 500 | 56 |
| 1974 | 290 | 66% | 37% | 987 | 67 | 857 | 500 | 56 |
| 1973 | 290 | 66% | 37% | 984 | 66 | 855 | 500 | 56 |
| 1972 | 290 | 76% | 37% | 1129 | 66 | 852 | 500 | 56 |
| 1971 | 290 | 76% | 37% | 1126 | 66 | 849 | 500 | 56 |
| 1970 | 290 | 76% | 37% | 1122 | 66 | 847 | 500 | 56 |
| 1969 | 290 | 76% | 37% | 1114 | 65 | 840 | 500 | 56 |
| 1968 | 290 | 76% | 37% | 1105 | 65 | 834 | 500 | 56 |
| 1967 | 290 | 76% | 37% | 1096 | 64 | 827 | 500 | 56 |
| 1966 | 290 | 76% | 37% | 1088 | 64 | 821 | 500 | 56 |
| 1965 | 290 | 76% | 37% | 1079 | 63 | 814 | 500 | 56 |
| 1964 | 290 | 76% | 37% | 1072 | 63 | 809 | 500 | 56 |
| 1963 | 290 | 76% | 37% | 1064 | 62 | 803 | 500 | 56 |
| 1962 | 290 | 76% | 37% | 1056 | 62 | 797 | 500 | 56 |
| 1961 | 290 | 76% | 37% | 1049 | 62 | 791 | 500 | 56 |
| 1960 | 290 | 76% | 37% | 1041 | 61 | 786 | 500 | 56 |
| 1959 | 290 | 76% | 37% | 1035 | 61 | 781 | 500 | 56 |
| 1958 | 290 | 76% | 37% | 1030 | 60 | 777 | 500 | 56 |
| 1957 | 290 | 76% | 37% | 1024 | 60 | 772 | 500 | 56 |
| 1956 | 290 | 76% | 37% | 1018 | 60 | 768 | 500 | 56 |
| 1955 | 290 | 76% | 37% | 1012 | 59 | 764 | 500 | 56 |
| 1954 | 290 | 76% | 37% | 1005 | 59 | 759 | 500 | 56 |
| 1953 | 290 | 76% | 37% | 998 | 59 | 753 | 500 | 56 |
| 1952 | 290 | 76% | 37% | 992 | 58 | 748 | 500 | 56 |

## Literature

Reports from Swedish EPA:

NV 3480: Svensk avfallshantering – Nuläge och problem. Naturvårdsverket May 1988.

NV 4158: Deponigasgenerering: Underlag för riktlinjer, Naturvårdsverket March 1993

NV 4271 Metangas från avfallsupplag, Naturvårdsverket 1993

NV 4434 Skogsindustrins utsläpp till vatten och luft, samt avfallsmängder 1994

NV 4611 Flöden av organiskt avfall

NV 4657: Skogsindustrins utsläpp till vatten och luft, samt avfallsmängder 1995

NV 4869: Skogsindustrins utsläpp till vatten och luft samt avfallsmängder och energiförbrukning 1996, Naturvårdsverket 1998

NV 4924: Skogsindustrins utsläpp till vatten och luft samt avfallsmängder och energiförbrukning 1997, Naturvårdsverket 1998

NV 4987: Skogsindustrins utsläpp till vatten och luft samt avfallsmängder och energiförbrukning 1998, Naturvårdsverket 1999

NV 5114: Skogsindustrins utsläpp till vatten och luft samt avfallsmängder och energiförbrukning 1999, Naturvårdsverket 2000

NV PM 970205: (unpublished) Metangas från avfallsupplag, underlag för statistik, Magnus Montelius, Naturvårdsverket, Kretsloppsavdelningen 1997

SNV PM 1652: Avfallsanläggningar i Sverige, Statistik och sammanställningar. Naturvårdsverket 1983.

SNV 1975:11 Avfallsanläggningar i Sverige

Reports from the Swedish Association of Waste Management:

RVF 90:9 Svensk avfallshantering 1990

RVF 96:5 Svenska deponier idag, kartläggning av miljöskydd, avgifter och mängder

RVF 96:8 Beskrivning av biologiskt avfall. Vägledning vid val av biologisk behandlingsmetod.

RVF 97:8 Deponering i Sverige, Statistik för 1996

RVF 98:9 Avfallsanläggningar med deponering, Statistik 1997

RVF 99:5 Avfallsanläggningar med deponering, Statistik 1998

RVF 00:14 Avfallsanläggningar med deponering 1999

Other reports:

AFR-report 264: Gas Emission from Land fills, An overview of issues and research needs

Börjesson 1997: Methane Oxidation in Landfill Cover Soils, Gunnar Börjesson, Department of Microbiology Uppsala, 1997

FoU 145: Plockanalys av hushållens säck- och kärlavfall. En studie i sex svenska kommuner. Stiftelsen REFORSK, mars 1998.

Ohlsson 1998: Plockanalys av hushållsavfall, metoder och trender, Tommy Ohlsson, 1998:226 CIV, Luleå tekniska universitet

# Analysis of Key Source Categories

The analysis of Sweden’s Key Source Categories is presented in appendix 13.

The analysis has been done completely according to IPCC Good Practice Guidance, chapter 7. Since inventory data are available for several years but lack sufficient information on uncertainty estimates, Tier 1 Level and Trend Assessment Method is applied. The analysis is presented according to what is described in IPCC Good Practice Guidance appendix 7A.1.

# Coming improvements

## Energy

### Emission factors for combustion

Emission factors for combustion has not been updated since 1996, and in the near future a revision of emission factors for combustion will be performed in order to improve the quality of emission factors especially for the years 1997 and later.

### Emission factors for waste combustion

Emissions factors for waste combustion will receive special attention since the quality of these emission factors is unknown.

### Difference of emissions between Reference approach and Sectoral approach

Emissions of CO2 differs some between Reference approach and Sectoral approach. The reasons for this will also be analysed in the near future.

### Flaring

Emissions from flaring should be reported in section 1B. Currently these emissions are included in section 1A due to lack of proper background data. This will be revised in the near future.

## Transport, emissions of CH4, N2O, NOx, CO, NMVOC and SO2 for the years 1990-1999

### Road Transportation

For the years 1995-1999 road transportation has been calculated with a national model, the so-called EMV-model. For the years 1990-1994 this sector has been calculated with an old model for CH4, N2O, NMVOC and SO2. The years 1990-1994 are thus not directly comparable with the years 1995-1999. The old model did not make any division between gasoline and diesel for road transportation. Data for the years 1990-1999 is going to be revised in the near future in order to make the whole time series consistent with regard to methodology, “Good Practice Guidance” and to make background data consistent with the methodology used for estimating CO2 emissions.

### Off-road vehicles

A review of the whole time series is going to be made in order to improve the quality of these data and to make the methodology consistent with methodology used for estimating CO2 emissions.

### Railways

It has not been possible to separate emissions from fossil fuels and emissions from electricity production for the years 1990-1999. This separation has been done for the year 2000 and is going to be made for the years 1990-1999 as soon as possible.

### Navigation

The quality of the separation made between national and international operations is not well documented. This has to be revised in the near future.

## Industrial processes

In the iron- and steel sector some SO2 emissions have been underestimated during the 1990´s. This concerns blast furnace combustion of coke, which has not been included during the 1990´s. It also concerns SO2 emissions arising from the S-content in the raw material at sinter plants, which has not been consistently treated during the 1990´s. In the reporting of emissions for the year 2000 these emissions have not been included. In the reporting of emissions for 2001, they will be included, as well as recalculated data from these sources for the period 1990-2000.

## Burning of biomass in the residential sector

The emissions from burning of biomass in the residential sector, for heating and “pleasure” (fireplaces) are now calculated by an emission factor times used energy. For heating purposes some of the furnaces have installed accumulator tanks for storing warm water and the used methodology does not take this into account. The methodology has to be developed and will be so in the near future.

## Solvent and Other Product Use

Methodology in this sector is not well documented and the quality is uncertain. This sector is going to be revised in the near future to better comply with the “Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories” and the “Good Practice Guidance”.

## Agriculture

### Methane from Enteric Fermentation

For emissions from cattle Sweden uses national methodologies. Methodologies are not considered to be documented well enough for reporting needs. For this reason a review is going to be made during 2002 and revised data for the years 1990-2000 is going to be submitted together with the submission regarding emissions 2001. According to preliminary results, the estimated total emission from cattle will approximately remain on the presently reported level.

SCB Mi 40 SM 9901

The calculations are based on the number of sows, under the assumption that a typical sow produces 23 piglets in 2,5 farrows a year.

NV 4144

According to current estimations, “other animal groups” produce only about 10 % of the total methane that result from enteric fermentation.

According to current estimations, cattle and swine produce about 85-90 % of the total methane that result from manure management.

Report RVF 96:8 on organic waste says that manure from cattle holds 89 % vs/ts, and manure from swine 88 %. Hence, the difference between these animal groups seems to be negligible.

Reports 1993:20 (on Swine) and 1995:10 (on Cattle). The given values are calculated according to the STANK-model – the official model for input/output accounting on farm level in Sweden. STANK is currently evaluated in a study launched by The European Commission. (JTI 281)

Values are calculated according to the STANK-model. A short presentation is given by JTI 281.

Fagerberg/Solomon 1992

SCB: Na 30 SM 9803

SCB: Na 30 SM 9803

SNV PM 1652; another value, 275 kg/inhabitant is used in SNV 1975:11.

RVF 90:9, NV 4158.

NV 4158

NV 4611

MI 28 SM 0001, Statistics Sweden.

Recounted from RVF 96:8.

Oral communication from VBB Viak.

NV 4158

AFR 264

National information kindly provided by Dr. Gunnar Börjesson, University of Linköping.

SNV PM 1652

SNV PM 1652