15 April 2003

Appendix 6 Transport

The emissions from transport referring to SNAP category 07 (road transport) and the sub-categories in 08 (other mobile sources) are made up in the following IPCC categories:

Table 1 Inventory sub-categories for transport according to SNAP and IPCC classification

| SNAP classification | IPCC classification |
| --- | --- |
| 07 Road transport | 1A3b Transport-Road |
| 0801 Military | 1A5 Other |
| 0802 Railways | 1A3c Railways |
| 0803 Inland waterways | 1A3d Transport-Navigation |
| 080402 National sea traffic | 1A3d Transport-Navigation |
| 080403 National fishing |  |
| 080404 International sea traffic |  |
| 080501 Dom. airport traffic (LTO < 1000 m) | 1A3a Transport-Civil aviation |
| 080502 Int. airport traffic (LTO < 1000 m) | 1A3a Transport-Civil aviation (international) |
| 080503 Dom. cruise traffic (> 1000 m) | 1A3a Transport-Civil aviation |
| 080504 Int. cruise traffic (> 1000 m) | 1A3a Transport-Civil aviation (international) |
| 0806 Agriculture | 1A4c Agriculture/forestry/fisheries |
| 0807 Forestry | 1A4c Agriculture/forestry/fisheries |
| 0808 Industry | 1A2f Industry-Other |
| 0809 Household and gardening | 1A4b Residential |

1. Methodology and references for Road Transport

For road transport the Tier 1 and Tier 2 approaches are used in parallel to make annual estimates of the Danish emissions as described in the IPCC guidelines (Penman et al. (2000). The actual calculations are made with the European COPERT III model (Ntziachristos et al. 2000). In COPERT III fuel use and emission simulations can be made for operationally hot engines taking into account gradually stricter emission standards and emission degradation due to catalyst wear. Furthermore the emission effects of cold start and evaporation are simulated.

1.1. Vehicle fleet and mileage data

Corresponding to the COPERT fleet classification all present and future vehicles in the Danish traffic are grouped into vehicle layers. This is a sub-division of all vehicle classes into groups of vehicles with the same average fuel use and emission behaviour. An overview of the different layers with years of implementation is given in annex 1.

Table 2 Model vehicle classes, trip speeds and mileage split.

| Vehicle classe | Fuel type | Engine size/weight | Trip speed |  |  | Mileage [%] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Urban | Rural | Highway | Urban | Rural | Highway |
| PC | Gasoline | < 1.4 l. | 40 | 70 | 100 | 35 | 46 | 19 |
| PC | Gasoline | 1.4 – 2 l. | 40 | 70 | 100 | 35 | 46 | 19 |
| PC | Gasoline | > 2 l. | 40 | 70 | 100 | 35 | 46 | 19 |
| PC | Diesel | < 2 l. | 40 | 70 | 100 | 35 | 46 | 19 |
| PC | Diesel | > 2 l. | 40 | 70 | 100 | 35 | 46 | 19 |
| PC | LPG |  | 40 | 70 | 100 | 35 | 46 | 19 |
| PC | 2-stroke |  | 40 | 70 | 100 | 35 | 46 | 19 |
| LDV | Gasoline |  | 40 | 65 | 80 | 35 | 50 | 15 |
| LDV | Diesel |  | 40 | 65 | 80 | 35 | 50 | 15 |
| Trucks | Gasoline |  | 35 | 60 | 80 | 32 | 47 | 21 |
| Trucks | Diesel | 3.5 – 7.5 tonnes | 35 | 60 | 80 | 32 | 47 | 21 |
| Trucks | Diesel | 7.5 – 16 tonnes | 35 | 60 | 80 | 32 | 47 | 21 |
| Trucks | Diesel | 16 – 32 tonnes | 35 | 60 | 80 | 19 | 45 | 36 |
| Trucks | Diesel | > 32 tonnes | 35 | 60 | 80 | 19 | 45 | 36 |
| Urban buses | Diesel |  | 30 | 50 | 70 | 51 | 41 | 8 |
| Coaches | Diesel |  | 35 | 60 | 80 | 32 | 47 | 21 |
| Mopeds | Gasoline |  | 30 | 30 | - | 81 | 19 | 0 |
| Motorcycles | Gasoline | 2 stroke | 40 | 70 | 100 | 47 | 39 | 14 |
| Motorcycles | Gasoline | < 250 cc. | 40 | 70 | 100 | 47 | 39 | 14 |
| Motorcycles | Gasoline | 250 – 750 cc. | 40 | 70 | 100 | 47 | 39 | 14 |
| Motorcycles | Gasoline | > 750 cc. | 40 | 70 | 100 | 47 | 39 | 14 |

From the Danish Road Directorate information of the vehicle stock and annual mileage is obtained. This covers data for the number of vehicles, annual mileage, mileage split between urban, rural and highway driving and the respective average speeds.

Figure 1 No. of vehicles in sub-classes in 1985-2001

The number of vehicles and annual mileages respectively, are provided per first registration year for all vehicle sub-classes. Subsequently the vehicle numbers are summed up in layers, j, for each year, y, by using the correspondance between layers and first registration year, i:

Nj,y=i=FYear(j)LYear(j)Ni,y   (1)

Weighted annual mileages per layer are calculated as the sum of all mileage driven per first registration year divided with the total number of vehicles in the specific layer.

Mj,y=i=FYear(j)LYear(j)Ni,y⋅Mi,yi=FYear(j)LYear(j)Ni,y   (2)

Both vehicle numbers and weighted annual mileages per layer are shown in annex 1.

Figure 2 Layer distribution of vehicle numbers per vehicle type in 1985-2001

Trip speed dependent fuel use and emission factors are taken from the COPERT model using trip speeds as shown in table 2. The factors are listed in annex 2. For new layers not represented by actual data, the emission factors are scaled according to reduction factors, see Ntziachristos et al. (2000) or Illerup et al. (2002).

1.2. Deteoriation factors

For three-way catalyst cars the emissions of NOx and NMVOC (and CO) gradually increase due to catalyst wear and are therefore modified as a function of total mileage by the so-called deteoriation factors. Even though the emission curves may be serrated for the individual vehicles, on average the emissions from catalyst cars stabilise after a given cut-off mileage is reached due to OBD (On Board Diagnostics) and the Danish inspection and maintenance programme. For each forecast year the deteoriation factors are calculated per first registration year by using deteoriation coefficients and cut-off mileages as given in Ntziachristos et al. (2000) or Illerup et al. (2002) for the corresponding layer. The deteoriation coefficients are given for the two driving cycles ”Urban driving Cycle” (UDF) and ”Extra Urban driving Cycle” (EUDF: urban and rural), with trip speeds of 19 and 63 km/h, respectively.

Firstly the deteoriation factors are calculated for the correspondent trip speeds of 19 and 63 km h-1 in each case determined by the total cumulated mileage less than or exceeding the cut-off mileage. The formulas 3 and 4 show the calculations for the ”Urban driving Cycle”:

UDF=UA⋅MTC+UB, MTC < UMAX   (3)

UDF=UA⋅UMAX+UB, MTC >= UMAX   (4)

Where UDF is the urban detoriation factors, UA and UB the urban deteoriation coefficients, MTC = total cumulated mileage, UMAX urban cut-off mileage.

In the case of trip speeds below 19 km h-1 the deteoriation factor, DF, equals UDF, whereas for trip speeds exceeding 63 km h-1 DF=EUDF. For trip speeds between 19 and 63 km h-1 the deteoriation factor, DF, is found as an interpolation between UDF and EUDF. Secondly the deteoriation factors, one for each of the three road types, are aggregated into layers by taking into account the vehicle numbers and annual mileages per first registration year:

DFj,y=i=FYear(j)LYear(j)DFi,y⋅Ni,y⋅Mi,yi=FYear(j)LYear(j)DFi,y⋅Ni,y   (5)

Where DF is the deteoriation factor.

1.3. Emissions and fuel use for hot engines

Emissions and fuel use results for operationally hot engines are calculated for each year and layer and road type. The procedure is to combine fuel use and emission factors (and deteoriation factors for catalyst vehicles), number of vehicles annual mileage numbers and their road type shares given in table 1. For non-catalyst vehicles this yields:

Ej,k,y=EFj,k,y⋅Sk⋅Nj,y⋅Mj,y   (6)

Here E = fuel use/emission, EF = fuel use/emission factor, S = road type share, k = road type.

For catalyst vehicles the calculations becomes:

Ej,k,y=DFj,k,y⋅EFj,k,y⋅Sk⋅Nj,y⋅Mj,y   (7)

1.4. Extra emissions and fuel use for cold engines

Extra emissions of SO2, NOx and NMVOC (as well as CO, PM, CH4, CO2 and FC) from cold start are simulated separately. In the model each trip is associated with an amount of cold start emission and is assumed to take place under urban driving conditions. The number of trips are distributed evenly in months. Firstly cold emission factors are calculated as the hot emission factor times the cold:hot emission ratio. Secondly the extra emission factor during cold start is found by subtracting the hot emission factor from the cold emission factor. Lastly this extra factor is applied on the fraction of the total mileage driven with a cold engine (the -factor) for all vehicles in the specific layer.

The cold:hot ratios depend on the average trip length and the monthly ambient temperature distribution and are equivalent for gasoline fuelled conventional passenger cars and vans and for diesel passenger cars and vans, respectively, see Ntziachristos et al. (2000). For conventional gasoline and all diesel vehicles the extra emissions become:

CEj,y=β⋅Nj,y⋅Mj,y⋅EFU,j,y⋅(CEr−1)   (8)

Where CE is the cold extra emissions,  = cold driven fraction, CEr = Cold:Hot ratio.

For catalyst cars the cold:hot ratio is also trip speed dependent. The ratio is however unaffected by catalyst wear. The EURO I ratio is used for all future catalyst technologies. However, in order to comply with gradually stricter emission standards the catalyst light-off temperature must be reached in even shorter time periods for future EURO standards. Correspondingly the -factor for gasoline vehicles is step-wise reduced for EURO II onwards.

For catalyst vehicles the cold extra emissions are found from:

CEj,y=βred⋅βEUROI⋅Nj,y⋅Mj,y⋅EFU,j,y⋅(CErEUROI−1)   (9)

Where red = the  reduction factor.

1.5. Evaporative emissions from gasoline vehicles

For each year evaporative emissions of hydrocarbons are simulated in the forecast model as hot and warm running loss, hot and warm soak, and diurnal emissions. All emission types are influenced by RVP (Reid Vapour Pressure) and ambient temperature. The emission factors are shown in Ntziachristos et al. (2000).

Running loss emissions originate from vapour generated in the fuel tank during operation. The destinction between hot and warm running loss emissions depend on the engine temperature. In the model hot and warm running loss occur for hot and cold engines, respectively. The emissions are calculated as the annual mileage – broken down on cold and hot mileage totals using the -factor - times respective emission factors. For vehicles equipped with evaporation control (catalyst cars) the emission factors are only one tenth of the uncontrolled factors used by conventional gasoline vehicles.

Rj,y=Nj,y⋅Mj,y⋅((1−β)⋅HR+β⋅WR)   (10)

Where R is the running loss emissions and HR and WR the hot and warm running loss emission factors, respectively.

In the model hot and warm soak emissions for carburettor vehicles also occurs for hot and cold engines, respectively. These emissions are calculated as number of trips – broken down into cold and hot trip numbers using the -factor - times respective emission factors:

Sj,yC=Nj,y⋅Mj,yltrip⋅((1−β)⋅HS+β⋅WS)   (11)

Where SC is the soak emissions, ltrip = the average trip length and HS and WS is the hot and warm soak emission factors, respectively. Since all catalyst vehicles are assumed to be carbon canister controlled no soak emissions are estimated for this vehicle type. Average maximum and minimum temperatures per month are used in combination with diurnal emission factors to estimate the diurnal emissions from uncontrolled vehicles Ed(U):

Ej,yd(U)=365⋅Nj,y⋅ed(U)   (12)

Each forecast year’s total is the sum of each layer’s running loss, soak and diurnal emissions.

1.6. Fuel use balance

The calculated fuel use in the model must equal the statistical fuel sale totals from the Danish Energy Agency (DEA) according to the UNFCCC emissions reporting format. The standard approach to achieve a fuel balance in annual emission inventories is to multiply the annual mileage with a fuel balance factor derived as the ratio between simulated and statistical fuel figures for gasoline and diesel, respectively. This method is also used in the present model.

For gasoline vehicles all mileage numbers are equally scaled in order to obtain a gasoline fuel equilibrium. For diesel fuel the balance is made by adjusting the mileage for light and heavy duty vehicles and buses, given that the mileage and fuel consumption factors for these vehicles are regarded as the most uncertain parameters in the diesel engine emission simulations.

The final fuel use and emission factors are shown in annex 3. The total fuel use and emissions are shown in annex 4 and 5 per vehicle category and as grand totals, respectively.

2. Methodologies and references for other mobile sources

The off road sector is divided into several sub-sectors; sea transport, fishery, air traffic, railways, military and the working machinery and materiel in the industry, forestry, agriculture and household and gardening sectors. The emission calculations are made using the Tier 2 method for air traffic and off road working machinery and equipment, while for the remaining sectors the Tier 1 method is used.

2.1. Activity data

The activity data for air traffic consist of air traffic statistics provided by the Danish Civil Aviation Agency (CAA-DK) and Copenhagen Airport. For 2001 records are given per flight by CAA-DK as data for aircraft type and origin and destination airports. Prior to 2001 detailed LTO/aircraft type statistics are provided by Copenhagen Airport (for this airport only), while CAA-DK has given information of total take off numbers for other Danish airports. Fuel statistics for jet fuel use and aviation gasoline are obtained from the DEA.

For off road working machinery and equipment the number of different types of machines, their load factors, engine sizes and annual working hours are taken from the Danish EPA (1992 and 1993). Fuel use statistics for diesel, gasoline and LPG are obtained from the DEA in relevant sectors.

The activity data for military, railways, sea transport and fishery consist of fuel use information provided by the DEA. For sea transport the basis is fuel sold in Danish ports and the traffic is defined as either national or international depending on the destination of the vessels in question as prescribed by the IPCC guidelines.

For all sectors fuel use figures are given in annex 6 for the years 1990 and 2001.

2.2. Emission factors

For military ground material and railways aggregated emission factors for gasoline and diesel are derived from the road traffic emission simulations made with the COPERT model. The emission factors for the remaining sectors come from the EMEP/CORINAIR guidebook, see CORINAIR (1999). For all sectors emission factors are given in annex 6 for the years 1990 and 2001.

2.3. Calculation method

For military, aircraft, railways, national sea traffic, fishing and aviation the emissions are estimated with the Tier 1 method using fuel related emission factors and fuel use from the DEA.

For aviation the estimates are made separately for Landing and Take Offs (LTOs < 3000 ft) and cruise (> 3000 ft). From 2001 the estimates are made on a city-pair level by combining activity data and emission factors and subsequently group the emission results into domestic and international totals. In a final step a fuel balance is made. The fuel ratio between model estimates and statistical sales is used to modify the model results of cruise fuel use and emissions according to the domestic and international cruise shares.

Prior to 2001 the calculation scheme is firstly to estimate each year’s fuel use and emissions for LTO. Secondly the total cruise fuel use is found year by year as the statistical fuel use total minus the calculated fuel use for LTO. Lastly the cruise fuel use is split into domestic and international parts by using the results from a Danish city pair emission inventory in 1998 (Winther, 2001a). For more details of this latter fuel allocation procedure, see Winther (2001b).

Off road working machines and equipment are placed in the sectors: Inland waterways, agriculture, forestry, industri and household and gardening. In general the fuel use and emissions are calculated by combining information of the number of different machine types and their respective load factors, engine sizes, annual working hours, and fuel use and emission factors. The simulations take into account the implementation of a two stage emission legislation directive depending on engine size for relevant types of diesel fuelled machinery. Stage I and II of the directive becomes effective for new machinery in use in 1999-2001 and 1999-2003 respectively.

A fuel balance is made for diesel between statistical sales and estimated fuel consumption  and the ratio is used to scale the annual working hours used in the emission calculations for all machinery types. The final simulated fuel use for gasoline and LPG is maintained by adjusting the amount of fuel used in the simulations for road traffic and household.

The calculated emissions for other mobile sources are shown per sector in annex 7 for the years 1990 and 2001.

3. Recalculations

From 2001 air traffic estimates are made up using city-pair statistics from the CAA-DK. Moreover the flights for Greenland and the Faroe Islands are included under domestic aviation as prescribed by the UNFCCC reporting guidelines. Previous years estimates (1990-2000) are updated in accordance with the new model.

For military and railways updated 1990-2001 emission factors are used for gasoline and diesel derived from the new road transport inventory results.

4. Uncertainties

Uncertainty estimates are made for road transport and other mobile sources using the guidelines and emission factor uncertainties formulated by Penman et al. (2000). The calculations are shown in annex 8.

| Category | Fuel type | Activity data | Emission factor |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Uncertainty | Uncertainty |  |  |
|  |  |  | CO2 | CH4 | N2O |
|  |  | % | % | % | % |
| Civil Aviation | Aviation Gasoline | 0 | 5 | 100 | 1000 |
|  | Jet Kerosene | 0 | 5 | 100 | 1000 |
| Road Transportation | Gasoline | 0 | 5 | 40 | 50 |
|  | Diesel Oil | 2 | 5 | 40 | 50 |
| Railways | Diesel Oil | 2 | 5 | 100 | 1000 |
| Navigation | Residual Oil | 2 | 5 | 100 | 1000 |
|  | Gas/Diesel Oil | 0 | 5 | 100 | 1000 |
| Agriculture | Diesel Oil | 20 | 5 | 100 | 1000 |
|  | Gasoline | 20 | 5 | 100 | 1000 |
| Forestry | Diesel Oil | 20 | 5 | 100 | 1000 |
|  | Gasoline | 20 | 5 | 100 | 1000 |
| Industry | Diesel Oil | 20 | 5 | 100 | 1000 |
|  | Gasoline | 20 | 5 | 100 | 1000 |
|  | LPG | 20 | 5 | 100 | 1000 |
| Household and gardening | Gasoline | 20 | 5 | 100 | 1000 |
| Overall uncertainty in 2001 |  |  | 3,7 | 32,1 | 58,8 |
| Trend uncertainty |  |  | 3,8 | 4,1 | 154,4 |

5. Quality assurance/quality control (QA/QC)

For road transport the Tier 1 and Tier 2 methods are used independently to provide a quality control of the emission estimations. Firstly the bottom up approach (Tier 2) is used as described in the sections 1.1 to 1.5. Secondly the estimates are modified according to a fuel balance using the statistical sale figures for road transportation fuel in Denmark (Tier 1), as described in section 1.6. The usage of the Tier 1 method ensures that all fuel for road transport is accounted for in the estimations and that no double counting of emissions for working equipment and machinery is made.

Also for air traffic and off road machinery and working equipment the Tier 1 and 2 methods are worked in parallel, see section 2.3. This enables the computation of more accurate sub-sectoral estimates and makes the level of total sector-wise emission contributions to be in compliance with anticipated results derived directly from national fuel statistics.

For the remaining transport sectors the Tier 1 method ensure that all fuel is accounted for in the emission estimations.

As a part of the general QA/QC work all time series of emissions in the CRF and SNAP source categories are examined and considerate changes are checked and explained. Moreover a comparison is made to the previous year’s estimate, and any major changes are verified. As a last point a data transfer control is made from SNAP source categories to aggregated CRF source categories.

6. References

CORINAIR (1999): Atmospheric Emission Inventory Guidebook Vol. 3, Second Edition, EMEP Task Force on Emission Inventories, European Environmental Agency, Copenhagen.

Illerup et al. (2002): Projection Models 2010. Danish Emissions of SO2 , NOx , NMVOC and NH3. (2002). NERI technical report no. 414. National Environmental Research Institute. 192 pp.

Ntziachristos, L., Samaras, Z. (2000): COPERT III Computer Programme to Calculate Emissions from Road Transport - Methodology and Emission Factors (Version 2.1). Tehnical report No 49. European Environment Agency, November 2000, Copenhagen.

Penman et al. (2000): Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, IPCC.

Winther, M. (2001): 1998 Fuel Use and Emissions for Danish IFR Flights. Environmental Project no. 628, 2001. 112 p. Danish EPA. Prepared by the National Environmental Research Institute, Denmark. Electronic report at homepage of Danish EPA http://www.mst.dk/udgiv/Publications/2001/87-7944-661-2/html/.

Winther, M. (2001): Improving fuel statistics for Danish aviation. National Environmental Research Institute, Denmark. 56 p. – NERI Technical Report No. 387.

Annex 1: Fleet data 1990-2001 for road transport

| Sector | Subsector | Tech | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passenger Cars | Gasoline <1,4 l | PRE ECE | 46209 | 44014 | 42804 | 36466 | 39959 | 37597 | 37130 | 3434 | 2761 | 2103 | 1744 | 1614 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/00-01 | 187912 | 161642 | 139010 | 119424 | 80742 | 67991 | 53302 | 44338 | 31104 | 22511 | 17980 | 15837 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/02 | 86959 | 80041 | 73306 | 66422 | 50119 | 43384 | 35052 | 26097 | 17585 | 10873 | 7348 | 5544 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/03 | 300791 | 294878 | 288227 | 280146 | 261998 | 250043 | 235177 | 215301 | 183113 | 147111 | 118929 | 97930 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/04 | 272012 | 270182 | 268686 | 268154 | 265860 | 262990 | 259954 | 265188 | 264791 | 254032 | 235890 | 219216 |
| Passenger Cars | Gasoline <1,4 l | Euro I | 10000 | 49608 | 87121 | 122067 | 177991 | 230063 | 282488 | 289374 | 275572 | 273582 | 270268 | 267260 |
| Passenger Cars | Gasoline <1,4 l | Euro II |  |  |  |  |  |  |  | 58502 | 119142 | 170981 | 209279 | 205833 |
| Passenger Cars | Gasoline <1,4 l | Euro III |  |  |  |  |  |  |  |  |  |  |  | 34695 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | PRE ECE | 35940 | 34233 | 33292 | 28362 | 31079 | 29242 | 28879 | 2671 | 2148 | 1635 | 1356 | 1255 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/00-01 | 127631 | 109641 | 94188 | 80844 | 54600 | 45991 | 36079 | 30465 | 21520 | 15647 | 12537 | 11077 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/02 | 55063 | 50674 | 46402 | 42040 | 31712 | 27445 | 22173 | 16509 | 11141 | 6870 | 4642 | 3500 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/03 | 174545 | 170750 | 166596 | 161592 | 150612 | 143386 | 133413 | 122642 | 103931 | 83270 | 67222 | 55300 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/04 | 180298 | 178950 | 177873 | 177525 | 176045 | 174195 | 172298 | 176155 | 179510 | 172582 | 160800 | 149915 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro I | 10000 | 45647 | 82427 | 119744 | 184854 | 250826 | 322960 | 330407 | 315731 | 313279 | 309587 | 306414 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro II |  |  |  |  |  |  |  | 80440 | 163821 | 235099 | 287758 | 283021 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro III |  |  |  |  |  |  |  |  |  |  |  | 47705 |
| Passenger Cars | Gasoline >2,0 l | PRE ECE | 3423 | 3260 | 3171 | 2701 | 2960 | 2785 | 2750 | 254 | 205 | 156 | 129 | 120 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/00-01 | 10781 | 9234 | 7914 | 6781 | 4567 | 3849 | 3022 | 2619 | 1881 | 1366 | 1110 | 986 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/02 | 4392 | 4043 | 3702 | 3355 | 2531 | 2191 | 1770 | 1318 | 888 | 549 | 371 | 280 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/03 | 24667 | 24157 | 23595 | 22912 | 21429 | 20432 | 19053 | 17571 | 14934 | 12016 | 9722 | 8009 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/04 | 15679 | 15524 | 15390 | 15339 | 15120 | 14844 | 14546 | 14977 | 23975 | 22975 | 21251 | 19699 |
| Passenger Cars | Gasoline >2,0 l | Euro I | 10000 | 13961 | 17871 | 21674 | 28044 | 34257 | 40813 | 41567 | 31121 | 30887 | 30519 | 30193 |
| Passenger Cars | Gasoline >2,0 l | Euro II |  |  |  |  |  |  |  | 7313 | 14893 | 21373 | 26160 | 25729 |
| Passenger Cars | Gasoline >2,0 l | Euro III |  |  |  |  |  |  |  |  |  |  |  | 4337 |
| Passenger Cars | Diesel <2,0 l | Euro I |  | 4041 | 8031 | 11912 | 18412 | 24751 | 31440 | 31580 | 31998 | 35415 | 39518 | 43826 |
| Passenger Cars | Diesel <2,0 l | Euro II |  |  |  |  |  |  |  | 7316 | 15312 | 24505 | 33856 | 37328 |
| Passenger Cars | Diesel <2,0 l | Euro III |  |  |  |  |  |  |  |  |  |  |  | 6313 |
| Passenger Cars | Diesel <2,0 l | Conventional | 79709 | 75788 | 72288 | 68529 | 62139 | 58843 | 55000 | 48153 | 43893 | 43004 | 42861 | 42885 |
| Passenger Cars | Diesel >2,0 l | Euro I |  | 213 | 423 | 627 | 969 | 1303 | 1655 | 1662 | 1684 | 1864 | 2087 | 2313 |
| Passenger Cars | Diesel >2,0 l | Euro II |  |  |  |  |  |  |  | 385 | 806 | 1290 | 1789 | 1971 |
| Passenger Cars | Diesel >2,0 l | Euro III |  |  |  |  |  |  |  |  |  |  |  | 332 |
| Passenger Cars | Diesel >2,0 l | Conventional | 3702 | 3556 | 3425 | 3281 | 3040 | 2905 | 2746 | 2461 | 2266 | 2237 | 2228 | 2229 |
| Passenger Cars | LPG | Conventional | 286 | 286 | 288 | 289 | 289 | 301 | 311 | 172 | 97 | 44 | 32 | 63 |
| Passenger Cars | 2-Stroke | Conventional | 5417 | 4804 | 4308 | 3747 | 3029 | 2443 | 1665 | 1248 | 761 | 400 | 300 | 200 |

| Sector | Subsector | Tech | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Light Duty Veh. | Gasoline <3,5t | Conventional | 42333 | 43215 | 44179 | 45486 | 47261 | 44601 | 41519 | 37209 | 34454 | 31489 | 28488 | 25423 |
| Light Duty Veh. | Gasoline <3,5t | Euro I |  |  |  |  |  | 4259 | 8524 | 12645 | 17212 | 16632 | 15979 | 15527 |
| Light Duty Veh. | Gasoline <3,5t | Euro II |  |  |  |  |  |  |  |  |  | 4705 | 9299 | 14017 |
| Light Duty Veh. | Diesel <3,5 t | Conventional | 155543 | 158781 | 162324 | 167129 | 173650 | 163877 | 152553 | 142109 | 131572 | 122992 | 115695 | 105397 |
| Light Duty Veh. | Diesel <3,5 t | Euro I |  |  |  |  |  | 15648 | 31318 | 48292 | 65727 | 64964 | 64894 | 64370 |
| Light Duty Veh. | Diesel <3,5 t | Euro II |  |  |  |  |  |  |  |  |  | 18376 | 37766 | 58112 |
| Heavy Duty Veh. | Gasoline >3,5 t | Conventional | 250 | 255 | 260 | 268 | 279 | 288 | 295 | 261 | 274 | 253 | 257 | 249 |
| Heavy Duty Veh. | Diesel 3,5 - 7,5 t | Conventional | 5108 | 5214 | 5330 | 5488 | 5205 | 4891 | 4532 | 3999 | 3692 | 3079 | 2406 | 1979 |
| Heavy Duty Veh. | Diesel 3,5 - 7,5 t | Euro I |  |  |  |  | 497 | 1004 | 1506 | 1440 | 1435 | 1269 | 1057 | 951 |
| Heavy Duty Veh. | Diesel 3,5 - 7,5 t | Euro II |  |  |  |  |  |  |  | 529 | 1087 | 1487 | 1703 | 1990 |
| Heavy Duty Veh. | Diesel 7,5 - 16 t | Conventional | 10286 | 10500 | 10734 | 11052 | 10482 | 9850 | 9126 | 7800 | 6603 | 5613 | 5085 | 4210 |
| Heavy Duty Veh. | Diesel 7,5 - 16 t | Euro I |  |  |  |  | 1001 | 2022 | 3033 | 2808 | 2566 | 2314 | 2235 | 2024 |
| Heavy Duty Veh. | Diesel 7,5 - 16 t | Euro II |  |  |  |  |  |  |  | 1032 | 1945 | 2710 | 3600 | 4234 |
| Heavy Duty Veh. | Diesel 16 - 32 t | Conventional | 13034 | 13306 | 13602 | 14005 | 13283 | 12481 | 11564 | 10720 | 9832 | 8982 | 7933 | 6814 |
| Heavy Duty Veh. | Diesel 16 - 32 t | Euro I |  |  |  |  | 1268 | 2562 | 3844 | 3859 | 3821 | 3702 | 3486 | 3276 |
| Heavy Duty Veh. | Diesel 16 - 32 t | Euro II |  |  |  |  |  |  |  | 1419 | 2896 | 4336 | 5616 | 6853 |
| Heavy Duty Veh. | Diesel >32t | Conventional | 11446 | 11684 | 11944 | 12298 | 11664 | 10960 | 10154 | 9337 | 8720 | 8180 | 7361 | 6527 |
| Heavy Duty Veh. | Diesel >32t | Euro I |  |  |  |  | 1114 | 2250 | 3376 | 3362 | 3389 | 3371 | 3234 | 3138 |
| Heavy Duty Veh. | Diesel >32t | Euro II |  |  |  |  |  |  |  | 1236 | 2568 | 3949 | 5211 | 6564 |
| Buses | Urban Buses | Conventional | 4753 | 4561 | 4522 | 4490 | 4083 | 3635 | 3261 | 2946 | 2792 | 2542 | 2319 | 2319 |
| Buses | Urban Buses | Euro I |  |  |  |  | 390 | 746 | 1084 | 1060 | 972 | 913 | 852 | 852 |
| Buses | Urban Buses | Euro II |  |  |  |  |  |  |  | 390 | 729 | 1053 | 1345 | 1345 |
| Buses | Coaches | Conventional | 3327 | 2868 | 3007 | 3086 | 2927 | 4507 | 4156 | 3662 | 3369 | 3007 | 2724 | 2724 |
| Buses | Coaches | Euro I |  |  |  |  | 280 | 925 | 1381 | 1318 | 1173 | 1080 | 1001 | 1001 |
| Buses | Coaches | Euro II |  |  |  |  |  |  |  | 485 | 879 | 1246 | 1579 | 1579 |
| Mopeds | <50 cm³ | Conventional | 120000 | 118000 | 113000 | 109000 | 105000 | 114167 | 123333 | 132500 | 141667 | 150833 | 150522 | 149460 |
| Mopeds | <50 cm³ | 97/24/EC I |  |  |  |  |  |  |  |  |  |  | 9478 | 15540 |
| Motorcycles | 2-stroke >50 cm³ | Conventional | 6617 | 6804 | 6904 | 7111 | 7406 | 7672 | 8214 | 8980 | 9598 | 10385 | 11054 | 11367 |
| Motorcycles | 4-stroke <250 cm³ | Conventional | 7499 | 7712 | 7824 | 8059 | 8394 | 8695 | 9310 | 10177 | 10878 | 11769 | 11916 | 11367 |
| Motorcycles | 4-stroke <250 cm³ | 97/24/EC |  |  |  |  |  |  |  |  |  |  | 613 | 1074 |
| Motorcycles | 4-stroke 250-750 cm³ | Conventional | 20622 | 21207 | 21516 | 22162 | 23083 | 23911 | 25602 | 27986 | 29914 | 32365 | 32768 | 33910 |
| Motorcycles | 4-stroke 250-750 cm³ | 97/24/EC |  |  |  |  |  |  |  |  |  |  | 1685 | 2953 |
| Motorcycles | 4-stroke >750 cm³ | Conventional | 9374 | 9639 | 9780 | 10074 | 10492 | 10869 | 11637 | 12721 | 13597 | 14712 | 14894 | 15414 |
| Motorcycles | 4-stroke >750 cm³ | 97/24/EC |  |  |  |  |  |  |  |  |  |  | 766 | 1342 |

Annex 1: Mileage data 1990-2001 for road transport

| Sector | Subsector | Tech | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passenger Cars | Gasoline <1,4 l | PRE ECE | 10352 | 11120 | 11847 | 12282 | 12679 | 12387 | 12152 | 11935 | 11988 | 11407 | 11507 | 11922 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/00-01 | 12073 | 12735 | 13119 | 13036 | 12679 | 12387 | 12152 | 11935 | 11988 | 11407 | 11507 | 11922 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/02 | 13225 | 13605 | 14495 | 15028 | 15513 | 13532 | 12152 | 11935 | 11988 | 11407 | 11507 | 11922 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/03 | 16370 | 16830 | 16920 | 16820 | 16701 | 15701 | 14867 | 14403 | 14225 | 13135 | 12591 | 11922 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/04 | 19941 | 20341 | 20763 | 20356 | 19947 | 18688 | 17564 | 16233 | 15648 | 14567 | 14356 | 12416 |
| Passenger Cars | Gasoline <1,4 l | Euro I | 22535 | 23984 | 25041 | 25397 | 25885 | 24768 | 23130 | 22344 | 21207 | 19362 | 18497 | 18409 |
| Passenger Cars | Gasoline <1,4 l | Euro II |  |  |  |  |  |  |  | 25981 | 25671 | 23766 | 24054 | 22954 |
| Passenger Cars | Gasoline <1,4 l | Euro III |  |  |  |  |  |  |  |  |  |  |  | 25954 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | PRE ECE | 10352 | 11120 | 11847 | 12282 | 12679 | 12387 | 12152 | 11935 | 11988 | 11407 | 11507 | 11922 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/00-01 | 12073 | 12735 | 13119 | 13036 | 12679 | 12387 | 12152 | 11935 | 11988 | 11407 | 11507 | 11922 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/02 | 13225 | 13605 | 14495 | 15028 | 15513 | 13532 | 12152 | 11935 | 11988 | 11407 | 11507 | 11922 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/03 | 16370 | 16830 | 16920 | 16820 | 16701 | 15701 | 14867 | 14403 | 14225 | 13135 | 12591 | 11922 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/04 | 19941 | 20341 | 20763 | 20356 | 19947 | 18688 | 17564 | 16233 | 15648 | 14567 | 14356 | 12416 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro I | 22535 | 23984 | 25041 | 25397 | 25885 | 24768 | 23130 | 22344 | 21207 | 19362 | 18497 | 18409 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro II |  |  |  |  |  |  |  | 25981 | 25671 | 23766 | 24054 | 22954 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro III |  |  |  |  |  |  |  |  |  |  |  | 25954 |
| Passenger Cars | Gasoline >2,0 l | PRE ECE | 10352 | 11120 | 11847 | 12282 | 12679 | 12387 | 12152 | 11935 | 11988 | 11407 | 11507 | 11922 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/00-01 | 12073 | 12735 | 13119 | 13036 | 12679 | 12387 | 12152 | 11935 | 11988 | 11407 | 11507 | 11922 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/02 | 13225 | 13605 | 14495 | 15028 | 15513 | 13532 | 12152 | 11935 | 11988 | 11407 | 11507 | 11922 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/03 | 16370 | 16830 | 16920 | 16820 | 16701 | 15701 | 14867 | 14403 | 14225 | 13135 | 12591 | 11922 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/04 | 19941 | 20341 | 20763 | 20356 | 19947 | 18688 | 17564 | 16233 | 15648 | 14567 | 14356 | 12416 |
| Passenger Cars | Gasoline >2,0 l | Euro I | 22535 | 23984 | 25041 | 25397 | 25885 | 24768 | 23130 | 22344 | 21207 | 19362 | 18497 | 18409 |
| Passenger Cars | Gasoline >2,0 l | Euro II |  |  |  |  |  |  |  | 25981 | 25671 | 23766 | 24054 | 22954 |
| Passenger Cars | Gasoline >2,0 l | Euro III |  |  |  |  |  |  |  |  |  |  |  | 25954 |
| Passenger Cars | Diesel <2,0 l | Euro I |  | 44822 | 44911 | 43972 | 44800 | 44746 | 43410 | 41641 | 39363 | 38090 | 35677 | 34320 |
| Passenger Cars | Diesel <2,0 l | Euro II |  |  |  |  |  |  |  | 47992 | 47256 | 46753 | 45221 | 42794 |
| Passenger Cars | Diesel <2,0 l | Euro III |  |  |  |  |  |  |  |  |  |  |  | 48385 |
| Passenger Cars | Diesel <2,0 l | Conventional | 30874 | 30888 | 30400 | 29591 | 29501 | 29228 | 28169 | 27809 | 27304 | 27242 | 26288 | 22832 |
| Passenger Cars | Diesel >2,0 l | Euro I |  | 44822 | 44911 | 43972 | 44800 | 44746 | 43410 | 41641 | 39363 | 38090 | 35677 | 34320 |
| Passenger Cars | Diesel >2,0 l | Euro II |  |  |  |  |  |  |  | 47992 | 47256 | 46753 | 45221 | 42794 |
| Passenger Cars | Diesel >2,0 l | Euro III |  |  |  |  |  |  |  |  |  |  |  | 48385 |
| Passenger Cars | Diesel >2,0 l | Conventional | 30874 | 30888 | 30400 | 29591 | 29501 | 29228 | 28169 | 27809 | 27304 | 27242 | 26288 | 22832 |
| Passenger Cars | LPG | Conventional | 16370 | 16830 | 16920 | 16820 | 16701 | 15701 | 14867 | 12868 | 14225 | 13135 | 12591 | 11922 |
| Passenger Cars | 2-Stroke | Conventional | 16370 | 16830 | 16920 | 16820 | 16701 | 15701 | 14867 | 12868 | 14225 | 13135 | 12591 | 11922 |

| Sector | Subsector | Tech | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Light Duty Veh. | Gasoline <3,5t | Conventional | 20255 | 20573 | 21208 | 21167 | 21003 | 20157 | 20005 | 19559 | 18762 | 17644 | 18101 | 16619 |
| Light Duty Veh. | Gasoline <3,5t | Euro I |  |  |  |  |  | 20157 | 20005 | 19559 | 18762 | 17644 | 18101 | 16619 |
| Light Duty Veh. | Gasoline <3,5t | Euro II |  |  |  |  |  |  |  |  |  | 17644 | 18101 | 16619 |
| Light Duty Veh. | Diesel <3,5 t | Conventional | 40234 | 40672 | 38721 | 37230 | 38732 | 36909 | 37282 | 37023 | 35010 | 33919 | 32789 | 32130 |
| Light Duty Veh. | Diesel <3,5 t | Euro I |  |  |  |  |  | 36909 | 37282 | 37023 | 35010 | 33919 | 32789 | 32130 |
| Light Duty Veh. | Diesel <3,5 t | Euro II |  |  |  |  |  |  |  |  |  | 33919 | 32789 | 32130 |
| Heavy Duty Veh. | Gasoline >3,5 t | Conventional | 24464 | 24848 | 25614 | 25565 | 25367 | 24346 | 24162 | 21140 | 21559 | 20756 | 22009 | 23191 |
| Heavy Duty Veh. | Diesel 3,5 - 7,5 t | Conventional | 41061 | 41509 | 39516 | 37996 | 39528 | 37666 | 38048 | 30803 | 31809 | 33314 | 34246 | 44099 |
| Heavy Duty Veh. | Diesel 3,5 - 7,5 t | Euro I |  |  |  |  | 39528 | 37666 | 38048 | 30803 | 31809 | 33314 | 34246 | 44099 |
| Heavy Duty Veh. | Diesel 3,5 - 7,5 t | Euro II |  |  |  |  |  |  |  | 30803 | 31809 | 33314 | 34246 | 44099 |
| Heavy Duty Veh. | Diesel 7,5 - 16 t | Conventional | 49634 | 50175 | 47767 | 45929 | 47781 | 45532 | 45991 | 43199 | 42348 | 38844 | 37550 | 20945 |
| Heavy Duty Veh. | Diesel 7,5 - 16 t | Euro I |  |  |  |  | 47781 | 45532 | 45991 | 43199 | 42348 | 38844 | 37550 | 20945 |
| Heavy Duty Veh. | Diesel 7,5 - 16 t | Euro II |  |  |  |  |  |  |  | 43199 | 42348 | 38844 | 37550 | 20945 |
| Heavy Duty Veh. | Diesel 16 - 32 t | Conventional | 68996 | 69748 | 66402 | 63845 | 66422 | 63294 | 63934 | 64717 | 65653 | 66302 | 64092 | 68254 |
| Heavy Duty Veh. | Diesel 16 - 32 t | Euro I |  |  |  |  | 66422 | 63294 | 63934 | 64717 | 65653 | 66302 | 64092 | 68254 |
| Heavy Duty Veh. | Diesel 16 - 32 t | Euro II |  |  |  |  |  |  |  | 64717 | 65653 | 66302 | 64092 | 68254 |
| Heavy Duty Veh. | Diesel >32t | Conventional | 68996 | 69748 | 66402 | 63845 | 66422 | 63294 | 63934 | 64717 | 65653 | 66302 | 64092 | 68254 |
| Heavy Duty Veh. | Diesel >32t | Euro I |  |  |  |  | 66422 | 63294 | 63934 | 64717 | 65653 | 66302 | 64092 | 68254 |
| Heavy Duty Veh. | Diesel >32t | Euro II |  |  |  |  |  |  |  | 64717 | 65653 | 66302 | 64092 | 68254 |
| Buses | Urban Buses | Conventional | 104096 | 107730 | 104029 | 103324 | 108850 | 103224 | 103953 | 103313 | 102311 | 99070 | 95765 | 93590 |
| Buses | Urban Buses | Euro I |  |  |  |  | 108850 | 103224 | 103953 | 103313 | 102311 | 99070 | 95765 | 93590 |
| Buses | Urban Buses | Euro II |  |  |  |  |  |  |  | 103313 | 102311 | 99070 | 95765 | 93590 |
| Buses | Coaches | Conventional | 93315 | 98696 | 98846 | 98624 | 108850 | 90022 | 85021 | 83545 | 82509 | 80371 | 77850 | 76121 |
| Buses | Coaches | Euro I |  |  |  |  | 108850 | 90022 | 85021 | 83545 | 82509 | 80371 | 77850 | 76121 |
| Buses | Coaches | Euro II |  |  |  |  |  |  |  | 83545 | 82509 | 80371 | 77850 | 76121 |
| Mopeds | <50 cm³ | Conventional | 2056 | 2137 | 2235 | 2304 | 2315 | 2211 | 2188 | 2141 | 2151 | 1751 | 1614 | 1175 |
| Mopeds | <50 cm³ | 97/24/EC I |  |  |  |  |  |  |  |  |  |  | 1614 | 1175 |
| Motorcycles | 2-stroke >50 cm³ | Conventional | 5815 | 6072 | 6372 | 6557 | 6584 | 6279 | 6220 | 6106 | 6168 | 5827 | 6029 | 5563 |
| Motorcycles | 4-stroke <250 cm³ | Conventional | 5815 | 6072 | 6372 | 6557 | 6584 | 6279 | 6220 | 6106 | 6168 | 5827 | 6029 | 5563 |
| Motorcycles | 4-stroke <250 cm³ | 97/24/EC |  |  |  |  |  |  |  |  |  |  | 6029 | 5563 |
| Motorcycles | 4-stroke 250-750 cm³ | Conventional | 5815 | 6072 | 6372 | 6557 | 6584 | 6279 | 6220 | 6106 | 6168 | 5827 | 6029 | 5563 |
| Motorcycles | 4-stroke 250-750 cm³ | 97/24/EC |  |  |  |  |  |  |  |  |  |  | 6029 | 5563 |
| Motorcycles | 4-stroke >750 cm³ | Conventional | 5815 | 6072 | 6372 | 6557 | 6584 | 6279 | 6220 | 6106 | 6168 | 5827 | 6029 | 5563 |
| Motorcycles | 4-stroke >750 cm³ | 97/24/EC |  |  |  |  |  |  |  |  |  |  | 6029 | 5563 |

Annex 2: Basis emission factors (g/km)

| Sector | Subsector | Tech | FCu | FCr | FCh | CO2u | CO2r | CO2h | CH4u | CH4r | CH4h | N2Ou | N2Or | N2Oh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passenger Cars | Gasoline <1,4 l | PRE ECE | 67,499 | 55,000 | 62,743 | 216 | 176 | 201 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/00-01 | 58,240 | 44,460 | 48,600 | 186 | 142 | 155 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/02 | 53,248 | 45,170 | 51,200 | 170 | 144 | 164 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/03 | 53,248 | 45,170 | 51,200 | 170 | 144 | 164 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/04 | 51,420 | 43,440 | 47,700 | 164 | 139 | 153 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline <1,4 l | Euro I | 51,136 | 37,996 | 43,936 | 164 | 121 | 140 | 0,038 | 0,018 | 0,021 | 0,053 | 0,016 | 0,035 |
| Passenger Cars | Gasoline <1,4 l | Euro II | 51,136 | 37,996 | 43,936 | 164 | 121 | 140 | 0,038 | 0,018 | 0,021 | 0,053 | 0,016 | 0,035 |
| Passenger Cars | Gasoline <1,4 l | Euro III | 51,136 | 37,996 | 43,936 | 164 | 121 | 140 | 0,038 | 0,018 | 0,021 | 0,053 | 0,016 | 0,035 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | PRE ECE | 79,277 | 67,000 | 76,386 | 253 | 214 | 244 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/00-01 | 67,779 | 51,090 | 60,300 | 217 | 163 | 193 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/02 | 61,731 | 50,686 | 59,680 | 197 | 162 | 191 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/03 | 61,731 | 50,686 | 59,680 | 197 | 162 | 191 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/04 | 61,652 | 49,112 | 52,052 | 197 | 157 | 166 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro I | 65,920 | 44,020 | 48,040 | 211 | 141 | 154 | 0,039 | 0,017 | 0,016 | 0,053 | 0,016 | 0,035 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro II | 65,920 | 44,020 | 48,040 | 211 | 141 | 154 | 0,039 | 0,017 | 0,016 | 0,053 | 0,016 | 0,035 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro III | 65,920 | 44,020 | 48,040 | 211 | 141 | 154 | 0,039 | 0,017 | 0,016 | 0,053 | 0,016 | 0,035 |
| Passenger Cars | Gasoline >2,0 l | PRE ECE | 96,536 | 80,000 | 88,267 | 309 | 256 | 282 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/00-01 | 73,798 | 57,090 | 66,300 | 236 | 183 | 212 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/02 | 75,270 | 63,260 | 70,700 | 241 | 202 | 226 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/03 | 75,270 | 63,260 | 70,700 | 241 | 202 | 226 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/04 | 71,055 | 58,080 | 69,900 | 227 | 186 | 223 | 0,092 | 0,029 | 0,026 | 0,005 | 0,005 | 0,005 |
| Passenger Cars | Gasoline >2,0 l | Euro I | 79,370 | 46,400 | 51,050 | 254 | 148 | 163 | 0,040 | 0,017 | 0,010 | 0,053 | 0,016 | 0,035 |
| Passenger Cars | Gasoline >2,0 l | Euro II | 79,370 | 46,400 | 51,050 | 254 | 148 | 163 | 0,040 | 0,017 | 0,010 | 0,053 | 0,016 | 0,035 |
| Passenger Cars | Gasoline >2,0 l | Euro III | 79,370 | 46,400 | 51,050 | 254 | 148 | 163 | 0,040 | 0,017 | 0,010 | 0,053 | 0,016 | 0,035 |
| Passenger Cars | Diesel <2,0 l | Euro I | 52,722 | 42,225 | 47,406 | 167 | 133 | 150 | 0,004 | 0,005 | 0,009 | 0,027 | 0,027 | 0,027 |
| Passenger Cars | Diesel <2,0 l | Euro II | 52,722 | 42,225 | 47,406 | 167 | 133 | 150 | 0,004 | 0,005 | 0,009 | 0,027 | 0,027 | 0,027 |
| Passenger Cars | Diesel <2,0 l | Euro III | 52,722 | 42,225 | 47,406 | 167 | 133 | 150 | 0,004 | 0,005 | 0,009 | 0,027 | 0,027 | 0,027 |
| Passenger Cars | Diesel <2,0 l | Conventional | 57,529 | 41,209 | 50,089 | 182 | 130 | 158 | 0,004 | 0,005 | 0,009 | 0,027 | 0,027 | 0,027 |
| Passenger Cars | Diesel >2,0 l | Euro I | 52,722 | 42,225 | 47,406 | 167 | 133 | 150 | 0,004 | 0,005 | 0,009 | 0,027 | 0,027 | 0,027 |
| Passenger Cars | Diesel >2,0 l | Euro II | 52,722 | 42,225 | 47,406 | 167 | 133 | 150 | 0,004 | 0,005 | 0,009 | 0,027 | 0,027 | 0,027 |
| Passenger Cars | Diesel >2,0 l | Euro III | 52,722 | 42,225 | 47,406 | 167 | 133 | 150 | 0,004 | 0,005 | 0,009 | 0,027 | 0,027 | 0,027 |
| Passenger Cars | Diesel >2,0 l | Conventional | 57,529 | 41,209 | 50,089 | 182 | 130 | 158 | 0,004 | 0,005 | 0,009 | 0,027 | 0,027 | 0,027 |
| Passenger Cars | LPG | Conventional | 59,000 | 45,000 | 54,000 | 176 | 135 | 161 | 0,080 | 0,035 | 0,025 | 0,015 | 0,015 | 0,015 |
| Passenger Cars | 2-Stroke | Conventional | 111,500 | 66,000 | 56,900 | 357 | 211 | 182 | 0,150 | 0,040 | 0,025 | 0,005 | 0,005 | 0,005 |
| Sector | Subsector | Tech | FCu | FCr | FCh | CO2u | CO2r | CO2h | CH4u | CH4r | CH4h | N2Ou | N2Or | N2Oh |
| Light Duty Vehicles | Gasoline <3,5t | Conventional | 82,270 | 59,883 | 56,470 | 263 | 191 | 181 | 0,150 | 0,040 | 0,025 | 0,006 | 0,006 | 0,006 |
| Light Duty Vehicles | Gasoline <3,5t | Euro I | 96,450 | 70,388 | 66,450 | 308 | 225 | 212 | 0,038 | 0,020 | 0,016 | 0,053 | 0,016 | 0,035 |
| Light Duty Vehicles | Gasoline <3,5t | Euro II | 96,450 | 70,388 | 66,450 | 308 | 225 | 212 | 0,038 | 0,020 | 0,016 | 0,053 | 0,016 | 0,035 |
| Light Duty Vehicles | Diesel <3,5 t | Conventional | 76,718 | 65,934 | 72,142 | 242 | 208 | 228 | 0,005 | 0,005 | 0,005 | 0,017 | 0,017 | 0,017 |
| Light Duty Vehicles | Diesel <3,5 t | Euro I | 68,860 | 58,185 | 63,660 | 218 | 184 | 201 | 0,005 | 0,005 | 0,005 | 0,017 | 0,017 | 0,017 |
| Light Duty Vehicles | Diesel <3,5 t | Euro II | 68,860 | 58,185 | 63,660 | 218 | 184 | 201 | 0,005 | 0,005 | 0,005 | 0,017 | 0,017 | 0,017 |
| Heavy Duty Vehicles | Gasoline >3,5 t | Conventional | 225,000 | 150,000 | 165,000 | 719 | 480 | 528 | 0,140 | 0,110 | 0,070 | 0,006 | 0,006 | 0,006 |
| Heavy Duty Vehicles | Diesel 3,5 - 7,5 t | Conventional | 95,822 | 87,060 | 109,160 | 303 | 275 | 345 | 0,085 | 0,023 | 0,020 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel 3,5 - 7,5 t | Euro I | 95,822 | 87,060 | 109,160 | 303 | 275 | 345 | 0,085 | 0,023 | 0,020 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel 3,5 - 7,5 t | Euro II | 95,822 | 87,060 | 109,160 | 303 | 275 | 345 | 0,085 | 0,023 | 0,020 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel 7,5 - 16 t | Conventional | 186,796 | 147,006 | 169,108 | 590 | 465 | 534 | 0,085 | 0,023 | 0,020 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel 7,5 - 16 t | Euro I | 186,796 | 147,006 | 169,108 | 590 | 465 | 534 | 0,085 | 0,023 | 0,020 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel 7,5 - 16 t | Euro II | 186,796 | 147,006 | 169,108 | 590 | 465 | 534 | 0,085 | 0,023 | 0,020 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel 16 - 32 t | Conventional | 295,313 | 227,040 | 230,740 | 933 | 717 | 729 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel 16 - 32 t | Euro I | 295,313 | 227,040 | 230,740 | 933 | 717 | 729 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel 16 - 32 t | Euro II | 295,313 | 227,040 | 230,740 | 933 | 717 | 729 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel >32t | Conventional | 392,838 | 311,460 | 297,380 | 1241 | 984 | 940 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel >32t | Euro I | 392,838 | 311,460 | 297,380 | 1241 | 984 | 940 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Heavy Duty Vehicles | Diesel >32t | Euro II | 392,838 | 311,460 | 297,380 | 1241 | 984 | 940 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Buses | Urban Buses | Conventional | 315,796 | 253,287 | 219,035 | 998 | 800 | 692 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Buses | Urban Buses | Euro I | 315,796 | 253,287 | 219,035 | 998 | 800 | 692 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Buses | Urban Buses | Euro II | 315,796 | 253,287 | 219,035 | 998 | 800 | 692 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Buses | Coaches | Conventional | 281,771 | 214,600 | 198,320 | 890 | 678 | 627 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Buses | Coaches | Euro I | 281,771 | 214,600 | 198,320 | 890 | 678 | 627 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Buses | Coaches | Euro II | 281,771 | 214,600 | 198,320 | 890 | 678 | 627 | 0,175 | 0,080 | 0,070 | 0,030 | 0,030 | 0,030 |
| Mopeds | <50 cm³ | Conventional | 25,000 | 25,000 | 0,000 | 80 | 80 | 0 | 0,219 | 0,000 | 0,000 | 0,001 | 0,000 | 0,000 |
| Mopeds | <50 cm³ | 97/24/EC I | 25,000 | 25,000 | 0,000 | 80 | 80 | 0 | 0,219 | 0,000 | 0,000 | 0,001 | 0,000 | 0,000 |
| Motorcycles | 2-stroke >50 cm³ | Conventional | 30,368 | 32,375 | 36,950 | 97 | 104 | 118 | 0,150 | 0,150 | 0,150 | 0,002 | 0,002 | 0,002 |
| Motorcycles | 4-stroke <250 cm³ | Conventional | 23,180 | 26,718 | 35,640 | 74 | 85 | 114 | 0,200 | 0,200 | 0,200 | 0,002 | 0,002 | 0,002 |
| Motorcycles | 4-stroke <250 cm³ | 97/24/EC | 23,180 | 26,718 | 35,640 | 74 | 85 | 114 | 0,200 | 0,200 | 0,200 | 0,002 | 0,002 | 0,002 |
| Motorcycles | 4-stroke 250 - 750 cm³ | Conventional | 28,620 | 28,640 | 34,700 | 92 | 92 | 111 | 0,200 | 0,200 | 0,200 | 0,002 | 0,002 | 0,002 |
| Motorcycles | 4-stroke 250 - 750 cm³ | 97/24/EC | 28,620 | 28,640 | 34,700 | 92 | 92 | 111 | 0,200 | 0,200 | 0,200 | 0,002 | 0,002 | 0,002 |
| Motorcycles | 4-stroke >750 cm³ | Conventional | 37,500 | 34,354 | 38,620 | 120 | 110 | 123 | 0,200 | 0,200 | 0,200 | 0,002 | 0,002 | 0,002 |
| Motorcycles | 4-stroke >750 cm³ | 97/24/EC | 37,500 | 34,354 | 38,620 | 120 | 110 | 123 | 0,200 | 0,200 | 0,200 | 0,002 | 0,002 | 0,002 |

| Sector | Subsector | Tech | COu | COr | COh | NOxu | NOxr | NOxh | NMVOCu | NMVOCr | NMVOCh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passenger Cars | Gasoline <1,4 l | PRE ECE | 27,505 | 19,333 | 15,520 | 1,849 | 2,062 | 2,023 | 2,262 | 1,568 | 1,221 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/00-01 | 18,966 | 14,480 | 18,620 | 1,849 | 2,062 | 2,023 | 1,770 | 1,227 | 1,095 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/02 | 15,859 | 8,200 | 8,260 | 1,619 | 2,102 | 2,909 | 1,757 | 1,032 | 0,924 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/03 | 16,752 | 8,793 | 7,620 | 1,680 | 2,253 | 3,276 | 1,757 | 1,032 | 0,924 |
| Passenger Cars | Gasoline <1,4 l | ECE 15/04 | 9,087 | 4,956 | 4,292 | 1,691 | 2,089 | 2,662 | 1,388 | 0,866 | 0,672 |
| Passenger Cars | Gasoline <1,4 l | Euro I | 1,898 | 0,557 | 3,176 | 0,314 | 0,356 | 0,593 | 0,175 | 0,064 | 0,082 |
| Passenger Cars | Gasoline <1,4 l | Euro II | 1,898 | 0,557 | 3,176 | 0,314 | 0,356 | 0,593 | 0,175 | 0,064 | 0,082 |
| Passenger Cars | Gasoline <1,4 l | Euro III | 1,898 | 0,557 | 3,176 | 0,314 | 0,356 | 0,593 | 0,175 | 0,064 | 0,082 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | PRE ECE | 27,505 | 19,333 | 15,520 | 2,164 | 2,683 | 3,130 | 2,262 | 1,568 | 1,221 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/00-01 | 18,966 | 14,480 | 18,620 | 2,164 | 2,683 | 3,130 | 1,770 | 1,227 | 1,095 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/02 | 15,859 | 8,200 | 8,260 | 1,831 | 2,377 | 3,283 | 1,757 | 1,032 | 0,924 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/03 | 16,752 | 8,793 | 7,620 | 1,917 | 2,580 | 3,472 | 1,757 | 1,032 | 0,924 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | ECE 15/04 | 9,087 | 4,956 | 4,292 | 2,122 | 2,757 | 3,524 | 1,388 | 0,866 | 0,672 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro I | 2,583 | 0,937 | 2,402 | 0,323 | 0,349 | 0,530 | 0,138 | 0,066 | 0,067 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro II | 2,583 | 0,937 | 2,402 | 0,323 | 0,349 | 0,530 | 0,138 | 0,066 | 0,067 |
| Passenger Cars | Gasoline 1,4 - 2,0 l | Euro III | 2,583 | 0,937 | 2,402 | 0,323 | 0,349 | 0,530 | 0,138 | 0,066 | 0,067 |
| Passenger Cars | Gasoline >2,0 l | PRE ECE | 27,505 | 19,333 | 15,520 | 2,860 | 4,090 | 5,500 | 2,262 | 1,568 | 1,221 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/00-01 | 18,966 | 14,480 | 18,620 | 2,860 | 4,090 | 5,500 | 1,770 | 1,227 | 1,095 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/02 | 15,859 | 8,200 | 8,260 | 2,066 | 2,675 | 3,680 | 1,757 | 1,032 | 0,924 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/03 | 16,752 | 8,793 | 7,620 | 2,806 | 3,441 | 4,604 | 1,757 | 1,032 | 0,924 |
| Passenger Cars | Gasoline >2,0 l | ECE 15/04 | 9,087 | 4,956 | 4,292 | 2,293 | 2,750 | 3,687 | 1,388 | 0,866 | 0,672 |
| Passenger Cars | Gasoline >2,0 l | Euro I | 3,838 | 0,814 | 0,976 | 0,427 | 0,406 | 0,521 | 0,232 | 0,147 | 0,105 |
| Passenger Cars | Gasoline >2,0 l | Euro II | 3,838 | 0,814 | 0,976 | 0,427 | 0,406 | 0,521 | 0,232 | 0,147 | 0,105 |
| Passenger Cars | Gasoline >2,0 l | Euro III | 3,838 | 0,814 | 0,976 | 0,427 | 0,406 | 0,521 | 0,232 | 0,147 | 0,105 |
| Passenger Cars | Diesel <2,0 l | Euro I | 0,432 | 0,109 | 0,165 | 0,679 | 0,488 | 0,619 | 0,073 | 0,028 | 0,020 |
| Passenger Cars | Diesel <2,0 l | Euro II | 0,432 | 0,109 | 0,165 | 0,679 | 0,488 | 0,619 | 0,073 | 0,028 | 0,020 |
| Passenger Cars | Diesel <2,0 l | Euro III | 0,432 | 0,109 | 0,165 | 0,679 | 0,488 | 0,619 | 0,073 | 0,028 | 0,020 |
| Passenger Cars | Diesel <2,0 l | Conventional | 0,651 | 0,472 | 0,384 | 0,520 | 0,433 | 0,528 | 0,141 | 0,081 | 0,052 |
| Passenger Cars | Diesel >2,0 l | Euro I | 0,432 | 0,109 | 0,165 | 0,679 | 0,488 | 0,619 | 0,073 | 0,028 | 0,020 |
| Passenger Cars | Diesel >2,0 l | Euro II | 0,432 | 0,109 | 0,165 | 0,679 | 0,488 | 0,619 | 0,073 | 0,028 | 0,020 |
| Passenger Cars | Diesel >2,0 l | Euro III | 0,432 | 0,109 | 0,165 | 0,679 | 0,488 | 0,619 | 0,073 | 0,028 | 0,020 |
| Passenger Cars | Diesel >2,0 l | Conventional | 0,651 | 0,472 | 0,384 | 0,824 | 0,723 | 0,861 | 0,141 | 0,081 | 0,052 |
| Passenger Cars | LPG | Conventional | 2,043 | 2,373 | 9,723 | 2,203 | 2,584 | 2,861 | 1,002 | 0,632 | 0,465 |

| Sector | Subsector | Tech | COu | COr | COh | NOxu | NOxr | NOxh | NMVOCu | NMVOCr | NMVOCh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Passenger Cars | 2-Stroke | Conventional | 20,700 | 7,500 | 8,700 | 0,300 | 1,020 | 0,720 | 15,250 | 7,160 | 5,875 |
| Light Duty Vehicles | Gasoline <3,5t | Conventional | 14,925 | 6,075 | 7,389 | 2,671 | 3,118 | 3,387 | 1,727 | 0,689 | 0,421 |
| Light Duty Vehicles | Gasoline <3,5t | Euro I | 4,187 | 0,862 | 1,087 | 0,427 | 0,400 | 0,429 | 0,181 | 0,090 | 0,062 |
| Light Duty Vehicles | Gasoline <3,5t | Euro II | 4,187 | 0,862 | 1,087 | 0,427 | 0,400 | 0,429 | 0,181 | 0,090 | 0,062 |
| Light Duty Vehicles | Diesel <3,5 t | Conventional | 1,124 | 1,009 | 1,060 | 1,673 | 0,843 | 0,834 | 0,126 | 0,101 | 0,096 |
| Light Duty Vehicles | Diesel <3,5 t | Euro I | 0,393 | 0,328 | 0,423 | 1,138 | 0,975 | 1,022 | 0,126 | 0,101 | 0,096 |
| Light Duty Vehicles | Diesel <3,5 t | Euro II | 0,393 | 0,328 | 0,423 | 1,138 | 0,975 | 1,022 | 0,126 | 0,101 | 0,096 |
| Heavy Duty Vehicles | Gasoline >3,5 t | Conventional | 70,000 | 55,000 | 55,000 | 4,500 | 7,500 | 7,500 | 6,860 | 5,390 | 3,430 |
| Heavy Duty Vehicles | Diesel 3,5 - 7,5 t | Conventional | 3,156 | 2,170 | 1,777 | 3,247 | 2,169 | 2,615 | 1,688 | 1,082 | 0,838 |
| Heavy Duty Vehicles | Diesel 3,5 - 7,5 t | Euro I | 3,156 | 2,170 | 1,777 | 3,247 | 2,169 | 2,615 | 1,688 | 1,082 | 0,838 |
| Heavy Duty Vehicles | Diesel 3,5 - 7,5 t | Euro II | 3,156 | 2,170 | 1,777 | 3,247 | 2,169 | 2,615 | 1,688 | 1,082 | 0,838 |
| Heavy Duty Vehicles | Diesel 7,5 - 16 t | Conventional | 3,156 | 2,170 | 1,777 | 6,684 | 4,293 | 4,091 | 1,688 | 1,082 | 0,838 |
| Heavy Duty Vehicles | Diesel 7,5 - 16 t | Euro I | 3,156 | 2,170 | 1,777 | 6,684 | 4,293 | 4,091 | 1,688 | 1,082 | 0,838 |
| Heavy Duty Vehicles | Diesel 7,5 - 16 t | Euro II | 3,156 | 2,170 | 1,777 | 6,684 | 4,293 | 4,091 | 1,688 | 1,082 | 0,838 |
| Heavy Duty Vehicles | Diesel 16 - 32 t | Conventional | 3,156 | 2,170 | 1,777 | 12,561 | 9,060 | 7,610 | 1,598 | 1,025 | 0,788 |
| Heavy Duty Vehicles | Diesel 16 - 32 t | Euro I | 3,156 | 2,170 | 1,777 | 12,561 | 9,060 | 7,610 | 1,598 | 1,025 | 0,788 |
| Heavy Duty Vehicles | Diesel 16 - 32 t | Euro II | 3,156 | 2,170 | 1,777 | 12,561 | 9,060 | 7,610 | 1,598 | 1,025 | 0,788 |
| Heavy Duty Vehicles | Diesel >32t | Conventional | 3,156 | 2,170 | 1,777 | 18,269 | 13,523 | 11,517 | 1,598 | 1,025 | 0,788 |
| Heavy Duty Vehicles | Diesel >32t | Euro I | 3,156 | 2,170 | 1,777 | 18,269 | 13,523 | 11,517 | 1,598 | 1,025 | 0,788 |
| Heavy Duty Vehicles | Diesel >32t | Euro II | 3,156 | 2,170 | 1,777 | 18,269 | 13,523 | 11,517 | 1,598 | 1,025 | 0,788 |
| Buses | Urban Buses | Conventional | 4,687 | 3,204 | 2,494 | 15,288 | 11,731 | 9,853 | 1,138 | 0,696 | 0,479 |
| Buses | Urban Buses | Euro I | 4,687 | 3,204 | 2,494 | 15,288 | 11,731 | 9,853 | 1,138 | 0,696 | 0,479 |
| Buses | Urban Buses | Euro II | 4,687 | 3,204 | 2,494 | 15,288 | 11,731 | 9,853 | 1,138 | 0,696 | 0,479 |
| Buses | Coaches | Conventional | 3,227 | 2,053 | 1,612 | 12,210 | 8,260 | 7,844 | 1,713 | 1,090 | 0,837 |
| Buses | Coaches | Euro I | 3,227 | 2,053 | 1,612 | 12,210 | 8,260 | 7,844 | 1,713 | 1,090 | 0,837 |
| Buses | Coaches | Euro II | 3,227 | 2,053 | 1,612 | 12,210 | 8,260 | 7,844 | 1,713 | 1,090 | 0,837 |
| Mopeds | <50 cm³ | Conventional | 15,000 | 15,000 | 0,000 | 0,030 | 0,030 | 0,000 | 8,781 | 9,000 | 0,000 |
| Mopeds | <50 cm³ | 97/24/EC I | 15,000 | 15,000 | 0,000 | 0,030 | 0,030 | 0,000 | 8,781 | 9,000 | 0,000 |
| Motorcycles | 2-stroke >50 cm³ | Conventional | 23,380 | 25,490 | 27,500 | 0,032 | 0,088 | 0,133 | 9,190 | 8,252 | 8,210 |
| Motorcycles | 4-stroke <250 cm³ | Conventional | 22,380 | 26,300 | 38,600 | 0,130 | 0,242 | 0,362 | 1,350 | 0,760 | 1,120 |
| Motorcycles | 4-stroke <250 cm³ | 97/24/EC | 22,380 | 26,300 | 38,600 | 0,130 | 0,242 | 0,362 | 1,350 | 0,760 | 1,120 |
| Motorcycles | 4-stroke 250 - 750 cm³ | Conventional | 20,440 | 21,517 | 25,810 | 0,136 | 0,251 | 0,374 | 1,150 | 0,744 | 0,810 |
| Motorcycles | 4-stroke 250 - 750 cm³ | 97/24/EC | 20,440 | 21,517 | 25,810 | 0,136 | 0,251 | 0,374 | 1,150 | 0,744 | 0,810 |
| Motorcycles | 4-stroke >750 cm³ | Conventional | 14,880 | 18,030 | 24,300 | 0,148 | 0,266 | 0,392 | 2,320 | 1,410 | 0,990 |
| Motorcycles | 4-stroke >750 cm³ | 97/24/EC | 14,880 | 18,030 | 24,300 | 0,148 | 0,266 | 0,392 | 2,320 | 1,410 | 0,990 |

Annex 3: Fuel use factors (MJ/km) and emission factors (g/km)

| Year | Sector | FCu (MJ) | FCr (MJ) | FCh (MJ) | CO2u | CO2r | CO2h | CH4u | CH4r | CH4h | N2Ou | N2Or | N2Oh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | Passenger Cars | 3,082 | 2,054 | 2,318 | 223 | 149 | 168 | 0,143 | 0,027 | 0,024 | 0,008 | 0,007 | 0,008 |
| 1991 | Passenger Cars | 3,121 | 2,036 | 2,297 | 226 | 148 | 166 | 0,157 | 0,026 | 0,024 | 0,011 | 0,008 | 0,010 |
| 1992 | Passenger Cars | 3,133 | 2,020 | 2,276 | 227 | 146 | 165 | 0,165 | 0,025 | 0,023 | 0,014 | 0,009 | 0,011 |
| 1993 | Passenger Cars | 3,161 | 2,003 | 2,255 | 229 | 145 | 163 | 0,181 | 0,025 | 0,023 | 0,017 | 0,009 | 0,013 |
| 1994 | Passenger Cars | 3,162 | 1,981 | 2,226 | 229 | 143 | 161 | 0,189 | 0,024 | 0,022 | 0,021 | 0,010 | 0,016 |
| 1995 | Passenger Cars | 3,213 | 1,960 | 2,202 | 233 | 142 | 160 | 0,208 | 0,023 | 0,021 | 0,025 | 0,011 | 0,018 |
| 1996 | Passenger Cars | 3,275 | 1,945 | 2,182 | 237 | 141 | 158 | 0,234 | 0,022 | 0,021 | 0,028 | 0,012 | 0,020 |
| 1997 | Passenger Cars | 3,225 | 1,912 | 2,142 | 234 | 139 | 155 | 0,215 | 0,020 | 0,019 | 0,032 | 0,013 | 0,023 |
| 1998 | Passenger Cars | 3,201 | 1,901 | 2,128 | 232 | 138 | 154 | 0,198 | 0,018 | 0,017 | 0,034 | 0,013 | 0,024 |
| 1999 | Passenger Cars | 3,383 | 1,886 | 2,110 | 245 | 137 | 153 | 0,234 | 0,016 | 0,015 | 0,036 | 0,014 | 0,026 |
| 2000 | Passenger Cars | 3,207 | 1,875 | 2,097 | 232 | 136 | 152 | 0,187 | 0,014 | 0,014 | 0,038 | 0,015 | 0,027 |
| 2001 | Passenger Cars | 3,241 | 1,865 | 2,085 | 235 | 135 | 151 | 0,188 | 0,013 | 0,013 | 0,040 | 0,015 | 0,028 |
| 1990 | Light Duty Vehicles | 3,842 | 2,792 | 3,007 | 282 | 205 | 221 | 0,038 | 0,009 | 0,007 | 0,016 | 0,016 | 0,016 |
| 1991 | Light Duty Vehicles | 3,872 | 2,792 | 3,007 | 284 | 205 | 221 | 0,040 | 0,009 | 0,007 | 0,016 | 0,016 | 0,016 |
| 1992 | Light Duty Vehicles | 3,873 | 2,790 | 3,002 | 284 | 205 | 220 | 0,042 | 0,010 | 0,008 | 0,016 | 0,016 | 0,016 |
| 1993 | Light Duty Vehicles | 3,907 | 2,790 | 2,999 | 286 | 205 | 220 | 0,044 | 0,010 | 0,008 | 0,016 | 0,016 | 0,016 |
| 1994 | Light Duty Vehicles | 3,945 | 2,791 | 3,002 | 289 | 205 | 220 | 0,044 | 0,010 | 0,008 | 0,016 | 0,016 | 0,016 |
| 1995 | Light Duty Vehicles | 3,937 | 2,771 | 2,979 | 289 | 203 | 219 | 0,044 | 0,009 | 0,007 | 0,016 | 0,016 | 0,016 |
| 1996 | Light Duty Vehicles | 3,973 | 2,752 | 2,959 | 291 | 202 | 217 | 0,046 | 0,009 | 0,007 | 0,017 | 0,016 | 0,016 |
| 1997 | Light Duty Vehicles | 3,900 | 2,732 | 2,939 | 286 | 200 | 216 | 0,041 | 0,009 | 0,007 | 0,017 | 0,016 | 0,017 |
| 1998 | Light Duty Vehicles | 3,851 | 2,714 | 2,918 | 282 | 199 | 214 | 0,040 | 0,008 | 0,007 | 0,018 | 0,016 | 0,017 |
| 1999 | Light Duty Vehicles | 4,000 | 2,697 | 2,901 | 293 | 198 | 213 | 0,042 | 0,008 | 0,007 | 0,018 | 0,016 | 0,017 |
| 2000 | Light Duty Vehicles | 3,816 | 2,681 | 2,883 | 280 | 197 | 211 | 0,036 | 0,008 | 0,007 | 0,018 | 0,016 | 0,017 |
| 2001 | Light Duty Vehicles | 3,821 | 2,663 | 2,866 | 280 | 195 | 210 | 0,034 | 0,007 | 0,006 | 0,019 | 0,016 | 0,018 |

| Year | Sector | FCu (MJ) | FCr (MJ) | FCh (MJ) | CO2u | CO2r | CO2h | CH4u | CH4r | CH4h | N2Ou | N2Or | N2Oh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | Heavy Duty Vehicles | 11,965 | 9,685 | 10,256 | 879 | 712 | 754 | 0,151 | 0,067 | 0,063 | 0,030 | 0,030 | 0,030 |
| 1991 | Heavy Duty Vehicles | 12,016 | 9,691 | 10,226 | 883 | 712 | 751 | 0,151 | 0,066 | 0,062 | 0,030 | 0,030 | 0,030 |
| 1992 | Heavy Duty Vehicles | 12,023 | 9,677 | 10,218 | 883 | 711 | 751 | 0,152 | 0,066 | 0,062 | 0,030 | 0,030 | 0,030 |
| 1993 | Heavy Duty Vehicles | 11,898 | 9,723 | 10,189 | 874 | 714 | 749 | 0,150 | 0,067 | 0,062 | 0,030 | 0,030 | 0,030 |
| 1994 | Heavy Duty Vehicles | 11,929 | 9,663 | 10,141 | 877 | 710 | 745 | 0,146 | 0,065 | 0,061 | 0,030 | 0,030 | 0,030 |
| 1995 | Heavy Duty Vehicles | 11,911 | 9,673 | 10,013 | 875 | 711 | 736 | 0,142 | 0,064 | 0,059 | 0,030 | 0,030 | 0,030 |
| 1996 | Heavy Duty Vehicles | 11,938 | 9,657 | 10,026 | 877 | 710 | 737 | 0,137 | 0,061 | 0,058 | 0,030 | 0,030 | 0,030 |
| 1997 | Heavy Duty Vehicles | 12,176 | 9,836 | 10,152 | 895 | 723 | 746 | 0,133 | 0,061 | 0,057 | 0,030 | 0,030 | 0,030 |
| 1998 | Heavy Duty Vehicles | 12,246 | 9,904 | 10,209 | 900 | 728 | 750 | 0,129 | 0,059 | 0,056 | 0,030 | 0,030 | 0,030 |
| 1999 | Heavy Duty Vehicles | 12,389 | 10,028 | 10,306 | 910 | 737 | 757 | 0,126 | 0,059 | 0,055 | 0,030 | 0,030 | 0,030 |
| 2000 | Heavy Duty Vehicles | 12,422 | 10,055 | 10,329 | 913 | 739 | 759 | 0,121 | 0,057 | 0,053 | 0,030 | 0,030 | 0,030 |
| 2001 | Heavy Duty Vehicles | 12,685 | 10,273 | 10,476 | 932 | 755 | 770 | 0,121 | 0,057 | 0,053 | 0,030 | 0,030 | 0,030 |
| 1990 | 2-wheelers | 1,173 | 1,245 | 1,578 | 85 | 90 | 114 | 0,209 | 0,130 | 0,192 | 0,001 | 0,001 | 0,002 |
| 1991 | 2-wheelers | 1,182 | 1,242 | 1,578 | 86 | 90 | 114 | 0,208 | 0,127 | 0,193 | 0,001 | 0,001 | 0,002 |
| 1992 | 2-wheelers | 1,178 | 1,253 | 1,578 | 85 | 91 | 114 | 0,209 | 0,137 | 0,192 | 0,001 | 0,001 | 0,002 |
| 1993 | 2-wheelers | 1,190 | 1,249 | 1,578 | 86 | 90 | 114 | 0,207 | 0,133 | 0,192 | 0,001 | 0,001 | 0,002 |
| 1994 | 2-wheelers | 1,188 | 1,258 | 1,578 | 86 | 91 | 114 | 0,207 | 0,141 | 0,193 | 0,001 | 0,001 | 0,002 |
| 1995 | 2-wheelers | 1,182 | 1,254 | 1,578 | 86 | 91 | 114 | 0,208 | 0,137 | 0,193 | 0,001 | 0,001 | 0,002 |
| 1996 | 2-wheelers | 1,185 | 1,256 | 1,578 | 86 | 91 | 114 | 0,208 | 0,139 | 0,193 | 0,001 | 0,001 | 0,002 |
| 1997 | 2-wheelers | 1,186 | 1,257 | 1,578 | 86 | 91 | 114 | 0,208 | 0,140 | 0,192 | 0,001 | 0,001 | 0,002 |
| 1998 | 2-wheelers | 1,186 | 1,257 | 1,578 | 86 | 91 | 114 | 0,208 | 0,140 | 0,193 | 0,001 | 0,001 | 0,002 |
| 1999 | 2-wheelers | 1,195 | 1,264 | 1,578 | 86 | 91 | 114 | 0,207 | 0,146 | 0,192 | 0,001 | 0,002 | 0,002 |
| 2000 | 2-wheelers | 1,198 | 1,265 | 1,572 | 87 | 92 | 114 | 0,202 | 0,150 | 0,193 | 0,001 | 0,002 | 0,002 |
| 2001 | 2-wheelers | 1,210 | 1,272 | 1,568 | 88 | 92 | 113 | 0,199 | 0,158 | 0,193 | 0,002 | 0,002 | 0,002 |

| Year | Sector | SO2u | SO2r | SO2h | NOxu | NOxr | NOxh | NMVOCu | NMVOCr | NMVOCh | COu | COr | COh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | Passenger Cars | 0,031 | 0,020 | 0,023 | 1,791 | 2,142 | 2,763 | 2,527 | 0,929 | 0,787 | 26,382 | 7,625 | 7,554 |
| 1991 | Passenger Cars | 0,030 | 0,019 | 0,023 | 1,718 | 2,014 | 2,609 | 2,520 | 0,864 | 0,732 | 26,666 | 7,061 | 7,065 |
| 1992 | Passenger Cars | 0,021 | 0,014 | 0,016 | 1,649 | 1,901 | 2,475 | 2,416 | 0,804 | 0,681 | 25,631 | 6,538 | 6,663 |
| 1993 | Passenger Cars | 0,012 | 0,008 | 0,009 | 1,595 | 1,798 | 2,356 | 2,402 | 0,746 | 0,633 | 25,790 | 6,025 | 6,323 |
| 1994 | Passenger Cars | 0,012 | 0,008 | 0,009 | 1,488 | 1,624 | 2,149 | 2,200 | 0,654 | 0,555 | 23,663 | 5,243 | 5,741 |
| 1995 | Passenger Cars | 0,013 | 0,008 | 0,009 | 1,430 | 1,495 | 1,993 | 2,157 | 0,584 | 0,497 | 23,607 | 4,701 | 5,507 |
| 1996 | Passenger Cars | 0,013 | 0,008 | 0,009 | 1,393 | 1,395 | 1,875 | 2,176 | 0,526 | 0,449 | 24,352 | 4,267 | 5,365 |
| 1997 | Passenger Cars | 0,013 | 0,008 | 0,009 | 1,283 | 1,240 | 1,694 | 1,835 | 0,435 | 0,376 | 20,485 | 3,464 | 4,975 |
| 1998 | Passenger Cars | 0,013 | 0,008 | 0,009 | 1,197 | 1,129 | 1,549 | 1,619 | 0,382 | 0,329 | 18,258 | 3,057 | 4,674 |
| 1999 | Passenger Cars | 0,011 | 0,006 | 0,007 | 1,053 | 0,888 | 1,200 | 1,647 | 0,301 | 0,255 | 19,037 | 2,333 | 3,140 |
| 2000 | Passenger Cars | 0,007 | 0,004 | 0,005 | 1,027 | 0,893 | 1,232 | 1,335 | 0,277 | 0,238 | 15,805 | 2,309 | 4,104 |
| 2001 | Passenger Cars | 0,007 | 0,004 | 0,005 | 0,958 | 0,796 | 1,107 | 1,261 | 0,234 | 0,203 | 15,723 | 2,064 | 4,048 |
| 1990 | Light Duty Vehicles | 0,312 | 0,233 | 0,255 | 1,960 | 1,117 | 1,141 | 0,546 | 0,171 | 0,135 | 5,044 | 1,620 | 1,823 |
| 1991 | Light Duty Vehicles | 0,314 | 0,233 | 0,254 | 1,979 | 1,118 | 1,143 | 0,568 | 0,172 | 0,135 | 5,242 | 1,622 | 1,826 |
| 1992 | Light Duty Vehicles | 0,202 | 0,150 | 0,164 | 1,981 | 1,138 | 1,165 | 0,586 | 0,177 | 0,138 | 5,453 | 1,666 | 1,881 |
| 1993 | Light Duty Vehicles | 0,079 | 0,058 | 0,063 | 2,004 | 1,147 | 1,176 | 0,620 | 0,179 | 0,140 | 5,785 | 1,688 | 1,908 |
| 1994 | Light Duty Vehicles | 0,080 | 0,058 | 0,064 | 2,006 | 1,135 | 1,162 | 0,618 | 0,176 | 0,138 | 5,767 | 1,661 | 1,874 |
| 1995 | Light Duty Vehicles | 0,080 | 0,058 | 0,063 | 1,945 | 1,115 | 1,144 | 0,605 | 0,170 | 0,134 | 5,616 | 1,553 | 1,758 |
| 1996 | Light Duty Vehicles | 0,080 | 0,057 | 0,062 | 1,907 | 1,091 | 1,121 | 0,612 | 0,163 | 0,130 | 5,646 | 1,441 | 1,636 |
| 1997 | Light Duty Vehicles | 0,079 | 0,057 | 0,062 | 1,811 | 1,064 | 1,094 | 0,543 | 0,154 | 0,125 | 4,957 | 1,317 | 1,500 |
| 1998 | Light Duty Vehicles | 0,078 | 0,056 | 0,062 | 1,735 | 1,050 | 1,082 | 0,507 | 0,149 | 0,122 | 4,638 | 1,232 | 1,410 |
| 1999 | Light Duty Vehicles | 0,045 | 0,031 | 0,034 | 1,725 | 1,025 | 1,058 | 0,536 | 0,141 | 0,117 | 4,820 | 1,111 | 1,273 |
| 2000 | Light Duty Vehicles | 0,009 | 0,006 | 0,007 | 1,629 | 1,018 | 1,052 | 0,459 | 0,137 | 0,115 | 4,159 | 1,064 | 1,229 |
| 2001 | Light Duty Vehicles | 0,009 | 0,006 | 0,007 | 1,593 | 0,998 | 1,032 | 0,442 | 0,130 | 0,111 | 3,903 | 0,964 | 1,119 |

| Year | Sector | SO2u | SO2r | SO2h | NOxu | NOxr | NOxh | NMVOCu | NMVOCr | NMVOCh | COu | COr | COh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | Heavy Duty Vehicles | 1,119 | 0,906 | 0,960 | 12,379 | 9,166 | 8,502 | 1,527 | 1,007 | 0,792 | 3,707 | 2,411 | 1,857 |
| 1991 | Heavy Duty Vehicles | 1,124 | 0,907 | 0,957 | 12,423 | 9,174 | 8,460 | 1,532 | 1,007 | 0,792 | 3,687 | 2,414 | 1,862 |
| 1992 | Heavy Duty Vehicles | 0,731 | 0,588 | 0,622 | 12,435 | 9,145 | 8,456 | 1,532 | 1,011 | 0,792 | 3,700 | 2,413 | 1,866 |
| 1993 | Heavy Duty Vehicles | 0,278 | 0,227 | 0,238 | 12,272 | 9,212 | 8,433 | 1,538 | 1,008 | 0,792 | 3,707 | 2,420 | 1,872 |
| 1994 | Heavy Duty Vehicles | 0,279 | 0,226 | 0,237 | 11,939 | 8,820 | 8,092 | 1,475 | 0,986 | 0,775 | 3,588 | 2,323 | 1,814 |
| 1995 | Heavy Duty Vehicles | 0,278 | 0,226 | 0,234 | 11,490 | 8,527 | 7,722 | 1,439 | 0,964 | 0,761 | 3,411 | 2,225 | 1,757 |
| 1996 | Heavy Duty Vehicles | 0,279 | 0,226 | 0,235 | 11,123 | 8,241 | 7,455 | 1,396 | 0,938 | 0,745 | 3,260 | 2,156 | 1,704 |
| 1997 | Heavy Duty Vehicles | 0,285 | 0,230 | 0,238 | 10,798 | 8,035 | 7,206 | 1,331 | 0,903 | 0,721 | 3,070 | 2,044 | 1,632 |
| 1998 | Heavy Duty Vehicles | 0,286 | 0,232 | 0,239 | 10,374 | 7,747 | 6,917 | 1,283 | 0,876 | 0,700 | 2,941 | 1,970 | 1,586 |
| 1999 | Heavy Duty Vehicles | 0,159 | 0,129 | 0,133 | 10,019 | 7,524 | 6,684 | 1,233 | 0,849 | 0,680 | 2,793 | 1,883 | 1,534 |
| 2000 | Heavy Duty Vehicles | 0,029 | 0,024 | 0,024 | 9,598 | 7,227 | 6,410 | 1,193 | 0,826 | 0,663 | 2,686 | 1,824 | 1,500 |
| 2001 | Heavy Duty Vehicles | 0,030 | 0,024 | 0,025 | 9,526 | 7,161 | 6,252 | 1,148 | 0,803 | 0,645 | 2,625 | 1,770 | 1,467 |
| 1990 | 2-wheelers | 0,003 | 0,004 | 0,005 | 0,064 | 0,164 | 0,340 | 6,532 | 4,283 | 2,011 | 16,842 | 19,852 | 27,917 |
| 1991 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,068 | 0,161 | 0,340 | 6,249 | 4,379 | 2,011 | 17,073 | 19,753 | 27,917 |
| 1992 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,066 | 0,171 | 0,340 | 6,372 | 4,042 | 2,011 | 16,972 | 20,099 | 27,917 |
| 1993 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,071 | 0,167 | 0,340 | 6,033 | 4,187 | 2,011 | 17,250 | 19,950 | 27,917 |
| 1994 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,070 | 0,175 | 0,340 | 6,097 | 3,880 | 2,011 | 17,197 | 20,267 | 27,917 |
| 1995 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,068 | 0,171 | 0,340 | 6,271 | 4,023 | 2,011 | 17,055 | 20,119 | 27,917 |
| 1996 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,069 | 0,173 | 0,340 | 6,184 | 3,960 | 2,011 | 17,126 | 20,184 | 27,917 |
| 1997 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,069 | 0,174 | 0,340 | 6,153 | 3,931 | 2,011 | 17,152 | 20,214 | 27,917 |
| 1998 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,069 | 0,174 | 0,340 | 6,145 | 3,924 | 2,011 | 17,158 | 20,221 | 27,917 |
| 1999 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,073 | 0,180 | 0,340 | 5,895 | 3,704 | 2,011 | 17,363 | 20,448 | 27,917 |
| 2000 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,077 | 0,186 | 0,342 | 5,549 | 3,417 | 1,980 | 17,021 | 20,004 | 27,414 |
| 2001 | 2-wheelers | 0,003 | 0,003 | 0,004 | 0,084 | 0,196 | 0,345 | 5,073 | 3,064 | 1,933 | 16,990 | 19,880 | 26,948 |

Annex 4:Fuel use (GJ) and emissions (tons) per vehicle category

| Year | Sector | FC (GJ) | CO2 | CH4 | N2O | SO2 | NOx | NMVOC | CO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | Passenger Cars | 69608254 | 5043700 | 2079 | 214 | 687 | 57332 | 71086 | 427990 |
| 1991 | Passenger Cars | 73816548 | 5348323 | 2334 | 280 | 709 | 57836 | 71423 | 443231 |
| 1992 | Passenger Cars | 77348442 | 5603904 | 2467 | 347 | 526 | 58614 | 70458 | 434504 |
| 1993 | Passenger Cars | 79376296 | 5750620 | 2719 | 407 | 307 | 57397 | 68127 | 438753 |
| 1994 | Passenger Cars | 82258416 | 5959390 | 2876 | 511 | 318 | 55069 | 63930 | 410796 |
| 1995 | Passenger Cars | 83450719 | 6046096 | 3038 | 587 | 329 | 52947 | 59334 | 396227 |
| 1996 | Passenger Cars | 84286667 | 6106833 | 3288 | 651 | 335 | 50991 | 55425 | 394948 |
| 1997 | Passenger Cars | 86483699 | 6265962 | 3139 | 761 | 344 | 48026 | 49139 | 347652 |
| 1998 | Passenger Cars | 88522087 | 6413827 | 2975 | 833 | 356 | 45484 | 44559 | 320454 |
| 1999 | Passenger Cars | 89328184 | 6473548 | 3347 | 877 | 294 | 36597 | 42347 | 303479 |
| 2000 | Passenger Cars | 88595697 | 6421379 | 2769 | 938 | 203 | 37329 | 32628 | 274087 |
| 2001 | Passenger Cars | 88434195 | 6410500 | 2751 | 981 | 202 | 33750 | 28933 | 267408 |
| 1990 | Light Duty Vehicles | 23115080 | 1695295 | 149 | 112 | 1906 | 10428 | 3091 | 21687 |
| 1991 | Light Duty Vehicles | 23957024 | 1757050 | 159 | 115 | 1974 | 10830 | 3248 | 23002 |
| 1992 | Light Duty Vehicles | 23479719 | 1721818 | 160 | 112 | 1248 | 10678 | 3305 | 23176 |
| 1993 | Light Duty Vehicles | 23450531 | 1719549 | 167 | 112 | 481 | 10731 | 3389 | 24109 |
| 1994 | Light Duty Vehicles | 24974574 | 1831494 | 167 | 120 | 516 | 11211 | 3480 | 24458 |
| 1995 | Light Duty Vehicles | 24505598 | 1796982 | 164 | 121 | 504 | 10790 | 3295 | 23171 |
| 1996 | Light Duty Vehicles | 25221292 | 1849446 | 170 | 127 | 519 | 10851 | 3235 | 23111 |
| 1997 | Light Duty Vehicles | 25453281 | 1866574 | 159 | 132 | 526 | 10671 | 2991 | 21012 |
| 1998 | Light Duty Vehicles | 24752576 | 1815060 | 151 | 132 | 509 | 10197 | 2750 | 19304 |
| 1999 | Light Duty Vehicles | 25246216 | 1851344 | 157 | 134 | 290 | 10113 | 2817 | 19299 |
| 2000 | Light Duty Vehicles | 25282013 | 1853875 | 141 | 139 | 59 | 10060 | 2415 | 17666 |
| 2001 | Light Duty Vehicles | 25520318 | 1871549 | 135 | 143 | 60 | 9976 | 2282 | 16600 |

Annex 5: Total emissions (tons) for road transport 1990-2001

| Year | SO2 | NOx | NMVOC | CH4 | CO | CO2 | N2O |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 5769 | 100118 | 80774 | 2620 | 467745 | 9277069 | 423 |
| 1991 | 5905 | 101506 | 81436 | 2891 | 484830 | 9681694 | 494 |
| 1992 | 3822 | 101402 | 80499 | 3024 | 476563 | 9847443 | 556 |
| 1993 | 1571 | 99862 | 78250 | 3283 | 481940 | 9971964 | 615 |
| 1994 | 1671 | 98904 | 74203 | 3444 | 454750 | 10467465 | 735 |
| 1995 | 1684 | 95730 | 69476 | 3602 | 438837 | 10563069 | 813 |
| 1996 | 1723 | 93410 | 65693 | 3862 | 437939 | 10734864 | 886 |
| 1997 | 1746 | 89086 | 59195 | 3700 | 388778 | 10938138 | 1000 |
| 1998 | 1769 | 85642 | 54617 | 3536 | 360719 | 11125879 | 1074 |
| 1999 | 1089 | 75772 | 52070 | 3904 | 342989 | 11259857 | 1121 |
| 2000 | 353 | 74501 | 41624 | 3299 | 311969 | 11140469 | 1185 |
| 2001 | 355 | 70348 | 37026 | 3259 | 302537 | 11183436 | 1230 |

Annex 6: Fuel use and emission factors for other mobile sources 1990 and 2001

| Year | SNAP ID | Category | Fuel type | Mode | Fuel | SO2 | NOx | NMVOC | CH4 | CO | CO2 | N2O |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | [GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [kg/GJ] | [g/GJ] |
| 1990 | 0801 | Military | Diesel |  | 146162 | 93,68 | 778,10 | 83,81 | 6,66 | 250,19 | 74 | 4,04 |
| 1990 | 0801 | Military | Gasoline |  | 986 | 2,28 | 871,06 | 1129,29 | 33,78 | 6687,29 | 73 | 2,24 |
| 1990 | 0801 | Military | Aviation gasoline |  | 4913 | 4,57 | 859,00 | 1242,60 | 21,90 | 6972,00 | 73 | 2,00 |
| 1990 | 0801 | Military | Jet fuel | < 3000 ft | 149678 | 4,60 | 250,57 | 24,94 | 2,65 | 229,89 | 72 | 2,30 |
| 1990 | 0801 | Military | Jet fuel | > 3000 ft | 1347105 | 4,60 | 250,57 | 24,94 | 2,65 | 229,89 | 72 | 2,30 |
| 1990 | 0802 | Railways | Diesel |  | 4010007 | 93,68 | 691,26 | 43,21 | 4,76 | 103,48 | 74 | 2,04 |
| 1990 | 0802 | Railways | Kerosene |  | 70 | 5,00 | 50,00 | 3,00 | 7,00 | 20,00 | 72 | 2,00 |
| 1990 | 0802 | Railways | Gasoline |  | 18954 | 2,28 | 871,06 | 1129,29 | 33,78 | 6687,29 | 73 | 2,24 |
| 1990 | 0803 | Inland waterways | Diesel |  | 304613 | 93,68 | 1249,33 | 270,13 | 4,35 | 595,20 | 74 | 3,05 |
| 1990 | 0803 | Inland waterways | Gasoline |  | 447167 | 2,28 | 64,34 | 10809,58 | 108,10 | 18485,08 | 73 | 0,52 |
| 1990 | 080402 | National sea traffic | Residual oil |  | 3559806 | 1466,99 | 1393,64 | 56,92 | 1,76 | 180,93 | 78 | 4,89 |
| 1990 | 080402 | National sea traffic | Diesel |  | 2782388 | 93,68 | 1334,89 | 54,52 | 1,69 | 173,30 | 74 | 4,68 |
| 1990 | 080402 | National sea traffic | Kerosene |  | 452 | 4,60 | 50,00 | 3,00 | 7,00 | 20,00 | 72 | 2,00 |
| 1990 | 080402 | National sea traffic | LPG |  | 1794 |  | 1249,00 | 384,90 | 20,30 | 443,00 | 65 | 2,00 |
| 1990 | 080403 | Fishing | Residual oil |  | 285426 | 1466,99 | 1393,64 | 56,92 | 1,76 | 180,93 | 78 | 4,89 |
| 1990 | 080403 | Fishing | Diesel |  | 10422380 | 93,68 | 1334,89 | 54,52 | 1,69 | 173,30 | 74 | 4,68 |
| 1990 | 080403 | Fishing | Kerosene |  | 25787 | 4,60 | 50,00 | 3,00 | 7,00 | 20,00 | 72 | 2,00 |
| 1990 | 080403 | Fishing | Gasoline |  | 9001 | 2,28 | 64,34 | 10809,58 | 108,10 | 18485,08 | 73 | 0,52 |
| 1990 | 080403 | Fishing | LPG |  | 42320 |  | 1249,00 | 384,90 | 20,30 | 443,00 | 65 | 2,00 |
| 1990 | 080404 | International sea traffic | Residual oil |  | 28644252 | 1711,49 | 2127,14 | 56,92 | 1,76 | 180,93 | 78 | 4,89 |
| 1990 | 080404 | International sea traffic | Diesel |  | 11632674 | 468,38 | 2037,47 | 54,52 | 1,69 | 173,30 | 74 | 4,68 |
| 1990 | 080501 | Air traffic, other airports | Jet fuel | Dom. < 3000 ft | 378795 | 2,30 | 310,41 | 16,54 | 1,76 | 100,94 | 72 | 6,35 |
| 1990 | 080501 | Air traffic, other airports | Aviation gasoline |  | 104947 | 4,57 | 859,00 | 1242,60 | 21,90 | 6972,00 | 73 | 2,00 |
| 1990 | 080502 | Air traffic, other airports | Jet fuel | Int. < 3000 ft | 136077 | 2,30 | 306,48 | 18,38 | 1,95 | 177,11 | 72 | 6,90 |
| 1990 | 080502 | Air traffic, other airports | Aviation gasoline |  | 30660 | 4,57 | 859,00 | 1242,60 | 21,90 | 6972,00 | 73 | 2,00 |
| 1990 | 080503 | Air traffic, other airports | Jet fuel | Dom. > 3000 ft | 910427 | 2,30 | 315,28 | 8,51 | 0,90 | 79,30 | 72 | 2,30 |
| 1990 | 080504 | Air traffic, other airports | Jet fuel | Int. > 3000 ft | 1612988 | 2,30 | 242,81 | 6,20 | 0,66 | 54,25 | 72 | 2,30 |

| Year | SNAP ID | Category | Fuel type | Mode | Fuel | SO2 | NOx | NMVOC | CH4 | CO | CO2 | N2O |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | [GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [kg/GJ] | [g/GJ] |
| 1990 | 0806 | Agriculture | Diesel |  | 14194891 | 93,68 | 1273,14 | 190,59 | 4,43 | 424,13 | 74 | 3,10 |
| 1990 | 0806 | Agriculture | Gasoline |  | 426773 | 2,28 | 244,33 | 1022,05 | 51,10 | 24741,09 | 73 | 1,80 |
| 1990 | 0807 | Forestry | Diesel |  | 4105 | 93,68 | 1255,79 | 238,29 | 4,37 | 526,70 | 74 | 3,06 |
| 1990 | 0807 | Forestry | Gasoline |  | 49540 | 2,28 | 48,66 | 18095,47 | 180,95 | 33391,26 | 73 | 0,48 |
| 1990 | 0808 | Industry | Diesel |  | 8091042 | 93,68 | 1285,59 | 176,89 | 4,48 | 395,14 | 74 | 3,13 |
| 1990 | 0808 | Industry | Gasoline |  | 117286 | 2,28 | 216,67 | 3096,74 | 119,76 | 44820,30 | 73 | 1,63 |
| 1990 | 0808 | Industry | LPG |  | 1991975 |  | 621,12 | 838,51 | 62,11 | 931,68 | 65 |  |
| 1990 | 0809 | Household and gardening | Gasoline |  | 974445 | 2,28 | 213,71 | 3726,00 | 116,17 | 42616,59 | 73 | 1,61 |
| 1990 | 080501 | Air traffic, Copenhagen airport | Jet fuel | Dom. < 3000 ft | 441215 | 2,30 | 280,41 | 23,40 | 2,49 | 144,24 | 72 | 5,03 |
| 1990 | 080501 | Air traffic, Copenhagen airport | Aviation gasoline |  | 8642 | 4,57 | 859,00 | 1242,60 | 21,90 | 6972,00 | 73 | 2,00 |
| 1990 | 080502 | Air traffic, Copenhagen airport | Jet fuel | Int. < 3000 ft | 2037255 | 2,30 | 326,94 | 34,43 | 3,66 | 159,73 | 72 | 3,76 |
| 1990 | 080502 | Air traffic, Copenhagen airport | Aviation gasoline |  | 5612 | 4,57 | 859,00 | 1242,60 | 21,90 | 6972,00 | 73 | 2,00 |
| 1990 | 080503 | Air traffic, Copenhagen airport | Jet fuel | Dom. > 3000 ft | 1160709 | 2,30 | 330,34 | 9,28 | 0,99 | 93,07 | 72 | 2,30 |
| 1990 | 080504 | Air traffic, Copenhagen airport | Jet fuel | Int. > 3000 ft | 20653862 | 2,30 | 291,18 | 8,79 | 0,93 | 36,07 | 72 | 2,30 |

| Year | SNAP ID | Category | Fuel type | Mode | Fuel | SO2 | NOx | NMVOC | CH4 | CO | CO2 | N2O |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | [GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [kg/GJ] | [g/GJ] |
| 2001 | 0801 | Military | Diesel |  | 884792 | 2,34 | 557,55 | 62,48 | 5,03 | 175,12 | 74 | 4,58 |
| 2001 | 0801 | Military | Jet fuel | < 3000 ft | 42908 | 4,60 | 250,57 | 24,94 | 2,65 | 229,89 | 72 | 2,30 |
| 2001 | 0801 | Military | Jet fuel | > 3000 ft | 386176 | 4,60 | 250,57 | 24,94 | 2,65 | 229,89 | 72 | 2,30 |
| 2001 | 0801 | Military | Gasoline |  | 0 | 2,28 | 397,09 | 400,36 | 35,56 | 3516,78 | 73 | 10,55 |
| 2001 | 0801 | Military | Aviation gasoline |  | 6904 | 4,57 | 859,00 | 1242,60 | 21,90 | 6972,00 | 73 | 2,00 |
| 2001 | 0802 | Railways | Diesel |  | 2853837 | 2,34 | 691,26 | 43,21 | 4,76 | 103,48 | 74 | 2,04 |
| 2001 | 0802 | Railways | Kerosene |  | 0 | 5,00 | 50,00 | 3,00 | 7,00 | 20,00 | 72 | 2,00 |
| 2001 | 0802 | Railways | Gasoline |  | 6373 | 2,28 | 397,09 | 400,36 | 35,56 | 3516,78 | 73 | 10,55 |
| 2001 | 0803 | Inland waterways | Diesel |  | 363808 | 23,42 | 1249,33 | 4,35 | 4,35 | 595,20 | 74 | 3,05 |
| 2001 | 0803 | Inland waterways | Gasoline |  | 534065 | 2,28 | 64,34 | 10809,58 | 108,10 | 18485,08 | 73 | 0,52 |
| 2001 | 080402 | National sea traffic | Residual oil |  | 1513156 | 702,68 | 1393,60 | 56,90 | 1,76 | 180,90 | 78 | 4,90 |
| 2001 | 080402 | National sea traffic | Diesel |  | 3240423 | 93,68 | 1334,90 | 54,50 | 1,69 | 173,30 | 74 | 4,70 |
| 2001 | 080402 | National sea traffic | Kerosene |  | 522 | 4,60 | 50,00 | 3,00 | 7,00 | 20,00 | 72 | 2,00 |
| 2001 | 080402 | National sea traffic | LPG |  | 0 | 0,00 | 1249,00 | 384,90 | 20,30 | 443,00 | 65 | 2,00 |
| 2001 | 080403 | Fishing | Residual oil |  | 0 | 702,68 | 1393,60 | 56,90 | 1,76 | 180,90 | 78 | 4,90 |
| 2001 | 080403 | Fishing | Diesel |  | 8908320 | 93,68 | 1334,90 | 54,50 | 1,69 | 173,30 | 74 | 4,70 |
| 2001 | 080403 | Fishing | Kerosene |  | 1496 | 4,60 | 50,00 | 3,00 | 7,00 | 20,00 | 72 | 2,00 |
| 2001 | 080403 | Fishing | Gasoline |  | 3022 | 2,28 | 64,34 | 10809,60 | 108,10 | 18485,10 | 73 | 0,52 |
| 2001 | 080403 | Fishing | LPG |  | 19182 | 0,00 | 1249,00 | 384,90 | 20,30 | 443,00 | 65 | 2,00 |
| 2001 | 080404 | International sea traffic | Residual oil |  | 25923684 | 1710,72 | 2127,10 | 56,90 | 1,76 | 180,90 | 78 | 4,90 |
| 2001 | 080404 | International sea traffic | Diesel |  | 21388985 | 468,38 | 2037,50 | 54,50 | 1,69 | 173,30 | 74 | 4,70 |
| 2001 | 080501 | Air traffic, other airports | Jet fuel | Dom. < 3000 ft | 265380 | 2,30 | 326,18 | 16,96 | 1,80 | 116,87 | 72 | 7,66 |
| 2001 | 080501 | Air traffic, other airports | Aviation gasoline |  | 98664 | 4,57 | 859,00 | 1242,60 | 21,90 | 6972,00 | 73 | 2,00 |
| 2001 | 080502 | Air traffic, other airports | Jet fuel | Int. < 3000 ft | 244140 | 2,30 | 306,48 | 18,38 | 1,95 | 177,11 | 72 | 6,90 |
| 2001 | 080502 | Air traffic, other airports | Aviation gasoline |  | 7282 | 4,57 | 859,00 | 1242,60 | 21,90 | 6972,00 | 73 | 2,00 |
| 2001 | 080503 | Air traffic, other airports | Jet fuel | Dom. > 3000 ft | 588726 | 2,30 | 323,65 | 8,91 | 0,95 | 89,93 | 72 | 2,30 |
| 2001 | 080504 | Air traffic, other airports | Jet fuel | Int. > 3000 ft | 2621135 | 2,30 | 242,81 | 6,20 | 0,66 | 54,25 | 72 | 2,30 |

| Year | SNAP ID | Category | Fuel type | Mode | Fuel | SO2 | NOx | NMVOC | CH4 | CO | CO2 | N2O |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | [GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [g/GJ] | [kg/GJ] | [g/GJ] |
| 2001 | 0806 | Agriculture | Diesel |  | 16953370 | 23,42 | 1258,27 | 189,04 | 4,43 | 424,13 | 74 | 3,10 |
| 2001 | 0806 | Agriculture | Gasoline |  | 509707 | 2,28 | 244,33 | 1022,05 | 51,10 | 24741,09 | 73 | 1,80 |
| 2001 | 0807 | Forestry | Diesel |  | 4903 | 23,42 | 1144,55 | 216,87 | 4,37 | 517,55 | 74 | 3,06 |
| 2001 | 0807 | Forestry | Gasoline |  | 59167 | 2,28 | 48,66 | 18095,47 | 180,95 | 33391,26 | 73 | 0,48 |
| 2001 | 0808 | Industry | Diesel |  | 9663366 | 23,42 | 1174,86 | 166,55 | 4,48 | 394,44 | 74 | 3,13 |
| 2001 | 0808 | Industry | Gasoline |  | 140078 | 2,28 | 216,67 | 3096,74 | 119,76 | 44820,30 | 73 | 1,63 |
| 2001 | 0808 | Industry | LPG |  | 2379073 | 0,00 | 621,12 | 838,51 | 62,11 | 931,68 | 65 | 3,11 |
| 2001 | 0809 | Household and gardening | Gasoline |  | 1163807 | 2,28 | 213,71 | 3726,00 | 116,17 | 42616,59 | 73 | 1,61 |
| 2001 | 080501 | Air traffic, Copenhagen airport | Jet fuel | Dom. < 3000 ft | 325930 | 2,30 | 301,48 | 20,93 | 2,22 | 156,45 | 72 | 6,25 |
| 2001 | 080501 | Air traffic, Copenhagen airport | Aviation gasoline |  | 800 | 4,57 | 859,00 | 1242,60 | 21,90 | 6972,00 | 73 | 2,00 |
| 2001 | 080502 | Air traffic, Copenhagen airport | Jet fuel | Int. < 3000 ft | 3055248 | 2,30 | 322,85 | 29,96 | 3,18 | 169,60 | 72 | 3,81 |
| 2001 | 080502 | Air traffic, Copenhagen airport | Aviation gasoline |  | 1158 | 4,57 | 859,00 | 1242,60 | 21,90 | 6972,00 | 73 | 2,00 |
| 2001 | 080503 | Air traffic, Copenhagen airport | Jet fuel | Dom. > 3000 ft | 1059388 | 2,30 | 302,98 | 9,70 | 1,03 | 63,23 | 72 | 2,30 |
| 2001 | 080504 | Air traffic, Copenhagen airport | Jet fuel | Int. > 3000 ft | 27098054 | 2,30 | 291,18 | 8,79 | 0,93 | 36,07 | 72 | 2,30 |

Annex 7: Total emissions per sector for other mobile sources 1990 and 2001

| Year | Sector | SNAP ID | SO2 | NOx | NMVOC | CH4 | CO | CO2 | N2O |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [tons] | [tons] | [tons] | [tons] | [tons] | [ktons] | [tons] |
| 2001 | Military | 801 | 4 | 607 | 75 | 6 | 302 | 97 | 5 |
| 2001 | Railways | 802 | 7 | 1975 | 126 | 14 | 318 | 212 | 6 |
| 2001 | Inland waterways | 803 | 10 | 489 | 5775 | 59 | 10089 | 66 | 1 |
| 2001 | National sea traffic | 80402 | 1367 | 6434 | 263 | 8 | 835 | 358 | 23 |
| 2001 | Fishing | 80403 | 835 | 11916 | 526 | 16 | 1608 | 661 | 42 |
| 2001 | International sea traffic | 80404 | 54366 | 98722 | 2641 | 82 | 8396 | 3605 | 228 |
| 2001 | Air traffic, Dom. < 3000 ft. | 80501 | 2 | 270 | 135 | 3 | 775 | 50 | 4 |
| 2001 | Air traffic, Int. < 3000 ft. | 80502 | 8 | 1068 | 107 | 10 | 620 | 238 | 13 |
| 2001 | Air traffic, Dom. > 3000 ft. | 80503 | 4 | 512 | 16 | 2 | 120 | 119 | 4 |
| 2001 | Air traffic, Int. > 3000 ft. | 80504 | 68 | 8527 | 255 | 27 | 1120 | 2140 | 68 |
| 2001 | Agriculture | 806 | 398 | 21457 | 3726 | 101 | 19801 | 1292 | 54 |
| 2001 | Forestry | 807 | 0 | 8 | 1072 | 11 | 1978 | 5 | 0 |
| 2001 | Industry | 808 | 227 | 12861 | 4038 | 208 | 12306 | 880 | 38 |
| 2001 | Household and gardening | 809 | 3 | 249 | 4336 | 135 | 49598 | 85 | 2 |
| 1990 | Military | 801 | 21 | 494 | 57 | 5 | 422 | 119 | 4 |
| 1990 | Railways | 802 | 376 | 2788 | 195 | 20 | 542 | 298 | 8 |
| 1990 | Inland waterways | 803 | 30 | 409 | 4916 | 50 | 8447 | 55 | 1 |
| 1990 | National sea traffic | 80402 | 5483 | 8678 | 355 | 11 | 1127 | 484 | 30 |
| 1990 | Fishing | 80403 | 1395 | 14365 | 698 | 20 | 2044 | 799 | 50 |
| 1990 | International sea traffic | 80404 | 54473 | 84632 | 2265 | 70 | 7199 | 3095 | 195 |
| 1990 | Air traffic, Dom. < 3000 ft. | 80501 | 2 | 339 | 158 | 4 | 894 | 67 | 5 |
| 1990 | Air traffic, Int. < 3000 ft. | 80502 | 5 | 739 | 118 | 9 | 602 | 159 | 9 |
| 1990 | Air traffic, Dom. > 3000 ft. | 80503 | 5 | 667 | 18 | 2 | 177 | 149 | 5 |
| 1990 | Air traffic, Int. > 3000 ft. | 80504 | 51 | 6406 | 192 | 20 | 832 | 1603 | 51 |
| 1990 | Agriculture | 806 | 1331 | 18176 | 3142 | 85 | 16579 | 1082 | 45 |
| 1990 | Forestry | 807 | 1 | 8 | 897 | 9 | 1656 | 4 | 0 |
| 1990 | Industry | 808 | 758 | 11664 | 3465 | 174 | 10310 | 737 | 26 |
| 1990 | Household and gardening | 809 | 2 | 208 | 3631 | 113 | 41528 | 71 | 2 |

Annex 8: Uncertainty estimate calculation schemes for CO2, CH4 and N2O in transport

|  |  | Gas | Base year emission | Year t emission | Activity data uncertainty | Emission factor uncertainty | Combined uncertainty | Combined uncertainty as % of total national emissions in year t |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Input data | Input data | Input data | Input data |  |  |
|  |  |  | Gg CO2 | Gg CO2 | % | % | % | % |
| Civil Aviation | Aviation Gasoline | CO2 | 8,29 | 7,26 | 2 | 5 | 5,385 | 0,003 |
|  | Jet Kerosene | CO2 | 208,16 | 161,24 | 2 | 5 | 5,385 | 0,061 |
| Road Transportation | Gasoline | CO2 | 4.914,00 | 6.024,98 | 2 | 5 | 5,385 | 2,269 |
|  | Diesel Oil | CO2 | 4.436,10 | 5.247,87 | 2 | 5 | 5,385 | 1,976 |
| Railways | Diesel Oil | CO2 | 298,13 | 211,65 | 2 | 5 | 5,385 | 0,080 |
| Navigation | Residual Oil | CO2 | 277,66 | 118,03 | 2 | 5 | 5,385 | 0,044 |
|  | Gas/Diesel Oil | CO2 | 228,47 | 266,75 | 2 | 5 | 5,385 | 0,100 |
| Agriculture | Diesel Oil | CO2 | 1050 | 1255 | 20 | 5 | 20,616 | 1,809 |
|  | Gasoline | CO2 | 31 | 37 | 20 | 5 | 20,616 | 0,054 |
| Forestry | Diesel Oil | CO2 | 0 | 0 | 20 | 5 | 20,616 | 0,001 |
|  | Gasoline | CO2 | 4 | 4 | 20 | 5 | 20,616 | 0,006 |
| Industry | Diesel Oil | CO2 | 599 | 715 | 20 | 5 | 20,616 | 1,031 |
|  | Gasoline | CO2 | 9 | 10 | 20 | 5 | 20,616 | 0,012 |
|  | LPG | CO2 | 129 | 155 | 20 | 5 | 20,616 | 0,187 |
| Household and gardening | Gasoline | CO2 | 71 | 85 | 20 | 5 | 20,616 | 0,103 |
|  | Total | CO2 | 12264,2312 | 14299,127 |  |  |  | 13,460 |
|  | Total uncertainties |  |  |  | Overall uncertainty in the year (%): |  |  |  | 3,669 |

|  |  | Gas | Type A sensitivity | Type B sensitivity | Uncertainty in trend in national emissions introduced by emission factor uncertainty | Uncertainty in trend in national emissions introduced by activity data uncertainty | Uncertainty introduced into the trend in total national emissions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | % | % | % | % | % |
| Civil Aviation | Aviation Gasoline | CO2 | -0,000196 | 0,0006 | -0,0010 | 0,0017 | 0,0019 |
|  | Jet Kerosene | CO2 | -0,006641 | 0,0131 | -0,0332 | 0,0372 | 0,0499 |
| Road Transportation | Gasoline | CO2 | 0,024009 | 0,4913 | 0,1200 | 1,3895 | 1,3947 |
|  | Diesel Oil | CO2 | 0,006152 | 0,4279 | 0,0308 | 1,2103 | 1,2107 |
| Railways | Diesel Oil | CO2 | -0,011082 | 0,0173 | -0,0554 | 0,0488 | 0,0738 |
| Navigation | Residual Oil | CO2 | -0,016769 | 0,0096 | -0,0838 | 0,0272 | 0,0882 |
|  | Gas/Diesel Oil | CO2 | 3,03E-05 | 0,0218 | 0,0002 | 0,0615 | 0,0615 |
| Agriculture | Diesel Oil | CO2 | 0,002431 | 0,1023 | 0,0122 | 2,8933 | 2,8933 |
|  | Gasoline | CO2 | 7,22E-05 | 0,0030 | 0,0004 | 0,0858 | 0,0858 |
| Forestry | Diesel Oil | CO2 | 7,04E-07 | 0,0000 | 0,0000 | 0,0008 | 0,0008 |
|  | Gasoline | CO2 | 8,38E-06 | 0,0004 | 0,0000 | 0,0100 | 0,0100 |
| Industry | Diesel Oil | CO2 | 0,001386 | 0,0583 | 0,0069 | 1,6492 | 1,6492 |
|  | Gasoline | CO2 | 1,98E-05 | 0,0007 | 0,0001 | 0,0197 | 0,0197 |
|  | LPG | CO2 | 0,0003 | 0,0106 | 0,0015 | 0,2986 | 0,2986 |
| Household and gardening | Gasoline | CO2 | 0,000165 | 0,0058 | 0,0008 | 0,1641 | 0,1641 |
|  | Total | CO2 |  |  |  |  | 14,64540184 |
|  | Total uncertainties |  |  |  | Trend uncertainty (%): |  | 3,827 |

|  |  | Gas | Base year emission | Year t emission | Activity data uncertainty | Emission factor uncertainty | Combined uncertainty | Combined uncertainty as % of total national emissions in year t |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Input data | Input data | Input data | Input data |  |  |
|  |  |  | Gg CH4 | Gg CH4 | % | % | % | % |
| Civil Aviation | Aviation Gasoline | CH4 | 0,00 | 0,00 | 2 | 100 | 100,020 | 0,058 |
|  | Jet Kerosene | CH4 | 0,00 | 0,00 | 2 | 100 | 100,020 | 0,076 |
| Road Transportation | Gasoline | CH4 | 2,27 | 2,92 | 2 | 40 | 40,050 | 31,285 |
|  | Diesel Oil | CH4 | 0,35 | 0,34 | 2 | 40 | 40,050 | 3,591 |
| Railways | Diesel Oil | CH4 | 0,02 | 0,01 | 2 | 100 | 100,020 | 0,369 |
| Navigation | Residual Oil | CH4 | 0,01 | 0,00 | 2 | 100 | 100,020 | 0,071 |
|  | Gas/Diesel Oil | CH4 | 0,01 | 0,01 | 2 | 100 | 100,020 | 0,189 |
| Agriculture | Diesel Oil | CH4 | 0 | 0 | 20 | 100 | 101,980 | 2,048 |
|  | Gasoline | CH4 | 0 | 0 | 20 | 100 | 101,980 | 0,710 |
| Forestry | Diesel Oil | CH4 | 0 | 0 | 20 | 100 | 101,980 | 0,001 |
|  | Gasoline | CH4 | 0 | 0 | 20 | 100 | 101,980 | 0,292 |
| Industry | Diesel Oil | CH4 | 0 | 0 | 20 | 100 | 101,980 | 1,179 |
|  | Gasoline | CH4 | 0 | 0 | 20 | 100 | 101,980 | 0,457 |
|  | LPG | CH4 | 0 | 0 | 20 | 100 | 101,980 | 4,027 |
| Household and gardening | Gasoline | CH4 | 0 | 0 | 20 | 100 | 101,980 | 3,684 |
|  | Total | CH4 | 3,03838247 | 3,7424274 |  |  |  | 1027,973 |
|  | Total uncertainties |  |  |  |  | Overall uncertainty i the year (%): |  | 32,062 |

|  |  | Gas | Type A sensitivity | Type B sensitivity | Uncertainty in trend in national emissions introduced by emission factor uncertainty | Uncertainty in trend in national emissions introduced by activity data uncertainty | Uncertainty introduced into the trend in total national emissions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | % | % | % | % | % |
| Civil Aviation | Aviation Gasoline | CH4 | -0,000292 | 0,0007 | -0,029176864 | 0,002027498 | 0,029247224 |
|  | Jet Kerosene | CH4 | -0,000564 | 0,0009 | -0,056442996 | 0,002653993 | 0,056505358 |
| Road Transportation | Gasoline | CH4 | 0,043366 | 0,9621 | 1,734653411 | 2,721343515 | 3,227186543 |
|  | Diesel Oil | CH4 | -0,032878 | 0,1104 | -1,315129826 | 0,312395732 | 1,351723919 |
| Railways | Diesel Oil | CH4 | -0,00345 | 0,0045 | -0,344968901 | 0,012856646 | 0,345208395 |
| Navigation | Residual Oil | CH4 | -0,001664 | 0,0009 | -0,166406514 | 0,002478984 | 0,166424978 |
|  | Gas/Diesel Oil | CH4 | -0,000116 | 0,0023 | -0,011582793 | 0,006574939 | 0,013318818 |
| Agriculture | Diesel Oil | CH4 | -0,000774 | 0,0247 | -0,077415121 | 0,699602936 | 0,70387312 |
|  | Gasoline | CH4 | -0,000268 | 0,0086 | -0,026834926 | 0,242475273 | 0,243955675 |
| Forestry | Diesel Oil | CH4 | -2,21E-07 | 0,0000 | -2,20869E-05 | 0,000199559 | 0,000200778 |
|  | Gasoline | CH4 | -0,00011 | 0,0035 | -0,011030683 | 0,099666939 | 0,100275494 |
| Industry | Diesel Oil | CH4 | -0,000446 | 0,0142 | -0,044561602 | 0,402669238 | 0,405127451 |
|  | Gasoline | CH4 | -0,000173 | 0,0055 | -0,017283274 | 0,15616438 | 0,15711787 |
|  | LPG | CH4 | -0,001522 | 0,0486 | -0,152184991 | 1,375575792 | 1,38396858 |
| Household and gardening | Gasoline | CH4 | -0,001392 | 0,0445 | -0,139242975 | 1,258551401 | 1,266230719 |
|  | Total | CH4 |  |  |  |  | 16,66551279 |
|  | Total uncertainties |  |  |  | Trend uncertainty (%): |  | 4,082 |

|  |  | Gas | Base year emission | Year t emission | Activity data uncertainty | Emission factor uncertainty | Combined uncertainty | Combined uncertainty as % of total national emissions in year t |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Input data | Input data | Input data | Input data |  |  |
|  |  |  | Gg N2O | Gg N2O | % | % | % | % |
| Civil Aviation | Aviation Gasoline | CH4 | 0,00 | 0,00 | 2 | 1000 | 1000,002 | 0,146 |
|  | Jet Kerosene | CH4 | 0,01 | 0,01 | 2 | 1000 | 1000,002 | 5,774 |
| Road Transportation | Gasoline | CH4 | 0,15 | 0,88 | 2 | 50 | 50,040 | 32,175 |
|  | Diesel Oil | CH4 | 0,27 | 0,35 | 2 | 50 | 50,040 | 13,048 |
| Railways | Diesel Oil | CH4 | 0,01 | 0,01 | 2 | 1000 | 1000,002 | 4,326 |
| Navigation | Residual Oil | CH4 | 0,02 | 0,01 | 2 | 1000 | 1000,002 | 5,447 |
|  | Gas/Diesel Oil | CH4 | 0,01 | 0,02 | 2 | 1000 | 1000,002 | 12,004 |
| Agriculture | Diesel Oil | CH4 | 0,04 | 0,05 | 20 | 1000 | 1000,200 | 38,656 |
|  | Gasoline | CH4 | 0,00 | 0,00 | 20 | 1000 | 1000,200 | 0,675 |
| Forestry | Diesel Oil | CH4 | 0,00 | 0,00 | 20 | 1000 | 1000,200 | 0,011 |
|  | Gasoline | CH4 | 0,00 | 0,00 | 20 | 1000 | 1000,200 | 0,021 |
| Industry | Diesel Oil | CH4 | 0,03 | 0,03 | 20 | 1000 | 1000,200 | 22,249 |
|  | Gasoline | CH4 | 0,00 | 0,00 | 20 | 1000 | 1000,200 | 0,167 |
|  | LPG | CH4 | 0,01 | 0,01 | 20 | 1000 | 1000,200 | 5,429 |
| Household and gardening | Gasoline | CH4 | 0,00 | 0,00 | 20 | 1000 | 1000,200 | 1,373 |
|  | Total | N2O | 0,55042987 | 1,3611741 |  |  |  | 3452,500 |
|  | Total uncertainties |  |  |  |  | Overall uncertainty i the year (%): |  | 58,758 |

|  |  | Gas | Type A sensitivity | Type B sensitivity | Uncertainty in trend in national emissions introduced by emission factor uncertainty | Uncertainty in trend in national emissions introduced by activity data uncertainty | Uncertainty introduced into the trend in total national emissions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | % | % | % | % | % |
| Civil Aviation | Aviation Gasoline | N2O | -0,000658 | 0,0004 | -0,658309891 | 0,001022577 | 0,658310685 |
|  | Jet Kerosene | N2O | -0,027884 | 0,0143 | -27,88419895 | 0,040389228 | 27,8842282 |
| Road Transportation | Gasoline | N2O | 0,910392 | 1,5901 | 45,51961232 | 4,497355926 | 45,74124305 |
|  | Diesel Oil | N2O | -0,57595 | 0,6448 | -28,797492 | 1,823815146 | 28,85518753 |
| Railways | Diesel Oil | N2O | -0,026232 | 0,0107 | -26,23185078 | 0,030261089 | 26,23186824 |
| Navigation | Residual Oil | N2O | -0,064715 | 0,0135 | -64,71489864 | 0,038097422 | 64,71490985 |
|  | Gas/Diesel Oil | N2O | -0,03303 | 0,0297 | -33,03046173 | 0,083959236 | 33,03056844 |
| Agriculture | Diesel Oil | N2O | -0,102237 | 0,0956 | -102,2371008 | 2,703274297 | 102,2728335 |
|  | Gasoline | N2O | -0,001787 | 0,0017 | -1,786635287 | 0,047203713 | 1,78725875 |
| Forestry | Diesel Oil | N2O | -2,92E-05 | 0,0000 | -0,02918608 | 0,000771099 | 0,029196265 |
|  | Gasoline | N2O | -5,49E-05 | 0,0001 | -0,054893645 | 0,001450295 | 0,0549128 |
| Industry | Diesel Oil | N2O | -0,058864 | 0,0550 | -58,86440652 | 1,55591886 | 58,88496615 |
|  | Gasoline | N2O | -0,000443 | 0,0004 | -0,442916035 | 0,011701919 | 0,443070592 |
|  | LPG | N2O | -0,014369 | 0,0134 | -14,36851828 | 0,37966012 | 14,3735333 |
| Household and gardening | Gasoline | N2O | -0,003634 | 0,0034 | -3,63397193 | 0,096012571 | 3,635240075 |
|  | Total | N2O |  |  |  |  | 23820,37525 |
|  | Total uncertainties |  |  |  | Trend uncertainty (%): |  | 154,339 |