15 April 2003

Appendix 9 Agriculture

9.1 Trends

9.2 Methods and assumptions

9.3 References – sources of information

9.4 CH4 emissions

9.4.1 Enteric Fermentation

9.4.2 Manure Management

9.4.3 Implied emission factor

9.5 N2O emissions

9.5.1 Manure Management

9.5.2 Agricultural Soils

9.6 Uncertainties

9.7 QA/QC

9.8 Recalculations

9.9 Improvements

9.10 References

Table A

Table A.9.1 CH4 Enteric Fermentation - tier 2, Cattle

Table A.9.2 CH4 Manure Management – tier 2

Table A.3.3 Data used to estimation of NH3 emission

Appendix 9 Agriculture

The emission of greenhouse gases from the agricultural sector includes emissions of CH4 from enteric fermentation and manure management and emission of N2O from manure management and agricultural soils. The emissions are reported in CRF – table 4.A, 4.B(a), 4.B(b) and 4.D.

9.1 Trends

In CO2 equivalents the agricultural sector contribute with ca. 20% of the total emission of greenhouse gases. From 1990 to 2001 the emissions have decreased from 11,4 Gg CO2-equ. to 11,5 Gg CO2-equ. Which responds to a 20%-reduction (table 1).

Table 1. Emission of GHG in the agricultural sector in Denmark 1990 – 2001 (CRF)

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Gg CO2 – equ. |  |  |  |  |  |  |  |
| N2O | 10,259 | 10,030 | 9,399 | 9,485 | 9,200 | 9,134 | 8,842 | 8,486 | 8,563 | 8,505 | 8,293 | 7,918 |
| CH4 | 4,089 | 4,066 | 4,042 | 4,133 | 3,974 | 3,977 | 3,960 | 3,868 | 3,897 | 3,578 | 3,575 | 3,632 |
| Total | 14,348 | 14,096 | 13,441 | 13,618 | 13,174 | 13,111 | 12,803 | 12,354 | 12,460 | 12,083 | 11,868 | 11,550 |

Figure 1 shows the distribution of greenhouse gas emission from the various sources. The figure shows that the reduction up to 2001 mainly is caused by a decrease of N2O emission from agricultural soils. This is due to the offensive National environmental policy during the last twenty years. Due to The Action Plan on the Aquatic Environment and The Ammonia Action Plan a series of measures to prevent loss of nitrogen in the agricultural production has been initiated. The measures have included i.e. demands to a improved utilisation of nitrogen in husbandry manure, ban against application of husbandry manure in winter, demand on establishment of green field during the winter, regulation of the number of animals per hectare and a ceiling for the supply of nitrogen to crops. This means that despite an increase in the livestock production both the evaporation of ammonia and the N-excretion per animal has been reduced considerably.

The emission of CH4 are in the period 1990 to 2001 reduced with 11%, which is coursed by a decrease of dairy cattle as a consequence of an improved milk yield.

Figure 1.Danish greenhouse gas emission 1990 - 2001

9.2 Methods and assumptions

The calculations of the emissions are based on methods described in the IPCC Guideline (IPCC, 1996) and in the Good Practice Guidance (IPCC, 2000). There has been no chance in the methods of the evaluation of the previous years. Emission from rice production, burning of savannas and crop residues is not relevant for the situation in Denmark. Therefor the table 4C, 4E and 4F has not been completed. Burning of plant residue has been prohibited since 1989 and may only take place in connection with cultivation of seed grass. It is estimated that the emissions from burning of straw from the seed grass production is so small that it is not included in the emission inventory.

Presently a thorough investigation of the method for the emission inventory of greenhouse gases from the agricultural sector is being performed. This work is expected to be completed the summer of 2003. Based on this investigation change in the data on activity and the emission factors for the whole period from 1990 to 2001 might occur. In section 9.9 these changes will be mentioned briefly. The revision is planned to be implemented in the emission inventory for 2002.

9.3 References – sources of information

Data on activity and emissions are collected, evaluated and discussed in corporation with institutes occupied with research or administration in the agricultural sector. These institutes are Statistic Denmark, Danish Institute of Agricultural Sciences, The Danish Agricultural Advisory Centre, The Danish Plant Directorate and Ministry of the Environment. A more detailed description of the methods of inventory is given by Andersen, 1999 and Olesen et al., 2001.

Table 2. List of institutes and persons involved

| References | Abbreviation |  | Data / information |
| --- | --- | --- | --- |
| National Environmental Research Institute Cand. scient. PhD, Ruth Grant Cand. agro. PhD, Steen Gyldenkærne Cand. agro. Mette Hjorth Mikkelsen | NERI |  | -reporting -data collecting |
| Statistics Denmark - Agricultural Statistic (www.dst.dk) | DS |  | -No. of animal -milk yield -slaughtering data -land use -crop production |
| Danish Institute of Agricultural Sciences Cand. agro. PhD, Sven G. Sommer Cand. agro. PhD, Jørgen E. Olesen Cand. agro. scient, PhD Søren O. Petersen Cand. agro. PhD, Arne Kyllingsbæk Cand. agro. PhD, Christen Duus Børgesen Cand. agro. scient, PhD Hanne D. Poulsen | DIAS |  | -N-excretion -feeding situation -growth -N-fixed crops -crop residue -N-leaching/runoff -NH3 emissions factor |
| The Danish Agricultural Advisory Centre Cand. agro. Niels Lundegaard Cand. agro. Jan Brøgger Rasmussen Cand. agro. Ole Aaes | AAC |  | -stable type -grassing situation -manure application time and methods |
| Danish Environmental Protection Agency | EPA |  | -sewage sludge used as fertiliser -industrial waste used as fertiliser |
| The Danish Plant Directorate | PD |  | -organic farming -synthetic fertiliser |

9.4 CH4 emissions

The majority of the CH4 emission from agriculture, corresponding to approximately 75%, origins from the enteric fermentation process. The contribution from cattle amounts to approximately 80%. The emissions from storage of husbandry manure make up 25% of total emission.

The numbers of animals are taken from the Agricultural Statistic (Statistic, Denmark) and number of horses and goats are collected at The Danish Agricultural Advisory Centre. Because the Agricultural statistic has a lower bound on agricultural farms by 5 hectare.

9.4.1 Enteric Fermentation

The CH4 emissions from enteric fermentation is given in CRF in table 4.A. The emission factor for cattle is based on tier 2 approach described in the IPCC Guideline. For the other categories of livestocks the IPCC default values are applied. Goats have not been included in the inventory, because there is only approximately 10.000 in Denmark. However, this is planned to be included in the emission inventory 2002.

In appendix 9.1 the data used for calculation of the emission factor is given. Data on food intake, milk yield and growth etc. are collected by Danish Institute of Agricultural Sciences (Poulsen et al., 2001). The data are based on an inventory on feeding praxis and inventory in relation to the National control system.

Table 3. Animal categories and emission factor for estimating CH4 emission from enteric fermentation.

| Activity data |  |  | Method | Emission factor |
| --- | --- | --- | --- | --- |
|  |  |  |  | kg CH4/head/yr |
| Cattle | Dairy Cattle |  | Tier 2 | 104 |
|  | Non-Dairy Cattle | Calves, heifers, bulls, suckling cattle | Tier 2 | 37 |
| Sheep |  | Breeding ewes | Tier 1 | 8 |
| Horses |  |  | Tier 1 | 18 |
| Swine | Sows |  | Tier 1 | 1,5 |
|  | Slaughter pigs |  | Tier 1 | 1,5 |
| Goats1 |  |  |  | Inventory - 2002 |
| Poultry |  |  |  | Not estimated |
| Fur farming |  |  |  | Not estimated |

1 Goats will be included in the emission inventory for 2002.

In the emission inventory the same emission factor for cattle has been applied all years. This implies that the data used for calculation of the emission factors in CRF given in table 4.a. and 4.B(a) are the same for all the years from 1990 to 2001. Due to change in the composition of food and developments in the field of genetic elevation the emission factors might differ. In Denmark data are available which makes it possibly to calculate the emission factor for each year. It is planned that annual emission factors will be calculated and applied in the emission inventory from 2002.

9.4.2 Manure Management

The emission from storage of husbandry manure is given in CRF in table 4.B(a). National data for all categories off livestock is applied. The tier 2 approach is given in the IPCC guideline (IPCC, 1996). The data used for calculation are given in appendix 9.2.

Table 4. Animal categories and emission factor for estimating CH4 emission from manure management.

| Activity data |  |  | Method | Emission factor |
| --- | --- | --- | --- | --- |
|  |  |  |  | kg CH4/head/yr |
| Cattle | Dairy Cattle |  | Tier 2 | 21,8 |
|  | Non-Dairy Cattle | Calves, heifers, bulls, suckling cattle | Tier 2 | 1,6 |
| Sheep |  | Breeding ewes | Tier 2 | 0,46 |
| Horses |  |  | Tier 2 | 1,1 |
| Swine | Sows |  | Tier 2 | 6,0 |
|  | Slaughter pigs |  | Tier 2 | 2,1 |
| Poultry | Laying hens |  | Tier 2 | 0,066 |
|  | Broilers |  | Tier 2 | 0,024 |
|  | Other poultry | Turkey, duck and geese | Tier 2 | 0,057 |
| Goats1 |  |  |  | Inventory - 2002 |
| Fur farming |  |  |  | Inventory - 2002 |

1 Goats will be included in emission inventory for 2002.

The emission factor for dairy cattle are higher than the IPCC default values and contrary for non dairy cattle the emission factor are lower than the IPCC default values. This can be explained by differences in manure systems. In appendix 9.3 all stable types for all livestock categories are given. A large part of the non-dairy cattle are in stable-systems with deep litter. This implies that the emission factor is relative low because the MCF (metan conversion factor) for dry lot according to IPCC is 1%.

The same emission factor is used for all years. However the emission factors are calculated based on manure conditions in 1995. Changes in stable types and in the grassing period will shift the manure types, which influences the annual variation in the emission factor. It is planned that this will be included in the emission inventory for 2002.

In Good Practice Guidance (IPCC, 2000) the MCF for liquid manure is changed to 39%. However, in this inventory the MCF is maintained on 10% since this match the climate conditions under which Massé et al., 2003 and Husted, 1994 performer their investigations.

Concerning the data used for the assignment of the distributions of husbandry manure on various manure systems (table 4.B(a)) it should be noted the distributions of non-dairy cattle is based on a weighted distribution of manure from calves, heifer, bulls and suckling cattle. The distribution in the separate categories varies substantially depending on stabling (table 5). Corresponding variation can be found for swine. The shown distribution cower only pigs for slaughtering

9.4.3 Implied emission factor

In table 6 the implied emission factor for each livestock category for each year is given. This is calculated as the total emission divided by the number of animals. It can been seen that the implied emission factor for manure management varies for dairy cattle, pigs and poultry, despite that for the whole period the same emission factor is applied.

For dairy cattle the CH4 emission in the period 1990-1998 is lower because of the production of biogas. However, in the period 1999-2001 the biogas production is not included in the calculations. Quantification of the expected reduction in the CH4 emission is currently being prepared and it is planned to apply in the emission inventory 2002.

The indirect emission factor for pigs varies in the period 1990-1998 between 2.45 to 2.47 which due to the difference in the distribution of slaughter pigs and sows having different emission factors – see table 4. The higher implied emission factor for the years 1999-2001 from 2.53 to 5.54 is caused by changes in the method. It is planned that this will be corrected in the emission inventory 2002.

The category of poultry is a combination of broilers, hens and other poultry as ducks, geese an turkeys which has different emission factors – see table 4. The indirect emission factor for all poultry varies through the years because the composition of different poultry groups differs. By a mistake there have been a double counting of poultry. The number of animal in they year of 1990 and 1992 in CRF table 4.B(a) should be revised to 21,327 and 22,907 (1000 head), respectively.

Table 6. Implied emission factor - CH4

| Implied emission factor | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | kg CH4/head/yr |  |  |  |  |  |  |  |  |  |  |  |
| 4.A Enteric Fermentation |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Cattle |  |  |  |  |  |  |  |  |  |  |  |  |
| Dairy Cattle | 104,00 | 104,00 | 104,00 | 104,00 | 104,00 | 104,00 | 104,00 | 104,00 | 104,00 | 104,00 | 104,00 | 104,00 |
| Non-Dairy Cattle | 37,40 | 37,00 | 37,00 | 37,00 | 37,00 | 37,00 | 37,00 | 37,00 | 37,00 | 37,00 | 37,00 | 37,00 |
| 3. Sheep | 8,00 | 8,00 | 8,00 | 8,00 | 8,00 | 8,00 | 8,00 | 8,00 | 8,00 | 8,00 | 8,00 | 8,00 |
| 6. Horses | 18,00 | 18,00 | 18,00 | 18,00 | 18,00 | 18,00 | 18,00 | 18,00 | 18,00 | 18,00 | 18,00 | 18,00 |
| 8. Swine | 1,50 | 1,50 | 1,50 | 1,50 | 1,50 | 1,50 | 1,50 | 1,50 | 1,50 | 1,50 | 1,50 | 1,50 |
| 9. Poultry | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 |
| 4.B Manure Management |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Cattle |  |  |  |  |  |  |  |  |  |  |  |  |
| Dairy Cattle | 21,43 | 21,19 | 21,01 | 20,87 | 20,50 | 20,17 | 20,05 | 19,61 | 19,29 | 21,80 | 21,80 | 21,80 |
| Non-Dairy Cattle | 1,60 | 1,60 | 1,60 | 1,60 | 1,60 | 1,60 | 1,60 | 1,60 | 1,60 | 1,60 | 1,60 | 1,60 |
| 3. Sheep | 0,46 | 0,46 | 0,46 | 0,46 | 0,46 | 0,46 | 0,46 | 0,46 | 0,46 | 0,46 | 0,46 | 0,46 |
| 6. Horses | 1,10 | 1,10 | 1,10 | 1,10 | 1,10 | 1,10 | 1,10 | 1,10 | 1,10 | 1,10 | 1,10 | 1,10 |
| 8. Swine | 2,47 | 2,47 | 2,47 | 2,45 | 2,45 | 2,46 | 2,46 | 2,47 | 2,45 | 2,54 | 2,54 | 2,53 |
| 9. Poultry | 0,030 | 0,032 | 0,031 | 0,031 | 0,032 | 0,032 | 0,031 | 0,031 | 0,030 | 0,034 | 0,033 | 0,033 |

9.5 N2O emissions

Figure 2 shows the distributions of sources of nitrogen oxide emission from the agricultural sector. The main part origins from agricultural soils as direct emission of which the largest sources are husbandry manure and synthetic fertiliser applied on agricultural soil and from crops residue left on the field after harvest. The indirect N2O emission includes emissions from nitrogen leaching and from atmospheric deposition.

Figure 2. N2O emission from agriculture 1990 - 2001

Emission of N2O is closely related to nitrogen balance. This means that a series of the data applied in relation to the inventory of ammonia emissions simultaneously are being applied in the calculation of N2O emissions. In Denmark a model based system for calculation of the emission of ammonia (Hutchings et al., 2001) is used. This system is also used to calculate the N2O emission from husbandry manure. The system is based on 27 different livestock categories depending on type of livestock and weight class. All several stable types are all together gives 150 different classes. Further each category is separated in to stable type systems. For each of these categories the NH3 emission is calculated based on information on i.e. N-excretion, days of grassing, nitrogen content in manure, the distribution of the different types of manure and NH3 evaporation in stables, during storage and in relation to the application of manure on the fields. Data for the calculation is collected at Danish Institute of Agricultural Sciences, Danish Agricultural Advisory Centre and Statistic Denmark. In appendix 9.3 a list over the different livestock categories are shown and some of the parameters from the NH3 model, which is applied in the calculations of the N2O emission.

9.5.1 Manure Management

The emission of N2O from manure management is given in CRF table 4.B(b). The data for the N-excretion and the distribution on the different types of manure is taken from the NH3 model. As emission factors for N2O the IPCC default values are applied.

Table 7. Data and emissions factor used in estimation of N2O emission from manure management

| Emission source | Activity data |  | Emission factor |  |
| --- | --- | --- | --- | --- |
|  |  | References | kg N2O -N/kg N | References |
| 4.B(b) Manure Management | - N-excretion in stable - Fraction of different manure type | NERI1 | Liquid + slurry = 0.001 Solid st. + dry lot = 0.02 | IPCC, 2000 -table 4.12 |

1 Data from the NH3 estimation model (appendix 9.3)

9.5.2 Agricultural Soils

In table 8 a list of data applied in the calculation of N2O emissions from agricultural soils is given including relevant references. The emission is given in CFR in table 4.D. National data for the evaporation of ammonia from the NH3 model is applied to calculate the nitrogen content of the manure. In the estimation of N2O emissions the IPCC default values are applied.

Table 8. Data and emissions factor used in estimation of N2O emission from agricultural soils.

| Emission source | Activity data |  | Emission factor |  |
| --- | --- | --- | --- | --- |
|  |  | References | kg N2O -N/kg N | References |
| 4.D Agricultural Soils |  |  |  |  |
| 1. Direct soil emissions |  |  |  |  |
| 1a. Synthetic Fertiliser | - N in fertiliser - fertiliser type - NH3 emission = 2% | PD - DIAS | 0.0125 | IPCC, 2000 -table 4.17 |
| 1.b Animal wastes applied to soils | - N-excretion in stable - NH3 emission = 28.3% | NERI1 | 0.0125 | IPCC, 2000 -table 4.17 |
| 1.c N-fixing crops | -landuse + crop product. -N-content, FracDM | DS DIAS | 0.0125 | IPCC, 2000 -table 4.17 |
| 1.d Crop Residue | -landuse + crop product -N-content and FracDM in harvest crops and residue, | DS DIAS | 0.0125 | IPCC, 2000 -table 4.17 |
| 1.e Cultivation of histosols | -Cultivated organic soils | NERI | 8 kg N2O -N/ha | IPCC, 2000 -table 4.17 |
| 2. Animal production | N-excretion on grass NH3 emission = 7% | NERI1 DIAS | Solid = 0.001 liquid = 0.02 | IPCC, 2000 - table 4.12 |
| 3. Indirect soil emissions |  |  |  |  |
| 3.a Atmospheric deposition | N-excretion, stable N-excretion, grass N in fertiliser NH3 emission | NERI1 - - DIAS | 0.01 | IPCC, 2000 – table 4.18 |
| 3.b Leaching and runoff | Amount of N leached | DIAS | 0.025 | IPCC, 2000 – table 4.18 |
| 4. Other |  |  |  |  |
| Sewage sludge and industrial waste used as fertiliser | Amount of sewage sludge N-content | EPA | 0.0125 | IPCC, 1996 -table 4.18 |

1 Data from the NH3 estimation model (appendix 9.3)

## FracGASM

The emission of NH3 + NOx from husbandry manure are in IPCC 0.2 (IPCC, 1996 – table 4-19). However, considering the Danish situation the calculation of FracGASM is higher i.e. having an average of 0.28 in period 1990 – 2001 (figures 3). From figure 3, which is based on the NH3 model, a decrease in the FracGASM in the period can be seen. In future inventories it is planned to apply a specific value for FracGASM for each year.

Figure 3. FracGASM estimated in the NH3-model, 1990 – 2001

## FracGASF

The emission of NH3 + NOx from synthetic fertiliser are in IPCC given as 0.1 (IPCC, 1996 - table 4-19). The Danish value for the FracGASF is considerably lower i.e. 0.02. This is because urea constitute less than 1% of the total use of nitrogen in the fertiliser (table 9). The primary type of fertiliser is Calcium ammonium nitrate and NPK fertiliser that has an emission factor of 0.02. Data on the use of commercial manure is based on the sale estimations (The Danish Plant Directorate). Data for emissions factor are collected from Danish Institute of Agricultural Sciences (Sommer et al, 1992, 1994 and 1996).

Table 9. Synthetic fertiliser consumption 2001 and emissions factor.

| Synthetic fertiliser year 2001 | Emissions factor1 kg NH3 + NOx/kg N | Consumption2 Mio. kg N |
| --- | --- | --- |
| Fertiliser type |  |  |
| Calcium and boron calcium nitrate | 0.02 | 0.4 |
| Ammonium sulphate | 0.05 | 2.6 |
| Calcium ammonium nitrate and other nitrate types | 0.02 | 100.2 |
| Ammonium nitrate | 0.02 | 20.5 |
| Liquid ammonia | 0.01 | 7.8 |
| Urea | 0.15 | 1.1 |
| Other nitrogen fertiliser | 0.05 | 10.3 |
| NPK-fertiliser | 0.02 | 82.7 |
| Diammonphosphate | 0.05 | 0.7 |
| Other NP fertiliser types | 0.02 | 2.2 |
| NK fertiliser | 0.02 | 5.3 |
| FracGASF Fraction of synthetic fertiliser N applied to soils that volatilizes as NH3 and NOx | 0.02 | 233.7 |

1 Danish Institute of Agricultural Sciences (Sommer et al., 1992, 1994 and 1996)

2 The Danish Plant Directorate (www.plantedirektoratet.dk)

## FracGRAZ

By estimation of NH3 emission from grassing is used a emission factor of 0.07 which, is based on English and Dutch investigations of grassing cattle (Jarvis et al., 1989a, Jarvis et al., 1989b and Bussink, 1994). This emission factor is used for all animal categories. FracGRAZ  is based on the NH3 model and in table 10 the values for the period 1990-2001 are given. The variations are due to changes in the grassing period.

Table 10. FracGRAZ – fraction of total livestock N excretion that is applied during gassing (CRF, 2003)

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FracGRAZ | 0.15 | 0.15 | 0.14 | 0.14 | 0.13 | 0.14 | 0.14 | 0.11 | 0.10 | 0.10 | 0.11 | 0.12 |

## FracLEACH

In table 11 is shown FracLEACH for the period 1990-2001. In the period 1990-1998 FracLEACH decreases from 0.33-0.31. There have been recalculations of nitrogen leaching in connection with the evaluation of The Action Plan on the Aquatic Environment. Recent data shows that the nitrogen leaching might be higher than previous estimations. This recalculation will be included in the emission inventory for 2002.

Table 11. FracLEACH – fraction of N input to soils that is lost through leaching and runoff (CRF, 2003)

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FracLEACH | 0.33 | 0.32 | 0.32 | 0.33 | 0.32 | 0.32 | 0.32 | 0.31 | 0.31 | 0.37 | 0.36 | 0.38 |

9.6 Uncertainties

Table 12 shows the estimated uncertainty for different emission sources which is based on expert judgement (Olesen et al., 2001). The uncertainties for activity data are relative small because they are based on National statistics and include measures based on a large number of data. For enteric fermentation the uncertainty for activity data is estimated to 2% (Statistic, Denmark). Uncertainties for manure management and agricultural soils are estimated to 10% and 20%, respectively. The uncertainties for agricultural soils is mainly related to estimation of nitrogen from crop residue, N-fixing crops and from N-leaching.

From enteric fermentation the uncertainty for the emission factor is estimated to 20% related to tier 2 method (IPCC, 2000). The emission factor for cattle is based on tier 2 method and this category contributes with 80% of the total emission from enteric fermentation. Uncertainty of the emission factor for manure management is estimated to 30% (Gyldenkærne, pers. comm., 2003). The emission factor depends among others on dry matter content in husbandry manure and the distribution of manure systems.

The emission factors for N2O is based on IPCC default values. These values are connected with high uncertainties as a result of different soil types and climatic conditions. The default values does not necessary correspond the Danish conditions. The estimate of 95% confidence limit range is between 0.25% to 6% as given in IPCC Guidance (2000). The uncertainties for the emission factor of agricultural soils and manure management are estimated to 500% (Gyldenkærne, pers. comm., 2003).

In connection to the thorough investigation of the method for the emission inventory it is planned to perform a calculation of uncertainties. This will bee included in the emission inventory for 2002.

Table 12. The estimated uncertainty associated with activities and emission factor for CH4 and N2O

| Emission | Source | Uncertainty1 |  | Quantitative estimation of uncertainy2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Activity data | Emission factor | Activity data | Emission factor |
| CH4 | 4.A Enteric Fermentation | * | * | 2% | 20% |
|  | 4.B(a) Manure Mangement | * | ** | 10% | 30% |
| N2O | 4.B(b) Manure Mangement | * | *** | 10% | 500% |
|  | 4.D Agricultural Soils |  |  | 20% | 500% |
|  | 1. Direct soil emissions |  |  |  |  |
|  | 1a. Syntehetic Fertilizer | * | *** |  |  |
|  | 1.b Animal waste applied to soils | * | *** |  |  |
|  | 1.c N-fixing crops | ** | *** |  |  |
|  | 1.d Crop Residue | *** | *** |  |  |
|  | 1.e Cultivation of histosols | ** | *** |  |  |
|  | 2. Animal production | * | *** |  |  |
|  | 3. Indirect soil emissions |  |  |  |  |
|  | 3.a Atmospheric deposition | * | *** |  |  |
|  | 3.b Leaching and runoff | ** | *** |  |  |
|  | 4. Other |  |  |  |  |
|  | Sewage sludge and industrial waste used as ferilizer | * | *** |  |  |

1 Olesen et al., 2001

2 Gyldenkærne pers. comm., 2003

*    uncertainty < 20%,

**   uncertainty 20-50%

*** uncertainty > 50%

9.7 QA/QC

To ensure the data quality the trend of the emission for both activity data and the emissions factors is checked out. Further the implied emission factors for CH4 and FracGASF, FracGRAZ, FracLEACH is compared for each year in the period 1990-2001. Considerable variations can reveal miscalculations or changes in methods.

The tier 2 assessment on livestock holdings and manure management is based on the Danish Ammonia Inventory System. This system is developed in corporation with and accepted by Danish Institute of Agricultural Science and the Danish Agricultural Advisory Centre and is therefor seen as a high standard. The in-house quality control is made by agronomist, Dr. PhD Steen Gyldenkærne.

Activity data and data for estimations of emission factors are collected and discussed in corporation with specialists and researcher at different institutes and research sections (table 2). It means that both the data and the methods will bee evaluated continuously according to the latest knowledge and information. At the moment there is no quality system review process. Presently it is planned to prepare a report, which in details describes the method of the emission inventory. This report will be circulate among research institutes and the environmental agency for consideration.

9.8 Recalculations

Years 1990-2001. CH4-emissions from Enteric fermentation and manure management have been changed since horses on small farms and on riding schools have been included.

Since the emission inventory report 2001 there have been recalculations for CH4-emissions from Enteric fermentation and manure management in the years 1990-2001. This is due to changes in number of horses since horses on small farms and on riding schools now are included. Previous inventory only includes horses stabling on farms bigger than five hectare as given in the Agricultural Statistics (Statistic Denmark). The number of horses is based on the stud book registration collected by The Danish Agricultural Advisory Centre. As a result of the recalculation the total emission of CH4 is increased with 1-3 Gg CH4 for each year in the period 1990-2001 (table 13).

Table 13. Changes in CH4 emission in the agricultural sector compared to CRF reported dec. 2002.

| #### CH4 - CRF | 1990 | 1991 |  | 1992 |  | 1993 |  | 1994 |  | 1995 |  | 1996 |  | 1997 |  | 1998 |  | 1999 |  | 2000 |  | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | ### Gg CH4 |  |  |  |  |  |  |  |  |  |  |  |
| CH4 – reported dec. 2002 | 193.9 | 191.6 |  | 190.4 |  | 194.5 |  | 186.9 |  | 187.0 |  | 186.2 |  | 182.2 |  | 183.5 |  | 168.3 |  | 168.1 |  | 170.9 |
| CH4 – reported april 2003 | 194.7 | 193.5 |  | 192.4 |  | 196.7 |  | 189.1 |  | 189.2 |  | 188.5 |  | 184.2 |  | 185.6 |  | 170.4 |  | 170.3 |  | 173,0 |
| Difference | 0.8 | 1.9 |  | 2.0 |  | 2.2 |  | 2.2 |  | 2.2 |  | 2.3 |  | 2.0 |  | 2.1 |  | 2.1 |  | 2.2 |  | 2.1 |

9.9 Improvements

As mentioned a recalculation of activity data and emission factors for the period 1990-2001 will be performed in accounts to the latest knowledge. It is planned that these changes will be implemented in inventory emission for 2002.

A system is established to estimate the CH4 emission similar to the NH3 model. A system which include categorisation of animal categories and distribution of different stable systems and manure types (app. 9.3). The CH4 emission will be estimated for each category based on tier 2 method. Data for estimation of emission factors are collected at NERI and Danish Institute of Agricultural Sciences.

The emission of CO2 from agricultural soils and absorption are not included in previous inventory caused by lack of data. An investigation of changes in C-content in agricultural soil is planned in 2003. Result of this investigation will be implemented in the inventory as soon as the work is completed

Planned improvement for emission inventory 2002 in punctual form:

* development of a uniform emission calculates model for NH3, N2O, CH4 and CO2 from the agricultural sector.

* calculation of an annual emission factor for CH4 from enteric fermentation and manure management

* include emission of CH4 from goats

* include the reduction of emission from biogas production in the year 1990-2002

* calculation of an annual emission factor for evaporation of NH3

* revision of N2O emission from crop residue

* revision of N2O emission from N-fixing crops

* revision of N2O emission from N-leaching and runoff from manure

* investigate the possibilities to use National data for N2O -emissions

* include CO2 emission from agricultural soils

9.10 References

Bussink, D.W., 1994 Realtionship between ammonia volatilisation and nitrogen fertilizer application rate, indtake and excretion of herbage nitrogen by cattle on grazed swards. Fertil. Res. 38, 111-121.

CFR, Common Reporting Format (http://cdr.eionet.eu.int/dk/Air_Emission_Inventories/Submission_EU)

Gyldenkærne, S. Reacher at NERI, Departement of Policy Analysis. Pers. Comm., 2003

Husted, 1994. Waste Management, Seasonal Variation in Methane Emission from Stored Slurry and Solid Manures. J. Environ. Qual. 23:585-592 (1994).

Jarvis, S.C., Hatch, D.J., Roberts, D.H., 1989a. The effects of grassland management on nitrogen losses from grazed swards through ammonia volatilization; the relationship to extral N returns from cattle. J. Agric. Sci. Camb. 112,205-216.

Jarvis, S.C., Hatch, D.J., Lockyer, D.R., 1989b. Ammonia fluxes from grazed grassland annual losses form cattle production systems and their relation to nitrogen inputs. J. Agric. Camp. 113, 99-108.

Hutchings, N.J., Sommer, S.G., Andersen, J.M., Asman, W.A.H., 2001. A detail ammonia emission inventory for Denmark. Atmospheric Environment 35 (2001) 1959-1968.

IPCC, 1996. IPCC Guidelines for National Greenhouse Gas Inventories: Reference Manual.

IPCC, 2000. IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories.

Massé, D.I., Croteau, F., Patni, N.K., Masse, L., 2003. Methane emissions from dairy cow and swine slurries stored at 10ºC and 15ºC. Agriculture and Agri-Food Canada, Canadian Biosystem Engineering, Volume 45 p. 6.1-6.6

Olesen, J.E., Fenhann, J.F., Petersen, S.O., Andersen, J.M., Jacobsen, B.H., 2001. Emission af drivhusgasser fra dansk landbrug. DJF rapport nr. 47, markbrug, Danmarks Jordbrugsforskning, 2001. (In Danish)

Poulsen, H.D., Børsting, C.F., Rom, H.B., Sommer, S.G., 2001. Kvælstof, fosfor og kalium i husdyrgødning – normtal 2000. DJF rapport nr. 36 – husdyrbrug, Danmarks Jordbrugsforskning. (In Danish)

Sommer, S.G. and Christensen, B. T. 1992. Ammonia volatilization after injection of anhydrous ammonia into arable soils of different moisture levels. Plant Soil. 142, 143-146.

Sommer, S.G. and Jensen, C. 1994. Ammonia volatilization from urea and ammoniacal fertilizers surface applied to winter wheat and grassland. Fert. Res. 37, 85-92.

Sommer, S.G. and Ersbøll, A.K. 1996. Effect of air flow rate, lime amendments and chemical soil properties on the volatilization of ammonia from fertilizers applied to sandy soils. Biol. Fertil Soils. 21, 53-60.

Statistic Denmark - Agricultural Statistic (www.dst.dk)