# 15 April 203

# Appendix 4 Stationary Combustion plants

Appendix 4 Stationary Combustion plants	1

1A1 Energy industries, 1A2 Manufacturing industries and 1A4 Other sectors	2

Methodology and references	2

SNAP / IPCC source categories	2

Large point sources	3

Area sources	3

Activity rates, fuel consumption	3

Emission factors	4

CO2 from the plastic part of municipal waste	5

Reference approach	6

Recalculation and changes	6

Uncertainty	7

QA/QC and verification	9

Future improvements	10

References	11

Appendix	12

Appendix 4.1 Emission factors	13

Appendix 4.2 Large point sources	20

Appendix 4.3 Fuel rate	24

Appendix 4.4 CO2 emission factors for municipal waste incineration	25

Appendix 4.5 CRF SNAP correspondence list	27

Appendix 4.6 Uncertainty calculations	29

## 1A1 Energy industries, 1A2 Manufacturing industries and 1A4 Other sectors

Stationary combustion plants are part of the CRF emission sources 1A1 Energy industries, 1A2 Manufacturing industries and 1A4 Other sectors.

## Methodology and references

The Danish emission inventory is based on the CORINAIR (CORe INventory on AIR emissions) system, which is a European program for air emission inventories. CORINAIR includes methodology structure and software for inventories. The methodology is described in the Emission Inventory Guidebook 3rd edition, prepared by the UNECE/EMEP Task Force on Emissions Inventories and Projections. Emission data are stored in an Access database, and data transferred to the CRF format.

The inventory of emissions from combustion processes in the CRF sources 1A1 Energy industries, 1A2 Manufacturing industries and 1A4 Other sectors is based on activity rates from the Danish energy statistics. Emission factors for different fuels, plants and sectors have been determined. Some large plants like e.g. power plants are registered individually as large point sources and emission data from the actual plants are used.

### SNAP / IPCC source categories

In the Danish emission database all activity rates and emissions are defined in SNAP sector categories (Selected Nomenclature for Air Pollution). 11 main categories are defined; categories that are further divided into a second and third level of snap. The CRF sources (IPCC source categories) 1A1, 1A2 and 1A4 are included in SNAP 01, 02, 03 and 08.

SNAP 01 includes power plants, district-heating plants, petroleum refining plants and oil/gas extraction. SNAP 02 includes commercial and institutional plants, residential plants and plants in agriculture, forestry and aquaculture. SNAP 03 includes industrial combustion in boilers, gas turbines and stationary engines and industrial combustion in processes. SNAP 08 includes amongst others mobile sources in households & gardening, in agriculture, in forestry, in industry and in national fishing. Mobile sources are not further discussed in this Appendix, please refer to Appendix 6.

Emission inventories are prepared from a complete emission database based on the SNAP categories. Aggregation to IPCC source categories in the CRF format is based on a correspondence list between SNAP and IPCC96 source categories which is included in the UNECE/EMEP Emission Inventory Guidebook. An emission source correspondence list of CRF sources 1A1, 1A2 and 1A4 is shown in Appendix 4.5.

In Denmark all municipal waste incineration is utilised for heat and power production. Thus incineration of waste is included in CRF categories 1A1, 1A2 and 1A4.

### Large point sources

Large point sources like power plants, industrial plants and refineries are included as point sources in the Danish emission database. Each point source might consist of more than one part e.g. a power plant with several units.

By registering the plants as point sources in the database it is possible to use plant specific emission factors for the plants. Plant specific emission data are obtained from:

- Annual environmental reports

- Annual plant specific reporting of SO2 and NOx from power plants >25MWe prepared for the Danish Energy Authority due to Danish legislation /3/

Annual environmental reports from the plants include a considerable number of emission data sets. In general emission data from annual environmental reports are based on emission measurements, but some emissions might have been calculated from general emission factors. Only some of the pollutants included in the UNFCCC inventory is reported in the annual environmental reports or in other plant specific reports. Emission of the remaining pollutants is based on general emission factors for the fuel and source category. CO2, CH4 and N2O from large point sources included in CRF 1A1, 1A2 and 1A4 are all based on the general emission factors.

In the year 2001 63 large point sources within CRF source category 1A1, 1A2 and 1A4 are specified in the Danish emission database. These point sources includes:

- Power plants and cogeneration plants (combined heat and power plants)

- Municipal waste incineration plants

- A few large industrial plants

- Petroleum refining plants

The fuel consumption of large point sources in 1A1, 1A2 and 1A4 is 320 PJ (2001). This corresponds to 53% of the overall fuel consumption of 1A1, 1A2 and 1A4.

A list of large point sources 2001 and the fuel consumption rates is shown in appendix 4.2. A table showing which large point source emissions are based on plant specific emission factors is also included in the appendix. The number of large point sources registered in the databases has been increasing from 1990 till 2001.

### Area sources

Fuels not combusted in large point sources are included as sector specific area sources in the emission database. Plants like residential boilers, small district heating plants, small cogeneration plants and industrial plants are defined as area sources. Emissions from area sources are based on fuel consumption data and emission factors. Further information about emission factors is given below.

### Activity rates, fuel consumption

The fuel consumption rates are based on the official Danish energy statistics prepared by the Danish Energy Authority. The Danish Energy Authority aggregates fuel consumption rates to SNAP categories. Some fuel categories of the official Danish energy statistics are added to obtain a less detailed fuel aggregation level.

The Danish energy statistics does not specify fuel consumption rates in specific industries. Thus all fuel consumption of 1A2 Manufacturing industries and construction is included in 1A2f Other.

Traded and not traded fuels are included in the Danish energy statistics. Thus e.g. an estimation of the annual consumption of not traded wood is included.

Emissions from petroleum coke bought abroad and combusted in Danish residential plants (border trade of 251 TJ) are included in the inventory.

The fuel consumption of district heating and power producing plants are each year reported by the plant owners to the Danish Energy Authority. The fuel consumption of large point sources specified in the Danish emission databases refers to a plant specific database from the Danish Energy Authority. The fuel consumption of area sources is calculated as total fuel consumption minus fuel consumption of large point sources.

Emissions from non-energy use of fuels are not yet included in the Danish national approach, but have been included in the reference approach.

Fuel rates of CRF sources 1A1, 1A2 and 1A4 is shown in Appendix 4.3.

### Emission factors

For each fuel and SNAP (sector and e.g. type of plant) a set of general emission factors has been determined. The emission factors are either national referenced or based on the EMEP/CorinAir Guidebook. Emission factors for area sources year 2001 are shown in Appendix 4.1. Time series of greenhouse gas emission factors are also shown.

Most country specific emission factors refers to:

- Danish legislation

- Danish research reports

- Calculations based on plant specific emissions from a considerable number of power plants

- Calculations based on plant specific emissions from a considerable number of municipal waste incineration plants

References of each area source emission factor are shown in Appendix 4.1. Note that SO2 and NOx emissions from large point sources are often based on emission measurements and thus they are plant specific. Some CO and NMVOC emissions are also plant specific. If emissions are not stated in annual environmental reports or other plant specific reports the general area source emission factor is used.

CO2 emission factors for fuels are shown in Table 1.

Table 1 Emission factors 2001, CO2

| Fuel | Biomass | Emission factor | Unit | Reference type 1) |
| --- | --- | --- | --- | --- |
| Coal |  | 95 | kg/GJ | cs |
| Petroleum coke |  | 92 | kg/GJ | cs |
| Wood | x | 102 | kg/GJ | c |
| Municipal waste (biomass part) | x | 97,8 | kg/GJ | cs |
| Municipal waste (plastic part) |  | 19,2 | kg/GJ | cs |
| Agricultural waste (straw) | x | 102 | kg/GJ | cs |
| Residual oil |  | 78 | kg/GJ | c |
| Gas oil |  | 74 | kg/GJ | c |
| Kerosene |  | 72 | kg/GJ | c |
| Liquid bio fuel | x | 102 | kg/GJ | c |
| Orimulsion |  | 80 | kg/GJ | cs |
| Natural gas |  | 57,25 | kg/GJ | cs |
| LPG |  | 52 | kg/GJ | c |
| Refinery gas |  | 57,1 | kg/GJ | cs |
| Biogas | x | 83,6 | kg/GJ | cs |

1) CS: country specific, C: CORINAIR

Some of the indicated emission factors (IEF) in Table 1A(a) of the CRF differ from IEF reported by other countries:

- Indicated emission factors for NMVOC and CH4 of gaseous fuels in CRF Table 1A(a) are above the general level. This is due to the considerable number gas fuelled lean burn engines installed in cogeneration plants. The emission of unburned hydrocarbon from lean burn gas engines is much higher than from boilers or gas turbines. The emission factors are documented in several Danish reports including /4/. There has been a rapid increase in number and gas consumption rate of these plants, thus resulting in a considerable increase of IEF since 1990.

- IEF of CH4 and NMVOC of biomass are relatively high in Denmark duel to a considerable number of biogas fuelled lean burn gas engines in cogeneration plants.

### CO2 from the plastic part of municipal waste

The CO2 emission from the plastic part of municipal waste incineration is included in the Danish national approach. The CO2 emission from incineration of municipal waste is split in two parts: The emission from combustion of the plastic content of the waste and the emission from combustion of the rest of the waste – the biomass part. The CO2 emission from the plastic part is included in the fuel category Other fuels with a fuel consumption rate of zero. Thus the CO2 emission from the plastic part of incinerated waste (Table 1A(a), Other fuels, 1A1a, 1A2f and 1A4a) occurs without at corresponding fuel consumption. The CO2 emission from the non-plastic part is included in the fuel category Biomass applying the total consumption rate of municipal waste. Calculation of the two CO2 emission factors for municipal waste is based on knowledge of plastic content of waste and the lower heating value of plastic (assumed to be the same as for crude oil) and biomass waste part respectively. Calculations are shown in Appendix 4.4.

Other pollutants from municipal waste incineration are all included in fuel category biomass.

### Reference approach

In addition to the detailed emission calculation in the national approach CO2 emission from fuel combustion is aggregated using the reference approach. Data used in the reference approach originate from the annual “basicdata” table prepared by the Danish Energy Authority and published on their home page. Some fuel categories have been aggregated to follow fuel categories in the CRF Table1A(b).

The fraction of carbon oxidised has been assumed to be 1,00. The carbon emission factor originates from the IPCC Reference Manual /8/.

In CRF Table 1A(c) the CO2 emission calculated in the national approach is compared with the CO2 emission calculated in the reference approach. To make results comparable:

- CO2 emission from the plastic part of municipal waste incineration is added in Reference Approach. (Other fuels of sources 1A1, 1A2 and 1A4).

- Non-energy use of fuels is not included in the Danish National Approach. Thus consumption for non-energy (10,63 PJ) is subtracted in Reference Approach to make results comparable. Inclusion of non-energy use of fuels in the national approach will be considered in future inventories.

In 2001 fuel consumption rates of the two approaches differ 1,19% and the CO2 emission differ 0,94%. In 1990-2001 the fuel consumption difference is within 1,91% and the CO2 emission difference is within 1,69%.

## Recalculation and changes

Emission factors for CH4, N2O, CO and NMVOC, 1990-2000

Emission factors for CH4, N2O, CO and NMVOC have been revised. Some emission factors have been changed and now refer to the EMEP Guidebook or to Danish references. Consistency of factors has been improved. References of the emission factors have been included in the 2001 emission database.

Correction of errors

Some corrections of emission factor errors (1990-2000) have been carried out since last year. The corrections are registered in a (manual) log table in the emission database. The errors however have not influenced the total emission considerably.

CO2 emission from incineration of municipal waste

Up to now the emission from the plastic part of the waste have been included in the fuel category Liquid fuels in CRF. Now this emission is included in the fuel category Other fuels. This change of aggregation does not change the total emission reported but improves comparability of IEFs to IEFs of inventories from other countries.

The CO2 emission from non-plastic part of waste is includes in the fuel category Biomass as in the former inventories.

In former inventories the energy content of municipal waste have been counted twice in the National approach. The energy content was included both as Biomass and as Liquid fuel. Now the energy content is included only as Biomass. Thus the CO2 emission from the plastic part of incinerated waste (Other fuels) occurs without at corresponding energy consumption.

Table 1A(a)s1-4:

The change of aggregation cause considerable changes of Table 1A(a)s1-4. The total CO2 emission is unchanged but a considerable part of the emission is now included in fuel category Other fuels instead of Liquid fuels. Energy consumption, CO2 emission and IEF change in the sources 1A1, 1A2 and 1A4. The CO2 IEFs of Liquid fuel are now comparable to those of other countries.

Table 1A(c):

In the National approach energy consumption and CO2 emission of Liquid fuels is lower than in former inventories. Emission from Other fuels is not included in Table 1A(c). Thus it is not necessary to make the correction in Table 1A(c) that was made last year.

Reference approach

The reference approach has been recalculated in 2003. Some specific fuels have been moved to other fuel categories in Table 1A(b) as a result of comments in the review last year. The recalculation is based on a revised energy statistics.

## Uncertainty

Estimation of uncertainty is based on the Tier 1 methodology in IPCC Good Practice Guidance (GPG) /2/.

The uncertainty of greenhouse gas emissions and the uncertainty of total Global Warming Potential (GWP) is shown in Table 2. The uncertainty of Global Warming Potential is estimated to be 11% and the uncertainty in trend of GWP is 1,8%.

Main sources of uncertainty of GWP are N2O emission and CO2 emission from coal combustion. Main sources of trend uncertainty of GWP are CO2 emission from natural gas consumption and CO2 emission from coal combustion. The uncertainties of these sources should be further analysed to improve uncertainty estimates in the future.

Table 2 Uncertainty of greenhouse gases

| Pollutant | Uncertainty of emission inventory [%] | Uncertainty of emission trend [%] |
| --- | --- | --- |
| CO2 | 2,6 | 1,8 |
| CH4 | 39 | 389 |
| N2O | 1000 | 3,1 |
| GWP | 11 | 1,8 |

CO2:

The source categories follow the suggested aggregation of key source analyses /2/, and thus CO2 emissions from stationary combustion plants are disaggregated only to fuel level.

The activity rate uncertainties refers to Table 2.6 in GPG. The fuel consumption rates of the different sectors are considered and the uncertainties of fuel rates determined - see appendix 2.6.

Uncertainties of CO2 emission factors for fuels are assumed to be less than 5% referring to GPG page 2.15. Uncertainty of emission factors for coal (5%), orimulsion (2%), fuel oil (2%) and natural gas (1%) is based on a Danish research report /6/.

Overall uncertainty of the Danish CO2 emission from stationary combustion plants year 2001 is 2,6% and the trend uncertainty is 1,8%. Calculation sheet is shown in appendix 4.6.

CH4:

Two emission sources are considered in the CH4 uncertainty analysis: Gas engines and other stationary combustion plants. This aggregation level differs from the key source analysis. This aggregation is preferred due to the much higher emission factor for gas engines and due to the fact that measurements of CH4 emissions from gas engines have been carried out on a large number of plants - contrary to other stationary combustion plants. Thus CH4 emission factors for gas engines are more accurate than emission factors for other stationary combustion plants.

Uncertainty of CH4 emission factors for gas engines is estimated to be less than 40% /5/. Uncertainty of other stationary sources is assumed to be 100% referring to GPG Table 2.5. The combined fuel rate uncertainty of stationary combustion plants is estimated to 2,2% (Appendix 4.6). Calculation of CH4 emissions from gas engines is based on the Danish emission database that specifies fuel consumption in reciprocating engines.

Overall uncertainty of the Danish CH4 emission from stationary combustion plants year 2001 is 39% and the trend uncertainty is 389%. Calculation sheet is shown in appendix 4.6.

N2O:

Stationary combustion plants are not disaggregated following the key source analysis. Uncertainty of N2O emission factors is assumed to be 1000% referring to GPG Table 2.5. The combined fuel rate uncertainty of stationary combustion plants is again estimated to be 2,0%.

The overall uncertainty of the Danish N2O emission from stationary combustion plants year 2001 is 1000% and the trend uncertainty is 2,3%. Calculation sheet is shown in appendix 4.6.

SO2, NOx, NMVOC and CO

The uncertainty estimation of SO2, NOx, NMVOC and CO are based on the methodology in Good Practice Guidance for CLRTAP Emission Inventories /7/. The methodology follows the tier 1 methodology of GPG. Source categories follow the SNAP main sectors thus 3 categories of stationary combustion plants are considered.

Uncertainty of fuel rates are assumed to be 2% based on /7/. Uncertainty of emission factors are taken from /7/ assuming that uncertainty is always in the lower end of the interval given in chapter 2.3.3 b. Estimated uncertainties of SO2, NOx, NMVOC and CO from stationary combustion plants are shown in Table 3. In general the calculated uncertainties in this first attempt estimation are assumed to be overestimated due to the fact that a considerable part of e.g. SO2 and NOx emissions in the Danish inventory is based on emissions measurements.

Table 3 Uncertainty, SO2, NOx, NMVOC and CO

| Pollutant | Uncertainty of emission inventory [%] | Uncertainty of emission trend [%] |
| --- | --- | --- |
| SO2 | 7 | 0,5 |
| NOx | 15 | 3,1 |
| NMVOC | 36 | 23 |
| CO | 45 | 6 |

## QA/QC and verification

A formal QA/QC plan has not yet been developed, but a number of quality control (QC) procedures are performed. The Danish QC includes:

- Check of time series of the CRF and SNAP source categories. Considerable changes are checked and explained.

- Comparison to inventory of the previous year. Any major changes are verified.

- Total emissions when aggregated to CRF source categories are compared to totals based on SNAP source categories (control of data transfer).

- A manual log table have been introduced in the emission databases to collect information about recalculations

In addition to the general QC procedures some additional QC are performed for stationary combustion plants:

General:

- The CRF reference approach validates the fuel consumption rates and CO2 emissions of fuel combustion. Fuel consumption rates are within 1,9% difference (1990-2001) and CO2 emissions are within 1,7% difference.

- The emission from each large point source is compared to the emission reported the previous year.

- Some automated checks have been prepared for the emission databases:

- Check of units for fuel rate, emission factor and plant specific emissions

- Check of emission factors of large point sources. Emission factors of pollutants that are not plant specific should be the same as the emission factor that are defined for area sources.

- Additional checks of database consistency

- Most emission factor references are now implemented in the emission database itself.

Country specific emission factors:

- Annual environmental reports are kept for subsequent control of plant specific emission data

- QA/QC checks of the country specific emission factors have not been performed but most factors are based on work from companies that have implemented some QA/QC work. The two major power plant owners / operators in Denmark: E2 and Elsam both obtained the ISO 14001 certification for environmental management system. Danish Gas Technology Centre and dk-Teknik both run accredited laboratories for emission measurements.

Uncertainty

QA/QC of uncertainty estimates have not yet been performed.

## Future improvements

Some improvements are planned for the future greenhouse gas emissions inventory.

A research project aiming at improving emission factors for cogeneration plants (combined heat and power production) <25MWe will be reported in May 2003 /9/. The emission factors determined in this work is better documented than the emission factors used presently. E.g. N2O emission measurements have been performed on a considerable number of plants. In addition to improvements of the emission factors themselves it will be possible to make better estimates of uncertainties. Further, time series of the CH4 emission factor for gas engines will be implemented.

Improved estimates of CO2 emission from incineration of the plastic part of municipal waste are planned. Up till now the lower heating value of plastic is assumed to equal the lower heating value of crude oil. A heating value for plastic will be used in future inventories.

The first attempt uncertainty analyse that has been reported this year is excepted to be improved next year when more country specific uncertainties for fuel consumption and emission factors have been incorporated.

## References

- Emission Inventory Guidebook 3rd edition, prepared by the UNECE/EMEP Task Force on Emissions Inventories and Projections

- Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, IPCC

- Bekendtgørelse om begrænsning af udledning af svovldioxid og kvælstofoxider fra kraftværker (Kvotebekendtgørelsen). Bekendtgørelse 885 af 18/12/1991 (Danish legislation)

- Emissionsforhold for gasdrevne kraftvarmeanlæg <25MWe, Arbejdsrapport fra Miljøstyrelsen nr 17 1997 (In Danish)

- Estimated by Danish Gas Technology Centre, mail 10.04.2003

- Kontrol af indberetning af CO2-udledning fra el-producenter i 2001, Carl Bro for Energistyrelsens 6. kontor (mail 13.08.2002) (In Danish, not published)

- Good Practice Guidance for CLRTAP Emission Inventories, Draft chapter for the UNECE Corinair Guidebook, 7 November 2001

- Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual

- Kortlægning af emissioner fra decentrale kraftvarmeværker, Eltra PSO projekt 3141 (In Danish, in print)

## Appendix

### Appendix 4.1 Emission factors

Table 4 Emission factors 2001, CO2

| Fuel | Biomass | Emission factor | Unit | Reference type | Reference |
| --- | --- | --- | --- | --- | --- |
| Coal |  | 95 | kg/GJ | cs | Elsam (major producer of electricity and district heating) |
| Petroleum coke |  | 92 | kg/GJ | cs | SK Energi |
| Wood | x | 102 | kg/GJ | c | - |
| Municipal waste (biomass part) | x | 97,8 | kg/GJ | cs | Calculation based on data from Danish Environmental Protection Agency. See appendix 4.4. |
| Municipal waste (plastic part) |  | 19,2 | kg/GJ | cs |  |
| Agricultural waste (straw) | x | 102 | kg/GJ | cs | - |
| Residual oil |  | 78 | kg/GJ | c | 1) |
| Gas oil |  | 74 | kg/GJ | c | 1) |
| Kerosene |  | 72 | kg/GJ | c | 1) (~73 kg/GJ) |
| Liquid bio fuel | x | 102 | kg/GJ | c | - |
| Orimulsion |  | 80 | kg/GJ | cs | The Danish Energy Authority |
| Natural gas |  | 57,25 | kg/GJ | cs | Danish Gas Technology Centre, calculation based on average gas quality stated by DONG |
| LPG |  | 52 | kg/GJ | c | 1) |
| Refinery gas |  | 57,1 | kg/GJ | cs | Assumed same emission factor as natural gas |
| Bio gas | x | 83,6 | kg/GJ | cs | Calculation based of typical manure gas stated by Danish Gas Technology Centre |

1) Emission Inventory Guidebook 3rd edition, prepared by the UNECE/EMEP

Table 5 Emission factors 1990-2001, CO2

| fuel | Unit | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coal | kg/GJ | 95 | 95 | 95 | 95 | 95 | 95 | 95 | 95 | 95 | 95 | 95 | 95 |
| Petroleum coke | kg/GJ | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 92 | 92 | 92 | 92 |
| Wood | kg/GJ | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 |
| Municipal waste (biomass part) | kg/GJ | 92,56 | 92,56 | 94,73 | 95,68 | 95,68 | 96,96 | 97,91 | 97,91 | 97,6 | 97,8 | 97,8 | 97,8 |
| Municipal waste (plastic part) | kg/GJ | 24,44 | 24,44 | 22,27 | 21,32 | 21,32 | 20,04 | 19,09 | 19,09 | 19,4 | 19,2 | 19,2 | 19,2 |
| Agricultural waste (straw) | kg/GJ | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 | 102 |
| Residual oil | kg/GJ | 78 | 78 | 78 | 78 | 78 | 78 | 78 | 78 | 78 | 78 | 78 | 78 |
| Gas oil | kg/GJ | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 | 74 |
| Kerosene | kg/GJ | 72 | 72 | 72 | 72 | 72 | 72 | 72 | 72 | 72 | 72 | 72 | 72 |
| Liquid bio fuel | kg/GJ |  |  |  |  |  |  |  |  |  |  |  | 102 |
| Orimulsion | kg/GJ |  |  |  |  |  | 80 | 80 | 80 | 80 | 80 | 80 | 80 |
| Natural gas | kg/GJ | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 57,1 | 57,25 |
| LPG | kg/GJ | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 65 | 52 |
| Refinery gas | kg/GJ | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 56,9 | 57,1 | 57,1 |
| Bio gas | kg/GJ | 83,6 | 83,6 | 83,6 | 83,6 | 83,6 | 83,6 | 83,6 | 83,6 | 83,6 | 83,6 | 83,6 | 83,6 |

Table 6 Emission factors 2001

| fuel | ipcc_id | snap | unit1 | unit2 | CH4 | Reference |  | N2O | Reference |  | SO2 | Reference |  | NOX | Reference |  | NMVOC | Reference |  | CO | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AGRICUL. WASTES | 1A1a | 010102 | g | GJ | 32 | C | 1 | 4 | C | 1 | 100 | CS | 5 19 | 153 | SC | 28 | 50 | C | 1 | 50 | CS | 3 |
| AGRICUL. WASTES | 1A1a | 010103 | g | GJ | 32 | C | 1 | 4 | C | 1 | 100 | CS | 5 19 | 156 | CS | 4 | 50 | C | 1 | 50 | CS | 3 |
| AGRICUL. WASTES | 1A1a | 010202 | g | GJ | 32 | C | 1 | 4 | C | 1 | 100 | CS | 5 19 | 153 | CS | 28 | 50 | C | 1 | 325 | CS | 4 5 |
| AGRICUL. WASTES | 1A1a | 010203 | g | GJ | 32 | C | 1 | 4 | C | 1 | 100 | CS | 5 19 | 156 | CS | 4 | 50 | C | 1 | 325 | CS | 4 5 |
| AGRICUL. WASTES | 1A2f | 030102 | g | GJ | 32 | C | 1 | 4 | C | 1 | 100 | CS | 5 19 | 153 | CS | 28 | 50 | C | 1 | 325 | CS | 4 5 |
| AGRICUL. WASTES | 1A2f | 030105 | g | GJ | 32 | C | 1 | 4 | C | 1 | 100 | CS | 5 19 | 153 | CS | 28 | 50 | C | 1 | 325 | CS | 4 5 |
| AGRICUL. WASTES | 1A4a | 0201 | g | GJ | 200 | C | 1 | 4 | C | 1 | 100 | CS | 5 19 | 153 | CS | 28 | 600 | C | 1 | 325 | CS | 4 5 |
| AGRICUL. WASTES | 1A4b | 0202 | g | GJ | 200 | C | 1 | 4 | C | 1 | 100 | CS | 5 19 | 153 | CS | 28 | 600 | C | 1 | 4000 | CS | 1 6 7 |
| AGRICUL. WASTES | 1A4c | 0203 | g | GJ | 200 | C | 1 | 4 | C | 1 | 100 | CS | 5 19 | 153 | CS | 28 | 600 | C | 1 | 325 | CS | 4 5 |
| AGRICUL. WASTES | 1A4c | 020302 | g | GJ | 200 | C | 1 | 4 | C | 1 | 100 | CS | 5 19 | 153 | CS | 28 | 600 | C | 1 | 325 | CS | 4 5 |
| BIOGAS | 1A1a | 010102 | g | GJ | 4 | C | 1 | 2 | C | 1 | 11 | CS | 26 | 31 | CS | 4 | 4 | C | 1 | 36 | CS | 8 |
| BIOGAS | 1A1a | 010103 | g | GJ | 4 | C | 1 | 2 | C | 1 | 11 | CS | 26 | 31 | CS | 4 | 4 | C | 1 | 36 | CS | 8 |
| BIOGAS | 1A1a | 010105 | g | GJ | 434 | CS | 2 | 2 | C | 1 | 11 | CS | 26 | 605 | CS | 2 | 4 | C | 1 | 255 | CS | 2 |
| BIOGAS | 1A1a | 010203 | g | GJ | 4 | C | 1 | 2 | C | 1 | 11 | CS | 26 | 31 | CS | 4 | 4 | C | 1 | 36 | CS | 8 |
| BIOGAS | 1A1c | 010405 | g | GJ | 434 | CS | 2 | 2 | C | 1 | 11 | CS | 26 | 605 | CS | 2 | 4 | C | 1 | 255 | CS | 2 |
| BIOGAS | 1A1c | 010505 | g | GJ | 434 | CS | 2 | 2 | C | 1 | 11 | CS | 26 | 605 | CS | 2 | 4 | C | 1 | 255 | CS | 2 |
| BIOGAS | 1A2f | 0301 | g | GJ | 4 | C | 1 | 2 | C | 1 | 11 | CS | 26 | 31 | CS | 4 | 4 | C | 1 | 36 | CS | 8 |
| BIOGAS | 1A2f | 030102 | g | GJ | 4 | C | 1 | 2 | C | 1 | 11 | CS | 26 | 66 | CS | 4 | 4 | C | 1 | 36 | CS | 8 |
| BIOGAS | 1A2f | 030105 | g | GJ | 434 | CS | 2 | 2 | C | 1 | 11 | CS | 26 | 605 | CS | 2 | 4 | C | 1 | 255 | CS | 2 |
| BIOGAS | 1A4a | 0201 | g | GJ | 4 | C | 1 | 2 | C | 1 | 11 | CS | 26 | 31 | CS | 4 | 4 | C | 1 | 36 | CS | 8 |
| BIOGAS | 1A4a | 020103 | g | GJ | 4 | C | 1 | 2 | C | 1 | 11 | CS | 26 | 31 | CS | 4 | 4 | C | 1 | 36 | CS | 8 |
| BIOGAS | 1A4a | 020105 | g | GJ | 434 | CS | 2 | 2 | C | 1 | 11 | CS | 26 | 605 | CS | 2 | 4 | C | 1 | 255 | CS | 2 |
| BIOGAS | 1A4c | 0203 | g | GJ | 4 | C | 1 | 2 | C | 1 | 11 | CS | 26 | 31 | CS | 4 | 4 | C | 1 | 36 | CS | 8 |
| BIOGAS | 1A4c | 020304 | g | GJ | 434 | CS | 2 | 2 | C | 1 | 11 | CS | 26 | 605 | CS | 2 | 4 | C | 1 | 255 | CS | 2 |
| BLACK LIQUOR | 1A1a | 010203 | g | GJ | 32 | CS | 15 | 4 | C | 4 | 100 | CS | 15 | 153 | CS | 15 | 50 | CS | 15 | 325 | CS | 15 |
| BLACK LIQUOR | 1A4c | 020304 | g | GJ | 200 | CS | 15 | 4 | C | 4 | 100 | CS | 15 | 153 | CS | 15 | 600 | CS | 15 | 325 | CS | 15 |
| GAS OIL | 1A1a | 0101 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 65 | CS | 28 | 1,5 | C | 1 | 15 | CS | 3 |
| GAS OIL | 1A1a | 010101 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 65 | CS | 28 | 1,5 | C | 1 | 15 | CS | 3 |
| GAS OIL | 1A1a | 010102 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 65 | CS | 28 | 1,5 | C | 1 | 15 | CS | 3 |
| GAS OIL | 1A1a | 010103 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 52 | CS | 4 | 1,5 | C | 1 | 15 | CS | 3 |
| GAS OIL | 1A1a | 010104 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 350 | C | 1 | 2 | C | 1 | 15 | CS | 3 |
| GAS OIL | 1A1a | 010105 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 700 | - | - | 100 | C | 1 | 100 | C | 1 |
| GAS OIL | 1A1a | 010202 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 65 | CS | 28 | 1,5 | C | 1 | 30 | C | 1 |
| GAS OIL | 1A1a | 010203 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 52 | CS | 4 | 1,5 | C | 1 | 30 | C | 1 |
| GAS OIL | 1A1a | 010205 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 700 | - | - | 100 | C | 1 | 100 | C | 1 |
| GAS OIL | 1A2f | 0301 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 52 | CS | 4 | 1,5 | C | 1 | 30 | C | 1 |
| GAS OIL | 1A2f | 030102 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 52 | CS | 4 | 1,5 | C | 1 | 30 | C | 1 |
| GAS OIL | 1A2f | 030103 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 52 | CS | 4 | 1,5 | C | 1 | 30 | C | 1 |
| GAS OIL | 1A2f | 030104 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 350 | C | 1 | 2 | C | 1 | 15 | CS | 3 |
| GAS OIL | 1A2f | 030105 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 700 | - | - | 100 | C | 1 | 100 | C | 1 |
| GAS OIL | 1A2f | 030106 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 52 | CS | 4 | 1,5 | C | 1 | 30 | C | 1 |
| GAS OIL | 1A4a | 0201 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 52 | CS | 4 | 3 | C | 1 | 30 | C | 1 |
| GAS OIL | 1A4a | 020103 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 52 | CS | 4 | 3 | C | 1 | 30 | C | 1 |
| GAS OIL | 1A4a | 020105 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 700 | - | - | 100 | C | 1 | 100 | C | 1 |
| GAS OIL | 1A4b | 0202 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 52 | CS | 4 | 3 | C | 1 | 43 | C | 1 |
| GAS OIL | 1A4c | 020304 | g | GJ | 1,5 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 700 | - | - | 100 | C | 1 | 100 | C | 1 |
| KEROSENE | 1A2f | 0301 | g | GJ | 7 | C | 1 | 2 | C | 1 | 23 | CS | 27 | 73 | C | 1 | 3 | C | 1 | 20 | C | 1 |
| KEROSENE | 1A4a | 0201 | g | GJ | 7 | C | 1 | 2 | C | 1 | 23 | CS | 30 | 73 | C | 1 | 3 | C | 1 | 20 | C | 1 |
| KEROSENE | 1A4b | 0202 | g | GJ | 7 | C | 1 | 2 | C | 1 | 23 | CS | 30 | 73 | C | 1 | 3 | C | 1 | 20 | C | 1 |
| KEROSENE | 1A4c | 0203 | g | GJ | 7 | C | 1 | 2 | C | 1 | 23 | CS | 30 | 73 | C | 1 | 3 | C | 1 | 20 | C | 1 |
| LPG | 1A1a | 010203 | g | GJ | 1 | C | 1 | 1 | C | 1 | 1 | - | - | 50 | C | 1 | 2 | C | 1 | 25 | C | 1 |
| LPG | 1A2f | 0301 | g | GJ | 1 | C | 1 | 2 | C | 1 | 1 | - | - | 50 | C | 1 | 2 | C | 1 | 25 | C | 1 |
| LPG | 1A4a | 0201 | g | GJ | 1 | C | 1 | 2 | C | 1 | 1 | - | - | 50 | C | 1 | 2 | C | 1 | 25 | C | 1 |
| LPG | 1A4b | 0202 | g | GJ | 1 | C | 1 | 2 | C | 1 | 1 | - | - | 50 | C | 1 | 2 | C | 1 | 25 | C | 1 |
| MUNICIP. WASTES | 1A1a | 010102 | g | GJ | 6 | C | 1 | 4 | C | 1 | 69 | CS | 9 | 150 | CS | 9 | 9 | C | 1 | 10 | CS | 9 |
| MUNICIP. WASTES | 1A1a | 010103 | g | GJ | 6 | C | 1 | 4 | C | 1 | 69 | CS | 9 | 150 | CS | 9 | 9 | C | 1 | 10 | CS | 9 |
| MUNICIP. WASTES | 1A1a | 010104 | g | GJ | 6 | C | 1 | 4 | C | 1 | 69 | CS | 9 | 150 | CS | 9 | 9 | C | 1 | 10 | CS | 9 |
| MUNICIP. WASTES | 1A1a | 010105 | g | GJ | 6 | C | 1 | 4 | C | 1 | 69 | CS | 9 | 150 | CS | 9 | 9 | C | 1 | 10 | CS | 9 |
| MUNICIP. WASTES | 1A1a | 010203 | g | GJ | 6 | C | 1 | 4 | C | 1 | 69 | CS | 9 | 150 | CS | 9 | 9 | C | 1 | 10 | CS | 9 |
| MUNICIP. WASTES | 1A4a | 0201 | g | GJ | 6 | C | 1 | 4 | C | 1 | 69 | CS | 9 | 150 | CS | 9 | 9 | C | 1 | 10 | CS | 9 |
| MUNICIP. WASTES | 1A4a | 020103 | g | GJ | 6 | C | 1 | 4 | C | 1 | 69 | CS | 9 | 150 | CS | 9 | 9 | C | 1 | 10 | CS | 9 |
| NATURAL GAS | 1A1a | 0101 | g | GJ | 6 | CS | 14 | 1 | C | 1 | 0,3 | CS | 17 | 100 | CS | 18 | 2 | CS | 14 | 15 | CS | 3 |
| NATURAL GAS | 1A1a | 010101 | g | GJ | 6 | CS | 14 | 1 | C | 1 | 0,3 | CS | 17 | 88 | CS | 9 | 2 | CS | 14 | 15 | CS | 3 |
| NATURAL GAS | 1A1a | 010102 | g | GJ | 6 | CS | 14 | 1 | C | 1 | 0,3 | CS | 17 | 88 | CS | 9 | 2 | CS | 14 | 15 | CS | 3 |
| NATURAL GAS | 1A1a | 010103 | g | GJ | 15 | CS | 11 | 1 | C | 1 | 0,3 | CS | 17 | 30 | CS | 4 | 2 | CS | 14 | 15 | CS | 3 |
| NATURAL GAS | 1A1a | 010104 | g | GJ | 4 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 88 | CS | 9 | 1 | CS | 2 | 7 | CS | 10 |
| NATURAL GAS | 1A1a | 010105 | g | GJ | 573 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 193 | CS | 2 | 163 | CS | 2 | 169 | CS | 2 |
| NATURAL GAS | 1A1a | 010202 | g | GJ | 6 | CS | 14 | 1 | C | 1 | 0,3 | CS | 17 | 100 | CS | 18 | 2 | CS | 14 | 28 | CS | 4 |
| NATURAL GAS | 1A1a | 010203 | g | GJ | 15 | CS | 11 | 1 | C | 1 | 0,3 | CS | 17 | 30 | CS | 4 | 2 | CS | 14 | 28 | CS | 4 |
| NATURAL GAS | 1A1b | 010304 | g | GJ | 4 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 174 | CS | 2 | 1 | CS | 2 | 7 | CS | 10 |
| NATURAL GAS | 1A1c | 010405 | g | GJ | 573 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 193 | CS | 2 | 163 | CS | 2 | 169 | CS | 2 |
| NATURAL GAS | 1A1c | 010502 | g | GJ | 6 | CS | 14 | 1 | C | 1 | 0,3 | CS | 17 | 100 | CS | 18 | 2 | CS | 14 | 28 | CS | 4 |
| NATURAL GAS | 1A1c | 010504 | g | GJ | 4 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 174 | CS | 2 | 1 | CS | 2 | 7 | CS | 10 |
| NATURAL GAS | 1A1c | 010505 | g | GJ | 573 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 193 | CS | 2 | 163 | CS | 2 | 169 | CS | 2 |
| NATURAL GAS | 1A2f | 0301 | g | GJ | 6 | CS | 14 | 1 | C | 1 | 0,3 | CS | 17 | 30 | CS | 4 | 2 | CS | 14 | 28 | CS | 4 |
| NATURAL GAS | 1A2f | 030103 | g | GJ | 15 | CS | 11 | 1 | C | 1 | 0,3 | CS | 17 | 30 | CS | 4 | 2 | CS | 14 | 28 | CS | 4 |
| NATURAL GAS | 1A2f | 030104 | g | GJ | 4 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 174 | CS | 2 | 1 | CS | 2 | 7 | CS | 10 |
| NATURAL GAS | 1A2f | 030105 | g | GJ | 573 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 193 | CS | 2 | 163 | CS | 2 | 169 | CS | 2 |
| NATURAL GAS | 1A2f | 030106 | g | GJ | 15 | CS | 11 | 1 | C | 1 | 0,3 | CS | 17 | 30 | CS | 4 | 2 | CS | 14 | 28 | CS | 4 |
| NATURAL GAS | 1A4a | 0201 | g | GJ | 6 | CS | 14 | 1 | C | 1 | 0,3 | CS | 17 | 30 | CS | 4 | 2 | CS | 14 | 28 | CS | 4 |
| NATURAL GAS | 1A4a | 020103 | g | GJ | 15 | CS | 11 | 1 | C | 1 | 0,3 | CS | 17 | 30 | CS | 4 | 2 | CS | 14 | 28 | CS | 4 |
| NATURAL GAS | 1A4a | 020104 | g | GJ | 4 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 174 | CS | 2 | 1 | CS | 2 | 7 | CS | 10 |
| NATURAL GAS | 1A4a | 020105 | g | GJ | 573 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 193 | CS | 2 | 163 | CS | 2 | 169 | CS | 2 |
| NATURAL GAS | 1A4b | 0202 | g | GJ | 6 | CS | 14 | 1 | C | 1 | 0,3 | CS | 17 | 30 | CS | 4 | 4 | CS | 11 | 20 | CS | 11 |
| NATURAL GAS | 1A4b | 020202 | g | GJ | 15 | CS | 11 | 1 | C | 1 | 0,3 | CS | 17 | 30 | CS | 4 | 4 | CS | 11 | 20 | CS | 11 |
| NATURAL GAS | 1A4b | 020204 | g | GJ | 573 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 193 | CS | 2 | 163 | CS | 2 | 169 | CS | 2 |
| NATURAL GAS | 1A4c | 0203 | g | GJ | 6 | CS | 14 | 1 | C | 1 | 0,3 | CS | 17 | 30 | CS | 4 | 2 | CS | 14 | 28 | CS | 4 |
| NATURAL GAS | 1A4c | 020303 | g | GJ | 4 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 174 | CS | 2 | 1 | CS | 2 | 7 | CS | 10 |
| NATURAL GAS | 1A4c | 020304 | g | GJ | 573 | CS | 2 | 1 | C | 1 | 0,3 | CS | 17 | 193 | CS | 2 | 163 | CS | 2 | 169 | CS | 2 |
| OTHER LIQ. FUEL (Orimulsion) | 1A1a | 010101 | g | GJ | 3 | CS | 16 | 2 | C | 16 | 10 | CS | 16 | 88 | CS | 9 | 3 | CS | 16 | 15 | CS | 16 |
| PETROLEUM COKE | 1A2f | 0301 | g | GJ | 15 | C | 1 | 3 | C | 1 | 573 | CS | 24 | 50 | C | 1 | 1,5 | C | 1 | 61 | CS | 4 |
| PETROLEUM COKE | 1A4a | 0201 | g | GJ | 15 | C | 1 | 3 | C | 1 | 573 | CS | 24 | 50 | C | 1 | 1,5 | C | 1 | 1000 | C | 1 |
| PETROLEUM COKE | 1A4b | 0202 | g | GJ | 15 | C | 1 | 3 | C | 1 | 573 | CS | 24 | 50 | C | 1 | 1,5 | C | 1 | 1000 | C | 1 |
| PETROLEUM COKE | 1A4c | 0203 | g | GJ | 15 | C | 1 | 3 | C | 1 | 573 | CS | 24 | 50 | C | 1 | 1,5 | C | 1 | 1000 | C | 1 |
| REFINERY GAS | 1A1b | 010303 | g | GJ | 2 | C | 1 | 2 | C | 1 | 0,3 | CS | 23 | 30 | CS | 23 | 4 | C | 1 | 15 | C | 1 |
| REFINERY GAS | 1A1b | 010304 | g | GJ | 2 | C | 1 | 2 | C | 1 | 0,3 | CS | 23 | 174 | CS | 23 | 4 | C | 1 | 15 | C | 1 |
| RESIDUAL OIL | 1A1a | 0101 | g | GJ | 3 | C | 1 | 2 | C | 1 | 315 | CS | 20 | 240 | CS | 18 | 3 | C | 1 | 15 | CS | 3 |
| RESIDUAL OIL | 1A1a | 010101 | g | GJ | 3 | C | 1 | 2 | C | 1 | 315 | CS | 20 | 240 | CS | 18 | 3 | C | 1 | 15 | CS | 3 |
| RESIDUAL OIL | 1A1a | 010102 | g | GJ | 3 | C | 1 | 2 | C | 1 | 315 | CS | 20 | 240 | CS | 18 | 3 | C | 1 | 15 | CS | 3 |
| RESIDUAL OIL | 1A1a | 010103 | g | GJ | 3 | C | 1 | 2 | C | 1 | 315 | CS | 20 | 142 | CS | 4 | 3 | C | 1 | 15 | CS | 3 |
| RESIDUAL OIL | 1A1a | 010104 | g | GJ | 3 | C | 1 | 2 | C | 1 | 315 | CS | 20 | 142 | CS | 4 | 3 | C | 1 | 15 | C | 1 |
| RESIDUAL OIL | 1A1a | 010202 | g | GJ | 3 | C | 1 | 2 | C | 1 | 315 | CS | 20 | 240 | CS | 18 | 3 | C | 1 | 30 | C | 1 |
| RESIDUAL OIL | 1A1a | 010203 | g | GJ | 3 | C | 1 | 2 | C | 1 | 315 | CS | 20 | 142 | CS | 4 | 3 | C | 1 | 30 | C | 1 |
| RESIDUAL OIL | 1A1b | 010303 | g | GJ | 3 | C | 1 | 2 | C | 1 | 315 | CS | 20 | 142 | CS | 4 | 3 | C | 1 | 30 | C | 1 |
| RESIDUAL OIL | 1A2f | 0301 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 130 | CS | 28 | 3 | C | 1 | 30 | C | 1 |
| RESIDUAL OIL | 1A2f | 030102 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 130 | CS | 28 | 3 | C | 1 | 30 | C | 1 |
| RESIDUAL OIL | 1A2f | 030103 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 142 | CS | 4 | 3 | C | 1 | 30 | C | 1 |
| RESIDUAL OIL | 1A2f | 030104 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 142 | CS | 4 | 3 | C | 1 | 15 | C | 1 |
| RESIDUAL OIL | 1A2f | 030105 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 142 | CS | 4 | 3 | C | 1 | 100 | C | 1 |
| RESIDUAL OIL | 1A4a | 0201 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 130 | CS | 28 | 3 | C | 1 | 30 | C | 1 |
| RESIDUAL OIL | 1A4a | 020105 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 142 | CS | 4 | 3 | C | 1 | 100 | C | 1 |
| RESIDUAL OIL | 1A4b | 0202 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 130 | CS | 28 | 3 | C | 1 | 30 | C | 1 |
| RESIDUAL OIL | 1A4c | 0203 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 130 | CS | 28 | 3 | C | 1 | 30 | C | 1 |
| RESIDUAL OIL | 1A4c | 020302 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 130 | CS | 28 | 3 | C | 1 | 30 | C | 1 |
| RESIDUAL OIL | 1A4c | 020304 | g | GJ | 3 | C | 1 | 2 | C | 1 | 344 | CS | 25 | 142 | CS | 4 | 3 | C | 1 | 100 | C | 1 |
| STEAM COAL | 1A1a | 010101 | g | GJ | 1,5 | C | 1 | 3 | C | 1 | 39 | CS | 21 | 139 | CS | 21 | 1,5 | C | 1 | 10 | CS | 3 |
| STEAM COAL | 1A1a | 010102 | g | GJ | 1,5 | C | 1 | 3 | C | 1 | 39 | CS | 21 | 139 | CS | 21 | 1,5 | C | 1 | 10 | CS | 3 |
| STEAM COAL | 1A1a | 010103 | g | GJ | 15 | C | 1 | 3 | C | 1 | 464 | CS | 20 | 95 | CS | 4 | 1,5 | C | 1 | 10 | CS | 3 |
| STEAM COAL | 1A1a | 010202 | g | GJ | 15 | C | 1 | 3 | C | 1 | 464 | CS | 20 | 95 | CS | 4 | 15 | C | 1 | 10 | CS | 3 |
| STEAM COAL | 1A1a | 010203 | g | GJ | 15 | C | 1 | 3 | C | 1 | 464 | CS | 20 | 95 | CS | 4 | 15 | C | 1 | 10 | CS | 3 |
| STEAM COAL | 1A2f | 0301 | g | GJ | 15 | C | 1 | 3 | C | 1 | 464 | CS | 20 | 95 | CS | 4 | 15 | C | 1 | 10 | C | 1 |
| STEAM COAL | 1A4b | 0202 | g | GJ | 15 | C | 1 | 3 | C | 1 | 464 | CS | 20 | 95 | CS | 4 | 15 | C | 1 | 10 | C | 1 |
| STEAM COAL | 1A4c | 0203 | g | GJ | 15 | C | 1 | 3 | C | 1 | 464 | CS | 20 | 95 | CS | 4 | 15 | C | 1 | 10 | C | 1 |
| WOOD AND SIMIL. | 1A1a | 010102 | g | GJ | 32 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 48 | C | 1 | 50 | CS | 3 |
| WOOD AND SIMIL. | 1A1a | 010103 | g | GJ | 32 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 48 | C | 1 | 50 | CS | 3 |
| WOOD AND SIMIL. | 1A1a | 010105 | g | GJ | 32 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 48 | C | 1 | 50 | CS | 3 |
| WOOD AND SIMIL. | 1A1a | 010202 | g | GJ | 32 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 48 | C | 1 | 240 | CS | 4 |
| WOOD AND SIMIL. | 1A1a | 010203 | g | GJ | 32 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 48 | C | 1 | 240 | CS | 4 |
| WOOD AND SIMIL. | 1A1a | 010205 | g | GJ | 32 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 48 | C | 1 | 240 | CS | 4 |
| WOOD AND SIMIL. | 1A2f | 0301 | g | GJ | 32 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 48 | C | 1 | 240 | CS | 4 |
| WOOD AND SIMIL. | 1A2f | 030102 | g | GJ | 32 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 48 | C | 1 | 240 | CS | 4 |
| WOOD AND SIMIL. | 1A2f | 030103 | g | GJ | 32 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 48 | C | 1 | 240 | CS | 4 |
| WOOD AND SIMIL. | 1A4a | 0201 | g | GJ | 200 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 600 | C | 1 | 240 | CS | 4 |
| WOOD AND SIMIL. | 1A4a | 020105 | g | GJ | 200 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 600 | C | 1 | 240 | CS | 4 |
| WOOD AND SIMIL. | 1A4b | 0202 | g | GJ | 200 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 600 | C | 1 | 9000 | CS | 12 13 |
| WOOD AND SIMIL. | 1A4c | 0203 | g | GJ | 200 | C | 1 | 4 | C | 1 | 25 | CS | 22 | 130 | CS | 22 | 600 | C | 1 | 240 | CS | 4 |

- Emission Inventory Guidebook 3rd edition, prepared by the UNECE/EMEP Task Force on Emissions Inventories and Projections

- Emissionsforhold for gasdrevne kraftvarmeanlæg <25MWe, Arbejdsrapport fra Miljøstyrelsen nr 17 1997 (In Danish)

- Elsam, Bo Sander, mail 17-05-2002

- Luftvejledningen, Begrænsning af luftforurening fra virksomheder, Vejledning fra Miljøstyrelsen Nr. 2 2001 (Danish legislation)

- Halm til energiformål, Teknik – Miljø – Økonomi, 2. udgave, 1998, Videncenter for halm og flisfyring (In Danish)

- Emissioner fra halm- og flisfyr, dk-Teknik 1990 (In Danish)

- Danish Technological Institute, personal comunication

- dk-Teknik, personal comunication

- Calculation based on annual environmental reports of Danish plants year 2000

- Technical note from Danish Gas Technology Centre, Eltra PSO 2002 (In Danish, not published)

- Energi- og miljøoversigt, Danish Gas Technology Centre  2000 (In Danish)

- Miljøprojekt 149/1990 Emissionsundersøgelse for pejse og brændeovne, Miljøstyrelsen (In Danish)

- Miljøprojekt 249/1994 Emissioner af dioxiner fra pejse og brændeovne, Miljøstyrelsen (In Danish)

- Naturgas – Energi og miljø, Danish Gas Technology Centre, 2000 (In Danish)

- Same emission factors as agricultural waste (straw) is assumed

- Same emission factors as residual oil assumed

- Calculation based on S content of natural gas 6mg(S)/mn3 gas. S content from the Danish natural gas transmission company DONG

- Indberetning af SO2 og NOx emissioner, Eltra og Elkraft System  (Reportingof SO2 and NOx emissions from power plants >25MWe due to Danish legislation)

- Fyring med biomassebaserede restprodukter, Miljøprojekt nr. 358 1997, Miljøstyrelsen

- Indberetning af SO2 og NOx emissioner, Eltra og Elkraft System, antaget ingen røggasrensning for arealkilder (Reportingof SO2 and NOx emissions from power plants >25MWe due to Danish legislation)

- Average of large point sources 2000

- Træ til energiformål, Teknik – Miljø – Økonomi, 2. udgave, 1999, Videncenter for halm og flisfyring (In Danish)

- Same emission factor as for natural gas assumed

- Bekendtgørelseom begrænsning af svovlindholdet i visse flydende og faste brændstoffer, Bekendtgørelse 698 af 22/09/1998 (Danish legislation)

- Assumed 0,7% S. Product data from Shell and Statoil

- S content stated by plant owners (~200 ppm H2S)

- Assumed 0,05% S. Bilag 750, Kom 97/0105 and product sheets from Q8, Shell and Statoil

- Bekendtgørelse om begrænsning af emissioner af svovldioxid, kvælstofoxider og støv fra store fyringsanlæg, Bekendtgørelse 689 af 15/10/1990 (Danish legislation)

- –

- Product sheet from Shell

Table 7 Emission factors of CH4 [g/GJ] Time series of area sources

| fuel_gr_abbr | snap_id | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STEAM COAL | 010101 and 010102 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 |
| STEAM COAL | Other | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| PETROLEUM COKE | all | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| WOOD AND SIMIL. | 01 and 03 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| WOOD AND SIMIL. | 02 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 |
| MUNICIP. WASTES | all | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| AGRICUL. WASTES | 01 and 03 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| AGRICUL. WASTES | 02 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 |
| RESIDUAL OIL | all | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| GAS OIL | all | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 |
| KEROSENE | all | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| BLACK LIQUOR | 010203 |  |  |  |  |  |  |  |  |  |  |  | 32 |
| BLACK LIQUOR | 020304 |  |  |  |  |  |  |  |  |  |  |  | 200 |
| OTHER LIQ. FUEL (Orimulsion) | 0101 |  |  |  |  |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| NATURAL GAS | Engines: 010105, 010405, 010505, 020105, 020204, 020304, 030105 | 573 | 573 | 573 | 573 | 573 | 573 | 573 | 573 | 573 | 573 | 573 | 573 |
| NATURAL GAS | Gas turbines: 010104, 010304, 010504, 020104, 020303, 030104 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| NATURAL GAS | 0101, 010101, 010102, 0102, 0103, 010302, 010502, 0201, 0202, 0203, 020302, 0301 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| NATURAL GAS | 010103, 010203, 010406, 020103, 020202, 030103, 030106 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| NATURAL GAS | 010202 |  |  |  |  |  |  |  |  |  | 15 | 6 | 6 |
| LPG | all | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| REFINERY GAS | all | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| BIOGAS | Engines: 010105, 010405, 010505, 020105, 020304, 030105 |  |  |  |  |  |  |  |  | 434 | 434 | 434 | 434 |
| BIOGAS | Other | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |

Table 8 Emission factors of N2O [g/GJ] Time series of area sources

| fuel_gr_abbr | snap_id | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STEAM COAL | all | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| PETROLEUM COKE | all | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| WOOD AND SIMIL. | all | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MUNICIP. WASTES | all | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| RESIDUAL OIL | all | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| GAS OIL | all | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| KEROSENE | all | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| BLACK LIQUOR | all |  |  |  |  |  |  |  |  |  |  |  | 4 |
| OTHER LIQ. FUEL (Orimulsion) | 0101 |  |  |  |  |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| NATURAL GAS | all | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| LPG | all | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 or 2 | 1 or 2 |
| REFINERY GAS | all | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| BIOGAS | all | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |

Table 9 Emission factors of NMVOC [g/GJ] Time series of area sources

| fuel_gr_abbr | snap_id | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STEAM COAL | 0101 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 |
| STEAM COAL | Other | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| PETROLEUM COKE | all | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 |
| WOOD AND SIMIL. | 01 and 03 | 48 | 48 | 48 | 48 | 48 | 48 | 48 | 48 | 48 | 48 | 48 | 48 |
| WOOD AND SIMIL. | 0201, 0202 and 0203 | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 600 |
| MUNICIP. WASTES | all | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 |
| AGRICUL. WASTES | 01 and 03 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| AGRICUL. WASTES | 02 | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 600 | 600 |
| RESIDUAL OIL | all | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| GAS OIL | 010105, 010205, 020105, 020304, 030105 |  |  |  |  |  |  |  |  | 100 | 100 | 100 | 100 |
| GAS OIL | 010104, 030104 |  |  |  |  |  |  |  |  | 2 | 2 | 2 | 2 |
| GAS OIL | 01 and 03 Other | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 |
| GAS OIL | 02 other | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| KEROSENE | all | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| BLACK LIQUOR | 010203 |  |  |  |  |  |  |  |  |  |  |  | 50 |
| BLACK LIQUOR | 020304 |  |  |  |  |  |  |  |  |  |  |  | 600 |
| OTHER LIQ. FUEL (Orimulsion) | 0101 |  |  |  |  |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| NATURAL GAS | Engines: 010105, 010405, 010505, 020105, 020204, 020304, 030105 | 163 | 163 | 163 | 163 | 163 | 163 | 163 | 163 | 163 | 163 | 163 | 163 |
| NATURAL GAS | Gas turbines: 010104, 010304, 010504, 020104, 020303, 030104 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| NATURAL GAS | 0202 (-02) | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| NATURAL GAS | Other | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| LPG | all | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| REFINERY GAS | all | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| BIOGAS | all | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |

Table 10 Emission factors of CO [g/GJ] Time series of area sources

| fuel_gr_abbr | snap_id | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STEAM COAL | all | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| PETROLEUM COKE | 010102 |  |  |  | 61 |  |  |  |  |  |  |  |  |
| PETROLEUM COKE | 02 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 |
| PETROLEUM COKE | 03 | 61 | 61 | 61 | 61 | 61 | 61 | 61 | 61 | 61 | 61 | 61 | 61 |
| WOOD AND SIMIL. | 0101 |  |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| WOOD AND SIMIL. | 0102 and 0201 | 400 | 373 | 347 | 320 | 293 | 267 | 240 | 240 | 240 | 240 | 240 | 240 |
| WOOD AND SIMIL. | 0202 | 9000 | 9000 | 9000 | 9000 | 9000 | 9000 | 9000 | 9000 | 9000 | 9000 | 9000 | 9000 |
| WOOD AND SIMIL. | 0203 and 0301 | 400 | 373 | 347 | 320 | 293 | 267 | 240 | 240 | 240 | 240 | 240 | 240 |
| MUNICIP. WASTES | all | 100 | 85 | 70 | 55 | 40 | 25 | 10 | 10 | 10 | 10 | 10 | 10 |
| AGRICUL. WASTES | 0101 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| AGRICUL. WASTES | 0102 and 0201 | 600 | 554 | 508 | 463 | 417 | 371 | 325 | 325 | 325 | 325 | 325 | 325 |
| AGRICUL. WASTES | 0202 | 8500 | 8500 | 8500 | 8500 | 8500 | 7500 | 6500 | 5500 | 4500 | 4000 | 4000 | 4000 |
| AGRICUL. WASTES | 0203 and 0301 | 600 | 554 | 508 | 463 | 417 | 371 | 325 | 325 | 325 | 325 | 325 | 325 |
| RESIDUAL OIL | 0101 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| RESIDUAL OIL | 0102, 0103, 02 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| RESIDUAL OIL | 0301 (-02 and -03) | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| RESIDUAL OIL | 030104 |  |  |  |  |  |  |  |  |  | 15 | 15 | 15 |
| RESIDUAL OIL | 030105 |  |  |  |  |  |  |  |  |  |  |  | 100 |
| GAS OIL | 0101 (not –05) | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| GAS OIL | 010105 |  |  |  |  |  |  |  |  | 100 | 100 | 100 | 100 |
| GAS OIL | 0102 (not –05) and 0103 and 0104 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| GAS OIL | 010205 |  |  |  |  |  |  |  |  |  |  | 100 | 100 |
| GAS OIL | 0201 (-03) | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| GAS OIL | 020105 |  |  |  |  |  |  |  |  |  |  | 100 | 100 |
| GAS OIL | 0202 (-02) | 43 | 43 | 43 | 43 | 43 | 43 | 43 | 43 | 43 | 43 | 43 | 43 |
| GAS OIL | 0203 (-02) | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |  |  |  |  |
| GAS OIL | 020304 |  |  |  |  |  |  |  |  |  |  | 100 | 100 |
| GAS OIL | 0301 (-02-, 03) | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| GAS OIL | 030104 |  |  |  |  |  |  |  |  |  |  | 15 | 15 |
| GAS OIL | 030105 |  |  |  |  |  |  |  |  |  |  | 100 | 100 |
| GAS OIL | 030106 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| KEROSENE | all | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| BLACK LIQUOR | all |  |  |  |  |  |  |  |  |  |  |  | 325 |
| OTHER LIQ. FUEL (Orimulsion) | 0101 |  |  |  |  |  | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| NATURAL GAS | 0101 (-01, -02, -03) | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| NATURAL GAS | 010104, 010304, 010504, 020104, 020303, 030104 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| NATURAL GAS | 010105, 010405, 010505, 020105, 020204, 020304, 030105 | 212 | 212 | 212 | 212 | 212 | 212 | 203 | 195 | 186 | 178 | 169 | 169 |
| NATURAL GAS | 0202 (-02) | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| NATURAL GAS | Other | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 | 28 |
| LPG | all | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| REFINERY GAS | all | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| BIOGAS | 010105, 010405, 010505, 020105, 020304, 030105 |  |  |  |  |  |  |  |  | 255 | 255 | 255 | 255 |
| BIOGAS | Other | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 |

### Appendix 4.2 Large point sources

Table 11 Large point sources, fuel consumption in 2001 (1A1, 1A2 and 1A4)

| lps_id | lps_name | part_id | snap_id | fuel | fuel cons. [GJ] | ipcc_id |
| --- | --- | --- | --- | --- | --- | --- |
| 001 | Amagervaerket | 01 | 010101 | STEAM COAL | 2785006 | 1A1a |
| 001 | Amagervaerket | 01 | 010101 | RESIDUAL OIL | 124657 | 1A1a |
| 001 | Amagervaerket | 02 | 010101 | STEAM COAL | 2765850 | 1A1a |
| 001 | Amagervaerket | 02 | 010101 | RESIDUAL OIL | 148840 | 1A1a |
| 001 | Amagervaerket | 03 | 010101 | STEAM COAL | 14334732 | 1A1a |
| 001 | Amagervaerket | 03 | 010101 | RESIDUAL OIL | 126698 | 1A1a |
| 002 | Svanemoellevaerket | 05 | 010101 | NATURAL GAS | 1882525 | 1A1a |
| 002 | Svanemoellevaerket | 07 | 010104 | NATURAL GAS | 4344799 | 1A1a |
| 003 | H.C.Oerstedsvaerket | 03 | 010101 | RESIDUAL OIL | 1177110 | 1A1a |
| 003 | H.C.Oerstedsvaerket | 03 | 010101 | NATURAL GAS | 1688830 | 1A1a |
| 003 | H.C.Oerstedsvaerket | 07 | 010101 | RESIDUAL OIL | 1931452 | 1A1a |
| 003 | H.C.Oerstedsvaerket | 07 | 010101 | NATURAL GAS | 2327121 | 1A1a |
| 004 | Kyndbyvaerket | 22 | 010101 | RESIDUAL OIL | 103088 | 1A1a |
| 004 | Kyndbyvaerket | 26 | 010101 | RESIDUAL OIL | 187734 | 1A1a |
| 004 | Kyndbyvaerket | 28 | 010101 | RESIDUAL OIL | 89515 | 1A1a |
| 004 | Kyndbyvaerket | 41 | 010105 | GAS OIL | 1009 | 1A1a |
| 004 | Kyndbyvaerket | 51 | 010104 | GAS OIL | 6186 | 1A1a |
| 004 | Kyndbyvaerket | 52 | 010104 | GAS OIL | 7801 | 1A1a |
| 005 | Masnedoevaerket | 12 | 010102 | WOOD AND SIMIL. | 95934 | 1A1a |
| 005 | Masnedoevaerket | 12 | 010102 | AGRICUL. WASTES | 484060 | 1A1a |
| 005 | Masnedoevaerket | 12 | 010102 | GAS OIL | 1108 | 1A1a |
| 005 | Masnedoevaerket | 31 | 010104 | GAS OIL | 5584 | 1A1a |
| 007 | Stigsnaesvaerket | 01 | 010101 | STEAM COAL | 330455 | 1A1a |
| 007 | Stigsnaesvaerket | 01 | 010101 | RESIDUAL OIL | 67899 | 1A1a |
| 007 | Stigsnaesvaerket | 02 | 010101 | STEAM COAL | 5558569 | 1A1a |
| 007 | Stigsnaesvaerket | 02 | 010101 | RESIDUAL OIL | 250782 | 1A1a |
| 008 | Asnaesvaerket | 01 | 010101 | RESIDUAL OIL | 144268 | 1A1a |
| 008 | Asnaesvaerket | 03 | 010101 | STEAM COAL | 2173576 | 1A1a |
| 008 | Asnaesvaerket | 03 | 010101 | RESIDUAL OIL | 66746 | 1A1a |
| 008 | Asnaesvaerket | 04 | 010101 | STEAM COAL | 7479179 | 1A1a |
| 008 | Asnaesvaerket | 04 | 010101 | RESIDUAL OIL | 82560 | 1A1a |
| 008 | Asnaesvaerket | 05 | 010101 | RESIDUAL OIL | 266167 | 1A1a |
| 008 | Asnaesvaerket | 05 | 010101 | OTHER LIQ. FUEL | 30243677 | 1A1a |
| 009 | Statoil Raffinaderi | 01 | 010306 | RESIDUAL OIL | 765537 | 1A1b |
| 009 | Statoil Raffinaderi | 01 | 010306 | REFINERY GAS | 6909888 | 1A1b |
| 010 | Avedoerevaerket | 01 | 010101 | STEAM COAL | 17199365 | 1A1a |
| 010 | Avedoerevaerket | 01 | 010101 | RESIDUAL OIL | 63657 | 1A1a |
| 010 | Avedoerevaerket | 01 | 010101 | GAS OIL | 40018 | 1A1a |
| 010 | Avedoerevaerket | 02 | 010104 | AGRICUL. WASTES | 101730 | 1A1a |
| 010 | Avedoerevaerket | 02 | 010104 | RESIDUAL OIL | 1715776 | 1A1a |
| 010 | Avedoerevaerket | 02 | 010104 | NATURAL GAS | 1279897 | 1A1a |
| 011 | Fynsvaerket | 03 | 010101 | STEAM COAL | 1684670 | 1A1a |
| 011 | Fynsvaerket | 03 | 010101 | WOOD AND SIMIL. | 920 | 1A1a |
| 011 | Fynsvaerket | 03 | 010101 | MUNICIP. WASTES | 185500 | 1A1a |
| 011 | Fynsvaerket | 03 | 010101 | AGRICUL. WASTES | 10070 | 1A1a |
| 011 | Fynsvaerket | 03 | 010101 | RESIDUAL OIL | 98330 | 1A1a |
| 011 | Fynsvaerket | 03 | 010101 | NATURAL GAS | 7666490 | 1A1a |
| 011 | Fynsvaerket | 07 | 010101 | STEAM COAL | 10731460 | 1A1a |
| 011 | Fynsvaerket | 07 | 010101 | RESIDUAL OIL | 88460 | 1A1a |
| 011 | Fynsvaerket | 08 | 010101 | MUNICIP. WASTES | 2623520 | 1A1a |
| 011 | Fynsvaerket | 08 | 010101 | GAS OIL | 42420 | 1A1a |
| 012 | Studstrupvaerket | 03 | 010101 | STEAM COAL | 15240450 | 1A1a |
| 012 | Studstrupvaerket | 03 | 010101 | RESIDUAL OIL | 132130 | 1A1a |
| 012 | Studstrupvaerket | 04 | 010101 | STEAM COAL | 11462760 | 1A1a |
| 012 | Studstrupvaerket | 04 | 010101 | AGRICUL. WASTES | 28160 | 1A1a |
| 012 | Studstrupvaerket | 04 | 010101 | RESIDUAL OIL | 136920 | 1A1a |
| 014 | Vendsysselvaerket | 03 | 010101 | STEAM COAL | 19831710 | 1A1a |
| 014 | Vendsysselvaerket | 03 | 010101 | RESIDUAL OIL | 206640 | 1A1a |
| 014 | Vendsysselvaerket | 03 | 010101 | GAS OIL | 15280 | 1A1a |
| 017 | Shell Raffinaderi | 01 | 010306 | RESIDUAL OIL | 677392 | 1A1b |
| 017 | Shell Raffinaderi | 01 | 010306 | REFINERY GAS | 4866618 | 1A1b |
| 017 | Shell Raffinaderi | 05 | 010304 | REFINERY GAS | 2457089 | 1A1b |
| 018 | Skaerbaekvaerket | 01 | 010101 | RESIDUAL OIL | 85000 | 1A1a |
| 018 | Skaerbaekvaerket | 03 | 010101 | GAS OIL | 25000 | 1A1a |
| 018 | Skaerbaekvaerket | 03 | 010101 | NATURAL GAS | 6950000 | 1A1a |
| 019 | Enstedvaerket | 03 | 010101 | STEAM COAL | 27937970 | 1A1a |
| 019 | Enstedvaerket | 03 | 010101 | RESIDUAL OIL | 153050 | 1A1a |
| 019 | Enstedvaerket | 04 | 010101 | AGRICUL. WASTES | 1549480 | 1A1a |
| 019 | Enstedvaerket | 04 | 010101 | RESIDUAL OIL | 15520 | 1A1a |
| 020 | Esbjergvaerket | 03 | 010101 | STEAM COAL | 19474710 | 1A1a |
| 020 | Esbjergvaerket | 03 | 010101 | RESIDUAL OIL | 74420 | 1A1a |
| 022 | Oestkraft | 05 | 010102 | RESIDUAL OIL | 17990 | 1A1a |
| 022 | Oestkraft | 06 | 010102 | STEAM COAL | 723407 | 1A1a |
| 022 | Oestkraft | 06 | 010102 | WOOD AND SIMIL. | 31020 | 1A1a |
| 022 | Oestkraft | 06 | 010102 | RESIDUAL OIL | 29102 | 1A1a |
| 023 | Danisco Ingredients | 01 | 030102 | STEAM COAL | 552101 | 1A2f |
| 023 | Danisco Ingredients | 01 | 030102 | NATURAL GAS | 10500 | 1A2f |
| 024 | Dansk Naturgas Behandlingsanlaeg | 01 | 010502 | NATURAL GAS | 352650,31 | 1A1c |
| 025 | Horsens Kraftvarmevaerk | 01 | 010102 | MUNICIP. WASTES | 875920 | 1A1a |
| 025 | Horsens Kraftvarmevaerk | 02 | 010104 | NATURAL GAS | 878260 | 1A1a |
| 026 | Herningvaerket | 01 | 010102 | RESIDUAL OIL | 21650 | 1A1a |
| 026 | Herningvaerket | 01 | 010102 | NATURAL GAS | 3806740 | 1A1a |
| 027 | Vestforbraendingen | 01 | 010102 | MUNICIP. WASTES | 2086623 | 1A1a |
| 027 | Vestforbraendingen | 01 | 010102 | GAS OIL | 17862 | 1A1a |
| 027 | Vestforbraendingen | 02 | 010102 | MUNICIP. WASTES | 3007981 | 1A1a |
| 028 | Amagerforbraendingen | 01 | 010102 | MUNICIP. WASTES | 3019970 | 1A1a |
| 029 | Randersvaerket | 01 | 010102 | STEAM COAL | 3229268 | 1A1a |
| 029 | Randersvaerket | 01 | 010102 | BIOGAS | 23338 | 1A1a |
| 029 | Randersvaerket | 02 | 010102 | GAS OIL | 46594 | 1A1a |
| 030 | Grenaavaerket | 01 | 010102 | STEAM COAL | 1017827 | 1A1a |
| 030 | Grenaavaerket | 01 | 010102 | WOOD AND SIMIL. | 217337 | 1A1a |
| 030 | Grenaavaerket | 01 | 010102 | MUNICIP. WASTES | 130991 | 1A1a |
| 030 | Grenaavaerket | 01 | 010102 | AGRICUL. WASTES | 853970 | 1A1a |
| 030 | Grenaavaerket | 01 | 010102 | RESIDUAL OIL | 77083 | 1A1a |
| 030 | Grenaavaerket | 01 | 010102 | GAS OIL | 7470 | 1A1a |
| 031 | Hilleroedvaerket | 01 | 010104 | NATURAL GAS | 3190810 | 1A1a |
| 032 | Helsingoervaerket | 01 | 010104 | NATURAL GAS | 2066843 | 1A1a |
| 033 | Staalvalsevaerket | 01 | 030102 | NATURAL GAS | 1803744 | 1A2f |
| 034 | Stora Dalum | 01 | 030102 | NATURAL GAS | 1054807,5 | 1A2f |
| 035 | Assens Sukkerfabrik | 01 | 030102 | STEAM COAL | 445279,5 | 1A2f |
| 035 | Assens Sukkerfabrik | 01 | 030102 | RESIDUAL OIL | 306379,05 | 1A2f |
| 035 | Assens Sukkerfabrik | 01 | 030102 | BIOGAS | 17020 | 1A2f |
| 036 | Kolding Kraftvarmevaerk | 01 | 010103 | MUNICIP. WASTES | 762059 | 1A1a |
| 036 | Kolding Kraftvarmevaerk | 02 | 010103 | MUNICIP. WASTES | 285185 | 1A1a |
| 037 | Maabjergvaerket | 02 | 010102 | WOOD AND SIMIL. | 432000 | 1A1a |
| 037 | Maabjergvaerket | 02 | 010102 | MUNICIP. WASTES | 1720000 | 1A1a |
| 037 | Maabjergvaerket | 02 | 010102 | AGRICUL. WASTES | 408000 | 1A1a |
| 037 | Maabjergvaerket | 02 | 010102 | NATURAL GAS | 218000 | 1A1a |
| 038 | Soenderborg Kraftvarmevaerk | 01 | 010102 | MUNICIP. WASTES | 461825 | 1A1a |
| 038 | Soenderborg Kraftvarmevaerk | 02 | 010104 | NATURAL GAS | 910366 | 1A1a |
| 039 | Kara Affaldsforbraendingsanlaeg | 01 | 010102 | MUNICIP. WASTES | 1962022 | 1A1a |
| 039 | Kara Affaldsforbraendingsanlaeg | 01 | 010102 | NATURAL GAS | 16851 | 1A1a |
| 040 | Viborg Kraftvarmevaerk | 01 | 010104 | NATURAL GAS | 2398896,9 | 1A1a |
| 042 | Nordforbraendingen | 01 | 010102 | MUNICIP. WASTES | 1018068 | 1A1a |
| 045 | Aalborg Portland | 01 | 030311 | STEAM COAL | 4718457,5 | 1A2f |
| 045 | Aalborg Portland | 01 | 030311 | PETROLEUM COKE | 7656733 | 1A2f |
| 045 | Aalborg Portland | 01 | 030311 | MUNICIP. WASTES | 795492 | 1A2f |
| 045 | Aalborg Portland | 01 | 030311 | RESIDUAL OIL | 784 | 1A2f |
| 046 | Aarhus Nord | 01 | 010102 | MUNICIP. WASTES | 1211385 | 1A1a |
| 046 | Aarhus Nord | 02 | 010102 | MUNICIP. WASTES | 648292 | 1A1a |
| 047 | Reno Nord | 01 | 010103 | MUNICIP. WASTES | 1487290 | 1A1a |
| 048 | Silkeborg Kraftvarmevaerk | 01 | 010104 | NATURAL GAS | 3571656 | 1A1a |
| 049 | Rensningsanlægget Lynetten | 01 | 020103 | MUNICIP. WASTES | 12669 | 1A4a |
| 049 | Rensningsanlægget Lynetten | 01 | 020103 | GAS OIL | 44010 | 1A4a |
| 049 | Rensningsanlægget Lynetten | 01 | 020103 | BIOGAS | 84512 | 1A4a |
| 050 | I/S Fasan | 01 | 010203 | MUNICIP. WASTES | 754394 | 1A1a |
| 051 | AVV Forbrændingsanlæg | 01 | 010103 | MUNICIP. WASTES | 632966 | 1A1a |
| 052 | I/S REFA Kraftvarmeværk | 01 | 010103 | MUNICIP. WASTES | 1040783 | 1A1a |
| 053 | Svendborg Kraftvarmeværk | 01 | 010102 | MUNICIP. WASTES | 470264 | 1A1a |
| 053 | Svendborg Kraftvarmeværk | 01 | 010102 | NATURAL GAS | 6727 | 1A1a |
| 054 | Kommunekemi | 02 | 010102 | MUNICIP. WASTES | 651702 | 1A1a |
| 054 | Kommunekemi | 02 | 010102 | RESIDUAL OIL | 40171 | 1A1a |
| 054 | Kommunekemi | 02 | 010102 | GAS OIL | 7604 | 1A1a |
| 054 | Kommunekemi | 03 | 010102 | MUNICIP. WASTES | 637250 | 1A1a |
| 054 | Kommunekemi | 03 | 010102 | RESIDUAL OIL | 49654 | 1A1a |
| 054 | Kommunekemi | 03 | 010102 | GAS OIL | 9110 | 1A1a |
| 054 | Kommunekemi | 04 | 010104 | NATURAL GAS | 1000 | 1A1a |
| 055 | I/S Fælles Forbrænding | 01 | 010203 | MUNICIP. WASTES | 235484 | 1A1a |
| 056 | Vestfyns Forbrænding | 01 | 010203 | MUNICIP. WASTES | 242970 | 1A1a |
| 058 | I/S Reno Syd | 01 | 010103 | MUNICIP. WASTES | 614638 | 1A1a |
| 059 | I/S Kraftvarmeværk Thisted | 01 | 010103 | MUNICIP. WASTES | 562958 | 1A1a |
| 059 | I/S Kraftvarmeværk Thisted | 01 | 010103 | AGRICUL. WASTES | 943 | 1A1a |
| 060 | Knudmoseværket | 01 | 010103 | MUNICIP. WASTES | 341618 | 1A1a |
| 060 | Knudmoseværket | 01 | 010103 | NATURAL GAS | 27263 | 1A1a |
| 061 | Kavo I/S Energien | 01 | 010103 | MUNICIP. WASTES | 699867 | 1A1a |
| 062 | VEGA | 01 | 010203 | MUNICIP. WASTES | 574350 | 1A1a |
| 063 | Hadsund Bys Fjernvarmeværk | 01 | 010203 | WOOD AND SIMIL. | 30286 | 1A1a |
| 063 | Hadsund Bys Fjernvarmeværk | 01 | 010203 | MUNICIP. WASTES | 198468 | 1A1a |
| 064 | Aars Fjernvarmeforsyning | 01 | 010103 | WOOD AND SIMIL. | 6410 | 1A1a |
| 064 | Aars Fjernvarmeforsyning | 01 | 010103 | MUNICIP. WASTES | 520526 | 1A1a |
| 065 | Haderslev Kraftvarmeværk | 01 | 010103 | MUNICIP. WASTES | 615093 | 1A1a |
| 065 | Haderslev Kraftvarmeværk | 01 | 010103 | NATURAL GAS | 53 | 1A1a |
| 066 | Frederiskhavn Affaldskraftvarmeværk | 01 | 010103 | MUNICIP. WASTES | 370380 | 1A1a |
| 066 | Frederiskhavn Affaldskraftvarmeværk | 01 | 010103 | GAS OIL | 1260 | 1A1a |
| 067 | Vejen Kraftvarmeværk | 01 | 010103 | MUNICIP. WASTES | 409800 | 1A1a |
| 068 | Bofa I/S | 01 | 010203 | MUNICIP. WASTES | 189372 | 1A1a |
| 068 | Bofa I/S | 01 | 010203 | RESIDUAL OIL | 561 | 1A1a |
| 069 | DTU | 01 | 010104 | NATURAL GAS | 1325022 | 1A1a |
| 070 | Næstved Kraftvarmeværk | 01 | 010104 | NATURAL GAS | 446555 | 1A1a |
| 071 | Maricogen | 01 | 030104 | NATURAL GAS | 2249317 | 1A2f |
| 072 | Hjørring KVV | 01 | 010104 | NATURAL GAS | 1473666 | 1A1a |

Table 12 Large point sources, plant specific emissions (CRF 1A1, 1A2 and 1A4)

| lps_id |  | lps_name | part_id | snap_id | ipcc_id | SO2 [Mg] | NOx [Mg] | NMVOC [Mg] | CO [Mg] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 001 | Amagervaerket |  | 01 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 02 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 03 | 010101 | 1A1a | x | x |  |  |
| 002 | Svanemoellevaerket |  | 05 | 010101 | 1A1a |  | x |  |  |
|  |  |  | 07 | 010104 | 1A1a |  | x |  |  |
| 003 | H.C.Oerstedsvaerket |  | 03 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 05 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 07 | 010101 | 1A1a | x | x |  |  |
| 004 | Kyndbyvaerket |  | 21 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 22 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 26 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 28 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 41 | 010105 | 1A1a | x | x |  |  |
|  |  |  | 51 | 010104 | 1A1a | x | x |  |  |
|  |  |  | 52 | 010104 | 1A1a | x | x |  |  |
| 005 | Masnedoevaerket |  | 12 | 010102 | 1A1a | x | x |  |  |
|  |  |  | 31 | 010104 | 1A1a | x | x |  |  |
| 007 | Stigsnaesvaerket |  | 01 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 02 | 010101 | 1A1a | x | x |  |  |
| 008 | Asnaesvaerket |  | 02 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 03 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 04 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 05 | 010101 | 1A1a | x | x |  |  |
| 009 | Statoil Raffinaderi |  | 01 | 010306 | 1A1b | x |  |  |  |
| 010 | Avedoerevaerket |  | 01 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 02 | 010104 | 1A1a | x | x |  |  |
| 011 | Fynsvaerket |  | 03 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 07 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 08 | 010101 | 1A1a | x | x |  | x |
| 012 | Studstrupvaerket |  | 03 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 04 | 010101 | 1A1a | x | x |  |  |
| 014 | Vendsysselvaerket |  | 03 | 010101 | 1A1a | x | x |  |  |
| 017 | Shell Raffinaderi |  | 01 | 010306 | 1A1b | x | x |  |  |
|  |  |  | 05 | 010304 | 1A1b |  | x |  |  |
| 018 | Skaerbaekvaerket |  | 01 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 03 | 010101 | 1A1a | x | x |  |  |
| 019 | Enstedvaerket |  | 03 | 010101 | 1A1a | x | x |  |  |
|  |  |  | 04 | 010101 | 1A1a | x | x |  |  |
| 020 | Esbjergvaerket |  | 03 | 010101 | 1A1a | x | x |  |  |
| 022 | Oestkraft |  | 06 | 010102 | 1A1a | x | x |  |  |
| 024 | Dansk Naturgas Behandlingsanlaeg |  | 01 | 010502 | 1A1c |  | x |  |  |
| 025 | Horsens Kraftvarme-vaerk |  | 01 | 010102 | 1A1a | x | x |  | x |
|  |  |  | 02 | 010104 | 1A1a |  | x |  |  |
| 026 | Herningvaerket |  | 01 | 010102 | 1A1a | x | x |  |  |
| 027 | Vestforbraendingen |  | 01 | 010102 | 1A1a | x | x |  | x |
|  |  |  | 02 | 010102 | 1A1a | x | x |  | x |
| 028 | Amagerforbraendingen |  | 01 | 010102 | 1A1a | x | x |  | x |
| 029 | Randersvaerket |  | 01 | 010102 | 1A1a | x | x |  |  |
| 030 | Grenaavaerket |  | 01 | 010102 | 1A1a | x | x |  |  |
| 031 | Hilleroedvaerket |  | 01 | 010104 | 1A1a |  | x |  |  |
| 032 | Helsingoervaerket |  | 01 | 010104 | 1A1a |  | x |  |  |
| 034 | Stora Dalum |  | 01 | 030102 | 1A2f |  | x |  |  |
| 035 | Assens Sukkerfabrik |  | 01 | 030102 | 1A2f | x |  |  |  |
| 036 | Kolding Kraftvarmevaerk |  | 01 | 010103 | 1A1a | x |  |  | x |
|  |  |  | 02 | 010103 | 1A1a | x |  |  | x |
| 037 | Maabjergvaerket |  | 02 | 010102 | 1A1a | x | x | x | x |
| 038 | Soenderborg Kraftvarmevaerk |  | 01 | 010102 | 1A1a | x | x |  | x |
|  |  |  | 02 | 010104 | 1A1a |  | x |  |  |
| 039 | Kara Affalds-forbraendingsanlaeg |  | 01 | 010102 | 1A1a | x |  |  | x |
| 040 | Viborg Kraftvarmevaerk |  | 01 | 010104 | 1A1a |  | x |  |  |
| 042 | Nordforbraendingen |  | 01 | 010102 | 1A1a | x |  |  | x |
| 045 | Aalborg Portland |  | 01 | 030311 | 1A2f | x | x |  | x |
| 046 | Aarhus Nord |  | 01 | 010102 | 1A1a | x |  |  |  |
|  |  |  | 02 | 010102 | 1A1a | x |  |  |  |
| 047 | Reno Nord |  | 01 | 010103 | 1A1a | x |  |  | x |
| 048 | Silkeborg Kraftvarmevaerk |  | 01 | 010104 | 1A1a |  | x |  |  |
| 049 | Rensningsanlægget Lynetten |  | 01 | 020103 | 1A4a | x |  |  |  |
| 050 | I/S Fasan |  | 01 | 010203 | 1A1a | x | x |  | x |
| 051 | AVV Forbrændingsanlæg |  | 01 | 010103 | 1A1a | x |  |  |  |
| 053 | Svendborg Kraftvarmeværk |  | 01 | 010102 | 1A1a | x | x |  | x |
| 054 | Kommunekemi |  | 02 | 010102 | 1A1a | x |  |  | x |
|  |  |  | 03 | 010102 | 1A1a | x |  |  | x |
| 055 | I/S Fælles Forbrænding |  | 01 | 010203 | 1A1a | x |  |  | x |
| 056 | Vestfyns Forbrænding |  | 01 | 010203 | 1A1a | x | x |  | x |
| 058 | I/S Reno Syd |  | 01 | 010103 | 1A1a | x |  | x | x |
| 059 | I/S Kraftvarmeværk Thisted |  | 01 | 010103 | 1A1a | x |  |  | x |
| 060 | Knudmoseværket |  | 01 | 010103 | 1A1a | x |  |  | x |
| 061 | Kavo I/S Energien |  | 01 | 010103 | 1A1a | x |  | x | x |
| 062 | VEGA |  | 01 | 010203 | 1A1a | x | x |  | x |
| 063 | Hadsund Bys Fjernvarmeværk |  | 01 | 010203 | 1A1a | x |  |  | x |
| 064 | Aars Fjernvarmeforsyning |  | 01 | 010103 | 1A1a | x |  |  | x |
| 065 | Haderslev Kraftvarmeværk |  | 01 | 010103 | 1A1a | x | x |  | x |
| 066 | Frederiskhavn Affaldskraftvarmeværk |  | 01 | 010103 | 1A1a | x | x |  | x |
| 067 | Vejen Kraftvarmeværk |  | 01 | 010103 | 1A1a | x | x |  | x |
| 068 | Bofa I/S |  | 01 | 010203 | 1A1a | x |  |  | x |
| 069 | DTU |  | 01 | 010104 | 1A1a |  | x |  |  |
| 070 | Næstved Kraftvarmeværk |  | 01 | 010104 | 1A1a |  | x |  |  |
| 071 | Maricogen |  | 01 | 030104 | 1A2f |  | x |  |  |
| 072 | Hjørring KVV |  | 01 | 010104 | 1A1a |  | x |  |  |
|  | Total |  |  |  |  | 11674,87 | 44889,83 | 11,64 | 3397,99 |

### Appendix 4.3 Fuel rate

| fuel | fuel_gr_abbr | ipcc_id | rate_TJ |
| --- | --- | --- | --- |
| 102 | STEAM COAL | 1A1a | 163987 |
| 102 | STEAM COAL | 1A2f | 10181 |
| 102 | STEAM COAL | 1A4b | 49 |
| 102 | STEAM COAL | 1A4c | 1234 |
| 110 | PETROLEUM COKE | 1A2f | 7785 |
| 110 | PETROLEUM COKE | 1A4a | 12 |
| 110 | PETROLEUM COKE | 1A4b | 262 |
| 110 | PETROLEUM COKE | 1A4c | 3 |
| 111 | WOOD AND SIMIL. | 1A1a | 6135 |
| 111 | WOOD AND SIMIL. | 1A2f | 6150 |
| 111 | WOOD AND SIMIL. | 1A4a | 633 |
| 111 | WOOD AND SIMIL. | 1A4b | 14102 |
| 111 | WOOD AND SIMIL. | 1A4c | 198 |
| 114 | MUNICIP. WASTES | 1A1a | 32114 |
| 114 | MUNICIP. WASTES | 1A2f | 795 |
| 114 | MUNICIP. WASTES | 1A4a | 128 |
| 117 | AGRICUL. WASTES | 1A1a | 8856 |
| 117 | AGRICUL. WASTES | 1A2f | 0 |
| 117 | AGRICUL. WASTES | 1A4a | 0 |
| 117 | AGRICUL. WASTES | 1A4b | 2901 |
| 117 | AGRICUL. WASTES | 1A4c | 1940 |
| 118 | SEWAGE SLUDGE | 1A2f | 0 |
| 121 | OTHER SOL. FUEL | 1A1a | 32114 |
| 121 | OTHER SOL. FUEL | 1A2f | 795 |
| 121 | OTHER SOL. FUEL | 1A4a | 128 |
| 203 | RESIDUAL OIL | 1A1a | 8689 |
| 203 | RESIDUAL OIL | 1A1b | 1670 |
| 203 | RESIDUAL OIL | 1A2f | 7852 |
| 203 | RESIDUAL OIL | 1A4a | 203 |
| 203 | RESIDUAL OIL | 1A4b | 27 |
| 203 | RESIDUAL OIL | 1A4c | 1651 |
| 204 | GAS OIL | 1A1a | 1600 |
| 204 | GAS OIL | 1A2f | 3505 |
| 204 | GAS OIL | 1A4a | 4781 |
| 204 | GAS OIL | 1A4b | 31506 |
| 204 | GAS OIL | 1A4c | 8911 |
| 205 | DIESEL OIL | 1A2f | 9663 |
| 205 | DIESEL OIL | 1A4c | 16958 |
| 206 | KEROSENE | 1A2f | 26 |
| 206 | KEROSENE | 1A4a | 80 |
| 206 | KEROSENE | 1A4b | 159 |
| 206 | KEROSENE | 1A4c | 24 |
| 208 | MOTOR GASOLINE | 1A2f | 140 |
| 208 | MOTOR GASOLINE | 1A4b | 1164 |
| 208 | MOTOR GASOLINE | 1A4c | 572 |
| 215 | BLACK LIQUOR | 1A1a | 191 |
| 215 | BLACK LIQUOR | 1A4c | 1 |
| 225 | OTHER LIQ. FUEL | 1A1a | 30244 |
| 301 | NATURAL GAS | 1A1a | 80675 |
| 301 | NATURAL GAS | 1A1b | 0 |
| 301 | NATURAL GAS | 1A1c | 24628 |
| 301 | NATURAL GAS | 1A2f | 42910 |
| 301 | NATURAL GAS | 1A4a | 7578 |
| 301 | NATURAL GAS | 1A4b | 31077 |
| 301 | NATURAL GAS | 1A4c | 6582 |
| 303 | LPG | 1A1a | 0 |
| 303 | LPG | 1A2f | 2390 |
| 303 | LPG | 1A4a | 119 |
| 303 | LPG | 1A4b | 722 |
| 303 | LPG | 1A4c | 19 |
| 308 | REFINERY GAS | 1A1b | 14234 |
| 309 | BIOGAS | 1A1a | 1748 |
| 309 | BIOGAS | 1A1c | 29 |
| 309 | BIOGAS | 1A2f | 110 |
| 309 | BIOGAS | 1A4a | 982 |
| 309 | BIOGAS | 1A4c | 178 |

### Appendix 4.4 CO2 emission factors for municipal waste incineration

11 April 2000

Jytte Boll Illerup

The National Environmental Research Institute

Denmark

CO2 emission from plastic in municipal waste

In the Danish air emission inventory the emission of CO2 from plastic in municipal waste is included in the total CO2 emission.

The Danish Environmental Agency has estimated the content of plastic in municipality waste (Cplast) to be 6.4 w/w%. The energy content in plastic (Eplast) from one ton waste can be calculated to be:

(1) Eplast(t) = Cplast x Hplast(t)

where Hplast(t)  is the lower heating value of plastic. It is assumed that Hplast equals the lower heating value of crude oil.

The emission of CO2 from the plastic part of the waste per GJ plastic (EMF,p,p) is

(2) EMFp,p = (CC,plast x MCO2)/MC

where MCO2 is the mole weight for CO2 and MC is the mole weight for carbon.

The emission of CO2 from the plastic part of the waste per GJ waste (EMFp,w) is

(3) EMFp,w(t)= EMFp,p x Eplast/Hwaste(t)

where Hwaste is the lower heating value of waste.

Equations (1)-(3) gives:

(4) EMFp,w(t) = CC,plast x Cplast x MCO2/MC x Hplast/Hwaste

Constants:

Cplast = 6,4 w/w% (Ref. The Danish Environmental Agency)

Cc,plast = 20 kg carbon/GJ in plastic (Ref. IPCC Guidelines)

MCO2 = 44 kg/kmol

MC = 12 kg/kmol

The values of EMFp,w for 1990 to 1998 used in the Danish air emission inventory are given in table 1.

Table 1 CO2-emissionfactor for plastic in municipal waste.

| Year | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hplast1) (GJ/ton) | 42,7 | 42,7 | 42,7 | 42,7 | 42,7 | 42,7 | 42,7 | 42,7 | 43 | 43 | 43 | 43 |
| Hwaste1) (GJ/ton) | 8,2 | 8,2 | 9 | 9,4 | 9,4 | 10 | 10,5 | 10,5 | 10,4 | 10,5 | 10,5 | 10,5 |
| EMFp,w (kg/GJ in waste) | 24,44 | 24,44 | 22,27 | 21,32 | 21,32 | 20,04 | 19,09 | 19,09 | 19,41 | 19,22 | 19,22 | 19,22 |

1) Ref.: The Danish Energy Agency

| Year | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Municipal waste (GJ) | 15,006 | 16,255 | 17,325 | 18,963 | 19,979 | 24,088 | 25,394 | 27,632 | 27,323 | 29,13 | 30,27 | 33,04 |
| CO2 emission from plastic (kton) | 367 | 397 | 386 | 404 | 426 | 483 | 485 | 527 | 530 | 560 | 582 | 635 |

### Appendix 4.5 CRF SNAP correspondence list

Table 13 Correspondence list of IPCC source categories 1A1, 1A2 and 1A4 and SNAP

| snap_id | snap_name | ipcc_id |
| --- | --- | --- |
| 01 | Combustion in energy and transformation industries |  |
| 0101 | Public power | 1A1a |
| 010101 | Combustion plants >= 300 MW (boilers) | 1A1a |
| 010102 | Combustion plants >= 50 and < 300 MW (boilers) | 1A1a |
| 010103 | Combustion plants < 50 MW (boilers) | 1A1a |
| 010104 | Gas turbines | 1A1a |
| 010105 | Stationary engines | 1A1a |
| 0102 | District heating plants | 1A1a |
| 010201 | Combustion plants >= 300 MW (boilers) | 1A1a |
| 010202 | Combustion plants >= 50 and < 300 MW (boilers) | 1A1a |
| 010203 | Combustion plants < 50 MW (boilers) | 1A1a |
| 010204 | Gas turbines | 1A1a |
| 010205 | Stationary engines | 1A1a |
| 0103 | Petroleum refining plants | 1A1b |
| 010301 | Combustion plants >= 300 MW (boilers) | 1A1b |
| 010302 | Combustion plants >= 50 and < 300 MW (boilers) | 1A1b |
| 010303 | Combustion plants < 50 MW (boilers) | 1A1b |
| 010304 | Gas turbines | 1A1b |
| 010305 | Stationary engines | 1A1b |
| 010306 | Process furnaces | 1A1b |
| 0104 | Solid fuel transformation plants | 1A1c |
| 010401 | Combustion plants >= 300 MW (boilers) | 1A1c |
| 010402 | Combustion plants >= 50 and < 300 MW (boilers) | 1A1c |
| 010403 | Combustion plants < 50 MW (boilers) | 1A1c |
| 010404 | Gas turbines | 1A1c |
| 010405 | Stationary engines | 1A1c |
| 010406 | Coke oven furnaces | 1A1c |
| 010407 | Other (coal gasification, liquefaction, ...) | 1A1c |
| 0105 | Coal mining, oil/gas extraction, pipeline compressors |  |
| 010501 | Combustion plants >= 300 MW (boilers) | 1A1c |
| 010502 | Combustion plants >= 50 and < 300 MW (boilers) | 1A1c |
| 010503 | Combustion plants < 50 MW (boilers) | 1A1c |
| 010504 | Gas turbines | 1A1c |
| 010505 | Stationary engines | 1A1c |
| 02 | Non-industrial combustion plants |  |
| 0201 | Commercial and institutional plants (t) | 1A4a |
| 020101 | Combustion plants >= 300 MW (boilers) | 1A4a |
| 020102 | Combustion plants >= 50 and < 300 MW (boilers) | 1A4a |
| 020103 | Combustion plants < 50 MW (boilers) | 1A4a |
| 020104 | Stationary gas turbines | 1A4a |
| 020105 | Stationary engines | 1A4a |
| 020106 | Other stationary equipments (n) | 1A4a |
| 0202 | Residential plants | 1A4b |
| 020201 | Combustion plants >= 50 MW (boilers) | 1A4b |
| 020202 | Combustion plants < 50 MW (boilers) | 1A4b |
| 020203 | Gas turbines | 1A4b |
| 020204 | Stationary engines | 1A4b |
| 020205 | Other equipments (stoves, fireplaces, cooking,...) | 1A4b |
| 0203 | Plants in agriculture, forestry and aquacultu | 1A4c |
| 020301 | Combustion plants >= 50 MW (boilers) | 1A4c |
| 020302 | Combustion plants < 50 MW (boilers) | 1A4c |
| 020303 | Stationary gas turbines | 1A4c |
| 020304 | Stationary engines | 1A4c |
| 020305 | Other stationary equipments (n) | 1A4c |
| 03 | Combustion in manufacturing industry |  |
| 0301 | Comb. in boilers, gas turbines and stationary | 1A2f |
| 030101 | Combustion plants >= 300 MW (boilers) | 1A2f |
| 030102 | Combustion plants >= 50 and < 300 MW (boilers) | 1A2f |
| 030103 | Combustion plants < 50 MW (boilers) | 1A2f |
| 030104 | Gas turbines | 1A2f |
| 030105 | Stationary engines | 1A2f |
| 030106 | Other stationary equipments (n) | 1A2f |
| 0302 | Proces furnaces without contact |  |
| 030203 | Blast furnace cowpers | 1A2a |
| 030204 | Plaster furnaces | 1A2f |
| 030205 | Other furnaces | 1A2f |
| 0303 | Processes with contact |  |
| 030301 | Sinter and pelletizing plants | 1A2a |
| 030302 | Reheating furnaces steel and iron | 1A2a |
| 030303 | Gray iron foundries | 1A2a |
| 030304 | Primary lead production | 1A2b |
| 030305 | Primary zinc production | 1A2b |
| 030306 | Primary copper production | 1A2b |
| 030307 | Secondary lead production | 1A2b |
| 030308 | Secondary zinc production | 1A2b |
| 030309 | Secondary copper production | 1A2b |
| 030310 | Secondary aluminium production | 1A2b |
| 030311 | Cement (f) | 1A2f |
| 030312 | Lime (includ. iron and steel and paper pulp industr.)(f) | 1A2f |
| 030313 | Asphalt concrete plants | 1A2f |
| 030314 | Flat glass (f) | 1A2f |
| 030315 | Container glass (f) | 1A2f |
| 030316 | Glass wool (except binding) (f) | 1A2f |
| 030317 | Other glass (f) | 1A2f |
| 030318 | Mineral wool (except binding) | 1A2f |
| 030319 | Bricks and tiles | 1A2f |
| 030320 | Fine ceramic materials | 1A2f |
| 030321 | Paper-mill industry (drying processes) | 1A2d |
| 030322 | Alumina production | 1A2b |
| 030323 | Magnesium production (dolomite treatment) | 1A2b |
| 030324 | Nickel production (thermal process) | 1A2b |
| 030325 | Enamel production | 1A2f |
| 030326 | Other | 1A2f |
| 08 | Other mobile sources and machinery |  |
| 0804 | Maritime activities |  |
| 080403 | National fishing | 1A4c |
| 0806 | Agriculture | 1A4c |
| 0807 | Forestry | 1A4c |
| 0808 | Industry | 1A2f |
| 0809 | Household and gardening | 1A4b |

### Appendix 4.6 Uncertainty calculations

Table 14 Uncertainty estimation, GWP

| IPCC Source category | Gas | Base year emission | Year t emission | Activity data uncertainty | Emission factor unceratinty | Combined uncertainty | Combined uncertainty as % of total national emissions in year t |  | Type A sensitivity |  |  | Type B sensitivity | Unceratinty i trend in national emissions introduced by emission factor uncertarinty | Uncertainty in trend in national emissions introduced by activity data uncertainty |  | Uncertainty introduced into the trend in total national emissions |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Input data | Input data | Input data | Input data |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Gg CO2 eq | Gg CO2 eq | % | % | % | % |  | % |  |  | % | % | % |  | % |  |
| Stationary Combustion, Coal | CO2 | 24209 | 16668 | 1 | 5 | 5,099 | 2,273 |  | -0,183 |  |  | 0,436 | -0,916 | 0,617 |  | 1,104 |  |
| Stationary Combustion, Petroleum coke | CO2 | 455 | 742 | 3 | 5 | 5,831 | 0,116 |  | 0,008 |  |  | 0,019 | 0,039 | 0,082 |  | 0,091 |  |
| Stationary Combustion, Residual oil | CO2 | 2505 | 1567 | 2 | 2 | 2,828 | 0,119 |  | -0,023 |  |  | 0,041 | -0,046 | 0,116 |  | 0,125 |  |
| Stationary Combustion, Gas oil | CO2 | 4858 | 3063 | 4 | 5 | 6,403 | 0,525 |  | -0,044 |  |  | 0,080 | -0,222 | 0,454 |  | 0,505 |  |
| Stationary Combustion, Kerosene | CO2 | 366 | 21 | 4 | 5 | 6,403 | 0,004 |  | -0,009 |  |  | 0,001 | -0,044 | 0,003 |  | 0,044 |  |
| Stationary Combustion, Orimulsion | CO2 | 0 | 2419 | 1 | 2 | 2,236 | 0,145 |  | 0,063 |  |  | 0,063 | 0,127 | 0,090 |  | 0,155 |  |
| Stationary Combustion, Natural gas | CO2 | 4330 | 11075 | 3 | 1 | 3,162 | 0,937 |  | 0,179 |  |  | 0,290 | 0,179 | 1,230 |  | 1,243 |  |
| Stationary Combustion, LPG | CO2 | 148 | 55 | 4 | 5 | 6,403 | 0,009 |  | -0,002 |  |  | 0,001 | -0,012 | 0,008 |  | 0,014 |  |
| Stationary Combustion, Refinery gas | CO2 | 806 | 813 | 3 | 5 | 5,831 | 0,127 |  | 0,001 |  |  | 0,021 | 0,003 | 0,090 |  | 0,090 |  |
| Stationary combustion plants, gas engines | CH4 | 5 | 443 | 2,2 | 40 | 40,060 | 0,475 |  | 0,011 |  |  | 0,012 | 0,459 | 0,036 |  | 0,460 |  |
| Stationary combustion plants, other | CH4 | 110 | 137 | 2,2 | 100 | 100,024 | 0,365 |  | 0,001 |  |  | 0,004 | 0,076 | 0,011 |  | 0,077 |  |
| Stationary combustion plants | N2O | 396 | 391 | 2,2 | 1000 | 1000,002 | 10,451 |  | 0,000 |  |  | 0,010 | 0,071 | 0,032 |  | 0,078 |  |
| Total |  | 38189 | 37394 |  |  |  | 115,965 |  |  |  |  |  |  |  |  | 3,302 |  |
| Total uncertainties |  | Overall uncertainty i the year (%): |  |  |  |  |  | 10,769 |  |  | Trend uncertainty (%): |  |  |  | 1,817 |  |  |

Table 15 Uncertainty estimation, CO2

| IPCC Source category | Gas | Base year emission | Year t emission | Activity data uncertainty | Emission factor unceratinty | Combined uncertainty | Combined uncertainty as % of total national emissions in year t |  | Type A sensitivity |  |  | Type B sensitivity | Unceratinty i trend in national emissions introduced by emission factor uncertarinty | Uncertainty in trend in national emissions introduced by activity data uncertainty | Uncertainty introduced into the trend in total national emissions |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Input data | Input data | Input data | Input data |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Gg CO2 | Gg CO2 | % | % | % | % |  | % |  |  | % | % | % | % |  |  |
| Stationary Combustion, Coal | CO2 | 24209 | 16668 | 1 | 5 | 5,099 | 2,333 |  | -0,178 |  |  | 0,442 | -0,888 | 0,626 | 1,086 |  |  |
| Stationary Combustion, Petroleum coke | CO2 | 455 | 742 | 3 | 5 | 5,831 | 0,119 |  | 0,008 |  |  | 0,020 | 0,040 | 0,084 | 0,093 |  |  |
| Stationary Combustion, Residual oil | CO2 | 2505 | 1567 | 2 | 2 | 2,828 | 0,122 |  | -0,023 |  |  | 0,042 | -0,045 | 0,118 | 0,126 |  |  |
| Stationary Combustion, Gas oil | CO2 | 4858 | 3063 | 4 | 5 | 6,403 | 0,539 |  | -0,043 |  |  | 0,081 | -0,216 | 0,460 | 0,508 |  |  |
| Stationary Combustion, Kerosene | CO2 | 366 | 21 | 4 | 5 | 6,403 | 0,004 |  | -0,009 |  |  | 0,001 | -0,044 | 0,003 | 0,044 |  |  |
| Stationary Combustion, Orimulsion | CO2 | 0 | 2419 | 1 | 2 | 2,236 | 0,149 |  | 0,064 |  |  | 0,064 | 0,128 | 0,091 | 0,157 |  |  |
| Stationary Combustion, Natural gas | CO2 | 4330 | 11075 | 3 | 1 | 3,162 | 0,962 |  | 0,183 |  |  | 0,294 | 0,183 | 1,247 | 1,260 |  |  |
| Stationary Combustion, LPG | CO2 | 148 | 55 | 4 | 5 | 6,403 | 0,010 |  | -0,002 |  |  | 0,001 | -0,012 | 0,008 | 0,014 |  |  |
| Stationary Combustion, Refinery gas | CO2 | 806 | 813 | 3 | 5 | 5,831 | 0,130 |  | 0,001 |  |  | 0,022 | 0,004 | 0,092 | 0,092 |  |  |
| Total | CO2 | 37678 | 36423 |  |  |  | 6,727 |  |  |  |  |  |  |  | 3,087 |  |  |
| Total uncertainties |  | Overall uncertainty i the year (%): |  |  |  |  |  | 2,594 |  |  | Trend uncertainty (%): |  |  |  |  | 1,757 |  |

Table 16 Uncertainty estimation, CH4

| IPCC Source category | Gas | Base year emission | Year t emission | Activity data uncertainty | Emission factor unceratinty | Combined uncertainty |  | Combined uncertainty as % of total national emissions in year t | Type A sensitivity |  |  | Type B sensitivity | Unceratinty i trend in national emissions introduced by emission factor uncertarinty | Uncertainty in trend in national emissions introduced by activity data uncertainty |  | Uncertainty introduced into the trend in total national emissions |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Input data | Input data | Input data | Input data |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mg CH4 | Mg CH4 | % | % | % |  | % | % |  |  | % | % | % |  | % |  |
| Stationary combustion plants, gas engines | CH4 | 237 | 21096 | 2,2 | 40 | 40,060 |  | 30,622 | 3,640 |  |  | 3,860 | 145,586 | 12,010 |  | 146,081 |  |
| Stationary combustion plants, other | CH4 | 5228 | 6502 | 2,2 | 100 | 100,024 |  | 23,566 | -3,607 |  |  | 1,190 | -360,673 | 3,702 |  | 360,692 |  |
| Total | CH4 | 5465 | 27598 |  |  |  |  | 1493,06 |  |  |  |  |  |  |  | 151439 |  |
| Total uncertainties |  | Overall uncertainty i the year (%): |  |  |  |  | 38,640 |  |  |  | Trend uncertainty (%): |  |  |  | 389,151 |  |  |

Table 17 Uncertainty estimation, N2O

| IPCC Source category | Gas | Base year emission | Year t emission | Activity data uncertainty | Emission factor unceratinty | Combined uncertainty |  | Combined uncertainty as % of total national emissions in year t | Type A sensitivity |  |  | Type B sensitivity | Unceratinty i trend in national emissions introduced by emission factor uncertarinty | Uncertainty in trend in national emissions introduced by activity data uncertainty |  | Uncertainty introduced into the trend in total national emissions |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Input data | Input data | Input data | Input data |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Gg N2O | Gg N2O | % | % | % |  | % | % |  |  | % | % | % |  | % |  |
| Stationary combustion plants | N2O | 1,279 | 1,261 | 2,200 | 1000,000 | 1000,002 |  | 1000,002 | 0,000 |  |  | 0,986 | 0,000 | 3,068 |  | 3,068 |  |
| Total | N2O | 1,279 | 1,261 |  |  |  |  | 1000005 |  |  |  |  |  |  |  | 9,411 |  |
| Total uncertainties |  | Overall uncertainty i the year (%): |  |  |  |  | 1000,002 |  |  |  | Trend uncertainty (%): |  |  |  | 3,068 |  |  |

Table 18 Uncertainties of fuel consumption rates

| Source | Mainly combusted in sector 1) | Uncertainty |
| --- | --- | --- |
| Stationary Combustion, Coal | Public power, cogeneration and district heating | 1% |
| Stationary Combustion, Petroleum coke | Industrial combustion | 3% |
| Stationary Combustion, Residual oil | Public power, cogeneration and district heating Industrial combustion | 2% |
| Stationary Combustion, Gas oil | Commercial, institutional, residential combustion | 4% |
| Stationary Combustion, Kerosene | Commercial, institutional, residential combustion | 4% |
| Stationary Combustion, Orimulsion | Public power, cogeneration and district heating | 1% |
| Stationary Combustion, Natural gas | Public power, cogeneration and district heating Industrial combustion Commercial, institutional, residential combustion | 3% |
| Stationary Combustion, LPG | Commercial, institutional, residential combustion | 4% |
| Stationary Combustion, Refinery gas | Public power, cogeneration and district heating Industrial combustion | 3% |
| Biomass: |  |  |
| Stationary Combustion, biomass in domestic plants | Biomass in small sources | 30% |

1) Sectors defined in table 2.6 of GPG

Table 19 Background data to determine combined fuel rate uncertainty

| fuel | fuel | Rate 2001 | Uncertainty |
| --- | --- | --- | --- |
| 102 | coal | 175451125 | 1 |
| 110 | Petroleum coke | 8062464 | 3 |
| 111 | Wood | 27217801 | 30 |
| 114 | Municipal waste | 33037670 | 20 |
| 117 | Straw | 13698057 | 20 |
| 203 | Residual oil | 20093058,05 | 2 |
| 204 | Gas oil | 41395271 | 4 |
| 206 | Kerosene | 286786 | 4 |
| 215 | Liquid bio fuel | 191475 | 20 |
| 225 | Orimulsion | 30243687 | 1 |
| 301 | Natural gas | 193449630,7 | 3 |
| 303 | LPG | 852741 | 4 |
| 308 | Refinery gas | 14233595 | 3 |
| 309 | Biogas | 3046907 | 20 |
| SUM |  | 561260267,8 |  |
| Combined uncertainty: | 2,0 |  |  |

Table 20 CH4 from gas engines 2001

| snap_id | fuel_id | asrat_val | emfa_val | uni_abbr | AS_EMIS | EMIS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 010105 | 301A | 28098555 | 573 | g | 16100472015 | 16100 |  |
| 010105 | 309A | 1589322 | 434 | g | 689765748 | 690 |  |
| 010405 | 301A | 0 | 573 | g | 0 | 0 |  |
| 010405 | 309A | 0 | 434 | g | 0 | 0 |  |
| 010505 | 301A | 11887 | 573 | g | 6811251 | 7 |  |
| 010505 | 309A | 28627 | 434 | g | 12424118 | 12 |  |
| 020105 | 301A | 1048143 | 573 | g | 600585939 | 601 |  |
| 020105 | 309A | 501385 | 434 | g | 217601090 | 218 |  |
| 020204 | 301A | 1450266 | 573 | g | 831002418 | 831 |  |
| 020304 | 301A | 2859644 | 573 | g | 1638576012 | 1639 |  |
| 020304 | 309A | 108819 | 434 | g | 47227446 | 47 |  |
| 030105 | 301A | 1641970 | 573 | g | 940848810 | 941 |  |
| 030105 | 309A | 23805 | 434 | g | 10331370 | 10 |  |
|  |  |  |  |  |  | 21096 | Mg |

Table 21 CH4 from gas engines 1990

| snap_id | fuel_id | asrat_val | emfa_val | uni_abbr | AS_EMIS | EMIS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 010105 | 301A | 313500 | 573 | g | 179635500 | 179,6355 |  |
| 010505 | 301A | 4900 | 573 | g | 2807700 | 2,8077 |  |
| 020105 | 301A | 26200 | 573 | g | 15012600 | 15,0126 |  |
| 020204 | 301A | 0 | 573 | g | 0 | 0 |  |
| 020304 | 301A | 66300 | 573 | g | 37989900 | 37,9899 |  |
| 030105 | 301A | 2600 | 573 | g | 1489800 | 1,4898 |  |
|  |  |  |  |  |  | 236,9355 | Mg |