GREENHOUSE GAS EMISSIONS IN FINLAND 1990–2001

NATIONAL INVENTORY REPORT

to the UNFCCC Secretariat

Common Reporting Formats (CRF)

1990–2001

28th March 2003

# PREFACE

Finland’s National Inventory Report under the UNFCCC (United Nations Framework Convention on Climate Change) contains the following three parts:

Part 1	Description of the greenhouse gas emission inventory according to the UNFCCC new reporting guidelines (FCCC/SBSTA/2002/L.5/Add.1) containing description of the organisation of the national greenhouse gas inventory, IPCC and other methods applied in calculation of the year 2001 emissions and exceptions to the previous inventories. A summarising table (Table 1) of the emissions data for the years 1990–2001 is included as well as a description of the current emission trends.

Part 2	CRF (Common Reporting Format) data tables of Finland’s updated greenhouse gas emission inventories for the years 1990–2001.

Part 3	Report on the methodology for calculating the greenhouse gas emission inventories has been sent to the UNFCCC Secretariat in the previous years. The report can also be downloaded from the website: . The website will be updated with the latest information by 15th April 2003.

Finland would like to thank the UNFCCC Secretariat for conducting the Individual Review of the greenhouse gas inventory of Finland in November 2001 and the In-depth Review of Finland’s Third National Communication in October 2002. Finland appreciates the excellent work carried out by the Expert Review Teams and would like to thank them. We welcome many suggestions and observations by the teams for improvements of our inventory in accordance with the Report of the Individual Review of the Greenhouse Gas Inventory of Finland submitted in the year 2001 (FCCC/WEB/IRI(2)/2002/FIN) and the draft report of the in-depth review of the third national communication (FCCC/IDR.3/FIN). Many of the suggested improvements have already been inserted into the present inventory submission.

Methodological improvements in accordance with the Revised (1996) IPCC Guidelines for National Greenhouse Gas Inventories and the Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories have been implemented in the present inventory as far as possible and will be implemented in their entirety as soon as possible.

Ministry of the Environment

P.O. Box 35

FIN-00023 GOVERNMENT

Finland

Tel. +358 9 16007, fax +358 9 1603 9545

http://www.environment.fi

CONTENTS (PART I)

PREFACE		2

EXECUTIVE SUMMARY	5

ES.1	Background information on greenhouse gas inventories and climate change	5

ES.2	Summary of national emissions and removals related to trends	6

ES.3	Overview of source and sink category emission estimates and trends	6

ES.4	Other information	7

1	INTRODUCTION	8

1.1	Background information on greenhouse gas inventories and climate change	8

1.2	A description of the institutional arrangement for inventory preparation	9

1.3	Brief description of the process of inventory preparation	10

1.4	Brief general description of methodologies and data sources	10

1.5	Brief description of key source categories	11

1.6	Information on the QA/QC plan including verification and treatment of confidentiality issues	14

1.7	General uncertainty evaluation, including data on the overall uncertainty for the inventory totals	16

1.8	General assessment of the completeness	16

1.9	Remarks and abbreviations	17

2	GREENHOUSE GAS EMISSION TRENDS	18

2.1	Description and interpretation of emission trends for aggregated greenhouse gas emissions	18

2.2	Description and interpretation of emission trends by gas	18

2.3	Description and interpretation of emission trends by source	20

2.4	Description and interpretation of emission trends of indirect greenhouse gases and sulphur oxides	21

3	ENERGY (CRF 1)	23

3.1	Overview of the sector	23

3.2	Fuel Combustion (CRF 1.A)	23

3.3	Fugitive Emissions from Fuels (CRF 1.B)	28

3.4	Reference approach	30

3.5	International bunkers	30

4	INDUSTRIAL PROCESSES (CRF 2)	32

4.1	Overview of the sector	32

4.2	Mineral Products (CRF 2.A)	32

4.3	Chemical Industry (CRF 2.B)	33

4.4	Metal Production (CRF 2.C)	34

4.5	Other Production (CRF 2.D)	35

4.6	Production of Halocarbons and SF6 (CRF 2.E)	36

4.7	Consumption of Halocarbons and SF6 (CRF 2.F)	37

4.8	Other (CRF 2.G)	39

5	SOLVENT AND OTHER PRODUCT USE (CRF 3)	40

5.1	Overview of the sector	40

5.2	Solvent and Other Product Use (CRF 3)	40

6	AGRICULTURE (CRF 4)	42

6.1	Overview of the sector	42

6.2	Enteric Fermentation (CRF 4.A)	43

6.3	Manure Management (CRF 4.B)	45

6.4	Rice Cultivation (CRF 4.C)	46

6.5	Agricultural Soils (CRF 4.D)	47

6.6	Burning of Savannas (CRF 4.E)	50

6.7	Field Burning of Agricultural Residues (CRF 4.F)	50

7	LAND-USE CHANGE AND FORESTRY (CRF 5)	51

7.1	Overview of the sector	51

7.2	Changes in Forest and Other Woody Biomass Stocks (CRF 5.A)	52

7.3	Forest and Grassland Conversion (CRF 5.B)	53

7.4	Abandonment of Managed Lands (CRF 5.C)	54

7.5	CO2 Emissions and Removals from Soil (CRF 5.D)	54

7.6	Other (CRF 5.E)	55

8	WASTE (CRF 6)	56

8.1	Overview of the sector	56

8.2	Solid Waste Disposal on Land (CRF 6.A)	56

8.3	Wastewater Handling (CRF 6.B)	58

8.4	Waste Incineration (CRF 6.C)	59

8.5	Other (CRF 6.D)	59

9	other (CRF 7)	60

9.1	Overview of the sector	60

9.2	Feedstock and Non-energy Use of Fuels (CRF 7)	60

10	RECALCULATIONS AND IMPROVEMENTS	61

10.1	Explanations and justifications for recalculations	61

10.2	Implications for emission levels	61

10.3	Implications for emission trends, including time series consistency	61

10.4	Recalculations, including response to the review process, and planned improvements to the inventory	62

REFERENCES	65

ANNEXES

Annex 1. Energy balance sheets	70

Annex 2. Emission factors implied in the energy sector	75

Annex 3. Additional information on uncertainty reporting	79

PART 2: CRF tables for the years 1990–2001

PART 3: Report on the Methodology: Greenhouse Gas Emissions and Removals in Finland

(Available from the website: )

PART I	EMISSION INVENTORIES

# EXECUTIVE SUMMARY

## ES.1	Background information on greenhouse gas inventories and climate change

Finland has carried out greenhouse gas inventories since the 1990’s to meet the obligations of the United Nations Framework Convention on Climate Change (UNFCCC). Inventory reports are delivered to the UNFCCC Secretariat and the European Commission annually.

A special task force, the Greenhouse Gas Working Group, was established 15th October 1998 by the Ministry of the Environment to act as an advisory body in the collection of greenhouse gas inventories and reporting of greenhouse gas emissions and reduction measures to the UNFCCC Secretariat and the European Commission. The Working Group was appointed to:

- Compile and report the Finnish total estimated emissions of the six greenhouse gases defined under the UNFCCC in 1990–2001, and their projected development up to 2010.

- Monitor and develop the greenhouse gas inventory methods and the work carried out for them internationally and in Finland.

- Make proposals for achieving more efficient data collection and improving the inventory systems.

The Working Group consists of representatives from ministries, agencies and research institutes, including: Ministry of the Environment, Ministry of Trade and Industry, Ministry of Transport and Communications, Ministry of Forestry and Agriculture, Finnish Meteorological Institute, MTT Agrifood Research Finland, Finnish Forest Research Institute, Finnish Environment Institute, Statistics Finland, and VTT Technical Research Centre of Finland.

In Finland, the Ministry of the Environment has the overall responsibility over the greenhouse gas inventory and reporting under the UNFCCC, but the Greenhouse Gas Working Group together with the supportive organisations takes care of the practical work. Several different organisations produce information that is used as base or emissions information for the inventory. The Finnish Environment Institute and Statistics Finland compile the inventory that is verified by the Working Group before the Ministry of the Environment reports it to the UNFCCC Secretariat and the European Commission.

IPCC Good Practice Guidelines and Revised 1996 Guidelines or national estimation methods are used in producing greenhouse gas emissions inventories. The Common Reporting Format (CRF) tables are used in reporting the emission figures. Emissions projections are usually presented by source and by gas, and as total emissions in CO2 equivalents. The responsibility of producing the emissions data is divided between several organisations, as presented in Section 1.2.

The national inventory and reporting system is being constantly developed and improved. The engaged organisations participate in the national and international research and development work related to the development of the methods and the inventory systems by recognising the development needs and maintaining the quality control system in their area of responsibility. Statistics Finland was appointed as the National Inventory Agency for the greenhouse gas inventory by a decision by the Government 30 January 2003.

## ES.2	Summary of national emissions and removals related to trends

TABLE 1. Summary of Finland’s greenhouse gas emissions data for 1990–2001.

(Remark: Due to rounding the sum of subtotals does not equal to total figures.)

| (Tg CO2 equivalents) | 1990 (Base year) | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |  |
| fuel combustion | 53.9 | 53.1 | 51.3 | 52.0 | 58.3 | 55.9 | 61.2 | 59.8 | 57.4 | 56.8 | 54.9 | 60.5 |
| fugitive emissions from fuels (peat, oil and natural gas) | 3.5 | 3.5 | 3.5 | 3.6 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 |
| industrial processes | 1.2 | 1.0 | 0.9 | 0.8 | 0.8 | 0.8 | 0.9 | 0.9 | 0.9 | 1.0 | 1.1 | 1.0 |
| agricultural soils | 3.2 | 2.8 | 2.3 | 2.2 | 2.1 | 1.7 | 1.8 | 2.1 | 2.0 | 2.0 | 2.0 | 1.9 |
| other | 0.6 | 0.6 | 0.6 | 0.6 | 0.7 | 0.7 | 0.7 | 0.6 | 0.7 | 0.8 | 0.7 | 0.7 |
| CH4 | 6.3 | 6.3 | 6.2 | 6.2 | 6.1 | 6.1 | 6.1 | 6.0 | 5.8 | 5.7 | 5.4 | 5.4 |
| N2O | 8.3 | 7.9 | 7.3 | 7.4 | 7.6 | 7.8 | 7.8 | 8.0 | 7.9 | 7.7 | 7.1 | 7.1 |
| SF6, HFCs, PFCs | 0.09 | 0.07 | 0.04 | 0.03 | 0.04 | 0.10 | 0.15 | 0.24 | 0.30 | 0.40 | 0.58 | 0.73 |
| TOTAL | 77.2 | 75.3 | 72.2 | 72.8 | 79.2 | 76.7 | 82.1 | 81.1 | 78.5 | 77.8 | 75.4 | 80.9 |
| Land-use change and forestry (removals) | -23.8 | -38.2 | -31.9 | -29.1 | -17.3 | -14.7 | -21.0 | -12.6 | -9.7 | -10.8 | -12.0 | -16.9 |

## ES.3	Overview of source and sink category emission estimates and trends

The main sources of greenhouse gas emissions have been officially divided into the following sectors: energy, industrial processes, agriculture, waste, and solvent and product use (Figure 1). The land use change and forestry sector (LUCF) acts both as a source and a sink for carbon dioxide emissions, currently absorbing approximately 20% of the annual emissions from other sectors as the CO2 removals in this category exceed its emissions.

The energy sector is the most significant source of greenhouse gas emissions with over 80% share of the total emissions. This includes all emissions related to the production, distribution and consumption of fuels. The energy sector releases three greenhouse gases, CO2, CH4 and N2O. Energy related CO2 emissions vary mainly according to the economic trend and the energy supply structure. Today, the availability of hydropower in the integrated Nordic electricity market has a notable effect on the emissions. Due to these reasons there was a 5.5 Tg CO2 increase in the energy sector between the years 2000 and 2001.

FIGURE 1. Finnish greenhouse gas emissions by source 1990–2001

Agriculture is the second most significant source of greenhouse gas emissions, with approximately 10% of Finland’s total emissions. Emissions from agriculture include CO2, CH4 and N2O. The total emissions from agriculture have a clearly decreasing trend. The annual emissions have reduced approximately by 25% since 1990 due to decreases in cultivation of organogenic land, in the number of livestock, and in nitrogen fertilisation.

The industrial processes category accounts for approximately 4% of the total greenhouse gas emissions, and includes non-energy related releases of CO2, CH4, N2O and F-gases. Emissions from industrial processes decreased significantly until 1993, but have since had a rapidly increasing trend. In 2001, the emissions were approximately equal to the 1990 level.

Also, the waste sector accounts for approximately 4% of Finland’s total greenhouse gas emissions. The emissions consist of CH4 and N2O, and have had a decreasing trend in the 1990’s. Overall, the annual emissions have decreased by over 20%. The emission volumes from the waste sector have changed to some extent for the years 1990–2001 compared to previous inventories, because the calculation methods for the waste sector have been revised by changing from the mass balance method used until 2000 to the first order decay (FOD) method drafted by the IPCC, as suggested by the IOR teams and according to the IPCC Good Practice Guidance.

Solvent and product use cause mainly N2O emissions. Emissions from this sector are only 0.1% of Finland’s total greenhouse gas emissions and have remained the same throughout the 1990’s.

## ES.4	Other information

[Space left intentionally blank.]

# INTRODUCTION

## Background information on greenhouse gas inventories and climate change

Climate change means anthropogenic alterations in the global climate, such as temperature increase, sea level rise, and severe and abrupt weather conditions caused by significant increases in greenhouse gas emissions. The latest report by the Intergovernmental Panel on Climate Change (IPCC) indicates that global temperatures could rise by 1.4–5.8 degrees by 2100. This means increases in average temperatures, which can have a significant impact on the local climates, weather conditions, and flora and fauna in areas around the world. The severity of the impacts will vary regionally and be difficult to predict in advance.

In Finland, climate change research is being conducted among others by CLIMTECH climate change and technology programme that investigates the development needs and possibilities of the technologies, which can be applied to control greenhouse gas emissions and climate change, and by FIGARE, the Finnish Global Change Research Programme. FINSKEN is a FIGARE project, co-ordinated by the Finnish Environment Institute, that is developing consistent global change scenarios for Finland, including for example climate change and sea-level scenarios. The climate scenarios anticipate both annual temperature increase (1.8–5.2ºC) and rainfall increase (4–28%) by 2050 compared to the mean values of 1961–1990. These trends are expected to continue in the second half of the century. According to the present information, climate change is anticipated to have both direct and indirect impacts in Finland. The impacts are related for example to the endurance of the Northern ecosystems, winter tourism, increased flooding and the prevalence of pests and diseases as well as the possible growth of agriculture and forestry and decreased need for heating energy.

According to the United Nations Framework Convention on Climate Change (UNFCCC), ratified in 1994, and the Kyoto Protocol 1997, Finland is committed to reduce its greenhouse gas emissions to the 1990 level by 2008–2012 and submit an annual inventory reports of its anthropogenic greenhouse gas emissions to the UNFCCC Secretariat. Finland has carried out greenhouse gas inventories since the early 1990’s, delivering inventory reports both to the UNFCCC Secretariat and the European Commission annually.

A special task force, the Greenhouse Gas Working Group (chaired by Mr Jaakko Ojala, counsellor, Ministry of the Environment, jaakko.ojala@environment.fi ), was established 15th October 1998 by the Ministry of the Environment to act as an advisory body in collection of greenhouse gas inventories and reporting of greenhouse gas emissions and emissions reduction activities to the UNFCCC Secretariat and the European Commission. The Working Group was appointed to:

- Compile and report the Finnish total estimated emissions of the six greenhouse gases defined under the UNFCCC in 1990–2001, and their projected development up to 2010.

- Monitor and develop the greenhouse gas inventory methods and the work carried out for them internationally and in Finland.

- Make proposals for achieving more efficient data collection and improving inventory systems.

The Working Group consists of representatives from ministries, agencies and research institutes, including Ministry of the Environment, Ministry of Trade and Industry, Ministry of Transport and Communications, Ministry of Forestry and Agriculture, Finnish Meteorological Institute, MTT Agrifood Research Finland, Finnish Forest Research Institute, Finnish Environment Institute, Statistics Finland, and VTT Technical Research Centre of Finland.

In Finland, the Ministry of the Environment has the overall responsibility over the greenhouse gas inventory and reporting under the UNFCCC, but the Greenhouse Gas Working Group together with supportive organisations takes care of the practical work. Several different organisations produce information that is used as base or emissions information for the inventory. The Finnish Environment Institute and Statistics Finland compile the inventory that is verified by the Working Group before the Ministry of the Environment reports it to the UNFCCC Secretariat and the European Commission. This National Inventory Report (Part 1) is compiled at the Finnish Environment Institute by Ms Kristina Saarinen (kristina.saarinen@environment.fi) and Mr Mikko Attila (mikko.attila@environment.fi), based on data from the above mentioned organisations.

IPCC Good Practice Guidelines and Revised 1996 Guidelines or national estimation methods are used in producing greenhouse gas emissions inventories. Common Reporting Format (CRF) tables are used in reporting the emission figures. Emissions projections are usually presented by source and by gas, and as total emissions in CO2 equivalents to enable comparisons between the different greenhouse gases. Responsibility of producing the emissions data according to the CRF data tables is divided between several organisations, as presented in Section 1.2.

The national inventory and reporting system is constantly being developed and improved. The engaged organisations participate in national and international research and development work related to the development of methods and inventory systems by recognising the development needs and maintaining the quality control system in their area of responsibility. Statistics Finland was appointed as the National Inventory Agency for the greenhouse gas inventory by a decision by the Government 30 January 2003.

## A description of the institutional arrangement for inventory preparation

According to the provisions of the COP/SBSTA/SBI decisions, Finland is carrying out the greenhouse reporting in consultation with the relevant ministries, institutes and experts. The Ministry of the Environment has set an inter-ministerial working group to assist in reporting greenhouse gases and their sinks. Facilitation and guidance of the work was chaired by the Ministry of the Environment.

Calculation of the Finnish 2001 greenhouse gas inventory data to the UNFCCC Secretariat was carried out by organisations presented in Table 2.

TABLE 2. Responsible institutes in calculation of the greenhouse gas emissions in Finland.

| CRF category | Organisation responsible for the inventory data in the year 2001 report to the UNFCCC Secretariat |
| --- | --- |
| 1.A | Statistics Finland |
| 1.B | Statistics Finland Ministry of the Environment Finnish Environment Institute (SYKE) |
| 2 (I) | Statistics Finland VTT Technical Research Centre of Finland Finnish Environment Institute (SYKE) |
| 2 (II) | Finnish Environment Institute (SYKE) |
| 3 | Finnish Environment Institute (SYKE) |
| 4 | MTT Agrifood Research Finland |
| 5 | Finnish Forest Research Institute (Metla) |
| 6 | Finnish Environment Institute (SYKE) |
| 7 | Statistics Finland |
| International bunkers | Statistics Finland |

Statistics Finland was appointed as the National Inventory Agency for the greenhouse gas inventory by a decision by the Government 30 January 2003.

## Brief description of the process of inventory preparation

The inventory report is in practice compiled by the Finnish Environment Institute (SYKE) from material provided by the research institutes, agencies and ministries participating in the inter-ministerial working group set by the Ministry of the Environment. The CRF tables are compiled by the Finnish Environment Institute and Statistics Finland from data provided by the above mentioned organisations.

The greenhouse gas inventory was prepared by Statistics Finland, Finnish Environment Institute, VTT Technical Research Centre of Finland, MTT Agrifood Research Finland, Finnish Forest Research Institute (Metla) and Ministry of the Environment.

The following ministries, agencies and research institutes also participated in the work: Ministry of the Environment, Ministry of Trade and Industry, Ministry of Transport and Communications, Ministry of Forestry and Agriculture, Finnish Meteorological Institute and VTT Technical Research Centre of Finland.

## Brief general description of methodologies and data sources

In this report, compilation of the year 2001 inventory is described more closely while compilation of the 1990–2000 inventories is presented at a general level. More detailed description of the methodologies is presented in the report “Greenhouse Gas Emissions and Removals in Finland” which is also available from the website: http://www.ymparisto.fi/eng/environ/state/air/emis/ghg/ghg.htm. Documentation of the F-gases inventory is presented in Oinonen (2003).

Calculation methods for the years 1990–2001

There are differences in the calculation methods used for the years 1990–2000 emissions and year 2001 emissions. To facilitate a review of time series consistency, these differences are listed below. Changes in emission factors and activity data are not accounted.

CRF 1.A	A new version of ILMARI calculation system is under development. The 2001 inventory has been calculated using preliminary version of the new system. Calculation methods are the same as in the previous ILMARI, but the database is new. Also some classifications have been revised (for example NACE instead of the previous national industrial classification). This has caused some changes in the allocation between subsectors 1.A.1 and 1.A.2.

1991 emissions are not calculated with ILMARI; they are based on top-down estimates.

International bunkers: Calculation is included in the ILMARI system; see comments on CRF 1.A.

CRF 1.B	NMVOC estimates for 2001 are updated according to the results from the Finnish NMVOC calculation model at the Finnish Environment Institute. The whole time series will be updated in the next submission.

CRF 2 (I)	Most parts of the calculation are included in the ILMARI system; see comments on CRF 1.A.

NMVOC estimates for 2001 are updated according to the results from the Finnish NMVOC calculation model at the Finnish Environment Institute. The whole time series will be updated in the next submission.

CRF 2 (II)	None.

CRF 3	NMVOC estimates for 2001 are calculated according to the results from the Finnish NMVOC calculation model at the Finnish Environment Institute.

CRF 4	CH4 emissions from enteric fermentation of reindeer have been included in the whole time series.

CRF 5	None.

CRF 6	For solid waste disposal on land the whole time series has been recalculated. The calculation method has been changed from the mass balance method to the FOD method.

CRF 7	None.

Use of bottom-up data

A specific feature for the Finnish emission inventories is use of compliance data reported by the industrial installations. The installations report their annual emissions data to the environmental supervising authorities according to the installations’ monitoring obligations determined in their individual environmental permits. After checking and approving the data the supervising authorities record the data into the database (VAHTI) from where it is available for emission inventory purposes. Statistics Finland and the Finnish Environment Institute collect this data and, after normal statistical checking (e.g. check of magnitude and trend) as well as an inter-comparison between the two institutes, the data is taken into the emissions inventory data systems of both Statistics Finland and the Finnish Environment Institute. For F-gases no VAHTI data is currently available.

## Brief description of key source categories

Key sources are the emission sources, which have a significant influence on the total inventory in terms of the absolute level (2001) of emissions, trend of emissions (change between 1990–2001) or both. There are two alternative methods for identifying key source categories: Tier 1 and Tier 2. In the Tier 2 method, also the relative uncertainties of source categories are taken into account. The level key sources are defined by multiplying the level assessment of the source category (i.e. the proportion of source category emissions of total emissions) with the source category uncertainty. The source categories are then sorted by descending order of magnitude by the above mentioned criteria.

Key source categories are those, which together represent 90% of the inventory uncertainty. In the level and trend assessment the number of identified key source categories was 25. The identified key source categories are presented in Tables 2 and 3 with descending order of importance. A summary of the key source categories is presented in Table E in Annex 3. The key source analysis were carried out at the VTT Technical Research Centre of Finland.

TABLE 2. Key source categories by level (Tier 2).

| A | B | C | D | #### E |
| --- | --- | --- | --- | --- |
| Source category number | Gas | Level key sources | Level assessment with uncertainty1 | Cumulative total of column D |
| 1.B | CO2 | Arable peatlands | 0.1702 | 0.1702 |
| 4.D | N2O | Agricultural soils | 0.1316 | 0.3018 |
| 1.B | CO2 | Peat production areas | 0.1068 | 0.4086 |
| 1.A.4 | CO2 | Other sectors: liquid fuels | 0.0881 | 0.4967 |
| 2.B.2 | N2O | Nitric acid production | 0.0644 | 0.5611 |
| 6.A | CH4 | Solid waste disposal on land | 0.0470 | 0.6081 |
| 1.A.1 | CO2 | Energy industries: other fuels | 0.0283 | 0.6364 |
| 1.A.5 | CO2 | Other: liquid fuels | 0.0271 | 0.6635 |
| 1.A.3 | N2O | Road transportation: gasoline (cars with catalytic converters) | 0.0269 | 0.6904 |
| 4.A | CH4 | Enteric fermentation | 0.0245 | 0.7149 |
| 1.A.4 | CH4 | Other sectors: biomass | 0.0239 | 0.7388 |
| 1.A.1 | CO2 | Energy industries: solid fuels | 0.0192 | 0.7579 |
| 2.F.1 | HFCs | Refrigeration and air conditioning equipment | 0.0183 | 0.7762 |
| 7 | CO2 | Other – non-energy use of fuels | 0.0178 | 0.7940 |
| 4.B | N2O | Manure management | 0.0166 | 0.8107 |
| 1.A | N2O | Indirect N2O from fuel combustion | 0.0161 | 0.8268 |
| 1.A.1 | N2O | Energy industries: other fuels | 0.0140 | 0.8408 |
| 6.B.2 | N2O | Domestic and commercial wastewater: densely populated areas | 0.0135 | 0.8542 |
| 1.A.2 | CO2 | Manufacturing industries and construction: solid fuels | 0.0101 | 0.8643 |
| 1.A.3 | N2O | Road transportation: diesel | 0.0081 | 0.8724 |
| 1.A.4 | N2O | Other sectors: liquid fuels | 0.0072 | 0.8797 |
| 1.A.2 | N2O | Manufacturing industries and construction: biomass | 0.0070 | 0.8867 |
| 1.A.3 | CO2 | Road transportation: diesel | 0.0066 | 0.8933 |
| 1.A.2 | CO2 | Manufacturing industries and construction: liquid fuels | 0.0064 | 0.8997 |
| 6.B.2 | CH4 | Domestic and commercial wastewater | 0.0062 | 0.9060 |

TABLE 3. Key source categories by trend (Tier 2).

| A | B | #### C | D | #### E |
| --- | --- | --- | --- | --- |
| Source category number | Gas | #### Trend key sources | Trend assessment with uncertainty1 | Cumulative total of column D |
| 4.D | N2O | Agricultural soils | 0.1657 | 0.1657 |
| 1.A.4 | CO2 | Other sectors: liquid fuels | 0.1120 | 0.2777 |
| 1.A.3 | N2O | Road transportation: gasoline (cars with catalytic converters) | 0.0829 | 0.3606 |
| 2.B.2 | N2O | Nitric acid production | 0.0775 | 0.4381 |
| 2.F.1 | HFCs | Refrigeration and air conditioning equipment | 0.0631 | 0.5013 |
| 6.A | CH4 | Solid waste disposal on land | 0.0569 | 0.5582 |
| 1.A.1 | CO2 | Energy industries: other fuels | 0.0439 | 0.6021 |
| 1.B | CO2 | Arable peatlands | 0.0381 | 0.6402 |
| 1.A | N2O | Indirect N2O from fuel combustion | 0.0320 | 0.6722 |
| 4.B | N2O | Manure management | 0.0299 | 0.7021 |
| 1.B | CO2 | Peat production areas | 0.0239 | 0.7260 |
| 1.A.5 | CO2 | Other: liquid fuels | 0.0237 | 0.7497 |
| 4.A | CH4 | Enteric fermentation | 0.0229 | 0.7726 |
| 1.A.3 | N2O | Road transportation: gasoline (cars without catalytic converters) | 0.0172 | 0.7898 |
| 1.A.1 | N2O | Energy industries: biomass | 0.0165 | 0.8063 |
| 1.A.3 | N2O | Civil aviation | 0.0147 | 0.8210 |
| 6.B.2 | N2O | Domestic and commercial wastewater (densely populated areas) | 0.0144 | 0.8355 |
| 1.A.4 | CH4 | Other sectors: biomass | 0.0123 | 0.8478 |
| 1.A.4 | N2O | Other sectors: biomass | 0.0093 | 0.8571 |
| 1.A.2 | N2O | Manufacturing industries and construction: other fuels | 0.0090 | 0.8661 |
| 1.A.2 | CO2 | Manufacturing industries and construction: solid fuels | 0.0086 | 0.8747 |
| 1.A.2 | N2O | Manufacturing industries and construction: liquid fuels | 0.0085 | 0.8832 |
| 1.A.2 | N2O | Manufacturing industries and construction: biomass | 0.0081 | 0.8913 |
| 1.A.1 | N2O | Energy industries: other fuels | 0.0079 | 0.8992 |
| 1.A.1 | CO2 | Energy industries: gaseous fuels | 0.0078 | 0.9070 |

1 For each source category, Equation 7.4 in IPCC Good Practice Guidance (Penman et al. 2000) is divided with the sum of all TUx,t

Key sources were also defined using a national method for the level assessment (2001). In the national method, a sensitivity analysis was performed by computing rank correlation coefficients between all input parameters (with the simulation tool Crystal Ball). The advantage of this method is that the sources of uncertainties are identified at a very disaggregated level rather than only at source category level. The results of this method are presented in Figure 2.

FIGURE 2. The level key sources in the Finnish 2001 emission inventory using the national method. The key sources are measured by their contribution to the variance. EF denotes emission factor, A activity data and E emissions.

## Information on the QA/QC plan including verification and treatment of confidentiality issues

Development of quality systems

The quality management system for the national greenhouse gas inventory is currently under development and will be implemented in the inventory of the year 2002 emissions. Statistics Finland as the National Inventory Agency will coordinate the project.

The expert institutes participating in the inventory are presently building up quality systems for the inventories in their areas of expertise. The quality systems shall be in place according to the following schedule in the participating expert institutes:

- Statistics Finland is building up a quality system for calculation of the Energy (CRF 1) and Industrial processes (CRF 2(I)) sectors during 2002–2003. The system is planned to be in use in the 2002 inventory.

- Finnish Environment Institute is building up a quality system for calculation of Industrial processes (CRF 2(II)), Solvent and other product use (CRF 3) and Waste (CRF 6) sectors during 2002–2003. The system is part of the air emissions inventory system for air pollutants and greenhouse gases at the Finnish Environment Institute, and is planned to be in use in the 2002 inventory.

- At the MTT Agrifood Research Finland the quality system for the agriculture (CRF 4) sector is under development and some measures have already been taken. The system is planned to be in use in the 2002 inventory

- At the Finnish Forest Research Institute the quality system work related to the Land use and forestry (CRF 5) sector is connected with the new biomass allocation models where reliability figures are under development and will be in use in 2006.

Archiving of the inventory

The annually reported CRF tables are archived both at the Finnish Environment Institute and Statistics Finland. The method descriptions together with documents of the original data sources are archived at the Finnish Environment Institute with the exception of confidential activity data in the energy sector that is archived at Statistics Finland.

Verification

The present inventory of the year 2001 emissions is not verified by a third party but in the inter-ministerial working group (see Executive Summary Chapter ES.1).

Confidentiality issues related to F-gases

The methodologies used to calculate potential and actual emissions of HFCs, PFCs and SF6 are based on data obtained directly from corporate entities, universities and research institutions. These sources of data have responded to annual surveys providing information on, for instance, chemical quantities imported, exported and used in manufacturing products. The data gathering has been based on voluntary inquiries and a promise not to disclose confidential information, or to report results in a manner that confidential information could be inferred.

Although there can be no absolute safeguards against breaches of confidentiality, care has been taken not to publish or otherwise release identified or identifiable data. To lessen the likelihood of such breaches, reporting has been based on anonymity. Moreover, to counteract the opportunities for others to infer confidential information, grouping of activity and emissions data have been carried out.

Because of the multidimensional structure of the tables in the CRF – emissions (and activity data in case of sectoral background data tables) are reported disaggregated to sub-source categories, to individual chemical species, to manufacturing, use and disposal emissions, to emissions calculated using different methods (Tier 1a, 1b, 2) – the grouping in many cases becomes an inadequate strategy to safeguard against breaches of confidentiality. The number of respondents is simply too small in certain categories to support such disaggregation.

For this reason it has not always been possible to report emissions on the most disaggregated level for a source category (paragraph 19 in UNFCCC Guidelines on Annual Inventories, FCCC/SBSTA/1999/6/Add. 1). In order to facilitate the assessment of completeness, the cells for which data cannot be reported due to confidentiality, have been marked with ‘C’ (paragraph 21.(e) of the Guidelines on Annual Inventories).

In the previous inventories, confidential data have been grouped over sub-source categories in order to include the emitted quantities, and to enable the data flow from sectoral report tables to summary tables. This does, however, inflict some damage to analysis possibilities, because components of emissions have been moved from one category to another. For instance, HFC-152a emissions from aerosols have been aggregated with HFC-152a emissions from refrigeration and air conditioning. With many such transfers from one category to another, the entire categorisation soon begins to lose its meaning. It is also a question of consistency if grouping over categories is carried out on an annual basis, and if the allocation of emissions to categories varies from one year to another. Moreover, also comparability becomes an issue: because a component of emissions from aerosols is confidential, emissions from aerosols in Finland cannot be compared to emissions from aerosols, say, in Denmark. Moreover, if one component of emissions from aerosols is added to emissions from refrigeration and air conditioning, this emission category also becomes non-comparable.

In order to minimise these damages to analysis possibilities, the grouping practice over categories has been given up. All confidential emissions data have been grouped and added to figures in the summary tables. Also, classification of one species-source category combination is kept constant over all inventory years. This means that a combination may be confidential in one year and not in the second year, but the combination for both years is classified confidential for the sake of consistency. These practices are hoped to inflict minimum damage to the usefulness of categorisation, consistency and comparability.

It should be noted that although every effort to protect the sources of data have been taken, the possibility of disclosing confidential information cannot be ruled out entirely. For example, it is possible to envisage a situation where the end-users of a certain chemical are numerous, but the importers of the chemical are few. In such a case, if only the end-users are surveyed, and not the entire chain of production, it is possible that the data is not confidential in case of the end-users, but it may be connected to the activity of just one or two importers.

## General uncertainty evaluation, including data on the overall uncertainty for the inventory totals

In the year 2001 inventory, the uncertainty assessment was performed for the first time using the Monte Carlo simulation. The uncertainties in the input parameters were estimated using the IPCC default uncertainties, expert elicitation, domestic and international literature and available measurement data. A separate report on the uncertainty estimates (Monni & Syri 2003) will be published in 2003. According to the calculations, the total uncertainty of the inventory was –5...+6% in 2001. The trend uncertainty was 5 percentage points. When the uncertainty estimate was performed using the Tier 1 method, the corresponding uncertainties were 7% and 6%. The share of CO2 emissions from fuel combustion, which has low uncertainties, is large in Finland, thus resulting in a rather low total inventory uncertainty, though some input parameters in other emission categories have very large uncertainties. Land use and land use change is not included in the present uncertainty analysis.

The uncertainties by gas were as follows: –4...+6% for CO2, –19...+20% for CH4, –33...+40% for N2O and

–53...+32% for HFCs, PFCs and SF6 together.

The results of the uncertainty estimates according to Tier 1 and Tier 2 methods are presented in Tables A-D in Annex 3. The sums of the total emissions in 1990 and 2001 differ from the sum in the inventory. This results from three reasons: first, a minor reason, is that rounding and simulation might alter the mean value slightly. Secondly, CO2 emissions from agricultural soils are not taken into account in the uncertainty estimation, because they are not dealt with in the “Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Emission Inventories” (Penman et al. 2000), but will be covered in the LULUCF Good Practice Guidance (these emissions can be reported in either Agriculture or LUCF sector). Thirdly, the uncertainty estimates are based on figures sent to the European Commission in December 2002. Thus some minor differences with the submission to the UNFCCC might occur. The uncertainty estimations were carried out at VTT Technical Research Centre of Finland.

## General assessment of the completeness

Completeness by emission sources

The inventory is complete in coverage of emission sources with the following exceptions:

CRF 1.B.2	Fugitive emissions from oil and natural gas: emissions of some substances are estimated to be nearly zero (negligible). This has to be re-checked in the future inventories.

CRF 2.A, B, D	Emissions from industrial processes: emissions from some source categories are estimated to be nearly zero (negligible). This has to be re-checked in the future inventories.

CRF 4	Rice cultivation and burning of savannas does not occur in Finland. Field burning of agricultural residues is occasional and small scale, thus these emissions are estimated negligible and not reported. Nitrogen fraction of certain vegetable and fruit crops will be estimated and included into the calculations in the future together with the improved evaluation of the areas of mineral and organic soils.

More detailed information on the non-covered emission sources is presented in the relevant CRF tables.

Completeness by geographical coverage

The inventory includes emissions from the autonomic territory of Aland (Ahvenanmaa). Information on the specified emissions for the territory of Aland estimated by the Finnish Environment Institute will be available at the website http://www.ymparisto.fi/eng/environ/state/air/emis/ghg/ghg.htm by end of April 2003.

Completeness by timely coverage

In general, complete CRF tables are provided for all years. In the energy sector, recent studies on emission factors, more developed estimation models and updated energy data have caused some inconsistencies in the time series. These are described in more detail in Section 3.2. The time series will be recalculated in the future inventories to remove inconsistencies.

## Remarks and abbreviations

The figures in the CRF tables are given at the calculation accuracy that the CRF programme uses (2 decimals). The actual emission estimates are not that accurate.

Abbreviations:

CS	country specific

D	IPCC default

DC	degradable components

H	high (high confidence in estimation), combined uncertainty (uc) of the activity data and emission factor, uc < 10%

IE	included elsewhere

L	low (high confidence in estimation), uc > 40%

M	medium (high confidence in estimation), 10% < uc < 40%

NE	not estimated

NO	not occurring

O	source estimated

PS	plant specific

# GREENHOUSE GAS EMISSION TRENDS

## Description and interpretation of emission trends for aggregated greenhouse gas emissions

The aggregated greenhouse gas emissions include the six gases defined in the Kyoto Protocol, carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O) and F-gases that include HFC and PFC combination groups and sulphur hexafluoride (SF6). The emission levels are presented in tera grammes (Tg) of carbon dioxide equivalents, which is identical with millions of tonnes.

In 2001, Finland’s total greenhouse gas emissions were approximately 81 Tg in CO2 equivalents. This was approximately 3.5 Tg above the 1990 baseline level. The inventory methods and certain time series still contain some inaccuracies and uncertainties which will be improved in the following years.

As illustrated in Figure 3, Finland’s greenhouse gas emission volumes have varied considerably during the 1990’s. This variation has mainly depended on what have been, at given times, the particular economic trends for the energy intensive sectors, the production level of hydropower, the level of imported electricity and the availability of alternative non-carbon energy sources. In Finland, the level of imported electricity is highly affected by the annual rainfall situation as the neighbouring states, Sweden and Norway, have significant hydropower capacities.

FIGURE 3. Finnish aggregated greenhouse gas emissions in 1990–2001.

The development of Finland’s greenhouse gas emissions in the future has been modelled and projected by scenarios. According to the National Climate Strategy (2001), the greenhouse gas emissions are expected to increase to approximately 90 Tg carbon dioxide equivalents during the obligation period by 2010 and to 95 Tg by 2020, if additional climate policy measures are not carried out.

## Description and interpretation of emission trends by gas

Carbon dioxide (CO2) is the main greenhouse gas causing the climate change (Figure 4). In 2001, its share of Finland’s total greenhouse gas emissions was as high as 80% and thus the CO2 trends tend to be the same as the aggregated emission trends above. In 2001, the total CO2 emissions were over 67 Tg, 8% above the 1990 baseline level. The largest source of carbon dioxide in Finland is fuel combustion in the energy industries. Other sources of CO2 are transport, industrial processes and agriculture.

FIGURE 4. Finnish greenhouse gas emissions trend 1990–2001 by gas.

Nitrous oxide (N2O) is the second most significant greenhouse gas in Finland with approximately 10% share of the total greenhouse gas emissions. In 2001, N2O emissions were 7.1 Tg in CO2 equivalents, reduced by 1.2 Tg from 1990. As illustrated in Figure 5 below, N2O emissions have an oscillating trend in the 1990’s with gradual reduction in emissions in the past 4 years. These emissions arise from agriculture, energy production, transport, industrial processes and waste, agriculture being clearly the largest source with over half of the total N2O emissions.

FIGURE 5. Finnish greenhouse gas emissions by gas excluding CO2 in 1990–2001.

Methane (CH4) emissions in Finland have been reduced significantly throughout the 1990’s. Figure 5 illustrates a steadily decreasing trend. In 2001, CH4 emissions were only 5.4 Tg in CO2 equivalents, decreased by one Tg from 1990. CH4 is still a significant greenhouse gas in Finland with 8% of the total net greenhouse gas emissions in 2001, but its significance has clearly decreased from 1990 when its share was approximately 12% of the total emissions. The reduction between 1990 and 2001 is explained by changes that have occurred in the waste sector. The waste and agricultural sectors are the largest sources of methane emissions, with approximately 56% and 33% of the total methane emissions, respectively.

F-gases, that include HFC and PFC combination groups and sulphur hexafluoride (SF6), are minor greenhouse gases but their emissions have strongly increased in the 1990’s. The increase was eight-fold from 0.09 Tg CO2 equivalents in 1990 to 0.73 Tg in 2001. This means that their share of the total greenhouse gas emissions has increased from 0.1% in 1990 to 0.9% in 2001. The rapid increase in the F-gas emissions began after 1995. F-gases, mainly HFC in Finland, are released from industrial processes.

## Description and interpretation of emission trends by source

The main sources of greenhouse gas emissions are divided into the following sectors: energy, industrial processes, agriculture, waste, and solvent and product use (Figure 6).

The energy sector is clearly the largest source of greenhouse gas emissions with over 80% share of the total emissions. This includes all fuel consumption (excluding non-energy use of fuels) and fugitive emissions related mainly to peat production and distribution of oil products and natural gas. This means that also the emissions from fuels used by the industry are reported under this sector. The energy industries caused most of the emissions from the energy sector, approximately 40%, but also manufacturing industries and construction (21%) and transport (19%) have a significant impact on the emissions of this sector.

The energy sector releases three greenhouse gases: CO2, mainly from fuel combustion in energy industries, CH4 and N2O. The emissions in the energy sector have varied considerably throughout the 1990’s with an overall slightly increasing trend being visible. One of the most essential emission reduction measures in the transport source category is the EU level agreement with car manufacturers on reducing the vehicle fuel consumption. The success of this and other EU climate policies will strongly shape the emission trends in the future.

Agriculture is the second most significant source of greenhouse gas emissions, with approximately 10% share of the total emissions. In Finland, the emissions are released mainly from agricultural soils but also through enteric fermentation and manure management. Emissions from agriculture include CO2, CH4 and N2O. The total emissions from agriculture have a clearly decreasing trend. The annual emissions have reduced approximately by 25% since 1990 due to decreases in cultivation of organogenic land, in the number of livestock, and in nitrogen fertilisation. The decreasing emissions trend will be safeguarded in the EU common agricultural policy by adopting support measures encouraging production that minimises the burden on the greenhouse gas balance.

The industrial processes sector releases approximately 4% of the total greenhouse gas emissions, and includes non-energy related releases from mineral products (CO2), chemical industry (CH4 and N2O), metal production (CH4), and the consumption of halocarbons and SF6. The most important industrial greenhouse gas emissions are the N2O emissions from nitric acid production and CO2 emissions from cement and lime production. The emissions from industrial processes decreased significantly until 1993, but have since had a rapidly increasing trend. The increase can be partly explained by the significant growth in the Finnish industry, mainly in the electronics but also in metal, forest and chemical industries. The significant growth in production has increased emissions both in the industrial processes sector and the energy sector, despite of the fact that the industry has made distinct improvements in eco-efficiency over the last two decades. In 2001, the emissions of the industrial processes sector were approximately equal to the 1990 level.

FIGURE 6. Finnish greenhouse gas emissions by source 1990–2001 excluding land-use change and forestry.

The waste sector accounts for approximately 4% of Finland’s total greenhouse gas emissions. The emissions consist of CH4 from solid waste disposal on land, and CH4 and N2O from waste water treatment. The emissions from waste had a decreasing trend in the 1990’s. Overall the annual emissions have decreased by more than 20%. The emissions trend from the waste sector has changed for 1990–2001. Emissions for each year have increased to some extent, because the calculation methods for the waste sector have been revised by changing from the mass balance method used until 2000 to the first order decay (FOD) method drafted by the IPCC. The difference between these methods is that the mass balance method assumes that the methane emissions are released at the time when the wastes are deposited in the landfill sites, whereas the FOD method takes into account the emissions development over time.

Solvent and product use cause mainly nitrous oxide emissions and volatile organic compounds. N2O is an actual greenhouse gas whereas the NMVOC compounds are indirect gases. Emissions from this sector are only 0.1% of Finland’s total greenhouse emissions and have stayed the same throughout the 1990’s.

## Description and interpretation of emission trends of indirect greenhouse gases and sulphur oxides

Emissions of indirect greenhouse gases by gas

The emissions trends of the indirect greenhouse gases, sulphur dioxides, nitrous oxides, carbon monoxide and non-methane volatile organic compounds, are presented in Figure 7. Detailed information on indirect greenhouse gases and other air pollutants is available at the Finnish Environment Institute website http://www.ymparisto.fi/ympsuo/ilma/laatu/airpollutant.htm.

The sulphur dioxide emissions totalled 87 Gg out of which 87% originated in the energy sector, where energy industries generated 56% of the total emissions and manufacturing industries and construction 31%.

Nitrous oxides were generated exclusively in the energy sector. The total emissions were 210.0 Gg. Transport sector was responsible for 44% of the emissions. Energy industries as well as manufacturing industries and construction generated 22% and 21% of the emissions, respectively.

Carbon monoxide emissions, total 603.5 Gg, originated also exclusively in the energy sector, where transport generated 68% and other sectors (including small scale combustion in the residential energy sector as well as off road machinery in forestry, agriculture and fishery) 21% of the total emissions.

The non-methane volatile organic compounds totalled 155.9 Gg in 2001. 71% of the total emissions were generated in the energy sector, where transport generated 34%, other sectors (including small scale combustion in the residential energy sector as well as off road machinery in forestry, agriculture and fishery) 23% and fugitive emissions from fuels 11% of the total emissions. 20% of the NMVOC emissions originated from solvent and other product use and 8% from industrial processes.

FIGURE 7. Finnish indirect greenhouse gas emissions trend 1990–2001 by gas.

Emissions by source

The energy sector generated 87% (76.3 Gg) of the sulphur dioxide emissions, 100% of both nitrous oxides emissions (210.0 Gg) and carbon monoxide emissions (603.5 Gg), and 71% (110.2 Gg) of the NMVOC emissions. Inside the energy sector transport generated 44% (92.3 Gg) of the nitrous oxides emissions, 68% (410.3 Gg) of the carbon monoxide emissions and 34% (53.4 Gg) of the NMVOC emissions.

Industrial processes generated 13% (10.9 Gg) of the sulphur dioxide emissions and 8% (12.3 Gg) of the NMVOC emissions.

Solvent and other product use generated 20% (31.4 Gg) of the NMVOC emissions.

The waste sector share was 1% (2.0 Gg) of the total NMVOC emissions.

# ENERGY (CRF 1)

## Overview of the sector

The energy sector is clearly the largest source of greenhouse gas emissions with over 80% share of the total emissions. In the 1990’s first the economic depression and later the recovery especially in the energy intensive export industries have effected the demand of energy. The energy supply structure has also caused fluctuation in the energy-related greenhouse gas emissions. In the integrated Nordic electricity market the annual rainfall and accordingly the availability of cheap hydro-power have decreased the Finnish greenhouse gas balance in the early 1990’s as well as during the three last years of the decade. In addition, in the late 1990’s, the sharp increase in the world-market oil prices, the increased capacity in existing nuclear power plants and decreased space-heating demand caused by exceptional warm winters have lowered the greenhouse gas emissions in the energy sector. Emissions from the energy sector in 2001 are presented in Figure 8.

FIGURE 8. Finnish greenhouse gas emissions from the energy sector 1990–2001.

## Fuel Combustion (CRF 1.A)

### Source category description

All the existing point sources are included in the categories 1.A.1 Energy industries and 1.A.2 Manufacturing Industries and Construction, i.e. power plants and boilers (Pfuel > 5 MW) as well as industrial plants with a boiler. The total number of plants is approximately 1 000, including 2 000 boilers or industrial processes. These point sources cover together two thirds of the total annual fuel combustion.

Category 1.A.3 Transport covers emissions from road transport, civil aviation, railways, navigation and other transportation.

Categories 1.A.4 Other sectors and CRF 1.A.5 Other cover all the remaining fuel combustion activities that are not covered by the categories 1.A.1–1.A.3.

### Methodological issues

All emissions from fuel combustion are calculated with Statistics Finland’s ILMARI calculation system which follows mostly the Tier 2 method in the IPCC Guidelines. The ILMARI calculation system has been used for national emission calculations of CO2, SO2, NO2, CO, CH4, N2O, NMVOC and PM emissions of fuel combustion from the year 1992. Also the year 1990 emissions have now been calculated by ILMARI. The CRF tables for the year 1991 are produced by top-down estimates based on data for 1990 and 1992.

ILMARI combines three types of source data:

1.	Detailed bottom-up data for point sources covers approximately 2/3 of the total annual fuel combustion (collected from the Regional Environment Centres’ VAHTI data system, Electricity Statistics, District Heating Statistics and Manufacturing Industry Statistics). The total fuel consumption data as well as some aggregate sectoral or subsectoral fuel data are from the national Energy Statistics.

2.	Aggregate transport and off-road vehicle data is originally calculated by the detailed calculation models LIPASTO and TYKO of the VTT Technical Research Centre of Finland and they cover approximately 1/6 of the total fuel combustion.

3.	Aggregate sectoral (subsectoral) data for other sources (small combustion, residential and others) covers approximately the rest 1/6 of the total fuel combustion.

#### Categories 1.A.1 Energy industries and 1.A.2 Manufacturing Industries and Construction

A new version of the ILMARI calculation system is under development. The 2001 inventory has been calculated using a preliminary version of the new system. The calculation methods are the same as in the previous ILMARI, but a new database system will be constructed.

At the moment the data sources are the same as in the previous system, but other data sources with plant level data will be included into the system to reduce uncertainties in the sector.

The previous years will be recalculated with the new system to remove inconsistencies in the sector. This will be done in sequence with the ongoing revision of the fuel consumption time series.

Some classifications have been revised (for example NACE instead of the previous national industrial classification). This has caused some changes in the allocation between the subsectors 1.A.1 and 1.A.2.

Calculation of the CO2 emissions is based on a country specific (Tier 2, Revised (1996) Guidelines) method using detailed activity (fuel consumption) data and fuel specific emission factors. The emission factors are either IPCC defaults, country or plant specific. The inventory uses bottom-up data always when available. The country specific emission factors are listed.

The CO2 emissions from coke and residual fuel oil used in the blast furnaces in iron and steel industry have been allocated to 1.A.2a Fuel consumption in Manufacturing Industry instead of 2.C Industrial Processes. There are two reasons for this: firstly, coke has usually been treated as energy producing material in the Finnish Energy Statistics. Secondly, the calculation of emissions is more accurate from the total coke consumption than from partly coke and partly blast furnace gases. The amount of CO2 emissions from coke and residual fuel oil used in the blast furnaces is approximately 5 Gg (of 53–62 Gg CO2 from the total fuel combustion). Other, non-energy based emissions have been allocated to the CRF category 2.G Industrial processes.

The SO2 and NO2 emissions are based on the emission data reported by the plants and recorded in the environmental authorities VAHTI database. The emissions are allocated to fuel based emissions (CRF 1) by each fuel and non-fuel-based (process) emissions (CRF 2).

The emissions of CH4, N2O and CO are based on a country specific method (Tier 2, Revised (1996) Guidelines) using detailed activity data and technology based emission factors for each boiler or process type (emission factors are available for approximately 250 categories of boilers and processes).

The calculation methods are consistent with the IPCC Guidelines except for calculation of the indirect N2O emissions from NOx deposition. The guidelines give in the agriculture sector (page 4.105 in the Guidelines) a method and an emission factor for estimation of indirect N2O emissions due to atmospheric deposition of NOx emissions. In the Finnish inventory these emissions are calculated also for energy-related and industrial NOx emissions using the methodology and the IPCC default emission factor (0.0125 kg N2O-N/kg NOx-N emitted). The indirect N2O emissions caused by nitrogen deposition due to NOx emissions in the energy sector are included in the emission estimates for the relevant sectors. The share of the indirect emissions is approximately 20–25% of the total N2O emissions of fuel combustion.

The indirect N2O emissions from NOx deposition are added to the direct N2O emissions in CRF table 1 sectoral report for energy as well as to all summary tables (1.A, 1.B, 2). However, they are not included in the sectoral background data tables for energy (1.A.a).

The activity data used in emission calculation is based on fuel consumption by boilers/processes and by fuel types. The basic data source is the VAHTI database which is complemented and cross-checked against other fuel data sources mentioned above.

Emission factors	The complete set of emission factors used in the inventory is presented in Annex 2.

CO2	PS/CS/D

CH4	PS/CS

N2O	PS/CS

SOx	Plant specific reported emission data (permitted installations’ annual reports) or CS for fuel specific emission factors.

NO2	Plant specific reported emission data (permitted installations’ annual reports) or CS for fuel specific emission factors.

CO, NMVOC	CS

#### Category 1.A.3 Transport

The calculation method is in general consistent with the IPCC Guidelines.

CO2 emissions within the transport sector and within the different transport modes are calculated using the LIPASTO calculation system () maintained and developed by the VTT Technical Research Centre of Finland. The LIISA sub-model of the LIPASTO calculates all emissions from the road transport for the years 1980–2022. The aggregate results of LIISA are taken into the ILMARI system. In the ILMARI calculation system road transport activity data covers diesel oil and motor gasoline consumption in road transport vehicles. Fuel consumption data is compared to the total diesel oil and motor gasoline consumption from the Energy Statistics (fuel sales). Road transport covers almost 100% of these fuels. Natural gas in road transport has been included since the 1999 inventory.

In the ILMARI system aggregate country specific emission factors for diesel oil and motor gasoline are used. The emissions are originally calculated by the very detailed VTT Technical Research Centre of Finland’s LIPASTO model. In the LIPASTO calculation model the emission factors have been adjusted to the Finnish car population.

The activity data (fuel consumption) for other transport sectors than road transport, follows the data in the Energy Statistics. Part of the data is obtained directly from companies, another part is coming from national authorities, and the rest have been estimated with the LIPASTO and TYKO models.

There are differences in the domestic aviation activity data between the LIPASTO calculation model and the Energy statistics. The differences have to be checked to enable the results to be used in the ILMARI system. For emission factors used in civil aviation, railways, navigation, other transport, see the references mentioned in Boström (1994). Since the 1999 inventory the emission factors are mostly taken from the LIPASTO and TYKO models and completed with some IPCC 1996 default emission factors for CH4 and N2O.

The emissions from inland waterways passenger transport are not included in the domestic navigation model MEERI (a sub-model of LIPASTO). At the moment there is no data available for estimation of these emissions.

The estimated emissions from off-road machinery (machinery used in construction, agriculture and other applications) are included in the inventories and reported under the relevant emission sectors. The results of the VTT Technical Research Centre of Finland’s new off-road machinery model (TYKO model) have been used in the ILMARI from 1999 onwards.

The ILMARI fuel consumption for off-road machinery is allocated according to the Energy statistics which leads to the following division in the CRFs: the emissions fall into CRF categories 1.A.3 Other Transportation, CRF 1.A.4c Agriculture/Forestry/Fisheries and CRF 1.A.2f Other/Construction

#### Categories 1.A.4 Other sectors and CRF 1.A.5 Other

The calculation method is consistent with the IPCC Guidelines. A closer description for the method is given in connection with categories CRF 1.A.1–1.A.2 for further information (ILMARI).

Activity data is obtained from the energy statistics. For the applied emission factors, see CRF 1.A.1–1.A.2 for further information.

### Uncertainty and time series’ consistency

Due to new results from the transport and off road machinery calculation models and other updates of activity data and emission factors there are some inconsistencies in the time series of the energy sector sub-categories. These will be recalculated in the following inventories. The recalculated time series will be included in the new version of ILMARI.

In the fuel combustion sector, the uncertainty in the year 2001 emissions was 3%. Finland has no fossil fuel production – all fossil fuels are imported and the statistics are very accurate. The factors that affect the uncertainty most are the liquid fuels activity data in Other sectors and Other. The next most important sources are other fuels (peat) activity data in Energy Industries and the CO2 emission factor in the same sector. The N2O (and CH4) emission factors from fuel combustion are highly uncertain, but they don’t affect much the total uncertainty, because the CO2 emissions dominate the emission level.

Although the total fuel sales for most fuels are quite well known, there are difficulties in allocation of some fuels. The most difficult fuels to allocate are gasoil, which is used in space heating, off-road machinery, water transport etc., as well as residual fuel oil, which is used in steam/heat production, industrial processes and space heating. There are large uncertainties in the fuel consumption data within these sectors (1.A.4 and 1.A.5). The errors made in the allocation do not affect the total CO2 emissions, but might cause relatively high errors in other emissions, because the emission factors vary a lot depending on the type of technology (for example NOx emissions from space heating compared to NOx emissions from diesel engines).

The activity and emission factor data for off-road machinery are among the most uncertain within the energy sector.

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. The quality system is under development and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

#### Categories 1.A.1 Energy industries and 1.A.2 Manufacturing industries and construction

Emissions for the year 1991 are not calculated with the same model. There are estimates of the total sectoral emissions in Table 11 (Section 6.5.2), but these may be revised in the future. There has been progress in the harmonisation of emissions from domestic air and water transport between the ILMARI and LIPASTO calculation models. However, all results for the years 1990–2000 have not yet been updated to the CRF tables. This will be done as soon as possible. At the moment, there are some differences in the activity data, which need to be checked.

### Source-specific planned improvements

#### Categories 1.A.1 Energy industries and 1.A.2 Manufacturing industries and construction

Some minor updates for the years 1992–1994 will be made in the future. There are some ongoing research activities for producing better data from residential, service sector and off-road machinery fuel consumption. The changes in the activity data should not affect the total CO2 emissions but the sectoral breakdowns may change.

The CO2 emission factor for municipal solid waste needs to be revised.

The non-CO2 emission factors used in ILMARI are based on research data from the beginning of the 1990’s and appropriate for emission calculations for that period. Since then, however, the combustion conditions and fuel mixes have changed, and an update of the emission factors based on research and measurements under current conditions is needed. This is especially needed for N2O emissions from fluidised bed combustion due to its growing importance for the Finnish emissions inventory.

In addition to measurements, the IPCC default emission factors and international research carried out after 1990 on the CH4 and N2O emissions from combustion should be evaluated and used when updating the emission factors. The N2O and CH4 emissions are very much dependent on both the fuel type and combustion conditions. This should be taken into consideration when deciding if and how the new data should be applied.

Results from new measurements on CH4 and N2O emission factors of certain boiler types have been used in the year 2000 inventory. As expected, the results showed that the N2O emission factors for fluidised bed combustion were overestimated. The time series will be updated in the future inventories.

The oxidation factor of peat combustion has been changed from 0.98 to 0.99 (IPCC default). However, this correction has not yet been made to years 1992–1994. There are also some other minor corrections for these years which will be made in the future.

The time series for the NMVOC emissions from residential fuel combustion is not constant due to changes in the activity data and emission factors. The time series will be updated in the future inventories. This applies also partly to CH4, N2O and CO emissions.

#### Category 1.A.3 Transport: Civil aviation, railways, navigation and other transport

There are differences in the domestic aviation activity data between the LIPASTO calculation model and the Energy statistics. The differences have to be studied to enable use of the results to be used in the ILMARI system.

#### Categories 1.A.4 Other sectors and CRF 1.A.5 Other

More detailed data is needed on the allocation of gas oil and residual fuel oil to different sectors and types of use. Emission factors for small combustion of wood need to be studied further to achieve better annual comparability. Recalculation of time series for all non-point sources is underway. The results of the recalculation will be updated to the CRF tables as soon as possible.

#### General

When preparing the CRF tables for 1990, some differences between total emissions of SO2 and NO2 were noticed compared to the previously published official time series. These differences will be checked in the future.

REMARK: In comparison between 2001 and previous years there is a difference in the allocation of emissions between CRF 1.A.1 and 1.A.2. Some power plants, which in the previous years had been allocated as autoproducers in the subsector CRF 1.A.2, have now been corrected to the CRF 1.A.1b. The effect of this is approximately 2.3 Gg of CO2 in 2001 data (1.6 Gg in 2000).

## Fugitive Emissions from Fuels (CRF 1.B)

### Source category description

CRF category 1.B.1 Fugitive emissions from solid fuels include in Finland the fugitive CO2 and CH4 emissions that arise particularly from peat production (preparation and profiling of peat soils and stockpiling of peat). The current inventory includes emissions from the production areas (surface emissions and emissions from stockpiles and ditches) and from arable peatland classified as reservoirs for future peat production. The areas and emission factors used in the inventory are uncertain and the calculated emissions have therefore been rounded to the nearest integer expressed in Tg CO2. The same emission estimates have been used for all years in the inventory (1990–2001). Efforts to improve the estimates are underway. There are no coal mines in Finland.

CRF category 1.B.2 Fugitive emission from oil and natural gas include CO2 emissions from venting and flaring from oil refineries (CRF 1.B.2c.i Venting). Only CO2 emissions are reported, other emissions are estimated to be negligible. Methane emissions in CRF 1.B.2b.iii Other leakage include emissions from emptying of natural gas pipelines for extension work. Other leakages from the pipelines are estimated to be negligible as the pipelines are relatively new and only 5% of the natural gas is distributed via local networks to small consumers (households, restaurants, greenhouses etc.) The NMVOC emissions originate from oil refineries and storage of chemicals at the refineries, road traffic evaporative emissions from cars, petrol distribution chain and refuelling of cars, ships and aircraft. There is no exploration or production of oil and natural gas in Finland.

### Methodological issues

#### Category 1.B.1 Fugitive emissions from solid fuels

The calculation method is in general consistent with the IPCC Guidelines. The calculation method for peat is country specific.

The peat production area in Finland is around 50 000–60 000 ha (e.g. Selin 1999) and the arable peatland classified as peat production reservoirs are estimated to be approximately 100 000–150 000 ha (Laine et al. 1998). The CO2 emissions from peat production sites are approximately 1 100 g CO2 m-2 a-1 (Nykänen et al 1996). The CO2 emission factor for cultivated peatland that are classified as peatland reservoirs is approximately 450 g C m-2 a-1.

#### Category 1.B.2 Fugitive emission from oil and natural gas

The calculation method is consistent with the IPCC Guidelines. The CO2 and CH4 emission estimates are based on plant specific information. The NMVOC emissions from oil storages are based on emission data from the Regional Environmental Centres’ VAHTI database. Evaporative emissions from cars is based on expert estimation at the VTT Technical Research Center of Finland and emissions from petrol distribution chain and refuelling of vehicles on expert estimation of Finnish Gas and Oil Federation.

### Uncertainty and time series’ consistency

The uncertainty in fugitive emissions from fuels is very high due to uncertainties in emissions from peat production areas and arable peatlands. The total uncertainty in 2001 was –59...+106%. The most important factors affecting the uncertainty are the area of arable peatlands, CO2 emission factor for peat production areas and CO2 emission factor for arable peatlands. The activity data on cultivated peatland classified as reservoirs for future peat production are uncertain and under review, as well as the allocation of the emissions (Energy, Agricultural or LUCF sector).

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. The quality system is under development and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

No recalculations have been made to the previous inventories.

### Source-specific planned improvements

Calculation of the emissions from 1.B.1 Fugitive emissions from solid fuels will be made for the whole period since 1990 when improved data on areas of peatland and the emission factors is available.

The results of the NMVOC calculation model of the Finnish Environment Institute have to be updated into the ILMARI model in the CRF category 1.B.2 Fugitive emission from oil and natural gas to years 1990–1999. Calculation of the fugitive emissions from distribution of oil and natural gas (which were estimated to be negligible) will be carried out according to the IPCC default methodology in the following inventories.

## Reference approach

Reference approach (RA) is done using import, export, production and stock change data from the energy balance (EB) sheet published in annual energy statistics. However, RA table requires liquid fuels reported in more disaggregated level than in the EB sheet. This data was taken from the background data of EB.

Another difference is that in the EB sheet stock changes and statistical differences are combined for certain fuels, whereas in the RA table only stock changes are reported.

In the 2001 inventory the difference of CO2 emissions between RA and Sectoral Approach (SA) was

–2.70%, which is probably due to the differences mentioned above.

Another reference calculation is included in Annex 1. In addition to the EB sheets there are CO2 emissions calculated directly from the EB sheet. This calculation shows 0.3% difference compared to the SA calculation.

## International bunkers

### Source category description

International bunkers cover international aviation and navigation according to the IPCC Guidelines.

### Methodological issues

The emissions are calculated by the ILMARI calculation model of Statistics Finland (see closer CRF 1.A). Fuel consumption by transport mode is obtained from the energy statistics and it includes fuel sales to ships and planes going abroad. The country specific emission factors used are the same as for domestic transport.

The case of Aland could be seen as an exception to the IPCC definitions. In the present inventory all trips going to Sweden via Aland are treated as international, because the number of passengers (or cargo) leaving or entering the ships in Aland is low. According to the IPCC methodology, the trip should be divided into domestic and international parts.

However, both the desk review report and the in-country review report of the Finnish greenhouse gas inventory accept the allocation used in the inventory to be consistent with the Revised 1996 IPCC Guidelines and the Good Practice Guidance.

### Uncertainty and time series’ consistency

No uncertainty estimation for international bunkers has been carried out.

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. The quality system is under development and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

None.

### Source-specific planned improvements

Harmonisation of emission factors in the ILMARI and LIPASTO calculation models is underway. The results will be updated to the CRF tables as soon as possible.

# INDUSTRIAL PROCESSES (CRF 2)

## Overview of the sector

Industrial greenhouse gas emissions contribute less than 4% to the total anthropogenic greenhouse gas emissions in Finland. The most important industrial greenhouse gas emissions are the N2O emissions from nitric acid production, and CO2 emissions from cement and lime production. F-gases are together only about 0.5% of the total greenhouse gas emissions in Finland. Coke and ethylene production release small amounts of CH4. The emissions have fluctuated somewhat during the 1990’s. The most significant change is the increase of emissions of F-gases which are now more than five fold compared to the 1990 emissions. The N2O emissions have decreased significantly almost equalling the increase of the F-gases. The CH4 emissions have increased by more than 30% but their contribution to the total industrial emissions is very small. Industrial CO2 emissions decreased considerably at the beginning of the 1990’s but have now almost reached the level of the 1990 emissions. The emission trend of greenhouse gas emissions in CO2 equivalents from industrial processes is presented in Figure 9.

FIGURE 9. Finnish greenhouse gas emissions trend from the industrial processes sector 1990–2001.

## Mineral Products (CRF 2.A)

### Source category description

At the moment the most important non-energy industrial process for non-energy CO2 emission sources are cement and lime production. Lime production includes production in steel industry.

The NMVOC emissions from road paving are included. Asphalt roofing is included in road paving.

The SO2 emissions are not split between fuel based and non-fuel based. All SO2 emissions are reported under CRF 1.A.2f.

### Methodological issues

CO2 emissions from cement and lime production are calculated by Statistics Finland. The calculation method is consistent with the IPCC Guidelines. Activity data for calculation of the CO2 emissions from cement and lime production are obtained from the Manufacturing Industry Statistics as well as from the industrial plants directly. The emission factors are the IPCC default factors.

The NMVOC emissions from road paving and asphalt roofing are calculated at the Finnish Environment Institute. The activity data and emission factors are from Fortum Oil and Gas Ltd. (Blomberg 2002).

### Uncertainty and time series’ consistency

The uncertainty in emissions from cement production and lime production together was around 6%. The total uncertainty in industry sector was –27...+43%.

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. Quality systems are under development both at Statistics Finland and the Finnish Environment Institute and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

No recalculations have been made to the previous inventories.

### Source-specific planned improvements

CO2 emissions from the use of mineral products need to be estimated.

## Chemical Industry (CRF 2.B)

### Source category description

All ammonia currently used in Finland is imported from other countries. In 1990–1993 small amounts (4–30 kt per year) were produced using mainly peat and saw dust as raw material. The CO2 emissions from these processes have not been estimated and included in the inventory.

Ethylene production is a source of CH4 emissions. Ethylene production in Finland has fluctuated from about 180 to 260 Gg ethylene per year between 1990 and 2001.

Nitric acid production is a source of N2O emissions. Nitric acid is produced at two sites (in 1990–1992 at three sites) in Finland. The production has varied from about 430 to 550 Gg nitric acid per year. Adipic acid is not produced in Finland.

The NMVOC emission sources include chemical industry and storage of chemicals at the sites.

### Methodological issues

For N2O emissions a country specific calculation method based on measurements is used. The method is consistent with the IPCC Guidelines.

The CH4 emissions have been calculated with the IPCC default emission factor 1 g CH4/kg ethylene produced. The annual ethylene production figures have been obtained from the production plants and manufacturing statistics. The annual nitric acid production figures have been obtained from the production plants. The N2O emission factors are based on measurements carried out at the factories in 1999. More measurements are carried out currently, and this may lead to adjustments in the emission factors used.

The NMVOC emissions are based on emission data from the Regional Environment Centres’ VAHTI database and collected by the Finnish Environment Institute.

### Uncertainty and time series’ consistency

The uncertainty in nitric acid production was –57...+99% in 2001. The uncertainty in ethylene production was around 21%.

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. Quality systems are under development both at Statistics Finland and the Finnish Environment Institute and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

No recalculations have been made to the previous inventories.

### Source-specific planned improvements

Industrial emission sources for CH4 and the suitability of the IPCC default emission factors should be studied further.

## Metal Production (CRF 2.C)

### Source category description

CH4 emissions from coke production are included in the inventory.

The NMVOC emission sources include iron and steel production and secondary aluminium production. Degreasing and painting are included in CRF 3.B.

SF6 emissions from magnesium die casting are included in the inventory.

### Methodological issues

CH4 emissions from coke production are calculated at Statistics Finland. Activity data for the calculation is obtained from the Energy Statistics. The emission factor is the IPCC default value (IPCC 1995). The calculation method for CH4 is consistent with the IPCC Guidelines. The CO2 emissions from coke and residual fuel oil used in blast furnaces are allocated in metal production in the energy sector CRF 1.A.

In the earlier inventories also CH4 emissions from pig iron and sinter production were reported. Based on the Revised 1996 IPCC Guidelines and measurements carried out at the Finnish plants, these emissions are now considered to be negligible and omitted from the inventory.

The NMVOC emissions from iron and steel production and secondary aluminium production NMVOC emission estimates are calculated at the Finnish Environment Institute based on emission data from the Regional Environment Centres’ VAHTI database and the Finnish Metal Industries Federation. The emission factors are taken from the Joint EMEP/Corinair Atmospheric Inventory Guidebook.

### Uncertainty and time series’ consistency

The uncertainty in iron and steel production (coke) was estimated to be at around 20% in 2001.

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. The quality system is under development and will be implied in the inventory of the year 2002 emissions. At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

None.

### Source-specific planned improvements

No work plan for future improvements currently exists for this category.

## Other Production (CRF 2.D)

### Source category description

The non-fuel based CO2 emissions the from pulp and paper industry as well as glass production are estimated to be negligible.

All NO2 emissions from the pulp and paper industry are reported as fuel based emissions under CRF 1.

Catalytic cracking of oil is identified as a possible N2O source in the IPCC Guidelines but no method or default emission factors are given. Catalytic cracking of oil is carried out at the refineries and, thus, this might be a possible emission source in Finland, too.

NMVOC emissions from the forest and food industries are included.

### Methodological issues

NMVOC emissions from the forest industry are calculated at the Finnish Environment Institute. Activity data for the calculation is obtained from the Finnish Forest Industries Federation and the emission factors from the Finnish Forest Industries Federation, Report August 1996 and The Finnish Forest Industries Federation, Annual report 2001, Sawmills and board production.

NMVOC emissions from the food industry are calculated at the Finnish Environment Institute. Activity data for calculation of the NMVOC emissions from the food industries is obtained from Suomen Hiiva Oy, the National Research and Development Centre for Welfare and Health (Stakes), the Finnish Food and Drink Industries' Federation, the Plant Production Inspection Centre (KTTK) and from the Finnish Fisheries Research Institute. The emission factors are taken from the NPI (1999), Joint EMEP/Corinair Atmospheric Inventory Guidebook (2001) and YTV (1995).

All SO2 emissions of different sulphur compounds are calculated as SO2 equivalents.

The calculation methods are consistent with the IPCC Guidelines.

### Uncertainty and time series’ consistency

No uncertainty estimation for non-fuel based CO2 emissions has been carried out due to missing emission data.

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. The quality system is under development and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

No recalculations have been made to the previous inventories.

### Source-specific planned improvements

No work plan for future improvements currently exists for this category.

## Production of Halocarbons and SF6 (CRF 2.E)

Halocarbons and SF6 are not produced in Finland.

## Consumption of Halocarbons and SF6 (CRF 2.F)

### Source category description

Historically, HFC, PFC and SF6 (for short: F-gases) emissions have been very low in Finland. This is explained by the absence of certain large industrial point sources that account for most of the F-gases emissions globally. First of all, F-gases are not produced in Finland, which means that there are no fugitive emissions from manufacturing. Moreover, there is no manufacturing of other fluorinated gases, such as HCFCs, that could lead to by-product emissions (e.g. HFC-23 from HCFC-22 manufacturing). Other point sources that have generated plenty of emissions elsewhere, but are absent in Finland, include primary aluminium and magnesium industry.

F-gases emissions from the Finnish sources thus follow from consumption of these gases in various applications:

- SF6 used as a cover gas in magnesium die casting

- SF6 used in electrical equipment (gas insulated switchgear and circuit breakers)

- SF6 used in “semi-prompt applications” (running shoes, windows)

- HFCs, PFCs and SF6 used in semiconductor manufacturing

- HFCs and PFCs used as refrigerants in refrigeration and air conditioning equipment

- HFCs used as propellants in aerosols and one-component polyurethane foam

- HFCs used as blowing agents in manufacturing various kinds of polyurethane, extruded polystyrene and other foam products

- HFCs used as extinguishing agents in fixed fire fighting systems.

There has been no indication so far of HFC use as a solvent in Finland.

In the inventory the “semi-prompt” applications of SF6 consist of SF6 emitted from shoes. The contribution of other “semi-prompt” applications to Finnish emissions of SF6 was considered negligible and these sources were not included in the inventory.

Emissions from refrigeration and air conditioning are reported as a single figure for all of the refrigeration and air conditioning sub-categories (domestic, commercial, industrial, mobile, etc.). This follows from a project finished in 2002 that looked for ways of reducing the companies’ reporting burden. Based on this project’s results, it is clear that the companies do not have statistics available for the disaggregated reporting, or, that such reporting would entail an excessive reporting burden. It was also clear that a simplified survey form yielded better response activity.

The sub-category of emissions from aerosols also includes one-component polyurethane foam cans (OCF), a very aerosol-like product. These products have been treated as aerosols in the Finnish inventory. This practice predates the Good Practice Guidance. In the Good Practice Guidance, OCF is discussed together with other foam types, and the methodology is slightly different from that applied to aerosols. It has been decided not to change the practice of including OCF in the aerosols sub-source category, because this would require recalculation of both the aerosol and foam time series, and because recalculation would lead to insignificant differences in the total Finnish emissions of F-gases.

In reporting the results, HFC-23 from refrigeration and air conditioning, HFCs used as extinguishing agents, SF6 in shoes, SF6 in magnesium die-casting and HFCs, PFCs and SF6 used in semiconductor manufacturing have been aggregated into one source category. This is to protect the sources of the data (see Section 1.6 on confidentiality).

### Methodological issues

Summary of the methods used in preparing the F-gases inventory is presented in Table 4. Please note that detailed descriptions of the methodologies for each source category are described in a separate document (Oinonen 2003). The document describes among other things how the data for calculations were gathered and how the uncertainties were quantified.

TABLE 4. Summary of methods used in the F-gases inventory.

| Source category | Methods used | Reference | Notes |
| --- | --- | --- | --- |
| Magnesium die-casting | Direct reporting method, Tier 1a | Palmer 2000, p. 3.48; Anon. 1997, p. 2.47 | Tier 1b is not applicable to this category because all SF6 used is imported in bulk. Emissions from this source are not reported separately due to confidentiality. |
| Electrical equipment | Tier 3c (country-level mass-balance), Tier 1b | Olivier & Bakker 2000, p. 3.56; Anon. 1997, p. 2.48 and section 3.1 of Oinonen (2003) | Tier 1a estimates can not be calculated for this source because of lack of historical data. Tier 1b estimates have been calculated, however, based on survey and emissions data, cf. section 3.1 of Oinonen (2003). |
| Running shoes | Method for adiabatic property applications, Tier 1b | Olivier & Bakker, p. 3.65; Anon. 1997, p. 2.48 | Tier 1a is not applicable to this category because all SF6 used is imported not in bulk, but in products (i.e. shoes). Emissions from this source are not reported separately due to confidentiality. |
| Semiconductor manufacturing | Tier 1, Tier 1a | Bartos & Burton 2000, p. 3.72; Anon. 1997, p. 2.47 | Tier 1b is not applicable to this category because all gases used are imported in bulk. |
| Refrigeration and air conditioning | Top-down Tier 2, Tier 1a, Tier 1b | Forte et al. 2000, pp. 3.100–3.106; Anon. 1997, p. 2.47–2.50 | Tier 2 top-down method is used for all sources in this category, both stationary and mobile. Data is not collected for separate sub-categories because such statistics are either not available or the preparation of such statistics would entail a very high reporting burden on companies. HFC-23 emissions from this source are not reported separately due to confidentiality. |
| Aerosols and one component foam | Tier 2, Tier 1a, Tier 1b | Forte et al. 2000, p. 3.85; Anon. 1997, p. 2.47–2.50 | One component foam cans are treated as aerosols in this inventory, cf. section 2.3.6 of Oinonen (2003). MDIs are not reported separately from other aerosols due to confidentiality. |
| Foam blowing | Tier 2, Tier 1a, Tier 1b | Forte et al. 2000, p. 3.93 and section 2.3.7 of this report; Anon. 1997, p. 2.47–2.50 | Revised 1996 IPCC Guidelines and the Good Practice Guidance give little advice on how to model the effect of leakage from products and the annually installed new foam products on HFCs banked in foams. See section 2.3.7 of Oinonen (2003) on how these effects were modelled. |
| Fixed fire fighting systems | Tier 2, Tier 1a, Tier 1b | Forte et al. 2000, p. 3.115; Anon. 1997, p. 2.47–2.50 | Emissions from this source are not reported separately due to confidentiality. |

### Uncertainty and time series’ consistency

Both the inventory year and the trend uncertainties have been quantified for sub-source categories shown in Table 4, and also for overall F-gases emissions. The setting of priorities for future improvements based on the uncertainty analysis, as well as detailed discussion on major sources of uncertainties for F-gases, are described in Oinonen (2003).

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. The quality system is under development and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

Recalculations have been made due to addition of a new source, and also due to changes in the methodology. Corrections were also made as a response to the in-country review (see CRF table 8.C.b1).

Following recalculation, emissions of HFCs and PFCs generally decreased whereas SF6 emissions increased. Total F-gases estimates for 1990–2000 increased by 30% on average. In absolute terms the change ranged from 5 to 60 Gg CO2 equivalents. The differences between previous and latest estimates are depicted in Figure 10.A. Compared to the overall level of emissions, the maximum change following from recalculation was 0.07%

Recalculation affected estimates of annual emissions differently. The change was smaller in years 1998–2000, because the level of emissions during those years was considerably higher compared to the previous years. On the other hand, in 1995, the fairly large change in estimate combined with a low level of emissions, caused the estimate to double due to recalculation (Figure 10.B).

The time series for electrical equipment was recalculated to ensure consistency following a change in methods.

(A)	(B)

FIGURE 10. Differences between previous and latest submissions following from recalculation. Differences are shown for each inventory year both in absolute (A) and proportional (B) terms.

### Source-specific planned improvements

The uncertainty analysis indicated that the major source of uncertainty in the F-gases inventory results from the under-coverage of the survey used to gather information from the companies in the refrigeration and air conditioning sector, and the required corrections in the input parameters because of this. The uncertainty is expected to become considerably smaller in the inventory for 2002 as a list of contact details for companies became available in mid-2002 following from adoption of new national legislation.

## Other (CRF 2.G)

No emission sources are included in this sector.

# SOLVENT AND OTHER PRODUCT USE (CRF 3)

## Overview of the sector

The only direct greenhouse gas source identified in the solvent and other product use sector is the use of N2O in industrial, medical and other applications. In Finland, N2O is used in hospitals and by dentists to relieve pain and for detoxification.

The main sources of NMVOC emissions in Finland is paint application while printing industry and domestic solvent use also contribute to the emissions. Emissions from the solvent and other product use sector are shown in Figure 11.

FIGURE 11. Emissions in the solvent and other product use sector 1990–2001.

## Solvent and Other Product Use (CRF 3)

### Source category description

In Finland N2O is used in medical applications, as well as in some other specific applications, e.g. in industry. All used N2O is imported.

Emission sources for the NMVOC compounds are paint application, degreasing and dry cleaning, chemical products, manufacture and processing: pharmaceutical industry, leather industry, plastic industry, textile industry, rubber conversion, manufacture of paints etc., and other solvent use: printing industry, preservation of wood, use of pesticides, glass and mineral wool enduction, domestic solvent use, vehicles dewaxing.

### Methodological issues

The N2O emissions are calculated by Statistics Finland. Tier 2 calculation method is consistent with the IPCC Guidelines. For estimation of N2O emissions sales data is obtained from a few companies for the years 1990 and 1998. The emission estimation is based on assumption that all used N2O is emitted to atmosphere the same year it is used. From 1999 onwards the data is partly estimated due to non-response.

The NMVOC emissions for CRF tables 3.A, 3.B, 3.C and 3.D are calculated at the Finnish Environment Institute:

3.A	The emissions are based on emission calculation by the Association for Finnish Paint Industry and emissions data from the Regional Environment Centres’ VAHTI database.

3.B	The emission are based on import of pure solvents and products containing chlorinated organic solvents and amounts of the solvent waste processed in hazardous waste treatment plants. The emission factor is an expert estimation at the VTT Technical Research Centre of Finland.

3.C	The emissions are based on emission data in the Regional Environment Centres’ VAHTI database and on questionnaires to small and medium size companies in textile, plastic and leather industry. The emission factors are based on expert estimations in the industries and at the Finnish Environment Institute.

3.D	The emissions are based on emission data in the Regional Environment Centres’ VAHTI database and activity data from the Finnish Environment Institute’s Chemicals Divisions database. The emission factors are country specific based on expert estimation at the Finnish Environment Institute’s Chemicals Division. Emissions from domestic solvent use are based on emission calculation by the Finnish Cosmetics, Toiletry and Detergents Association.

### Uncertainty and time series’ consistency

The uncertainty of the N2O emissions in solvent and other product use was –34...+38% in 2001.

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. Quality systems are under development both at Statistics Finland and the Finnish Environment Institute and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

No recalculations were made to the previous inventories.

### Source-specific planned improvements

The quality of the activity data for N2O emissions inventory will be improved.

# AGRICULTURE (CRF 4)

## Overview of the sector

Finland reports agricultural emissions from the following sources: CH4 emissions from enteric fermentation in domestic livestock, N2O emissions from manure management, direct N2O emissions from agricultural soils, indirect N2O emissions from nitrogen used in agriculture and CH4 emissions from manure management. CO2 emissions from agricultural soils are also reported under agricultural emissions (Pipatti 2001).

Agricultural key sources in 2001 identified by the IPCC Tier 2 method were CH4 emissions from enteric fermentation, N2O emissions from agricultural soils and N2O emission from manure management (Monni & Syri 2003). In the year 1999 inventory analysis, also CO2 emissions from agricultural soils was identified as a key source (Aaltonen et. al. 2001) (this category was not handled in 2001 key source analysis).

Previous inventories in the agricultural sector have been made at the VTT Technical Research Centre of Finland (Pipatti 2001) with the assistance of the MTT Agrifood Research Finland (Kulmala & Esala 2000). From the year 2001 the inventory was made completely at the MTT Agrifood Research Finland and a systematic calculation system for the whole sector is currently under further development. Closer studies in some areas will be launched. Main focuses are the improved collecting of activity data, development of emission factors based on national data and enhancement of QA/QC measures.

Finnish agricultural greenhouse gas emissions in 2001 were 7.45 Tg CO2 equivalents in total. The summary of agricultural emissions by source and gas is illustrated in Table 5 and Figure 12. The main agricultural sources of greenhouse gases in 2001 were agricultural soils emitting 3.34 Tg CO2 equivalents nitrous oxide and 1.95 Tg CO2 equivalents carbon dioxide in total.

TABLE 5. Finnish agricultural greenhouse gas emissions by source and gas in 1990-2001.

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CH4 Tg CO2 equivalents |  |  |  |  |  |  |  |  |  |  |  |  |
| Enteric fermentation | 1.87 | 1.79 | 1.74 | 1.74 | 1.73 | 1.63 | 1.64 | 1.66 | 1.62 | 1.59 | 1.58 | 1.56 |
| Manure management | 0.20 | 0.19 | 0.18 | 0.18 | 0.19 | 0.22 | 0.22 | 0.22 | 0.22 | 0.21 | 0.21 | 0.20 |
| N2O |  |  |  |  |  |  |  |  |  |  |  |  |
| Agricultural soils | 4.27 | 3.98 | 3.64 | 3.69 | 3.68 | 3.75 | 3.61 | 3.52 | 3.44 | 3.33 | 3.38 | 3.34 |
| Manure management | 0.55 | 0.53 | 0.52 | 0.52 | 0.52 | 0.47 | 0.46 | 0.46 | 0.44 | 0.42 | 0.41 | 0.40 |
| CO2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Organic soils | 1.62 | 1.60 | 1.58 | 1.55 | 1.53 | 1.38 | 1.36 | 1.34 | 1.31 | 1.29 | 1.27 | 1.25 |
| Mineral soils | 0.97 | 0.78 | 0.41 | 0.22 | 0.09 | –0.03 | 0.05 | 0.25 | 0.27 | 0.29 | 0.34 | 0.31 |
| Liming | 0.62 | 0.43 | 0.33 | 0.45 | 0.44 | 0.37 | 0.41 | 0.47 | 0.44 | 0.43 | 0.42 | 0.39 |
| Total | 10.10 | 9.29 | 8.39 | 8.36 | 8.18 | 7.79 | 7.75 | 7.92 | 7.74 | 7.56 | 7.61 | 7.45 |

FIGURE 12. Finnish greenhouse gas emissions in agriculture in 1990–2001 by main source category and gas.

Greenhouse gas emissions from agriculture have declined over the period of 1990–2001. One reason for this is Finland’s membership in the EU that resulted in changes in the economic structure followed by an increase in the average farm size and a decrease in the number of small farms (Pipatti 2001). Those changes caused also a decrease in the livestock numbers except for the number of horses that has increased in the recent years. The reduced use of nitrogen fertilisers and improved manure management resulting from the measures taken by the farmers as a part of an agro-environmental program aiming to minimise nutrient loading to water courses has also decreased the emissions.

## Enteric Fermentation (CRF 4.A)

### Source category description

The emission sources cover domestic livestock. Finland reports emissions from cattle (including dairy cows, mother cows, heifers, bulls and calves), swine, horses, goats, sheep and reindeer.

### Methodological issues

Calculation of emissions is based on methods described in the Revised 1996 IPCC Guidelines (IPCC 1996a) and the IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories (IPCC 2000). CH4 emissions from enteric fermentation of cattle have been estimated using the Tier 2 methodology. The average daily feed intake has been calculated from data on animal weight, daily weight gain etc. as in previous inventories. These input data have been obtained from agricultural experts. The method and the activity data acquisition are described in more detail in the references (Pipatti 2001; Pipatti et al. 2000; Pipatti 1997; Pipatti et al. 1996; Savolainen et al. 1996). The method is consistent with the IPCC Guidelines. For the other animal types the Tier 1 method and the default values given in the IPCC Guidelines have been used. CH4 emissions from enteric fermentation of reindeer have been calculated on basis of Finnish literature (Nieminen et al. 1998).

Activity data have been collected from annual agricultural statistics (animal numbers, milk production for dairy cows) and surveys (monthly and annual publications by the Information Centre of the Ministry of Agriculture and Forestry). Some information has been received from the “Matilda” database of the Ministry of Agriculture and Forestry, and agricultural experts. Summary of the emission factors used in the 2001 inventory is presented in Table 6.

TABLE 6. Methods and emission factors used in calculating methane emissions from enteric fermentation.

| Animal category | Method | #### Type of EF | #### EF (kg CH4/animal/year) |
| --- | --- | --- | --- |
| Dairy cows | IPCC Tier 2 | National | 114.00 |
| Mother cows | IPCC Tier 2 | National | 66.05 |
| Bulls | IPCC Tier 2 | National | 60.15 |
| Heifers | IPCC Tier 2 | National | 61.24 |
| Calves | IPCC Tier 2 | National | 25.08 |
| Swine | IPCC Tier 1 | IPCC default | 1.50 |
| Sheep | IPCC Tier 1 | IPCC default | 8.00 |
| Goats | IPCC Tier 1 | IPCC default | 5.00 |
| Horses | IPCC Tier 1 | IPCC default | 18.00 |
| Reindeer | IPCC Tier 1 | National | 8.85 |

### Uncertainty and time series’ consistency

The uncertainty in enteric fermentation of domestic animals was 30% in the year 2001. The uncertainty estimates of activity data are based on national information, and the uncertainties of emission factors are mainly the IPCC default uncertainties. The total uncertainty in the agriculture sector was –37...+47%.

Part of the input data is not found in the agricultural statistics or surveys and is therefore based on expert knowledge. As the sources of data do not vary between years the time series can be considered consistent. Numerical estimates of uncertainty for all emission sources will be presented in Monni & Syri (2003).

The emission factor used for reindeer is national and was calculated on basis of literature and comments of an expert of reindeer nutrition (Nieminen et.al. 1998; M. Nieminen, pers. comm.). The value is very preliminary and needs to be studied further. There has been discussion with Sweden and Norway concerning the reindeer emission factor, and co-operation in developing a common emission factor in the future is recommended by all parties.

### Source-specific QA/QC and verification

Calculation of greenhouse gas emissions in the agricultural sector is currently carried out at the MTT Agrifood Research Finland. The QA/QC in the agricultural sector is still under development but some measures have been taken. Collecting of the activity data and documentation of the method have been improved. Some activity data has been compared to previously reported and a few corrections have been made. The importance of careful documentation of all the activity data sources, calculation methods and changes compared to the previous inventory has been noticed and will be further developed in the future.

### Source-specific recalculations including changes made in response to the review process

The emissions from enteric fermentation have been calculated using the same methodology as in the 2000 inventory. Recalculation has been made because the emissions from reindeer have been added to the whole time series.

### Source-specific planned improvements

The time series concerning the activity data has been improved but some of the values may still need fine-tuning and improvement. An idea of changing the method so that it would be based on the feed consumption of cattle instead of estimating this indirectly from the data on animal weight, daily weight gain etc. has been put forward by the VTT Technical Research Centre of Finland and will be further examined at the MTT Agrifood Research Finland.

## Manure Management (CRF 4.B)

### Source category description

The emission sources cover management of manure from domestic livestock. Finland reports emissions from cattle (including dairy cows, mother cows, heifers, bulls and calves), swine, horses, goats, sheep and poultry.

### Methodological issues

Calculation of methane emissions is based on methods described in the Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories (IPCC 1996a) and the IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories (Penman et al. 2000). Tier 2 methodology for all animal categories has been used. Calculation of the emissions is based on annual agricultural statistics (animal numbers, milk production for dairy cows) and surveys (MKL 1993; Seppänen & Matinlassi 1998). Additional information has been received from agricultural experts. VS (volatile solid excretion) values for cattle have been calculated from feed intake values obtained from the calculations on emissions from enteric fermentation (Pipatti 2001). Other input data used are the IPCC default values. The method and activity data acquisition is described in more detail in the references (Pipatti 2001; Pipatti et al. 2000; Pipatti 1997; Pipatti et al. 1996; Pipatti 1994). No changes in the existing methods have been made in the 2001 inventory. The emission factors used in calculating the methane emissions from manure management are presented in Table 7.

TABLE 7. Emission factors used in calculating methane emissions from manure management.

| Animal category | EF (kg CH4/head/year) | Type of EF |
| --- | --- | --- |
| Dairy cows | 7.92 | National |
| Mother cows | 1.41 | National |
| Bulls | 4.37 | National |
| Heifers | 3.07 | National |
| Calves | 1.31 | National |
| Swine | 3.37 | National |
| Sheep | 0.19 | National |
| Goats | 1.12 | National |
| Horses | 1.42 | National |
| Poultry | 0.09 | National |

Calculation of nitrous oxide emissions from manure management is also based on the IPCC methods (IPCC 1996a; Penman et al. 2000) and is described in detail in Pipatti (2001). Annual average N excretion per head of animal is national data (Grönroos et al.1998) as well as the ratio of liquid/slurry to the total manure managed (Seppänen & Matinlassi 1998). IPCC default emission factors were used for each manure management system (Pipatti 2001).

### Uncertainty and time series’ consistency

The magnitude of methane emissions from manure management depends on the climate, temperature, animal type and feeding as well as the type of the manure management system used. IPCC default and also some national parameters have been used in calculating the emissions. More research will be needed in the future in developing national parameters. As the sources of data do not vary between years the time series can be considered consistent.

The magnitude of nitrous oxide emissions from manure management depend on the amount of nitrogen excreted in manure per animal as well as the proportion of manure managed in each manure management system. The values are based on national surveys and need to be updated regularly. IPCC default emission factors are used at the moment but field measurements are needed to ensure their suitability to Finnish climate conditions. As the sources of data do not vary between years the time series can be considered consistent.

The uncertainty of CH4 emissions from manure management was estimated at around 17%, and in N2O emissions –82...+35%. The negatively skewed uncertainty distribution results from the assumption, that some of the calculation parameters used were too high.

### Source-specific QA/QC and verification

Calculation of greenhouse gas emissions in the agricultural sector is currently carried out at the MTT Agrifood Research Finland. The QA/QC in the agricultural sector is still under development but some measures have been taken. Collecting of the activity data and documentation of the method have been improved. Some activity data has been compared to previously reported and a few corrections have been made. The importance of careful documentation of all the activity data sources, calculation methods and changes compared to previous inventory has been noticed and will be further developed in the future.

### Source-specific recalculations including changes made in response to the review process

There is a little change in the time series of nitrous oxide emissions from manure management in 1999–2001 because of the correction in the proportion of dairy cow manure that is applied in pasture. The effect for the total emissions is negligible.

### Source-specific planned improvements

Annual or periodic data collection on e.g. the manure management systems need to be developed. Some of the input parameters in the calculation may need fine-tuning.

## Rice Cultivation (CRF 4.C)

Rice cultivation does not occur in Finland.

## Agricultural Soils (CRF 4.D)

### Source category description

Carbon dioxide emissions from organic soils and liming are included in the inventory together with estimates of the changes in carbon stocks in mineral soils due to land-use change. The emissions are reported in category 4.D as they are caused by agricultural activities. They are also very analogous to the N2O emissions from agricultural soils and they also share some activity data used in calculations (Pipatti 2001).

Direct N2O emissions from nitrogen in mineral N fertilisers, animal manure, biological nitrogen fixation, crop residues, cultivation of Histosols and sewage sludge are included in the inventory. Nitrous oxide emissions from sewage sludge are reported as “other source” in the CRF tables. The emissions from nitrogen excreted to pasture range and paddocks by animals are reported under “animal production” in CRF tables.

Indirect emissions from atmospheric deposition of NH4 and NOx as well as from leaching and run-off of the applied or deposited nitrogen are included in the inventory.

### Methodological issues

Calculation of emissions is consistent with the IPCC Guidelines and is based on activity data obtained from the annual agricultural statistics, publications, databases and agricultural experts. Both the IPCC default and the national values for emission factors and other parameters have been used. Activity data sources are presented in Table 8. No changes in the methodology compared to the previous inventories have been made.

TABLE 8. Activity data sources for calculating nitrous oxide and carbon dioxide emissions from agricultural soils.

| Activity data | Source of information |
| --- | --- |
| Crop statistics | The Information Centre of the Ministry of Agriculture and Forestry |
| Data of sludge spreading | VAHTI database (Regional Environment Centres) |
| N excretion by animal types | Rural Advisory Centres, e.g. Grönroos et al. 1998 |
| Data of animal waste management systems | Rural Advisory Centres, MKL 1993; Seppänen & Matinlassi 1998 |
| Energy model for ammonia emission estimate | VTT Technical Research Centre of Finland, Savolainen et al. 1996, and agricultural experts |
| Amount of lime used in agriculture | Finnish Liming Society |

#### N2O emissions from soils

Nitrous oxide emissions from agricultural soils are calculated using the emission factors presented in Table 9. Direct emissions from sewage sludge are reported under Other source in the CRF tables and direct emissions from N-excretion on pasture range and paddock are reported under Animal production in the CRF tables. Other parameters used for calculating nitrous oxide emissions from agricultural soils are presented in Table 10.

TABLE 9. Emission sources and emission factors used for calculating nitrous oxide emissions from agricultural soils.

| Emission source | Emission factor | Reference |
| --- | --- | --- |
| Direct soil emissions |  |  |  |  |
| Synthetic fertilisers | 0.0125 kg N2O-N/kg N | Penman et al. 2000, p. 4.60, Table 4.17 |
| Animal wastes applied to soils | 0.0125 kg N2O-N/kg N | Penman et al. 2000, p. 4.60, Table 4.17 |
| N-fixing crops | 0.0125 kg N2O-N/kg dry biomass | Penman et al. 2000, p. 4.60, Table 4.17 |
| Crop residue | N2O-N/kg dry biomass | Penman et al. 2000, p. 4.60, Table 4.17 |
| Cultivation of Histosols | 8 kg N2O-N/ha/a | Penman et al. 2000, p. 4.60, Table 4.17, Klemedtsson et al. 1999 |
| Indirect soil emissions |  |  |  |  |
| Atmospheric deposition | 0.01 kg N2O-N/kg NH4-N & NOx-N deposited | Penman et al. 2000, p. 4.73, table 4.18 |
| Nitrogen leaching and run-off | 0.025 kg N2O-N/kg N/a | Penman et al. 2000, p. 4.73, table 4.18 |
| Animal production |  |  |  |  |
| N-excretion on pasture range and paddock | 0.020 kg N2O-N/kg N/a | IPCC 1996a, p. 4.97 |
| Other sources |  |  |
| Sludge spreading | 0.0125 kg N2O-N/kg N load | IPCC 1996b, p. 4.37, EF1 |

TABLE 10. Other parameters used for calculating nitrous oxide emissions from agricultural soils.

| Parameter | Abbreviation | Value | Reference |
| --- | --- | --- | --- |
| Fraction of N input that is lost through leaching or runoff | FracLEACH | 0.15 | Pipatti 2001; Pipatti et al. 2000; Rekolainen et al.1995 |
| Fraction of N input that volatilises as NH3 and NOx from synthetic fertilisers. | FracGASF | 0.006 | Pipatti 2001; Keränen & Niskanen 1987 |
| Fraction of manure N input that volatilises as NH3 and NOx | FracGASM | 0.31 | Energy model for ammonia emission estimate (VTT Technical Research Centre of Finland), Savolainen et al.1996 |

#### CO2 emissions from soils

Calculation of CO2 emissions from mineral soils is based on changes in the carbon stocks resulting from changes in land use in the period of 20 years. The area of agricultural soils is calculated from the annual agricultural statistics published by the Ministry of Agriculture and Forestry. Determination of the area of organic soils is based on analyses of soil testing results (Finnish Soil Analysis Service) made at the MTT Agrifood Research Finland. (Myllys 2002, pers. comm.). Organic soils have been divided into peat soils and other organic soils according to their content of soil organic matter. Emissions from peat soils and other organic soils have been calculated using both the IPCC default and national emission factors (Table 11). The emissions from liming have been calculated using the IPCC method and data from the Finnish Liming Association.

TABLE 11. Emission factors used for calculating CO2 emissions from organic soils.

| Emission source | EF (Mg C/ha/a) | Reference |
| --- | --- | --- |
| Peat soils Soil organic matter content > 40% (w) |  |  |
| Pasture | 2 | National by Berglund 1989 |
| Upland crops | 4 | National by Nykänen et al. 1995 |
| Other organic soils Soil organic matter content 20–40% (w) |  |  |
| Pasture | 0.5 | National by Berglund 1989 |
| Upland crops | 1 | IPCC default, IPCC 1996b, p. 5.29 |

### Uncertainty and time series’ consistency

The new uncertainty analysis (Monni & Syri 2003) does not include CO2 from agricultural soils. According to the earlier uncertainty analysis (Pipatti 2001), the uncertainty in N2O emissions from agricultural soils was –58...+73%. Uncertainties in the emission estimates from agricultural soils are high because the area of organic soils used in agriculture is not very well known in Finland. Also the area of mineral soils and the emission factors for organic soils need to be updated.

FracLEACH for which the use of the default was estimated to lead to an overestimation of the emissions by a factor of 2; support for the value used can be found e.g. in Rekolainen et al. (1995).

### Source-specific QA/QC and verification

Calculation of greenhouse gas emissions in the agricultural sector is currently carried out at the MTT Agrifood Research Finland. The QA/QC in the agricultural sector is still under development but some measures have been taken. Collecting of the activity data and documentation of the method have been improved. Some activity data have been compared to previously reported and a few corrections have been made. The importance of careful documentation of all the activity data sources, calculation methods and changes compared to previous inventory has been noticed and will be further developed in the future.

### Source-specific recalculations including changes made in response to the review process

There has been a little change in the time series because of recalculation. Some corrections in the carbon stocks in mineral soils have been done based on new calculations. The nitrogen fraction of wheat residue has been corrected to correspond to the IPCC Good Practice Guidance for the whole time series. Also, the amount of sludge spread has been revised in 1998 and 1999. Carbon dioxide emissions from liming have been revised for the whole time series based on the new calculations.

### Source-specific planned improvements

The suitability of the IPCC default emission factors for the Finnish conditions is questioned as the climatic conditions and agricultural practices differ very much from those from which the default values have been derived. Country specific emission factors are used in the estimates to a certain extent but more research is still needed in the future. Further development of national emission factors will continue at the MTT Agrifood Research Finland. A project is planned which will give new information about changes in the soil carbon stocks in Finland. The project will also yield new information about the area of organic soils in Finland.

The current guidelines are not considered logically or scientifically correct in all aspects considering the estimation of N2O emissions. Also the development of the IPCC method in determining land use classifications for organic soils needs further examination.

Crop residue nitrogen of certain vegetable and fruit crops will be included into the calculations in the future.

## Burning of Savannas (CRF 4.E)

Burning of savannas does not occur in Finland.

## Field Burning of Agricultural Residues (CRF 4.F)

Field burning of agricultural residues is taking place in Finland only occasionally on small scale and the emissions from this source are estimated to be negligible.

# LAND-USE CHANGE AND FORESTRY (CRF 5)

## Overview of the sector

The CO2 balance of the trees from 1990 to 2001 is presented in Table 12 and Figure 13. The annual increment of trees has increased almost steadily wherefore the CO2 uptake has also increased. The total drain of trees is very much affected by commercial fellings and the global market situation. The demand of timber products was low in the beginning of 1990’s wherefore fellings were also at low level and the CO2 sink of trees high. The fellings in the end of 1990’s were exceptionally high. The total drain of the years 1994 and 1995 are nearer to a long-term average. Strong fluctuation in the CO2 sink in the 1990’s is very much affected by these facts.

TABLE 12. Carbon dioxide uptake and release of trees 1990–2001.

| Year | Tg CO2 |  |  |
| --- | --- | --- | --- |
|  | Uptake | Release | Balance |
| 1990 | 95.9 | 72.1 | 23.8 |
| 1991 | 96.8 | 58.6 | 38.2 |
| 1992 | 98.6 | 66.7 | 31.9 |
| 1993 | 99.5 | 70.4 | 29.1 |
| 1994 | 97.8 | 80.6 | 17.3 |
| 1995 | 97.8 | 83.1 | 14.7 |
| 1996 | 98.0 | 77.0 | 21.0 |
| 1997 | 98.6 | 85.9 | 12.6 |
| 1998 | 100.1 | 90.4 | 9.7 |
| 1999 | 101.3 | 90.4 | 10.8 |
| 2000 | 103.1 | 91.1 | 12.0 |
| 2001 | 105.1 | 88.2 | 16.9 |

FIGURE 13. Carbon dioxide uptake and release of trees in Finland 1990–2001.

## Changes in Forest and Other Woody Biomass Stocks (CRF 5.A)

### Source category description

Emissions are supposed to be caused by tree fellings and unrecovered natural losses, uptakes are caused by tree growth.

### Methodological issues

The invention of the land use change and forestry sector is carried out by the Finnish Forest Research Institute (Metla). A national method is applied. It gives more accurate figures than the IPCC methodology.

Cutting statistics are collected by the Finnish Forest Research Institute, volume increment statistics are based on the Finnish National Forest Inventory (the Finnish Forest Research Institute).

The total annual drain estimates of forests are based on the statistics of cutting removals reported by the forest industry companies and collected by the Finnish Forest Research Institute, the estimates of household use of timber is based on enquires, the estimate of cutting waste is based on an extensive field study by the Finnish Forest Research Institute. The volume of natural losses is estimated in a study by the Finnish National Forest Inventory (Finnish Statistical Yearbook of Forestry 1999, Finnish Forest Research Institute).

The volume increment of the growing stock of trees is estimated using field measurements on sample plots of the Finnish National Forest Inventory (FNFI). The increment figures concern increment of the tree stem volume. An average increment of five years preceding the measurement time is applied. The FNFI progresses by regions and thus the data for the whole country comes from different years for different parts of the country (Tomppo 1999 and 2000b; Tomppo et al. 1997 and 1998). Conversion factors are applied for converting the tree stem volume for the whole tree biomass and carbon content (Karjalainen & Kellomäki 1996).

### Uncertainty and time series’ consistency

Total drain figures (corresponding emissions of CO2) are estimated annually and are considered to be very accurate. Total increment figures (corresponding uptake of CO2) are updated annually but the figures for different parts of the country come from different years. The averages of increments of five years preceding the measurement year are applied. This is a commonly used practice in forest inventories. The reliability figures can be assessed by means of statistical methods (Tomppo 1999 and 2000a).

### Source-specific QA/QC and verification

New biomass allocation models with reliability estimates are under development and will be in use in 2006. The reliability figures both for sinks and sources of growing stock of trees will be developed and presented using that information.

### Source-specific recalculations including changes made in response to the review process

The figures are updated annually.

### Source-specific planned improvements

Either the IPCC practice of the use of annual figures must be changed or a method to produce annual increment figures must be developed. This is very much a question of measurement costs. One should also note that the real annual variation of the increment of trees is of a magnitude plus minus 20%. It is caused by climatic variation and biological cycles of trees. The use of five years instead of one reduces this variation which is not essential for the carbon balance budgeting. The new carbon allocation models with error estimates are under development and will be ready by 2005.

## Forest and Grassland Conversion (CRF 5.B)

### Source category description

Emission sources for the category cover deforestation, afforestation and reforestation. The country specific calculation method is consistent with the IPCC Guidelines.

### Methodological issues

Activity data figures for calculation of the emissions are indirectly included in the forest tree biomass change figures given by the Finnish Forest Research Institute1 and the Finnish National Forest Inventory (the Finnish Forest Research Institute). The calculation method and applied emission factors are as in Changes in Forest and Other Woody Biomass Stocks.

### Uncertainty and time series’ consistency

The error estimates for the increment of the tree stem volume are available. The error estimates for the total tree biomass, biomass of increment and biomass of drain are under development. The consistency over time is good due to the fact that the same method has been used.

### Source-specific QA/QC and verification

See Section 7.2.4.

### Source-specific recalculations including changes made in response to the review process

No recalculations are needed.

### Source-specific planned improvements

New carbon allocation models of trees with error estimates are under development. The inventory cycle is also under consideration.

## Abandonment of Managed Lands (CRF 5.C)

### Source category description

Emission sources cover afforestation and reforestation.

### Methodological issues

The activity data are indirectly included in the forest tree biomass change figures given by the Finnish Forest Research Institute1 and the Finnish National Forest Inventory (the Finnish Forest Research Institute). For the calculation method and emission factors, see Changes in Forest and Other Woody Biomass Stocks. The country specific method is consistent with the IPCC Guidelines.

### Uncertainty and time series’ consistency

See Section 7.3.3.

### Source-specific QA/QC and verification

See Section 7.2.4.

### Source-specific recalculations including changes made in response to the review process

See Section 7.3.4.

### Source-specific planned improvements

See Section 7.3.5.

## CO2 Emissions and Removals from Soil (CRF 5.D)

### Source category description

Emission sources in this category include forest soil; for agricultural soils, see Section 6.5.

Emissions or removals from forest soils are not reported at the moment. The changes in the carbon content of the forest soil are slow. Many factors, both human induced and not human induced, affect the changes. Several studies have been reviewed but a unique method covering both mineral soils and peatland soils does not exist.

### Methodological issues

A method to estimate changes in the carbon content of forest soil is under development at the Finnish Forest Research Institute. The method is ready and the estimates are available by the end of year 2005.

### Uncertainty and time series’ consistency

Annual changes in the carbon content of forest soil are very small and their estimates may be unreliable. The estimates of the annual changes are not necessarily due to the slow processes, either.

### Source-specific QA/QC and verification

See Section 7.2.4.

### Source-specific recalculations including changes made in response to the review process

No recalculations are carried out.

### Source-specific planned improvements

See Section 7.5.1.

## Other (CRF 5.E)

There are no emission sources to be included in this category.

# WASTE (CRF 6)

## Overview of the sector

Solid waste disposal on land (landfills and dumps) causes relatively large CH4 emissions in Finland while emissions from wastewater treatment are smaller. CH4 emissions from landfills are the most important greenhouse gas emissions in the waste sector. During the 1990’s these emissions have decreased by more than 20%. The decrease has been mainly due to the preparation and implementation of the new waste law in Finland in 1994. At the beginning of the 1990’s, around 80% of the generated municipal waste was taken to solid waste disposal sites (landfills). After the implementation of the new waste law, minimisation of waste generation, recycling and reuse of waste material and alternative treatment methods to landfills have been endorsed. Similar developments have occurred in the treatment of industrial waste, and municipal and industrial sludges. CH4 emissions from solid waste disposal on land have been identified as a key source in Finland. Both solid waste disposal on land and wastewater handling have been recalculated. Emission trends in the waste sector in CO2 equivalents are presented in Figure 14.

FIGURE 14. Finnish emissions trends in the waste sector 1990–2001.

## Solid Waste Disposal on Land (CRF 6.A)

### Source category description

The emission sources include solid waste disposal sites including solid municipal, industrial, construction and demolition wastes and municipal (domestic) and industrial sludges.

### Methodological issues

CH4 emissions from solid waste disposal on land have been calculated at the Finnish Environment Institute using the Tier 2 method given in the IPCC Good Practice Guidance and in the Revised (1996) Guidelines (first order decay method) and using national and IPCC emission factors.

Activity data for landfilled waste is obtained from the Finnish Environment Institute using both the VAHTI database information and information from the Register of Landfill Sites. Also data from publications of Statistics Finland and the Ministry of the Environment are used. Data on landfill gas recovery is obtained from the Finnish Biogas Plant Register.

The NMVOC emissions are based on expert estimation (Arnold et al. 1998).

Emission factors are either country specific or IPCC default values as follows:

D/CS	Methane correction factor (MCF) between 0.98 and 1

CS	Degradable organic carbon in municipal solid waste (DOC) 0.20 (based on waste composition in 1990)

CS	Expert knowledge. Fraction of DOC dissimilated, DOCf = 0.50 	(mean temperature in landfills 10–15ºC)

D	Fraction of methane in landfill gas, F = 0.5

CS	Expert knowledge: Oxidation factor (OX) 0.1

D/CS	k1 = 0.2 (Food waste in MSW and sludges)

k2 = 0.03 (wood waste in MSW and in construction and demolition waste, paper waste containing lignin in MSW)

k3 = 0.05 (industrial solid waste and other fractions of MSW than above)

Equation 5.1 in the Good Practice Guidance has been used so that the term MCF(t) has substituted for the term MCF(x) in the calculation of L0(x). It has been thought that the situation in year t defines the MCF to be used for the emissions caused by waste amounts landfilled in the previous years also. In Finland this is valid for closed landfills also (which have been unmanaged when used) because all the closed landfills have been covered at present.

Estimations of waste amounts before 1990 are presented in Tuhkanen (2002).

### Uncertainty and time series’ consistency

The uncertainty in Solid Waste Disposal on Land is estimated by applying uncertainty distributions into the parameters of FOD (First Order Decay) model. Most of the uncertainty estimates are based on expert judgement, IPCC default values, and some also on measurement data. Total uncertainty was –29...+31%. The most important factors affecting the uncertainty are the fraction of methane in landfill gas and the fraction of degradable organic carbon dissimilated, as well as the fraction of organic carbon in waste.

The uncertainties in the estimates on CH4 emissions from waste are large. The data on composition and amount of waste landfilled is still often based on rough estimates and, when looking at the past composition and amount of waste landfilled, the lack of data is even greater. Statistics on both municipal and industrial waste management are currently improving, and future emissions will at least to some extent be based on more reliable data. The improved statistics are assumed to improve the estimation accuracy of the emissions for the years from 1997 onwards.

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. The quality system is under development at the Finnish Environment Institute and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

The whole time series has been recalculated. The calculation method has been changed from the mass balance method to the FOD method.

### Source-specific planned improvements

The activity data before 1990 for waste as well as the waste composition data for MSW may be reviewed if better information is available.

## Wastewater Handling (CRF 6.B)

### Source category description

The emission sources cover municipal (domestic) and industrial wastewater handling plants and uncollected domestic waste waters for CH4 emissions.

N2O emissions are generated from nitrogen input of fish farming as well as domestic and industrial wastewaters into waterways.

NMVOC emissions from wastewater handling are included.

### Methodological issues

A national methodology that corresponds to the methodology given in the Revised (1996) Guidelines is used in estimation of the CH4 emissions. The emissions from municipal wastewater treatment are based on the BOD7 load of the wastewaters. The emissions from industrial wastewater treatment are based on the COD load. These DC values of wastewaters with shared methane conversion factors have been used for both wastewater and sludge handling. The emission estimate is very uncertain as parameters are based on expert opinions.

The IPCC Guidelines have only two default values for the methane conversion completely aerobic or anaerobic. There are no plant specific measurements for the degradable organic component of the sludge in Finland. Especially for domestic wastewater there are good measurement results for DC of wastewaters in Finland. DC values of wastewaters with shared methane conversion factors have been used for both wastewater and sludge handling. The estimated methane conversion factors for collected wastewater handling systems (industrial and domestic) are low in Finland because the handling systems included in the inventory are either aerobic or anaerobic with complete methane recovery. The emission factors mainly illustrate exceptional operation conditions. For uncollected domestic waste waters the Check method with the default parameters (IPCC Good Practice Guidance) is used.

Activity data is based on

- municipal (domestic) wastewaters and BOD (BOD7) value of the wastewater from the Finnish Environment Institute using both VAHTI database and the Water and Sewage Works Register and population (Check method);

- industrial wastewaters, COD value of the wastewater from the Finnish Environment Institute using both VAHTI database and the Register for industrial Water Pollution Control.

Estimates are based on expert knowledge.

Emission factors for municipal (domestic) wastewaters are IPCC default factors for maximum methane producing capacity Bo = 0.625 (= 2.5 · 0.25) kg CH4/kg BOD and country specific expert knowledge for the methane conversion factor MCF = 0.01. For the industrial wastewaters the emission factor is the IPCC default for maximum methane producing capacity Bo = 0.25 kg CH4/kg COD and country specific expert knowledge for the methane conversion factor MCF = 0.005.

In the Revised (1996) Guidelines a methodology to calculate the N2O emissions from sewage is given in the Agriculture sector. The methodology is very rough and the N input into waterways is based on population data. In Finland, the N input from fish farming and from municipal and industrial wastewaters into the waterways is collected into the VAHTI database.

The NMVOC emissions are based on expert estimation (Arnold et al.1998).

### Uncertainty and time series’ consistency

The uncertainty in wastewater sector was very high, namely –55...+118%. The total uncertainty was dominated by the uncertainty in the N2O emission factor.

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. The quality system is under development at the Finnish Environment Institute and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

There were no recalculations to the previous years.

### Source-specific planned improvements

Wastewater treatment causes also N2O emissions, although their importance is minor. In emission inventories the emissions have been estimated to be negligible. This assumption should be confirmed and the international development of the estimation methods for these emissions should be followed.

## Waste Incineration (CRF 6.C)

Emissions of greenhouse gases CO2, N2O and CH4 from Waste Incineration are reported in the energy sector.

## Other (CRF 6.D)

There are no emission sources to be included in this category.

# other (CRF 7)

## Overview of the sector

The sector includes emissions from feedstock and non-energy use of fuels.

## Feedstock and Non-energy Use of Fuels (CRF 7)

### Source category description

This source covers the CO2 emissions from non-energy use of oil products and natural gas. At the moment there is not enough data available to identify the processes and actual source categories.

### Methodological issues

The calculation method is the IPCC default method. The emissions are estimated at Statistics Finland based on activity data from the energy statistics and IPCC default emission factors. The emissions are calculated assuming that all non-stored carbon is combusted. This assumption may require more studies.

### Uncertainty and time series’ consistency

The uncertainty in non-energy use of fuels was estimated at 50% in 2001 based on expert knowledge on activity data and emission factor uncertainties.

### Source-specific QA/QC and verification

Normal statistical quality checkings related to assessment of magnitude and trends have been carried out. The quality system is under development at Statistics Finland and will be implied in the inventory of the year 2002 emissions.

At present, no verification has been carried out for the specific source-sector emissions.

### Source-specific recalculations including changes made in response to the review process

No recalculation has been carried out.

### Source-specific planned improvements

The fractions of carbon stored (and carbon released) need to be checked. There is a possibility of double counting in the present inventory. The whole category will be checked and moved to the Industrial Processes in the future inventories.

# RECALCULATIONS AND IMPROVEMENTS

## Explanations and justifications for recalculations

CRF 2.F		Recalculations have been made due to addition of a new source, and also due to changes in the methodology. Corrections were also made as a response to the in-country review (see CRF table 8.C.b1).

CRF 4	Data checking as a part of QA/QC measures resulted in few corrections in activity data and changes in nitrous oxide and carbon dioxide emissions. The new emission source, methane emissions from enteric fermentation of reindeer, was also added.

CRF 6.A		The whole time series has been recalculated. The calculation method has been changed from the mass balance method to the FOD method.

## Implications for emission levels

CRF 2.F	Following recalculation, emissions of HFCs and PFCs generally decreased whereas SF6 emissions increased. Total F-gases estimates for 1990–2000 increased by 30% on average. In absolute terms the change ranged from 5 to 60 Gg CO2 equivalents. Compared to the overall level of emissions, the maximum change following from recalculation was 0.07%.

CRF 4	Methane emissions from the enteric fermentation of reindeer were included in the time series. The emissions represents approximately 2% of the total agricultural emissions as CO2 equivalents.

CRF 6.A	Emission level in the waste sector is now 50% higher due to recalculations in 2001.

## Implications for emission trends, including time series consistency

CRF 2.F	Recalculation affected estimates of annual emissions differently. The change was smaller in years 1998–2000, because the level of emissions during those years was considerably higher compared to the previous years. On the other hand, in 1995, the fairly large change in estimate combined with a low level of emissions, caused the estimate to double due to recalculation.

The time series for electrical equipment was recalculated to ensure consistency following a change in methods.

CRF 6.A	Emission trend in the waste sector decreased only 20% (in the previous submission 50%) due to recalculations (FOD method).

## Recalculations, including response to the review process, and planned improvements to the inventory

### Recalculations

The following recalculations were made to the inventory of the year 2001 emissions:

CRF 2.F	The entire time series of emissions from 1990 to 2000 were recalculated for electrical equipment and semiconductor manufacturing. Also, a new source, SF6 emissions from shoes, was added (see CRF Table 8(b)).

CRF 4	CH4 emissions from enteric fermentation have been recalculated for the whole time series because a new source, the emissions from reindeer, was added. The time series of N2O emissions from manure management as well as agricultural soils have been recalculated because of the corrections made in the activity data. The time series of CO2 emissions was recalculated because of the correction made in the activity data.

CRF 5	The figures are updated annually.

CRF 6.A	The whole time series has been recalculated due to implementation of new method (FOD) and due to completing emissions from wastewater handling with uncollected wastewaters.

### Improvements in response to the review process

CRF 1 and 2.A-G

The number of staff has been increased (from 1 to 2) as proposed in the IDR report. Energy balance sheets and emission factor tables have been included in this NIR. The method of compiling reference approach has been revised (secondary fuels in more disaggregated level, oxidation factors and stock changes checked).

Implementation of a new database system has been started and the work is still going on. Recalculations of the time series will be done with the new system.

National classification of fuels has been revised to allow comparison and linking of separate data sources and to reduce uncertainties in the emission factors.

The production processes of the national energy statistics and GHG inventory have been linked more closely. The energy consumption time series have been revised. New results will be included in the following inventories. Some studies on the non-energy use of fuels have been started.

There are some additional issues identified by the ERT which have not yet been started, but will be done in the future inventories.

### Planned improvements

The structure and contents of this report on the Finnish greenhouse gas emission inventory will be further developed and completed to meet the new UNFCCC reporting guidelines accepted by the COP8 in New Delhi.

#### CRF 1.A	See Section 3.2.6.

CRF 1.B	Calculation of the emissions from 1.B.1 Fugitive emissions from solid fuels will be made for the whole period since 1990 when improved data on areas of peatland and the emission factors is available.

The results of the NMVOC calculation model of the Finnish Environment Institute have to be updated into the ILMARI model in the CRF category 1.B.2 Fugitive emission from oil and natural gas to years 1990–1999. Calculation of the fugitive emissions from distribution of oil and natural gas (which were estimated to be negligible) will be carried out according to the IPCC default methodology in the following inventories.

CRF 2.A	CO2 emissions from the use of mineral products need to be estimated.

CRF 2.F	In the 2001 inventory, uncertainties in the F-gases estimates were quantified for the first time. Most of the uncertainty originates from the non-coverage of the survey used to gather data from companies handling refrigerants. The uncertainty is expected to become considerably smaller in the inventory for 2002 as a list of contact details for all companies became available in mid-2002 through national regulation. This list has already been used in gathering data for 2002.

CRF 3	The quality of the activity data for N2O emissions inventory will be improved.

CRF 4.A	The time series concerning the activity data has been improved but some of the values may still need fine-tuning and improvement. An idea of changing the method so that it would be based on the feed consumption of cattle instead of estimating this indirectly from the data on animal weight, daily weight gain etc. has been put forward by the VTT Technical Research Centre of Finland and will be further examined at the MTT Agrifood Research Finland.

CRF 4.B	Annual or periodic data collection on e.g. the manure management systems need to be developed. Some of the input parameters in the calculation may need fine-tuning.

CRF 4.D	The suitability of the IPCC default emission factors for the Finnish conditions is questioned as the climatic conditions and agricultural practices differ very much from those from which the default values have been derived. Country specific emission factors are used in the estimates to a certain extent but more research is still needed in the future. Further development of national emission factors will continue at the MTT Agrifood Research Finland. A project is planned which will give new information about changes in the soil carbon stocks in Finland. The project will also yield new information about the area of organic soils in Finland.

The current guidelines are not considered logically or scientifically correct in all aspects considering the estimation of N2O emissions. Also the development of the IPCC method in determining land use classifications for organic soils needs further examination. Crop residue nitrogen of certain vegetable and fruit crops will be included into the calculations in the future.

CRF 5.A	Either the IPCC practice of the use of annual figures must be changed or a method to produce annual increment figures must be developed. This is very much a question of measurement costs. One should also note that the real annual variation of the increment of trees is of a magnitude ±20%. It is caused by climatic variation and biological cycles of trees. The use of five years instead of one reduces this variation which is not essential for the carbon balance budgeting. The new carbon allocation models with error estimates are under development and will be ready by 2005.

CRF 5.B	New carbon allocation models of trees with error estimates are under development. The inventory cycle is also under consideration.

CRF 5.C	No recalculations are needed.

CRF 6.A	The activity data before 1990 for waste as well as the waste composition data for MSW may be reviewed if better information is available.

CRF 6.B	Wastewater treatment causes also N2O emissions, although their importance is minor. In emission inventories the emissions have been estimated to be negligible. This assumption should be confirmed and the international development of the estimation methods for these emissions should be followed.

CRF 7	The fractions of carbon stored (and carbon released) need to be checked. There is a possibility of double counting in the present inventory. The whole category will be checked and moved to the Industrial Processes in the future inventories.

# REFERENCES

Aaltonen, J., Palosuo, T. and Pipatti, R. (2001). Key source identification in the Finnish 1999 greenhouse gas inventory. VTT Energy Reports 34/2001. Espoo, Finland. ISSN 1457-3350. www.vtt.fi/ene/tuloksia/enerap/2001/rep34_01.pdf

Advisory board for Waste Management (1992). Development programme on municipal waste management 2000. Ministry of the Environment, Environmental Protection Department Report 1992 (106). 55 p.

Anon. (1997) Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories, Reference Manual.

Arnold, M., Kuusisto, S. and Mroueh, U.-M. (1998). Emissions from volatile organic compounds (VOC) in 1996. VTT Publications 1921. Technical Research Centre of Finland. 35 p. (In Finnish).

Bartos, S. and Burton, C. S. (2000) Chapter 3.6 in IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Pp. 3.69–3.78.

Berglund, K. (1989). Ytsänkning på mosstorvjord. Sveriges Lantbruksuniversitet. Institutionen för markvetenskap. Avdelningsmeddelande 89:3 (In Swedish).

Blomberg, T. (2002). Recalculation of NMVOC emissions from road paving.

Boström, S. (1994). Greenhouse Gas Inventory, Finland 1990. Energy and Industry.

Boström, S., Bachman, R. and Hupa, M. (1992). Greenhouse Gas Emissions in Finland 1988 and 1990; Energy, Industrial and Transport Activities. Turku, Finland. Insinööritoimisto Prosessikemia Ky. Prepared for the Ministry of Trade and Industry and the Ministry of the Environment.

Finnish Environment Institute. (2000) Calculation model for Finnish NMVOC emissions. 32 p.

Forte, R. Jr., McCulloch, A. and Midgley, P. (2000). Chapter 3.7 in IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Pp. 3.79–3.122.

Grönroos, J., Nikander, A., Syri, S., Rekolainen, S. and Ekqvist, M. (1998). Agricultural ammonia emissions in Finland. Finnish Environment Institute. The Finnish Environment 206. 65 p. (In Finnish).

Houghton, J. T., Meira Filho, L.G., Lim, B., Treanton, K., Mamaty, I., Bonduki, Y., Griggs, D. J. and Callander, B. A. (1997) Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories. Vol 1–3. London: IPCC, OECD and IEA.

IPCC 1996a. Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories. Reference Manual. http://www.ipcc-nggip.iges.or.jp/public/gl/invs6.htmIPCC 1996b. Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories. Workbook. http://www.ipcc-nggip.iges.or.jp/public/gl/invs5.htm

Isaksson, K.-E. (1993). Wastes from construction. Talonrakennustoiminnan jätteet. Official Statistics of Finland, Environment 1993 (7). 55 p. (In Finnish).

Isännäinen, S. (1994). Utilisation of wastewater sludge. Jätevesilietteistä ja niiden hyötykäytöstä. In: Seminaariesitelmiä: Vesiensuojelu. Ympäristönsuojelutekniikan julkaisuja 1994(4): 19–39. Helsinki University of Technology. Laboratory of Environmental Pollution Prevention Technology. (In Finnish).

Joint EMEP/Corinair Atmospheric Emission Inventory Guidebook (2001). Third Edition. Copenhagen, European Environment Agency.

Karjalainen, T. and Kellomäki, S. (1996). Greenhouse gas inventory for land use changes and forestry in Finland based on international guidelines. Mitigation and Adaptation Strategies for Global Climate 1: 51–71.

Keränen, S. and Niskanen, R. (1987). Typpilannoituksen vaikutus happamoitumiseen Suomessa. The acidification impact of nitrogen fertilisation in Finland. Helsinki. Ministry of the Environment (HAPRO). 64 p. (In Finnish).

Klemedtsson, L., Klemedtsson, Å. K., Esala, M. and Kulmala, A. (1999). Inventory of N2O emissions from farmed European Peatland. In: Freibauer, A. and Kaltschimtt, M. (eds.) (1998). Approaches to greenhouse gas inventories of biogenic sources in agriculture. Proceedings of the Workshop at Lökeberg, Sweden, 9 B 10.7.1998. Pp. 79 B 94. (EU consorted action FAIR3 – CT96 – 1877 “Biogenic emissions of greenhouse gases caused by arable and animal agriculture”. Band 53, Stuttgart, Universität Stuttgart/IER).

Kulmala, A. and Esala, M. (2000). Maatalous ja kasvihuonekaasupäästöt. Agriculture and Greenhouse Gas Emissions. Literature Survey. Jokioinen. Agricultural Research Centre of Finland. 67 p. (A Series 76).(In Finnish).

Kuusisto, E. and Hämekoski, K. (eds.) (2001) Finland’s Third National Communication under the United Nations Framework Convention on Climate Change. Hämeenlinna 2001. 187 p + annexes.

Laine, K. I., Selin, P. and Nypönen T. (1998) The role of peat and peat utilization in carbon balance. Peat memorandum. 11 p. (Not published).

Leinonen, S. and Kuittinen, V. (1999). The Finnish Biogas Plant Register. Suomen Biokaasulaitosrekisteri II. Tiedot vuosilta 1996–1998. University of Joensuu. Karelian Institute. Working Papers 1999 (1). 54 p. (In Finnish).

LIPASTO. Calculation system for traffic exhaust emissions and energy consumption in Finland.

Ministry of the Environment (2002). Final report of the greenhouse gas working group. Recommendations for development of the national system for greenhouse gas inventories. January 2000. The Finnish Environment 548. 152 p. In Finnish.

MKL (1993). Environmental care programs 1990 B 1992. Maaseutukeskusten liitto (Rural Advisory Centres). 11 p. (In Finnish).

Monni, S. and Syri, S. 2003. Uncertainties in the Finnish 2001 Greenhouse Gas Emission Inventory. VTT Research Notes. (Forthcoming).

Myllys, M. (2002). MTT Agrifood Research Finland. Personal communication 15.10.2002.

Mäkelä, K., Laurikko, J. and Kanner, H. (2002). Road traffic exhaust gas emissions in Finland. LIISA 2001.1 calculation model. Technical Research Centre of Finland. VTT Research Notes 2177. (In Finnish).

National Climate Strategy Finland. Government Report to Parliament. (2001). Ministry of Trade and Industry. Reports 2/2001. 96 p.

Nieminen, M. (2002). Finnish Game and Fisheries Research Institute. Personal communication 25.9.2002.

Nieminen, M., Maijala, V. & Soveri, T. (1998). Reindeer feeding. (Poron ruokinta). Finnish Game and Fisheries Research Institute. (In Finnish).

NPI (1999). NPI Industry Handbooks. National Pollutant Inventory. Environment Australia. November 1999.

Nykänen, H., Alm, J., Lång, K., Silvola, J. and Martikainen, P. J. (1995). Emissions of CH4, N2O and CO2 from a virgin fen and a fen drained for grassland in Finland. Journal of Biogeography 22: 351–357.

Nykänen, H., Silvola, J., Alm, J. and Martikainen, P. (1996). Fluxes of greenhouse gases CH4, CO2 and N2O on some peat mining areas in Finland. In: Laiho, R., Laine, J. and Vasander, H. (eds.) (1996). Northern Peatland in global climate change. Helsinki: The Academy of Finland. Pp. 141–147. (Proceedings of the International Workshop held in Hyytiälä, Finland, 8–12 October 1995, The Finnish Research Programme on Climate Change – SILMU).

Oinonen, T. (2003). Finnish 2001 Inventory of HFC, PFC and SF6 Emissions. (Forthcoming). Copy from the author by request (.

Olivier, J. and Bakker, J. (2000). Chapter 3.5 in IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Pp. 3.53–3.68.

Palmer, B. (2000). Chapter 3.4 in IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Pp. 3.48–3.52.

Penman, J., Kruger, D., Galbally, I., Hiraishi, T., Nyenzi, B., Emmanuel, S., Buendia, L., Hoppaus, R., Martinsen, T., Meijer, J., Miwa, K. and Tanabe, K. (2000). Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Hayama: Intergovernmental Panel on Climate Change (IPCC). http://www.ipcc-nggip.iges.or.jp/public/gp/gpgaum.htm

Perälä, A.-L. and Nippala, E. (1998). Utilisation of wastes from construction. Rakentamisen jätteet ja niiden hyötykäyttö. Technical Research Centre of Finland, Research Notes 1936. 67 p. (In Finnish).

Pipatti, R. (2001). Greenhouse gas emissions and removals in Finland. VTT Research Notes 2094. 59 p. + app. 95 p. Technical Research Centre of Finland. http://www.inf.vtt.fi/pdf/tiedotteet/2001/T2094.pdf

Pipatti, R. (1997). Potential and cost-effectiveness of reducing methane and nitrous oxide emissions in Finland. VTT Research Notes 1835. 62 p. + app. 4 p. Technical Research Centre of Finland. (In Finnish).

Pipatti, R. (1994). Estimates, scenarios and reduction potential for methane and nitrous oxide emissions in Finland. VTT Research Notes 1548. 68 p. Technical Research Centre of Finland. (In Finnish).

Pipatti, R., Tuhkanen, S., Mälkiä, P. and Pietilä, R. (2000). Agricultural greenhouse gas emissions and abatement options and costs in Finland. VTT Publications 841. 72 p. Technical Research Centre of Finland. (In Finnish).

Pipatti, R., Hänninen, K., Vesterinen, R., Wihersaari, M. and Savolainen, I. (1996). Impact of waste management alternatives on greenhouse gas emissions. Jätteiden käsittelyvaihtoehtojen vaikutus kasvihuonekaasupäästöihin. VTT Publications 811. Technical Research Centre of Finland. 85 p. (In Finnish).

Pipatti, R., Savolainen, I. & Sinisalo, J. (1996). Greenhouse impacts of anthropogenic CH4 and N2O emissions in Finland. Environmental Management, Vol. 20 (1996), No:2, 219–233.

Rekolainen, S., Pitkänen, H., Bleeker, A. and Felix, S. (1995). Nitrogen and phosphorus fluxes from Finnish agricultural areas to the Baltic Sea. Nordic Hydrology 26: 55 B 72.

Savolainen, I., Tähtinen, M., Wistbacka, M., Pipatti, R. and Lehtilä, A. (1996). Economic reduction of acidifying deposition by decreasing emissions in Finland, Estonia and Russia. VTT Research Notes 1744. 60 p. Technical Research Centre of Finland. 60 p. (In Finnish).

Selin P. (1999). Industrial use of peatland and the re-use of cut-away areas in Finland (Thesis). University of Jyväskylä, Finland. 239 p.

Seppänen, H. & Matinlassi, T. (1998). Environmental care programs at Finnish farms 1995 B 1997. Maaseutukeskusten liitto (Rural Advisory Centres). 19 p. (In Finnish).

Statistics Finland (2002). Energy Statistics, 2001. Series: Official Statistics of Finland (SVT) Energy 2002:2. 151 p.

Statistics Finland (1998). The development program of energy statistics. Final Report. Statistics Finland Reviews 1988:11. Helsinki. (In Finnish with English summary).

Statistics Finland (1995). Wastes from Manufacturing and Related Industries 1992. Official Statistics of Finland, Environment 1995 (7). 162 p.

Tomppo, E. (2000a). Annual inventory and reporting practice on land use change and forestry for the Climate Convention (Ilmastosopimuksen mukainen vuosittainen maankäytön muutoksiin ja metsiin liittyvä inventointi- ja raportointikäytäntö). In: Ilmastosopimuksen ja Kioton pöytäkirjan metsien hiilivarastoja ja nieluja käsittelevän työryhmän muistio. Työryhmämuistio 2000:5. The Ministry of Agriculture and Forestry. (In Finnish).

Tomppo, E. (2000b). National Forest Inventory of Finland and its role in estimating the carbon balance forests. Biotechnol. Agron. Soc. Environ. 4(4): 281–284.

Tomppo, E. (1999). Carbon sequestrations and releases by tree of Finnish forests in 1995, 1996, 1997, 1998. A report. Finnish Forest Research Institute.

Tomppo, E., Henttonen, H., Korhonen, K. T., Aarnio, A., Ahola, A., Heikkinen, J., Ihalainen, A. Mikkelä, H., Tonteri, T. and Tuomainen, T. (1998). Forest resources of Forestry Centre Etelä-Pohjanmaa and their changes in 1968–97 (Etelä-Pohjanmaan metsäkeskuksen alueen metsävarat ja niiden kehitys 1968B97). Folia Forestalia 1998 (2B): 293–374. (In Finnish).

Tomppo, E., Varjo, J., Korhonen, K., Ahola, A., Ihalainen, A., Heikkinen, J., Hirvelä, H., Mikkelä, H., Mikkola, E., Salminen, S. and Tuomainen, T. (1997). Country report for Finland. In: Study on European Forestry Information and Communication Systems. Reports on forestry inventory and survey systems. Vol. 1. European Commission. Pp. 145–226. ISBN 92-827-9847-X.

Tuhkanen, S. (2002). Mitigation of greenhouse gases from waste management in Finland. Methane (CH4) emissions from landfills and landfill gas recovery. Espoo 2002. VTT Research Notes 2142. 46 p.

UNFCCC Secretariat (2002). Guidelines for reporting and review of greenhouse gas inventories from parties included in Annex I to the Convention (Implementing decisions 3/CP.5 and 6/CP.5). FCCC/SBSTA/2002/ L.5/Add.1.

YTV (1995). Emissions from volatile organic compounds at the metropolitan region. Metropolitan Area Publications C1995:19. Helsinki. 32 p. (In Finnish).

| ## Annex 1. Energy balance sheets Energy balance sheet 2001 (ktoe) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Coal | Crude oil & NGL | Petroleum products | Natural gas | Nuclear energy | Hydro & wind power | Peat fuel | Wood and recycled fuels | Electricity | District heat & heat pumps | TOTAL |
| Indigenous production | – | – | – | – | 5 694 | 1 125 | 1 436 | 6 320 | – | 65 | 14 641 |
| Imports | 4 130 | 10 941 | 3 987 | 3 716 | – | – | – | 248 | 1 012 | – | 24 035 |
| Exports | -1 | – | -4 913 | – | – | – | -18 | – | -156 | – | -5 088 |
| International marine bunkers | – | – | -572 | – | – | – | – | – | – | – | -572 |
| Stock changes | -34 | -20 | -15 | – | – | – | 587 | – | – | – | 519 |
| Total primary energy supply | 4 095 | 10 921 | -1 513 | 3 716 | 5 694 | 1 125 | 2 006 | 6 568 | 856 | 65 | 33 535 |
| Statistical difference | – | – | -60 | -7 | – | – | – | – | – | – | -67 |
| Electricity generation | -1 500 | – | -53 | -173 | -5 694 | -1 125 | -657 | -214 | 3 914 | – | -5 502 |
| Combined district heat and power | -1 160 | – | -80 | -1 489 | – | – | -760 | -341 | 1 212 | 2 043 | -575 |
| Co-generation electricity in industry | -44 | – | -110 | -210 | – | – | -76 | -911 | 998 | – | -353 |
| District heat production | -70 | – | -222 | -166 | – | – | -74 | -190 | – | 695 | -27 |
| Oil refinery | – | -10 921 | 10 520 | – | – | – | – | – | – | – | -402 |
| Coal transformation | -558 | – | – | – | – | – | – | – | – | – | -558 |
| Transmission and distributions losses | – | – | – | – | – | – | – | – | -260 | -230 | -490 |
| TFC (total final energy) | 764 | – | 8 482 | 1 672 | – | – | 438 | 4 912 | 6 721 | 2 573 | 25 562 |
| Industry | 762 | – | 1 466 | 1 550 | – | – | 414 | 3 754 | 3 719 | 250 | 11 913 |
| Transport | – | – | 4 385 | 22 | – | – | – | – | 49 | – | 4 455 |
| Residential | 2 | – | 835 | 23 | – | – | 11 | 991 | 1 507 | 1 446 | 4 814 |
| Agriculture | – | – | 662 | 14 | – | – | 11 | 108 | 73 | 10 | 877 |
| Commerce and public services | – | – | 358 | 31 | – | – | 2 | 60 | 1 214 | 869 | 2 533 |
| Other consumption | – | – | 478 | – | – | – | – | – | 159 | – | 332 |
| Non-energy use | – | – | 299 | 33 | – | – | – | – | – | – | 637 |

- Energy balance sheet 2001 (TJ)

|  | Coal | Crude oil & NGL | Petroleum products | Natural gas | Nuclear energy | Hydro & wind power | Peat fuel | Wood and recycled fuels | Electricity | District heat & heat pumps | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indigenous production | – | – | – | – | 238 407 | 47 115 | 60 120 | 264 602 | – | 2 730 | 612 975 |
| Imports | 172 922 | 458 080 | 166 935 | 155 590 | – | – | – | 10 400 | 42 368 | – | 1 006 296 |
| Exports | -45 | – | -205 714 | – | – | – | -738 | – | -6 516 | – | -213 013 |
| International marine bunkers | – | – | -23 943 | – | – | – | – | – | – | – | -23 943 |
| Stock changes | -1 418 | -834 | -622 | – | – | – | 24 590 | – | – | – | 21 716 |
| Total primary energy supply | 171 459 | 457 247 | -63 345 | 155 590 | 238 407 | 47 115 | 83 972 | 275 002 | 35 852 | 2 730 | 1 404 031 |
| Statistical difference | – | – | -2 512 | -286 | – | – | – | – | – | – | -2 799 |
| Electricity generation | -62 809 | – | -2 211 | -7 255 | -238 407 | -47 115 | -27 506 | -8 954 | 163 885 | – | -230 374 |
| Combined district heat and power | -48 561 | – | -3 332 | -62 351 | – | – | -31 823 | -14 269 | 50 738 | 85 536 | -24 061 |
| Cogeneration electricity in industry | -1 826 | – | -4 623 | -8 786 | – | – | -3 195 | -38 150 | 41 800 | – | -14 780 |
| District heat production | -2 934 | – | -9 288 | -6 930 | – | – | -3 108 | -7 965 | – | 29 088 | -1 136 |
| Oil refinery | – | -457 247 | 440 438 | – | – | – | – | – | – | – | -16 808 |
| Coal transformation | -23 351 | – | – | – | – | – | – | – | – | – | -23 351 |
| Transmission and distributions losses | – | – | – | – | – | – | – | – | -10 897 | -9 612 | -20 509 |
| TFC (total final energy) | 31 978 | – | 355 128 | 69 982 | – | – | 18 341 | 205 664 | 281 380 | 107 742 | 1 070 214 |
| Industry | 31 888 | – | 61 364 | 64 906 | – | – | 17 321 | 157 164 | 155 696 | 10 450 | 498 789 |
| Transport | – | – | 183 569 | 900 | – | – | – | – | 2 034 | – | 186 503 |
| Residential | 90 | – | 34 944 | 942 | – | – | 470 | 41 500 | 63 083 | 60 522 | 201 551 |
| Agriculture | – | – | 27 715 | 576 | – | – | 470 | 4 500 | 3 060 | 406 | 36 727 |
| Commerce and public services | – | – | 14 990 | 1 290 | – | – | 80 | 2 500 | 50 846 | 36 364 | 106 070 |
| Other consumption | – | – | 20 021 | – | – | – | – | – | 6 660 | – | 13 893 |
| Non-energy use | – | – | 12 525 | 1 368 | – | – | – | – | – | – | 26 681 |

| Comparison to CRF categories: Data from energy balance | Coal | Oil products | Natural gas | Peat | Wood and recycled fuels | TOTAL |  |  | CRF 2001 sector totals excl. biomass | Difference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | excluding biomass |  | including biomass |  |  |
| Transformation (CRF 1.A.1) | 139 481 | 36 263 | 85 322 | 65 632 | 69 339 | 326 697 |  | 396 035 | 330 732 | -1.2% |
| Industry (CRF 1.A.2) | 31 888 | 61 364 | 64 906 | 17 321 | 157 164 | 175 479 |  | 332 642 | 174 808 | 0.4% |
| Transport (CRF 1.A.3) | – | 183 569 | 900 | – | – | 184 469 |  | 184 469 | 172 557 | 6.9% |
| Commerce and public services (CRF 1.A.4a) | – | 14 990 | 1 290 | 80 | 2 500 | 16 360 |  | 18 860 | 18 502 | -11.6% |
| Residential (CRF 1.A.4b) | 90 | 34 944 | 942 | 470 | 41 500 | 36 446 |  | 77 946 | 36 220 | 0.6% |
| Agriculture (CRF 1.A.4c) | – | 27 715 | 576 | 470 | 4 500 | 28 761 |  | 33 261 | 27 995 | 2.7% |
| Other (CRF 1.A.5) | – | 20 021 | – | – | – | 20 021 |  | 20 021 | 19 752 | 1.4% |
| Totals by fuel | 171 459 | 378 865 | 153 936 | 83 972 | 275 002 | 788 232 |  | 1 063 234 | 780 566 | 1.0% |
| Aviation bunkers correction |  | -14 890 |  |  |  |  |  |  |  |  |
| Totals | 171 459 | 363 975 | 153 936 | 83 972 | 275 002 | 773 342 |  | 1 048 344 |  |  |
|  | Solid fuels | Liquid fuels | Gaseous fuels | Other | Biomass |  |  |  |  |  |
| CRF totals by fuel | 167 656 | 362 900 | 156 664 | 93 076 | 255 535 | 780 296 |  | 1 035 831 |  |  |
| Difference | 2.3% | 0.3% | -1.7% | -9.8% | 7.6% | -0.9% |  | 1.2% |  |  |

Energy balance sheet 2001 (Gg CO2)

|  | Coal | Crude oil & NGL | Petroleum products | Natural gas | Nuclear energy | Hydro & wind power | Peat fuel | Wood and recycled fuels | Electricity | District heat & heat pumps | TOTAL (fossil & peat) | TOTAL (incl. biomass) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indigenous production | – | – | – | – | 0 | 0 | 6 309 | 28 710 | – | 0 | 6 309 | 35 019 |
| Imports | 16 031 | 33 814 | 12 134 | 8 685 | – | – | – | 1 128 | 0 | – | 70 664 | 71 792 |
| Exports | -4 | – | -14 952 | – | – | – | -77 | – | 0 | – | -15 034 | -15 034 |
| International marine bunkers | – | – | -1 740 | – | – | – | – | – | – | – | -1 740 | -1 740 |
| Stock changes | -131 | -62 | -45 | – | – | – | 2 580 | – | – | – | 2 342 | 2 342 |
| Total primary energy supply | 15 896 | 33 752 | -4 604 | 8 685 | 0 | 0 | 8 812 | 29 839 | 0 | 0 | 62 540 | 92 379 |
| Statistical difference | – | – | -183 | -16 | – | – | – | – | – | – | -199 | -199 |
| Electricity generation | 5 823 | – | 161 | 405 | 0 | 0 | 2 887 | 972 | 0 | – | 9 275 | 10 247 |
| Combined district heat and power | 4 502 | – | 242 | 3 480 | – | – | 3 339 | 1 548 | 0 | 0 | 11 564 | 13 112 |
| Co-generation electricity in industry | 169 | – | 336 | 490 | – | – | 335 | 4 139 | 0 | – | 1 331 | 5 470 |
| District heat production | 272 | – | 675 | 387 | – | – | 326 | 864 | – | 0 | 1 660 | 2 524 |
| Oil refinery | – | 33 752 | -32 014 | – | – | – | – | – | – | – | 1 738 | 1 738 |
| Coal transformation | 2 165 | – | – | – | – | – | – | – | – | – | 2 165 | 2 165 |
| Transmission and distributions losses | – | – | – | – | – | – | – | – | 0 | 0 | 0 | 0 |
| TFC (total final energy) | 2 965 | – | 24 902 | 3 830 | – | – | 1 925 | 22 315 | 0 | 0 | 33 622 | 55 937 |
| Industry | 2 956 | – | 4 460 | 3 623 | – | – | 1 818 | 17 053 | 0 | 0 | 12 857 | 29 910 |
| Transport | – | – | 13 343 | 50 | – | – | – | – | – | – | 13 393 | 13 393 |
| Residential | 8 | – | 2 540 | 53 | – | – | 49 | 4 503 | 0 | 0 | 2 650 | 7 153 |
| Agriculture | – | – | 2 014 | 32 | – | – | 49 | 488 | 0 | 0 | 2 096 | 2 584 |
| Commerce and public services | – | – | 1 090 | 72 | – | – | 8 | 271 | 0 | 0 | 1 170 | 1 441 |
| Other consumption | – | – | 1 455 | – | – | – | – | – | 0 | – | 1 455 | 1 455 |
| Non-energy use | – | – | 910 | 76 | – | – | – | – | – | – | 987 | 987 |
| Total CO2 emissions (excluding non-energy use) | 15 896 |  | 28 055 | 8 593 |  |  | 8 812 | 29 839 |  |  | 61 355 | 91 194 |
| CO2 emission factor g/MJ | 94.6 | 74.6 | 73.4 | 56.1 | 0.0 | 0.0 | 106.0 | 109.6 | 0.0 | 0.0 |  |  |
| Oxidation factor | 0.98 | 0.99 | 0.99 | 0.995 | 0.00 | 0.00 | 0.99 | 0.99 | 0.00 | 0.00 |  |  |

| Comparison to CRF categories: Data from energy balance | Coal | Petroleum products | Natural gas | Peat fuel | Wood and recycled fuels | TOTAL excluding biomass | including biomass | CRF 2001 sector totals excl. biomass | Difference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transformation (CRF 1.A.1) | 12 931 | 3 152 | 4 763 | 6 887 | 7 524 | 27 733 | 35 257 | 26 762 | 3.6% |
| Industry (CRF 1.A.2) | 2 956 | 4 460 | 3 623 | 1 818 | 17 053 | 12 857 | 29 910 | 13 855 | -7.2% |
| Transport (CRF 1.A.3) | – | 13 343 | 50 | – | – | 13 393 | 13 393 | 12 569 | 6.6% |
| Commerce and public services (CRF 1.A.4a) | – | 1 090 | 72 | 8 | 271 | 1 170 | 1 441 | 1 317 | -11.2% |
| Residential (CRF 1.A.4b) | 8 | 2 540 | 53 | 49 | 4 503 | 2 650 | 7 153 | 2 658 | -0.3% |
| Agriculture (CRF 1.A.4c) | – | 2 014 | 32 | 49 | 488 | 2 096 | 2 584 | 2 047 | 2.4% |
| Other (CRF 1.A.5) | – | 1 455 | – | – | – | 1 455 | 1 455 | 1 284 | 13.3% |
| Totals by fuel | 15 896 | 28 055 | 8 593 | 8 812 | 29 839 | 61 355 | 91 194 | 60 492 | 1.4% |
| Aviation bunkers correction |  | -1 054 |  |  |  |  |  |  |  |
| Totals | 15 896 | 27 001 | 8 593 | 8 812 | 29 839 | 60 301 | 90 140 |  |  |
| CRF totals by fuel | 15 707 | 26 530 | 8 735 | 9 520 | 27 681 | 60 492 | 88 173 |  |  |
| Difference | 1.2% | 1.8% | -1.6% | -7.4% | 7.8% | -0.3% | 2.2% |  |  |

## Annex 2. Emission factors implied in the energy sector

- Emission factors of stationary sources in the ILMARI calculation system

| Combustion technique code |  | Main category / main fuel code |  | CH4 kg/TJ | N2O kg/TJ |
| --- | --- | --- | --- | --- | --- |
| 8 | CFB | 10 | Coal fired boiler(> 80% coal) | 4–5 | 70 |
|  |  | 40 | Peat fired boiler (> 80% peat) | 2–7 | 30 |
|  |  | 84 | Multi-fuel/peat fired boiler (> 50% peat) | 5 | 30 |
|  |  | 50 | Wood/bark fired boiler (> 80% wood) | 30 | 10 |
|  |  | 85 | Multi-fuel/wood/bark fired boiler (> 50% wood) | 4–35 | 10 |
|  |  | 88 | Multi-fuel fired boiler | 30 | 30 |
| 7 | BFB | 10 | Coal fired boiler(> 80% coal) | 5 | 70 |
|  |  | 40 | Peat fired boiler (> 80% peat) | 2–7 | 2 |
|  |  | 84 | Multi-fuel/peat fired boiler (> 50% peat) | 2–5 | 2 |
|  |  | 50 | Wood/bark fired boiler (> 80% wood) | 30 | 2 |
|  |  | 85 | Multi-fuel/wood/bark fired boiler (> 50% wood) | 4–35 | 2 |
|  |  | 88 | Multi-fuel fired boiler | 15 | 2 |
| 14 | PFB | 81 | Multi-fuel/coal fired boiler (> 50% coal) | 4 | 2 |
| 3 | Stoker, grate | 10 | Coal fired boiler(> 80% coal) | 4–8 | 4 |
|  |  | 40 | Peat fired boiler (> 80% peat) | 2–7 | 2 |
|  |  | 84 | Multi-fuel/peat fired boiler (> 50% peat) | 2–15 | 2 |
|  |  | 50 | Wood/bark fired boiler (> 80% wood) | 30–50 | 2 |
|  |  | 85 | Multi-fuel/wood/bark fired boiler (> 50% wood) | 20–35 | 2 |
|  |  | 88 | Multi-fuel fired boiler | 10–35 | 2 |
| 1, 4, 5 | Burners | 10 | Coal fired boiler(> 80% coal) | 4 | 2 |
|  |  | 30 | Oil fired boiler(> 80% oil) | 8 | 2 |
|  |  | 40 | Peat fired boiler (> 80% peat) | 2–7 | 2 |
|  |  | 50 | Wood/bark fired boiler (> 80% wood) | 50 | 2 |
|  |  | 60 | Gas fired boiler (> 80% gas) | 3 | 1 |
|  |  | 70 | Soda recovery boiler (> 80% black liquor) | 1 | 1 |
|  |  | 81–88 | Multi-fuel fired boiler | 2–50 | 1–2 |
| 10 | Gas turbine | 121 | Gas turbine plant (oil) | 8 | 1 |
|  |  | 122 | Gas turbine plant (gas) | 3 | 1 |
| 10x12 | Gas turbine (Combined cycle) | 130 | Gas turbine /Combined cycle | 3 | 1 |
| 11 | Diesel engine | 141 | Diesel power plant (oil) | 2 | 31 |
| 11 | Diesel engine | 142 | Diesel power plant (gas) | 2 | 31 |
| 115 | Internal combustion engine (Otto) | 143 | Other combustion engine power plant | 2 | 31 |
|  | Other combustion (not specified) | 90, 150 | Not specified | 8–10 | 2 |
|  |  | 91 | Mesa kiln | 8 | 2 |
|  |  | 92 | Hospital waste incineration | 8–50 | 2 |
|  |  | 93 | Asphalt station | 8 | 2 |
|  |  | 94 | Coking plant | 0 | 2 |
|  |  | 95 | Drying oven | 8 | 2 |
|  |  | 96 | Blast furnace | 0 | 2 |
|  |  | 97 | Sinter plant | 4 | 2 |
|  |  | 98 | Rolling mill | 0 | 2 |
|  |  | 99 | Melting oven | 0 | 2 |
|  |  | 100 | Brick furnace | 8 | 2 |
|  |  | 101 | Cupola oven | 8–10 | 2 |

B. Emission factors of small combustion in ILMARI calculation system

| Small combustion boilers < 1 MW | CH4 kg/TJ | N2O kg/TJ | CO kg/TJ | NMVOC kg/TJ |
| --- | --- | --- | --- | --- |
| Oil | 10 | 2 | 20 | 5 |
| Coal | 300 | 4 | 200 | 200 |
| Natural gas | 3 | 1 | 20 | 5 |
| Peat | 300 | 2 | 200 | 200 |
| Wood | 300 | 2 | 2 100 | 600 |
| References: | IPCC Table 1–7 Boström (1994) | IPCC Table 1–8 Boström (1994) | IPCC Table 1–10 Boström (1994) | IPCC Table 1–11 Peat: the same EF as for coal |

C. Emission factors of transport sectors and off-road machinery in ILMARI calculation system

| CRF | Sector | Fuel | Detailed calculation model | CH4 kg/TJ | N2O kg/TJ | CO kg/TJ | NMVOC kg/TJ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CRF 1.A.3a | Domestic aviation | Aviation gasoline | ILMI | 30.6 | 3.0 | 23 600 | 275 |
| CRF 1.A.3a | Domestic aviation | Jet fuel | ILMI | 2.7 | 3.0 | 112 | 24 |
| CRF 1.A.3b | Road transport | Diesel oil | LIISA | 3.7 | 2.2 | 247 | 72 |
| CRF 1.A.3b | Road transport | Gasoline | LIISA | 30.1 | 11.1 | 4 207 | 447 |
| CRF 1.A.3b | Road transport | Natural gas | LIISA | 610.0 | 1.1 | 730 | 110 |
| CRF 1.A.3c | Railways | Gasoil (diesel) | RAILI | 4.0 | 30.0 | 233 | 94 |
| CRF 1.A.3d | Domestic navigation / Passengers & cargo | Gasoil (diesel) | MEERI | 3.7 | 30.0 | 180 | 58 |
| CRF 1.A.3d | Domestic navigation / Passengers & cargo | Residual fuel oil | MEERI | 5.0 | 2.0 | 81 | 37 |
| CRF 1.A.3d | Domestic navigation / Leisure boats | Gasoline | MEERI | 199.0 | 2.0 | 11 368 | 3 781 |
| CRF 1.A.3d | Domestic navigation / Leisure boats | Gasoil (diesel) | MEERI | 8.2 | 30.0 | 377 | 129 |
| CRF 1.A.4c | Fishing vessels | Gasoil (diesel) | MEERI | 4.1 | 30.0 | 212 | 64 |
| CRF 1.A.3d | Domestic navigation/Other | Gasoil (diesel) | MEERI | 3.7 | 30.0 | 180 | 58 |
| CRF IntBunk | International aviation | Jet fuel | ILMARI | 2.7 | 3.0 | 112 | 24 |
| CRF IntBunk | International navigation | Gasoil (diesel) | ILMARI | 3.7 | 30.0 | 180 | 58 |
| CRF IntBunk | International navigation | Residual fuel oil | ILMARI | 5.0 | 2.0 | 81 | 37 |
| CRF 1.A.4c | Off-road machinery / Agriculture | Gasoil (diesel) | TYKO | 4.0 | 31.8 | 390 | 153 |
| CRF 1.A.4c | Off-road machinery / Agriculture | Gasoline | TYKO | 129.8 | 1.6 | 29 100 | 1 520 |
| CRF 1.A.4c | Off-road machinery / Forestry | Gasoline | TYKO | 139.0 | 0.3 | 27 482 | 9 364 |
| CRF 1.A.4c | Off-road machinery / Forestry | Gasoil (diesel) | TYKO | 4.3 | 32.5 | 379 | 119 |
| CRF 1.A.2f | Off-road machinery / Construction | Gasoil (diesel) | TYKO | 4.3 | 31.7 | 383 | 153 |
| CRF 1.A.2f | Off-road machinery / Construction | Gasoline | TYKO | 133.4 | 1.7 | 27 897 | 1 438 |
| CRF 1.A.3e | Off-road machinery / Other | Gasoline | TYKO | 95.0 | 1.2 | 18 712 | 1 963 |
| CRF 1.A.3e | Off-road machinery / Other | LPG | TYKO | 64.6 | 3.2 | 2 661 | 99 |
| CRF 1.A.3e | Off-road machinery / Other | Gasoil (diesel) | TYKO | 4.1 | 31.5 | 387 | 149 |

D. CO2 emission factors

|  | g CO2/MJ | kg C/GJ | Ref. | Oxidation factor | NCV |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gasoline | 72.7 | 19.8 | 2 | 1 | 43.0 | GJ/t |  |
| Diesel oil | 73.0 | 19.9 | 2 | 1 | 42.8 | GJ/t |  |
| Light fuel oil (gasoil) | 74.1 | 20.2 | 1 | 0.99 | 42.4 | GJ/t |  |
| Residual fuel oil | 77.4 | 21.1 | 1 | 0.99 | 40.7 | GJ/t | 40.5–41.1 GJ/t |
| Jet fuel | 71.5 | 19.5 | 1 | 0.99 | 42.3 | GJ/t |  |
| Kerosene | 71.5 | 19.5 | ? | 0.99 | 43.4 | GJ/t |  |
| Naphta | 72.7 | 19.8 | 1 | 0.99 | 44.3 | GJ/t |  |
| LPG | 63.1 | 17.2 | 1 | 0.99 | 45.7 | GJ/t |  |
| Waste oil | 77.4 | 21.1 | 4 | 0.99 | 40.9 | GJ/t |  |
| Refinery gas | 65.0 | 17.7 | 7 | 0.99 | 47.5 | GJ/1 000 m3 |  |
| Refinery coke | 97.0 | 26.5 | 7 | 0.99 | 33.3 | GJ/t |  |
| Hard coal | 94.6 | 25.8 | 1 | 0.99 | 25.5 | GJ/t |  |
| Coke | 108.0 | 29.5 | 1 | 0.98 | 29.3 | GJ/t |  |
| Anthracite and briquettes | 94.6 | 25.8 | 1 | 0.98 | 33.5 | GJ/t |  |
| Blast furnace gas | 0.0 | 0.0 | 9 |  | 3.8 | GJ/1 000 m3 |  |
| Coke oven gas | 40.5 | 11.0 | 7 | 0.98 | 16.7 | GJ/1 000 m3 |  |
| Natural gas | 56.1 | 15.3 | 1 | 0.995 | 36.0 | GJ/1 000 m3 |  |
| Peat | 106.0 | 28.9 | 1 | 0.99 | 10.1–12.3 | GJ/t |  |
| Fuelwood | 109.6 | 29.9 | 1 | 0.99 |  |  | varying NCVs |
| Bark | 109.6 | 29.9 | 1 | 0.99 |  |  | varying NCVs |
| Wood chips | 109.6 | 29.9 | 1 | 0.99 |  |  | varying NCVs |
| Sawdust | 109.6 | 29.9 | 1 | 0.99 |  |  | varying NCVs |
| Other residues from wood proc. industry | 109.6 | 29.9 | 5 | 0.99 |  |  | varying NCVs |
| Black liquor | 110.0 | 30.0 | 3 | 0.99 | 12.6 | GJ/tdm |  |
| Sulphite liquor | 112.0 | 30.5 | 3 | 0.99 | 9.9 | GJ/tdm |  |
| Malodorous gases from wood proc. industry | 59.0 | 16.1 |  | 0.99 | 44.9 |  |  |
| 0-fibres / biosludge | 109.6 | 29.9 | 5 | 0.99 | 5.4 | GJ/t |  |
| Waste paper | 109.6 | 29.9 | 5 | 0.99 | 14.0 | GJ/t |  |
| Municipal waste | 31.8 | 8.7 | 10 | 0.99 | 10–21 | GJ/t | share of fossil carbon |
| Construction & demolition waste | 31.8 | 8.7 | 10 | 0.99 | 10.0 | GJ/t | share of fossil carbon |
| Industrial waste | 75.0 | 20.5 | 8 | 0.99 | 42.9 | GJ/t |  |
| Plastic waste | 74.1 | 20.2 | 6 | 0.99 | 40.0 | GJ/t |  |
| Other wastes | 75.0 | 20.5 | 8 | 0.99 | 8.8 | GJ/t |  |
| Other fuels |  |  | 6 | 0.99 | 5–40 | GJ/t | 74–150 g CO2/MJ |

References

| 1 | IPCC Guidelines 1995 | 6 | Depends on type of fuel; assumed same as for corresponding fuels |
| --- | --- | --- | --- |
| 2 | VTT, LIISA model | 7 | Plant specific data |
| 3 | Boström et al. 1992 | 8 | Ref. not specified (expert guess) |
| 4 | Assumed same as for residual fuel oil | 9 | Assumed zero to avoid double-counting (CO2 emissions from blast furnaces included in coke and RFO used in these plants) |
| 5 | Assumed same as for fuelwood | 10 | Expert estimate by VTT and Statistics Finland |

E. Comments to emission factors in ILMARI calculation system

In the ILMARI system emissions are calculated by a bottom-up method using annual fuel consumption for boilers and processes. ILMARI includes also technical data of boilers, such as combustion technique, fuel capacity, emission reduction equipment etc. The CH4, N2O, CO and NMVOC emission factors used in the Finnish inventory are largely based on the compilation of research data by Prosessikemia Oy (Boström et al. 1992; Boström 1994) in the inventory calculations for the year 1990 for Finland’s first national communication to the UNFCCC.

The emission factor database from Prosessikemia Oy has been expanded to fit ILMARI’s more detailed classification of boilers and processes. As new boiler types have been included in the boiler database, the emission factors have been determined on basis of expert opinion (when no data has been available from other sources). In the future, emission factors have to be checked against the Revised 1996 IPCC Guidelines and new data from national research.

Emission factors for small combustion are partly IPCC default and partly taken from the reference Boström et.al. (1992).

In the transport sectors the emission factors used in the ILMARI system are taken as average emission factors from the calculation models of the VTT. Emission factors for international bunker emissions are the same as for corresponding domestic transport sectors.

CH4 and N2O emissions from railway, navigation and aviation have been calculated with the ILMARI system from aggregated fuel data, using selected emission factors from the IPCC tables.

## Annex 3. Additional information on uncertainty reporting

TABLE A. Tier 2 uncertainty reporting

| A | B | C | D | E | F | G | H | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Greenhouse gas source and sink categories | Gas | Base year (1990) emissions | Year t (2001) emissions | Uncertainty in year 2001 emissions as % of emissions in the category |  | Uncertainty introduced on national total in year 2001 | % change in emissions between year t and base year | Range of likely % change between year t and base year |  |
|  |  | Gg CO2 equivalent | Gg CO2 equivalent | % below (2.5 percentile) | % above (97.5 percentile) | % | % | Lower % (2.5 percentile) | Upper % (97.5 percentile) |
| 1.A. Fuel combustion activities |  |  |  |  |  |  |  |  |  |
| 1.A.1 Energy industries |  |  |  |  |  |  |  |  |  |
| Liquid fuels | CO2 | 2 607 | 2 881 | 3 | 3 | 0.104 | 11 | 8 | 14 |
|  | CH4 | 6 | 6 | 75 | 12 | 0.006 | 10 | 0 | 16 |
|  | N2O | 26 | 30 | 76 | 12 | 0.029 | 17 | 0 | 22 |
| Solid fuels | CO2 | 9 279 | 10 211 | 4 | 4 | 0.472 | 10 | 7 | 13 |
|  | CH4 | 9 | 10 | 75 | 12 | 0.010 | 16 | 0 | 22 |
|  | N2O | 85 | 105 | 50 | 50 | 0.067 | 24 | 11 | 38 |
| Gaseous fuels | CO2 | 2 659 | 5 882 | 1 | 1 | 0.107 | 121 | 118 | 124 |
|  | CH4 | 4 | 8 | 76 | 11 | 0.007 | 118 | 27 | 132 |
|  | N2O | 18 | 42 | 49 | 50 | 0.027 | 137 | 69 | 207 |
| Biomass | CH4 | 2 | 11 | 52 | 57 | 0.008 | 448 | 211 | 717 |
|  | N2O | 10 | 73 | 71 | 149 | 0.137 | 655 | 188 | 1 638 |
| Other fuels | CO2 | 3 972 | 7 679 | 7 | 7 | 0.696 | 93 | 82 | 105 |
|  | CH4 | 5 | 6 | 50 | 51 | 0.004 | 8 | 0 | 19 |
|  | N2O | 141 | 180 | 70 | 151 | 0.344 | 27 | 7 | 75 |
| 1.A.2 Manufacturing industries and construction |  |  |  |  |  |  |  |  |  |
| Liquid fuels | CO2 | 4 294 | 4 374 | 3 | 3 | 0.158 | 2 | -1 | 5 |
|  | CH4 | 8 | 40 | 75 | 12 | 0.038 | 430 | 31 | 549 |
|  | N2O | 83 | 152 | 75 | 12 | 0.145 | 83 | -57 | 169 |
| Solid fuels | CO2 | 6 410 | 5 476 | 4 | 4 | 0.249 | -15 | -17 | -12 |
|  | CH4 | 4 | 3 | 74 | 12 | 0.003 | -32 | -38 | -7 |
|  | N2O | 108 | 97 | 50 | 50 | 0.062 | -10 | -17 | -3 |
| Gaseous fuels | CO2 | 2 094 | 2 294 | 1 | 1 | 0.042 | 10 | 8 | 11 |
|  | CH4 | 5 | 6 | 74 | 11 | 0.006 | 32 | 7 | 38 |
|  | N2O | 17 | 19 | 50 | 50 | 0.012 | 13 | 5 | 21 |
| Greenhouse gas source and sink categories | Gas | Base year (1990) emissions | Year t (2001) emissions | Uncertainty in year 2001 emissions as % of emissions in the category |  | Uncertainty introduced on national total in year 2001 | % change in emissions between year t and base year | Range of likely % change between year t and base year |  |
|  |  | Gg CO2 equivalent | Gg CO2 equivalent | % below (2.5 percentile) | % above (97.5 percentile) | % | % | Lower % (2.5 percentile) | Upper % (97.5 percentile) |
| Biomass | CH4 | 20 | 23 | 51 | 54 | 0.016 | 14 | -9 | 41 |
|  | N2O | 111 | 89 | 70 | 154 | 0.173 | -20 | -65 | 0 |
| Other fuels | CO2 | 1 561 | 1 656 | 7 | 7 | 0.149 | 6 | -1 | 13 |
|  | CH4 | 4 | 4 | 50 | 51 | 0.002 | -2 | -12 | 6 |
|  | N2O | 56 | 25 | 70 | 148 | 0.048 | -54 | -135 | -16 |
| 1.A.3 Transport |  |  |  |  |  |  |  |  |  |
| a. Civil aviation | CO2 | 403 | 360 | 5 | 5 | 0.025 | -11 | -17 | -4 |
|  | CH4 | 11 | 0 | 57 | 98 | 0.000 | -97 | -193 | -41 |
|  | N2O | 57 | 5 | 70 | 149 | 0.009 | -92 | -230 | -27 |
| b. Road transportation |  |  |  |  |  |  |  |  |  |
| Gasoline | CO2 | 6 202 | 5 202 | 2 | 2 | 0.001 | -16 | -17 | -15 |
|  | CH4 | 44 | 45 | 49 | 50 | 0.029 | 4 | -2 | 11 |
| Cars with catalytic converters | N2O | 35 | 348 | 70 | 150 | 0.663 | 882 | 266 | 2 207 |
| Cars without catalytic converters | N2O | 67 | 21 | 81 | 189 | 0.049 | -69 | -191 | -12 |
| Diesel | CO2 | 4 909 | 5 705 | 2 | 2 | 0.163 | 16 | 15 | 18 |
|  | CH4 | 15 | 6 | 50 | 50 | 0.004 | -60 | -91 | -30 |
|  | N2O | 78 | 77 | 80 | 205 | 0.199 | -2 | -27 | 17 |
| Natural gas | CO2 | 0 | 3 | 2 | 2 | 0.000 | * | * | * |
|  | CH4 | 0 | 1 | 50 | 51 | 0.000 | * | * | * |
|  | N2O | 0 | 0 | 70 | 146 | 0.000 | * | * | * |
| c. Railways | CO2 | 192 | 136 | 5 | 5 | 0.009 | -29 | -35 | -23 |
|  | CH4 | 0 | 0 | 60 | 109 | 0.000 | 40 | 14 | 89 |
|  | N2O | 25 | 17 | 70 | 147 | 0.032 | -31 | -81 | -8 |
| d. Navigation |  |  |  |  |  |  |  |  |  |
| Residual oil & gas/Diesel oil | CO2 | 227 | 311 | 10 | 10 | 0.040 | 37 | 20 | 54 |
|  | CH4 | 0 | 0 | 57 | 99 | 0.001 | 233 | 98 | 471 |
|  | N2O | 29 | 24 | 70 | 147 | 0.044 | -18 | -59 | -1 |
| Gasoline | CO2 | 0 | 153 | 20 | 20 | 0.039 | * | * | * |
|  | CH4 | 0 | 9 | 58 | 103 | 0.012 | * | * | * |
|  | N2O | 0 | 1 | 71 | 150 | 0.002 | * | * | * |
| e. Other transportation (other off-road-machinery) |  |  |  |  |  |  |  |  |  |
| Greenhouse gas source and sink categories | Gas | Base year (1990) emissions | Year t (2001) emissions | Uncertainty in year 2001 emissions as % of emissions in the category |  | Uncertainty introduced on national total in year 2001 | % change in emissions between year t and base year | Range of likely % change between year t and base year |  |
|  |  | Gg CO2 equivalent | Gg CO2 equivalent | % below (2.5 percentile) | % above (97.5 percentile) | % | % | Lower % (2.5 percentile) | Upper % (97.5 percentile) |
| Gasoline | CO2 | 74 | 97 | 30 | 30 | 0.037 | 30 | -19 | 79 |
|  | CH4 | 0 | 1 | 54 | 64 | 0.001 | 2 187 | 1 004 | 3 595 |
|  | N2O | 1 | 1 | 72 | 154 | 0.002 | -17 | -79 | 22 |
| Diesel | CO2 | 469 | 609 | 5 | 5 | 0.040 | 30 | 22 | 38 |
|  | CH4 | 0 | 6 | 49 | 50 | 0.004 | 2 187 | 1 111 | 3 283 |
|  | N2O | 74 | 61 | 91 | 55 | 0.070 | -17 | -32 | 4 |
| 1.A.4 Other sectors |  |  |  |  |  |  |  |  |  |
| Liquid fuels | CO2 | 7 274 | 5 662 | 30 | 30 | 2.168 | -22 | -60 | 16 |
|  | CH4 | 19 | 16 | 75 | 40 | 0.015 | -18 | -60 | 21 |
|  | N2O | 201 | 183 | 77 | 40 | 0.178 | -9 | -53 | 33 |
| Solid fuels | CO2 | 57 | 8 | 11 | 11 | 0.001 | -85 | -96 | -75 |
|  | CH4 | 0 | 1 | 75 | 20 | 0.001 | 440 | 107 | 540 |
|  | N2O | 0 | 0 |  |  |  | -100 | -152 | -50 |
| Gaseous fuels | CO2 | 99 | 274 | 5 | 5 | 0.018 | 178 | 164 | 193 |
|  | CH4 | 0 | 0 | 76 | 15 | 0.000 | 150 | 35 | 178 |
|  | N2O | 1 | 2 | 50 | 50 | 0.001 | 150 | 75 | 228 |
| Biomass | CH4 | 245 | 306 | 70 | 151 | 0.588 | 25 | -2 | 81 |
|  | N2O | 24 | 60 | 71 | 152 | 0.116 | 149 | 42 | 380 |
| Other fuels | CO2 | 141 | 121 | 30 | 33 | 0.051 | -14 | -48 | 19 |
|  | CH4 | 5 | 1 | 52 | 60 | 0.001 | -76 | -126 | -34 |
|  | N2O | 1 | 2 | 71 | 155 | 0.003 | 25 | -16 | 96 |
| 1.A.5 Other |  |  |  |  |  |  |  |  |  |
| Liquid fuels | CO2 | 736 | 1 050 | 50 | 50 | 0.666 | 43 | -44 | 130 |
|  | CH4 | 2 | 2 | 79 | 61 | 0.002 | 6 | -69 | 82 |
|  | N2O | 6 | 12 | 78 | 61 | 0.012 | 90 | -17 | 209 |
| Gaseous fuels | CO2 | 236 | 305 | 25 | 25 | 0.096 | 29 | -12 | 70 |
|  | CH4 | 0 | 0 | 75 | 35 | 0.000 | 14 | -25 | 55 |
|  | N2O | 1 | 2 | 53 | 59 | 0.001 | 25 | -15 | 73 |
| Indirect N2O from fuel combustion | N2O | 462 | 312 | 79 | 100 | 0.396 | -32 | -147 | 71 |
| 1.B Fugitive emissions from fuels |  |  |  |  |  |  |  |  |  |
| 1.B.1 Solid fuels |  |  |  |  |  |  |  |  |  |
| Greenhouse gas source and sink categories | Gas | Base year (1990) emissions | Year t (2001) emissions | Uncertainty in year 2001 emissions as % of emissions in the category |  | Uncertainty introduced on national total in year 2001 | % change in emissions between year t and base year | Range of likely % change between year t and base year |  |
|  |  | Gg CO2 equivalent | Gg CO2 equivalent | % below (2.5 percentile) | % above (97.5 percentile) | % | % | Lower % (2.5 percentile) | Upper % (97.5 percentile) |
| Arable peatlands | CO2 | 2 500 | 2 500 | 68 | 132 | 4.189 | 0 | -14 | 14 |
| Peat production areas | CO2 | 1 000 | 1 000 | 80 | 207 | 2.630 | 0 | -22 | 20 |
|  | CH4 | 21 | 21 | 80 | 202 | 0.054 | 0 | -24 | 22 |
| 1.B.2 Oil and natural gas | CO2 | 42 | 25 | 22 | 23 | 0.007 | -41 | -57 | -28 |
|  | CH4 | 4 | 19 | 22 | 23 | 0.006 | 434 | 335 | 540 |
| 2. Industrial processes |  |  |  |  |  |  |  |  |  |
| 2.A.1 Cement production | CO2 | 777 | 625 | 7 | 7 | 0.056 | -20 | -26 | -13 |
| 2.A.2 Lime production | CO2 | 398 | 417 | 11 | 11 | 0.060 | 5 | -10 | 20 |
| 2.B.2 Nitric acid production | N2O | 1 598 | 1 262 | 57 | 99 | 1.585 | -21 | -48 | -7 |
| 2.B.5 Other | CH4 | 4 | 5 | 21 | 21 | 0.001 | 39 | 27 | 51 |
| 2.C Iron and steel production | CH4 | 5 | 10 | 20 | 20 | 0.002 | 87 | 68 | 105 |
| 2.F.1 Refrigeration and air conditioning equipment | HFCs | 0 | 486 | 73 | 44 | 0.450 | * | * | * |
| 2.F.2 Foam blowing | HFCs | 0 | 52 | 26 | 25 | 0.017 | * | * | * |
| 2.F.4 Aerosols | HFCs | 0 | 74 | 4 | 4 | 0.004 | * | * | * |
| 2.F.7 Electrical equipment | SF6 | 87 | 36 | 7 | 12 | 0.006 | -59 | -109 | -8 |
| 2.F Other (grouped data) | HFCs, PFCs, SF6 | 8 | 20 | 39 | 39 | 0.010 | 157 | 46 | 267 |
| 3. Total solvent and other product use | N2O | 62 | 62 | 34 | 38 | 0.030 | 0 | -43 | 43 |
| 4. Agriculture |  |  |  |  |  |  |  |  |  |
| 4.A Enteric fermentation | CH4 | 1 868 | 1 565 | 30 | 30 | 0.602 | -16 | -23 | -10 |
| 4.B Manure management | CH4 | 200 | 202 | 17 | 16 | 0.042 | 1 | -3 | 5 |
| 4.B Manure management | N2O | 562 | 393 | 82 | 35 | 0.409 | -30 | -56 | -2 |
| 4.D Agricultural soils | N2O | 4 291 | 3 349 | 59 | 76 | 3.240 | -22 | -57 | -1 |
| 6. Waste |  |  |  |  |  |  |  |  |  |
| 6.A Solid waste disposal on land | CH4 | 3 679 | 2 901 | 29 | 31 | 1.157 | -21 | -62 | 17 |
| 6.B.1 Industrial wastewater | CH4 | 22 | 19 | 56 | 108 | 0.026 | -13 | -45 | 6 |
| 6.B.2 Domestic and commercial wastewater | CH4 | 132 | 111 | 55 | 109 | 0.153 | -16 | -40 | -4 |
| 6.B.2 Domestic and commercial wastewater |  |  |  |  |  |  |  |  |  |
| sparsely pop. areas | N2O | 21 | 17 | 92 | 370 | 0.081 | -19 | -106 | 10 |
| densely pop. areas | N2O | 85 | 69 | 92 | 378 | 0.332 | -19 | -102 | 6 |
| Greenhouse gas source and sink categories | Gas | Base year (1990) emissions | Year t (2001) emissions | Uncertainty in year 2001 emissions as % of emissions in the category |  | Uncertainty introduced on national total in year 2001 | % change in emissions between year t and base year | Range of likely % change between year t and base year |  |
|  |  | Gg CO2 equivalent | Gg CO2 equivalent | % below (2.5 percentile) | % above (97.5 percentile) | % | % | Lower % (2.5 percentile) | Upper % (97.5 percentile) |
| 6.B.3 N input from fish farming | N2O | 8 | 5 | 92 | 397 | 0.023 | -44 | -221 | -3 |
| 6.B.3 N input from ind. wastewater | N2O | 27 | 18 | 92 | 396 | 0.091 | -32 | -176 | -1 |
| 7. Other – non-energy use of fuels | CO2 | 640 | 690 | 50 | 50 | 0.439 | 8 | -66 | 81 |
| TOTAL |  | 74 058 | 78 853 | 5 | 6 | 6.383 | 6 | 2 | 11 |

*Trend not calculated, when base year emissions ≈ 0

TABLE B. Tier 1 uncertainty reporting, columns A-M.

| A | B | C | D | E | F | G | H | I | J | K | L | M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPCC greenhouse gas source and sink categories | Direct greenhouse gas | Base year emissions, 1990 | Current year emissions, 2001 | Activity data uncertainty | Emission factor uncertainty1 | Combined uncertainty | Combined uc as part of total national emissions in 2001 | Type A sensitivity | Type B sensitivity | Uncertainty in trend in national emissions introduced by emission factor uncertainty | Uncertainty in trend in national emissions introduced by activity data uncertainty | Uncertainty introduced into the trend in total national emissions |
| 1.A Fuel combustion activities |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.A.1 Energy industries |  |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | CO2 | 2 607 | 2 881 | 2% | 2% | 3% | 0.00103 | 0.00143 | 0.03891 | 0.00003 | 0.00110 | 0.00110 |
|  | CH4 | 6 | 6 | 2% | 75% | 75% | 0.00006 | 0.00000 | 0.00008 | 0.00000 | 0.00000 | 0.00000 |
|  | N2O | 26 | 30 | 2% | 75% | 75% | 0.00029 | 0.00004 | 0.00041 | 0.00003 | 0.00001 | 0.00003 |
| Solid fuels | CO2 | 9 279 | 10 211 | 2% | 3% | 4% | 0.00467 | 0.00446 | 0.13788 | 0.00013 | 0.00390 | 0.00390 |
|  | CH4 | 9 | 10 | 2% | 75% | 75% | 0.00009 | 0.00001 | 0.00013 | 0.00001 | 0.00000 | 0.00001 |
|  | N2O | 85 | 105 | 2% | 50% | 50% | 0.00067 | 0.00020 | 0.00142 | 0.00010 | 0.00004 | 0.00011 |
| Gaseous fuels | CO2 | 2 659 | 5 882 | 1% | 1% | 1% | 0.00105 | 0.04118 | 0.07943 | 0.00041 | 0.00112 | 0.00120 |
|  | CH4 | 4 | 8 | 1% | 75% | 75% | 0.00007 | 0.00005 | 0.00010 | 0.00004 | 0.00000 | 0.00004 |
|  | N2O | 18 | 42 | 1% | 50% | 50% | 0.00027 | 0.00031 | 0.00057 | 0.00016 | 0.00001 | 0.00016 |
| Biomass | CH4 | 2 | 11 | 20% | 50% | 54% | 0.00008 | 0.00012 | 0.00015 | 0.00006 | 0.00004 | 0.00008 |
|  | N2O | 10 | 73 | 20% | 150% | 151% | 0.00140 | 0.00084 | 0.00098 | 0.00127 | 0.00028 | 0.00130 |
| Other fuels | CO2 | 3 972 | 7 679 | 5% | 5% | 7% | 0.00689 | 0.04656 | 0.10369 | 0.00233 | 0.00733 | 0.00769 |
|  | CH4 | 5 | 6 | 5% | 50% | 50% | 0.00004 | 0.00000 | 0.00008 | 0.00000 | 0.00001 | 0.00001 |
|  | N2O | 141 | 180 | 5% | 150% | 150% | 0.00342 | 0.00040 | 0.00243 | 0.00060 | 0.00017 | 0.00062 |
| 1.A.2 Manufacturing industries and construction |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | CO2 | 4 294 | 4 374 | 2% | 2% | 3% | 0.00157 | -0.00266 | 0.05907 | -0.00005 | 0.00167 | 0.00167 |
|  | CH4 | 8 | 40 | 2% | 75% | 75% | 0.00038 | 0.00044 | 0.00054 | 0.00033 | 0.00002 | 0.00033 |
|  | N2O | 83 | 152 | 2% | 75% | 75% | 0.00144 | 0.00086 | 0.00205 | 0.00064 | 0.00006 | 0.00065 |
| Solid fuels | CO2 | 6 410 | 5 476 | 2% | 3% | 4% | 0.00250 | -0.01820 | 0.07394 | -0.00055 | 0.00209 | 0.00216 |
|  | CH4 | 4 | 3 | 2% | 75% | 75% | 0.00003 | -0.00002 | 0.00004 | -0.00002 | 0.00000 | 0.00002 |
|  | N2O | 108 | 97 | 2% | 50% | 50% | 0.00062 | -0.00023 | 0.00131 | -0.00012 | 0.00004 | 0.00012 |
| Gaseous fuels | CO2 | 2 094 | 2 294 | 1% | 1% | 1% | 0.00041 | 0.00087 | 0.03097 | 0.00001 | 0.00044 | 0.00044 |
|  | CH4 | 5 | 6 | 1% | 75% | 75% | 0.00006 | 0.00002 | 0.00008 | 0.00001 | 0.00000 | 0.00001 |
|  | N2O | 17 | 19 | 1% | 50% | 50% | 0.00012 | 0.00001 | 0.00026 | 0.00001 | 0.00000 | 0.00001 |

| IPCC greenhouse gas source and sink categories | Direct greenhouse gas | Base year emissions, 1990 |  | Current year emissions, 2001 | Activity data uncertainty | Emission factor uncertainty1 | Combined uncertainty | Combined uc as part of total national emissions in 2001 | Type A sensitivity | Type B sensitivity | Uncertainty in trend in national emissions introduced by emission factor uncertainty | Uncertainty in trend in national emissions introduced by activity data uncertainty | Uncertainty introduced into the trend in total national emissions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Biomass | CH4 | 20 |  | 23 | 15% | 50% | 52% | 0.00015 | 0.00002 | 0.00031 | 0.00001 | 0.00007 | 0.00007 |
|  | N2O | 111 |  | 89 | 15% | 150% | 151% | 0.00170 | -0.00040 | 0.00120 | -0.00060 | 0.00025 | 0.00065 |
| Other fuels | CO2 | 1 561 |  | 1 656 | 5% | 5% | 7% | 0.00149 | -0.00008 | 0.02237 | 0.00000 | 0.00158 | 0.00158 |
|  | CH4 | 4 |  | 4 | 5% | 50% | 50% | 0.00002 | 0.00000 | 0.00005 | 0.00000 | 0.00000 | 0.00000 |
|  | N2O | 56 |  | 25 | 5% | 150% | 150% | 0.00049 | -0.00046 | 0.00034 | -0.00069 | 0.00002 | 0.00069 |
| 1.A.3 Transport |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a. Civil aviation | CO2 | 403 |  | 360 | 5% | 2% | 5% | 0.00025 | -0.00093 | 0.00486 | -0.00002 | 0.00034 | 0.00034 |
|  | CH4 | 11 |  | 0 | 5% | 100% | 100% | 0.00000 | -0.00015 | 0.00000 | -0.00015 | 0.00000 | 0.00015 |
|  | N2O | 57 |  | 5 | 5% | 150% | 150% | 0.00009 | -0.00075 | 0.00006 | -0.00113 | 0.00000 | 0.00113 |
| b. Road transportation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gasoline | CO2 | 6 202 |  | 5 202 | 1% | 2% | 2% | 0.00148 | -0.01890 | 0.07025 | -0.00038 | 0.00099 | 0.00106 |
|  | CH4 | 44 |  | 45 | 1% | 50% | 50% | 0.00029 | -0.00002 | 0.00061 | -0.00001 | 0.00001 | 0.00001 |
| Cars with catalytic converters | N2O | 35 |  | 348 | 1% | 150% | 150% | 0.00663 | 0.00419 | 0.00470 | 0.00629 | 0.00007 | 0.00629 |
| Cars without catalytic converters | N2O | 67 |  | 21 | 1% | 179% | 179% | 0.00047 | -0.00069 | 0.00028 | -0.00123 | 0.00000 | 0.00123 |
| Diesel | CO2 | 4 909 |  | 5 705 | 1% | 2% | 2% | 0.00162 | 0.00645 | 0.07703 | 0.00013 | 0.00109 | 0.00110 |
|  | CH4 | 15 |  | 6 | 1% | 50% | 50% | 0.00004 | -0.00014 | 0.00008 | -0.00007 | 0.00000 | 0.00007 |
|  | N2O | 78 |  | 77 | 1% | 200% | 200% | 0.00194 | -0.00009 | 0.00103 | -0.00018 | 0.00001 | 0.00018 |
| Natural gas | CO2 | 0 |  | 3 | 1% | 2% | 2% | 0.00000 | 0.00004 | 0.00004 | 0.00000 | 0.00000 | 0.00000 |
|  | CH4 | 0 |  | 1 | 1% | 50% | 50% | 0.00000 | 0.00001 | 0.00001 | 0.00001 | 0.00000 | 0.00001 |
|  | N2O | 0 |  | 0 | 1% | 150% | 150% | 0.00000 | 0.00000 | 0.00000 | 0.00000 | 0.00000 | 0.00000 |
| c. Railways | CO2 | 192 |  | 136 | 5% | 2% | 5% | 0.00009 | -0.00093 | 0.00183 | -0.00002 | 0.00013 | 0.00013 |
|  | CH4 | 0 |  | 0 | 5% | 110% | 110% | 0.00000 | 0.00000 | 0.00000 | 0.00000 | 0.00000 | 0.00000 |
|  | N2O | 25 |  | 17 | 5% | 150% | 150% | 0.00033 | -0.00013 | 0.00023 | -0.00019 | 0.00002 | 0.00019 |
| d. Navigation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Residual oil & gas/Diesel oil | CO2 | 227 |  | 311 | 10% | 2% | 10% | 0.00040 | 0.00094 | 0.00420 | 0.00002 | 0.00059 | 0.00059 |
|  | CH4 | 0 |  | 0 | 10% | 100% | 100% | 0.00001 | 0.00000 | 0.00001 | 0.00000 | 0.00000 | 0.00000 |
|  | N2O | 29 |  | 24 | 10% | 150% | 150% | 0.00045 | -0.00010 | 0.00032 | -0.00014 | 0.00005 | 0.00015 |
| Gasoline | CO2 | 0 |  | 153 | 20% | 2% | 20% | 0.00039 | 0.00206 | 0.00206 | 0.00004 | 0.00058 | 0.00058 |
| IPCC greenhouse gas source and sink categories | Direct greenhouse gas | Base year emissions, 1990 |  | Current year emissions, 2001 | Activity data uncertainty | Emission factor uncertainty1 | Combined uncertainty | Combined uc as part of total national emissions in 2001 | Type A sensitivity | Type B sensitivity | Uncertainty in trend in national emissions introduced by emission factor uncertainty | Uncertainty in trend in national emissions introduced by activity data uncertainty | Uncertainty introduced into the trend in total national emissions |
|  | CH4 | 0 |  | 9 | 20% | 100% | 102% | 0.00011 | 0.00012 | 0.00012 | 0.00012 | 0.00003 | 0.00012 |
|  | N2O | 0 |  | 1 | 20% | 150% | 151% | 0.00002 | 0.00002 | 0.00002 | 0.00003 | 0.00000 | 0.00003 |
| e. Other transportation (other off-road-machinery) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gasoline | CO2 |  | 74 | 97 | 30% | 2% | 30% | 0.00037 | 0.00024 | 0.00130 | 0.00000 | 0.00055 | 0.00055 |
|  | CH4 |  | 0 | 1 | 30% | 50% | 58% | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
|  | N2O |  | 1 | 1 | 30% | 150% | 153% | 0.00002 | 0.00000 | 0.00001 | 0.00000 | 0.00000 | 0.00001 |
| Diesel | CO2 |  | 469 | 609 | 30% | 2% | 30% | 0.00232 | 0.00148 | 0.00822 | 0.00003 | 0.00349 | 0.00349 |
|  | CH4 |  | 0 | 6 | 30% | 50% | 58% | 0.00005 | 0.00008 | 0.00008 | 0.00004 | 0.00004 | 0.00005 |
|  | N2O |  | 74 | 61 | 30% | 90% | 95% | 0.00073 | -0.00023 | 0.00082 | -0.00021 | 0.00035 | 0.00041 |
| 1.A.4 Other sectors |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | CO2 |  | 7 274 | 5 662 | 30% | 2% | 30% | 0.02159 | -0.02811 | 0.07645 | -0.00056 | 0.03243 | 0.03244 |
|  | CH4 |  | 19 | 16 | 30% | 75% | 81% | 0.00016 | -0.00006 | 0.00021 | -0.00005 | 0.00009 | 0.00010 |
|  | N2O |  | 201 | 183 | 30% | 75% | 81% | 0.00187 | -0.00042 | 0.00247 | -0.00031 | 0.00105 | 0.00109 |
| Solid fuels | CO2 |  | 57 | 8 | 10% | 5% | 11% | 0.00001 | -0.00070 | 0.00011 | -0.00004 | 0.00002 | 0.00004 |
|  | CH4 |  | 0 | 1 | 10% | 75% | 76% | 0.00001 | 0.00001 | 0.00001 | 0.00000 | 0.00000 | 0.00000 |
|  | N2O |  | 0 | 0 | 10% | 0% | 10% | 0.00000 | 0.00000 | 0.00000 | 0.00000 | 0.00000 | 0.00000 |
| Gaseous fuels | CO2 |  | 99 | 274 | 5% | 1% | 5% | 0.00018 | 0.00229 | 0.00370 | 0.00002 | 0.00026 | 0.00026 |
|  | CH4 |  | 0 | 0 | 5% | 75% | 75% | 0.00000 | 0.00000 | 0.00000 | 0.00000 | 0.00000 | 0.00000 |
|  | N2O |  | 1 | 2 | 5% | 50% | 50% | 0.00001 | 0.00001 | 0.00002 | 0.00001 | 0.00000 | 0.00001 |
| Biomass | CH4 |  | 245 | 306 | 15% | 150% | 151% | 0.00586 | 0.00062 | 0.00414 | 0.00092 | 0.00088 | 0.00127 |
|  | N2O |  | 24 | 60 | 15% | 150% | 151% | 0.00115 | 0.00047 | 0.00081 | 0.00070 | 0.00017 | 0.00072 |
| Other fuels | CO2 |  | 141 | 121 | 25% | 20% | 32% | 0.00049 | -0.00039 | 0.00164 | -0.00008 | 0.00058 | 0.00058 |
|  | CH4 |  | 5 | 1 | 25% | 50% | 56% | 0.00001 | -0.00005 | 0.00002 | -0.00003 | 0.00001 | 0.00003 |
|  | N2O |  | 1 | 2 | 25% | 150% | 152% | 0.00003 | 0.00000 | 0.00002 | 0.00000 | 0.00001 | 0.00001 |
| 1.A.5 Other |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Liquid fuels | CO2 |  | 736 | 1 050 | 50% | 2% | 50% | 0.00666 | 0.00359 | 0.01418 | 0.00007 | 0.01002 | 0.01002 |
|  | CH4 |  | 2 | 2 | 50% | 75% | 90% | 0.00003 | 0.00000 | 0.00003 | 0.00000 | 0.00002 | 0.00002 |
|  | N2O |  | 6 | 12 | 50% | 75% | 90% | 0.00013 | 0.00007 | 0.00016 | 0.00005 | 0.00011 | 0.00012 |
| Gaseous fuels | CO2 |  | 236 | 305 | 25% | 1% | 25% | 0.00097 | 0.00072 | 0.00412 | 0.00001 | 0.00146 | 0.00146 |
| IPCC greenhouse gas source and sink categories | Direct greenhouse gas | Base year emissions, 1990 |  | Current year emissions, 2001 | Activity data uncertainty | Emission factor uncertainty1 | Combined uncertainty | Combined uc as part of total national emissions in 2001 | Type A sensitivity | Type B sensitivity | Uncertainty in trend in national emissions introduced by emission factor uncertainty | Uncertainty in trend in national emissions introduced by activity data uncertainty | Uncertainty introduced into the trend in total national emissions |
|  | CH4 |  | 0 | 0 | 25% | 75% | 79% | 0.00000 | 0.00000 | 0.00000 | 0.00000 | 0.00000 | 0.00000 |
|  | N2O |  | 1 | 2 | 25% | 50% | 56% | 0.00001 | 0.00000 | 0.00002 | 0.00000 | 0.00001 | 0.00001 |
| Indirect N2O from fuel combustion | N2O |  | 462 | 312 | 100% | 0% | 100% | 0.00396 | -0.00242 | 0.00422 | 0.00000 | 0.00596 | 0.00596 |
| 1.B Fugitive emissions from fuels |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.B.1 Solid fuels |  |  |  |  |  |  |  |  |  |  |  |  |  |
| arable peatlands | CO2 |  | 2 500 | 2 500 | 109% | 50% | 120% | 0.03808 | -0.00218 | 0.03376 | -0.00109 | -0.00239 | 0.00262 |
| peat production areas | CO2 |  | 1 000 | 1 000 | 10% | 208% | 208% | 0.02634 | -0.00087 | 0.01350 | -0.00181 | -0.00009 | 0.00182 |
|  | CH4 |  | 21 | 21 | 10% | 208% | 208% | 0.00055 | -0.00002 | 0.00028 | -0.00004 | 0.00000 | 0.00004 |
| 1.B.2 Oil and natural gas | CO2 |  | 42 | 25 | 10% | 20% | 22% | 0.00007 | -0.00027 | 0.00033 | -0.00005 | 0.00005 | 0.00007 |
|  | CH4 |  | 4 | 19 | 10% | 20% | 22% | 0.00005 | 0.00021 | 0.00026 | 0.00004 | 0.00004 | 0.00006 |
| 2. Industrial processes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.A.1 Cement production | CO2 |  | 777 | 625 | 5% | 5% | 7% | 0.00056 | -0.00275 | 0.00843 | -0.00014 | 0.00060 | 0.00061 |
| 2.A.2 Lime production | CO2 |  | 398 | 417 | 10% | 5% | 11% | 0.00059 | -0.00009 | 0.00564 | 0.00000 | 0.00080 | 0.00080 |
| 2.B.2 Nitric acid production | N2O |  | 1 598 | 1 262 | 5% | 100% | 100% | 0.01602 | -0.00593 | 0.01703 | -0.00593 | 0.00120 | 0.00606 |
| 2.B.5 Other | CH4 |  | 4 | 5 | 5% | 20% | 21% | 0.00001 | 0.00002 | 0.00007 | 0.00000 | 0.00001 | 0.00001 |
| 2.C Iron and steel production | CH4 |  | 5 | 10 | 3% | 20% | 20% | 0.00002 | 0.00006 | 0.00013 | 0.00001 | 0.00001 | 0.00001 |
| 2.F.1 Refrigeration and air conditioning equipment | HFCs |  | 0 | 486 | 73% | 0% | 73% | 0.00449 | 0.00656 | 0.00656 | 0.00000 | 0.00676 | 0.00676 |
| 2.F.2 Foam blowing | HFCs |  | 0 | 52 | 26% | 0% | 26% | 0.00017 | 0.00071 | 0.00071 | 0.00000 | 0.00026 | 0.00026 |
| 2.F.4 Aerosols | HFCs |  | 0 | 74 | 4% | 0% | 4% | 0.00004 | 0.00100 | 0.00100 | 0.00000 | 0.00005 | 0.00005 |
| 2.F.7 Electrical equipment | SF6 |  | 87 | 36 | 12% | 0% | 12% | 0.00005 | -0.00076 | 0.00048 | 0.00000 | 0.00008 | 0.00008 |
| 2.F Other (grouped data) | HFCs, PFCs, SF6 |  | 8 | 20 | 39% | 0% | 39% | 0.00010 | 0.00016 | 0.00028 | 0.00000 | 0.00015 | 0.00015 |
| 3. Total Solvent and other product use | N2O |  | 62 | 62 | 30% | 20% | 36% | 0.00028 | -0.00005 | 0.00084 | -0.00001 | 0.00036 | 0.00036 |
| 4. Agriculture |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.A Enteric fermentation | CH4 |  | 1 868 | 1 565 | 30% | 0% | 30% | 0.00588 | -0.00573 | 0.02113 | 0.00000 | 0.00885 | 0.00885 |
| 4.B Manure management | CH4 |  | 200 | 202 | 16% | 0% | 16% | 0.00042 | -0.00014 | 0.00273 | 0.00000 | 0.00064 | 0.00064 |
| 4.B Manure management | N2O |  | 562 | 393 | 83% | 0% | 83% | 0.00412 | -0.00276 | 0.00531 | 0.00000 | 0.00620 | 0.00620 |
| 4.D Agricultural soils | N2O |  | 4 291 | 3 349 | 75% | 0% | 75% | 0.03190 | -0.01646 | 0.04522 | 0.00000 | 0.04804 | 0.04804 |
| 6. Waste |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IPCC greenhouse gas source and sink categories | Direct greenhouse gas | Base year emissions, 1990 |  | Current year emissions, 2001 | Activity data uncertainty | Emission factor uncertainty1 | Combined uncertainty | Combined uc as part of total national emissions in 2001 | Type A sensitivity | Type B sensitivity | Uncertainty in trend in national emissions introduced by emission factor uncertainty | Uncertainty in trend in national emissions introduced by activity data uncertainty | Uncertainty introduced into the trend in total national emissions |
| 6.A Solid waste disposal on land | CH4 |  | 3 679 | 2 901 | 32% | 0% | 32% | 0.01159 | -0.01372 | 0.03917 | 0.00000 | 0.01745 | 0.01745 |
| 6.B.1 Industrial wastewater | CH4 |  | 22 | 19 | 10% | 104% | 105% | 0.00026 | -0.00006 | 0.00026 | -0.00006 | 0.00004 | 0.00007 |
| 6.B.2 Domestic and commercial wastewater | CH4 |  | 132 | 111 | 5% | 104% | 105% | 0.00147 | -0.00041 | 0.00150 | -0.00043 | 0.00011 | 0.00044 |
| 6.B.2 Domestic and commercial wastewater |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sparsely pop. areas | N2O |  | 21 | 17 | 10% | 380% | 380% | 0.00083 | -0.00007 | 0.00023 | -0.00028 | 0.00003 | 0.00028 |
| densely pop. areas | N2O |  | 85 | 69 | 5% | 380% | 380% | 0.00333 | -0.00029 | 0.00093 | -0.00110 | 0.00007 | 0.00110 |
| 6.B.3 N input from fish farming | N2O |  | 8 | 5 | 10% | 380% | 380% | 0.00022 | -0.00005 | 0.00006 | -0.00021 | 0.00001 | 0.00021 |
| 6.B.3 N input from ind. wastewater | N2O |  | 27 | 18 | 5% | 380% | 380% | 0.00087 | -0.00014 | 0.00024 | -0.00054 | 0.00002 | 0.00054 |
| 7.Other – non-energy use of fuels | CO2 |  | 640 | 690 | 50% | 5% | 50% | 0.00440 | 0.00011 | 0.00932 | 0.00001 | 0.00659 | 0.00659 |
| TOTAL |  |  | 74 058 | 78 853 |  |  |  | 0.06624 |  |  |  |  | 0.06490 |

1When uncertainties are calculated with a separate model, resulting uncertainty in emissions is reported in column E, thus resulting in 0% in column F.

TABLE C. Tier 1 uncertainty reporting, columns A-B and N-Q.

| A | B | N | O | P | Q |
| --- | --- | --- | --- | --- | --- |
| IPCC Greenhouse Gas Source and Sink Categories | Direct Greenhouse Gas | Emission factor quality indicator | Activity data quality indicator | Expert judgement reference numbers1 | Footnote Reference numbers1 |
| 1.A. Fuel combustion activities |  |  |  |  |  |
| 1.A.1 Energy industries |  |  |  |  |  |
| Liquid fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 | M2 |
|  | N2O | R | R | E1 | M2 |
| Solid fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 | M2 |
|  | N2O | R | R | E1 |  |
| Gaseous fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 | M2 |
|  | N2O | R | R | E1 |  |
| Biomass | CH4 | R | R | E1 |  |
|  | N2O | R | R | E1 |  |
| Other fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 |  |
|  | N2O | R | R | E1 |  |
| 1.A.2 Manufacturing industries and construction |  |  |  |  |  |  |
| Liquid fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 | M2 |
|  | N2O | R | R | E1 | M2 |
| Solid fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 | M2 |
|  | N2O | R | R | E1 |  |
| Gaseous fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 |  |
|  | N2O | R | R | E1 |  |
| Biomass | CH4 | R | R | E1 |  |
|  | N2O | R | R | E1 |  |
| Other fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 |  |
|  | N2O | R | R | E1 |  |
| 1.A.3 Transport |  |  |  |  |  |
| a. Civil aviation | CO2 | R | R |  |  |
|  | CH4 | D | R |  | L4 |
|  | N2O | R | R |  |  |
| b. Road transportation |  |  |  |  |  |
| Gasoline | CO2 | R | R |  |  |
|  | CH4 | M | R |  | L5 |
| Cars with catalytic converters | N2O | M | R |  | L6, L7, L8, L9, L10 |
| Cars without cat. converters | N2O | M | R |  | L6, L9, L10 |
| Diesel | CO2 | R | R |  |  |
|  | CH4 | M | R |  | L5 |
|  | N2O | M | R |  | L6, L8, L11 |
| Natural gas | CO2 | R | R |  |  |
|  | CH4 | M | R |  | L5 |
|  | N2O | R | R |  |  |
| c. Railways | CO2 | M | R |  |  |
|  | CH4 | M | R |  | M3 |
|  | N2O | R | R |  | M3 |
| d. Navigation |  |  |  |  |  |
| Residual oil & gas/Diesel oil | CO2 | R | R |  |  |
| IPCC Greenhouse Gas Source and Sink Categories | Direct Greenhouse Gas | Emission factor quality indicator | Activity data quality indicator | Expert judgement reference numbers1 | Footnote Reference numbers1 |
|  | CH4 | D | R |  | L4 |
|  | N2O | R | R |  |  |
| Gasoline | CO2 | R | R |  |  |
|  | CH4 | D | R |  | L4 |
|  | N2O | R | R |  |  |
| e. Other transportation (other off-road-machinery) |  |  |  |  |  |  |
| Gasoline | CO2 | R | R |  |  |
|  | CH4 | R | R |  |  |
|  | N2O | R | R |  |  |
| Diesel | CO2 | R | R |  |  |
|  | CH4 | R | R |  |  |
|  | N2O | R | R |  |  |
| 1.A.4 Other sectors |  |  |  |  |  |
| Liquid fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 | M2 |
|  | N2O | R | R | E1 | M2 |
| Solid fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 | M2 |
|  | N2O | R | R | E1 |  |
| Gaseous fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 | M2 |
|  | N2O | R | R | E1 |  |
| Biomass | CH4 | R | R | E1 |  |
|  | N2O | R | R | E1 |  |
| Other fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 |  |
|  | N2O | R | R | E1 |  |
| 1.A.5 Other |  |  |  |  |  |
| Liquid fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 | M2 |
|  | N2O | R | R | E1 | M2 |
| Gaseous fuels | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 | M2 |
|  | N2O | R | R | E1 |  |
| Indirect N2O from fuel combustion | N2O | R | R |  | L1, M3, L2 |
| 1.B Fugitive emissions from fuels |  |  |  |  |  |
| 1.B.1 Solid fuels |  |  |  |  |  |
| arable peatlands | CO2 | R | R |  | L3 |
| peat production areas | CO2 | R | R |  |  |
|  | CH4 | R | R |  |  |
| 1.B.2 Oil and natural gas | CO2 | R | R | E1 |  |
|  | CH4 | R | R | E1 |  |
| 2. Industrial processes |  |  |  |  |  |
| 2.A.1 Cement production | CO2 | R | R | E1 |  |
| 2.A.2 Lime production | CO2 | R | R | E1 |  |
| 2.B.2 Nitric acid production | N2O | R | R |  | M1 |
| 2.B.5 Other | CH4 | R | R | E1 |  |
| 2.C Iron and steel production | CH4 | R | R | E1 |  |
| 2.F.1 Refrigeration and air conditioning equipment | HFCs | R | R | E2 |  |
| 2.F.2 Foam blowing | HFCs | R | R | E2 |  |
| 2.F.4 Aerosols | HFCs | R | R | E2 |  |
| 2.F.7 Electrical equipment | SF6 | R | R | E2 |  |
| IPCC Greenhouse Gas Source and Sink Categories | Direct Greenhouse Gas | Emission factor quality indicator | Activity data quality indicator | Expert judgement reference numbers1 | Footnote Reference numbers1 |
| 2.F Other (grouped data) | HFCs, PFCs, SF6 | R | R | E2 |  |
| 3. Total solvent and other product use | N2O | R | R | E1 |  |
| 4. Agriculture |  |  |  |  |  |
| 4.A Enteric fermentation | CH4 | D/R | R |  | L4, L13 |
| 4.B Manure management | CH4 | R | R |  |  |
| 4.B Manure management | N2O | R | R |  | L12, L14, L15, L16, L17, L4 |
| 4.D Agricultural soils | N2O | D/R | R |  | L2, L18 |
| 6. Waste |  |  |  |  |  |
| 6.A Solid waste disposal on land | CH4 | D/R | R | E3 | L4, M4 |
| 6.B.1 Industrial wastewater | CH4 | R | R | E3 | L4 |
| 6.B.2 Domestic and commercial wastewater | CH4 | R | R | E3 | L4 |
| 6.B.2 Domestic and commercial wastewater |  |  |  |  |  |
| sparsely pop. areas | N2O | R | R | E3 | L2 |
| densely pop. areas | N2O | R | R | E3 | L2 |
| 6.B.3 N input from fish farming | N2O | R | R | E3 | L2 |
| 6.B.3 N input from ind. wastewater | N2O | R | R | E3 | L2 |
| 7. Other – non-energy use of fuels | CO2 | R | R | E1 |  |

1 See Table D.

TABLE D. References of table C.

| Expert Elicitations |  |
| --- | --- |
| E1 | Grönfors, Kari and Äikäs, Mikko (Statistics Finland) 27 August 2002 |
| E2 | Oinonen, Teemu (Finnish Environment Institute) 21 November 2002 |
| E3 | Petäjä, Jouko (Finnish Environment Institute) 21 November 2002 |
| Measurement data |  |
| M1 | Confidential measurement data from nitric acid production plants. |
| M2 | Korhonen, S., Fabritius, M and Hoffren, H. (2001). Methane and nitrous oxide emissions in the Finnish energy production. Fortum Power and Heat Oy. TECH-4615. Helsinki. |
| M3 | Korhonen, R. and Määttänen, M. (1999). To solve the specific emissions of locomotive diesel engines, Final Report. MOBILE 237T-1. Kymenlaakso Polytechnic, Kotka. 15 pp. |
| M4 | Leinonen, S. and Kuittinen, V. (2001). Finnish Biogas Plant Register. Data from year 2000. University of Joensuu, Karelian Institute working papers 4/2001. (In Finnish). |
| Literature |  |
| L1 | Rypdal, K. (2002).Uncertainties in the Norwegian emission inventories of acidifying pollutants and volatile organic compounds. Pages 233–246 in Environmental Science and Policy 5 (2002). |
| L2 | IPCC 1996a. Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories. Reference manual. http://www.ipcc-nggip.iges.or.jp/public/gl/invs6c.htm. |
| L3 | Minkkinen, K. and Laine, J. (2001). Survey of further development needs of the greenhouse gas effects of the use of peat. Report, Ministry of Trade and Industry. Helsinki. 56 p. (In Finnish). |
| L4 | Penman, J., Kruger, D., Galbally, I., Hiraishi, T., Nyenzi, B., Emmanuel, S., Buendia, L., Hoppaus, R., Martinsen, T., Meijer, J., Miwa, K. and Tanabe, K. (2000). Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Hayama: Intergovernmental Panel on Climate Change (IPCC). |
| L5 | Tarantola, S. and Kioutsioukis, I. (2001). The JRC-IPCS in the ARETEMIS project: summary of the second year of activity. Institute for the Protection and Security of the Citizen. Technological and Economic Risk Management. I-21020 Ispra (VA), Italy. |
| L6 | Pringent M. and de Soete, G. (1989). Nitrous Oxide N2O in engines exhaust gases – A first appraisal of catalyst impact – SAE Technical Paper Series 890792. |
| L7 | Potter, D. (1990). N2O emissions from cars with catalyst. – Report OOK 90:02. Chalmers University of Technology and University of Gothenburg. ISSN 0283-8575. (In Swedish). |
| L8 | Becker, K.H., Lörzer, J.C., Kurtenbach, R. and Wiesen, P. (1999). Nitrous Oxide (N2O) Emissions from Vehicles. Pages 4134–4139 in Environmental Science and Technology. Vol. 33 No. 22. American Chemical Society. |
| L9 | Perby, H. (1990). N2O emissions from road traffic. Preliminary emission factors and budget assesments. VTI bulletin 629. National Road and Traffic Institute. Linköping. ISSN 0347-6049. (In Swedish). |
| L10 | Egebäck, K. E. and Bertilsson, B. M. (1983). Chemical and biological characterization of exhaust emissions from vehicles fuelled with gasoline, alcohol, LPG and diesel. SNV pm 1635. |
| L11 | Sjöberg, K., Lindskog, A., Rosen, Å and Sundström, L. (1989). N2O-emissions from motor vehicles. TFB bulletin nr 75. (In Swedish). |
| L12 | Finnish Grassland Society. http://www.agronet.fi/nurmiyhdistys/ |
| L13 | Nieminen, M., Maijala, V. and Soveri, T. (1998). Reindeer feeding. Finnish Game and Fisheries Research Institute. (In Finnish). |
| L14 | Dustan, A. (2002). Review of methane and nitrous oxide emissions factors for manure management in cold climates. JTI-rapport 299. Institutet för jordbruks- och miljöteknik. ISSN 1401-4963. |
| L15 | Amon, B., Amon, T:, Boxberger, J. and Alt, C. (2001). Emissions of NH3, N2O and CH4 from dairy cows housed in a farmyard manure tying stall (housing, manure storage, manure spreading). Nutrient Cycling in in Agroecosystems 60:103–113. |
| L16 | Hüther, L. (1999). Entvicklung analytischer Methoden und untersuchung von Einflussfactoren auf Ammoniak-, Methan- und Distickskstoffmonoxidemissionen aud Flüssing- und Festmist. Landbauforschung Völkenrode, Sonderheft 200. |
| L17 | Amon, B., Boxberger, J., Amon, T., Zaussinger, A. and Pöllinger, A. (1997). Emission data of NH3, N2O and CH4 from fattening bulls, milking cows and during different ways of storing liquid manure. Proc. Int. Symp. Ammonia and Odour Control from Animal Production Facilities. 6–10 October 1997, Vinkeloors, The Netherlands. |
| L18 | IPCC 1996b. Revised IPCC Guidelines for National Greenhouse Gas Inventories. Workbook. http://www.ipcc-nggip.iges.or.jp/public/gl/invs5c.htm |

TABLE E. Key Source Category Analysis summary.

| Quantitative method used: Tier 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| A | B | C | D | E |
| IPCC source categories | Direct greenhouse gas | Key source categoty flag (Yes or No) | If C is Yes, criteria for identification | Comments |
| 1.A. Fuel combustion activities |  |  |  |  |
| 1.A.1 Energy industries |  |  |  |  |
| Liquid fuels | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Solid fuels | CO2 | Yes | Level |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Gaseous fuels | CO2 | Yes | Trend |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Biomass | CH4 | No |  |  |
|  | N2O | Yes | Trend |  |
| Other fuels | CO2 | Yes | Level, Trend |  |
|  | CH4 | No |  |  |
|  | N2O | Yes | Level, Trend |  |
| 1.A.2 Manufacturing industries and construction |  |  |  |  |
| Liquid fuels | CO2 | Yes | Level |  |
|  | CH4 | No |  |  |
|  | N2O | Yes | Trend |  |
| Solid fuels | CO2 | Yes | Level, Trend |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Gaseous fuels | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Biomass | CH4 | No |  |  |
|  | N2O | Yes | Level, Trend |  |
| Other fuels | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | Yes | Trend |  |
| 1.A.3 Transport |  |  |  |  |
| a. Civil aviation | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | Yes | Trend |  |
| b. Road transportation |  |  |  |  |
| Gasoline | CO2 | No |  |  |
|  | CH4 | No |  |  |
| Cars with catalytic converters | N2O | Yes | Level, Trend |  |
| Cars without catalytic converters | N2O | Yes | Trend |  |
| Diesel | CO2 | Yes | Level |  |
|  | CH4 | No |  |  |
|  | N2O | Yes | Level |  |
| Natural gas | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| c. Railways | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| d. Navigation |  |  |  |  |
| Residual oil & gas/Diesel oil | CO2 | No |  |  |
| IPCC source categories | Direct greenhouse gas | Key source categoty flag (Yes or No) | If C is Yes, criteria for identification | Comments |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Gasoline | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| e. Other transportation (other off-road-machinery) |  |  |  |  |
| Gasoline | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Diesel | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| 1.A.4 Other sectors |  |  |  |  |
| Liquid fuels | CO2 | Yes | Level, Trend |  |
|  | CH4 | No |  |  |
|  | N2O | Yes | Level |  |
| Solid fuels | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Gaseous fuels | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Biomass | CH4 | Yes | Level, Trend |  |
|  | N2O | Yes | Trend |  |
| Other fuels | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| 1.A.5 Other |  |  |  |  |
| Liquid fuels | CO2 | Yes | Level, Trend |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Gaseous fuels | CO2 | No |  |  |
|  | CH4 | No |  |  |
|  | N2O | No |  |  |
| Indirect N2O from fuel combustion | N2O | Yes | Level, Trend |  |
| 1.B Fugitive emissions from fuels |  |  |  |  |
| 1.B.1 Solid fuels |  |  |  |  |
| Arable peatlands | CO2 | Yes | Level, Trend |  |
| Peat production areas | CO2 | Yes | Level, Trend |  |
|  | CH4 | No |  |  |
| 1.B.2 Oil and natural gas | CO2 | No |  |  |
|  | CH4 | No |  |  |
| 2. Industrial Processes |  |  |  |  |
| 2.A.1 Cement production | CO2 | No |  |  |
| 2.A.2 Lime production | CO2 | No |  |  |
| 2.B.2 Nitric acid production | N2O | Yes | Level, Trend |  |
| 2.B.5 Other | CH4 | No |  |  |
| 2.C Iron and steel production | CH4 | No |  |  |
| 2.F.1 Refrigeration and air conditioning equipment | HFCs | Yes | Level, Trend |  |
| 2.F.2 Foam blowing | HFCs | No |  |  |
| 2.F.4 Aerosols | HFCs | No |  |  |
| 2.F.7 Electrical equipment | SF6 | No |  |  |
| 2.F Other (grouped data) | HFCs, PFCs, SF6 | No |  |  |
| 3. Total solvent and other product use | N2O | No |  |  |
| IPCC source categories | Direct greenhouse gas | Key source categoty flag (Yes or No) | If C is Yes, criteria for identification | Comments |
| 4. Agriculture |  |  |  |  |
| 4.A Enteric fermentation | CH4 | Yes | Level, Trend |  |
| 4.B Manure management | CH4 | No |  |  |
| 4.B Manure management | N2O | Yes | Level, Trend |  |
| 4.D Agricultural soils | N2O | Yes | Level, Trend |  |
| 6. Waste |  |  |  |  |
| 6.A Solid waste disposal on land | CH4 | Yes | Level, Trend |  |
| 6.B.1 Industrial wastewater | CH4 | No |  |  |
| 6.B.2 Domestic and commercial wastewater | CH4 | Yes | Level, Trend |  |
| 6.B.2 Domestic and commercial wastewater |  |  |  |  |
| sparsely pop. areas | N2O | No |  |  |
| densely pop. areas | N2O | Yes | Level |  |
| 6.B.3 N input from fish farming | N2O | No |  |  |
| 6.B.3 N input from ind. wastewater | N2O | No |  |  |
| 7. Other – non-energy use of fuels | CO2 | Yes | Level |  |