# Belgium

## Sources of Information

- Belgium’s Third National Communication under the United Nations Framework Convention on Climate Change, April 2002.

- Projections of GHG Emissions by 2010 for Belgium, report to the Monitoring Mechanism December 2003.

## Quality and Transparency of Reporting

The information on policies and measures is taken from the Third National Communication. Existing policies and measures are well described but their effect is not disaggregated.  The with measures projections are taken from the report to the Monitoring Mechanism in 2003.  Additional technical measures for CO2 reductions are listed in the report to the monitoring mechanism and their incorporation into the model described.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - +++ |  |
| - Objectives of policies | - +++ |  |
| - Which GHGs? | - CO2, (CH4 and N2O with measures only) |  |
| - Status of Implementation | - ++ |  |
| - Implementation body specified | - +++ |  |
| - Quantitative assessment of implementation | - + | - Only aggregated |
| - Interaction with other P&Ms discussed | - - | - Not included |

- With measures and with additional measures projections are given for CO2.  For the other gases, only a with measures projection is given.  The existing policies are those implemented since 1990.  The projections are to the year 2020 and are disaggregated by gas and four sectors: energy; industrial process, agriculture and waste.  The energy-related emissions are further disaggregated to: energy transformation; industry; residential and service; and transport.  Industrial process emissions are also disaggregated.

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - With measures - With additional measures |  |
| - Expressed relative to inventory for previous years | - Yes |  |
| - Starting year | - 2000 |  |
| - Split of projections | - +++ |  |
| - Presentation of results | - ++ | - It is difficult from the tables presented to understand the overall picture. |
| - Description of model (level of detail, approach and assumptions) | - +++ |  |
| - Discussion of uncertainty | - - | - The fact that two models (Hermes and Markal) were used for the projection period up to 2010 provides some insights on the uncertainty in projections |
| - Details of parameters and assumptions | - +++ | - Good level of detail on model parameters – policies and measures less detail (Table 1) |

## Assessment of Policies and Measures

The effect of the policies and measures has not been quantified separately from the effect of parameter assumptions on the projections.  For additional measures, it has been assumed here that the difference between the two scenarios is the result of the additional measures.  The additional measures are directed towards reduction of energy use, the main effects are in energy transformation and manufacturing and construction.  Table 3 summarises the effect of policies and measures.  In Table 4, details of policies and measures are given.  The policies in the first part of the table are those identified as being included in the with measures projections.  The second part of the table contains the details given in the third national communication.   It is not clear which of the measures in Table 2 are included explicitly in the projections.

- Table 3: Summary of the Effect of additional Policies and Measures in relation to with measures projections Included in the Projections (MtCO2 equivalents)

|  | With measures | With additional measures |
| --- | --- | --- |
| Energy transformation |  | 5.7 |
| Manufacturing and construction |  | 5.1 |
| Transport |  | 0.7 |
| Residential and service |  | 2.2 |
| Industrial processes |  | 0.1 |
| Agriculture |  | - |
| Waste |  | - |
| Total |  | 13.8 |

- Table 4: Detailed Information on Polices and Measures

| Sector | Name | Objective | GHG affected | Type of instrument | Status | Implementing entity | Estimate of savings (MtCO2) |  | ### CCPM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2010 | 2020 |  |
| Policies and measure in the with measures projection |  |  |  |  |  |  |  |  |  |
| Residential and service sector | Improving thermal insulation of new buildings in the housing and service sectors | To reduce energy losses by 20% | CO2 |  | Implemented | Government and regions |  |  | n |
| Residential and service sector | Promoting the use of energy-efficient household appliances and lighting | Reduction of energy use | CO2 | Education and labelling | Implemented | Government and regions |  |  | In part (Labelling Directive) |
| Residential and service sector | K55 insulation level for new buildings in the residential sector |  | CO2 |  | Implemented |  |  |  |  |
| Residential and service sector | Insulation standards for the service sector |  | CO2 |  | Implemented |  |  |  | n |
| Residential and service sector | Subsidies for highly efficient bulbs through agreement with the electricity producing and distributing companies |  | CO2 | Economic | Implemented |  |  |  | n |
| Energy transformation | Penetration of renewables | To give priority to renewable energy sources | CO2 |  | Implemented |  |  |  | N |
| Energy transformation | Investment plan in the electricity sector |  | CO2 |  | Implemented |  |  |  | N |
| Energy transformation | Subsidy of 0.05 euro/kWh for electricity from renewable sources |  | CO2 |  | Implemented |  |  |  | N |
| Energy transformation | New STAG power plants are built in 1995 and 2000 |  | CO2 |  | Implemented |  |  |  | N |
| Energy transformation | No new nuclear power stations and maximum lifetime for existing nuclear power stations of 40 years |  | CO2 |  | Implemented |  |  |  | N |
| Energy use | Fuel taxes |  | CO2 | Fiscal | Implemented |  |  |  | N |
| Waste | Measures to reduce landfilling of organic waste and recovery of landfill gas |  | CH4 |  | Implemented |  |  |  | Y (Landfill directive) |
| Energy transformation (fugitive emissions from distribution) | Measures to reduce leaks |  | CH4 |  | Implemented |  |  |  | N |
| Agriculture | Reduction of fertiliser use |  | N2O |  | Implemented |  |  |  | ? |
| Fluorinated gases | Measures to reduce emissions |  | HFC |  | Implemented | Flemish region |  |  | N |

| Sector | Name | Objective | GHG affected | Type of instrument | Status | Implementing entity | Estimate of savings (MtCO2) |  | ### CCPM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2010 | 2020 |  |
| Policies and measure from the third national communication |  |  |  |  |  |  |  |  |  |
| Energy | E1 – Purchase at a guaranteed price of electricity produced from RES | Promotion of RES | CO2 | Financial | Adopted | Federal State Ministry of Economic Affairs (Energy Administration) |  |  | N |
|  | E2 – Green certificates | Promotion of electricity derived from RES or from CHP | CO2 | Regulation | Adopted | Federal State Ministry of Economic Affairs (Energy Administration) |  |  | N |
|  |  |  |  |  | Implemented | Ministry of Flemish Community (energy Administration) |  |  |  |
|  |  |  |  |  | Adopted | Walloon Region General Directorate of Technology, Research and Energy |  |  |  |
|  |  |  |  |  | Adopted | Brussels-Capital Region IBGE/BIM |  |  |  |
| Energy | E3 – Eligibility of producers and consumers of green electricity | Promotion of electricity derived from RES or from CHP | CO2 | Regulation | Adopted | Federal State Ministry of Economic Affairs (Energy Administration) |  |  | N |
|  |  |  |  |  | Adopted | Ministry of the Flemish Community (Energy Administration) |  |  |  |
|  |  |  |  |  | Adopted | Walloon Region General Directorate of Technology, Research and Energy |  |  |  |
|  |  |  |  |  | Adopted | Brussels-Capital Region IBGE/BIM |  |  |  |
| Energy | E4 – Priority access to the network for green electricity | Promotion of electricity derived from RES or from CHP | CO2 | Regulation | Adopted | Ministry of Flemish Community (Energy Administration) |  |  | N |
|  |  |  |  |  | Adopted | Walloon Region General Directorate of Technology, Research and Energy |  |  |  |
| Energy | E5 – Obligations of public service in the area of energy efficiency | Energy efficiency | CO2 | Regulation | Adopted | Ministry of the Flemish Community (Energy Administration) |  |  | N |
|  |  |  |  |  | Adopted | Walloon Region General Directorate of Technology, Research and Energy |  |  |  |
| Energy | E6 – Tax Reductions on investments (industrial sector) | Improvements in energy efficiency (industry) | CO2 | Financial | Implemented | Federal State Ministry of Finance |  |  | N |
| Energy | E7 – Tax reductions on investments (residential sector) | Improvements in energy efficiency(residential) | CO2 | Financial | Implemented | Federal State Ministry of Finance |  |  | N |
| Energy | E8- Financing CHP installations | Promotion of CHP | CO2 | Financial | Adopted | Federal State Ministry of Economic Affairs (Energy Administration) |  |  | N |
| Energy | E9 – Subsidies to companies for investments in energy economy | Improved energy efficiency and promotion of RES | CO2 | Financial | Implemented | Ministry of the Flemish Community (Energy Administration) |  |  | N |
| Energy | E10 – Subsidies for the installation of photovoltaic panels | Promotion of solar energy(photovoltaic) | CO2 | Financial | Implemented | Ministry of Flemish Community (Energy Administration) |  |  | N |
| Energy | E11 – Subsidies for CHP installations | Promotion of CHP systems | CO2 | Financial | Implemented | Ministry of the Flemish Community (Energy Administration) |  |  | N |
| Energy | E12 – Subsidies to households for improving their energy efficiency | Improvement of energy efficiency | CO2 | Financial | Implemented | Walloon Region General Directorate of Technology, Research and Energy |  |  | N |
| Energy | E13 – Subsidies to municipalities, schools and hospitals for investments leading to energy economy | Improvement of energy efficiency and promotion of RES | CO2 | Financial | Implemented | Walloon Region General Directorate of Technology, Research and Energy |  |  | N |
| Energy | E14 - Subsidies to federations of companies for investments leading to energy efficiency | Improvement Of energy efficiency | CO2 | Financial | Implemented | Walloon Region General Directorate of Technology, Research and Energy |  |  | N |
| Energy | E15 – Promotion of the Soltherm solar water heater | Promotion of solar water heaters | CO2 | Educational/ Financial | Implemented | Walloon Region General Directorate of Technology, Research and Energy |  |  | N |
| Energy | E16 – Subsidies for investments leading to energy efficiency | Improvement of energy efficiency | CO2 | Financial | Implemented | Brussels – Capital Region IBGE/BIM |  |  | N |
| Energy | E17 – Subsidies for investments in solar water heaters | Promotion of solar water heaters | CO2 | Educational/ Financial | Implemented | Brussels-Capital Region IBGE/BIM |  |  | N |
| Energy | E18 – Free energy pre-audits | Energy efficiency | CO2 | Educational/ Financial | Implemented | Ministry of the Flemish community (Energy Administration) |  |  | N |
| Energy | E19 – Grant towards the cost of an audit energy | Energy Efficiency, promotion of RES | CO2 | Educational/ Financial | Implemented | Walloon Region General Directorate of Technology, Research and Energy |  |  | N |
| Energy | E20 – Free consultation service for SME’s | Energy efficiency, promotion of RUE | CO2 | Educational/ Financial | Implemented | Walloon Region General Directorate of Technology, Research and Energy |  |  | N |
| Energy | E21 – Subsidies for energy audits | Energy efficiency | CO2 | Educational/ Financial | Implemented | Brussels-Capital Region IBGE/BIM |  |  | N |
| Energy | E22 – Reductions in tariff for the clients of CHP installations | Promotion of CHP | CO2 | Financial | Implemented | Federal State Ministry of Economic Affairs – Energy Administration |  |  | N |
| Energy | E23 – RUE/electricity Fund | Improvement of energy efficiency, promotion of the RES and the RUE | CO2 | Financial | Implemented | Federal State Ministry of Economic Affairs – Energy Administration |  |  | N |
| Energy | E24 – Measures to encourage energy efficiency (gas industry) | Energy efficiency | CO2 | Financial | Implemented | Federal State Ministry of Economic Affairs – Energy Administration |  |  | N |
| Energy | E25 – Financial support for the production of electricity from RES | Promotion of RES | CO2 | Financial | Implemented | Federal State Ministry of Economic Affairs – Energy Administration |  |  | N |
| Energy | E26 – Energy certification of buildings | Energy efficiency of buildings | CO2 | Regulation | Adopted | Energy Administration (federal + regional) |  |  | N |
| Energy | E27 – Introduction energy performance standards | Energy efficiency of buildings | CO2 | Regulation | Flemish Region – implemented: other regions: planned | Energy Administration (regions) |  |  | N |
| Energy | E28 – Standards of energy efficiency for boilers and water heaters | Improvement of the energy efficiency of boilers and water heaters | CO2 | Regulation | Implemented | Federal State Ministry of Social Affairs, Public Health and environmental health |  |  | N |
| Energy | E29 - Energy efficiency labels | Improvement of the energy efficiency of domestic appliances | CO2 | Regulation | Implemented | Federal State Ministry of Social Affairs, Public Health and environmental health |  |  | Y? |
| Energy | E30 – Benchmarking Agreements (Flemish Region) | Energy efficiency in the industrial sector | CO2 | Voluntary | Implemented | Ministry of the Flemish Community (Energy Administration) |  |  | N |
| Energy | E31 – Voluntary agreements with industry(Walloon Region) | Energy efficiency in the industrial sector | CO2 | Voluntary | Implemented | Walloon Region General Directorate of Technology, Research and Energy |  |  | N |
| Energy | E32 – “Eco-dynamic Company” Label (Brussels-Cap. Region | Improving energy efficiency in companies | CO2 | Voluntary | Implemented | Brussels-Capital Region IBGE/BIM |  |  | N |
| Energy | E33 – AMPERE Commission | Analysis aimed at planning energy policy | CO2 | Planning | Implemented | Federal State Ministry of Economic Affairs – Energy Administration |  |  | N |
| Energy | E34 – New infrastructure projects | New infrastructure (wind,photovoltaic) | CO2 | Planning | Adopted | Ministry of the Flemish Community (Energy Administration) |  |  | N |
| Energy | E35 – Demonstration projects (Flemish Region) | Information and public awareness (energy efficiency and RES) | CO2 | Education | Implemented | Energy Administration (regions) |  |  | N |
| Energy | E36- Code of good behaviour for the gas distribution sector | Analysis and prevention of CH4 leaks | CH4 | Voluntary | Planned | Ministry of the Flemish Community (Administration of Environment) |  |  | N |
| Industry | I1- Adaptation of the VLAREM regulations (N2O) catalysts | Limitations of N2O emissions released by the production of nitric acid | N2O | Regulation | Planned | Flemish Region |  |  | N |
| Industry | I2-Regulations aiming to limit the use of fluorinated greenhouse gasses | Reduced emissions of fluorinated greenhouse gases | HFCs, PFCs, SF8 | Regulation | Implemented/Adopted | Flemish Region |  |  | N |
| Industry | I3 -“Best Available Technologies” (Flemish Region) | Implementation of the IPPC directive | All | Regulation | Adopted | Flemish Region |  |  | Y |
| Industry | I4 – Sector agreements (Walloon Region) | Reduction of GHC emissions (Chemistry, paper, mining industries, cement) | All | Voluntary | Adopted/planned | Walloon Region |  |  | N |
| Industry | I5 – “Best Available Technologies” (Walloon Region) | Implementation of the IPPC directive | All | Regulation | Adopted | Walloon Region |  |  | Y |
| Agriculture and forestry | A1 – Manure Action Plan 2” | Reduction of agricultural inputs | N2O, CH4 | Regulation | Adopted | Flemish Region VLM – manure bank |  |  | N |
| Agriculture and forestry | A2 – Reduction in pork rearing | Reduction of agricultural inputs | N2O, CH4 | Financial | Implemented | Flemish Region ALT (Agricultural and horticultural administration) |  |  | N |
| Agriculture and forestry | A3 – Ammonia reduction plan | Reduction in NH3 emissions | N2O | Regulation | Adopted | Flemish Region |  |  | N |
| Agriculture and forestry | A4 – “Organic Agriculture” action plan | 10% of farmed land to be “organic” by 2010 | N2O, CH4 | Financial /Educational | Planned | Flemish Region ALT (Agricultural and horticultural administration) |  |  | N |
| Agriculture and forestry | A5 – Flanders Rural Development Programme | Improvement of agricultural practices | N2O, CH4 | Financial /Educational | Adopted/ Planned | Flemish Region |  |  | N |
| Agriculture and forestry | A6 – Rural Development Plan (RDP) | Improvement of agricultural policies | CO2 N2O, CH4 | Financial /Educational | Adopted/ Planned | Walloon Region |  |  | N |
| Agriculture and forestry | A7 – Agri-environmental measures | Improvement of agricultural practices | N2O, | Regulation | Adopted/ Planned | Walloon Region |  |  | N |
| Agriculture and forestry | A8 – Storage, treatment, spreading of farmyard manure | Optimisation of fertilisation | N2O, CH4 | Financial/ Voluntary | Planned | Walloon Region |  |  | N |
| Agriculture and forestry | A9 – Reduced input of mineral nitrogen | Reduction of added nitrogen | N2O, | Regulation | Planned | Walloon Region |  |  | N |
| Agriculture and forestry | A10 – Measure to support reforestation | Reforestation | CO2 | Financial | Implemented (end:2000) | Federal State Min of Agriculture |  |  | N |
| Agriculture and forestry | A11 – Reconversion of ground (Flemish Region) | Reforestation | CO2 | Financial | Implemented | Flemish Region Department of Woods and Countryside |  |  | N |
| Agriculture and forestry | A12 – Prohibition of deforestation | Preservation of wooded land | CO2 | Regulation | Implemented | Flemish Region Department of Woods and Countryside |  |  | N |
| Agriculture and forestry | A13 – Preservation of the ecological stability of forests | Forest Conservation | CO2 | Financial | Adopted | Walloon Region |  |  | N |
| Agriculture and forestry | A14 – Wood Energy Plan | Recovery of energy from wood | CO2 | Educational | Adopted | Walloon Region |  |  | N |
| Agriculture and forestry | A15 – Investigation of carbon sequestration | Improved knowledge of carbon sinks | CO2 | R&D | Implemented | Walloon Region |  |  | N |
| Waste | W1- Moratorium on dumping waste | Closure of biologically active landfill sites | CH4 | Regulation | Implemented | Flemish region OVAM |  |  | Y? |
| Waste | W2 – Modifications to the VLAREM legislation | Elimination and recovery of discharged gases | CH4 | Regulation | Planned | Flemish Region AMINAL |  |  | N |
| Waste | W3 – Introduction of specific channels of waste management | Optimised management and recovery of industrial waste | CO2/ CH4 | VOL | Planned | Walloon Region |  |  | N |
| Waste | W4 – Ban on dumping biodegradable organic waste | To stop waste going to landfill sites | CH4 | Regulation | Planned | Walloon Region |  |  | Y |
| Waste | W5 – Remedial treatment of old landfill sites | Recovery of discharged gas | CH4 | Regulation | Implemented | Walloon Region |  |  | N |
| Waste | W6 – Modifications of waste incineration installations | To improve the environmental performance of incinerators | CO2/ CH4 | Regulation | Implemented | Brussels-Capital Region |  |  | N |
| Waste | W7 – Reduction of waste at source | To reduce the quantity and harmfulness of waste at source | CO2/ CH4 | Educational | Implemented | Brussels-Capital Region |  |  | N |
| Waste | W8 – Recovery of waste | Waste recovery, prevention | CO2/ CH4 | Mixed | Implemented | Brussels-Capital Region |  |  | N |
| Transport | T1 – Improvements to public transport (Flemish Region) | Increase the provision and improve the quality of public transport | CO2, N2 O, Ozone | Mixed | Adopted | Flemish Region Mobility Cell |  |  | N |
| Transport | T2 Improvements to public transport (Walloon Region) | Ditto | CO2, N2 O, Ozone | Mixed | Implemented | Walloon Region (MET(DG3), SRWT,TEC) + SNCB/NMBS) |  |  | N |
| Transport | T3 – Improvements to public transport (Brussels-Cap,. Region) | ditto | CO2, N2 O, Ozone | Mixed | Implemented | Brussels-Capital Region AED+STIB/MVIB(Brussels Public transport Company) + Federal State |  |  | N |
| Transport | T4 – Federal Government’s rail investment plan | Ditto | CO2, N2 O, Ozone | Planned | Adopted | Federal state (Min. Communications and Infrastructure) + agreement for State/Region co-operation) |  |  | N |
| Transport | T5 – Tax exemption for employer’s contribution to the price of public transport season tickets and car pooling | The use of public transport and car pooling for home/work journeys | CO2, N2 O, Ozone | Financial | Adopted | Federal State Min. of Finance |  |  | N |
| Transport | T6 – Journey expenses become tax deductible | Use of alternative means of transport for home/work journeys | CO2, N2 O, Ozone | Financial | Adopted | Federal State Min. of Finance |  |  | N |
| Transport | T7 – Agreed employer’s expenses for operating collective transport for members of staff becomes tax deductable | Collective transport of company personnel | CO2, N2 O, Ozone | Financial | Adopted | Federal State Min. of Finance |  |  | N |
| Transport | T8 – Implementation of local transport plans(schools and companies) | Collective transport (school children and company personnel) | CO2, N2 O, Ozone | Voluntary | Implemented | Flemish Region Mobility Cell |  |  | N |
| Transport | T9 – Pricing policy in favour of public transport | Promotion of the use of public transport | CO2, N2 O, Ozone | Financial | Implemented | Flemish Region Mobility Cell, De Lijn company |  |  | N |
| Transport | T10 – Promotion of company staff transport plans | Collective transport of company personnel | CO2, N2 O, Ozone | Voluntary | Planned | Walloon Region |  |  | N |
| Transport | T11 – Promotion of the use of the bicycles for daily home-school journeys | Use of bicycles for home-school journeys | CO2, N2 O, Ozone | Educational | Adopted | Walloon Region municipalities |  |  | N |
| Transport | T12 – Plans for transporting company personnel | Use of alternative means of transport for home/work journeys | CO2, N2 O, Ozone | Voluntary | Implemented | Brussels Region MCI+IBGE/BIM +AED |  |  | N |
| Transport | T13 – Improvement of the infrastructure for cyclists and pedestrians | Use of bicycles and walking for short journeys | CO2, N2 O, Ozone | Infrastructure | Adopted | Flemish Region Mobility Cell |  |  | N |
| Transport | T14 – Improvement of infrastructure for cyclists | Promotion of the use of bicycles in towns & cities | CO2, N2 O, Ozone | Mixed | Implemented/ Adopted | Brussels Region AED + municipalities |  |  | N |
| Transport | T15 – Improvement of transport infrastructure around the Port of Antwerp | Use of different modes of transport | CO2, N2 O, Ozone | Infrastructure | Implemented | Federal State + SNCB/NMBS = Port of Antwerp |  |  | N |
| Transport | T16 – Improvement of the quality of transport by navigable waterway (Flemish Region) | Increase of the proportion of goods transported by navigable waterway | CO2, N2 O, Ozone | Infrastructure | Adopted | Flemish Region: Mobility Cell |  |  | N |
| Transport | T17 –Improvement of systems to promote the use of alternative modes of transport | Changing between modes of transport | CO2, N2 O, Ozone | Infrastructure | Planned | Flemish Region: Mobility Cell |  |  | N |
| Transport | T18 – Improvement of the quality of navigable waterways | Increased proportion of goods transported by navigable waterway | CO2, N2 O, Ozone | Infrastructure | Adopted | Walloon Region |  |  | N |
| Transport | T19 – Creation of multimodal platforms (hubs) (Walloon Region) | Changing between modes of transport | CO2, N2 O, Ozone | Infrastructure | Adopted | Walloon region |  |  | N |
| Transport | T20 – Rationalisation of the traffic flow and parking for heavy loads in cities | Freeing up city traffic | CO2, N2 O, Ozone | Regulation | Planned | Brussels Region Police + AED |  |  | N |
| Transport | T21 – Creation of multimodal platforms (hubs) (Region Brussels) | Changing between modes of transport | CO2, N2 O, Ozone | Infrastructure | Adopted | Brussels Region Port of Brussels + SNCB/NMBS + SDRB |  |  | N |
| Transport | T22 – Modulation of the size of the European road tax | Promotion of less polluting vehicles | CO2, N2 O, Ozone | Financial | Implemented | Federal State Min of Finance |  |  | N |
| Transport | T23 – LPG Allowance | Promotion of less polluting vehicles | CO2, N2 O, Ozone | Financial | Implemented | Federal State Min of the Environment |  |  | N |
| Transport | T24 – Reduction of the tax for bringing LPG vehicles onto the road | Promotion of less polluting vehicles | CO2, N2 O, Ozone | Financial | Adopted | Regions (Cooperation agreement) |  |  | N |
| Transport | T25 – Reduction of the tax for bringing “Euro 4” cars onto the road | Promotion of less polluting vehicles | CO2, N2 O, Ozone | Financial | Adopted | Regions (Cooperation agreement) |  |  | N |
| Transport | T26 – Revision of the tax for bringing second-hand vehicles onto the road | Promotion of less polluting vehicles | CO2, N2 O, Ozone | Financial | Adopted | Regions (Cooperation agreement) |  |  | N |
| Transport | T27 – Modulation of the tax for getting the vehicle on the road depending on polluting emissions | Promotion of less polluting vehicles | CO2, N2 O, Ozone | Financial | Adopted | Regions (Cooperation agreement) |  |  | N |
| Transport | T28 – Promotion of “clean” vehicles | Changing the behaviour patterns of consumption | CO2, N2 O, Ozone | Educational | Implemented | Flemish Region: Aminabel-air section |  |  | N |
| Transport | T29 – Traffic Regulation | Reduce polluting emissions due to car traffic | CO2, N2 O, Ozone | Planned | Implemented | Flemish Region Mobility Cell |  |  | N |
| Transport | T30 – Training of mobility advisors | To strengthen the authority of mobility organisations | CO2, N2 O, Ozone | Educational | Implemented | Walloon Region MET(DG1, DG2, DG3,) + DGRNE + DGATLP) |  |  | N |
| Transport | T31,T32 – Mobility observatory | Monitoring and evaluation of policies | CO2, N2 O, Ozone | Planned | Adopted | Walloon Region |  |  | N |
|  |  |  |  |  | Adopted | Brussels Region AED |  |  |  |
| Transport | T33 – Campaigns promoting the use of bicycles in the city | Promotion of the use of bicycles in the city | CO2, N2 O, Ozone | Educational | Implemented | Brussels Region AED + municipalities |  |  | N |

## Evaluation of Projections

- Table 5 shows the projections by greenhouse gas for 2010 and Table 6 summarises the projections by sector.  A new with measures projection was prepared in 2003 and presented in the report to the Monitoring Mechanism.  The with additional measures projection has been obtained by subtracting the effect of the additional measures given in Table 3 from the with measures projection.  The with measures projection is for an increase in emissions relative to the base year.  Additional measures have been identified that are projected to reduce the emissions below the base year.

- Table 5: Summary of projections by gas in 2010 (MtCO2 equivalent)

|  | Base year | 2010 - with measures | 2010 - with additional measures |
| --- | --- | --- | --- |
| CO2 | 116.4 | 130.1 | 116.3 |
| CH4 | 11.1 | 6.8 | 6.8 |
| N2O | 13.2 | 11.0 | 11.0 |
| HFC, PFC, SF6 | 0.3a | 2.2 | 2.2 |
| Total without sinks | 141 | 150.1 | 136.3 |
| Sinks | -2.1 | -2 | -2 |
| Total with sinks | 138.9 | 148.1 | 134.3 |

- a This figure is the inventory for fluorinated gases in 1995

- Table 6: Summary of projections by sector in 2010 (MtCO2 equivalent)

|  | Base year | 2010 - with measures | Relative to base year | 2010 - with additional measures | Relative to base year |
| --- | --- | --- | --- | --- | --- |
| Energy | 111.3 | 120.9 | 9% | 107.2 | -4% |
| Energy transformation sector | 30.5 | 28.2 | -8% | 22.5 | -26% |
| Manufacturing and construction | 32.7 | 30.1 | -8% | 25.0 | -23% |
| Transport sector | 20.2 | 27.7 | 37% | 27.0 | 34% |
| Residential and services | 26.8 | 33.8 | 26% | 31.6 | 18% |
| Fugitive emissions | 0.8 | 1.0 | 30% | 1.0 | 0% |
| Industrial processes | 11.3 | 15.8 | 40% | 15.7 | 39% |
| Agriculture | 14.0 | 8.2 | -41% | 8.2 | -41% |
| Waste | 4.2 | 3.0 | -28% | 3.0 | -28% |
| Fluorinated gas | 0.3 | 2.2 | 633% | 2.2 | 633% |
| Total | 141.0 | 150.1 | 6.5% | 136.3 | -3% |
| Sinks | -2.1 | -2 | -5% | -2 | -5% |
| Total incl sinks | 138.9 | 148.1 | 7% | 134.3 | -3% |

An assessment of the position relative to the target is given in Table 7.  Belgium’s commitment under the Burden Sharing Agreement is –7.5% of the 1990 six gas basket.  Based on the without sinks total, this gives a commitment to reduce emissions by 10.6 MtCO2 eq.  The ‘with measures’ projection is for an increase of 9.1 MtCO2 eq, giving a gap of 19.7 MtCO2 eq or 14% of the 1990 total.  Additional policies and measures totalling 13.8 MtCO2 eq or around 70% of the gap have been identified.  The Belgium Government have indicated that the Kyoto mechanisms will be used to meet their commitments.

Table 7: Assessment of Target

|  | - Without sinks |  | - With sinks |  |
| --- | --- | --- | --- | --- |
|  | - MtCO2 equiv. | - % of 1990 level (six gas basket) | - MtCO2 equiv. | - % of 1990 level (six gas basket) |
| - Base year (from projections) | - 141.0 |  | - 138.9 |  |
| - Commitment | - 130.4 | - -7.5% | - 130.4 | - -6.1% |
| - With existing P&Ms | - 150.1 | - 6.5% | - 148.1 | - 6.6% |
| - Gap (-ve means no gap) | - 19.7 | - 14.0% | - 18.0 | - 13.0% |
| - Effect of additional P&Ms | - 13.8 | - 9.8% | - 13.8 | - 9.8% |
| - With additional P&Ms | - 136.3 | - -3% | - 134.3 | - -3% |

## Description of Modelling Approach

### Background

Projections are produced for the medium term (2010) and the long term (2020).

Medium term projections.  These are produced using a combination of the Hermes and EPM models.  For the with measures scenario the results are emissions of the three main greenhouse gases from all sectors.  The additional measures scenario results is only for energy related emissions.

Long term projections.  A combination of the GEM-E3 and Markal models is used to estimate emissions for the long term.  The three main greenhouse gases are covered but only energy related emissions.

### Model Descriptions

EPM

The EPM model (Energy/Emissions Projection Model) is a techno-economic bottom-up simulation model.  The model explains the consumption of energy and corresponding greenhouse gas emissions by looking a activity variables in physical units.  It contains a detailed representation of emission sources and of the main factors that determine the evolution in demand.  Expected technological developments are taken into account and the model can simulate technical options.  Non-energy related emissions can also be determined.

EPM distinguishes between 90 industrial activities within the energy intensive industry sectors, 14 different building types in the residential sector and 30 subsectors in commercial.

Hermes

Hermes is a macro-sectoral econometric top-down simulation model.  It is a dynamic and annual model that allows for simulations to be performed of the impacts of macro-economic and sectoral policies, multinational economic polices, energy policies and new technologies.  The model is oriented towards the demand side, which is modelled in some detail, but also includes the supply side.  The demand for energy is estimated through econometric analysis.

The exogenous variables for the model are the detailed prices for energy products, monetary policy variables (interest rates, exchange rates), fiscal and budgetary variables, European policy and demographic variables.  The output of the model consists of input/output tables for 9 sectors, the demand for production factors, the accounts of economic agents, the detailed consumption structure for households and the equilibrium between supply and demand for 8 energy products.

In the most recent version of HERMES, the environmental component was further developed.  This allowed the analysis to be extended to the evolution in CO2 emissions per sector and the effect of fiscal and other measures.

To calculate the joint effect of non-fiscal and fiscal measures, the bottom-up estimates of non-fiscal measures from EPM have been integrated into the top-down model HERMES.

Markal

Markal is a long-term multi-period energy technology optimisation model,.  It contains a technology description of the main energy transformation and energy use processes in Belgium.  Each potentially available energy technology is described by its technological characteristics, its costs and its current installed capacity.  The model chooses energy production and consumption options that maximises the net total welfare of the energy users, given exogenous bounds on availability of energy sources or technologies or on total emissions.

GEM-E3 Model

The GEM-E3 model is an applied general equilibrium model, representing the EU Member States individually linked through trade,.  It aims to cover the interactions between the economy, the energy system and the environment.  The model computes simultaneously the different market equilibrium under the Walras law and within the macroeconomic equilibrium it determines the optimum balance for energy demand/supply and emission/abatement.  The projected energy demand derived from the GEM-E3 model is used as an input into the MARKAL model, which subsequently determines the least cost way to satisfy the demand.

### Macro-economic assumptions

The assumptions about the Belgian macro-economic development are based on predictions from the European Commission and the OECD for the period up to 2006.  For the period 2007-2012, no external information was available so the hypothesis was based on the growth trends over the last 20 years.  Parameters for the modelling are shown in Table 8.  The structure of the electricity supply for 2001 and 2010 is given in Table 9.  The estimates for wind and water energy are consistent with other sources but do not necessarily correspond the EU or Belgian targets.

### Table 8 Parameters for the projections

| Parameter | 2005/10 | Unit |
| --- | --- | --- |
| Population |  |  |
| GDPa | 2.3 | Annual ave. growth rate |
| Oil (International price) |  | Price growth rate (USD 90/bl) |
| Gas (International price) |  | Price growth rate (USD 90/boe) |
| Coal (International price) |  | Price growth rate |
| Transport Passenger growth |  | Annual ave. growth rate |
| Freight growth |  | Annual ave. growth rate |

Table 9 Production of electricity (TWh)

|  | 2001 | 2010 | relative to 2000 |
| --- | --- | --- | --- |
| Water and wind energy | 0.5 | 2.7 | 440% |
| Nuclear | 46.3 | 48.1 | 4% |
| Thermal | 31.7 | 42.4 | 34% |
| Petroleum products | 1.3 | 0.9 | -31% |
| Steel oven and coke gases | 2.1 | 2 | -5% |
| Natural gas | 16.5 | 32.5 | 97% |
| Biomass and waste | 1.4 | 2 | 43% |
| other combustibles | 0.5 | 0.5 | 0% |
| coal | 9.8 | 4.5 | -54% |
| Total | 78.5 | 93.2 | 19% |

Source: Report to the Monitoring Mechanism, December 2003

## Country Conclusions

The main documents used for this summary were the document provided under the Monitoring Mechanism and the Third National Communication.  The level of detail in the documents is good but quantification of the effect of policies and measures is weak.  The details of the methodology for the projections are available and contain a reasonable level of detail on the models and parameters. Additional policies and measures are only for CO2.

The with measures projections is for an increase of 6.5% compared to 1990.  Belgium’s burden sharing agreement is a reduction of 7.5%, giving a gap of 19.7MtCO2 eq (14%).   Additional policies and measures are identified to give savings of 13.8 MtCO2 eq (70% of the gap to the commitment).  The Kyoto Mechanisms will be used to close the gap.

There is an active programme of policies and measures in all sectors.  The Federal nature of Belgium means that policies may be implemented in some regions but not in others.  In the energy sector, most of the policies and measures have been implemented, but in other sectors more of the policies are at the adoption or planning stage.  Details of policies and measures are given in the 3rd National Communication but there is no clear indication which have been included in the with measures projection. The model used is an econometric model. Its projections are based upon historical activity and emissions data, which include the effects of all measures taken up to the year 2000.

ECONOTEC “Application du modèle EPM au dévelopment de scenarios d’émissions de CO2 à l’horizon 2010 pour la Belgique  - rapport d’avancement” étude effectuée pour les Services fèdéraux des Affaires scientifiques, techniques et culturelles, 2001

“A description of the HERMES II model for Belgium” Working paper no 05-00 of the Federal Planning Bureau 2000. http://www.plan.be/en/pub/wp/wplist.stm

“MARKAL, a model to support greenhouse gas reduction policies”.  2001. DWTC/SSTC – Final report CES-KULeuvan, VITO

“How to achieve the Kyoto target in Belgium – modelling methodology and some results”, Working paper series no2000-09, KULeuven-CES

P Capros, T Geogakopoulos, D. Van Regermorter, S Proost, T.F.N Schmidt and K Conrad, “European Union: the GEM-E3 General Equilibrium Model”, in Economic and Financial Modelling, Special Double Issue Vol 4, No 2&3 pp51-160, 1997

“GEM-E3: The GEM-E3 model:  Reference Manual (detailed technical documentation of the model)”, Capros et al 1997 available on http://gem-e3.zew.de