# Germany

## Sources of Information

Germany has not yet submitted a formal Monitoring report: this was due by 31 December 2003 under the Council Decision for a monitoring mechanism of Community CO2 and other greenhouse gas emissions (1999/296/EC). Germany advised the use of information included in its National allocation plan under the EU emission trading directive for updating greenhouse gas projections.

Therefore, this report is based mainly on the German Monitoring Report 2002:

- "Bericht 2002 der Bundesrepublik Deutschland über ein System zur Beobachtung der Emissionen von CO2 und anderen Treibhausgasen –entsprechend der Ratsentscheidung 1999/296/EG", submitted to the European Commission on 31 January 2003.

The Monitoring Report 2002 has not been submitted in English. Therefore, the German version was analysed.

With regard to policies and measures the Monitoring Report 2002 includes recent policy developments such as those in the coalition agreement of the new government after elections for the German Bundestag in September 2002.

However, the greenhouse gas projections have been taken from a study submitted by the German Environmental Agency that is still on the way carrying out an update of Germany’s GHG projections which will take into account the effects of the recent policies and measures:

- Preliminary draft "Endbericht zum Forschungsvorhaben Politikszenarien für den Klimaschutz – Langfristszenarien und Handlungsempfehlungen ab 2012 (Politikszenarien III)", Study commissioned by the Federal Environment Ministry; authors: DIW, FZJ, Fraunhofer-ISI, Öko-Institut, submitted by German Environmental Agency in June 2003;

These results are preliminary and still not adopted by the German Government. However, because of Germany’s large contribution to EU-total greenhouse gas emissions the Commission urged and Germany allowed to replace Germany’s projections reported in the Monitoring report 2002, as they are out-dated. But it must be mentioned that the data available now are not in compliance with the reporting guidelines of the Monitoring Mechanism concerning their split by sectors and by gases.

Additional sources of information have also been used mainly to complete descriptions of the reported policies and measures:

- Third Report by the Government of the Federal Republic of Germany in accordance with the Framework Convention of the United Nations.

Recent developments in Germany’s climate policies have been taken into account by using Germany’s National allocation plan under the EU emission trading Directive (2003/87/EC):

- Nationaler Allokationsplan für die Bundesrepublik Deutschland 2005-2007, Bundesministerium für Umwelt, Naturschutz und Reaktorsicherheit, Berlin, 31. März 2004

## Quality and Transparency of Reporting

The Monitoring Report 2002 (abbreviated MR2002 in this report) and Germany’s Third National Communication (abbreviated 3NC) are the basis for the initial analysis in this section.

- Table 1: Information Provided on Policies and Measures

- (from MR 2001 and CPP 2000)

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - +++ |  |
| - Objectives of policies | - + | - Not always stated in MR2002, additional info taken from 3NC. |
| - Which greenhouse gases? | - +++ | - CO2 , CH4, N2O, HFC, PFC, SF6 |
| - Status of Implementation | - ++ | - In most cases stated in MR2002, additional info taken from 3NC |
| - Implementation body specified | - + | - Not specified in MR2002, fully taken from 3NC |
| - Quantitative assessment of implementation | - ++ | - In most cases stated. |
| - Interaction with other P&Ms discussed | - + | - Only in very few cases stated. |

- +, ++, +++ level of information available increases as the number of + signs increases

In the Monitoring Report 2002 inventory data for all six Kyoto greenhouse gases are presented for the years between 1990 and 2001. The data for 1999 to 2001 are preliminary. As announced in the Monitoring Report 2002 Germany has delivered updates of emission inventory data to the Commission (and subsequently to EEA and ETC/ACC) by end of March 2003. As emission inventories are of minor importance in this analysis, the updates of inventory data are not included in the figures below.

Projections for the six greenhouse gases are presented for 2005 and 2010. Projections for 2020 are not given.

- Table 2: Information provided on Projections

- (from MR2002)

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - + | “With measures” (for references see above section on sources of information) “With additional measures”: None |
| - Expressed relative to inventory for previous years | - + | - It is given in the preliminary update (for references see above section on sources of information) relative to 1990 emissions. |
| - Starting year | - 1990 |  |
| - Split of projections |  | - No sectoral split |
| - Presentation of results | - + | - Draft study report with updated projections available |
| - Description of model (level of detail, approach and assumptions) |  | - Not given in MR2002; but can be taken from the series of “Politik-Szenarien” studies as Germany explained already in previous years. |
| - Discussion of uncertainty |  | - . |
| - Details of parameters and assumptions | - * | - For the time being, that is not available finally as the report of “Politikszenarien III” is still being drafted. |

- +, ++, +++ level of information available increases as the number of + signs increases

## Assessment of Policies and Measures

Germany’s Monitoring report 2002 includes policies and measures (i) taken in the 1990s, (ii) being implemented following the National Climate Protection Programme (CPP) published in Autumn 2000 and (iii) regarding recent policy developments such as those in the coalition agreement of the new government after elections were held for the German Bundestag in September 2002. Major political decisions have been taken in the time period 1998 to 2002 - namely ecological tax reform, Renewable Energy Sources Act and financial support programmes for renewable energy investments. Furthermore some of them are now in their later stages of increased effectiveness or were added by subsequent regulations. The Monitoring Report 2002 refers to all policies and measures which came into force or, at least, have already been decided by December 2002. Some policies and measures in the MR2002 were updated in their quantitative effects compared to the MR2001. However, the projections given in MR2002 do not fully include these recent policies and measures. The MR2002 notes that the projections are currently under revision (See section on evaluation of projections below.).

The MR 2002 splits policies and measures into the following sectors: industry, residential, transport, energy transformation sector, agriculture,  waste and cross sectoral issues. That reporting structure is different from the IPCC guidelines on emission inventories making further analyses difficult. To achieve as much as possible consistency with the greenhouse gas source and sink categories of the IPCC guidelines the attempt has been made in this Member states analysis to identify the sectors reported in the MR2002 as GHG source and sink categories and some policies and measures have been moved between sectors/categories. In table 4 policies and measures are given under the sectors where they are reported in the MR2002 and - if a shift seemed appropriate – also under the IPCC category. The allocation of policies and measures to a certain sector/IPCC category has been attempted by following the description of the policies and measures given in the MR2002. Only a few policies and measures have been reallocated, in particular, all those reported under ‘cross sectoral’ and those under the industry sector have been split into energy use in industry and industrial processes.

- Additionally, in table 4 it has been noted whether policies and measures are presented:

- -new in MR2002,

- -still reported (no indication),

- -from last year’s report, and no longer reported in MR2002.

For the summary of effects in table 3 only IPCC categories are given and policies and measures are counted only once in their respective IPCC category as noted in table 4.

- The MR2002 refers only in very few cases - ecological tax reform, nitrate directive - to common and co-ordinated policies and measures (CCPM) of the European Union. Therefore, the link to CCPM had to be estimated. A list of  CCPMs  referred to in table 4 is given in the annex.

- Table 3: Summary of the Effect of Policies and Measures (MtCO2-eq.) (only those quantified in the MR2002, see Tab. 4)

|  | With measures | With additional measures |
| --- | --- | --- |
| Energy supply (roughly 1.A.1) | 89.0 – 97.0 | not given |
| Industry - energy use (roughly 1.A.2) | 27.0 | not given |
| Domestic & Tertiary (roughly 1.A.4) | 19.0 – 21.0 | not estimated |
| Transport (roughly 1.A.3) | 38.5 - 41.5 | 0.2 |
| Industrial processes (roughly 2.) | 34.3 - 38.7 | 12.8 |
| Waste management (roughly 6.) | 31.3 | 7.0 |
| Agriculture (roughly 4.) | 2.1 | not given |
| Total | 241.2 - 258.6 | 20.0 |

- Note: Allocation roughly to IPCC source categories in brackets

- It must be mentioned here that because of double counting of policies and measures in the last year’s Member states analysis for Germany the total for With measures was about 30 MtCO2-eq. higher compared to this analysis.

- Table 4: Detailed Information on Polices and Measures

- Existing policies and measures:

| Sector | Name | Objective | GHG affected | Type of instrument | Status | Implementing entity | Estimate of savings (Mt CO2-equiv.) |  | CCPM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2010 | 2020 |  |
| Cross sectoral (new in MR 2002) | Market introduction programme renewable resources | Promotion of biogenious resources, for example, biogenious fuels and lubricants (extention to other product areas planned) | CO2 | E | Existing |  | Not given |  |  |
| Cross sectoral (accounted under energy supply) | Ecological tax reform | Reduce fuel consumption of heating oil, natural gas and liquid gas and transport fuels due to stepwise increase of prices. Side effect raising of funds for national pension scheme | CO2 | F, R, E | Existing (1999) further increases until 2003 | Federal Government | 20.0 |  | yes (b, c, e) |
| Cross sectoral (accounted under residential) | Promotion of the use of "contracting" | Increasing the use of contracting | CO2 | D | ongoing project |  | 5.0 |  |  |
| Cross sectoral (accounted under energy supply) | Renewable Energy Sources Act (EEG) (fixed feed in grid prices) | At least double the share of renewable energy in the production of electricity until 2010 | CO2, CH4 from waste | R, E | Existing (since April 2000) | Federal Government | 15.0 |  | yes (h, k) |
| Cross sectoral (accounted under energy supply) | “100 000 roofs” PV programme | 300 MW new PV capacity within 6 years | CO2 | E | Existing (since January 1999) | Federal Government, Kreditanstalt fuer Wiederaufbau (KfW) | 0.2 |  | yes (h, k) |
| Cross sectoral (accounted under energy supply) | Market Introduction Programme Renewable Energies | Promotion of thermal solar and biomass energy by credits | CO2 | E | Existing (since September 1999) | Federal Government | 6.0 |  | yes (h, k) |
| Cross sectoral (accounted under transport) | Non-sulphur fuels | Supporting the use of non-sulphur or less-sulphur fuels through taxation | CO2 | E | In force since November 2001, will be improved in November 2003 |  | 2.0 – 5.0 (by 2005) |  |  |
| Energy supply (new in MR 2002) | Energy-related tax adaption | ecological adaption of taxation on the use of natural gas | CO2 | E | In force 1 Jan 2003 | Federal Government | not given |  |  |
| Energy supply | CHP law on maintenance, modernisation and expansion of heat and power cogeneration and introduction of fuel cells on the market | Supplement to the climate protection agreement in 2000 between the Federal Government and the industry, aiming at energy efficiency improvement in electricity and heat production by as most as possible utilisation of energy contents in used fuels | CO2 | R | In force April 2002 | Federal Government | 20.0 - 23.0 |  | yes (i) |
| Energy supply | Construction of more gas-fired combined cycle power plants | Modernisation of power plants; aiming at increase of efficiency of electricity production by use of low carbon fuels and most efficient turbine technology | CO2 | V | Ongoing project | Industry | 15.0 - 20.0 |  | yes (i) |
| Energy supply (reported as cross sectoral) | Ecological tax reform | Reduce fuel consumption of heating oil, natural gas and liquid gas and transport fuels due to stepwise increase of prices. Side effect raising of funds for national pension scheme | CO2 | F, R, E | Existing (1999) further increases until 2003 | Federal Government | 20.0 |  | yes (b, c, e) |
| Energy supply (reported as cross sectoral) | Renewable Energy Sources Act (EEG) (fixed feed in grid prices) | At least double the share of renewable energy in the production of electricity until 2010 | CO2, CH4 from waste | R, E | Existing (since April 2000) | Federal Government | 15.0 |  | yes (h, k) |
| Energy supply (reported as cross sectoral) | “100 000 roofs” PV programme | 300 MW new PV capacity within 6 years | CO2 | E | Existing (since January 1999) | Federal Government, Kreditanstalt fuer Wiederaufbau (KfW) | 0.2 |  | yes (h, k) |
| Energy supply (reported as cross sectoral) | Market Introduction Programme Renewable Energies | Promotion of thermal solar and biomass energy by credits | CO2 | E | Existing (since September 1999) | Federal Government | 6.0 |  | yes (h, k) |
| Energy supply | Use of pit gas from hard coal mining | Increasing use of pit gas as energy source from 70% to 78%, aiming at avoidance of CH4 emissions, substitution of fossil fuels | CH4 | V | Existing | Industry | 11.7 |  | no |
| Energy supply | Renewable Energy Sources Act ("Erneuerbare-Energien-Gesetz –EEG") | Economic incentives for the construction of bio gas supply utilities in waste sector | CH4 | E | Existing | Federal Government | not given |  |  |
| Energy supply | Renewable Energy Sources Act ("Erneuerbare-Energien-Gesetz –EEG") in agriculture | Economic incentives for the construction of bio gas supply utilities in agriculture | CH4 | E | Existing | Federal Government | 1.1 |  |  |
| Industry - energy use (new in MR 2002) | Reduce of exemptions of ecological tax | reduction of initially lower tax rates for energy intensive production branches to enforce energy efficiency efforts | CO2 | E | In force 1 Jan 2003 | Federal Government | not given |  |  |
| Industry - energy use | Continuation and Improvement of Voluntary commitments by industries | 28 % CO2 reduction per output unit by 2005; 35% reduction of all six “Kyoto gases” together per output unit by 2010 | All six gases | V | Adopted (November 2000) | Industry associations with Federal Government | 10.0 CO2 plus 10.0 CO2 –equiv. |  | no |
| Industry - energy use | Promoting “contracting” as management tool | Support for development of financing and operator concepts Aim: to enhance energy efficiency in facility management | CO2 | D | Review | Federal Government, possibly also other entities | 1.0 (by 2005) |  |  |
| Industry - energy use | Improvement of ERP, DtA and KfW credit programmes | Strengthening economic incentives with regard to energy efficiency | CO2 | E | Under evaluation | Federal Government together with public financial institutions such as KfW | Not given |  |  |
| Industry - energy use | Training in energy saving and efficiency for SMEs | Increase in energy efficiency | CO2 | ET | Encouragement to the economy | Small and middle sized enterprises | Not given |  |  |
| Industry - energy use | Energy Savings Ordinance (“Energie-einspar-verordnung”) | Combines and tightens existing requirements from the Thermal Insulation Ordinance Wärmeschutzverordnung) and the Heating-Systems Ordinance Heizungsanlagenverordnung). Aim: to reduce energy requirements in new buildings by 30%, and to develop of economically reasonable potential for improving energy efficiency of existing buildings | CO2 | R | Existing, in force February 2002 | Federal Government | 6.0 (by 2005. both in industry and trade and service) |  |  |
| Industry - energy use | Working Group "Emission trading to combat the greenhouse effect" | Cooperation of government, industry entities and NGO's during the configuration phase of the emission trading system to regard interests and assure acceptance | CO2 | V | Implemented since 2000 |  | not given |  | yes (o) |
| Transport (new in MR 2002) | Change of petroleum tax law | Exemption of biogenious fuels from petroleum tax to offer an incentive to substitute fossil fuels and reduce carbon intensity | CO2 | R | Implemented | Federal Government | Not given |  |  |
| Transport (new in MR 2002) | Promotion of cars powered by natural gas | Extension up to 2020 of tax reduction on cars powered by natural gas to shift to less carbon-intensive fuels | CO2 | E | In force 1 Jan 2003 | Federal Government | Not given |  |  |
| Transport | Voluntary commitment of German automobile industry ("Selbstverpflichtung der deutschen Automobilindustrie") | Reduction of new car's fuel consumption until 2005 by 25% compared to 1990, after 2005 - by 30% compared to 1990 | CO2 | V | Existing | Automobile industry associations | 10.0 |  | yes (a) |
| Transport | Distance based motorway tolls for heavy duty vehicles (Schwerlast- abgabe) | Shift in the modes of goods transport from street to railway and waterway | CO2 | E | Adopted, implementation 2003 | Federal Government | 5.0 (by 2005) |  | yes (c) |
| Transport | Information campaign on driving and maintenance behaviour |  | CO2 | I |  | Federal Government | 5.0 (by 2005) |  | no |
| Transport | Fiscal promotion of fuel efficient vehicles | Temporary vehicle tax rebates for cars with CO2 emissions of up to 120g/km | CO2 | F, E | Government decision | Federal Government | 1.0 (by 2005) |  |  |
| Transport | Energy efficient equipment in new cars | Use of low friction oil and low friction tyres in new cars, use of consumption indicators | CO2 | V | Request directed at industry | Federal Government | 11.00 |  | no |
| Transport | Air traffic: Emission based levies at Germany's airports | Introduction of emission-based take-off and landing fees at German airports Aim: to enhance energy efficiency in air transports | CO2 | F, E | Government decision | Federal Government | 1.0 (by 2005) |  | yes (b) |
| Transport | Improving intermodal transport | Telematics, logistics management and fleet management; avoidance of dead heads, and congestions to increase fuel efficiency in goods transport | CO2 | E | Planned as of 2003 | Industry | 3.5 (by 2005) |  |  |
| Transport (reported under Cross sectoral) | Non-sulphur fuels | Supporting the use of non-sulphur or less-sulphur fuels through taxation | CO2 | E | In force since November 2001, will be improved in November 2003 |  | 2.0 – 5.0 (by 2005) |  |  |
| Transport (from last year report, no more reported in MR 2002) | Additional budgetary means for investments in the infrastructure of the German railways | Total of 6000 million DM for the next 3 years (future investment programme) | CO2 | E | Existing | Federal Government in connection with Deutsche Bahn AG | Not quantifiable |  | yes (c) |
| Residential | Voluntary commitment of Federal Government to reduce CO2 emissions in federal ministries by 25% in 2005 and by 30% in 2010 ("Selbstverpflichtung der Bundesregierung im eigenen Geschäftsbereich") | In order to provide a clear signal for the public, the Federal Government plans to reduce CO2 emissions from its own real estates by 25% by 2005 and by 30% by 2010. Aim: to enhance energy efficiency in buildings and structures, in equipment and devices and in transports | CO2 | V | Existing | Federal Government | Not given |  |  |
| Residential | CO2 reduction programme of the Reconstruction Loan Corporation ("Kreditanstalt fuer Wiederaufbau (KfW)") | Aims to promote heat insulation and modernisation of heating facilities in existing houses especially in the new Federal Länder | CO2 | E | Existing | Federal Government, Kreditanstalt fuer Wiederaufbau (KfW) | 5.0 - 7.0 (by 2005) |  | yes (m) |
| Residential | Energy Savings Ordinance (“Energieeinspar-verordnung”) | Combines existing requirements from the Thermal Insulation Ordinance (Wärmeschutzverordnung) and the Heating-Systems Ordinance (Heizungsanlagenverordnung). Aims on reduction of energy requirements in new buildings by 30%, and on development of economically reasonable potential for improving energy efficiency of existing buildings | CO2 | R | In force February 2002 | Federal Government | 4.0 (by 2005) |  | yes (e, m) |
| Residential | Measures to reduce electricity consumption of appliances | Voluntary agreements and labelling; Particular attention to stand-by losses | CO2 | ET, V, R | Mainly proposed | Federal Government | 5.0 (by 2005) |  | yes (d) |
| Residential (reported under Cross sectoral) | Promotion of the use of "contracting" | Increasing the use of contracting | CO2 | D | ongoing project |  | 5.0 |  |  |
| Industry - processes (new in MR 2002) | Regulation of limitations of N2O emission in nitric acid production | Limit value of 800 mg N2O per m3 to be implemented by 2007 | N2O | R | In force since October 2002 |  | ca. 0,2 - 4,7 |  |  |
| Industry - processes | Technical measures for adipic acid production | Agreement of Federal Government with adipic acid manufactures to introduce highly effective technical measures that will reduce N2O emissions | N2O | V | Existing | Industry together with Federal Government | 28.0 |  | no |
| Industry - processes | XPS-rigid foam ("XPS-Hartschäume") | Replacement of HFC with CO2 and ethyl alcohol by half of the production, starting in 2000 | HFC | V | Existing | Industry | 1.3 |  |  |
| Industry - processes (increasing amount of assessed effect compared to MR2001) | Declaration of German primary aluminium industry | Voluntary commitment to reduce CF4/CF6 emissions by 50% by the year 2005 compared to 1987 level | PFC | V | Existing | Industry together with Federal Government | 1.25 |  | no |
| Industry - processes | Provision for the scrapping of electronic utilities ("Vorsorge bei der Verschrottung elektronischer Betriebsmittel") | Emission avoidance and recycling | SF6 | V | Existing |  | 1.2 |  |  |
| Industry - processes | Voluntary abandonment of filling tyres with SF6 by vehicle industry | Substitution of SF6 by air | SF6 | I | Proposed |  | 2.3 |  |  |
| Agriculture | Fertiliser Ordinance | Determination of proper practice in fertiliser use | N2O | R | Existing (1996) | Transposes EC Nitrate directive, Federal Government | 2.1 |  | yes (n) |
| Waste | Technical Instruction on Waste from Human Settlements and waste placement ordinance ("Technische Anleitung Siedlungsabfall und Abfallablagerungsverordnung") | Avoiding the development of gas in new landfills; energetic use of gas from old landfills | CH4 | R | Existing | Federal Government | 31.0 |  |  |
| Waste | Technical Instructions on Waste Management, Part 1 ("TA Abfall, Teil 1") | Reduction of CH4-emissions in organic part of hazardous waste | CH4 | R | Existing | Federal Government | 0.3 |  |  |

- Additional policies and measures:

| Sector | Name | Objective | GHG affected | Type of instrument | Status | Implementing entity | Estimate of savings (Mt CO2 equiv.) |  | CCPM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2010 | 2020 |  |
| Transport (new in MR 2002) | Value added tax (VAT) on fuel for the domestic share of international flight routes | Abolishment of the exemption of value added tax on all fuel used for international flights; VAT on fuel used for the domestic share of international flight routes | CO2 | E | Under governmental legislation process | Federal Government | 0.2 (by 2005) |  |  |
| Residential (new in MR 2002) | Subsidy on ecological construction ("Eigenheimzulage") | Extension and adjustment of subsidies on ecological construction of new buildings and also in the building stock | CO2 | E | Under governmental legislation drafting process | Federal Government | Not given |  |  |
| Industry - processes | Semi-conductor production ("Halbleiterfertigung") | Improving flue gas cleaning by 2000, replacement of old plants by high efficient plants by 2009 | HFC | V | Proposed | Industry | 0.1 |  |  |
| Industry - processes | Air conditioning in buildings ("Kälte-stationäre Klimatechnik") | As of 2003, mandates maintenance requirements for systems with fill amounts of at least 1 kg; to reduce leak rates | HFC | R | Proposed | Federal Government | 3.6 |  | yes (n) |
| Industry - processes | Air conditioning in cars ("Mobile Klimatechnik") | As of 2007, motor-vehicle HFC air conditioners are to be supplanted by CO2 systems; substitution of HFC by CO2 | HFC |  | Proposed | Federal Government, industry | 1.0 |  |  |
| Industry - processes | PU foam for construction ("PU-Montageschäume") | Substitution of HFC by propane or butane | HFC | R, E, V | Proposed | Federal Government | 2.6 |  |  |
| Industry - processes | PU foam products ("PU- Schaumprodukte") | Substitution of HFC by pentane or cyclopentane | HFC | V | Proposed | Industry | 0.2 |  |  |
| Industry - processes | XPS-rigid foam ("XPS-Hartschäume") | Substitution of HFC by CO2 or ethyl alcohol through all producers | HFC | V | Proposed | Industry | 1.0 |  |  |
| Industry - processes | Aerosol ("Dosieraerosole") | Substitution of aerosols by powder inhaler through market initiatives | HFC | V | Proposed | Industry | 0.5 |  |  |
| Industry - processes (increasing amount of assessed effect compared to MR2001) | Production of aluminium | Provides for additional modernisation measures and process optimisation | PFC | V | Proposed | Industry | 0.84 |  |  |
| Industry - processes | Semi-conductor production ("Halbleiterherstellung") | Provides for use of a new process for cleaning plasma chambres; use of NF3 as a substitute for PFCs in etching | PFC | V | Proposed | Industry | 1.3 |  |  |
| Industry - processes | Prohibition of SF6 in noise insulation windows ("Verwendungsverbot bei Schallschutzscheiben") | By 2005, SF6 is to be phased out in new soundproof windows. Aiming at the use of alternative window technologies that do not require SF6. | SF6 | R | Proposed | Federal Government | 1.0 |  |  |
| Industry - processes | Prohibition of filling tyres with SF6 ("Verwendungsverbot") |  | SF6 | R | Proposed | Federal Government | 0.7 |  |  |
| Waste (New in MR 2002) | Improvement of energy use in waste combustion | Promotion of waste combustion for electricity and heat production and extension of waste as co-fuel in other combustion processes | CO2 | E, R | Suggestion and examination | Federal Government, industry | 7.0 |  |  |

## Evaluation of Projections

The projections given in MR2002 do not fully include the recent policies and measures developments. The MR2002 notes that the projections are currently under revision.

Updated projections have been submitted by Germany’s Federal Environmental Agency in June 2003 as has been announced by Germany’s Environment Ministry in the Monitoring Mechanism committee meeting 31 March 2003.

Therefore, the figures in Tables 5 and 6 show the updated data taken from the draft “Politikszenarien III” report (See above sources of information), table 3.12-2.

The projections data are not in compliance with reporting guidelines of the Monitoring Mechanism. No split of greenhouse gases according the Kyoto basket is available. Only total greenhouse gases and CO2 emissions are reported.

- Table 5: Summary of projections by gas in 2010 (MtCO2-equiv.)

| Greenhouse gases | Base year | With measures | With additional measures |
| --- | --- | --- | --- |
| Carbon dioxide | not given | not given | not given |
| Carbon dioxide (excl. sinks) | 1014.4 | 861.7 | not given |
| CH4 | not given | not given | not given |
| N2O | not given | not given | not given |
| HFC | not given | not given | not given |
| PFC | not given | not given | not given |
| SF6 | not given | not given | not given |
| Total (excl. sinks) | 1218.2 | 977.8 | not given |
| Total (incl. sinks) | not given | not given | not given |
| % change relative to base year (excl. sinks |  | -19.7% |  |
| % change relative to base year (incl. sinks |  |  |  |

And there is no sectoral split of greenhouse gas emission projections provided.

- Table 6: Summary of projections by sector in 2010 (MtCO2-equiv.)

| Sectors/IPCC source and sinks categories | Base year | with measures | % change rel base year | with additional measures | % change rel base year (additional measures) |
| --- | --- | --- | --- | --- | --- |
| 1. Energy | not given | not given | not given | not given | not given |
| 1.A. Fuel Combustion | not given | not given | not given | not given | not given |
| 1.A.1. Energy Industries | not given | not given | not given | not given | not given |
| 1.A.2. Manufacturing Industries and Construction | not given | not given | not given | not given | not given |
| 1.A.3. Transport | not given | not given | not given | not given | not given |
| 1.A.4. Other Sectors | not given | not given | not given | not given | not given |
| 1.A.5. Other (Military) | not given | not given | not given | not given | not given |
| 1.B. Fugitive Emissions from Fuels | not given | not given | not given | not given | not given |
| 2. Industrial Processes | not given | not given | not given | 46.48 | -28% |
| 3. Solvents and Other Product Use | not given | not given | not given | not given | not given |
| 4. Agriculture | not given | not given | not given | not given | not given |
| 5. Land-Use Change and Forestry | not given | not given | not given | not given | not given |
| 6. Waste | not given | not given | not given | not given | not given |
| Total (without LUCF) | 1218.2 | 977.8 | -19.7 |  |  |

Table 7: Assessment of the Target

|  | - Excl. sinks |  | - Incl. sinks |  |
| --- | --- | --- | --- | --- |
|  | - MtCO2 equiv. | - % of 1990 level (six gas basket) | - MtCO2 equiv. | - % of 1990 level (six gas basket) |
| - Base year (from projections) | 1218.2 |  |  |  |
| - Commitment | 962.4 | 79 |  |  |
| - With existing P&Ms | 977.8 | 80.3 |  |  |
| - Gap (-ve means no gap) | +15.4 | +1.3 |  |  |
| - Effect of additional P&Ms | not given |  |  |  |

## Description of Modelling ApproacH

A new description of the modelling approach is not given in Monitoring Report 2002. Therefore, the following description is the same as in the last year’s Member State Analysis 2002.

The CO2 projection used in the Monitoring Report is the latest projection and is based on the study “Politikszenarien III” that is still under way. It uses the dynamic (MARKAL) version of the IKARUS model. This tool, a linear programming model, optimises simultaneously the use of energy in all sectors, assuming certain “bounds” (like boundary conditions of the economic system, limitations of resources, etc.) and assuming that individual optimisation leads to some overall inefficiencies. The model separates nine industry branches and four other sectors (transport, energy transformation, residential, and commercial & institutional). The application of the IKARUS model is done in combination with other (sector specific) bottom-up analyses based on expert opinions. Parameters for the updated projections cannot be given for the time being. In the table below parameters that have been used for the old projections assessment in the MR2002 are shown to gain a preliminary impression of Germany’s boundary conditions.

### Modelling parameters

| Parameter | 2000 (only 1995 data available) | 2010 | Unit |
| --- | --- | --- | --- |
| Population |  |  | Millions |
| GDP |  |  | GDP |
| Oil (International price) |  | 22.8 | USD (1995) / bbl |
| Coal (International price) | - | - | USD (2000) / tonne |
| Crude Oil (National price) | 4.36 | 6.47 | DM/GJ in prices of 1995 |
| Natural Gas, Type A (National Price) | 4.06 | 5.31 | DM/GJ in prices of 1995 |
| Coal (National Price) | 2.58 | 3.59 | DM/GJ in prices of 1995 |
| Transport Passenger growth | 900 | 1063 | 1000 million passenger times km |
| Freight growth | 413 | 723 | 1000 million tonnes times km |
| Housing units | 36 |  | Million units |
| Households | 36.9 |  | Million households |
| Living space/cap. | 36.8 |  | m2/cap. |
| Share of renewables |  | 2 1 / 4 2 | Percent of primary energy supply |
| Discount rate | 5 % | 5 % | Real prices over all sectors |
| Liberalisation considered? |  |  | Yes, according to DIW |

Source: Member State Analysis 2002

These economic and demographic factors are exogenic data and form the base for demand values determining energy consumption. These energy consumption values, in turn, form the endogenic part of the model. The same applies for national energy prices: they are determined by the import prices and the outcome of the optimisation simulations.

Strengths of the approach:

- Consistent model based analysis of the national energy system, including emissions and costs,

- Identification of cost efficient technologies,

- Modification of model results by experts, with regard to real behaviour of firms and households, institutional factors, and the impact of policies and measures.

Weaknesses of the approach:

- Only „soft“ link to the economic model (MIS),

- Restricted to the analysis of energy related emissions or CO2

- The optimisation model (by itself) does not simulate or predict the influence of policies on human behaviour.

Incorporation of estimated effects of P&Ms into the projections:

The following figure illustrates the interaction of estimation methods and sources in developing a comprehensive approach that combines overall scenarios with estimated effects of individual P&Ms.

Within this framework, the optimisation model makes a special contribution towards identifying sectoral action fields in the sense of cost-efficient technology options. Other important factors are general economic and demographic data and the technology options available in the period under consideration characterised by specific costs, technical parameters and specific emissions.

Paying attention to these technology-oriented action fields, expert judgements have the aim of quantifying the impacts of appropriate policy measures on sectoral energy consumption and emissions. An essential prerequisite is the most accurate possible description of the policy options to be analysed.

The methodology used is described in “Policy Scenarios for Climate Protection, Volume 4: Methodological Guideline for Assessing the Impact of Measures for Emission Mitigation"; Study commissioned by the Federal Environment Ministry; authors: DIW, FZJ, Fraunhofer-ISI, Öko-Institut, 1998

## Country Conclusions

Germany’s Monitoring report 2002 (MR2002) was submitted in January 2003. It took into account most recent policies and measures which came into force until end of 2002 or were decided in the coalition agreement of the new government after elections for the German Bundestag in September 2002.  However, the effect of most of the new policies and measures has been not estimated in that report, yet. Preliminary updates of Germany’s GHG projections were presented in June 2003 but they were very limited.

Germany’s GHG emissions are projected preliminarily to be reduced to about 978 Mt CO2-equiv. in 2010 (See Table 7).  This means a shortfall of about 15 Mt CO2-equiv. in 2010 relative to the burden sharing commitment (-21% until 2010 for all six “Kyoto gases” relative to base year emissions), i.e. the “With measures” projections are about 1.3 percent points above the target. For that estimate the recent policy developments until the end of 2002 are assumed. All policies and measures (PAMs) in place are taken into account. Their originally expected effect in the German Climate Change program adopted by the Government in 2000 has been revised according the recent developments. The revision of projections replaces previously reported over-delivery of 144 Mt CO2-equiv. or 12 percent points below the target by a shortfall of 15 Mt CO2-equiv., i.e. 1.3 percent points above the target.

A comparison of the emission reductions achieved by the ETS and the reductions projected previously cannot be conducted because no sectoral split of projections is available consistent with Germany’s projections update of June 2003 (Table 6).

No “With additional measures” projection is given for the time being.

The MR2002 contains detailed policies and measures that aim to fulfil Germany's Kyoto commitments. The description of policies and measures has been improved compared to the MR2001. Further improvement could be made with information on whether each policy or measure is connected to EU-wide Common and Coordinated Policies and measures (CCPM), as the Monitoring Mechanism guidelines request. The authors of this report attempted to allocate national policies and measures to CCPMs.

The sectoral split of the policies and measures reported in the MR2002 are different compared to the projections and therefore, also different to the IPCC source categories. A reallocation of policies and measures to IPCC categories was attempted and documented in Table 4 by the authors of this report and the savings from policies and measures were estimated by IPCC categories (see Table 3).

Annex I:

List of Common and co-ordinated policies and measures (CCPMs)

(Conclusions of the Council, June 1998)

Abbreviations used in the analysis

Common measures:

A	reduction of CO2 emissions from cars to 120 g CO2 per km by means of:

A I	agreement with the automotive industry on energy efficiency in new cars, including a monitoring system

A II	energy labelling of cars

A III	financial incentives for energy-efficient cars

A IV	nitrous oxide from catalysers in vehicles

B	common minimum levies on energy products, including taxes on fuel for aircraft and ferries

C	lowering of subsidies for fossil fuels

D	improved and dynamic demands for energy efficiency in appliances in the form of standards, labelling or agreements

E	encourage the spreading of energy-efficient technologies and techniques, e.g., by differentiated levies on energy-economising products

F	waste handling

G	action plan for methane

H	encouragement of renewable energy in a liberalised power market

Co-ordinated policies and measures:

I	stimulate the market for CHP and remove barriers for a significant rise in use of CHP in the EU

J	encourage environmental targets in the liberalisation of power and natural gas markets

K	increased use of renewable energy through co-operation and co-ordinated policies and measures

L	influence transport needs towards types of transport which are less stressful to the environment

M	standards for energy efficiency in buildings

N	other

Annex II:

Type of instruments

D	research and development

E	economic instrument

ET	education and training

F	fiscal

I	information

R	regulation, law, guideline

V	voluntary agreement

For CO2, CH4 and N2O the base year is 1990; for HFCs, PFCs and SF6 the base year is 1995.

For CO2, CH4 and N2O the base year is 1990; for HFCs, PFCs and SF6 the base year is 1995.