# Greece

## Sources of Information

The information in this appendix is based on:

- Third National Communication to the UNFCCC, 2003.

- Ministry for the Environment, Physical Planning and Public Works, Climate Change:  Emissions projections, policies and measures, National Observatory of Athens, February 2004

## Quality and Transparency of Reporting

- The evaluation of the quality and transparency of reporting is based on the information provided in the February 2004 report on emissions projections, policies and measures.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - Some | - Detailed policy names not always provided |
| - Objectives of policies | - Yes |  |
| - Which greenhouse gases? | - All |  |
| - Status of Implementation | - +++ | - Policies clearly identified as implemented, adopted or planned |
| - Implementation body specified | - +++ |  |
| - Quantitative assessment of implementation | - +++ | - The effect of policies is quantified |
| - Interaction with other P&Ms discussed | - - | - No information on interaction with other policies |

- +, ++, +++ level of information available increases as the number of + signs increases

- The February 2004 report on projections and policies and measures presents a with measures projection and also contains quantification on the effect of additional policies and measures.  Less detail on policies and measures is provided in this report than in the 3NC.

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - ++ | - With measures projection presented & quantification of additional measures |
| - Expressed relative to inventory for previous years | - Not stated |  |
| - Starting year | - 2000 | - First year for which projections are given |
| - Split of projections | - +++ (WM) - + (WAM) | - Most detail given for with measures projection |
| - Presentation of results | - +++ | - Results are clearly presented |
| - Description of model (level of detail, approach and assumptions) | - ++ | - Good overview of model used for energy sector, including strengths & weaknesses. Less detail for other sectors. |
| - Discussion of uncertainty | - - | - No discussion of uncertainty |
| - Details of parameters and assumptions | - ++ | - Parameters and assumptions provided for with measures scenario |

- +, ++, +++ level of information available increases as the number of + signs increases

## Assessment of Policies and Measures

The February 2004 updates the information presented in the 3NC on the effects of both implemented/adopted and planned policies and measures.

- Table 3: Summary of the Effect of Policies and Measures Included in the Projections (MtCO2)

|  | With measures | With additional measures* |
| --- | --- | --- |
| Energy supply | 14.0 | 7.6 |
| Industry | 2.8 | 0.6 |
| Domestic & Tertiary | 2.1 | 3.8 |
| Transport | 0.4 | 0.9 |
| Industrial processes |  | 4.7 |
| Waste | 5.9 | 0.1 |
| Agriculture |  | 0.1 |
| Total | 25.2 | 17.7 |

*The figure presented here for the effect of additional policies and measures does not take account of interactions between them.

- Table 4: Detailed Information on Polices and Measures

| Sector | Name | Objective | GHG affected | Type of instrument |  | Status | Implementing entity | Estimate of savings (MtCO2) |  | ### CCPM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 2010 | 2020 |  |
| Implemented and adopted policies and measures (in the with measures projection) |  |  |  |  |  |  |  |  |  |  |
| Energy supply | Promotion of natural gas |  |  |  |  |  |  | 15.1 |  |  |
|  | Operational Programme for Energy (OPE) | Support for natural gas power plants | CO2 |  | Economic | Implemented | PPC | 6.1 |  | no |
|  | Law 2773/99 | Liberalisation of electricity market | CO2 |  | Economic | Adopted | PPC/Private | 4.4 |  | yes |
|  |  | Electricity generation from industrial autoproducers using natural gas | CO2 |  | Economic | Adopted | MD | 1.9 |  |  |
|  |  | Cogeneration | CO2 |  | Economic | Implemented | MD | 0.1 |  |  |
|  |  | Cogeneration | CO2 |  | Economic | Adopted | MD | 0.6 |  |  |
|  | OPE and Development Law 2601/98 | Natural gas in industry | CO2 |  | Economic | Implemented | MD | 0.9 |  | no |
|  | Law 2364/95 | Tax exemptions for natural gas systems in residential/tertiary sectors |  |  |  |  |  | 1.1 |  | no |
|  | Athens Urban Transport Association | Purchase of CNG buses | CO2 |  | Economic (public investments) | Implemented | MTT | 0.006 |  | no |
| Energy supply | Improvements in the conventional power generation system |  |  |  |  |  |  | 0.55 |  |  |
|  | Promotion of combined heat and power systems | Setting up district heating system in northern Greece | CO2 |  | Economic (PPC investments) | Implemented | PPC | 0.025 |  | no |
|  | Limitation of distribution losses | Includes replacement of normal loss distribution transformers | CO2 |  | Economic (PPC investments) | Implemented | PPC | 0.106 |  | no |
|  | Efficiency improvements in existing lignite power stations | Enhanced operation of cooling towers, lignite mills, turbines etc | CO2 |  | Economic (PPC investments) | Implemented | PPC | 0.419 |  | no |
| Energy supply | Promotion of renewable energy sources |  |  |  |  |  |  | 3.2 |  |  |
|  | OPE and Development Law, plus Law 2244/93 (favourable by back tariffs for RE) | Wind energy | CO2 |  | Economic, regulatory | Implemented | RAE/MD/Private | 0.99 |  | no |
|  | OPE and Development Law, plus Law 2244/93 (favourable by back tariffs for RE) | Small hydro | CO2 |  | Economic | Implemented | RAE/MD/Private | 0.144 |  | no |
|  | OPE and Development Law, plus Law 2244/93 (favourable by back tariffs for RE) | Large hydro | CO2 |  | Economic (PPC investments) | Implemented | RAE/MD/Private | 0.826 |  | no |
|  | OPE and Development Law, plus Law 2244/93 (favourable by back tariffs for RE) | Photovoltaics | CO2 |  | Economic | Implemented | RAE/MD/Private | 0.003 |  | no |
|  | OPE and Development Law, plus Law 2244/93 (favourable by back tariffs for RE) | Biomass | CO2 |  | Economic, regulatory | Implemented | RAE/MD/Private | 0.273 |  | no |
|  | OPE and Development Law, plus Law 2244/93 (favourable by back tariffs for RE) | Solar energy in residential sector | CO2 |  | Economic | Implemented | MEPPPW | 0.951 |  | no |
|  | OPE and Development Law, plus Law 2244/93 (favourable by back tariffs for RE) | Solar energy in tertiary and energy sectors | CO2 |  | Economic | Implemented | MEPPPW | 0.004 |  | no |
| Residential & tertiary sectors | Energy conservation |  |  |  |  |  |  | 0.01 |  |  |
|  | Presidential decrees 335/1993 and 178/1998 | Minimum standards for non-industrial boilers & refrigerators/freezers | CO2 |  | Regulation | Implemented |  |  |  | yes |
|  | Presidential decree 180/1994 | Energy labelling | CO2 |  | Information | Implemented |  |  |  | yes |
|  | Promotion of CFLs | Replacement of incandescent lamps | CO2 |  | Economic | Implemented | MD | 0.011 |  | no |
| Transport | Interventions in vehicles |  |  |  |  |  |  | 0.4 |  |  |
|  | ACEA agreement | Improve fuel efficiency of new cars | CO2 |  | Voluntary agreement | Implemented | MTT | 0.396 |  | yes |
|  | Law establishing private Centres for Technical Control | Regular vehicle inspections | CO2 |  | Regulation | Implemented |  |  |  | no |
|  | Exhaust control card | Exhaust emission inspections | CO2 |  | Regulation | Implemented |  |  |  | no |
|  | Law 1682/99 | Differentiated vehicle taxation according to pollution control technology | CO2 |  | Regulation | Implemented |  |  |  | no |
| Transport | Interventions in transport system | Enhancement of infrastructure | CO2 |  |  |  |  |  |  | no |
|  | Interventions in public transport | New metro lines, improved efficiency of buses, priority lanes for buses | CO2 |  |  |  |  |  |  | no |
| Waste | Waste management |  |  |  |  |  |  | 5.9 |  |  |
|  |  | Landfill Directive | CH4 |  | Regulatory | Adopted | MEPPPW | 5.9 |  | yes |

Planned policies and measures

| Energy supply | Further promotion of natural gas | Operation of gas plants as baseload, expansion of natural gas use in demand sectors | CO2 | Economic, regulatory | Planned | MD/RAE/PPC/MEPWPP/MTT | 3.4 |  | no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Further promotion of renewable energy sources | Covers wind, small hydro, PV, geothermal, biomass, biofuels, solar | CO2 | Economic, regulatory | Planned | MD/RAE/private/MTT/MEPWPP | 6.4 |  | no |
| Industry | Energy conservation | Both thermal & electricity | CO2 | Economic, regulatory | Planned | MD | 0.3 |  | no |
| Residential/tertiary | Energy conservation | Thermal improvements in buildings. Promotion of RES. Promotion of energy efficiency appliances. Further penetration of gas. Other energy conservation | CO2 | Economic, regulatory | Planned | MEWPP/MD | 2.3 |  | no |
| Transport | Various | Energy efficiency improvements (by 2%) in vehicles. Fuel switching. Transport management including improvements in public transport | CO2 | Regulatory, economic (public investments) | Planned | MTT | 0.6 |  | no |
| Waste | Waste management | Flaring of landfill gas | CH4 | Regulatory, economic (public investments) | Planned | MEPWPP | 0.1 |  | no |
| Industrial processes | Various | Restructure chemical industry & recover f-gases from discarded equipment | HFCs | Voluntary agreement, regulatory | MD | MD | 4.7 |  | no |
| Agriculture | New practices | Manure management systems & organic farming | CH4, N2O | Economic, regulatory | Planned | MA | 0.1 |  | no |

## Evaluation of Projections

- Table 5: Summary of projections by gas in 2010 (MtCO2)

|  | Base year* | with measures | with additional measures |
| --- | --- | --- | --- |
| Carbon dioxide | 84.3 | 122.5 |  |
| Carbon dioxide (excl. sinks) | 82.7 | 120.5 |  |
| Methane | 9.0 | 8.8 |  |
| Nitrous oxide | 14.1 | 14.9 |  |
| HFCs | 3.4 | 7.2 |  |
| PFCs | 0.1 | 0.1 |  |
| SF6 | 0.0 | 0.0 |  |
| Total (excl. sinks) | 109.3 | 151.4 | 133.7 |
| Total (incl. sinks) | 110.9 | 153.4 | 135.7 |
| % change relative to base year (excl. sinks) |  | 38.6% | 22.4% |
| % change relative to base year (incl. sinks) |  | 38.4% | 22.4% |

*base year for F-gases is 1995

With currently implemented and adopted policies and measures, the with measures scenario shows that greenhouse emissions increase will be almost 39 % above base year levels in 2010.  The effect of planned policies is to limit this increase to just over 22 %, which is slightly lower than the commitment of Greece under the EU burden sharing agreement (25%).

Table 6: Summary of projections by sector in 2010 (MtCO2)

| 6 GHGs | Base year (MtCO2) | With measures (MtCO2) | % change relative to 1990 (with measures) | with additional measures (MtCO2) | % change relative 1990 (additional measures) |
| --- | --- | --- | --- | --- | --- |
| Energy (total) | 81.0 | 119.6 | 47.6% | 106.7 | 31.7% |
| Energy supply | 44.0 | 62.7 |  |  |  |
| Industry | 12.5 | 15.6 |  |  |  |
| Domestic | 5.1 | 9.0 |  |  |  |
| Commerce | 0.6 | 1.4 |  |  |  |
| Agriculture | 3.2 | 3.3 |  |  |  |
| Transport | 15.6 | 27.5 |  |  |  |
| Industrial processes | 10.5 | 15.8 | 50.0% | 11.1 | 5.3% |
| Waste | 4.2 | 3.4 | -17.8% | 3.3 | -20.2% |
| Agriculture | 13.6 | 12.6 | -7.0% | 12.5 | -7.8% |
| Sinks | 1.6 | 2.0 | 26.3% | 2.0 | 26.3% |
| Total (without sinks) | 109.3 | 151.4 | 38.6% | 133.7 | 22.4% |
| Total (with sinks) | 110.9 | 153.4 | 38.4% | 135.7 | 22.4% |

Table 7: Assessment of the Target

|  | - Excl. sinks |  | - Incl. sinks |  |
| --- | --- | --- | --- | --- |
|  | - MtCO2 equiv. | - % of 1990 level (six gas basket) | - MtCO2 equiv. | - % of 1990 level (six gas basket) |
| - Base year (from projections) | 109.3 | 100.0 | 110.9 | 100.0 |
| - Commitment | 136.6 | 125.0 | 138.6 | 125.0 |
| - With existing P&Ms | 151.4 | 138.6 | 153.4 | 138.4 |
| - Gap (-ve means no gap) | 14.8 | 13.6 | 14.8 | 13.4 |
| - Effect of additional P&Ms | 17.7 | 16.2 | 17.7 | 16.0 |
| - Remaining gap (-ve means no gap) | -2.9 | -2.6 | -2.9 | -2.6 |

## Description of Modelling ApproacH

The energy sector in Greece is modelled using the Energy and Power Evaluation Program (ENPEP), developed by Argonne National Laboratory.  The model contains a set of analytical tools for use in integrated energy/electricity system planning and the quantification of environmental burdens. Its basic module (BALANCE) is used to trace the flow of energy throughout the entire energy system from resource extraction, through processing and conversion, to meet demands for useful energy (e.g. heating, transportation, electrical appliances) and employs a market-based simulation approach to project future energy supply/demand balances.

The ENPEP model uses a non-linear, equilibrium approach to determine the energy supply demand balance.  A fundamental assumption of the model is that producers and consumers both respond to changes in price.  For all energy supply forms and energy uses ENPEP finds a set of equilibria (given by a set prices and quantities) that satisfy all relevant equations and restrictions.

The data that are necessary to calibrate the model for a base year as well as to project the future energy needs are:

- Macro-economic data that correspond to demographic national accounts, sectoral activity and income variables.

- Structure of energy consumption in the base year and structure of activity variables (production, dwellings, passenger-kilometres, etc.).

- Technical-economic data for technologies and sub-sectors (e.g. capital cost, unit efficiency, variable costs, lifetime, etc.).

The Greek Energy system is presented in ENPEP model by sub-systems and sectors, which

covers the main economic and energy activities; including energy supply, energy conversion and final demand.

The strength of the approach is that it allows for a comprehensive assessment of the various interactions between the different sectors of the energy system. The market-based equilibrium together with the detailed technical description of the energy sectors and uses enables the realistic representation of the energy system as well as the modelling of different policy instruments. However, the solution obtained is closely related to the level of detail of the developed energy network. The main limitations of the model are that useful energy demand is not endogenously adjusted to changes of prices and market simulation parameters are set exogenously.

Greenhouse gases emissions in the non-energy sectors are calculated using simplified spreadsheet models that calculate emissions based on activity data, emission factors and sector specific assumptions about the growth of activities and the degree of emissions control.

In estimating the growth factor, time-series analysis and/or regression analysis using an appropriate determinant parameter of the available activity data is used. Potential determinant parameters include population; value added, product output, etc.  The control factor accounts for changes in emission factors. In determining the future year emission factor, three basic parameters must be quantified: regulation control, rule effectiveness, and rule penetration.  Regulation control is the level of reduction expected by assuming a fully complied measure. Rule effectiveness accounts for the level of expected compliance with the regulation.  Rule penetration indicates the fraction of emissions within a source category, which are subject to the regulation, accounting for possible exemptions. These parameters are quantified by experts' assessments in close consultation with the responsible governmental departments.

The main drawback of this analysis is that the models do not take into account any overlaps or synergies between sectors or policy areas.  In addition, the extrapolation of historical relationships overlooks the effects of faster technological advance.

Parameters used in the with measures scenario are given in the Table below.

Modelling parameters

| Parameter | 1990 | 1995 | 2000 | 2000-2005 | 2005-2010 | 2010-2015 | 2015-2020 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Population (M) | 10.2 | 10.5 | 10.9 | 0.6% | 0.4% | 0.3% | 0.2% |
| Household size | 3.2 | 3.1 | 3 | -0.7% | -0.8% | -0.8% | -0.8% |
| GDP (bn Euro 2000) | 75.1 | 79.9 | 94.6 | 3.9% | 3.4% | 3.0% | 2.9% |
| International fuel prices |  |  |  |  |  |  |  |
| Coal ($90/t) | 51.3 | 40.6 | 28.5 | 31.2 |  |  |  |
| Oil ($90/bbl) | 22.2 | 17.2 | 22.8 | -4.8% | 0.0% | 1.8% | 1.7% |
| Natural gas ($90/toe) | 119.1 | 92.5 | 90.1 | -4.8% | 0.0% | 1.8% | 1.7% |
| Transport activity |  |  |  |  |  |  |  |
| Passenger transport (bn pkm) | 83.6 | 100.6 | 118.1 | 3.3% | 3.0% | 2.6% | 2.3% |
| Goods transport (bn tkm) | 18.1 | 22.6 | 26 | 2.6% | 2.5% | 2.2% | 2.0% |

## Country Conclusions

The February 2004 report on projections, policies and measures from Greece has provided comprehensive information in most of the areas necessary for evaluation under the Monitoring Mechanism.  The information is presented in a similar way to the 3NC and is clearly set out, with good use of tables.  One area that could be improved further is in the reporting of the with additional measures projection, which unlike the 3 NC is not provided as a separate projection, but rather has to be derived from the aggregated effect of a set of additional policies and measures.   Since these policies and measures have been quantified separately without taking into account any interaction between them, it is likely that the aggregated effect will be less than the sum of the individual measures, but there is no information presented on the size of this effect.

The new projections show that with existing policies and measures, emissions will be almost 39 % higher in 2010 than in the base year, compared to the target of a 25 % increase.  While planned policies and measures have been identified to meet (and perhaps exceed) the gap, it will be important for both the existing and new policies to deliver the expected savings if Greece is to meet its commitment under the EU burden sharing agreement.