# Czech Republic

## Sources of Information

For policies and measures: Czech Republic’s Third National Communication (NC3) under the United Nations Framework Convention on Climate Change, 2001.

For projections: GHG Emission Projections update (made in September 2003) as in the National Action Plan 2004 approved by the Czech Government on 3 March 2004 (Gov. Decision No. 187/2004). Only short summary in English with main assumptions and resulting tables with emission estimates (2005-2020) were available.

Note that the updated projections in the National Action Plan 2004 differ from projections in NC3.

## Reporting

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Name of measure | - +++ |  |
| - Target and/or affected activity | - +++ |  |
| - Type of measure | - ++ |  |
| - Which GHGs? | - CO2, CH4, N2O, all GHGs |  |
| - Status of Implementation | - +++ |  |
| - Implementation body specified | - +++ |  |
| - Quantitative assessment of implementation | - + | - Only some measures are quantified |
| - Interaction with other P&Ms discussed | - ++ | - In some cases |

- +, ++, +++ level of information available increases as the number of + signs increases

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - Reference, low and High scenarios with measures, reference scenario with additional measures | - Scenarios are given for key parameters, and national totals of CO2, CH4 and N2O, sectors with measures only |
| - Expressed relative to inventory for previous years | - no |  |
| - Starting year | - 2000? | - Not clear |
| - Projections | - 2005, 2010, 2015, 2020 |  |
| - Split of projections | - +++ | - Projections split by IPCC main sectors and gases (not for reference scenario with additional measures) |
| - Presentation of results | - ++ | - Results presented in tabular form |
| - Description of model (level of detail, approach and assumptions) |  | - no description of the models and further references were provided in the English summary of the National Action Plan 2004 |
| - Discussion of uncertainty |  | - no information provided in the English summary of the National Action Plan 2004 |
| - Details of parameters and assumptions | - + | - no information on parameters used in scenarios was provided in the English summary of the National Action Plan 2004; main assumptions are listed |

- +, ++, +++ level of information available increases as the number of + signs increases

## Policies and Measures

A number of measures are carried out in the Czech Republic to mitigate emissions of greenhouse gases. These measures are concentrated on a specific aspect or sector and also include framework measures. The targets and consequences of a number of adopted measures can, however, be much broader, because it is particularly necessary to decrease detrimental impacts on the environment as a whole. Key measures with the greatest expected benefit include the following framework multi-sectoral measures:

- adopting a Strategy for protection of the Earth’s climate system in the Czech Republic,

- inclusion of the aspect of protection of the climate and incorporation of a National Program to Mitigate Changes in the Climate of the Earth in the newly prepared air protection legislation,

- adopting a new Energy Act and new Act on energy management.

Measures included in the Third National Communication can be divided into (a) implemented measures and (b) measures being prepared, classified according to the

relevant sector into:

- framework/multi-sector measures,

- measures in the sector of energy production and energy consumption,

- measures in the transport sector,

- measures in the industrial sector,

- measures in the sector of agriculture and forestry,

- measures in the waste management sector.

Additional measures should contribute to decreased emissions through implementation of  the new Air Act (01.01.2002), implementation of Directive 96/61/EC on IPPC and new act on waste and packaging

- Table 3: Summary of the Effect of Policies and Measures by 2010 included in the Projections (MtCO2  eq.)

|  | with measures - reference scenario | with measures - low scenario | with measures - high scenario | with additional measures - reference scenario |
| --- | --- | --- | --- | --- |
| CO2 (incl. sinks) | -50,6 | -54,6 | -45,1 | -60,7 |
| CO2 (excl. sinks) | -48,1 | -52,1 | -42,5 | -164,0 |
| CH4 | -7,2 | -7,3 | -7,6 | -7,3 |
| N2O | -2,9 | -2,6 | -2,8 | -3,3 |
| F-gases | 0,670 | 0,624 | 0,714 | 0,670 |
| Total by gas (excl. sinks) | -57,5 | -61,4 | -52,3 |  |
| Total by gas (incl. sinks) | -60,0 | -63,9 | -54,8 | -70,5 |
| Energy (IPCC Sector 1) | -52,6 | -56,6 | -47,9 |  |
| - of which transport | ? |  |  |  |
| Industry (IPCC Sector 2) | -0,2 | -0,3 | 0,1 |  |
| Solvents (IPPC Sector 3) | -0,2 | -0,2 | -0,2 |  |
| Agriculture (IPCC Sector 4) | -4,4 | -4,4 | -4,4 |  |
| Forestry (IPCC Sector 5) | -2,5 | -2,5 | -2,6 |  |
| Waste (IPCC Sector 6) | -0,1 | -0,1 | 0,1 |  |
| Total by sector (excl. sinks) | -57,6 | -61,8 | -52,3 |  |
| Total by sector (incl. sinks) | -60,0 | -64,2 | -60,0 | -70,5 |

Note: Here the effect of policies and measures is estimated compared to base year (1990) rather than to baseline scenario; figures based on projections given in National Action Plan 2004

Table 4 gives details of individual policies and measures.

Table 4 Detailed Information on Polices and Measures (estimated mitigation effect in 2010, in million tons CO2 eq.)

## Projections

Short scenario description:

- high scenario – GDP average annual increase by 4 % in 2000 – 2030

- reference scenario - GDP average annual increase by about 3.5 % in 2000 – 2030 (most plausible)

- low scenario - GDP average annual increase less then 3 % in 2000 – 2030

Assumptions valid for all three scenarios:

- existing nuclear power plant Temelin continuously in operation during the all projection period;

- existing nuclear power plant Dukovany will be reconstructed and continuously in operation during the all projection period;

- since 2004 no limits for oil, gas and coal will be applied;

- annual export of electric energy up to 10 TWh by 2010 and 5 TWh after 2010;

- existing mining lignite limit for during the all projection period in place.

Additional policies and measures included in NAP 2004 are as follows:

- implementation of National program to support energy saving and to promote use of renewables (as follow-up to the Act No.406/2000 on energy saving);

- ecological tax-reform; and

- more favourable purchasing tariffs for renewable energy.

- Table 5: Summary of projections by gas in 2010 (MtCO2eq.)

|  | Base year 1990 | with measures - reference scenario | with measures - low scenario | with measures - high scenario | with additional measures - reference scenario |
| --- | --- | --- | --- | --- | --- |
| Carbon dioxide (incl. sinks) | 161,9 | 111,3 | 107,3 | 116,7 | 101,2 |
| Carbon dioxide (excl.sinks) | 164,0 | 115,9 | 111,8 | 121,4 |  |
| Methane | 16,8 | 9,5 | 9,5 | 9,1 | 9,5 |
| Nitrous oxide | 11,3 | 8,4 | 8,7 | 8,4 | 8,0 |
| HFCs | 0,002 | 0,618 | 0,585 | 0,650 | 0,618 |
| PFCs | 0,000 | 0,016 | 0,014 | 0,018 | 0,016 |
| SF6 | 0,167 | 0,205 | 0,194 | 0,215 | 0,205 |
| Total (excl. sinks) | 192,2 | 134,7 | 130,8 | 139,9 |  |
| Total (incl. sinks) | 190,1 | 130,0 | 126,2 | 135,2 | 119,5 |
| % change rel.to base year (excl. sinks) |  | -29,9% | -31,9% | -27,2% |  |
| % change rel.to base year (incl. sinks) |  | -31,6% | -33,6% | -28,9% | -37,1% |

- Note: 1990 data based on CRF 2003-2001v2 table 10.; 1995 used as base year for F-gases

- Note: data for the base year do not include emission from international bunkers

- Note: data for WM scenarios and WAM - reference scenario do not include emission from international bunkers

- Note: Total CO2 eq. [Mt] figure for 1990 in Tab. A.1 of Summary of the GHG Emission Projections 2004 update includes int'l bunker and excludes F-gas emission

- The with measures projections show that the currently implemented or adopted measures in Czech Republic could reduce greenhouse gas emissions by 2010 by 31.6% in the reference scenario, thus meeting their commitment.  Even if high growth in the economy is achieved, emissions are projected to decrease in 2010 by 28.9% compared to 1990. In case of the low scenario the expected reduction would be 33.6%.

- Additional measures are estimated to bring emission reduction by 37.1 % in 2010.

- Table 6:  Summary of projections by sector in 2010 (MtCO2e)

|  | Base year 1990 | scenario type | with measures | with additional measures |
| --- | --- | --- | --- | --- |
| Energy | 171,018 | low | 114,4 |  |
|  |  | reference | 118,4 |  |
|  |  | high | 123,2 |  |
| Industrial processes | 4,877 | low | 4,5 |  |
|  |  | reference | 4,7 |  |
|  |  | high | 5,0 |  |
| Solvents | 0,734 | low | 0,5 |  |
|  |  | reference | 0,5 |  |
|  |  | high | 0,5 |  |
| Agriculture | 12,521 | low | 8,1 |  |
|  |  | reference | 8,1 |  |
|  |  | high | 8,1 |  |
| LUCF | -2,073 | low | -4,5 |  |
|  |  | reference | -4,6 |  |
|  |  | high | -4,7 |  |
| Waste | 2,983 | low | 2,8 |  |
|  |  | reference | 2,9 |  |
|  |  | high | 3,1 |  |
| Total (excl. CO2 sinks) | 192,2 | low | 130,4 |  |
|  |  | reference | 134,6 |  |
|  |  | high | 139,9 |  |
| Total (incl. sinks) | 190,1 | low | 125,9 |  |
|  |  | reference | 130,0 | 119,5 |
|  |  | high | 135,2 |  |

- Note: 1990 data based on CRF 2003 - 2001v2

- Note: data for Industrial processes include F-gas emission in 1995 and 2010 respectively

- Note: data for LUCF include CO2 sinks and emissions of CH4 and N2O

Table 7: Assessment of the Target

|  |  | excluding sinks |  | with sinks |  |
| --- | --- | --- | --- | --- | --- |
| Reference scenario | scenario | MtCO2e | % 1990 level | MtCO2e | % 1990 level |
| Base year 1990 (from projections) |  | 192,2 | 100,0 | 190,1 | 100,0 |
| Commitment |  | 176,8 | 92,0 | 174,9 | 92,0 |
| With existing P&Ms | low | 130,4 | 68,8 | 125,9 | 67,2 |
|  | reference | 134,6 | 70,0 | 130,0 | 68,4 |
|  | high | 139,9 | 72,8 | 135,2 | 72,1 |
| Gap | low | -46,4 | -24,1 | -49,0 | -25,8 |
|  | reference | -42,2 | -22,0 | -44,8 | -23,6 |
|  | high | -36,9 | -19,2 | -39,7 | -20,9 |
| Effect of additional P&Ms | low |  |  |  |  |
|  | reference |  |  | 10,5 | 5,5 |
|  | high |  |  |  |  |
| Remaining gap | low |  |  |  |  |
|  | reference |  |  | -55,3 | -29,1 |
|  | high |  |  |  |  |
| low scenario |  |  |  |  |  |
| Base year 1990 (from projections) |  | 192,2 | 100,0 | 190,1 | 100,0 |
| Commitment |  | 176,8 | 92,0 | 174,9 | 92,0 |
| With existing P&Ms |  | 130,4 | 67,9 | 125,9 | 66,2 |
| Gap |  | -46,4 | -24,1 | -49,0 | -25,8 |
| Effect of additional P&Ms |  |  |  |  |  |
| Remaining gap |  |  |  |  |  |
| reference scenario |  |  |  |  |  |
| Base year 1990 (from projections) |  | 192,2 | 100,0 | 190,1 | 100,0 |
| Commitment |  | 176,8 | 92,0 | 174,9 | 92,0 |
| With existing P&Ms |  | 134,6 | 70,0 | 130,0 | 68,4 |
| Gap |  | -42,2 | -22,0 | -44,8 | -23,6 |
| Effect of additional P&Ms |  |  |  | 10,5 | 5,5 |
| Remaining gap |  |  |  | -55,3 | -29,1 |
| high scenario |  |  |  |  |  |
| Base year 1990 (from projections) |  | 192,2 | 100,0 | 190,1 | 100,0 |
| Commitment |  | 176,8 | 92,0 | 174,9 | 92,0 |
| With existing P&Ms |  | 139,9 | 72,8 | 135,2 | 71,1 |
| Gap |  | -36,9 | -19,2 | -39,6 | -20,9 |
| Effect of additional P&Ms |  |  |  |  |  |
| Remaining gap |  |  |  |  |  |

## Description of Modelling ApproacH

Only short summary in English with main assumptions behind the projections and resulting tables with emission estimates (2005-2020) were available.

Main assumptions for "with measures" and "with additional measure" scenarios

| Statistics | Projection |
| --- | --- |
|  | 2000-2030 |
| Inter-annual change of GDP (%) |  |
| High scenario | 4 |
| Reference scenario | 3,5 (most plausible) |
| Low scenario | less than 3 |
| Assumptions valid for all three scenarios: |  |
| existing nuclear power plant Temelin continuously in operation during the all projection period; |  |
| existing nuclear power plant Dukovany will be reconstructed and continuously in operation during the all projection period; |  |
| since 2004 no limits for oil, gas and coal will be applied; |  |
| annual export of electric energy up to 10 TWh by 2010 and 5 TWh after 2010; |  |
| existing mining lignite limit for during the all projection period in place. |  |
| Additional policies and measures included in NAP 2004 are as follows: |  |
| implementation of National program to support energy saving and to promote use of renewables (as follow-up to the Act No.406/2000 on energy saving); |  |
| ecological tax-reform and more favourable purchasing tariffs for renewable energy |  |

## Country Conclusions

The Czech Republic project that with existing measures the Kyoto Commitment will be exceeded, even with assumptions of high growth.  Nevertheless, additional measures to deliver further emissions reductions have been identified.  The ‘reference scenario’ is taken as the main ‘with measures’ projection for the purpose of this analysis.  Under this scenario the emissions for 2010 are predicted to be lower than the commitment by 22%, thus comfortably meeting the Kyoto target.